IEEE P802.11
Wireless LANs

| dot11AssociationResponseTimeOut Negotiation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | SRC Soft | Fukuoka, JAPAN |  | hmorioka@src-soft.com |

Abstract

This text is a proposed resolution for CID4504.

# 6. Layer Management

## 6.3 MLME SAP interface

Modify clause 6.3.5.3 as following:

## 6.3.5.3 MLME-AUTHENTICATE.confirm

MLME-AUTHENTICATE.confirm(

PeerSTAAddress,

AuthenticationType,

ResultCode,

Content of FT Authentication elements,

Content of SAE Authentication Frame,

Multi-band local,

Multi-band peer,

FILSWrappedData,

AssociationResponseTimeOutInfo

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssociationResponseTimeOutInfo | Association Timeout Info element | As defined in 8.4.2.X Association Timeout Info element | Minimum Association Response timeout that the non-AP STA to be set to dot11AssociationResponseTimeOut. |

Modify clause 6.3.5.5 as following:

## 6.3.5.5 MLME-AUTHENTICATE.response

MLME-AUTHENTICATE.confirm(

PeerSTAAddress,

AuthenticationType,

ResultCode,

Content of FT Authentication elements,

Content of SAE Authentication Frame,

Multi-band local,

Multi-band peer,

FILSWrappedData,

AssociationResponseTimeOutInfo

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssociationResponseTimeOutInfo | Association Timeout Info element | As defined in 8.4.2.X Association Timeout Info element | Minimum Association Response timeout that the non-AP STA to be set to dot11AssociationResponseTimeOut. |

# 8. Frame formats

## 8.4 Management and Extension(11ad) frame body components

## 8.4.2 Elements

Insert new clause 8.4.2.X:

### 8.4.2.X Association Timeout Info element

The Association Timeout Info element is used to inform the minimum Association Response timeout to the non-AP STA. The format of the Association Timeout Info element is shown in Figure 8-x (Association Timeout Info element format).

|  | Element ID | Length | Association Timeout Info |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 8-x – Association Timeout Info element format

The Element ID and Length fields are defined in 8.4.2.1 (General).

The value of the Association Timeout Info field is the minimum Association Response timeout in TU.

# Annex B

Add the following column at the end of the table in clause B.4.28:

## B.4.28 FILS features

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| FILS7 | Association Response Timeout Info | 8.4.2.X (Association Timeout Info element) | CF1 and CF31: O CF2.1 and CF31: M | Yes  No  N/A  |