IEEE P802.11
Wireless LANs

| dot11AssociationResponseTimeOut Negotiation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | SRC Soft | Fukuoka, JAPAN |  | hmorioka@src-soft.com |

Abstract

This text is a proposed resolution for CID4504.

# 6. Layer Management

## 6.3 MLME SAP interface

Modify clause 6.3.5.3 as following:

## 6.3.5.3 MLME-AUTHENTICATE.confirm

MLME-AUTHENTICATE.confirm(

PeerSTAAddress,

AuthenticationType,

ResultCode,

Content of FT Authentication elements,

Content of SAE Authentication Frame,

Multi-band local,

Multi-band peer,

FILSWrappedData,

AssociationResponseTimeOutInfo

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssociationResponseTimeOutInfo | Association Timeout Info element | As defined in 8.4.2.X Association Timeout Info element | Minimum Association Response timeout that the non-AP STA to be set to dot11AssociationResponseTimeOut. |

Modify clause 6.3.5.5 as following:

## 6.3.5.5 MLME-AUTHENTICATE.response

MLME-AUTHENTICATE.confirm(

PeerSTAAddress,

AuthenticationType,

ResultCode,

Content of FT Authentication elements,

Content of SAE Authentication Frame,

Multi-band local,

Multi-band peer,

FILSWrappedData,

AssociationResponseTimeOutInfo

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| AssociationResponseTimeOutInfo | Association Timeout Info element | As defined in 8.4.2.X Association Timeout Info element | Minimum Association Response timeout that the non-AP STA to be set to dot11AssociationResponseTimeOut. |

# 8. Frame formats

## 8.3 Format of individual frame types

## 8.3.3 Management frames

## 8.3.3.11 Authentication frame format

Add the following column at the end of Table 8-32:

Table 8-32—Authentication frame body

| Order | Information | Notes |
| --- | --- | --- |
| 23 | Association Timeout Info | The Association Timeout Info is present in FILS Authentication as defined in table 8-33 (Presence of fields and elements in Authentication frames) |

Modify Table 8-33 as following:

Table 8-33—Presence of fields and elements in Authentication frames

| Authentication algorithm | Authentication transaction sequence no. | Status code | Presence of fields 4-23 |
| --- | --- | --- | --- |
| FILS | 2 | Status | FILS Session element is present. RSNE is present. FILS Authentication Type is present if Status is 0. FILS Nonce is present if Status is 0. FILS Wrapped Data element is present if Status is 0 and FILS shared key authentication is used. Finite cyclic group is present if FILSAuthentication type field indicates PFS or if FILS public key authentication is used. Element is present if FILSAuthentication type field indicates PFS or if FILS public key authentication is used. PMKID list is present if AP agrees to perform PMK caching. Association Timeout Info element is present if AP expects time to transmit Association Response exceeds 1TU. |

## 8.4 Management and Extension(11ad) frame body components

## 8.4.2 Elements

Insert new clause 8.4.2.X:

### 8.4.2.X Association Timeout Info element

The Association Timeout Info element is used to inform the minimum Association Response timeout to the non-AP STA. The format of the Association Timeout Info element is shown in Figure 8-x (Association Timeout Info element format).

|  | Element ID | Length | Association Timeout Info |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 8-x – Association Timeout Info element format

The Element ID and Length fields are defined in 8.4.2.1 (General).

The value of the Association Timeout Info field is the minimum Association Response timeout in TU.

# Annex B

Add the following column at the end of the table in clause B.4.28:

## B.4.28 FILS features

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| FILS7 | Association Response Timeout Info | 8.4.2.X (Association Timeout Info element) | CF1 and CF31: O CF2.1 and CF31: M | Yes  No  N/A  |