IEEE P802.11
Wireless LANs

| Amendment Text for the [TBD CAG] Number Feature for TGaq |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| Zhigang Rong | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 |  | zhigang.rong@huawei.com |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document provides the draft amendment text for the [TBD CAG] Number feature for TGaq.

- Definitions, acronyms and abbreviations

  - Definitions specific to IEEE Std 802.11

Insert new definitions retaining alphabetic order as follows:

Partial Advertisement Protocol ID: A value corresponding to the five least significant bits (LSBs) of an Advertisement Protocol ID as defined in Table 8-220 in subclause 8.4.2.92 (Advertisement Protocol element).

  - Definitions Abbreviations and acronyms

Insert new abbreviations and acronyms retaining alphabetic order as follows:

CAG	Common Advertisement Group [13/1373r0 but shortened to be an abbreviation]

- Layer Management

- MLME SAP interface

- Scan

- MLME-SCAN.confirm

- Semantics of the service primitive

Insert the following rows at the end of the second table in this subclause:

[editor note: if TGai, TGaq, TGmc later decide to consolidate the CAG Number IE (in 11ai Draft) into  the [TBD CAG] Number IE in this document, replace “when dot11PADActivated is true” with “when dot11FILSActivated or dot11PADActivated is true” in the Description column for the [TBD CAG] Number row in the table below.]

| Name | Type | Valid Range | Description | IBSS Adoption |
| --- | --- | --- | --- | --- |
| [TBD CAG] Number | Octet string | As defined in 8.4.2.[aq1] ([TBD CAG] Number element) | The [TBD Common Advertisement Group] Number of the found BSS. This parameter is optionally present when dot11PADActivated is true.[CID 2813, 2640, 13/1041r1 13/1269r6] | Do not adopt. |

- Frame formats

- Format of individual frame types

- Management frames

- Beacon frame format

Insert new rows/elements before the last order in Table 8-35 as follows:

[editor note: if TGai, TGaq, TGmc later decide to consolidate the CAG Number IE (in 11ai Draft) into the [TBD CAG] Number IE in this document, replace “when dot11PADActivated is true” with “when dot11FILSActivated or dot11PADActivated is true” in the Notes column for the [TBD CAG] Number row in the table below.]

| - Table 8-35 – Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| [TBD] | [TBD CAG] Number[13/1373r0] [CID 2890] | [TBD CAG] Number element (see 8.4.2.[aq1] [TBD CAG] Number element ) is optionally present when dot11PADActivated is true. [13/1373r0] |

- Probe Response frame format

Insert new rows/elements before the last order in Table 8-42 as follows:

[editor note: if TGai, TGaq, TGmc later decide to consolidate the CAG Number IE (in 11ai Draft) into the [TBD CAG] Number IE in this document, replace “when dot11PADActivated is true” with “when dot11FILSActivated or dot11PADActivated is true” in the Notes column for the [TBD CAG] Number row in the table below.]

| - Table 8-42 – Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <TBD> | [TBD CAG] Number[13/1373r0] [CID 2891] | [TBD CAG] Number elements specified in 8.4.2.[aq1] [TBD CAG] Number element is optionally present when dot11PADActivated is true. [CID 2650] [13/1373r0] |

[CID 3363]

- Elements

- General

Insert the following new rows/Element IDs to Table 8-85:

| Element | Element ID | Extensible |
| --- | --- | --- |
| [TBD CAG] Number (8.4.2.[aq1] [TBD CAG] Number element ) [13/1373r0] | <ANA> |  |

Insert the new subclause as follows:

[editor note: in this TGaq document, this new subclause (8.4.2.[a1]) is different than 11ai’s subclause 8.4.2.173 (CAG Number elemnt).  Terminologies of [TBD CAG] and [TBD Scope] are used here to avoid potential confusions with 11ai’s  CAG and Scope, while showing the similarities between the 11ai CAG and 11aq CAG features .  The intent is to facilitate possible convergence between these two CAGs.]

[editor note: if TGai, TGaq, TGmc later decide to consolidate the CAG Number IE (in 11ai Draft) into the [TBD CAG] Number IE in this document, replace “dot11PADActivated is true” with “dot11FILSActivated or dot11PADActivated is true” in the last sentence of the first paragraph under subclause 8.4.2.[aq1].]

8.4.2.[aq1] [TBD CAG] Number element [13/1373r0]

The [TBD CAG] Number element provides the current version number(s) of the [TBD CAG(s)] of the associated advertisement protocol(s). The [TBD CAG] Number element is used by the STA to determine if a change in the [TBD CAG] of the associated advertisement protocol occurred from the last visit of the STA to a particular AP. The [TBD CAG] Number element is optionally present[CID 2009] in the Beacon or Probe Response frame to reduce GAS exchanges when dot11InterworkingServiceActivated is true and dot11PADActivated is true. [13/1373r0]

|  | Element ID | Length | [TBD CAG] Information #1 | [TBD CAG] Information #2 (optional) | … | [TBD CAG] Information #n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 0 or 2 |  | 0 or 2 |

Figure 8-[aq2] − [TBD CAG] Number element [CID 2656, 2893, 2443] [13/1373r0]

The Element ID and Length fields are defined in General [CID 2010]. The [TBD CAG] Number element includes one or more 2-octet [TBD CAG] Information fields. The format of a [TBD CAG] Information field is shown in Figure 8-[aq3].

|  | B0 B7 | B8 B10 | B11 B15 |
| --- | --- | --- | --- |
|  | [TBD CAG] Version | [TBD Scope] | Partial Advertisement Protocol ID |
| Bits: | 8 | 3 | 5 |

Figure 8-[aq3] − CAG Information field [CID 2656, 2893, 2443] [13/1373r0]

The [TBD CAG] Version is a 1-octet subfield whose value is an unsigned positive integer that indicates the current version of the [TBD CAG] of the associated advertisement protocol. A [TBD CAG] Version is always positive, therefor a value of zero in this subfield will be neglected by the receiving STA.[CIDs 2658, 2298][13/1373r0, CIDs 2931, 3109, 3111, 3112, 3288, 3292 - Accepted resolution to CID 2818 contradicts the submittal approved in the same motion, submittal version was used.]

The [TBD Scope] is a 3-bit subfield that indicates the valid scope of the information represented by the [TBD CAG] Version in the same [TBD CAG] Information field. A subfield value of 0 indicates that the [TBD CAG] is only valid in the current BSS (identified by the BSSID value), a subfield value of 1 indicates that the [TBD CAG] is only valid in the Homogeneous ESS (identified by the HSSID value), and a subfield value of 2 indicates that the [TBD CAG] is valid for the ESS (union of BSSs with the same SSID). Subfield values of 3-7 are reserved.

The Partial Advertisement Protocol ID is a 5-bit subfield carrying a value equal to the five least significant bits (LSBs) of the Advertisement Protocol ID, as defined in Table 8-220 in subclause 8.4.2.92 (Advertisement Protocol element), of the advertisement protocol associated with the [TBD CAG] Version in the same [TBD CAG] Informaiton field.  [CID 2894] [13/1373r0, CIDs 2300, 2999, 3290]

8.4.6 Pre-association Discovery Protocol elements

8.4.6.1 General

Insert a new table at the end of this subclause as follows:

Table 8-[aq4] − PADP-element definitions [CID 2656, 2893, 2443] [13/1373r0]

| PADP-element name | Info ID | PADP-element subclause |
| --- | --- | --- |
| Reserved | 0-[aq5] | N/A |
| [TBD Common Advertisement Group] | [aq6] | 8.4.6.[aq8] ([TBD Common Advertisement Group] PADP-element) |
| Reserved | [aq7]-56796 | N/A |
| Vendor Specific | 56797 | 8.4.2.25 (Vendor Specific elemnt) |
| Reserved | 56798-65535 | N/A |

Insert a new subclause as follows:

8.4.6.[aq8] [TBD CAG] PADP-element

The [TBD CAG] PADP-element provides the current version of the [TBD CAG] associated with the PADP advertisement procotol.

|  | Info ID | Length | PADP CAG Version |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 |

Figure 8-[aq9] − CAG PADP-element [CID 2656, 2893, 2443] [13/1373r0]

[13/1373r0 CIDs 2905, 2334, 3288, 3268 - CIDs 2904 & 2905 were accepted in same motion as 13/1373r0 but offers conflicting resolution to this paragraph, implementing the submission version]The Info ID is a 2-octet field whose value is equal to the value in Error: Reference source not found. [13/1373r0]

The Length field is a 2-octet unsigned positive number whose value is set to the length in octets of remaining fields. [13/1373r0 CID 2682]

The PADP CAG Version field is a 1-octet field and it indicates the current version of the [TBD CAG] associated with the PADP advertisement procotol. The value of PADP CAG Version is determined by the proxy entity as in Annex AQ2.

10. MLME

10.25.3.4 PADP procedures

Insert a new subclause as follows:

10.25.3.4.[aq10] [TBD CAG] procedure

A PADP-capable STA should store, for later use, the PADP CAG Version value received, via an AP, from a proxy entity, the BSSID of the AP, and the service information corresponding to the PADP CAG Version. A PADP-capable STA uses the PADP CAG Version to determine if the [TBD CAG] associated with the PADP at a particular AP has changed from the last STA's visit to this AP. In order to reduce the service discovery delay, if a valid [TBD CAG] Version value associated with the PADP is received from this AP in the [TBD CAG] Number IE, a PADP-capable STA should compare its stored PADP CAG Version for this AP with this newly received [TBD CAG] Version, to decide whether it should initiate a GAS query for new service information. If the newly received [TBD CAG] Version associated with the PADP equals to the stored value of the PADP CAG Version for this AP, the STA shall not initiate a GAS query request to this AP for service information. The [TBD CAG] Version value associated with the PADP is determined by the proxy entity as in Annex AQ2. How an AP obtains the current [TBD CAG] Version value associated with the PADP from the proxy entity is out of the scope of this document. [13/1374r0, CIDs 3305, 3269, 3231, 2084, 3075, 20

The [TBD CAG] Version associated with the PADP is a positive number. If a STA receives a value of zero for the [TBD CAG] Version associated with the PADP, the value will be discarded and no action should be taken.

Annex AQ2

(normative)

AQ2.1 Proxy Entity

Insert the new paragraph as follows:

The [TBD CAG] Version value associated with the PADP is determined by the proxy entity and shall be a positive number that increases by 1 when there is any change in the service inormation available at the proxy entity. How the proxy entity determines that a change in the service information has occurred is out of the scope of this document.  [13/1374r0, CIDs 3305, 3269, 3231, 2084, 3075, 2084]

[13/1374r0]