IEEE P802.11
Wireless LANs

| Annex B proposed technical changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | SRC Software | Fukuoka JAPAN |  | hmorioka@src-soft .com |

Abstract

Proposed technical changes to Annex B based on 11-14/944r1

# Annex B

## B.4.28 FILS features

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| FILS1 | FILS Discovery Frame | 10.44.2 (FILS Discovery Frame generation and usage) | (CF1 or CF2.1) and CF31: M | Yes  No  N/A  |
|  |  | 8.6.8.34 (FILS Discovery frame format) |  |  |
| FILS2 | Differentiated Initial Link Setup | 10.44.5 (Differentiated initial link setup) 8.4.2.183 (Differentiated Initial Link Setup element) | CF2.1 and CF31: M CF1 and CF31: O | Yes  No  N/A  |
| FILS3 | ANQP Procedure for FILS | 8.4.4.20 (Query AP List ANQP-element) | (CF1 or CF2.1) and CF20 and CF31: M | Yes  No  N/A  |
| FILS3.1 | Query AP List | 8.4.4.21 (AP List Response ANQP-element) | FILS3: M | Yes  No  N/A  |
| FILS3.2 | FILS Domain Information | 8.4.4.22 (FILS Domain Information ANQP-element) | FILS3: M | Yes  No  N/A  |
| FILS3.3 | Common ANQP Group | 8.4.4.23 (CAG ANQP-element) | FILS3: M | Yes  No  N/A  |
|  |  | 8.4.2.173 (CAG Number element) |  |  |
| FILS4 | FILS Authentication | 11.11 (Authentication for Fast Initial Link Setup) 8.4.1.56 (FILS Authentication Type field) 8.4.1.57 (FILS Nonce field) 8.4.2.175 (FILS Key Confirmation element) 8.4.2.176 (FILS Session element) 8.4.2.179 (FILS Indication element) | (CF1 or CF2.1) and CF31: M | Yes  No  N/A  |
| FILS4.1 | FILS shared key authentication without PFS | 8.4.2.184 (FILS Wrapped Data element) 11.11.2.2.1 (Key establishment with FILS shared key authentication) | (FILS4 and not FILS4.3): M (FIILS4 and FILS4.3): O | Yes  No  N/A  |
| FILS4.2 | FILS shared key authentication with PFS | 8.4.2.184 (FILS Wrapped Data element) 11.11.2.2.1 (Key establishment with FILS shared key authentication) | FILS4: O | Yes  No  N/A  |
| FILS4.3 | FILS public key authentication | 8.4.2.177 (FILS Public Key element) 8.4.2.185 (Fragment element) 11.11.2.2.2 (Key establishment with FILS public key authentication) | (FILS4 and not FILS4.1): M (FIILS4 and FILS4.1): O | Yes  No  N/A  |
| FILS4.1 | FILS Authentication Type | 8.4.1.56 (FILS Authentication Type field) | FILS4: M | Yes  No  N/A  |
| FILS4.2 | FILS Nonce | 8.4.1.57 (FILS Nonce field) | FILS4: M | Yes  No  N/A  |
| FILS4.3 | FILS Key Confirmation | 8.4.2.175 (FILS Key Confirmation element) | FILS4: M | Yes  No  N/A  |
| FILS4.4 | FILS Session | 8.4.2.176 (FILS Session element) | FILS4: M | Yes  No  N/A  |
| FILS4.5 | FILS public key authentication | 8.4.2.177 (FILS Public Key element) | FILS4: M | Yes  No  N/A  |
| FILS4.6 | FILS Indication | 8.4.2.179 (FILS Indication element) | FILS4: M | Yes  No  N/A  |
| FILS4.7 | FILS Wrapped Data | 8.4.2.184 (FILS Wrapped Data element) | FILS4: M | Yes  No  N/A  |
| FILS4.8 | Element Fragmentation | 8.4.2.185 (Fragment element) | (FILS4 or FILS5): M | Yes  No  N/A  |
| FILS5 | Higher Layer Setup During Association/Reassociation Procedure | 10.44.3 (Higher layer setup during association/reassociation procedure) | (CF1 or CF2.1) and CF31: M | Yes  No  N/A  |
| FILS5.1 | HLP Packet Encapsulation | 10.44.3.1 (Higher Layer Protocol Packet Encapsulation) 8.4.2.180 (FILS HLP Container element) | FILS5: M | Yes  No  N/A  |
| FILS5.2 | FILS IP Address Configuration | 10.44.3.2 (FILS IP Address Configuration) 8.4.2.181 (FILS IP Address Assignment element) | FILS5: O | Yes  No  N/A  |
| FILS6 | Scanning Enhancement | 10.1.4.(Acquiring synchronization, scanning) | (CF1 or CF2.1) and CF31: M | Yes  No  N/A  |
| FILS6.1 | Probe Request Reduction | 10.1.4.3.2(Active scanning procedure) | FILS6: M | Yes  No  N/A  |
| FILS6.2 | Probe Response Reduction | 10.1.4.3.3(Probe response criteria) 8.4.2.174 (FILS Request Parameter element) | FILS6: M | Yes  No  N/A  |
| FILS6.3 | AP Configuration Sequence Number | 8.4.2.178 (AP Configuration Sequence Number element) | FILS6: M | Yes  No  N/A  |
| FILS6.4 | Reduced Neighbor Report | 10.43.8 (Reduced Neighbor Report) 8.4.2.171 (Reduced Neighbor Report element) | FILS6: M | Yes  No  N/A  |