IEEE P802.11
Wireless LANs

| Proposed Resolution to CID 6,7 on TGaj D0.2 in CC19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jiamin Chen | Huawei/HiSilicon |  |  | Jiamin.chen@mail01.huawei.com |

Abstract

This document proposes resolutions to TGaj D0.2 CIDs: 6 and 7.

Technical Comments

| 6 | 8.4.2.160.1 | 19 | 50 | TR | Why is the "STA Address" field in the Capabilities element format? This element will be transmitted in frames that already have addressing. | Remove "STA Address" field. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Reject

Referring to the structure of the DMG Capabilities element (defined in 802.11ad, see 8.4.2.127 in 802.11 REVmc), a CDMG PCP or AP can respond an Information Response frame containing the CDMG capabilities of other CDMG STAs in the same BSS after receiving an Information request frame from an associated CDMG STA. Therefore, the STAs can be identified by receiving MAC address with corresponding capabilities information. So the MAC address field needs to be included in the CDMG Capabilities element.

| 7 | 8.4.2.160.1 | 19 | 50 | TR | Capabilities are static attributes of a device. The AID is a dynamic attribute assigned by the AP. It should not be in the capabilities structure. | Remove "AID" field. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Revised

The CDMG Capabilities element includes the new capabilities defined in 802.11aj and does not include the capabilities defined in the DMG Capabilities element. So, the capabilities of a CDMG STA shall include the capabilities indicated by both the DMG Capabilities element and the CDMG Capabilities element. In other words, a CDMG STA shall transmit both DMG Capabilities element and CDMG Capabilities element to indicate its capabilities. Thus, the AID field does not need to be included in the CDMG Capabilities element. So, Change the definition of the CDMG and remove “AID” field from CDMG Capabilities element as follows:

“The capabilities of a CDMG STA shall include the capabilities indicated by the DMG Capabilities element and the CDMG Capabilities element. The CDMG Capabilities element contains a STA identifier and several fields to indicate the capabilities that are applicable for a CDMG STA; they are not applicable for a DMG STA. A CDMG STA shall use both the DMG Capabilities element and the CDMG Capabilities to advertise the support of optional CDMG capabilities. The CDMG Capabilities element contains a STA identifier and several fields that are used to advertise the support of optional CDMG capabilities of a CDMG STA. The element shall be is present in Association Request, Association Response, Reassociation Request, Reassociation Response, Probe Request and Probe Response frames and can be present in CDMG Beacon and Information request and response frames. The CDMG Capabilities element is formatted as illustrated in Figure 8-401bt.

|  | Element ID | Length | STA Address | AID | CDMG STA Capability Information | CDMG AP or PCP Capability Information |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 | 10 | 3 |

Figure 8–401bt CDMG Capabilities element format

The Element ID field is equal to the value for the CDMG Capabilities, specified in Table 8-54 (Element IDs).

The Length field is set to 20.

The STA Address field contains the MAC address of the STA.

The AID field contains the AID assigned to the STA by the AP or PCP. The value of the AID field is ignored in Association Request, Reassociation Request and Probe Request frames and when used in an IBSS. ”

“