IEEE P802.11
Wireless LANs

| 802.11 CID 3060 (relative to Draft 3.0) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-08-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission resolves to CID 3060.

- Cl. 6.3.58.3 describes MLME-FINETIMINGMSMT.confirm that includes parameters that do not belong in the confirm (CID #3060) – these parameters are moved to the indication, below.

- Cl. 6.3.58.4 describes MLME-FINETIMINGMSMT.indication that does not include all the required parameters.

- Cl. 6.3.70 addresses primitives related to Fine Timing Measurement

- Cl. 6.3.69 addresses primitives related to Timing Measurement Request

| Comment | Suggested Remedy | Resolution |
| --- | --- | --- |
| The (FINE)TIMINGMSMT primitives do not follow the usual 2 or 4 primitive model. There is no ".response" primitive. This brings into question where the values returned in the confirm come from. As shown in Figure 6-17, the .confirm is issues on receipt of an Ack. But where do the LCI Report, Location Civic Report, Fine Timing Measurement Parameter and Vendor Specific parameters come from? They are certainly not in the ACK. | Either: 1. Add a .response primitive, modify Figure 6-17 to show response primitive generating a management frame containing these parameters, or 2. Delete cited parameters from this primitive. | Revise |

# Editor: P386L6 Replace “6.3.69.3.4 1Effect of receipt” with “6.3.69.3.4 Effect of receipt”

# Editor: Move Table in P388L1-30 to P386L40

# Editor: Move Table in P387L1-30 to P387L40

# Editor: Move LCI Report, Location Civic Report and Fine Timing Measurement Parameter from MLME-FINETIMINGMSMT.confirm primitive (Cl. 6.3.58.3.2 P338) to MLME-FINETIMINGMSMT.indication primitive (Cl. 6.3.58.4.2 P339).

# Editor: Move the rows in the table in Cl. 6.3.58.3.2 (P339 L8-30) LCI Report, Location Civic Report and Fine Timing Measurement Parameter from MLME-FINETIMINGMSMT.confirm primitive (Cl. 6.3.58.3.2 P338) to the bottom of the table (P340 L57)  in MLME-FINETIMINGMSMT.indication primitive (Cl. 6.3.58.4.2 P339).