IEEE P802.11
Wireless LANs

| REVmc D3.0 Comment Resolutions for Clauses 22.3.3, 8, 10, 11, 19, 21, & 9.32.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-08-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Mingguang Xu | Marvell | 5488 Marvell Ln, Santa Clara, CA 95054 | 408-222-7766 | mxu@marvell.com |
| Hongyuan Zhang | Marvell |  |  | hongyuan@marvell.com |
| Edward Au | Marvell |  |  | edwardau@marvell.com |
| Liwen Chu | Marvell |  |  | liwenchu@marvell.com |

Abstract: This document contains proposed resolutions for comments in Clauses 22.3 & 9.32.2.1 from REVmc D3.0 with the CIDs below.

| Clauses 22.3 & 9.32.2.1 |  |  |
| --- | --- | --- |
| - 3166 - 3176 |  |  |
| - 3182 |  |  |
| - 3183 |  |  |
| - 3184 |  |  |
| - 3185 - 3186 - 3187 - 3188 - 3189 - 3190 - 3192 - 3194 - 3196 - 3197 - 3199 - 3200 - 3476 - 3667 (Clause 9.32.2.1) |  |  |

CIDs for Clause 22.3

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3166 | Sigurd Schelstraete | 22 | 2453.27 | Illustration of the transmitter block diagram for 80+80 VHT-SIG-B may not be correct. The same comment applies in a number of places where segment parsing is mentioned for 80+80 VHT-SIG-B. There are a number of contradictions in the text between the way 80+80 VHT-SIG-B is described in 22.3.8.3.6 and the way it is depicted here. | The main question is whether 80+80 VHT-SIG-B really uses segment parsing. This comment needs to be resolved together with other related comments. | Revised. Refer to the resolution of CID 3189. |
| 3176 | Sigurd Schelstraete | 22 | 2466.20 | N_CBPSS for 80+80 VHT-SIG-B | Section 22.3.8.3.6 states that the 80 MHz format is used in each of the frequency segments of 80+80 (see p2493, L34). As such, the number of coded bits is ambiguous. | Revised. Refer to the resolution of CID 3189. |
| 3189 | Sigurd Schelstraete | 22 | 2490.42 | There are inconsistencies in the description of VHT-SIG-B for 80+80. Formulas (22-47), (22-52) and the statement on page 2493, line 34 clearly show that the signal at the input of the spatial mapper is the same on both 80 MHz segments. Yet, page 2490, line 42 states that the input bits to VHT-SIG-B are segment parsed. This is also shown in Figure 22-9. However, Segment parsing as described in 22.3.10.7 would result in the even bits on one segment and the odd bits on the other, contradicting the requirements that the frequency signal is the same in both 80 MHz segments. | Make description of 80+80 VHT-SIG-B consistent. This will require changes in a number of places: 1. page 2490, line 42: no segment parsing for 80+80. Instead perform 80 MHz processing and duplicate. 2. There is no need to generate 468 bits for 80+80 VHT-SIG-B. Hence Figure 22-22 and accompanying text need to be corrected. 3. Correct Figure 22-9. 4. Possible other places (will do detailed scrub after comment is resolved in principle). | Revised. For VHT-SIG-B in the case of 80+80 MHz, the input bits should be segment parsed based on previous concensus in the 11ac document 11-13-0983r1, and as reflected in the text in page 2490 line 42. Propose to add dependence of frequency segment iSeg onto complex values D and d. Then there will be no ambiguity about segment parsed bits in each 80MHz segment. Refer to the text revisions as shown below. |

REVmc editor: please make the following changes in Clause 22.3:

- On Page 2490.62 (CIDs #3189): Replace “” in Equation (22-47) with “”.

- On Page 2491.21 (CIDs #3189): In Equation (22-48), replace “” with “”, and “” with “”.

- On Page 2491.42 (CIDs #3189): In Equation (22-50), replace “” with “”, and “” with “”.

- On Page 2492.4 (CIDs #3189): In Equation (22-52), replace “” with “”, and “” with “”.

- On Page 2492.30 (CIDs #3189): In Equation (22-54), replace “” with “”, and “” with “”.

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3182 | Sigurd Schelstraete | 22 | 2476.35 | Clarify relationship between transmit chains and frequency segments | Replace "signal on frequency segment i_Seg in transmit chain i_TX" with "signal on transmit chain i_TX of frequency segment i_Seg". This is consistent with the interpretation of N_TX proposed in previous comment. | Revised. There are multiple ways of implementing 80+80 with multiple Tx antennas. For convenience, we may make it consistent with single segment cases, and with the statement in page 2472 line1 “on frequenct segment i_seg of transmit chain i_tx”. |
| 3183 | Sigurd Schelstraete | 22 | 2477.20 | Add segment dependency | Replace "signal on transmit chain i_TX" with "signal on transmit chain i_TX of frequency segment i_Seg" | Revised. Change to as in the resolution of CID3182. |
| 3184 | Sigurd Schelstraete | 22 | 2478.12 | Add segment and transmit chain dependency | Replace "The time domain waveform of the L-SIG field" with "The time domain waveform of the L-SIG field on transmit chain i_TX of frequency segment i_Seg" | Revised. Change to as in the resolution of CID3182. |
| 3185 | Sigurd Schelstraete | 22 | 2480.51 | Add segment and transmit chain dependency | Replace "The time domain waveform for the VHT-SIG-A field in a VHT PPDU" with "The time domain waveform of the VHT-SIG-A field in a VHT PPDU on transmit chain i_TX of frequency segment i_Seg". [Editorial]: we propose to start this sentence on a new line, separate form the paragraph it currenlty belongs to. | Revised. Change to as in the resolution of CID3182. |
| 3186 | Sigurd Schelstraete | 22 | 2484.57 | Clarify relationship between transmit chains and frequency segments | Replace "signal on frequency segment i_Seg in transmit chain i_TX" with "signal on transmit chain i_TX of frequency segment i_Seg". | Revised. Change to as in the resolution of CID3182. |
| 3188 | Sigurd Schelstraete | 22 | 2487.52 | Clarify relationship between transmit chains and frequency segments | Replace "on frequency segment i_Seg in transmit chain i_TX" with "on transmit chain i_TX of frequency segment i_Seg". | Revised. Change to as in the resolution of CID3182. |
| 3197 | Sigurd Schelstraete | 22 | 2513.06 | Add segment dependency | Replace "transmit chain i_TX" with "transmit chain i_TX of frequency segement i_Seg" | Revised. Change to as in the resolution of CID3182. |

REVmc editor: please make the following changes in Clause 22.3:

- On Page 2470.38 and 2476.35 (CID #3182): Replace “in transmit chain iTx” with “of transmit chain iTx”.

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3187 | Sigurd Schelstraete | 22 | 2487.29 | P_VHTLTF not defined in (22-40) | Add "P_VHTLTF is defined in (22-43)" | Accept. |
| 3190 | Sigurd Schelstraete | 22 | 2491.25 | Notation d^(u) isn't explained until much later (page 2493, line 37). | move sentence from page 2493, line 37 to after equation (22-48). Also clarify "constellation point of VHT-SIG-B for user u". | Revised. The description for notation d has been changed, see text revisions below. |
| 3192 | Sigurd Schelstraete | 22 | 2496.45 | N_ES is user dependent | Replace N_ES with N_ES,u | Accept. |
| 3194 | Sigurd Schelstraete | 22 | 2496.58 | Use correct terminology | "FEC input sequence" is not defined. Replace "FEC input sequence" with "BCC encoder parser output sequence" | Accept. |
| 3196 | Sigurd Schelstraete | 22 | 2504.48 | The range of k in (22-84) is not correct for 160 MHz. (22-84) says that k =0, ..., N_SD-1 and l=0,1 for 160 MHz. This means there are 2 N_SD = 2x468 complex values per spatial stream and per symbol. | For 160 MHz, the range of k before segment deparsing is k=0, ..., N_SD/2 -1. Same comment on page 2509, lines 6 and 44 | Revised. N_SD is changed to be N_SD/2 for 160 MHz, and see text revisions below. |
| 3199 | Sigurd Schelstraete | 22 | 2517.05 | User correct terminology: replace NUM_USERS with N_user | As in comment | Accept. |
| 3200 | Sigurd Schelstraete | 22 | 2538.43 | This statement should not be a note | Remove "NOTE -" | Revised. See text revisions below. |
| 3476 | Mark RISON | 22 | 2531.41 | This subclause starts "For the operating classes requiring CCA-Energy Detect (CCA-ED)" but examination of Annex E shows that the only operating classes requiring CCA-ED are in the 3G band and the maximum channel width is 20 MHz, so this subclause seems otiose at best and misleading at worst | Delete this subclause (also 23.3.19.6.2) | Reject. It doesnot hurt to keep the subclauses of CCA-ED in HT, VHT, and TVHT PHYs. |
| 3667 | David Hunter | 9 | 1399.63 | "use only HT and non-HT PPDUs": uhh, are there any other kinds of PPDUs? Since the sentence is about HT procedures, should "and non-HT" be deleted? | Delete "and non-HT". | Reject. The implicit beamforming procedure may include frames that are carried by non-HT modulated packets. |

REVmc editor: please make the following changes in Clause 22.3:

- On Page 2487.51 (CID #3187):  Insert a line:  “P_VHTLTF is defined in (22-43)".

- On Page 2493.36 (CID #3190):

……

In Equation (22-48), Equation (22-50), Equation (22-52), and Equation (22-54), is the constellation point of VHT-SIG-B for user u (starting with 0) at subcarrier k  is the stream of complex numbers generated for VHT-SIG-B for user u at subcarrier k (logical index, starting with 0) in frequency segment iSeg (prior to multiplication by PVHTLTF).

… …

- On Page 2496.45 (CID #3192): Replace "NES" with "NES,u".

- On Page 2496.58 (CID #3194): Replace "FEC input sequence" with "BCC encoder parser output sequence".

- On Page 2504.48 (CID #3196): Change the first line of Equation (22-84) into:

for 20 MHz, 40 MHz, 80 MHz, and 80+80 MHz;

for 160 MHz;

- On Page 2509.06 (CID #3196): Change the first line of Equation (22-85) into:

for 20 MHz, 40 MHz, 80 MHz, and 80+80 MHz;

for 160 MHz;

- On Page 2509.43 (CID #3196): Change the first line of Equation (22-87) into:

for 20 MHz, 40 MHz, 80 MHz, and 80+80 MHz;

for 160 MHz;

- On Page 2509.01 (CID #3196): Remove the bandwidth attributive in the first sentence:

… …

For a 20 MHz, 40 MHz, and 80 MHz VHT PPDU transmission, the LDPC tone mapping for LDPC-coded streams corresponding to user u is done by permuting the stream of complex numbers generated by the constellation mappers (see Equation (22-84)) to obtain

… …

- On Page 2157.03 (CID #3199): Replace “NUM_USERS” with “Nuser”.

- On Page 2538.44 (CID #3200):

……

NOTE—where LENGTH in Equation (22-105) is the LENGTH field in L-SIG.

……