IEEE P802.11
Wireless LANs

| MDR Action Item 1 – Proposed Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document contains a proposed resolution to the MDR Action Item 1

Following the MDR (Mandatory Draft Review), the actions pending include:

- Adrian to solicit Stephen McCann’s help to disambiguate:

“When an AP is located in a regulatory domain that requires location capabilities, the ESR field shall onlymay be set to 1 and the Network Type shall onlymay be set to “Emergency services only network” (see Table 8-219 (Access network type)), if location capability is enabled on the AP; the ESR field shall not be set to 1 and the Network Type shall not be set to “Emergency services only network” otherwise. ”

Proposed Resolution:

(IEEE P802.11-REVmc/D3.0, June 2014, Page 1766, Line 15)

10.25.6 Interworking procedures: emergency services support

Change this text to read:

“When an AP is located in a regulatory domain that requires emergency location capabilities, the ESR field shall be set to 1; otherwise the ESR field shall be set to 0.  When an AP is located in a regulatory domain that requires emergency location capabilities and location capability is enabled on the AP, the Network Type shall be set to “Emergency services only network” (see Table 8-219 (Access network type)), if location capability is enabled on the AP; otherwise the ESR field shall not be set to 1 and the Network Type shall not be set to “Emergency services only network”.”