IEEE P802.11
Wireless LANs

| LB 203 MAC Comment Resolution for Clauses 10.1.2 and 10.1.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Jae Seung Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1326 | jasonlee@etri.re.kr |
| Jaewoo Park | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5635 | parkjw@etri.re.kr |
| Yu Ro Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5693 | yurolee@etri.re.kr |
| Jeeyon Choi | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5247 | jychoi@etri.re.kr |
| Il Gyu Kim | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5490 | igkim@etri.re.kr |

Abstract

This submission proposes comment resolutions on Clauses 10.1.2 and 10.1.3.

- CIDs: 3164, 3046, 3166, 3049, 3050, 3167, 3051, 3630, and 3052  (9 CIDs)

Changes in the text refer to: Draft P802.11ah/D2.1 and P802.11-REVmc/D3.1

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGah Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGah Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGah Editor: Editing instructions preceded by “TGah Editor” are instructions to the TGah editor to modify existing material in the TGah draft.  As a result of adopting the changes, the TGah editor will execute the instructions rather than copy them to the TGah Draft.

CID 3164

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3164 | 10.1.2 | 315 | 20 | If the S1G Beacon frame is the only beacon frame generated in an S1G BSS then there is nowhere else in an S1G BSS that can be stated otherwise. Remove the "unless explicitly stated otherwise" in P315L20 and replace "refers to that" with and all references to it, refer to that". Also, the correct terminology is "TSF timer" rather than "TSF counter". For consistency replace "TSF counter" with "TSF timer" in P315L34. | As in comment. | Accepted |

CIDs 3046, 3166, 3049, and 3050

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3046 | 101.3.10.2 | 317 | 3 | "S1G Beacon frames shall be transmitted in an S1G BSS and S1G IBSS." This statement is either unnecessary or incomplete. It is incomplete because it does not state who transmits the frames. | Either reword to informative, delete, or add a subject. | Revised - Agree with the commenter in principle. Changed the text accordingly. TGah Editor to make changes shown in 14/1156 |
| 3166 | 10.1.3.10.2 | 317 | 7 | The presence of the Next TBTT field in the S1G Beacon needs to be better specified for both cases, with and without TSBTT existence. Also there is already an acronym for "TBTT or TSBTT" i.e., T(S)BTT. | Replace "If n is greater than 1, the Next TBTT Present field shall be set to 1 and the Next TBTT field shall be present in S1G Beacon frames." with " If n is greater than 1, the Next TBTT field shall be present in S1G Beacon frames; otherwise it shall not be present in S1G Beacon frames." Replace "TBTT or TSBTT" with T(S)BTT" throughout this paragraph. Also it seems we dont need the Next TBTT all the time in TSBTTs. Consider relaxing the requirement. | Revised - Agree with the commenter in principle. Relaxed the requirement on Next TBTT field since it is not necessary at TBTT. Changed the text accordingly. TGah Editor to make changes shown in 14/1156 |
| 3049 | 10.1.3.10.2 | 317 | 11 | "At each TBTT or TSBTT the AP should suspend the decrementing of the backoff timer for any pending nonbeacon transmission and transmit the S1G Beacon frame according to the medium access rules specified in Clause 9 (MAC sublayer functional description)." There is no justificaiton for this being a "should". Failure to suspend gives an AP an unfair advantage. | Change "should" to "shall" | Rejected - The sentence is based on the baseline spec. In clause 10.1.3.2 Beacon generation in non-DMG infrastructure networks in REVmc D3.1, it is specified “At each TBTT the AP should suspend the decrementing of the backoff timer ~ “ . There is no reason to change the same behavior specified in the baseline. |
| 3050 | 10.1.3.10.2 | 317 | 11 | "At each TBTT or TSBTT the AP should suspend the decrementing of the backoff timer for any pending nonbeacon transmission and transmit the S1G Beacon frame according to the medium access rules specified in Clause 9 (MAC sublayer functional description)." This rule should generally be followed with a rule that says when decrementing the backoff timer is un-suspended. I don't see it. | Add a matching statement indicating when the backoff timer is restarted. | Rejected - The sentence is based on the baseline spec. In clause 10.1.3.2 Beacon generation in non-DMG infrastructure networks in REVmc D3.1. In REVmc, there is no matching statement that says when decreamenting the backoff timer is unsuspended. To add this statement to 11ah spec, REVmc should be updated first and then 11ah should adopt the same statement. |

Proposed Remedy (CID 3046 and 3166):

Instructions to TGah Editor: Change the subclause 10.1.3.10.2 as follows:

- Generation of S1G Beacon frames

S1G Beacon frames shall be transmitted in an S1G BSS and S1G IBSS. In an S1G infrastructure BSS, S1G Beacon frames shall be transmitted by an AP. Each STA in an S1G IBSS shall transmit S1G Beacon frames. An AP may further define the timing for the BSS by sending S1G Beacon frames according to the dot11ShortBeaconPeriod. The value for the dot11ShortBeaconPeriod shall be such that dot11BeaconPeriod = ndot11ShortBeaconPeriod, where n is a positive integer. This defines a series of TSBTTs exactly dot11ShortBeaconPeriod TUs apart. If n is greater than 1, the Next TBTT Present field shall be set to 1 and the Next TBTT field shall be present in S1G Beacon frames transmitted at TSBTT that is not a TBTT; otherwise it shall not be present in S1G Beacon frames. Time 0 is defined to be a T(S)BTT or TSBTT with the S1G Beacon frame being a DTIM. At each T(S)BTT or TSBTT, the AP shall schedule an S1G Beacon frame as the next frame for transmission. At each T(S)BTT or TSBTT the AP should suspend the decrementing of the backoff timer for any pending non-beacon transmission and transmit the S1G Beacon frame according to the medium access rules specified in Clause 9 (MAC sublayer functional description). The beacon period is included in S1G Beacon and (Short) Probe Response frames, and a STA shall adopt that beacon period when joining the S1G BSS and S1G IBSS, i.e., the STA shall set its dot11BeaconPeriod variable to that beacon period. If dot11ShortBeaconInterval is equal to true, the Short Beacon Interval element is included in an S1G Beacon and (Short) Probe Response frames, and a STA shall adopt that short beacon period when joining the S1G BSS or S1G IBSS, i.e., the STA shall set its dot11ShortBeaconPeriod variable to that short beacon period.

CID 3167

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3167 | 10.1.3.10.3 | 317 | 55 | list explicitly which value shall be increased by one unit, i.e., "the value obtained from the TSF Completion field" | As in comment. | Accepted |

CID 3051

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3051 | 10.1.3.10.4 | 317 | 62 | "If the ScanType parameter indicates a passive scan, the S1G STA shall listen to each channel scanned for no longer than a maximum duration defined by the MaxChannelTime parameter" Why is this under "Passive scanning with S1G Beacon", as it appears not to have any dependency on it? Why is this any different to 10.1.4.2.1? | Remove subclause. | Rejected – During LB200 comment resolution, the same CID (CID 2790) has been discussed and rejected. Although 10.1.4.2.1 and 10.1.3.10.4 both contain identical content, 10.1.3.10.4 implies that S1G Beacon frames are used instead of Beacon frames for passive scanning in 11ah. So it is better to insert the clause 10.1.3.10.4. |

CIDs 3630 and 3052

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 3630 | 10.1.3.10.5 | 318 | 7 | Does this refer to IBSS or Infrastructure BSS | If IBSS,specify IBSS. If infrastructure,specify if the transmitting STA is an AP or a non-AP STA. | Revised - Agree with the commenter in principle. Since 11ah does not exclude IBSS, both IBSS and infrastructure BSS should be included in the description. Changed the text accordingly. TGah Editor to make changes shown in 14/0xxx |
| 3052 | 10.1.3.10.5 | 318 | 6 | There is no need for a separate subclause, and it's in the wrong place anyway. | Move "an S1G STA shall stop transmitting S1G Beacon frames." to 10.1.6 and delete this subclause. | Revised - Agree with the commenter in principle. Changed the text accordingly. TGah Editor to make changes shown in 14/0xxx |

Proposed Remedy (CID 3630 and 3052):

Instructions to TGah Editor: Delete Subcluase 101.3.10.5 Terminating a BSS from 11ah D2.1

10.1.3.10.5 Terminating a BSS

An infrastructure BSS may be terminated at any time. In addition to procedures described in 10.1.6 (Terminating a BSS)(#3287), upon receipt of an MLME-STOP.request primitive, an S1G STA shall stop transmitting S1G Beacon frames.

Instructions to TGah Editor: Modify the subclause 10.1.6 in REVmc D3.1 as follows:

10.1.6 Terminating a BSS

An infrastructure BSS or PBSS(11ad) may be terminated at any time. A STA may cease support for an IBSS that it formed at any time. Upon receipt of an MLME-STOP.request primitive, a non-S1G STA shall stop transmitting Beacon, Probe Response, DMG Beacon, and Announce frame(11ad)(Ed) and deauthenticate all associated STAs. Upon receipt of an MLME-STOP.request primitive, an S1G STA shall stop transmitting S1G Beacon frames and (Short) Probe Response frames and deauthenticate all associated STAs.