IEEE P802.11
Wireless LANs

| Proposed 11ai Amendment Text for Extending CAG Number – Part 1 (common part for both ANQP and PADP) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| Zhigang Rong | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 |  | zhigang.rong@huawei.com |

Abstract

This document provides the 11ai Amendment Draft text for extending the CAG Number as proposed in document 11-14-1253-01-00aq.  The text included in this document is common for both ANQP and PADP.

[The first change against 802.11ai D2.1 text]

  - Definitions specific to IEEE Std 802.11

Insert new definitions retaining alphabetic order as follows:

Partial Advertisement Protocol ID: A value corresponding to the five least significant bits (LSBs) of an Advertisement Protocol ID as defined in Table 8-220 in 8.4.2.92 (Advertisement Protocol element).

[The second change against 802.11ai D2.1 text]

  - Definitions Abbreviations and acronyms

Insert new abbreviations and acronyms retaining alphabetic order as follows:

CAG	Common Advertisement Group [13/1373r0 but shortened to be an abbreviation]

[The third change against 802.11ai D2.1 text]

- MLME-SCAN.confirm

- Semantics of the service primitive

Insert the following rows at the end of the second table in this subclause.

| Name | Type | Valid Range | Description | IBSS Adoption |
| --- | --- | --- | --- | --- |
| CAG Tuple | Sequence of Intergers | As defined in 8.4.2.173 CAG Number element | The values of CAG Version, Scope, and Partial Advertisement Protocol ID from the CAG Number element if such an element was present in the Probe Response or Beacon frame, else null. The parameter is optionally present only if dot11FILSActivated is true.[CID 2813, 2640, 13/1041r1 13/1269r6] | Do not adopt. |

[The fourth change against 802.11ai D2.1 text]

- Beacon frame format

Insert new rows/elements before the last order in the table as follows:

| - Table 8-24 – Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 69 | common advertisement group ( CAG) Number[13/1373r0] [CID 2890] | CAG Number element (see 8.4.2.173 CAG Number element ) is optionally present if dot11FILSActivated is true. [13/1373r0] |

[The fifth and last change against 11ai D2.1 text]

Insert the new subclause as follows:

8.4.2.173 CAG Number element [13/1373r0]

The Common Advertisement Group (CAG) is a group of advertisement-protocol-elements, which do not change on a rapid basis, of the same advertisement protocol. The CAG Number element provides the current version number of the CAG. The CAG Number element is used by the STA to determine if a change in the CAG occurred from the last visit of the STA to a particular AP. The CAG Number element is optionally present[CID 2009] in the Beacon or Probe Response frame to reduce GAS frame exchanges when dot11InterworkingServiceActivated is true and dot11FILSActivated is true. [13/1373r0]

|  | Element ID |  |  |  |
| --- | --- | --- | --- | --- |

|  | Element ID | Length | CAG Tuple #1 | CAG Tuple #2 (optional) | … | CAG Tuple #n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 0 or 2 |  | 0 or 2 |

Figure 8-401cm−CAG Number element [CID 2656, 2893, 2443] [13/1373r0]

The Element ID and Length fields are defined in Error: Reference source not found[CID 2010]. The CAG Number element includes one or more 2-octet CAG Tuple fields as shown in Figure 8-401cm. The format of a CAG Tuple field is shown in Figure 8-[TBD].

|  | B0 B7 | B8 B10 | B11 B15 |
| --- | --- | --- | --- |
|  | CAG Version | Scope | Partial Advertisement Protocol ID |
| Bits: | 8 | 3 | 5 |

Figure 8-[TBD]–CAG Tuple field [CID 2656, 2893, 2443] [13/1373r0]

The CAG Version is a 1-octet subfield whose value is an unsigned positive integer indicating the version of the CAG of the associated advertisement protocol.

The CAG Version is incremented every time the value of, or presence of, an advertisement-protocol-element within the CAG of the associated advertisement protocol or InfoID are added, deleted, or changed.

CAG Version is always positive, therefor a value of zero in this subfield will be neglected by the receiving STA. [CIDs 2658, 2298][13/1373r0, CIDs 2931, 3109, 3111, 3112, 3288, 3292 - Accepted resolution to CID 2818 contradicts the submittal approved in the same motion, submittal version was used.]

The Scope is a 3-bit subfield that indicates the valid scope of the information represented by the CAG Version in the same CAG Tuple field. A subfield value of 0 indicates that the CAG is only valid in the current BSS (identified by the BSSID value), a subfield value of 1 indicates that the CAG is only valid in the Homogeneous ESS (identified by the HSSID value), and a subfield value of 2 indicates that the CAG is valid for the ESS (union of BSSs with the same SSID). Subfield values of 3-7 are reserved.

The Partial Advertisement Protocol ID is a 5-bit subfield carrying a value equal to the five least significant bits (LSBs) of the Advertisement Protocol ID of the advertisement protocol associated with the CAG Version in the same CAG Tuple field. The Advertisement Protocol ID is defined in Table 8-220 in 8.4.2.92 (Advertisement Protocol element).  [CID 2894] [13/1373r0, CIDs 2300, 2999, 3290]