IEEE P802.11
Wireless LANs

| Proposed 11aq Amendment Text for Adopting the Revised CAG Number Concept |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| Zhigang Rong | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 |  | zhigang.rong@huawei.com |

Abstract

This document provides the 11aq Amendment Draft text for adopting the revised CAG Number as proposed in document 11-14-1253-01-00aq.  The text included in this document is specific for PADP.

[The first change against revised 802.11ai text]

- MLME-SCAN.confirm

- Semantics of the service primitive

Insert the following rows at the end of the second table in this subclause.

| Name | Type | Valid Range | Description | IBSS Adoption |
| --- | --- | --- | --- | --- |
| CAG Tuple | Sequence of Intergers | As defined in 8.4.2.173 CAG Number element | The values of CAG Version, Scope, and Partial Advertisement Protocol ID from the CAG Number element if such an element was present in the Probe Response or Beacon frame, else null. The parameter is optionally present if dot11FILSActivated or dot11PADActivated is true.[CID 2813, 2640, 13/1041r1 13/1269r6] | Do not adopt. |

[The second change against revised 802.11ai  text]

- Beacon frame format

Insert new rows/elements before the last order in the table as follows:

| - Table 8-24 – Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 69 | common advertisement group ( CAG) Number[13/1373r0] [CID 2890] | CAG Number element (see Error: Reference source not found) is optionally present if dot11FILSActivated or dot11PADActivated is true. [13/1373r0] |

[The third change against revised 802.11ai text]

- Probe Response frame format

Change Table 8-31 as follows, inserting 5 new rows and changing one existing row:

| - Table 8-31 – Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 70 | CAG Number[13/1373r0] [CID 2891] | CAG Number element is optionally present if dot11FILSActivated or dot11PADActiviated is true. [CID 2650] [13/1373r0] |

[The fourth change against 802.11aq D0.01 text]

8.4.6.1 General

Insert a new table at the end of this subclause as follows:

Table 8-[aq1] −PADP-element definitions [CID 2656, 2893, 2443] [13/1373r0]

| PADP-element name | Info ID | PADP-element subclause |
| --- | --- | --- |
| Reserved | 0-[aq2] | N/A |
| Common Advertisement Group | [aq3] | 8.4.6.[aq5] (Common Advertisement Group PADP-element) |
| Reserved | [aq4]-56796 | N/A |
| Vendor Specific | 56797 | 8.4.2.25 (Vendor Specific elemnt) |
| Reserved | 56798-65535 | N/A |

[The fifth change against 802.11aq D0.01 text]

Insert a new subclause as follows:

8.4.6.[aq5] Common Advertisement Group PADP-element

The Common Advertisement Group PADP-element provides the Info IDs for the PADP-elements contained within a CAG associated with PADP and the current value of the CAG version number, indicating the version number of information within the CAG. The selection of the specific number of InfoIDs and the specific values of InfoIDs in a CAG associated with PADP is left to the implementation and is beyond the scope of this document.

|  | Info ID | Length | PADP CAG version number | Info ID 1 | … | Info ID n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | 2 | … | 0 or 2 |

Figure 8-[aq6] –Common Advertisement Group PADP-element [CID 2656, 2893, 2443] [13/1373r0]

[13/1373r0 CIDs 2905, 2334, 3288, 3268 - CIDs 2904 & 2905 were accepted in same motion as 13/1373r0 but offers conflicting resolution to this paragraph, implementing the submission version]The Info ID is a 2-octet field whose value is equal to the value in Error: Reference source not found.

[13/1373r0]The Length field is a 2-octet unsigned positive number whose value is set to the length in octets of remaining fields. [13/1373r0 CID 2682]

The PADP CAG version number is a 1-octet field and it indicates the current version number of the CAG associated with PADP. The PADP CAG version number changes when the CAG associated with PADP changes (either the Info IDs of the PADP-elements contained in the CAG associated with PADP or any value of the attributes and information within those PADP-elements). The Info ID field is a 2-octets field whose value represents a PADP-element Info ID specified in Table 8-[aq1] (PADP-element definitions).

[The sixth change against 802.11aq D0.01 text]

10.25.3.4 PADP procedures

Insert a new subclause as follows:10.25.3.4.1 General

A PADP-capable STA should store, for later use, the PADP CAG version number available from an AP, and also store the PADP attributes and information corresponding to that CAG version number. The STA stores the PADP CAG version number and the values of BSSID, HESSID, or SSID associated with the responding AP. A PADP-capable STA uses this CAG version number to determine if the CAG association with PADP changed from the last STA's visit to that AP. In order to reduce the network selection delay, if PADP CAG version number is provided from the AP, a PADP-capable STA should check the locally stored PADP CAG version number, to decide whether it should initiate a PADP query for new information. If the received PADP CAG version number equals the stored value of a PADP CAG version number associated with the corresponding value of BSSID, HESSID, or SSID for the AP, the STA shall not initiate a PADP query request for any of the PADP-elements of the CAG and shall use the stored PADP attributes and information within that STA instead.

| - Table 10-[aq7]—PADP usageinput.doc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | BSS |  | IBSS |
| PADP-element name | PADP-element (subclause) | PADP-element type | AP | Non-AP STA | STA |
| Common Advertisement Group | 8.4.6.[aq5] (Common Advertisement Group PADP-element) | Q,S | T, R | T, R |  |
| Symbols Q element is an PADP query S element is an PADP response T PADP-element may be transmitted by MAC entity R PADP-element may be received by MAC entity — PADP-element is neither transmitted nor received by MAC entity |  |  |  |  |  |

[The seventh and last change against 802.11aq D0.01 text]

Insert a new subclause as follows:

10.25.3.4.[aq7] CAG procedure

The CAG PADP-elemet is used by a requesting STA to perform a PADP Query to retrieve the Info IDs contained within the CAG associated with PADP and the current CAG version number associated with these Info IDs. For this purpose a STA shall use the PADP Query procedure defined in 10.25.3.4.1 (General). The requesting STA shall include in the PADP Query the Info ID of the CAG PADP-element as shown in Table 10-[aq7] (PADP Usage). When an AP receives a PADP Query List that contains the Info ID of CAG PADP-element, the AP shall include in the response frame the PADP CAG version number and the Info IDs of the PADP-elements that are comprised in the CAG associated with PADP in the increasing order of the Info ID values. The response frame should also include the (other) PADP elements that a STA requested in the PADP query list in the increasing order of the Info ID values.

The PADP CAG version number is a positive number that increases by 1 when there is any change in the CAG associated with PADP, including a change of the Info ID of the PADP-elements of the CAG or a change in the values of the PADP elements included in the CAG. If a STA receives a value of zero for the PADP CAG version number, the value will be discarded and no action should be taken.[13/1374r0]