IEEE P802.11
Wireless LANs

| TGaq – Simplification of Service Identifiers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes to remove the Service Identifier Name, as a separate type of service identifier, thus simplifiying the TGaq draft D0.01.

This uses Draft P802.11REVmc_D3.0.pdf as a baseline.

8.4.2.122a Pre-association Discovery Capabilities

The PAD Capabilities element contains information identifying Service Types and Upper Layer Protocols (ULPs) that are supported by a STA.  The PAD Capabilities element is transmitted in beacons and probe responses.

|  | Element ID | Length | Service Type mask | PAD Mode | ULP List count (optional) | ULP ID#1 (optional) | … | ULP ID#n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |

Figure 8-401aq – PADP Capabilities element format

The Element ID field is set to the value given in Table 8-54 {this table requires a new value added to it}.

The value of the Length field is 2 plus the number of optional ULP ID elements

The Service Type mask is a 1 octet bit mask, indicating the Service Types which are supported by an AP.  The format of the mask is shown in Figure 8-402aq.

|  | Peripheral | WEB Service | Streaming Multimedia | Interactive Multimedia | Location Based Services | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bit : | 0 | 1 | 2 | 3 | 4 | 5-7 |

Figure 8-402aq – Service Type mask format

The PAD Mode field indicates the mode in which PAD is operated by an AP.  The PAD Mode is one of the following:

Table 8-401aq – PAD Mode of operation

| Description | Mode |
| --- | --- |
| PAD request/response using a Service Identifier Hash | 0 |
| Encapsulated ULP | 1 |
| Reserved | 2-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

The ULP List length field is a 1 octet field indicating the length of the ULP ID sub-elements.

The ULP ID field is a 1 octet field indicating a value of a ULP that is supported by the AP.  A suggested mapping of ULP ID is shown in Table 8-402aq:

Table 8-402aq – Upper Layer Protocol Mappings

| ULP name | ULP Abbreviation | ULP ID |
| --- | --- | --- |
| List of available ULPs | - | 0 |
| DNS Service Discovery, part of Apple’s Bonjour technology | DNS-SD, Bonjour | 1 |
| Service Location Protocol | SLP | 2 |
| Simple Service Discovery Protocol as used in Universal Plug and Play | SSDP, UPnP | 3 |
| Universal Description Discovery and Integration for web services | UDDI | 4 |
| Jini for Java objects. | JINI | 5 |
| Bluetooth Service Discovery Protocol | SDP | 6 |
| Salutation | Salutation | 7 |
| XMPP Service Discovery | XEP-0030 | 8 |
| Web Services Dynamic Discovery | WS-Discovery | 9 |
| multicast DHCP | MDHCP | 10 |
| Internet Storage Name Service | iSNS | 11 |
| Web Proxy Autodiscovery Protocol | WPAD | 12 |
| Dynamic Host Configuration Protocol | DHCP | 13 |
| eXtensible Resource Descriptor Sequence | XRDS | 14 |
| JSON | JSON | xxx |
| e911 (Emergency Service) | e911 | 15 |
| Next Generation 911 (Emergency Service) | NG911 | 16 |
| Location Service | Location | 17 |
| Reserved | - | 18-220 |
| Vendor Specific | - | 221 |
| Reserved | - | 222-255 |

Insert the following new subclause 8.4.6

8.4.6 Pre-association Discovery Protocol elements

PADP provides a means to exchange service discovery information between STAs.  The elements support multiple service discovery protocols.

8.4.6.1 General

PADP-elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet Length field, and a variable-length element-specific Information field. The PADP-element format is shown in Figure 8-403aq:

|  | Info ID | Length | PADP Protocol-specific Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 8-403aq – PADP-element format

Each PADP-element in 8.4.6 is assigned a value of 276.

The Length field is a 2-octet field that indicates the number of octets in the Information field and is encoded following the conventions given in 8.2.2 (Conventions).

The PADP Protocol-Specific Information field is a variable length field that contains a specific PADP element definition.

8.4.6.2 PADP Request/Response using a Service Identifier Hash

The PADP Request/Response using a Service Identifier Hash (SIH) is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a hash as explained in Annex AQ1.  The hash is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specific Information fields are defined as follows:

8.4.6.2.1 Request PADP-element

The Request PADP-element is used to request service information between STAs using the PADP Request/Response protocol. The Request PADP-element is included in a GAS Query Request.

|  | Token | SIH | Service Type Mask |
| --- | --- | --- | --- |
| Octets: | 1 | 6 | 1 |

Figure 8-404aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

The Service Identifier Hash (SIH) field is a 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information.

8.4.6.2.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs using the GAS protocol in response to a Request PADP-element. The Response PADP-element is used in a GAS Query Response.

|  | Token | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-405aq – Response PADP-element format

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor (SD) sub-field is shown in Figure 8-406aq

|  | Length | SIH | Attributes Count | Attribute#1 (optional) | … | Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-406aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SIH sub-field is described in section 8.4.6.2.1.

The Attributes Count is a 1 octet field indicating the number of Attribute sub-fields.

The Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SIH, for example, a service name string.

The ULP sub-field is an optional 1 octet field that identifies an ULP associated with the SIH and is defined in section 8.4.2.122a.