IEEE P802.11
Wireless LANs

| Proposed Resolutions to CID 19 in CC20 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-11-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| DejianLi | Huawei/HiSilicon |  |  | lidejian@huawei.com |

Abstract

This document proposes resolutions to TGaj D0.5 CID 19.

Technical Comments

| CID | Clause | Page | Line | Type | Comment | Suggested Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 19 | 9.34a.2.1 | 40 | 38 | T | The definition of "synchronized PCP or AP" and "synchronizing PCP or AP" are not clear. | Define the "synchronized PCP or AP" and "synchronizing PCP or AP" in subclause 3.2. |  |

Proposed resolution: Accept

Do as the suggested remedy:

“Insert the following definitions into 3.2 in alphabetic order:

Synchronizing AP or PCP: A CDMG AP or PCP that is operating on a 1.08 GHz channel but still transmitting its DMG Beacon frames on the relevant 2.16 GHz channel with the PCP/AP Role subfield of Dynamic Bandwidth Control element (8.4.2.160) set to 0, and providing synchronization service to a synchronized AP or PCP on the relevant 2.16 GHz channel.

Synchronized AP or PCP: A CDMG AP or PCP that is operating on a 1.08 GHz channel but still transmitting its DMG Beacon frames on the relevant 2.16 GHz channel with the PCP/AP Role subfield of Dynamic Bandwidth Control element (8.4.2.160) set to 1, and synchronizing with the synchronizing AP or PCP on the relevant 2.16 GHz channel.”