IEEE P802.11
Wireless LANs

| LB 204 Resolution to Some CIDs on Clause 8.4.2.173 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-11-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Jae Seung Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1326 | jasonlee@etri.re.kr |
| Jaewoo Park | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5635 | parkjw@etri.re.kr |
| Yu Ro Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5693 | yurolee@etri.re.kr |
| Jeeyon Choi | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5247 | jychoi@etri.re.kr |
| Il Gyu Kim | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 5490 | igkim@etri.re.kr |

Abstract

This submission proposes comment resolutions to some CIDs on Clause 8.4.2.173

- CIDs: 6892, 6498, 6496, 6228, 6519, 6145, and 6520 (7 CIDs)

Changes in the text refer to: Draft P802.11ai/D3.0

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGai Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGai Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGai Editor: Editing instructions preceded by “TGai Editor” are instructions to the TGai editor to modify existing material in the TGai draft.  As a result of adopting the changes, the TGai editor will execute the instructions rather than copy them to the TGai Draft.

CIDs 6892, 6498, 6496, 6228, 6519, 6145, and 6520

| CID | Clause | Page | Line | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 6892 | 8.4.2.173 | 45 | 33 | The width of the PHY Support Criteria subfield is 3, not 2. | Change the width of PHY Support Criteria subfield in the Figure 8-574e to 3. | Revised - Agree with the commenter. TGah Editor to make changes shown in 14/1498 |
| 6498 | 8.4.2.173 | 46 | 41 | This should be a table | Make it into a table, and just refer to the table | Revised - Agree with the commenter in general. TGah Editor to make changes shown in 14/1498 |
| 6496 | 8.4.2.173 | 46 | 48 | "~" | Change to "to"; also delete "A" and change "value" to "values" | Revised - The sentence has been converted into a table and “ ~ “ has been removed. TGah Editor to make changes shown in 14/1498 |
| 6228 | 8.4.2.173 | 46 | 48 | 2-bit field can't have value bigger than 4 | 2-bit field can't have value bigger than 4 | Revised - The width of the PHY Support Criteria subfield is 3, not 2. The typo in the figure has been corrected. TGah Editor to make changes shown in 14/1498 |
| 6519 | 8.4.2.173 | 47 | 15 | "Max Channel Time of the MLME-SCAN.request" -- what of the what? | "MaxChannelTime parameter of the MLME-SCAN.request primitive" | Revised - Agree with the commenter. TGah Editor to make changes shown in 14/1498 |
| 6145 | 8.4.2.173 | 47 | 17 | change "It presents" to "It indicates" since "It presents" is different from standards language | as in comment | Revised - Agree with the commenter. TGah Editor to make changes shown in 14/1498 |
| 6520 | 8.4.2.173 | 47 | 18 | "since it contains the value of Max Channel Time as shown in Figure 10-3b (Active scanning when a non-DMG STA transmits Probe Request to individual address)." is quite superfluous here | Delete the cited text | Revised - Agree with the commenter. TGah Editor to make changes shown in 14/1498 |

Proposed Remedy (CIDs 6892, 6498, 6496, 6228, 6519, 6145, and 6520):

Instructions to TGai Editor: Change the subclause 8.4.2.173 as follows:

8.4.2.173 FILS Request Parameters element

…..

B0                  B2     B3                           B5    B6                 B7

| BSS Delay Criteria | PHY Support Criteria | Reserved |
| --- | --- | --- |

Bits:              3                                  2         3                        2

Figure 8-574e—FILS Criteria field

………….

P.46 Line 41:

A PHY Support Criteria field value of 0 indicates that PHY Support Criteria is not in use. A PHY Support Criteria field value of 1 indicates that a responding FILS STA is HT capable.

A PHY Support Criteria field value of 2 indicates that a responding FILS STA is VHT capable.

A PHY Support Criteria field value 3 ~ 7 are reserved.

The PHY Support Criteria field indicates the supported PHY type of the responding STA that is applied in the decision to respond to the Probe Request frame as described in 10.1.4.3.4 (Criteria for sending a response). The meaning of the values for the PHY Support Criteria is shown in Table 8-xxxx (PHY Support Criteria field).

Table 8-xxxx - PHY Support Criteria field

| Value | Meaning |
| --- | --- |
| 0 | Indicates that PHY Support Criteria is not in use. |
| 1 | Indicates that a responding FILS STA is HT capable. |
| 2 | Indicates that a responding FILS STA is VHT capable. |
| 3 | Reserved |
| 4 | Reserved |
| 5 | Reserved |
| 6 | Reserved |
| 7 | Reserved |

……………….

P.47 Line 15

The Max Channel Time field contains the value of Max Channel Time MaxChannelTime parameter of the MLME-SCAN.request primitive represented in an unsigned integer of units of 200 microseconds. It presents indicates the time that the transmitter of the Probe Request frame will be available after the transmission of the Probe Request to receive the Probe Responses since it contains the value of Max Channel Time as shown in Figure 10-3b (Active scanning when a non-DMG STA transmits Probe Request to individual address).