IEEE P802.11
Wireless LANs

| Proposed Resolution Text for CIDs 6903 6503 6530 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-01-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Abraham | Qualcomm Inc. | 5775 Morehouse Drive, San Diego, CA 92121 |  | Sabraham@qti.qualcomm.com |

Abstract

This document presents resolutions to CID 6903, 6503, 6530

All changes are indicated with respect to Draft 3.1

8 Frame Formats

8.4 Management and Extension frame Body Components

8.4.2 Elements

- FILS Indication element

Replace the paragraph on page 60 lines 57-62 as follows: (CID 6903)

The format of the Public Key Information field (when present) of the FILS Indication element is shown in Figure 8-574o (Format of Public Key Information field).The Subnet ID token is present only if indicated by the Subnet ID token present field of the FILS information field.  Public Key Information Type of the FILS indication element and the Public Key entry are set as given Table 8-XXX:

Table 8-XXX: Public Key Information type and Public Key entry setting

| Public Key Information type | Public Key entry | Description |
| --- | --- | --- |
| 0 | Not present | No Public Key entry present |
| 1 | X.500 Distinguished Name (DN) of the issuer of the AP's certificate. | A certified public key is used. |
| 2 | SHA256 hash of the AP's raw public key | A raw public key is used, |
| 3-255 | NA | Values 3-255 Reserved |

Note that the length of the Public Key entry can be inferred from the length of the FILS Indication element and the length of the Public Key Identifier field. Public Key Info 3-255 are reserved.