IEEE P802.11
Wireless LANs

| Proposed Resolution Text for CID 6064 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-1-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Abraham | Qualcomm Inc. | 5775 Morehouse Drive, San Diego, CA 92121 |  | sabraham@qti.qualcomm.com |

Abstract

This document presents resolutions to CID 6064

“In the Probe Request, STA should be able to include the Security domains it is looking for. It could be "hashed domain name", or a full domain name”

The proposed resolution is to add a field in the FILS Request Parameters Element to include the Hashed domain name.  Revised text is presented in the following.

All changes are indicated with respect to Draft 3.0

- FILS Request Parameters element

Update Figure 8-574d as shown below

|  | Element ID | Length | Parameter Control Bitmap | FILS Criteria | Max Delay Limit | Minimum Data Rate | RCPI Limit | OUI Response Criteria | Max Channel Time [CIDs 6460, 6144] | Hashed Domain Information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 0 or 1 | 0 or 1 | 0 or 3 | 0 or 1 | 0 or 2 | 0 or 1 | 0 or Var |
| - FILS Request Parameters element format [13/1269r0 but with problem, adding the last column but not recognizing “Minimum Data Rate” column and not showing it as being deleted][CID 4453] |  |  |  |  |  |  |  |  |  |  |

Update Figure 8-574e as shown below

|  | B0 |  | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | FILS Criteria Present |  | Max Delay Limit Present | Minimum Data Rate Present | RCPI Limit Present [CID 2710] | OUI Response Criteria Present | Max Channel Time Present [13/1269r0] | Hashed Domain Information Present | Reserved |
| Bits: | 1 |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | - Parameter Control Bitmap field |  |  |  |  |  |  |  |

Change Line 21 on Page 45 as follows

Bits 0 to 6 of the Parameter Control Bitmap field correspond to the Parameter fields that are[CID 2304] present in the element.[CID 2304]

Insert the following after line 21 on Page 47

|  | Number of Hashed Domain Names | Hashed Domain Name | …... | Hashed Domain Name |
| --- | --- | --- | --- | --- |
| Octets | 1 | 2 |  | 2 |

Figure 8-XXX Hashed Domain Information field

The Number of Hashed Domain Names field indicates a positive unsigned number of Hashed Domain Name fields in the Hashed Domain Information field.

The value of the Hashed Domain Names in the Hashed Domain Information field is computed from the domain name that is compliant with the preferred name syntax defined in IETF RFC 1035 (same as the domain name used in 8.4.4.15 (Domain Name ANQP-element)). The exact computation method for the Hashed Domain Name is given in 10.45.4 (FILS authentication and higher layer setup capability indications)

10. MLME
10.1 Synchronization
10.1.4 Acquiring synchronization, scanning

10.1.4.3 Active scanning

10.1.4.3.4 Criteria for sending a probe response

Insert the following after line 22 of page 83.

g) If Hashed Domain Names are present in the FILS element and none of them match with any of the Hashed domain names of the domains supported by the AP.