IEEE P802.11
Wireless LANs

| LB207 Comment Resolutions for Clause 24.2.2 and the CA Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Mingguang Xu | Marvell | 5488 Marvell Ln, Santa Clara, CA 95054 | 408-222-7766 | mxu@marvell.com |
| Hongyuan Zhang |  |  |  | Hongyuan@marvell.com |
| Liwen Chu |  |  |  | liwenchu@marvell.com |
| Naveen Kakani | CSR |  |  | Naveen.Kakani@csr.com |

Abstract: This document contains proposed resolutions for comments in Clause 24.2.2 from TGah D4.0 and the CA document (doc #: 11-13/1088r3) with the CIDs below.

| Clause 24.2.2 |  |  |  |
| --- | --- | --- | --- |
| - 6053 - 6100 CA document - 6147 - 6148 |  |  |  |

CIDs for Clause 24.2.2

| CID | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6053 | 24.2.2 | 395.51 | The entries for FORMAT is S1G_DUP_2M and AGGREGATION is AGGREGATED is repeated. Same with corresponding entry for S1G_DUP_1M. | Change AGGREGATED to NOT_AGGREGATED, and in the description box change "packet duration in number of symbols" to "packet duration in number of octets". | Accepted. See the instructions for Editor below with doc #:11-15/0394r2. |
| 6100 | 24.2.2 | 417.00 | Includes description twice for the length of: - FORMAT is S1G_DUP_2M and AGGREGATION is AGGREGATED - FORMAT is S1G_DUP_2M and AGGREGATION is AGGREGATED Missing the length for "Not Aggregated" | Change "Aggregated" to "Not_Aggregated" | Revised. See the resolution for CID #6053. The referred entry is corrected. Besides the change to the condition as suggested by this comment, the value part is also modified, i.e., “symbols” is changed to be “octets”. |

TGah editor: please make the following changes in Clause 24.2.2 of D4.0:

- On P395L51 (CIDs #6053 and #6010):  Change the 5th and 6th rows for the LENGTH part of the table with the following ones:

| FORMAT is S1G_DUP_2M and AGGREGATION is NOT_AGGREGATED. | Indicates the packet duration in number of octets in the S1G 2 MHz Duplicate PSDU.. | Y | Y |
| --- | --- | --- | --- |
| FORMAT is S1G_DUP_1M and AGGREGATION is NOT_AGGREGATED. | Indicates the packet duration in number of octets in the S1G 1 MHz Duplicate PSDU.. | Y | Y |

CIDs for CA document (11-13/1088r3)

| CID | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 6147 |  | 2 | TBD in E1-4 have to be updated in CA document, i.e. 2. Channelzation. | Update as in D4.0. | Revised. TGah Editor: Please replace Table E-1, E-2, E-3, and E-4 in 11-13/1088r3 with Table E-4a in TGah Draft 4.0. |
| 6148 |  | 3 | CA document should include adequate statement with regard to coexistence analysis, e.g. prospective coexisting system of IEEE802.15.4g. | As in comment. | Rejected. As in 11n/ac, the main mechanism for coexistence with non-802.11 devices is clear channel assessment (CCA). The section of “3 Clear Channel Assessment” in 11-13/1088r3 already provides adequate statements with regard to coexistence with IEEE802.15.4g. |

CIDs from offline discussion

| CID | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| From offline discussion | 24.3.4 | 413.31 | Section 24.3.4.5 should be more elaborate. It does not describe that there should be a phase rotation while duplicating the initial 1 MHz or 2 MHz preamble in frequency. | Explicitly mention that the duplicate transmission should be phase rotated. | Revised. Explicitly mention that phase ration should be applied for duplicate transmission. See the instructions for Editor below with doc #: 11-15/0394r2. |

Discussion:  The cited text by the commenter is attached below:

Clearly, phase rotation on each duplicated transmission is not explicitly mentioned.  It is desirable to add some texts for the purpose of clarification.

TGah editor: please make the following changes in Clause 24.3.4 of D4.0:

- On P413L31:  Add more clarification texts as below:

… …

In the case of S1G_DUP_2M, the initial 2MHz preamble is duplicated in frequency with phase rotation as defined in 24.3.4.3 (Construction of the Preamble part in an S1G_SHORT PPDU) to fill the final transmission bandwidth (i.e. 4 MHz, 8 MHz, or 16 MHz) indicated by the CH_BANDWIDTH parameter in TXVECTOR.

In the case of S1G_DUP_1M, the initial 1MHz preamble is duplicated in frequency with phase rotation as defined in 24.3.4.4 (Construction of the Preamble part in an S1G_1M PPDU) to fill the final transmission bandwidth (i.e. 2 MHz, 4 MHz, 8 MHz, or 16 MHz) indicated by the CH_BANDWIDTH parameter in TXVECTOR.

… …