IEEE P802.11
Wireless LANs

| Resolution to CID 7288 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-03-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jarkko Kneckt | Nokia | Otaniementie 19b, 02150 Espoo Finland |  | Jarkko.kneckt@nokia.com |
| Xiaofei Wang | Interdigital |  |  |  |

Abstract

The submission contains resolutions to CID7288.

Two alternative wordings are proposed. 802.11ai group should select the most suitable wording.

Please Select either Alternative A or B:

Alternative A, Current Draft 4.0

If the STA is a FILS STA, the STA sets the FILSProbeTimer to 0 and starts the FILSProbeTimer. While the FILSProbeTimer is less than dot11FILSProbeDelay the STA may skip a probe request transmission and proceed to step h) after setting the ActiveScanningTimer to 0 and starting the ActiveScanningTimer, if one of the following conditions matches:

1) The STA receives a broadcast addressed probe request.

2) The STA receives oneor more of Probe Response, Beacon, Measurement Pilot, or a FILS Discovery frame.

Alternative B

If the STA is a FILS STA, the STA sets the FILSProbeTimer to 0 and starts the FILSProbeTimer. While the FILSProbeTimer is less than dot11FILSProbeDelay the STA may skip a probe request transmission and proceed to step h) after setting the ActiveScanningTimer to 0 and starting the ActiveScanningTimer, if one of the following conditions matches:

1) The STA receives a broadcast addressed probe request that the STA considers to be suitable to discover a candidate AP for association.

2) The STA receives oneor more of Probe Response, Beacon, Measurement Pilot, or a FILS Discovery frame that identify an AP which the STA considers a suitable candidate for association.

NOTE – The logic how a STA considers the probe request suitable or the AP as a suitable candidate for association is out of the scope of this standard.

Background: Draft 3.0 text

If the STA is a FILS STA, the STA sets the FILSProbetimer to 0 and starts the FILSProbetimer. While the FILSProbetimer is less than dot11FILSProbeDelay the STA may skip a probe request transmission and proceed to step h) after setting the ActiveScanningTimer to 0 and starting the timer, if one of the following conditions matches:

1) The STA receives a broadcast addressed Probe Request frame that the STA considers to be suitable to discover a candidate AP for association. The logic how the STA considers the Probe Request suitable is out of the scope of this standard.

2) The STA receives a Probe Response or a Beacon or a Measurement Pilot or an FILS Discovery frame that identify an AP which the STA considers a suitable candidate for association. The logic how the STA considers the AP as suitable candidate for association is out of the scope of this standard.

References:

802.11ai D3.0

802.11ai D4.0