IEEE P802.11
Wireless LANs

| Reconsideration of comment resolution to CID 7083 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |

Abstract

This document proposes to reconsider the comment resolution of CID 7083 as in Doc. 11-15/447r1 and to accept the changes proposed in this document instead.

Discussion:

The issue raised by CID 7083 is agreed. However, the adopted Resolution to CID 7083 as in Doc. 11-15/447r1 (by Motion #246) is incorrect for the following reasons:

1.    Short SSID Indictor, just like many other presence indicators in the FILS Discovery frame, is used for parsing the frame properly. If we don’t pass the other presence indicators through MLME-SAP, we shouldn’t pass along this one either.

2.    The parameters that are truly interested by the upper layer is the SSID and Short SSID, not the indicator.

Proposal: to reconsider the resolution to CID 7083 by accepting the changes to 11ai D4.0 as follows:

- TGai Editor: Replace the SSID/Short SSID row in the table for BSSDescriptionFromFD in clause 6.3.3.3.2 in 11ai D4.0 with the following two rows , excluding the title row:

| Name | Type | Valid range | Description | IBSS adoption |
| --- | --- | --- | --- | --- |
| SSID | Octet string. | As defined in the SSID element. | The SSID of the found BSS. This parameter is present if the Short SSID Indicator in the received FILS Discovery frame is equal to 0. | Do not adopt |
| Short SSID | Integer | As defined in the Reduced Neighbor Report element. | The Short SSID of the found BSS. This parameter is present if the Short SSID Indicator in the received FILS Discovery frame is equal to 1. | Do not adopt |