IEEE P802.11
Wireless LANs

| 802.11 Resolutions to some GLK-GCR related Comments (Part 1) (relative to P802.11ak D1.0 and IEEE REVmc Draft 4.0) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-07-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission resolves comments 309, 396 from Letter Ballot 212 that are related to GLK-GCR.

History:

- r0 – initial submission

- r1 – updated with feedback from discussions in TGak teleconferences

- r2 – updated with feedback at the July 09-10, 2015 .11ak ad hoc

- r3 – fixed tables 8-22 and 8-24 for consistency.

| 309 | 61.27 | 27 |  | 10.24.16.4.3 |  | There needs to be an GCR for GLK variant of Block ACK (Cl. 8.3.1.9.1) to differentiate between GCR MSDUs and GCR for GLK MSDUs whose successful receipt (or otherwise) is indicated in the Block Ack Bitmap | The GCR for GLK Block Ack feature description is not complete. A variant of the GCR Block ACK needs to be defined for use with GLK. The commenter will bring a submission to resolve this comment. | Revise |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 396 | 53.32 |  | 32 |  | 9.24.6 | "The GCR Mode subfield": 802.11mc's BAR Control field contains no "GCR Mode subfield". | If the intention is to create such a subfield, then 11ak needs to modify the definition of the BAR Control field -- see 11mc 8.3.1.8.1 -- including the additon of the nevv GLK-GCR value for this revised subfield. (By the way, the current GCR subfield is a single bit, so if a modification of that is planned, it will be difficult to fit in a third value.) | Revise |

#### 8.3.1.8 BlockAckReq frame format

Editor: Edit Cl. 8.3.1.8.1 as shown below:

#### 8.3.1.8.1 Overview

The Duration/ID field value is set as defined in 8.2.5 (Duration/ID field (QoS STA)).

The RA field of the BlockAckReq frame is the address of the recipient STA.

The TA field value is the address of the STA transmitting the BlockAckReq frame or a bandwidth signaling TA. In a BlockAckReq frame transmitted by a VHT STA in a non-HT or non-HT duplicate format and where the scrambling sequence carries the TXVECTOR parameter CH_BANDWIDTH_IN_NON_HT, the TA field value is a bandwidth signaling TA.

The BAR (for block acknowledgment request) Control field is shown in Figure 8-26 (BAR Control field).

|  | B0 | B1 | B2 | B3 B4 | B5 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- | --- |
|  | BAR Ack Policy | Multi-TID | Compressed Bitmap | GCR Mode | Reserved | TID_INFO |
| Bits | 1 | 1 | 1 | 2 | 7 | 4 |

Figure 8-26—BAR Control field

The GCR Mode bits are described in Table 8-20a (GCR Mode values)

Table 8-20a GCR Mode  values

| GCR Mode (B3 B4) | Description |
| --- | --- |
| 00 | Reserved (non-GCR Modes) |
| 01 | The BAR is a GLK-GCR Block Ack Request |
| 10 | The BAR is a GCR Block Ack Request |
| 11 | Reserved |

For BlockAckReq frames sent under Delayed and HT-delayed agreements, the BAR Ack Policy subfield of the BAR Control field has the meaning shown in Table 8-21 (BAR Ack Policy subfield). For BlockAckReq frames sent under other types of agreement, the BAR Ack Policy subfield is reserved.

Table 8-21—BAR Ack Policy subfield

| Value | Meaning |
| --- | --- |
| 0 | Normal Acknowledgment. The BAR Ack Policy subfield is set to this value when the sender requires immediate acknowledgment. The addressee returns an Ack frame. See 9.29.2.7 (PSMP acknowledgment rules). The value 0 is not used in frames transmitted by a DMG STA. |
| 1 | No Acknowledgment. The addressee sends no immediate response upon receipt of the frame. The BAR Ack Policy subfield is set to this value when the sender does not require immediate acknowledgment. The value 1 is not used in a Basic BlockAckReq frame outside a PSMP sequence. The value 1 is not used in an Multi-TID BlockAckReq frame. |

The values of the Multi-TID, Compressed Bitmap and GCR Mode subfields determine the possible BlockAckReq frame variants that can be represented, as indicated in Table 8-22 (BlockAckReq frame variant encoding).

Table 8-22—BlockAckReq frame variant encoding

|  |  | subfi |  |
| --- | --- | --- | --- |
|  |  | 1 |  |
|  |  | 1 |  |
|  |  | 1 |  |

| Multi-TID subfield value | Compressed Bitmap subfield value | GCR Mode subfield value | BlockAckReq frame variant |
| --- | --- | --- | --- |
| 0 | 0 | 00 | Basic BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |
| 0 | 1 | 00 | Compressed BlockAck |
|  |  | 01 | GLK-GCR BlockAck |
|  |  | 10 | GCR BlockAck |
|  |  | 11 | Reserved |
| 1 | 0 | 00 | Extended Compressed BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |
| 1 | 1 | 00 | Multi-TID BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |

Editor: Add Cl. 8.3.1.8.7 as shown below:

8.3.1.8.7 GLK-GCR BlockAckReq variant

The TID_INFO subfield of the BAR Control field of the GLK-GCR BlockAckReq frame is set to 0. The BAR Information field of the GLK-GCR BlockAckReq frame contains the Block Ack Starting Sequence Control subfield and GLK-GCR Group Address subfield, as shown in Figure 8-30 (BAR Information field format (GLK-GCR BlockAckReq)). The Block Ack Starting Sequence Control subfield is shown in Figure 8-27 (Block Ack Starting Sequence Control subfield). The Starting Sequence Number subfield of the Block Ack Starting Sequence Control subfield contains the sequence number of the first MSDU or A-MSDU for which this BlockAckReq frame is sent. The Fragment Number subfield of the Block Ack Starting Sequence Control subfield is set to 0.

|  | B0 B15 |
| --- | --- |
|  | Block Ack Starting Sequence Control |
| bits | 16 |

Figure 8-30—BAR Information field format (GLK-GCR BlockAckReq)

8.3.1.9 BlockAck frame format

Editor: Edit Cl. 8.3.1.9.1 as shown below:

#### 8.3.1.9.1 Overview

The BA Control field is defined in Figure 8-32 (BA Control field).

|  | B0 | B1 | B2 | B3 B4 | B5 B11 | B12 B15 |
| --- | --- | --- | --- | --- | --- | --- |
|  | BA Ack Policy | Multi-TID | Compressed Bitmap | GCR Mode | Reserved | TID_INFO |
| Bits | 1 | 1 | 1 | 2 | 7 | 4 |

Figure 8-26—BA Control field

The GCR Mode bits are described in Table 8-20a (GCR Mode values)

Table 8-22a GCR Mode  values

|  | (non-GC |
| --- | --- |

For BlockAck frames sent under Delayed and HT-delayed agreements, the BA Ack Policy subfield of the BA Control field has the meaning shown in Table 8-23 (BA Ack Policy subfield). For BlockAck frames sent under other types of agreement, the BA Ack Policy subfield is reserved

Editor: Edit the following in Cl.8.3.1.9.1 as shown below:

The values of the Multi-TID, Compressed Bitmap, and GCR Mode subfields of the BA Control field determine which of five possible BlockAck frame variants is represented, as indicated in the Table 8-24 (BlockAck frame variant encoding).

Table 8-24—BlockAck frame variant encoding

|  |  | subfi |  |
| --- | --- | --- | --- |
|  |  | 10 or 11 |  |
|  |  | 10 or 11 |  |
|  |  | 10 or 11 |  |

| Multi-TID subfield value | Compressed Bitmap subfield value | GCR Mode subfield value | BlockAck frame variant |
| --- | --- | --- | --- |
| 0 | 0 | 00 | Basic BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |
| 0 | 1 | 00 | Compressed BlockAck |
|  |  | 01 | GLK-GCR BlockAck |
|  |  | 10 | GCR BlockAck |
|  |  | 11 | Reserved |
| 1 | 0 | 00 | Extended Compressed BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |
| 1 | 1 | 00 | Multi-TID BlockAck |
|  |  | 01 | Reserved |
|  |  | 10 | Reserved |
|  |  | 11 | Reserved |

NOTE—Reference to “a BlockAck” frame without any other qualification from other subclauses applies to any of the variants, unless specific exclusions are called out.

The GCR Mode field indicates whether the BlockAck frame was sent in response to a GCR Mode BlockAckReq or a GLK-GCR BlockAckReq frame. The GCR Mode field is set to 10 when the BlockAck frame is sent in response to a GCR BlockAckReq frame and set to 01 when the BlockAck frame is sent in response to a GLK-GCR BlockAckReq and set to 00 otherwise.

The meaning of the TID_INFO subfield of the BA Control field depends on the BlockAck frame variant type. The meaning of this subfield is explained within the subclause for each of the BlockAck frame variants.

The meaning of the BA Information field depends on the BlockAck frame variant type. The meaning of this field is explained within the subclause for each of the BlockAck frame variants.

Editor: Add Cl. 8.3.1.9.7 as shown below:

8.3.1.9.7 GLK-GCR BlockAck variant

The TID_INFO subfield of the BA Control field of the GCR BlockAck frame contains the TID for which

this BlockAck frame is sent.

The BA Information field of the GCR BlockAck frame comprises the Block Ack Starting Sequence Control,

and the Block Ack Bitmap subfields, as shown in Figure 8-37a (BA Information field

format (GLK-GCR BlockAck)). The Block Ack Starting Sequence Control subfield is shown in Figure 8-27

(Block Ack Starting Sequence Control subfield). The Starting Sequence Number subfield of the Block Ack

Starting Sequence Control subfield contains the sequence number of the first A-MSDU for which this

BlockAck frame is sent. The value of this subfield is defined in 9.24.10 (GCR block ack). The Fragment

Number subfield of the Block Ack Starting Sequence Control subfield is set to 0.

|  | Block Ack Starting Sequence Control | Block Ack Bitmap |
| --- | --- | --- |
| Octets | 2 | 8 |

Figure 8-37a—BA Information field format (GLK-GCR BlockAck)

The Block Ack Bitmap subfield is 8 octets in length and is used to indicate the received status of up to

64 MSDUs and A-MSDUs. Each bit that is equal to 1 in the Block Ack Bitmap subfield acknowledges the

successful reception of a single MSDU or A-MSDU in the order of sequence number, with the first bit of the

Block Ack Bitmap subfield corresponding to the MSDU or A-MSDU with the sequence number that

matches the value of the Starting Sequence Number subfield of the Block Ack Starting Sequence Control

subfield.