IEEE P802.11
Wireless LANs

| TGay Evaluation Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-07-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ganesh Venkatesan | Intel Corporation | 2111NE 25th Ave, Hillsboro, OR 97124 | +1 503 334 6720 | Ganesh.venkatesan@intel.com |
| Laurent Cariou | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 |  | laurent.cariou@intel.com |

Abstract

This document describes the TGay evaluation methodology. As appropriate, it reuses elements from the .11ad evaluation methodology (09-11-296r16). Since .11ay Use Cases span a wide spectrum, the simulation scenarios can be very large. An approach needs to be adopted by the TG to limit the number of simulation scenarios to something manageable while addressing all the requirements. For each system level simulation, key simulation parameters and corresponding performance characteristics should be identified. Proposals will be evaluated based on how well they deliver against the identified performance characteristics.

# Contributors

(This will grow to reflect those providing explicit contributions / review comments of this document.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Carlos Cordiero | Intel Corporation | 2111NE 25th Ave, Hillsboro, OR 97124 | +1 (503) 712-9356 | Carlos.cordiero@intel.com |
| Artyom Lomayev | Intel Corporation |  | +78312969444 | Artyom.lomayev@intel.com |
| Solomon Trainin | Intel Corporation |  |  | Solomon.trainin@intel.com |
| Kerstin Johnsson | Intel Corporation |  |  | Kerstin.johnsson@intel.com |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | initial contribution | 12 July 2015 |

# Introduction

The evaluation methodology defines conditions for functional requirements compliance, PHY performance, and a limited set of simulation scenarios and comparison criteria for evaluating proposals.

# Conditions for Functional Requirement Compliance

## Point-to-point link simulation

Synthetic test case to demonstrate compliance with requirements in [1], requirement 11ay.

- Two stations

  - STA 1 is source

  - STA 2 is sink

- traffic from STA1 to STA2

  - protocol: UDP

  - offered load: infinite

  - MSDU size: 8 Kbytes

- PHY channel impulse response and pathloss model (Use Usage Model document as the base)

  - (Indoor) home living room

  - (outdoor) backhaul – at least street level backhaul

  - Ultra Short Range (USR)

  - Outdoor access (MU-case)

  - Wearables

- Meet 11ay requirement in [1]: throughput measured at the MAC layer is at least 20 Gbps.

## Power Efficiency

Excerpt from .11ax Evaluation Methodology for Energy Efficiency

Per STA Energy per Transmit Bit

The metric of per STA energy per transmitted bit, measured in units of joules per bit, is defined as the total energy consumed by a STA divided by the total number of successful data bits transmitted by the STA.

Per STA Energy per Receive Bit

The metric of per STA energy per received bit, measured in units of joules per bit, is defined as the total energy consumed by a STA divided by the total number of successful data bits received by the STA.

Energy Efficiency Ratio (EER)

Energy efficiency ratio is defined as the ratio of average energy consumed during one successfully exchanged data bit between STAs using any new proposed power save mechanism over the baseline power save mechanism.

The values for voltage and current may be chosen from Power Model Parameter table 14/980r10.

End of Excerpt from .11ax Evaluation Methodology

## Link budget parameters

The table below needs to be discussed and updated.

The values included are to initiate discussion but should be considered as TBD.

| Parameters | Units | Value |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Indoor | Access (Aps fo outdoor /large indoor) | Outdoor (backhaul Single Hop) | Ultrashort Range |  |
| Max Range | Meters | 10/20 | 100 | 1000 | 0.15 |  |
| FCC EIRP limit | dBm | 40 | 82/40 | 82 (if antenna gain >=51) | 40 |  |
| Max Tx Power | dBm | 10 | 10 | 31 | 1 |  |
| Noise Figure | dB | 10 | 10 | 7 | <10 |  |
| Max Tx antenna gain (AP/STA) | dBi | 14/14 | 14/14 | 51/51 |  |  |
| Max Rx antenna gain (AP/STA) | dBi | 14/14 | 14/14 | 51/51 |  |  |
| Min Pathloss | dB | 98 | ? | 185 | Range is a few centimeters | 10 meters LOS + 10 dB additional pathloss for NLOS |

# PHY Performance

The criterion for comparison of PHY characteristics are PER vs. SNR curves for different operation modes and different modulation and coding schemes of the system.

## PHY Channel Impulse Response

In order to calculate PER vs. SNR curves, decoupling of the channel impulse response and path loss characteristics of the general channel model is required as follows:

- channel impulse response (CIR) is to be normalized on an instantaneous basis (packet-by-packet). Instantaneous normalization of the CIRs is performed after application of beamforming

- standardized antennas

Several models are proposed below and depending on the simulation scenario one of the proposed models will be chosen. Also, it is proposed that the SISO and SU-MIMO models below be used for Link Level Simulations while the MU-MIMO model will be for System Level Simulation.

- SISO

a.       Antenna configuration:

i.      Steerable antenna model with 300 main lobe beamwidth and -20 dB back lobe (legacy from 11ad, basic antenna model for channel model development)

ii.     Phased antenna array with rectangular geometry of M by N elements, (this antenna can be used for collecting the CIRs/SNRs data base and then using it as a golden set for PHY performance verification)

b.      Antenna combinations:

i.      Omni TX to omni RX (viable for LOS short range applications with definition for omni as for one element of the array, can be considered as optional)

ii.    Omni TX to directional RX, directional TX to omni RX (applied for control PHY only with TXSS/RXSS beamforming, can be considered as optional)

iii.   Directional TX to directional RX (this is a basic one applied for high rate SC/OFDM PHY transmission, mandatory)

c.       Polarization types:

i.      Linear at both sides (V-V or H-H), (mandatory)

ii.      Circular for Access Point (AP) and single linear for user (V or H), (optional)

- SU-MIMO

a.       Antenna configurations, phased antenna array is assumed, maximum configuration is 4 x 4 (TBD):

- 2 x 2 configuration: single array with single polarization type (V or H), creates MIMO spatial diversity by steering two beams in different directions (reflections), NLOS

- 2 x 2 configuration: single array with dual polarized antenna (each array has V and H polarized elements), creates MIMO diversity using polarization, LOS, NLOS

- 2 x 2 configuration:  two arrays separated by the distance of d = 30 cm (typical laptop’s lid  edge length) with identical linear polarization (V or H), creates MIMO diversity exploiting antennas separation, LOS NLOS

- 4 x 4 configuration: assumes combination of i and ii or ii and iii

b.      Antenna combinations:

- Directional TX to directional RX (mandatory)

c.       Polarization types:

- Linear single polarization: V or H identical at both sides (mandatory)

- Linear dual polarization: V and H at both sides (mandatory)

- Circular for Access Point (AP) and single linear for user (V or H), (optional)

- Circular for AP and dual polarization for user (V and H), (optional)

- MU-MIMO

<tbd>

## Hardware impairments

- phase noise:

- model

- parameters

- PSD(0) =  -90 dBc/Hz

- Pole frequency fp = 1 MHz

- Zero frequency fz =  100 MHz

- Corresponding PSD(infinity) = -130 dBc/Hz

- impairment is modeled at both transmitter and receiver

- PA non-linearity model:

- Rapp AM-AM

- Modified Rapp AM-PM

in degrees

- CMOS PA model parameters

  - AM-AM

    - g = 4.65

    - Asat = 0.58

    - s = 0.81

  - AM-PM

    -  = 2560

    -  = 0.114

    - q1 = 2.4

    - q2 = 2.3

- Calculate backoff as the output power backoff from full saturation:

    - PA Backoff = 10 log10(Average TX Power/Psat)

    - Disclose: (a) EIRP and how it was calculated, (b) PA Backoff

    - Note: a PA Backoff equal to 8 dB for OFDM and 0.5 dB for single carrier is recommended.

- carrier frequency offset and symbol clock:

- fixed carrier frequency offset of  –13.675 ppm at the receiver, relative to the transmitter

- The symbol clock shall have the same relative offset as the carrier frequency offset

- Downlink multi-user simulations for all comparisons except offset compensation shall be run using a fixed carrier frequency offset selected from the array [N(1) ,N(2),……,N(8) ], relative to the transmitter, where N(j) corresponds to the frequency offset of the j-th client and is randomly chosen from [-20,20] ppm with a uniform distribution.

- Transmitter and receiver I/Q imbalance:

- Imbalance model

- distortion coefficients

## Comparison Criteria

    - PER vs. SNR curves

- all MCS’s

- channel impulse responses from channel model document

    - AWGN

    - home living room

    - office conference room

    - enterprise cubicle

    - indoor large area – Classroom, hotel lobby, etc

    - USR

    - Outdoor backhaul

    - Outdoor Multi-User

    - Wearables

- antenna combinations for each channel model: (antenna models for outdoor and outdoor Backhaul cases)

    - Omni TX to omni RX; LOS

    - Omni TX to directional RX; NLOS

    - Directional Tx to omni Rx

    - Directional TX to directional RX; NLOS

    - Directional TX to directional RX; LOS (USR, outdoor backhaul)

- Antenna combination for MIMO and MU-MIMO (TBD)

    - single array with single polarization type

    - single array with dual polarized antenna

    - multiple arrays separated by the distance of d = 30 cm with identical linear polarization

    - …

- simulations must include:

    - all impairments outlined in 3.2

    - timing acquisition on a per-packet basis

    - preamble detection on a per-packet basis

# System Evaluation

## Traffic Models

Full buffer model is baseline – users always have DATA to send and receive.

This model should represent sufficiently well the requirements for the high throughput traffic:

- Lightly compressed video

- Backhaul traffic

- Local file transfer

- Productivity docking

- Hard disk file transfer

- gaming

A more realistic FTP traffic model may be used based on [7] to better represent traffic in specific scenarios.  Specific parameters are TBD.

Traffic models are described in appendix 1

| Traffic | Subtype | Description | Rate, Mbps (Average/Peak) | Packet Error Rate | Jitter, ms | Delay, ms |
| --- | --- | --- | --- | --- | --- | --- |
| Video | Video Lightly Compressed† | HD, 1920x1080 pixels, Motion JPEG2000[3] | 150 | 1.00E-07 | 20 | 20 |
|  |  | VHD, 4k*2k, Motion JPEG2000 | 600 | 1.00E-07 | 20 | 20 |
|  |  | UHD, 8k*4k, Motion JPEG2000 | 2400 | 1.00E-07 | 20 | 20 |
|  |  | 3D VHD, 4kp, Motion JPEG2000 | 900 | 1.00E-07 | 20 | 20 |
|  |  | 3D UHD, 8kp, Motion JPEG2000 | 3600 | 1.00E-07 | 20 | 20 |
| Outdoor backhaul |  |  | 20000 | 3.00E-07 | 30 | 30 |
| Productivity docking | Monitor 4K lightly compressed |  | 300/1500 |  | 10 | 10 |
|  | Monitor 5K lightly compressed |  | 500/2700 |  | 10 | 10 |
|  | Monitor 8K lightly compressed |  | 1600/8000 |  | 10 | 10 |
|  | USB HID |  | 100 |  | 10 | 10 |
|  | USB total |  | 8000 |  | NA | NA |
|  | Ethernet |  | 2000 |  | NA | NA |
|  | Mobile to mobile |  | 8000 |  | NA | NA |
|  | combined wireless link | 2 monitors+USB+Ethernet | 13200 |  | 10 | 10 |
| Gaming | First-person Shooter | Like CS and games in Xbox 360 | 20 | 1.00E-03 | 10 | 10 |
|  | Real-time strategy |  | 0.08 | 1.00E-02 | 40 | 40 |
|  | Turn based games |  | 0.005 | 1.00E-02 | 400 | 400 |
|  | Interactive real-time gaming (assumption) |  | 100 | 1.00E-03 | 100 | 100 |
| Internet Access | FTP |  | 200 | 1.00E-03 | 100 | 100 |
|  | Internet Browsing |  | 0.2 | 1.00E-03 | 50 | 50 |
|  | Twitter & Facebook |  | 20 | 1.00E-03 | 50 | 50 |
|  | IM |  | 0.2 | 1.00E-03 | 50 | 50 |
|  | VoIP |  | 0.02 | 1.00E-03 | 50 | 50 |
|  | High-def audio |  | 0.05 | 1.00E-02 | 10 | 10 |
|  | Online Videos |  | 20 | 1.00E-03 | 20 | 20 |

## PHY Model

PHY abstraction and path loss models should be used for system level simulations.

PER vs. SNR curves obtained for PHY performance evaluation as described in Section 2.1 may be used for the PHY abstraction. Alternatively, a different PHY abstraction mechanism could be used, a description for which should be provided.

Path loss models developed in [Error: Reference source not found, Section 7] should be used for system evaluation.

As explained in Section 2.1, a PHY model (PER vs SNR curves and path loss models) may be derived if parameters of the antenna and beamforming algorithm are fixed. Section 2.1 defines the standard set of antenna and beamforming parameters. However, TGay proposals may also include system evaluation results based on their proposed antenna and beamforming algorithm.

## Simulation Scenarios

As explained in the introduction, as Use Cases span a wide spectrum, the simulation scenarios can be very large. An approach needs to be adopted by the TG to limit the number of simulation scenarios to something manageable while addressing all the requirements.

The current section incorporates a list of scenarios, as a starting point to apply the previously defined approach.

The current section includes the following scenarios:

- Home living room (from 11ad with rate adaptation for 4K/8K)

- Office conference room (from 11ad: TBD adaptations)

- Enterprise cubicle (from 11ad: TBD adaptations)

- Dense Enterprise cubicle (extension of the enterprise cubicle scenario as for 11ax enterprise scenario)

- Outdoor backhaul (TBD)

- Outdoor hotspot access (TBD: layout could be inspired from hotspot scenario in 11ax)

- Ultra short range (TBD)

- Dense wearables use in airplane (new proposition)

### Home living room

An example of a home living room floor plan is show in Figure 1.

Figure 1: Example of home living room floor plan

Set top box transmitting lightly compressed video, and TV receiving lightly compressed video

- configuration as show in Figure 1, with 3 meter separation between STB and TV (Note: PHY channel model was developed with STB in the rectangular sector as show in Figure 1)

- traffic type

- Lightly compressed video (includes 4K/8K rates with similar slice sizes but more slices to accommodate the higher resolutions)

- PHY channel impulse and pathloss model

- Home living room

- NLOS

### Office conference room (need to add Multi-User DL modes)

The office conference room floor plan is shown in Figure 2.

Figure 2: Office conference room floor plan

Mix of uses:  Laptop transmitting lightly compressed video to projector.  Multiple laptops connected to an AP that in the default scenario has a 60 GHz radio; and in the optional scenario does not have a 60 GHz radio.  Laptop connected to device performing sync-and-go file transfer.  Laptops connected to other laptops performing local file transfer.  Links between devices are logical, e.g. STA 3 and STA 5 are performing local file transfer between each other but the physical link could be direct or through the AP.

Note: In the optional scenario where the AP does not have a 60 GHz radio, then all client-to-AP communication is assumed to occur using other bands such as the 2.4 or 5 GHz band.

- configuration

- Room dimensions (length, width, height) in meters is 3.0 x 4.5 x 3

- Devices (coordinates of devices are calculated using coordinate axes shown in Figure 2)

    - AP (may or may not have a 60 GHz radio): location (x = 1.50 m, y = 0.50 m, z = 2.90 m – in ceiling)

    - STA 1:

      - Projector

      - location: ( x = 1.75 m,  y =2.30 m, z fixed at 1 m)

      - Traffic type: receiving lightly compressed video from STA 2 (LOS link)

    - STA 2:

      - Mobile Device

      - location:  (x = 1.90 m, y = 1.50 m,  z fixed at 1m)

      - Traffic type:

        - transmitting lightly compressed video to STA 1 (LOS link)

        - Local file transfer from AP

      - Antenna capability: >=60 degree HPBW pointed towards STA 1

    - STA 3:

      - Laptop

      - location:  (x = 1.35 m, y = 3.00 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 5  (NLOS link)

        - web browsing

    - STA 4:

      - Laptop

      - location:  (x = 1.30 m, y = 2.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to AP

        - web browsing

    - STA 5:

      - Laptop

      - location:  (x = 1.25 m,y = 1.40 m, z fixed at 1m)

      - Traffic type:

        - Local file transfer to/from STA 3 (NLOS link)

        - web browsing (if AP has a 60 GHz radio) or file transfer to STA 7 (if the AP does not have a 60 GHz radio)

    - STA 6:

      - Laptop

      - location:  (x = 1.55 m, y = 1.20 m, z fixed at 1m)

      - Traffic type:  web browsing

    - STA 7:

      - Laptop

      - location:  (x = 1.85 ,y = 3.10, z fixed at 1m)

      - Traffic type:

        - local file transfer to STA 8 (LOS link)

        - Local file transfer from AP (if the AP has 60 GHz radio) or file transfer from STA 5 (if AP does not have a 60 GHz radio)

    - STA 8:

      - mobile device

      - location: (x = 1.60, y = 3.25, , z fixed at 1m)

      - Traffic type: Local file transfer from STA 7 (LOS link)

      - Antenna capability: >= 60 degree HPBW pointed towards STA 7

- PHY channel impulse and pathloss model

- office conference room

- All links to the AP are LOS links that may be blocked by people. Model of the human blockage is in [Error: Reference source not found]. Type of links (LOS or NLOS) between two STAs is specified above.

- The following non-communicating pairs have NLOS channels: STA2  STA7, STA2  STA8, STA6  STA7, STA1 STA8. All other pairs have LOS channels, except due to human blockage.

- Interference modeling

    - Generate a set of channel realizations with inter-cluster parameters for all the links defined in the scenario with ray-tracing.  Store the generated channel realizations as a channel realization table in the simulator.

    - Total number of the channel realizations to be generated before simulations: 
2 STAs out of 9 STAs =92=36channel realizations

    - Signal power calculation procedure over the n-th link for a MAC simulator:

      - A packet received over the n-th link (link-n)

      - Read the link-n’s channel realization from the channel realization table

      - Block part of clusters and apply human blockage if necessary

      - Generate intra-cluster

      - Apply antenna model and beamforming

      - Calculate received signal power

Figure 3: Signal power calculation procedure over the n-th link

### Enterprise cubicle

Figure 4: Enterprise cubicle floor plan

Figure 5: Locations of the STAs within a cube

Mix of uses:  Laptop transmitting lightly compressed video to monitor.  Laptop connected to AP.  Laptop connected to hard drive.

- configuration

  - cubicle layout

    - single cubicle (length, width) in meters: 2.5m x 1.8 m

    - 8 cubicles in 4 rows, 2 columns

    - ceiling height 3m

    - Populate three cubes at Cubicle 1, Cubicle 2, and Cubicle 5 in the fixed locations as shown in Figure 4 using same frequency channel

  - Floor dimension: 25m x 25m

  - AP: AP located in the ceiling in the middle of the group of cubicles as indicated in Figure 4, at a height of 2.9m)

  - Devices in each of the populated cubes

    - STA 1:

      - monitor

      - location: (x=0.5 m,y=0.5 m) from the reference point of each cube shown in Figure 5, z fixed at 1m

      - Traffic type: receiving lightly compressed video from STA 2

    - STA 2:

      - Laptop

      - location: (x=1 m,y=0.25 m) from the reference point of each cube shown in Figure 5, z fixed at 1m

      - Traffic type:

        - transmitting lightly compressed video to STA 1 with target bit rate (p)

        - Local file transfer to/from AP

        - Local file transfer to/from STA 3

        - web browsing to/from AP

    - STA 3:

      - hard drive

      - location: (x=0.25m,y=1m)from the reference point of each cube shown in Figure 5, z fixed at 1m

      - Traffic type: Hard disk file transfer to/from STA 2

- PHY channel and pathloss model

- Enterprise cubicle

- TBD mix of LOS and NLOS in cubicle, NLOS between cubicles, TBD mix of LOS/NLOS to AP

- Interference modeling

    - Use the same procedure as described in Figure 3.

    - Total number of the channel realizations to be generated before simulations: 
2 STAs out of 9 STAs + 3 links between the laptops to the AP =92+3=39channel realizations

### Dense enterprise cubicles

- Proposition to be inspired from the enterprise scenario in TGax.

- Multiple rooms in a building floor, multiple cubicles and BSSs per room and STA positioning per cubicle.

- STA positioning and traffic in cubicle can be inspired from the new enterprise cubicle scenario or from 11ax enterprise scenario.

Example:

Figure 1 - BSSs within the building floor

Figure 2 - STAs clusters (cubicle) and AP positions within a BSS

Figure 3 - STAs within a cluster

### Outdoor (backhaul)

### Outdoor hotspot access

### Ultra Short Range

### Dense wearables use in airplane

An example of a usage scenario for wearables is given in Figure 7.  We use an airplane as our sample scenario since it poses a potential threat to the technology’s ability to serve users given that planes are often full, and passengers are highly likely to use high-end wearables for entertainment during long flights. The cabin layout is given in Figure 7, while cabin and seat dimensions are given in Figure 8.

Figure 7.  The Boeing 737-700 has 128 seats; 8 in First and 120 in Economy.

Figure 8.  Cabin length is 950”, width is 139.2”, and height is 86.6”.  First class seats are ~22” wide (not incl. armrests) with a 36” pitch, while Economy seats are ~17” wide with a 32” pitch.  Other relevant measurements can be found in the diagrams.

- Configuration

  - Full flight with all passengers seated and flight personnel (3 persons) in random standing locations throughout the cabin.

  - Passenger’s high-end wearable is head-worn, and it communicates with the passenger’s smartphone for data content and processing.

    - Head-worn wearable may have antenna structure on front, side, or back of head.

    - Smartphone may be laying on the seat tray or held in lap.

  - Transmissions between devices of different passengers are considered interference.

  - 40% of passengers (randomly distributed throughout seats) are using their high-end wearable during the flight.

  - There are no other in-band devices are active in the airplane.

- Traffic type

  - Compressed video

  - Interactive gaming

- PHY channel and pathloss model

  - Wearables in commuter scenario (TBD)

## Comparison Criteria

- goodput (aggregate and per flow)

  - average

- delay (per flow)

  - average

  - # of packets that exceed delay requirement

- packet loss rate (per flow)

- Provide description of PHY abstraction & antenna model

- Provide description of scheduling algorithm

# References

- NG60 PAR: 14-11-1151r8

- NG60 CSD: 14-11-1152r8

- 11-14-1386r1: NG60 Usage Models

- 11-15-0328r4 NG60 Use Cases (update to refer to the new .11ay Use Case document)

- 11-14-1486r0 Channel Models for NG60

- <add Functionial Requirements document>

- 3GPP TR 36.814 Annex A.2.1.3.1 FTP Traffic model 1

# Appendix 1: traffic models

- Outdoor (backhaul)

  - Throughput: > 20Gbps

  - Range: > <tbd> m LoS

  - Latency: 5-35 msecs

  - Availability: 99.99% in heavy rain

- lightly compressed video (assuming H.264 I-frame only)

  - Requirements

    - Application PLR: 1e-8

    - Delay: 10 ms

  - Parameters

    - Slice inter-arrival time (IAT) = 1/4080 seconds (1/8100 and 1/16200 seconds for 4K and 8K respectively)

    - µ = 15.798 Kbytes

    - σ = 1.350 Kbytes

  - b = 515, 1023, 2047 Mbps (for 1080p, 2160p and 4320p respectively Algorithm for each video source – Input: target bit rate in Mbps (p); Output: slice size in Kbytes (L)

    - At each IAT, generate a slice size L with the following distribution: Normal(µ*(p/b), σ*(p/b))

      - If L > 92.160 Kbytes, set L = 92.160 Kbytes (1080p)

      - If L > 180 Kbytes, set L  = 180 Kbytes (2160p aka 4K)

      - if L > 360 Kbytes, set L = 360 Kbytes (4320p aka 8K)

- Productivity docking traffic [TBD]

- Local file transfer

  - protocol: TCP (Reno)

  - offered load: infinite

  - MSDU sizes: 64 bytes for TCP connection establishment (3-way handshake) and 1500 bytes for payload data.

  - Algorithm: at the start of simulation, generate a TCP connection establishment with the following TCP parameter configuration (as appropriate for the simulation platform):

| TCP Model Parameters |  |
| --- | --- |
| MSS | Ethernet (1500) |
| Receive Buffer (bytes) | 65535 |
| Receive Buffer Adjustment | None |
| Delayed ACK Mechanism | Segment/Clock based |
| Maximum ACK Delay (sec) | 0.05 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Enabled |
| Selective ACK (SACK) | Disabled |
| ECN Capability | Disabled |
| Segment Send Threshold | Byte Boundary |
| Active Connection Threshold | Unlimited |
| Karn's Algorithm | Enabled |
| Nagle Algorithm | Disabled |
| Initial Sequence Number | Auto Complete |
| Initial RTO (sec) | 3.0 |
| Min RTO (sec) | 1.0 |
| Max RTO (sec) | 64.0 |
| RTT Gain | 0.125 |
| Deviation gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity | 0.5 |

- Web browsing

  - Protocol: HTTP (version 1.0 or above)

  - MSDU sizes: 350 bytes for HTTP requests and 1500 bytes for payload data

  - Algorithm: After each reading time the new requests for pages are generated by the user (mean of 31 seconds), generate a HTTP request with the following parameters enlisted below. The parsing time is the time taken by the HTTP page to fill in all subpage requests which appear from the master page. After going through few of the subpages the user quits the session which is indicated by the last packet of the session. This is shown in Error: Reference source not found.

Reading Time

First Packet of Session

Last Packet of Session

Reading Time

Parsing Time

Figure 6: HTTP traffic pattern

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes SD = 25032 bytes Min = 100 bytes Max = 2 Mbytes (before truncation) | σ=1.37,μ=8.37 if x>max or x<min, discard and generate a new value for x |
| Embedded object size (SE) | Truncated Lognormal | Mean = 7758 bytes SD = 126168 bytes Min = 50 bytes Max = 2 Mbytes (before truncation) | σ=2.36,μ=6.17 f x>max or x<min, discard and generate a new value for x |
| Number of embedded objects per page (Nd) | Truncated Pareto | Mean = 5.64 Max. = 53 (before truncation) | Subtract k from the generated random value to obtain Nd if x>max, discard and regenerate a new value for x |
| Reading time (Dpc) | Exponential | Mean = 30 sec |  = 0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec | fx=λe−λx,x≥0 λ=7.69 |

- Hard disk file transfer  (should be modified to reflect UASP)

  - Transaction Model

    - A transaction consists of a READ request from host to drive for a specific block of data

    - Followed by the data transfer from drive to host

  - Algorithm

    - Compute sequence of inter-arrival times

    - Compute corresponding sequence of transaction data sizes

  - Parameters

    - READ request is a short (256B) packet sent from host to drive

    - fixed 1ms delay between receipt of READ request and data offered

    - Compute sequence of inter-arrival times of transaction requests with following discrete random variable distribution

    - Compute corresponding sequence of transaction data sizes with following discrete random variable distribution

- Outdoor access traffic model

- Full buffer traffic model

- Gaming traffic model