| Complete Proposal for IEEE802.11aj(45GHz) |  |  |
| --- | --- | --- |
| Date: 2015-07-13 |  |  |
| Author(s)/Supporter(s): |  |  |
| Name | Company | Email |
| HE, Shiwen | SEU /CWPAN | shiwenhe@seu.edu.cn |
| WANG, Haiming | SEU /CWPAN | hmwang@seu.edu.cn |
| HUANG, Yongming | SEU /CWPAN | huangym@seu.edu.cn |
| HONG, Wei | SEU /CWPAN | weihong@seu.edu.cn |
| YANG, Luxi | SEU /CWPAN | lxyang@seu.eud.cn |
| CHEN, Jixin | SEU /CWPAN | 101010789@seu.edu.cn |
| ZHOU, Jianyi | SEU /CWPAN | jyzhou@seu.edu.cn |
| ZHANG, Nianzu | SEU /CWPAN | nzzhang@seu.edu.cn |
| ZHU, Xiaowei | SEU /CWPAN | xwzhu@seu.edu.cn |
| ZHANG, Yan | SEU /CWPAN | yanzhang@emfield.org |
| YANG, Guangq | SEU /CWPAN | eyed@seu.edu.cn |
| ZHAI, Jianfeng | SEU /CWPAN | zhaijianfeng@seu.edu.cn |
| CHEN, Zhe | SEU /CWPAN | zhechen@seu.edu.cn |
| MA, Chunli | SEU /CWPAN | 220130640@seu.edu.cn |
| LI, Yuanwen | SEU /CWPAN | 220130681@seu.edu.cn |
| MA, Shimin | SEU /CWPAN | 220130729@seu.edu.cn |
| YU, Denggao | SEU /CWPAN | 220130708@seu.edu.cn |
| SUN, Yu | SEU /CWPAN | 220130773@seu.edu.cn |
| YU, Guangshi | SEU /CWPAN | 220130796@seu.edu.cn |
| ZUO, Gan | SEU /CWPAN | 220130606@seu.edu.cn |
| WEI, Mingjun | SEU /CWPAN | 220140696@seu.edu.cn |
| LIAO, Shuri | SEU /CWPAN | 220140692@seu.edu.cn |
| XUE, Chunlin | SEU /CWPAN | 220140810@seu.edu.cn |
| Ou, Feifei | SEU /CWPAN | 220140736@seu.edu.cn |
| ZHANG, Jun | ZTE/CWPAN | zhang.jun7@zte.com.cn |
| JIANG, Hua | ZTE/CWPAN | Jiang.Hua@zte.com.cn |
| SUN, Bo | ZTE/CWPAN | sun.bo1@zte.com.cn |
| LI, Liguang | ZTE/CWPAN | li.liguang9@zte.com.cn |
| TIAN, Kaibo | ZTE/CWPAN | tian.kaibo@zte.com.cn |
| XING, Weiming | ZTE/CWPAN | xing.weimin@zte.com.cn |
| TANG, Shanshan | ZTE/CWPAN | tang.shanshan3@zte.com.cn |
| XUE, Haitao | ZTE/CWPAN | xue.haitao125@zte.com.cn |
| GAO, Hui | ZTE/CWPAN | gao.hui39@zte.com.cn |
| LIU, Jianhan | Media Tek/CWPAN | Jianhan.liu@mediatek.com |
| HSU, Frank | Media Tek/CWPAN | frank.hsu@mediatek.com |
| LI, Yan | Gigaray/CWPAN | Yan_li@gigaray.cn |
| HUANG, Feng | Gigaray/CWPAN | feng_huang@gigaray.cn |
| CHEN, Jiamin | Huawei /CWPAN | Jiamin.chen@mail01.huawei.com |
| CHEN, Qian | I2R / CWPAN | qchen@i2r.a-star.edu.sg |
| LI, Dejian | Huawei/CWPAN | lidejian@huawei.com |
| LIU, Pei | Hisilicon /CWPAN | liupei@hisilicon.com |
| PENG, Xiaoming | I2R/CWPAN | pengxm@i2r.a-star.edu.sg |
| ZHUO, Lan | CESI/CWPAN | this_zl7812@163.com |
| ZOU, Weixia | BUPT/CWPAN | zwx0218@bupt.edu.cn |
| CHEN Peng | SEU /CWPAN | topcp@sina.com |
| TIAN, Lin | SEU /CWPAN | 380947194@qq.com |
| WANG, Yu | SEU /CWPAN | wangyu359@outlook.com |
| YAN, Wen | SEU /CWPAN | wyan@seu.edu.cn |
| WU, Bo | SEU /CWPAN | 220130654@seu.edu.cn |
| CHEN, Feng | SEU /CWPAN | ansonred@qq.com |
| GAO, Su | SEU /CWPAN | 834474783@qq.com |
| LI, Changgen | SEU /CWPAN | 249366414@qq.com |

Abstract

This document is in support of the complete proposal (45GHz) described in 802.11-15/0707r1 (slides). This presentation is edited by using IEEE 802.11 draft template and is not a formal draft specification.

Contents

1	Overview	1

2	Normative references	3

3	Definitions, acronyms and abbreviations	4

3.1	Definitions	4

3.2	Definitions specific to IEEE 802.11	4

3.3	Abbreviations and acronyms	4

4	General description	6

5	MAC service definition	7

6	Layer management	8

7	PHY service specification	9

8	Frame formats	10

8.1	General requirements	10

8.2	MAC frame formats	10

8.2.3	General frame format	10

8.2.4	Frame fields	10

8.2.4.1	Frame Control field	10

8.2.4.2	Duration/ID field	11

8.2.4.6	QMG Control field	11

8.3	Format of individual frame types	14

8.3.1	Control frames	14

8.3.1.10	Control Wrapper frame	14

8.3.2	Data frames	15

8.3.3	Management frames	15

8.3.3.1	Format of management frames	15

8.3.3.2	Beacon frame format	15

8.3.3.5	Association Request frame format	15

8.3.3.6	Association Response frame format	15

8.3.3.7	Reassociation Request frame format	15

8.3.3.8	Reassociation Response frame format	16

8.3.3.9	Probe Request frame format	16

8.3.3.10	Probe Response frame format	16

8.4	Management and Extension frames body components	16

8.4.1	Fields that are not elements	16

8.4.1.9	Status Code field	16

8.4.1.11	Action field	17

8.4.1.23	PCO Phase Control Field	17

8.4.1.26	QMG MIMO Control field	17

8.4.1.27	QMG CSI Report field	19

8.4.1.28	QMG Noncompressed Beamforming Report field	20

8.4.1.29	QMG Compressed Beamforming Report field	21

8.4.1.36	Band ID field	25

8.4.1.37	Operating Mode field	25

8.4.2	Information elements	26

8.4.2.1	General	26

8.4.2.161	QMG Capabilities element	27

8.4.2.162	QMG Operation element	36

8.4.2.163	Operating Mode Notification element	37

8.4.2.164	Extended BSS Load element	37

8.4.2.165	QMG Transmit Power Envelope	37

8.4.2.166	Channel Switch Wrapper	37

8.4.2.169	QMG Link Margin element	37

8.4.2.170 QMG Link Adaptation Acknowledgment element	38

8.5	Action frame format details	39

8.5.2	Spectrum management Action frames	39

8.5.12	QMG Action frame details	39

8.5.12.1	QMG Action field	39

8.5.12.3	SM Power Save frame format	39

8.5.12.7	QMG Set PCO Phase frame format	39

8.5.12.11	QMG CSI frame format	40

8.5.12.12	QMG Noncompressed Beamforming frame format	40

8.5.12.13	QMG Compressed Beamforming frame format	40

8.5.12.14	Operating Mode Notification frame format	41

9	MAC sublayer functional description	42

9.3.2.6	CTS and QMG CTS procedure	42

9.3.4	DCF access procedure	42

9.3.4.4	Recovery procedures and retransmit limits	42

9.7	Multirate Support	42

9.7.1	Overview	42

9.7.3	Basic STBC MCS	43

9.7.4	Basic Rate Set and Basic MCS Set for mesh STA	43

9.7.5	Rate selection for data and management frames	43

9.7.5.1	Rate selection for non-STBC Beacon	43

9.7.5.2	Rate selection for STBC group addressed data and management frames	43

9.7.5.3	Rate selection for other group addressed data and management frames	44

9.7.5.4	Rate selection for polling frames	44

9.7.5.6	Rate selection for other data and management frames	44

9.7.6	Rate selection for control frames	45

9.7.6.1	General rules for rate selection for control frames	45

9.7.6.2	Rate selection for control frames that initiate a TXOP	46

9.7.6.4	Rate selection for control frames that are not control response frames	46

9.7.6.5	Rate selection for control response frames	47

9.7.6.7	Control frame TXVECTOR parameter restrictions	49

9.7.7	Multiple BSSID Rate Selection	49

9.7.8	Modulation classes	49

9.7.11	Rate Selection Constraints for QMG STAS	50

9.7.11.1	Rx Supported QMG-MCS and NSS Set	50

9.7.11.2	Tx Supported QMG-MCS and NSS Set	50

9.9	QMG Control field operation	50

9.10	Control Wrapper operation	51

9.12	A-MSDU operation	51

9.14	PPDU duration constraint	52

9.15	STBC operation	52

9.16	Operation across regulatory domains	52

9.19	HCF	54

9.19.2	HCF contention based channel access (EDCA)	54

9.19.2.2	EDCA TXOPs	54

9.19.2.3	Obtaining an EDCA TXOP	55

9.19.2.4	Multiple frame transmission in an EDCA TXOP	55

9.19.2.8	EDCA channel access in a QMG BSS	55

9.24	Block Acknowledgment (Block Ack)	56

9.24.8	QMG delayed Block Ack extensions	56

9.25	Reverse Direction Protocol	56

9.26	PSMP operation	56

9.27	Sounding PPDUs	56

9.28	Link adaptation	57

9.28.1	Introduction	57

9.28.2	Link adaptation using the QMG Control field	57

9.28.3	Link adaptation using the QMG Link Measurement	61

9.28.3.1	General	61

9.28.3.2	QMG TPC	62

9.28.3.3	QMG Fast Link Adaptation	62

9.29	Transmit beamforming	63

9.29.1	General steering matrix calculations	63

9.29.2	Transmit beamforming with implicit feedback	63

9.29.3	Explicit feedback beamforming	64

9.30	Antenna selection (ASEL)	66

9.31	Null data packet (NDP) sounding	66

9.31.1	NDP rules	66

9.31.2	Transmission of a QMG NDP	67

9.37	DMG or QMG AP or PCP Cluster	68

9.37.1	General	68

9.37.2	Cluster formation	69

9.37.2.1	Decentralized AP or PCP cluster formation	69

9.37.2.2	Centralized AP or PCP cluster formation	70

9.37.3	Cluster maintenance	73

9.37.3.1	General cluster maintenance	73

9.37.3.2	Decentralized AP or PCP cluster maintenance	73

9.37.3.3	Centralized AP or PCP cluster maintenance	74

9.37.3.4	Centralized AP or PCP cluster MAC requirements	75

9.37.4	Cluster report and re-scheduling	75

9.37.5	Decentralized AP or PCP cluster request	77

10	MLME	78

10.1	Synchronization	78

10.2	Power management	78

10.2.1.9	Receive operation for STAs in PS mode during the CFP	78

10.2.1.19	QMG TXOP power save	78

10.3	STA authentication and association	79

10.8	TPC procedures	79

10.9	DFS procedures	80

10.11	Radio measurement procedures	80

10.16	Phased coexistence operation (PCO)	80

10.16.1	General description of PCO	80

10.16.2	Operation at a PCO active AP	81

10.16.3	Operation at a PCO active non-AP STA	82

10.22	Tunneled direct link setup	83

10.23	Quality-of-Service management frame (QMF)	83

10.33	Multi-band operation	83

10.33.1	General	83

10.34	MMSL cluster operation	84

10.40	QMG BSS operation	84

10.40.1	Basic QMG BSS functionality	84

10.40.2	Channel selection methods for a QMG BSS	84

10.40.3	Scanning requirements for QMG STA	85

10.40.4	Channel switching methods for a QMG BSS	85

10.40.5	NAV assertion in a QMG BSS	87

10.40.6	QMG STA antenna indication	88

10.40.7	BSS basic QMG-MCS and NSS set operation	88

10.41	Notification of operating mode changes	88

11	Security	89

13	MLME mesh procedures	90

26	QMG PHY specification	91

26.1	Introduction	91

26.1.1	Introduction to the PHY	91

26.1.2	Scope	91

26.1.3	QMG PHY functions	91

26.1.3.1	QMG PLCP sublayer	92

26.1.3.2	QMG PMD sublayer	92

26.1.3.3	PHY management entity (PLME)	92

26.1.3.4	Service specification method	92

26.1.4	QMG PHY functions	92

26.1.4.1	General	92

26.1.4.2	PHY management entity (PLME)	92

26.1.4.3	Service specification method	92

26.1.4.4	PPDU format	92

26.2	QMG PHY service interface	93

26.2.1	Introduction	93

26.2.2	TXVECTOR and RXVECTOR parameters	93

26.3	Common Parameters	97

26.3.1	Introduction	97

26.3.2	Common requirements	97

26.3.2.1	Introduction	97

26.3.2.2	Transmit RF Delay	97

26.3.2.3	Center frequency tolerance	97

26.3.3	Time-related parameters	98

26.3.4	Mathematical conventions in the signal description	100

26.3.4.1	Notation	100

26.3.4.2	Subcarrier indices in use	100

26.3.4.3	Transmitted signal	100

26.3.4.4	Definition of the tone rotation	102

26.3.4.5	The windowing function	102

26.3.5	QMG PHY Preamble	103

26.3.5.1	General	103

26.3.5.2	QMG Control PHY STF	103

26.3.5.3	QMG Control PHY CEF	103

26.3.5.4	QMG SC and OFDM PHY STF	104

26.3.5.5	QMG SC PHY CEF	104

26.3.5.6	QMG OFDM PHY CEF	105

26.3.6	CRC calculation	105

26.3.7	Scrambler	106

26.3.8	Common LDPC parity matrices	107

26.3.8.1	Introduction	107

26.3.8.2	General	107

26.3.8.3	Rate-1/2 LDPC code matrix	108

26.3.8.4	Rate-5/8 LDPC code matrix	108

26.3.8.5	Rate-3/4 LDPC code matrix	108

26.3.8.6	Rate-13/16 LDPC code matrix	108

26.3.9	QMG PHY SIG	108

26.3.9.1	General	108

26.3.9.2	Encoding of SIG	112

26.3.10	Transmission of the preamble and the SIG field on 1080 MHz channel	112

26.3.11	definition	113

26.3.12	Encoding of Data	113

26.3.13	Received channel power indicator (RCPI) measurement	115

26.4	QMG Control PHY	115

26.4.1	Introduction	115

26.4.2	Frame format	115

26.4.3	QMG Preamble Transmission	116

26.4.4	QMG Control PHY SIG Transmission	116

26.4.4.1	General	116

26.4.4.2	Generation of CRC bits	116

26.4.4.3	Scrambler	116

26.4.4.4	Encoding	116

26.4.4.5	Modulation	116

26.4.4.6	Spreading	116

26.4.5	QMG Control PHY Data field	117

26.4.5.1	General	117

26.4.5.2	Scrambler	117

26.4.5.3	Encoder	117

26.4.5.4	Modulation	117

26.4.5.5	Spreading	117

26.4.6	QMG Control PHY Performance requirements	117

26.4.6.1	Transmit requirements	117

26.4.6.2	Receive requirements	118

26.5	QMG SC PHY	118

26.5.1	Introduction	118

26.5.2	Transmitter block diagram	118

26.5.3	Overview of  QMG SC PHY PPDU encoding process	119

26.5.3.1	Construction of QMG SC PHY SIG	119

26.5.3.2	Construction of QMG SC PHY PPDU	119

26.5.4	QMG SC PHY PPDU Format	120

26.5.5	QMG SC PHY SIG Transmission	120

26.5.5.1	SIG Fields	120

26.5.5.2	Generation of the CRC bits of SIG	120

26.5.5.3	Scrambler	120

26.5.5.4	Encoding and modulations	120

26.5.6	Data Field Transmission	121

26.5.6.1	General	121

26.5.6.2	Scrambler	121

26.5.6.3	Encoding	121

26.5.6.4	Symbol Block Padding Zeros	121

26.5.6.5	Stream Parser	122

26.5.6.6	Constellation mapping	122

26.5.6.7	Spatial Expansion	125

26.5.7	Performance Requirements	128

26.5.7.1	Transmit Requirements	128

26.5.7.2	Receive Requirements	129

26.6	QMG OFDM PHY	129

26.6.1	Introduction	129

26.6.2	QMG OFDM PHY PPDU Format	129

26.6.3	Transmission Block Diagram	129

26.6.4	Overview of QMG OFDM PHY PPDU encoding process	130

26.6.4.1	Construction of QMG OFDM PHY SIG	130

26.6.4.2	Construction of QMG OFDM PHY PPDU	131

26.6.4.3	Construction of OSTF	131

26.6.4.4	Construction of OCEF	132

26.6.5	QMG OFDM PHY SIG Fields	132

26.6.5.1	SIG Fields	132

26.6.5.2	Modulation	132

26.6.5.3	Sample rate change	132

26.6.6	OSTF definition	133

26.6.7	OCEF Definition	133

26.6.8	Data Fields	136

26.6.8.1	General	136

26.6.8.2	Scrambler	136

26.6.8.3	Encoding	136

26.6.8.4	OFDM symbol padding zeros	136

26.6.8.5	Stream Parser	136

26.6.8.6	Subcarrier modulation mapping	136

26.6.8.7	Tone Mapping	139

26.6.8.8	Space-time block coding	140

26.6.8.9	Pilot Subcarriers	141

26.6.8.10	Cyclic shift diversity (CSD) for OFDM PHY data fields Transmissioin	141

26.6.8.11	OFDM Modulation	141

26.6.8.12	Beamforming	143

26.6.8.13	QMG Preamble format for sounding PPDUs	144

26.6.9	Performance requirements	145

26.6.9.1	Transmit requirements	145

26.6.9.2	TX flatness	147

26.6.9.3	Receive requirements	147

26.7	Analog Beamforming PHY frame format	147

26.7.1	TX sector sweep	148

26.7.2	Beam refinement	148

26.7.2.1	General	148

26.7.2.2	BRP packet structure	148

26.7.2.3	BRP packet SIG fields	148

26.7.2.4	Beam refinement AGC field	148

26.7.2.5	Beam refinement TRN-R field	149

26.7.2.6	Beam refinement TRN-T field	149

26.7.2.7	Channel measurement	150

26.7.2.8	BRP resampling in an OFDM packet	150

26.8	ZCZ sequence	150

26.9	Regulatory requirements	155

26.10	Channelization	155

26.11	Transmit spectrum mask	156

26.12	PHY Transmit procedure	156

26.12.1	PHY transmit procedure for an SC transmission	156

26.12.2	PHY transmit procedure for an OFDM transmission	157

26.13	Receive procedure	160

26.13.1	SC Receive procedure	160

26.13.2	OFDM receiver procedure	160

26.14	QMG PLME	163

26.14.1	PLME SAP sublayer management primitive	163

26.14.2	PHY MIB	164

26.14.3	TXTIME calculation	165

26.14.4	PHY characteristics	165

26.15	Parameters for QMG-MCSs	166

26.15.1	Parameters for QMG-MCSs with SC Transimission	166

26.15.2	Parameters for QMG-MCSs with OFDM Transimission	168

26.16	QMG PMD sublayer	171

26.16.1	Scope and field of application	171

26.16.2	Overview of interactions	171

26.16.3	Overview of interactions	171

26.16.4	Basic service and options	171

26.16.4.1	Status of service primitives	171

26.16.4.2	PMD_SAP peer-to-peer service primitives	172

26.16.4.3	PMD_SAP sublayer-to-sublayer service primitives	172

26.16.5	PMD_SAP detailed service specification	172

26.16.5.1	Introduction to PMD_SAP service specification	172

26.16.5.2	PMD_DATA.request	172

26.16.5.3	PMD_DATA.indication	172

26.16.5.4	PMD_TXSTART.request	173

26.16.5.5	PMD_TXEND.request	173

26.16.5.6	PMD_TXEND.confirm	173

26.16.5.7	PMD_TXPWRLVL.request	174

26.16.5.8	PMD_TX_PARAMETERS.request	174

26.16.5.9	PMD_RSSI.indication	174

26.16.5.10	PMD_RCPI.indication	175

26.16.5.11	PMD_CHAN_MAT.indication	175

# Overview

# Normative references

# Definitions, acronyms and abbreviations

## Definitions

## Definitions specific to IEEE 802.11

Destination Q-band millimeter-wave multiple gigabit (QMG) station (STA): A QMG STA identified by the destination association identifier (AID) field contained in a Grant frame or Extended Schedule element that caused the allocation of a service period (SP) or a contention based access period (CBAP).

Q-band millimeter-wave multiple gigabit (QMG): Pertaining to operation in a frequency band containing a channel with the channel starting frequency above 45GHz.

Q-band millimeter-wave multiple gigabit (QMG) access point (AP): An AP whose radio transmitter is capable of transmitting and receiving QMG physical layer (PHY) protocol data units (PPDUs).

Q-band millimeter-wave multiple gigabit (QMG) basic service set (BSS): A BSS in which QMG Beacon frames are transmitted by QMG stations (STAs).

Q-band millimeter-wave multiple gigabit (QMG) frame: A frame transmitted or received within a QMG physical layer (PHY) protocol data unit (PPDU).

Q-band millimeter-wave multiple gigabit (QMG) physical layer (PHY) protocol data unit (PPDU): A Clause 26 (Q-band millimeter-wave multiple gigabit (QMG) PHY specification) PPDU transmitted or received using the Clause 26 (Q-band millimeter-wave multiple gigabit (QMG) PHY specification) physical layer (PHY).

Q-band millimeter-wave multiple gigabit (QMG) station (STA): A STA whose radio transmitter is capable of transmitting and receiving QMG physical layer (PHY) protocol data units (PPDUs).

Q-band millimeter-wave multiple gigabit (QMG) beamformee: A QMG station (STA) that receives a QMG physical layer (PHY) protocol data unit (PPDU) that was transmitted using a beamforming steering matrix and that supports the QMG transmit beamforming feedback mechanism as described in 9.34.5 (QMG sounding protocol).

Q-band millimeter-wave multiple gigabit (QMG) beamformer: A QMG station (STA) that transmits a QMG physical layer (PHY) protocol data unit (PPDU) using a beamforming steering matrix.

Q-band millimeter-wave multiple gigabit modulation and coding scheme (QMG-MCS): A specification of the QMG physical layer (PHY) parameters that consists of modulation order (e.g., BPSK, QPSK, 16-QAM, 64-QAM) and forward error correction (FEC) coding rate (e.g., 1/2, 2/3, 3/4, 5/6) and that is used in a QMG PHY protocol data unit (PPDU).

Q-band millimeter-wave multiple gigabit (QMG) single medium access control (MAC) protocol data unit (QMG single MPDU): An MPDU that is the only MPDU in an aggregate MPDU (A-MPDU) carried in a QMG physical layer (PHY) protocol data unit (PPDU) and that is carried in an A-MPDU subframe with the EOF subfield of the MPDU delimiter field equal to 1.

## Abbreviations and acronyms

Insert the following abbreviations into 3.3 in alphabetical order:

CEF: Channel Estimation Field

OCEF: OFDM Channel Estimation Field

QMG: Q-band Millimeter-wave Multiple Gigabit.

QMG MIMO: QMG Multiple Input and Multiple Output.

STF: Short Training Field

SCEF: Single carrier Channel Estimation Field

OSTF: OFDM Short Training Field

UW: Unique Words

ZCZ: Zero Correlation Zone (ZCZ) Sequence

# General description

# MAC service definition

# Layer management

# PHY service specification

# Frame formats

## General requirements

The format of the MAC frames is specified in this clause. A STA shall be able properly to construct a subset of the frames specified in this clause for transmission and to decode a (potentially different) subset of the frames specified in this clause upon validation following reception. The particular subset of these frames that a STA constructs and decodes is determined by the functions supported by that particular STA. All STAs shall be able to validate every received frame using the frame check sequence (FCS) and to interpret certain fields from the MAC headers of all frames.

A compliant STA shall transmit frames using only the frame formats described in Clause 8.

## MAC frame formats

### General frame format

Change the second paragraph of 8.2.3, and insert Figure 8-1a as indicated below:

The Frame Body field is of variable size. The maximum frame body size is determined by the maximum MSDU size (2304 octets) or the maximum A-MSDU size (3839 or 7935 octets, depending upon the STA’s capability), plus any overhead from security encapsulation.

Figure 8-1a MAC frame format

### Frame fields

#### Frame Control field

##### Type and Subtype fields

##### More Data field

Insert the following paragraph after the 13th paragraph of 8.2.4.1.8:

The More Data field is set to 1 in individually addressed frames transmitted by a QMG AP to a QMG STA when both support the TXOP power save feature (as determined from their QMG Capabilities elements) to indicate that at least one additional buffered BU is present for the STA. See 10.2.1.19.

##### Order field

#### Duration/ID field

#### QMG Control field

The QMG Control field is always present in a Control Wrapper frame and is present in QoS Data and Management frames as determined by the Order bit of the Frame Control field as defined in 8.2.4.1.10 (Order field).

NOTE 1—The only Control frame subtype for which QMG Control field is present is the Control Wrapper frame. A Control frame that is described as +QMGC (e.g., an RTS+QMGC, CTS+QMGC, BlockAck+QMGC or BlockAckReq+QMGC frame) implies the use of the Control Wrapper frame to carry that Control frame.

The format of the 4-octet QMG Control field is shown in Figure 8-7 (QMG Control field).

Figure 8-5a—QMG Control field

The link adaption fields include the fisrt six subfileds that are defined in Table 8-7a.

Table 8-7a—Subfields corresponding to link adaption

| Subfield | Meaning | Definition |
| --- | --- | --- |
| MRQ | QMG-MCS Feedback request | Set to 1 to request QMG-MCS feedback (solicited MFB); otherwise, set to 0. |
| MSI/STBC | MRQ sequence identifier/STBC indication | If the Unsolicited MFB subfield is 0 and the MRQ subfield is 1, the MSI/STBC subfield contains a sequence number in the range 0 to 6 that identifies the specific MCS feedback request. If the Unsolicited MFB subfield is 0 and the MRQ subfield is 0, the MSI/STBC subfield is reserved. If the Unsolicited MFB subfield is 1 and the MFB does not contain the value representing “no feedback is present,” the MSI/STBC field contains the Compressed MSI and STBC Indication subfields as shown in Figure 8-7a (MSI/STBC subfield when the Unsolicited MFB subfield is 1). The STBC Indication subfield indicates whether the estimate in the MFB subfield is computed based on a PPDU using STBC encoding: Set to 0 if the PPDU was not STBC encoded Set to 1 if the PPDU was STBC encoded The Compressed MSI subfield contains a sequence number that identifies the specific MCS feedback request. It is in the range 0 to 3 if STBC Indication equals 0 or in the range 0 to 2 if STBC Indication equals 1. Otherwise, the MSI/STBC subfield is reserved. |
| MFSI | MFB sequence identifier/LSBs of group ID | Set to the received value of MSI contained in the frame to which the MFB information refers. Set to 7 for unsolicited MFB. |
| MFB | NUM_STS, QMG-MCS, BW and SNR feedback | MFB subfield is interpreted as defined in Table 8-8a (MFB subfield in the QMG Control field). This subfield contains the recommended MFB. The combination of QMG-MCS=31 and NUM_STS=3 indicates that no feedback is present. |
| FB Tx Type | Transmission type of the measured PPDU | If the Unsolicited MFB subfield is 1, the MFB does not contain the value representing “no feedback is present,” and FB Tx Type subfield is 0, then the unsolicited MFB is estimated from a QMG PPDU with RXVECTOR parameter BEAMFORMED equal to 0. If the Unsolicited MFB subfield is 1, the MFB does not contain the value representing “no feedback is present,” and the FB Tx Type subfield is 1, then the unsolicited MFB is estimated from a QMG PPDU with RXVECTOR parameter BEAMFORMED equal to 1. Otherwise, this subfield is reserved. |
| Unsolicited MFB | Unsolicited QMG-MCS feedback indicator | Set to 1 if the MFB is not a response to an MRQ. Set to 0 if the MFB is a response to an MRQ. |

The format of the MSI/STBC subfield when the Unsolicited MFB subfield is 1 is shown in Figure 8-7a (MSI/STBC subfield when the Unsolicited MFB subfield is 1).

Figure 8-7a—MSI/STBC subfield when the Unsolicited MFB subfield is 1

The format of the MFB subfield in the QMG Control field is shown in Figure 8-7b (MFB subfield in the QMG Control field).

Figure 8-7b—MFB subfield in the QMG Control field

The subfields of the MFB subfield in the QMG Control field are defined in Table 8-8a (MFB subfield in the QMG Control field).

Table 8-8a—MFB subfield in the QMG Control field

| Subfield | Meaning | Definition |
| --- | --- | --- |
| NUM_STS | Recommended NUM_STS | Indicates the recommended NUM_STS as defined in 9.25 (Link adaptation). The NUM_STS subfield contains an unsigned integer representing the number of space-time streams minus 1. |
| QMG-MCS | Recommended QMG-MCS | Indicates the recommended QMG-MCS as defined in 9.25 (Link adaptation). The QMG-MCS subfield contains an unsigned integer in the range 0 to 31 representing a QMG-MCS Index value (defined in 1.6.8.6 (Parameters for QMG -MCSs)). |
| BW | Bandwidth of the recommended QMG-MCS | If the Unsolicited MFB subfield is 1, the BW subfield indicates the bandwidth for which the recommended QMG-MCS is intended, as defined in 9.25 (Link adaptation): For a QMG STA: Set to 0 for 540 MHz Set to 1 for 1080 MHz. If the Unsolicited MFB subfield is 0, the BW subfield is reserved. |
| SNR | Average SNR | Indicates the average SNR, which is an SNR averaged over data subcarriers and space-time streams. The SNR is averaged over all the space-time streams and data subcarriers and is encoded as a 6-bit 2s complement number of SNR_average – 22, where SNR_average is the sum of the values of SNR per frequency tone (in decibels) per space-time stream divided by the product of the number of space-time streams, as indicated in the NUM_STS subfield, and the number of frequency tones represented in the bandwidth in which the MFB was estimated. This encoding covers the SNR range from –10 dB to 53 dB in 1 dB steps. |

The CSI/Steering subfield of the QMG Control field indicates the type of feedback, as shown in Table 8-11 (CSI/Steering subfield values).

Table 8-11a CSI/Steering subfield values

| Value | Definition |
| --- | --- |
| 0 | No feedback required |
| 1 | CSI |
| 2 | Noncompressed beamforming |
| 3 | Compressed beamforming |

The QMG NDP Announcement subfield of the QMG Control field indicates that an NDP will be transmitted after the frame (according to the rules described in 9.31 (Null data packet (NDP) sounding)). It is set to 1 to indicate that an NDP will follow; otherwise, it is set to 0.

The AC Constraint subfield of the QMG Control field indicates whether the the mapped AC of an RD Data frame is constrained to a single AC, as defined in Table 8-11b (AC Constraint subfield values).

Table 8-11b—AC Constraint subfield values

| Value | Description |
| --- | --- |
| 0 | The response to a reverse direction grant (RDG) may contain Data frames from any TID |
| 1 | The response to a RDG may contain Data frames only from the same AC as the last Data frame received from the RD initiator |

NOTE 2—The AC of the last Data frame received from the RD initiator is determined directly from the TID of the received frame if the TID is between 0 and 7 inclusive or from the UP field of the TSPEC identified by the TID of the received frame if the TID is between 8 and 15 inclusive.

The RDG/More PPDU subfield of the QMG Control field is interpreted differently depending on whether it is transmitted by an RD initiator or an RD responder, as defined in Table 8-13a (RDG/More PPDU subfield values).

Table 8-13a—RDG/More PPDU subfield values

| Value | Role of transmitting STA | Interpretation of value |
| --- | --- | --- |
| 0 | RD initiator | No reverse grant |
|  | RD responder | The PPDU carrying the frame is the last transmission by the RD responder |
| 1 | RD initiator | An RDG is present, as defined by the Duration/ID field |
|  | RD responder | The PPDU carrying the frame is followed by another PPDU |

## Format of individual frame types

### Control frames

#### Control Wrapper frame

Change the Figure 8-29 as follows.

The format of the Control Wrapper frame is shown in Figure 8-29a.

Figure 8-29a Control Wrapper frame

Change the second paragraph as follows.

The Control Wrapper frame is used to carry any other control frame (i.e., excluding the Control Wrapper frame) together with a QMG Control field.

Insert the following sentence after the seventh paragraph:

The QMG Control field is defined in 8.2.4.6.

### Data frames

### Management frames

#### Format of management frames

In Figure 8-34 (Management frame format), change the octet range for Frame Body from 0–2320 to 0–2328.

#### Beacon frame format

Insert the following rows into Table 8-20 before the Last row:

Table 8-20 Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 66 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |
| 67 | QMG Operation | The QMG Operation element is present when the dot11QMGOptionImplemented is true; otherwise, it is not present. |

#### Association Request frame format

Insert the following rows into Table 8-22 before the Last row:

Table 8-22 Association Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 24 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |

#### Association Response frame format

Insert the following rows into Table 8-23 before the Last row:

Table 8-23 Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 30 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |
| 31 | QMG Operation | The QMG Operation element is present when the dot11QMGOptionImplemented is true; otherwise, it is not present. |

#### Reassociation Request frame format

Insert the following rows into Table 8-24 before the Last row:

Table 8-24 Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 29 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |

#### Reassociation Response frame format

Insert the following rows into Table 8-25 before the Last row:

Table 8-25 Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 34 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |
| 35 | QMG Operation | The QMG Operation element is present when the dot11QMGOptionImplemented is true; otherwise, it is not present. |

#### Probe Request frame format

Insert the following rows into Table 8-26 before the Last row:

Table 8-26 Probe Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 18 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |

#### Probe Response frame format

Insert the following rows into Table 8-27 before the Last row:

Table 8-27 Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 68 | QMG Capabilities | The QMG Capabilities element is present when the dot11QMGOptionImplemented is true. |
| 69 | QMG Operation | The QMG Operation element is present when the dot11QMGOptionImplemented is true; otherwise, it is not present. |

## Management and Extension frames body components

### Fields that are not elements

#### Status Code field

Insert the following rows into Table 8-37 before the Last row:

Table 8-37 Status codes

| Order | Information | Notes |
| --- | --- | --- |
| 108 |  | Association denied because the requesting STA does not support QMG features. |

#### Action field

Insert the following row into Table 8-38 in numeric order, and change the reserved values accordingly:

Table 8-38—Category values

| Code | Meaning | See subclause | Robust | Group Addressed privacy | Action frame |
| --- | --- | --- | --- | --- | --- |
| 22 | QMG | 8.5.23 | No | No |  |

#### PCO Phase Control Field

Change 8.4.1.23 as follows:

The PCO Phase Control field is used in a QMG Set PCO Phase frame (see 8.5.12.5) to indicate the phase of PCO operation (see 10.16). The length of the field is 1 octet. The PCO Phase Control field is illustrated in Figure 8-58.

Figure 8-58—PCO Phase Control fixed field

The PCO Phase Control field indicates the current PCO phase. The values of the PCO Phase Control field are defined in Table 8-41.

Table 8-41—Settings of the PCO Phase Control field

| Value | Meaning |
| --- | --- |
| 0 | 540MHz Phase |
| 1 | 1080MHz Phase |
| 2-255 | Reserved |

#### QMG MIMO Control field

Change 8.4.1.26 as follows:

The QMG MIMO Control field is used to manage the exchange of QMG MIMO channel state or transmit beamforming feedback information. It is used in the QMG CSI (see 8.5.12.1), QMG Noncompressed Beamforming (see 8.5.12.2), and QMG Compressed Beamforming (see 8.5.12.3) frames.

The QMG MIMO Control is 5 octets in length and is defined in Figure 8-62a (QMG MIMO Control field).

Figure 8-62a—QMG MIMO Control field

The subfields of the QMG MIMO Control field are defined in Table 8-42a (Subfields of the QMG MIMO Control field).

Table 8-42a—Subfields of the QMG MIMO Control field

| Subfield | Description |
| --- | --- |
| Nc Index | Indicates the number of columns in a matrix minus 1: Set to 0 for Nc = 1 Set to 1 for Nc = 2 Set to 2 for Nc = 3 Set to 3 for Nc = 4 |
| Nr Index | Indicates the number of rows in a matrix minus 1: Set to 0 for Nr = 1 Set to 1 for Nr = 2 Set to 2 for Nr = 3 Set to 3 for Nr = 4 |
| Channel Width | Indicates the width of the channel in which a measurement was made: Set to 0 for 540 MHz Set to 1 for 1080 MHz |
| Grouping | Indicates the number of subcarriers grouped into one: Set to 0 for Ng = 1 (No grouping) Set to 1 for Ng = 2 Set to 2 for Ng = 4 Set to 3 for Ng = 6 |
| Coefficient Size | Indicates the number of bits in the representation of the real and imaginary parts of each element in the matrix. For QMG CSI feedback: Set to 0 for Nb=4 Set to 1 for Nb=5 Set to 2 for Nb=6 Set to 3 for Nb=8 For QMG Noncompressed Beamforming feedback: Set to 0 for Nb=4 Set to 1 for Nb=5 Set to 2 for Nb=6 Set to 3 for Nb=8 |
| Codebook Information | Indicates the size of codebook entries： Set to 0 for 2 bits for ψ, 4 bits for ϕ Set to 1 for 3 bits for ψ, 5 bits for ϕ The value 2 and 3 are reserved. |
| Remaining Matrix Segments | Contains the remaining segment number for the associated measurement report. Valid range: 0 to 7. Set to 0 for the last segment of a segmented report or the only segment of an unsegmented report. |
| Sounding Timestamp | Contains the lower 3 octets of the TSF timer value sampled at the instant that the MAC received the PHY-CCA.indication(IDLE) primitive that corresponds to the end of the reception of the sounding packet that was used to generate feedback information contained in the frame. |

#### QMG CSI Report field

The QMG CSI Report field is used by the QMG CSI frame (see 8.5.12.1) to carry explicit channel state information to a QMG transmit beamformer, as described in 9.26.3 (Explicit feedback beamforming).

The CSI Matrix subfields in the QMG CSI Report field shown in Table 8-59 (QMG CSI Report field) are matrices whose elements are taken from the CHAN_MAT parameter of RXVECTOR (see Table 20-1 (TXVECTOR and RXVECTOR parameters)).

Table 8-43a—QMG CSI Report field

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| SNR in receive chain 1 | 8 | Signal-to-noise ratio in the first receive chainof the STA sending the report |
| … | … | … |
| SNR in receive chain Nr | 8 | Signal-to-noise ratio in the Nr-th receive chain of the STA sending the report |
| CSI Matrix for carrier -NSR | 3+2×Nb×Nr×Nc | CSI Matrix |
| … |  |  |
| CSI Matrix for carrier -2 | 3+2×Nb×Nr×Nc | CSI Matrix |
| CSI Matrix for carrier 2 | 3+2×Nb×Nr×Nc | CSI Matrix |
| … |  |  |
| CSI Matrix for carrier NSR | 3+2×Nb×Nr×Nc | CSI Matrix |

The structure of the field depends on the value of the QMG MIMO Control Channel Width subfield. In Table 8-59,

Nb is the number of bits determined by the Codebook Information field of the QMG MIMO Control field

Nc is the number of columns in a CSI matrix determined by the Nc Index field of the QMG MIMO Control field

NSR is the highest data subcarrier index per OFDM symbol (see Table 26-3).

The signal-to-noise ratio (SNR) values in Table 8-59 (QMG CSI Report field) are encoded as an 8-bit 2s complement value of 4 × (SNR_average – 22), where SNR_average is the decibel representation of linearly averaged values over the tones represented. This encoding covers the SNR range from –10 dB to 53.75 dB in 0.25 dB steps.

Grouping is a method that reduces the size of the QMG CSI Report field by reporting a single value for each group of Ng adjacent subcarriers. With grouping, the size of the CSI Report field is Nr×8+Ns×(3+2×Nb×Nc×Nr) bits, where the number of subcarriers sent, Ns, is a function of Ng and whether matrices for 540 MHz or 1080 MHz are sent. The value of Ns and the specific carriers for which matrices are sent are shown in Table 8-67 (Number of matrices and carrier grouping). If the size of the QMG CSI Report field is not an integral multiple of 8 bits, up to 7 0s are appended to the end of the report to make its size an integral multiple of 8 bits.

The CSI matrix Heff for a single carrier has the structure defined in Figure 8-63a (CSI matrix coding). The encoding rules for the elements of the Heff matrix are given in 1.6.8.12.2 (CSI matrices feedback).

For each subcarrier include { Carrier Matrix Amplitude of 3 bits For each of Nr rows in each CSI matrix in order: (1,…,Nr) { Include Nc complex coefficients of CSI matrix Heff in order: (1,…,Nc); each element of Heff includes the real part of the element (Nb bits) and imaginary part of the element (Nb bits) in that order. } }

Figure 8-63a—CSI matrix coding

#### QMG Noncompressed Beamforming Report field

The QMG Noncompressed Beamforming Report field is used by the QMG Noncompressed Beamforming frame (see 8.5.12.2) to carry explicit feedback in the form of noncompressed beamforming feedback matrices V for use by a transmit QMG beamformer to determine steering matrices Q, as described in 9.26.3 (Explicit feedback beamforming) and 1.6.8.12 .

Table 8-46a—QMG Noncompressed Beamforming Report field

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| SNR for space-time stream 1 | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream 1. |
| … |  |  |
| SNR for space-time stream Nc | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream Nc. |
| Beamforming Feedback Matrix for subcarrier -NSR | 2×Nb×Nr×Nc | Beamforming feedback matrix V |
| … |  |  |
| Beamforming Feedback Matrix for subcarrier -2 | 2×Nb×Nr×Nc | Beamforming feedback matrix V |
| Beamforming Feedback Matrix for subcarrier 2 | 2×Nb×Nr×Nc | Beamforming feedback matrix V |
| … |  |  |
| Beamforming Feedback Matrix for subcarrier NSR | 2×Nb×Nr×Nc | Beamforming feedback matrix V |

The structure of the field depends on the value of the QMG MIMO Control Channel Width subfield. In Table 8-62,

Nb is the number of bits determined by the Codebook Information field of the QMG MIMO Control field

Nc is the number of columns in a CSI matrix determined by the Nc Index field of the QMG MIMO Control field

NSR is the highest data subcarrier index per OFDM symbol (see Table 26-3).

The SNR values in Table 8-62 (Noncompressed Beamforming Report field) are encoded as an 8-bit 2s complement value of 4 ×(SNR_average – 22), where SNR_average is the sum of the values of SNR per tone (in decibels) divided by the number of tones represented. This encoding covers the SNR range from –10 dB to 53.75 dB in 0.25 dB steps. The SNR in space-time stream i corresponds to the SNR associated with the column i of the beamforming feedback matrix V. Each SNR corresponds to the predicted SNR at the QMG beamformee when the QMG beamformer applies the matrix V.

Grouping is a method that reduces the size of the QMG Noncompressed Beamforming Report field by reporting a single value for each group of Ng adjacent subcarriers. With grouping, the size of the QMG Noncompressed Beamforming Report field is Nc×8+Ns×(2×Nb×Nc×Nr) bits. The number of subcarriers sent, Ns, is a function of Ng and whether matrices for 540 MHz or 1080 MHz are sent. The value of Ns and the specific carriers for which matrices are sent is shown in Table 8-67 (Number of matrices and carrier grouping). If the size of the QMG Noncompressed Beamforming Report field is not an integral multiple of 8 bits, up to 7 0s are appended to the end of the report to make its size an integral multiple of 8 bits.

A beamforming feedback matrix V for a single carrier has the structure defined in Figure 8-64a (V matrix coding (noncompressed beamforming)).

Encoding rules for elements of the V matrix are given in 1.6.8.12.3 (Noncompressed beamforming feedback matrix).

For each subcarrier include { For each of Nr rows in the noncompressed beamforming feedback matrix in order: (1,…,Nr) { Include Nc complex coefficients of the noncompressed beamforming feedback matrix V in order: (1,…,Nc); each element of V includes the real part of the element (Nb bits) and imaginary part of the element (Nb bits) in that order. } }

Figure 8-64a—V matrix coding (noncompressed beamforming)

#### QMG Compressed Beamforming Report field

The QMG Compressed Beamforming Report field is used by the QMG Compressed Beamforming frame (see8.5.12.3) to carry explicit feedback information in the form of angles representing compressed beamforming feedback matrices V for use by a transmit QMG beamformer to determine steering matrices Q, as described in 9.26.3 (Explicit feedback beamforming) and 1.6.8.12 (SU-MIMO beamforming).

The size of the QMG Compressed Beamforming Report field depends on the values in the QMG MIMO Control field.

The QMG Compressed Beamforming Report field contains the channel matrix elements indexed, first, by matrix angles in the order shown in Table 8-48a (Order of angles in the Compressed Beamforming Report field) and, second, by data subcarrier index from lowest frequency to highest frequency. The explanation on how these angles are generated from the beamforming feedback matrix V is given in 1.6.8.12.4 (Compressed beamforming feedback matrix).

Table 8-48a—Order of angles in the Compressed Beamforming Report field

| Size of V (Nr×Nc) | Number of angles (Na) | The order of angles in the Compressed Beamforming Feedback Matrix subfield |
| --- | --- | --- |
|  | 2 | ， |
|  | 2 | ， |
|  | 4 | ,，， |
|  | 6 | ，，，，， |
|  | 6 | ，，，，， |
|  | 6 | ，，，，， |
|  | 10 | ，，，，，，，，， |
|  | 12 | ，，，，，，，，，，， |
|  | 12 | ，，，，，，，，，，， |

The angles are quantized as defined in Table 8-49a (Quantization of angles). All angles are transmitted LSB to MSB.

Table 8-49a—Quantization of angles

| Quantized | Quantized |
| --- | --- |
| radians where is the number of bits used to quantize defined by the Codebook Information field of the QMG MIMO Control field | radians where is the number of bits used to quantize defined by the Codebook Information field of the QMG MIMO Control field |

The Compressed Beamforming Report information has the structure and order defined in Table 8-66, whereis the number of angles used for the QMG compressed beamforming feedback matrix subfield.

Table 8-50a—Compressed Beamforming Report information

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| Average SNR of Space-Time Stream 1 | 8 | Signal-to-noise ratio at the beamformee for space-time stream 1 averaged over all data subcarriers |
| ... | ... | ... |
| Average SNR of Space-Time Stream | 8 | Signal-to-noise ratio at the beamformee for space-time streamaveraged over all data subcarriers. |
| Compressed Beamforming Feedback Matrix V for subcarrier |  | Compressed beamforming feedbackmatrix |
| Compressed Beamforming Feedback Matrix V for subcarrier |  | Compressed beamforming feedbackmatrix |
| Compressed Beamforming Feedback Matrix V for subcarrier |  | Compressed beamforming feedbackmatrix |
| ... | ... | ... |
| Compressed Beamforming Feedback Matrix V for subcarrier |  | Compressed beamforming feedbackmatrix |
| NOTE—scidx() is defined in Table 8-65a. |  |  |

Ns is the number of subcarriers for which the QMG Compressed Beamforming Feedback Matrix subfield is sent back to the QMG beamformer. A QMG beamformee may choose to reduce Ns by using a method referred to as grouping, in which only a single Compressed Beamforming Feedback Matrix is reported for each group of Ng adjacent subcarriers. Ns is a function of the Channel Width and Grouping subfields in the QMG MIMO Control field. Table 8-5a lists Ns, the exact subcarrier indices and their order for which the Matrix subfield is sent back. No padding is present between angles in the QMG Compressed Beamforming Report information, even if they correspond to different subcarriers. If the size of the QMG Compressed Beamforming Report information is not an integral multiple of 8 bits, up to seven zeros are appended to the end of the field to make its size an integral multiple of 8 bits.

Table 8-65a—Subcarriers for which a Matrix subfield is sent back

| Channel width | Ng | Ns | Subcarriers for which Matrix subfield is sent: scidx(0), scidx(1), …, scidx(Ns–1) |
| --- | --- | --- | --- |
| 540 MHz | 1 | 168 | –89, –88, –87, –86, –85, –84, –83, –82, –81, –80, –79, –78, –76, –75, –74, –73, –72, –71, –70, –69, –68, –67, –66, –65, –64, –63, –62, –61, –60, –59, –58, –57, –56, –54, –53, –52, –51, –50, –49, –48, –47, –46, –45, –44, –43, –42, –41, –40, –39, –38, –37, –36, –35, –34, –32, –31, –30, –29, –28, –27, –26, –25, –24, –23, –22, –21, –20, –19, –18, –17, –16, –15, –14, –13, –12, –10, –9, –8, –7, –6, –5, –4, –3, –2, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 78, 79, 80, 81, 82, 83, 84, 85, 86, 87, 88, 89 NOTE—Pilot subcarriers (±77, ±55, ±33, ±11) and DC subcarriers (0, ±1) are skipped |
|  | 2 | 90 | –89, –88, –86, –84, –82, –80, –78, –76, –74, –72, –70, –68, –66, –64, –62, –60, –58, –56 ,–54, –52, –50, –48, –46, –44, –42, –40, –38, –36, –34, –32, –30, –28, –26, –24, –22, –20, –18, –16, –14, –12, –10, –8, –6, –4, –2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 89 |
|  | 4 | 46 | –89, –86, –82, –78, –74, –70, –66, –62, –58, –54, –50, –46, –42, –38, –34, –30, –26, –22, –18, –14, –10, –6, –2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 89 |
|  | 6 | 32 | –89, –86, –80, –74, –68, –62, –56, –50, –44, –38, 32, –26, –20, –14, –8, –2, 2, 8, 14, 20, 26, 32, 38, 44, 50, 56, 62, 68, 74, 80, 86, 89 |
| 1080 MHz | 1 | 336 | –177, –176, –175, –174, –173, –172, –171, –170, –169, –168, –167, –166, –164, –163, –162, –161, –160, –159, –158, –157, –156, –155, –154, –153, –152, –151, –150, –149, –148, –147, –146, –145, –144, –142, –141, –140, –139, –138, –137, –136, –135, –134, –133, –132, –131, –130, –126, –125, –124, –123, –122, –120, –119, –118, –117, –116, –115, –114, –113, –112, –111, –110, –109, –108, –107, –106, –105, –104, –103, –102, –101, –100, –98, –97, –96, –95, –94, –93, –92, –91, –90, –89, –88, –87, –86, –85, –84, –83, –82, –81, –80, –79, –78, –76, –75, –74, –73, –72, –71, –70, –69, –68, –67, –66, –65, –64, –63, –62, –61, –60, –59, –58, –57, –56, –54, –53, –52, –51, –50,–49, –48, –47, –46, –45, –44, –43, –42, –41, –40, –39, –38, –37, –36, –35, –34, –32, –31, –30, –29, –28, –27, –26, –25, –24, –23, –22, –21, –20, –19, –18, –17,–16, –15, –14, –13, –12, –10, –9, –8, –7, –6, –5, –4, –3, –2, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 56, 57, 58, 59, 60, 61,62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 78, 79, 80, 81, 82, 83,84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97, 98, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, 120, 122, 123, 124, 125, 126, 130, 131, 132, 133, 134, 135, 136, 137, 138, 139, 140, 141, 142, 144, 145, 146, 147, 148, 149, 150, 151, 152, 153, 154, 155, 156, 157, 158, 159, 160, 161, 162, 163, 164, 166, 167, 168, 169, 170, 171, 172, 173, 174, 175, 176, 177 NOTE—Pilot subcarriers (±165, ±143, ±121, ±99, ±77, ±55, ±33, ±11) and DC subcarriers (0, ±1) are skipped |
|  | 2 | 170 | –177, –176, –174, –172, –170, –168, –166, –164, –162, –160, –158, –156, –154, –152, –150, –148, –146, –144, –142, –140, –138, –136, –134, –132, –130, –128, –126, –124, –122, –120, –118, –116, –114, –112, –110, –108, –106, –104, –102, –100, –98, –96, –94, –92, –90, –88, –86, –84, –82, –80, –78, –76, –74, –72, –70, –68, –66, –64, –62, –60, –58, –56, –54, –52, –50, –48, –46, –44, –42, –40, –38, –36, –34, –32, –30, –28, –26, –24, –22, –20, –18, –16, –14, –12, –10, –8, –6, –4, –2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 90, 92, 94, 96, 98, 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122, 124, 126, 128, 130, 132,134, 136, 138, 140, 142, 144, 146, 148, 150, 152, 154, 156, 158, 160, 162, 164, 166, 168, 170, 172, 174, 176, 177 |
|  | 4 | 90 | –177, –174, –170, –166, –162, –158, –154, –150, –146, –142, –138, –134, –130, –126, –122, –118, –114, –110, –106, –102, –98, –94, –90, –86, –82, –78, –74, –70, –66, –62, –58, –54, –50, –46, –42, –38, –34, –30, –26, –22, –18, –14, –10, –6, –2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102, 106, 110, 114, 118, 122, 126, 130, 134, 138, 142, 146, 150, 154, 158, 162, 166, 170, 174, 177 |
|  | 6 | 62 | –177, –176, –170, –164, –158, –152, –146, –140, –134, –128, –122, –116, –110, –104, –98, –92, –86, –80,–74,–68, –62, –56, –50, –44, –38, –32, –26, –20, –14, –8, –2, 2, 8, 14, 20, 26, 32, 38, 44, 50, 56, 62, 68, 74, 80, 86, 92, 98, 104, 110, 116, 122, 128, 134, 140, 146, 152, 158, 164, 170, 176, 177 |

The SNR values are encoded as an 8-bits twos complement value of 4×(SNR_average-22), where SNR_average is the sum of the values of SNR per tone (in decibels) divided by the number of tones represented.

The average SNR of space-time stream i subfield is an 8-bits twos complement integer whose definition is shown in Table 8-66.

Table 8-66—Average SNR of space–time stream i subfield

| Average SNR of space-time stream i subfield | AvgSNRi |
| --- | --- |
| –128 | –10dB |
| –127 | –9.75dB |
| –126 | –9.5dB |
| … | … |
| +126 | 53.5dB |
| +127 | 53.75dB |

#### Band ID field

Insert the following rows into Table 8-37 before the Last row:

Table 8-70—Band ID field

| Band ID value | Meaning |
| --- | --- |
| 6 | 45GHz |
| 7-255 | Reserved |

#### Operating Mode field

The Operating Mode field is present in the Operating Mode Notification frame (see 8.5.13) and Operating Mode Notification element (see 8.4.2.163).

The Operating Mode field is shown in Figure 8-80f.

Figure 8-80f Operating Mode field

The STA transmitting this field indicates its current operating channel width and the number of spatial streams it can receive using the settings defined in Table 8-53l.

Table 8-53l Subfield values of the Operating Mode field

| Subfield | Description |
| --- | --- |
| Channel Width | If the Rx NSS Type subfield is 0, indicates the supported channel width: Set to 0 for 540 MHz Set to 1 for 1080 MHz. Reserved if the Rx NSS Type subfield is 1. |
| Rx NSS | If the Rx NSS Type subfield is 0, indicates the maximum number of spatial streams that the STA can receive. If the Rx NSS Type subfield is 1, indicates the maximum number of spatial streams that the STA can receive as a beamformee in an SU PPDU using a beamforming steering matrix derived from a QMG Compressed Beamforming report with Feedback Type subfield indicating SU in the corresponding QMG Compressed Beamforming frame sent by the STA. Set to 0 for NSS = 1 Set to 1 for NSS = 2 … Set to 3 for NSS = 4 |
| Rx NSS Type | Set to 0 to indicate that the Rx NSS subfield carries the maximum number of spatial streams that the STA can receive. Set to 1 to indicate that the Rx NSS subfield carries the maximum number of spatial streams that the STA can receive in an SU PPDU using a beamforming steering matrix derived from a QMG Compressed Beamforming report with the Feedback Type subfield indicating SU in the corresponding QMG Compressed Beamforming frame sent by the STA. NOTE—An AP always sets this field to 0. |

### Information elements

#### General

Insert the following rows into Table 8-54 in numeric order, and change the reserved values accordingly:

Table 8-54—Element IDs

| Element | Element ID | Length of indicated element (in octets) | Extensible |
| --- | --- | --- | --- |
| QMG Capabilities (see 8.4.2.161) | 207 | 20 | YES |
| QMG Operation (see 8.4.2.162) | 208 | 5 | YES |
| Operating Mode Notification (see 8.4.2.163) | 209 | 3 | YES |
| Extended BSS Load element (see 8.4.2.164) | 210 | - | YES |
| QMG Transmit Power Envelope (see 8.4.2.165) | 211 | - | YES |
| Channel Switch Wrapper (see 8.4.2.166) | 212 | - | Subelements |
| QMG Link Margin (see 8.4.2.169) | 215 | 10 | YES |
| QMG Link Adaptation Acknowledgment (see 8.4.2.170) | 216 | 7 | YES |

#### QMG Capabilities element

##### QMG Capabilities element structure

A QMG STA declares that it is a QMG STA by transmitting the QMG Capabilities element.

The QMG Capabilities element contains a number of fields that are used to advertise the QMG capabilities of a QMG STA. The QMG Capabilities element is defined in Figure 8-249 (QMG Capabilities element format).

Figure 8-249 QMG Capabilities element format

The Element ID field is set to the value for QMG Capabilities element defined in Table 8-54.

The Length field of the QMG Capabilities element is set to 7.

##### QMG Capabilities Info field

The structure of the QMG Capabilities Info field is defined in Figure 8-327.

Figure 8-327 QMG Capabilities Info field

The subfields of the QMG Capabilities Info field are defined in Table 8-183v.

Table 8- 183v Subfields of the QMG Capabilities Info field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| Maximum MPDU Length | Indicates the maximum MPDU length (see 9.12). | Set to 0 for 3895 octets. Set to 1 for 7991 octets. Set to 2 for 11 454 octets. The value 3 is reserved. |
| Supported Channel Width Set | Indicates the channel widths supported by the STA. See 10.14. | Set to 0 if the STA does not support 1080 MHz. Set to 1 if the STA supports 1080 MHz. |
| Tx STBC | Indicates support for the transmission of at least 2x1 STBC. | Set to 0 if not supported. Set to 1 if supported. |
| Rx STBC | Indicates support for the reception of PPDUs using STBC. | Set to 0 for no support. Set to 1 for support of one spatial stream. Set to 2 for support of one and two spatial streams. The value 3 is reserved. |
| Short GI for 540 MHz | Indicates short GI support for the reception of packets transmitted with TXVECTOR parameter CH_BANDWIDTH set to QMG_CBW540. | Set to 0 if not supported. Set to 1 if supported. |
| Short GI for 1080 MHz | Indicates support for operation as an SU beamformee. | Set to 0 if not supported. Set to 1 if supported. |
| TPC | Indicates whether the STA supports TPC | The TPC field is set to 1 if the STA supports the TPC as defined in 10.8 (TPC procedures) and is set to 0 otherwise. |
| Number of Sounding Dimensions | Beamformer’s capability indicating the maximum value of the TXVECTOR parameter NUM_STS for a QMG NDP. | If SU beamformer capable, set to the maximum supported value of the TXVECTOR parameter NUM_STS minus 1. Otherwise, reserved. |
| QMG TXOP PS | Indicates whether the AP supports QMG TXOP Power Save Mode or whether the non-AP STA has enabled QMG TXOP Power Save mode. | Set to 0 if the AP does not support TXOP Power Save Mode. Set to 1 if the AP supports TXOP Power Save Mode. Set to 0 if the non-AP STA does not enable TXOP Power Save Mode. Set to 1 if the non-AP STA enables TXOP Power Save Mode. |
| QMG-Delayed Block Ack | Indicates support for QMG delayed Block Ack operation. See 9.21.1. | Set to 0 if not supported Set to 1 if supported Support indicates that the STA is able to accept an ADDBA request for QMG-delayed Block Ack |
| QMG Link Adaptation Capable | Indicates whether the STA supports link adaptation using QMG Control field | Set to 0 (No Feedback) if the STA does not provide QMG MFB. Set to 2 (Unsolicited) if the STA provides only unsolicited QMG MFB. Set to 3 (Both) if the STA can provide QMG MFB in response to QMG MRQ and if the STA provides unsolicited QMG MFB. The value 1 is reserved. |
| Rx Antenna Pattern Consistency | Indicates the possibility of a receive antenna pattern change. | Set to 0 if the receive antenna pattern might change during the lifetime of the current association. Set to 1 if the receive antenna pattern does not change during the lifetime of the current association. See10.14.6. |
| Tx Antenna Pattern Consistency | Indicates the possibility of a transmit antenna pattern change. | Set to 0 if the transmit antenna pattern might change during the lifetime of the current association. Set to 1 if the transmit antenna pattern does not change during the lifetime of the current association. See 10.14.6. |
| Fast Link Adaptation | Indicates whether the STA supports fast link adaptation | The Fast Link Adaptation field is set to 1 to indicate that the STA supports the QMG fast link adaptation procedure described in 9.28.3.3 (QMG Fast link adaptation). Otherwise, it is set to 0. |
| COLOR | Indicates the value that is used for the TXVECTOR parameter COLOR in frames transmitted by members of this BSS, as described in 9.17a. | Set to an unsigned integer in the range 0 to 7 if sent by an AP. Otherwise reserved. |
| NOTE—The value for the Maximum MPDU Length in the QMG Capabilities Info field imposes a constraint on the allowed value of the Maximum MPDU Length in the QMG Capabilities Info field of the QMG Capabilities element carried in the same frame (see 9.11). |  |  |

##### A-MPDU Parameters field

The structure of the A-MPDU Parameters field of the QMG Capabilities element is shown in Figure 7-328

Figure 8-328 A-MPDU Parameters field

Table 8- Subfields of the A-MPDU Parameters field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| Maximum AMPDU Length Exponent | Indicates the maximum length of A-MPDU that the STA can receive. EOF padding is not included in this limit. | This field is an integer in the range of 0 to 7. The length defined by this field is equal to 2（13 + Maximum A-MPDU Length Exponent） octets. |
| Minimum MPDU Start Spacing | Determines the minimum time between the start of adjacent MPDUs within an A-MPDU that the STA can receive, measured at the PHY-SAP. See 9.7d.3. | Set to 0 for no restriction Set to 1 for 16 ns Set to 2 for 32 ns Set to 3 for 64 ns Set to 4 for 128 ns Set to 5 for 256 ns Set to 6 for 512 ns Set to 7 for 1024 ns |

##### Supported QMG-MCS and NSS Set field

The Supported QMG-MCS and NSS Set field is used to convey the combinations of QMG-MCSs and spatial streams that a STA supports for reception and the combinations that it supports for transmission. The structure of the field is shown in Figure 8-401br.

Figure 8-401br Supported QMG-MCS and NSS Set field

The Supported QMG-MCS and NSS Set subfields are defined in Table 8-183w.

Table 8-183w Supported QMG-MCS and NSS Set subfields

| Subfield | Definition | Encoding |
| --- | --- | --- |
| Rx QMG-MCS Map | Indicates the maximum value of the RXVECTOR parameter MCS of a PPDU that can be received at all channel widths supported by this STA for each number of spatial streams. | The format and encoding of this subfield are defined in Figure 8-401bs (Rx QMG-MCS Map and Tx QMG-MCS Map subfields and Basic QMG-MCS and NSS Set field) and the associated description. |
| Rx Highest Supported Long GI Data Rate | Indicates the highest QMG PPDU data rate that the STA is able to receive. | The largest integer value less than or equal to the highest QMG PPDU data rate in Mb/s the STA is able to receive (see 9.7.11.1 (Rx Supported QMG-MCS and NSS Set)). The value 0 indicates that this subfield does not specify the highest QMG PPDU data rate that the STA is able to receive. |
| Tx QMG-MCS Map | Indicates the maximum value of the TXVECTOR parameter MCS of a PPDU that can be received at all channel widths supported by this STA for each number of spatial streams. | The format and encoding of this subfield are defined in Figure 8-401bs and the associated description. |
| Tx Highest Supported Long GI Data Rate | Indicates the highest QMG PPDU data rate that the STA is able to transmit at. | The largest integer value less than or equal to the highest QMG PPDU data rate in Mb/s the STA is able to receiveTransmit (see9.7.11.2(Tx Supported QMG-MCS and NSS Set)). The value 0 indicates that this subfield does not specify the highest QMG PPDU data rate that the STA is able to receive transmit. |

The Rx QMG-MCS Map subfield and the Tx QMG-MCS Map subfield have the structure shown in Figure 8-401bs.

Figure 8-401bs Rx QMG-MCS Map and Tx QMG-MCS Map subfields

and Basic QMG-MCS and NSS Set field

The Max QMG-MCS For n SS subfield (where n = 1, ... , 4) is encoded as follows:

- 0 indicates support for SC QMG -MCS 1-5 or OFDM QMG -MCS 9-13 for n spatial streams

- 1 indicates support for SC QMG -MCS 1-6 or OFDM QMG -MCS 9-14 for n spatial streams

- 2 indicates support for SC QMG -MCS 1-7 or OFDM QMG -MCS 9-15 for n spatial streams

- 3 indicates support for SC QMG -MCS 1-8 or OFDM QMG -MCS 9-16 for n spatial streams

- 4-7 indicates that n spatial streams is not supported

##### QMG Extended Capabilities field

The structure of the QMG Extended Capabilities field of the QMG Capabilities element is defined in Figure 8-252a.

Figure 8-252a QMG Extended Capabilities field

Table 160 Subfields of the QMG Extended Capabilities field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| PCO | Indicates support for PCO. When transmitted by an AP: indicates whether the AP can operate its BSS as a PCO BSS. When transmitted by a non-AP STA: indicates whether the STA can operate as a PCO active STA when the Transition Time subfield in its QMG Extended Capabilities field meets the intended transition time of the PCO capable AP. | Set to 0 if not supported Set to 1 if supported |
| PCO Transition Time | When transmitted by a non-AP STA: indicates that the STA can switch between 540 MHz channel width and 1080 MHz channel width within the specified time. When transmitted by an AP: indicates the PCO Transition Time to be used during PCO operation. The value contained in this field is dynamic when transmitted by an AP, i.e., the value of this field may change at any time during the lifetime of the association of a STA with the AP. See 10.16.3. | If the PCO subfield is equal to 0, this field is reserved. Otherwise: Set to 1 for 400 μs Set to 2 for 1.5 ms Set to 3 for 5 ms Set to 0 for no transition. In this case, the PCO active STA does not change its operating channel width and is able to receive 1080 MHz PPDUs during the 540 MHz phase (see 10.16). |
| MCS Feedback | Indicates whether the STA can provide MFB | Set to 0 (No Feedback) if the STA does not provide MFB Set to 2 (Unsolicited) if the STA provides only unsolicited MFB Set to 3 (Both) if the STA can provide MFB in response to MRQ (either Delayed or Immediate, see 9.28.1) as well as unsolicited MFB The value 1 is reserved |
| +QMGC Support | Indicates support of the QMG Control field. See 9.9 | Set to 0 if not supported Set to 1 if supported |
| RD Responder | Indicates support for acting as a reverse direction responder, i.e., the STA may use an offered RDG to transmit data to an RD initiator using the Reverse Direction Protocol described in 9.23. | Set to 0 if not supported Set to 1 if supported |

##### Transmit Beamforming Capabilities

The structure of the Transmit Beamforming Capabilities field is defined in Figure 253a.

Figure 8-253a —Transmit Beamforming Capabilities field

The subfields of the Transmit Beamforming Capabilities field are defined in Table 128a (Subfields of the Transmit Beamforming Capabilities field).

Table 128a-—Subfields of the Transmit Beamforming Capabilities field

| Subfield | Definition | Encoding |
| --- | --- | --- |
| Receive Staggered Sounding Capable | Indicates whether this STA can receive staggered sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Transmit Staggered Sounding Capable | Indicates whether this STA can transmit staggered sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Receive NDP Capable | Indicates whether this receiver can interpret null data packets as sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Transmit NDP Capable | Indicates whether this STA can transmit null data packets as sounding frames. | Set to 0 if not supported Set to 1 if supported |
| Explicit CSI Transmit Beamforming Capable | Indicates whether this STA can apply transmit beamforming using CSI explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit Noncompressed Steering Capable | Indicates whether this STA can apply transmit beamforming using noncompressed beamforming feedback matrix explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit Compressed Steering Capable | Indicates whether this STA can apply transmit beamforming using compressed beamforming feedback matrix explicit feedback in its transmission | Set to 0 if not supported Set to 1 if supported |
| Explicit Transmit Beamforming CSI Feedback | Indicates whether thisreceiver can return CSI explicit feedback | Set to 0 if not supported Set to 1 for delayed feedback Set to 2 for immediate feedback Set to 3 for delayed and immediate feedback |
| Explicit Noncompressed Beamforming Feedback Capable | Indicates whether this receiver can return noncompressed beamforming feedback matrix explicit feedback. | Set to 0 if not supported Set to 1 for delayed feedback Set to 2 for immediate feedback Set to 3 for delayed and immediate feedback |
| Explicit Compressed Beamforming Feedback Capable | Indicates whether thisreceiver can return compressed beamforming feedback matrix explicit feedback | Set to 0 if not supported Set to 1 for delayed feedback Set to 2 for immediate feedback Set to 3 for delayed and immediate feedback |
| Minimal Grouping | Indicates the minimal grouping used for explicit feedback reports | Set to 0 if the STA supports groups of 1 (no grouping) Set to 1 indicates groups of 1, 2 Set to 2 indicates groups of 1, 4 Set to 3 indicates groups of 1, 2, 4 Set to 4 indicates groups of 1, 6 Set to 5 indicates groups of 1, 2,6 Set to 6 indicates groups of 1, 4, 6 Set to 7 indicates groups of 1,2, 4, 6 |
| CSI Number of Beamformer Antennas Supported | Indicates the maximum number of beamformer antennas the QMG beamformee can support when CSI feedback is required | Set to 0 for single Tx antenna sounding Set to 1 for 2 Tx antenna sounding Set to 2 for 3 Tx antenna sounding Set to 3 for 4 Tx antenna sounding |
| Noncompressed Steering Number of Beamformer Antennas Supported | Indicates the maximum number of beamformer antennas the QMG beamformee can support when noncompressed beamforming feedback matrix is required | Set to 0 for single Tx antenna sounding Set to 1 for 2 Tx antenna sounding Set to 2 for 3 Tx antenna sounding Set to 3 for 4 Tx antenna sounding |
| Compressed Steering Number of Beamformer Antennas Supported | Indicates the maximum number of beamformer antennas the beamformee can support when compressed beamforming feedback matrix is required | Set to 0 for single Tx antenna sounding Set to 1 for 2 Tx antenna sounding Set to 2 for 3 Tx antenna sounding Set to 3 for 4 Tx antenna sounding |
| CSI Max Number of Rows Beamformer Supported | Indicates the maximum number of rows of CSI explicit feedback from the QMG beamformee that a QMG beamformer can support when CSI feedback is required. | Set to 0 for a single row of CSI Set to 1 for 2 rows of CSI Set to 2 for 3 rows of CSI Set to 3 for 4 rows of CSI |
| Channel Estimation Capability | Indicates the maximum number of space-time streams (columns of the MIMO channel matrix) for which channel dimensions can be simultaneously estimated when receiving an NDP sounding PPDU or the extension portion of the OCEFs in a staggered sounding PPDU. | Set 0 for 1 space-time stream Set 1 for 2 space-time streams Set 2 for 3 space-time streams Set 3 for 4 space-time streams |
| NOTE—The maximum number of space-time streams for which channel coefficients can be simultaneously estimated using the OCEFs corresponding to the data portion of the packet is limited by the Rx MCS Bitmask subfield of the Supported MCS Set field and by the Rx STBC subfield of the QMG Capability Information field. Both fields are part of the QMG Capabilities element. |  |  |

Insert the following subclauses, 8.4.2.161.7 (including Figure 8-254a), after 8.4.2.161.6:

##### QMG AP or PCP Capability Information field

The QMG AP or PCP Capability Information field, illustrated in Figure 8-254a, represents the capabilities when the transmitting STA performs in the role of AP or PCP and that are in addition to the capabilities in the QMG STA Capability Information field.

Figure 8-254a QMG AP or PCP Capability Information field format

The TDDTI (time division data transfer interval) subfield is set to 1 if the STA, while operating as an AP or PCP, is capable of providing channel access as defined in 9.36.6 (Channel access in scheduled DTI) and 10.4 (TS operation) and is set to 0 otherwise.

The Pseudo-static Allocations subfield is set to 1 if the STA, while operating as an AP or PCP, is capable of providing pseudo-static allocations as defined in 9.36.6.4 (Pseudo-static allocations) and is set to 0 otherwise. The Pseudo-static Allocations subfield is set to 1 only if the TDDTI subfield in the QMG AP or PCP Capability Information field is set to 1. The Pseudo-static Allocations subfield is reserved if the TDDTI subfield in the QMG AP or PCP Capability Information field is set to 0.

The PCP Handover field is set to 1 if the STA, while operating as a PCP, is capable of performing a PCP Handover as defined in 10.29.2 and is set to 0 if the STA does not support PCP Handover.

The MAX Associated STA Number field indicates the maximum number of STAs that the STA can perform association with if operating as an AP or PCP. The value of this field includes the STAs, if any, that are collocated with the AP or PCP.

The Power Source field is set to 0 if the STA is battery powered and is set to 1 otherwise.

The Decentralized AP or PCP Clustering field is set to 1 if the STA, when operating as an AP or PCP, is capable of performing Decentralized AP or PCP clustering and is set to 0 otherwise.

The PCP Forwarding field is set to 1 if the STA, while operating as a PCP, is capable of forwarding frames it receives from a non-PCP STA and destined to another non-PCP STA in the PBSS. This field is set to 0 otherwise.

The Centralized AP or PCP Clustering field is set to 1 if the STA, when operating as an AP or PCP, is capable of performing centralized AP or PCP clustering and is set to 0 otherwise. An AP or PCP that is incapable of performing centralized AP or PCP clustering is subject to requirements as described in 9.37.2.2.

#### QMG Operation element

The operation of QMG STAs in the BSS is controlled by the QMG Operation element. The format of the QMG Operation element is defined in Figure 8-401bv.

Figure 8-401bv QMG Operation element format

The Element ID field is set to the value for QMG Operation element defined in Table 8-54.

The structure of the QMG Operation Information field is defined in Figure 8-401bw.

Figure 8-401 bw QMG Operation Information field

The subfield of the QMG Operation Information field is defined in Table 8-401bs.

Table 8-401bs QMG Operation Information field format

| Field | Definition | Encoding |
| --- | --- | --- |
| Primary Channel | Indicates the channel number of the primary channel, see 1.10. | Channel number of the primary channel |
| Channel Width | This field, defines the BSS operating channel width. | Set to 0 for 540 MHz operating channel width. Set to 1 for 1080 MHz operating channel width. |
| PCO Active | Indicates whether PCO is active in the BSS Present in Beacon/Probe Response frames transmitted by an AP. Otherwise reserved. Non-PCO STAs regard the BSS as a 540/1080 MHz BSS and may associate with the BSS without regard to this field. See 10.16(Phased coexistence operation (PCO)) | Set to 0 if PCO is not active in the BSS Set to 1 if PCO is active in the BSS |
| PCO Phase | Indicates the PCO phase of operation. Defined only in a Beacon and Probe Response frames when PCO Active is 1. Otherwise reserved. See 10.16 (Phased coexistence operation (PCO)). | Set to 0 indicates switch to or continue 540 MHz phase; Set to 1 indicates switch to or continue 1080 MHz phase |

The Basic QMG-MCS and NSS Set field indicates the QMG-MCSs for each number of spatial streams in QMG PPDUs that are supported by all QMG STAs in the BSS (including IBSS and MBSS). The Basic QMG MCS and NSS Set field is a bitmap of size 2 bits. The Basic QMG-MCS and NSS Set field is defined in Figure8-401bs.

#### Operating Mode Notification element

The Operating Mode Notification element is used to notify STAs that the transmitting STA is changing its operating channel width, the maximum number of spatial streams it can receive, or both. The format of the Operating Mode Notification element is defined in Figure 8-401cc.

Figure 8-401cc Operating Mode Notification element

The Element ID field is set to the value for the Operating Mode Notification element in Table 8-54.

The Length field is set to 1.

The Operating Mode field is defined in 8.4.1.12.

#### Extended BSS Load element

#### QMG Transmit Power Envelope

#### Channel Switch Wrapper

#### QMG Link Margin element

##### General

The format of the QMG Link Margin element is shown in Figure 8-577a (QMG Link Margin element format). The QMG Link Margin element is included in a Link Measurement Report frame.

|  | Element ID | Length | Activity | MCS | Link Margin | SNR | Reference Timestamp |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 4 |

Figure 8-577a—QMG Link Margin element format

The Element ID and Length fields are defined in 8.4.2.1 (General).

The Activity field is set to a preferred action that the STA sending this element recommends that the peer STA indicated in the RA field of the Link Measurement Report frame execute. The method by which the sending STA determines a suitable action for the peer STA is implementation specific. The Activity field is defined in 8.4.2.169.2 (Activity field).

The MCS field is set to an MCS value that the STA sending this element recommends that the peer STA indicated in the RA field of the Link Measurement Report frame use to transmit frames to this STA. The reference PER for selection of the MCS is  for an MPDU length of 4096 octets. The method by which the sending STA determines a suitable MCS for the peer STA is implementation specific. The Link Margin field contains the measured link margin of Data frames received from the peer STA indicated in the RA field of the Link Measurement Report frame and is coded as a 2s complement signed integer in units of decibels. A value of –128 indicates that no link margin is provided. The measurement method of link margin is beyond the scope of this standard.

The SNR field indicates the SNR measured during the reception of a PHY packet. Values are from –13 dB to 50.75 dB in 0.25 dB steps.

The Reference Timestamp field contains the lower 4 octets of the TSF timer value sampled at the instant that the MAC received the PHY-CCA indication (IDLE) signal that corresponds to the end of the reception of the PPDU that was used to generate the feedback information contained in the Link Measurement Report frame.

##### Activity field

The Activity field values are defined in Table 8-248a (Activity field values).

Table 8-248a—Activity field values

| Preferred Action value | Meaning |
| --- | --- |
| 0 | No change preferred |
| 1 | Change(d) MCS |
| 2 | Decrease(d) transmit power |
| 3 | Increase(d) transmit power |
| 4 | Fast session transfer (FST) |
| 5 | Power conserve mode |
| 6 | Perform SLS |
| 7-255 | Reserved |

#### 8.4.2.170 QMG Link Adaptation Acknowledgment element

The format of the QMG Link Adaptation Acknowledgment element is shown in Figure 8-577b (QMG Link Adaptation Acknowledgment element format). The QMG Link Adaptation Acknowledgment element is carried in the Optional Subelements field of the Link Measurement Report frame.

|  | Element ID | Length | Activity | Reference Timestamp |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 |

Figure 8-577b—DMG Link Adaptation Acknowledgment element format

The Element ID and Length fields are defined in 8.4.2.1 (General).

The Activity field is set to the action that the STA sending this element has executed following the reception of the recommended activity in a Link Measurement Report frame. The method by which the sending STA determines the action is described in 9.25 and the Activity field is defined in 8.4.2.169.2 (Activity field).

The Reference Timestamp field contains the lower 4 octets of the TSF timer value sampled at the instant that the MAC received the PHY-CCA indication (IDLE) signal that corresponds to the end of the reception of the PPDU that was used to generate the feedback information contained in the Link Measurement Report frame.

## Action frame format details

### Spectrum management Action frames

.

### QMG Action frame details

#### QMG Action field

Several Action frame formats are defined to support QMG functionality. A QMG Action field, in the octet immediately after the Category field, differentiates the QMG Action frame formats. The QMG Action field values associated with each frame format within the QMG category are defined in Table 8-281ah.

Table 8-281ah—QMG Action field values

| Value | Meaning | Time Priority |
| --- | --- | --- |
| 0 | QMG Compressed Beamforming | Yes |
| 1 | QMG Noncompressed Beamforming | Yes |
| 2 | QMG CSI | Yes |
| 3 | QMG PCO Set Phase | Yes |
| 4-255 | Reserved |  |

#### SM Power Save frame format

#### QMG Set PCO Phase frame format

Set PCO Phase is an Action frame of category QMG that announces the phase change between 540 MHz and 1080MHz. The format of its Action field is defined in Table 8-233a. The operation of the PCO feature is defined in 10.16.

Table 8-233a—Set PCO Phase frame Action field format

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | QMG Action |
| 3 | PCO Phase Control field |

The Category field is set to the value for QMG, specified in Table 8-38.

The QMG Action field is set to the value for Set PCO Phase, specified in Table 8-281ah.

This frame is sent by a PCO active AP.

#### QMG CSI frame format

The CSI frame is an Action or an Action No Ack frame of category QMG. The format of its Action field is defined in Table 8-322 (CSI frame Action field format).

Table 8-322 CSI frame Action field format

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | QMG Action |
| 3 | QMG MIMO Control (see 8.4.1.26 (QMG MIMO Control field)) |
| 4 | QMG CSI Report (see 8.4.1.27 (QMG CSI Report field)) |

The Category field is set to the value for QMG, specified in Table 8-54 (Category values).

The QMG Action field is set to the value for CSI, specified in Table 8-317 (QMG Action field values).

In a CSI frame, the fields of the QMG MIMO Control field (see 8.4.1.1 (QMG MIMO Control field)) are used as described in Table 8-58 (Subfields of the QMG MIMO Control field).

#### QMG Noncompressed Beamforming frame format

The Noncompressed Beamforming frame is an Action or an Action No Ack frame of category QMG. The format of its Action field is defined in Table 8-235a (Noncompressed Beamforming frame Action field format).

The Category field is set to the value for QMG, specified in Table 8-38 (Category values).

Table 8-235a Noncompressed Beamforming frame Action field format

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | QMG Action |
| 3 | QMG MIMO Control (see 8.4.1.1 (QMG MIMO Control field)) |
| 4 | QMG Noncompressed Beamforming Report (see 8.4.1.4 (QMG Noncompressed Beamforming Report field)) |

The QMG Action field is set to the value for Noncompressed Beamforming, specified in Table 8-229a (QMG Action field values).

In a Noncompressed Beamforming frame, the fields of the QMG MIMO Control field (see 8.4.1.1 (QMG MIMO Control field)) are used as described in Table 8-58 (Subfields of the QMG MIMO Control field).

#### QMG Compressed Beamforming frame format

The Compressed Beamforming frame is an Action or an Action No Ack frame of category QMG. The format of its Action field is defined in Table 8-236a (Compressed Beamforming frame Action field format).

Table 8-236a Compressed Beamforming frame Action field format

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | QMG Action |
| 3 | QMG MIMO Control (see 8.4.1.1 (QMG MIMO Control field)) |
| 4 | QMG Compressed Beamforming Report (see 8.4.1.48 (QMG Compressed Beamforming Report field)) |

The Category field is set to the value for QMG, specified in Table 8-38 (Category values).

The QMG Action field is set to the value for Compressed Beamforming, specified in Table 8-229a (QMG Action field values).

In a Compressed Beamforming frame, the fields of the QMG MIMO Control field (see 8.4.1.26 (QMG MIMO Control field)) are used as described in Table 8-58 (Subfields of the QMG MIMO Control field).

#### Operating Mode Notification frame format

The Operating Mode Notification frame is an Action frame of category QMG. It is used to notify STAs that the transmitting STA is changing its operating channel width, the maximum number of spatial streams it can receive, or both.

The Action field of the Operating Mode Notification frame contains the information shown in Table 8-281al.

Table 8-281al Operating Mode Notification frame Action field format

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | QMG Action |
| 3 | Operating Mode (see 8.4.1.12) |

# MAC sublayer functional description

Insert the following subclause, 9.3.2.5a, after 9.3.2.5:

9.3.2.5a  QMG RTS procedure

A QMG STA transmits an RTS frame carried in QMG or QMG duplicate format. If the STA sending the RTS frame is capable of dynamic bandwidth operation (see 9.3.2.6), the QMG STA shall set the TXVECTOR parameter DYN_BANDWIDTH to Dynamic. Otherwise, the STA shall set the TXVECTOR parameter DYN_BANDWIDTH to Static.

#### CTS and QMG CTS procedure

Insert the following paragraphs at the beginning of 9.3.2.6:

A QMG STA that is addressed by an RTS frame in a QMG or QMG duplicate PPDU that has the RXVECTOR parameter DYN_BANDWIDTH equal to Static behaves as follows:

—  If the NAV indicates idle and CCA has been idle for the secondary channel (secondary 540MHz channel) in the channel width indicated by the RTS frame’s RXVECTOR parameter CH_BANDWIDTH for a PIFS period prior to the start of the RTS frame, then the STA shall respond with a CTS frame carried in a QMG or QMG duplicate PPDU after a SIFS period. The CTS frame’s TXVECTOR parameters CH_BANDWIDTH shall be set to the same value as the RTS frame’s RXVECTOR parameter CH_BANDWIDTH.

—  Otherwise, the STA shall not respond with a CTS frame.

A QMG STA that is addressed by an RTS frame in a QMG or QMG duplicate PPDU that has the RXVECTOR parameter DYN_BANDWIDTH equal to Dynamic behaves as follows:

—  If the NAV indicates idle, then the STA shall respond with a CTS frame in a QMG or QMG duplicate PPDU after a  SIFS period. The CTS frame’s TXVECTOR parameters CH_BANDWIDTH may be set to any channel width for which CCA has been idle for a PIFS prior to the start of the RTS frame and that is equal to or less than the channel width indicated in the RTS frame’s RXVECTOR parameter indicated in the RTS frame’s RXVECTOR parameter CH_BANDWIDTH.

—  Otherwise, the STA shall not respond with a CTS frame.

### DCF access procedure

#### Recovery procedures and retransmit limits

Insert the following paragraph at the end of 9.3.4.4:

An AP that fails to receive an acknowledgment after the AP transmits a frame with the More Data field set to 0 to a non-AP QMG STA that is in QMG TXOP power save mode retransmits the frame within the current TXOP under certain conditions as described in 10.2.1.19.

## Multirate Support

### Overview

For the Clause 17, Clause 18, Clause 19, Clause 21, Clause 22, Clause 23, Clause 24, Clause 25, and Clause 26 PHYs, the time required to transmit a frame for use in calculating the value for the Duration/ID field is determined using the PLME-TXTIME.request primitive (see 6.5.7) and the PLME-TXTIME.confirm primitive (see 6.5.8), both defined in 17.3.4, 18.4.3, 19.8.3.2, 19.8.3.3, 19.8.3.4, 20.4.3， or  26，depending on the PHY options. In QoS STAs, the Duration/ID field may cover multiple frames and may involve using the PLME-TXTIME.request primitive several times.

### Basic STBC MCS

The basic STBC MCS has the value null when any of the following conditions is true:

- No QMG Operation element is present in the most recently received Association Response frame that was addressed to this STA.

- The BSSBasicMCSSet is empty or does not exist.

- The lowest MCS of the BSSBasicMCSSet has NSS value greater than 1 (the mapping of MCS to NSS is PHY dependent, for QMG PHY see 26).

If none of the above conditions is true, then the basic STBC MCS is the lowest MCS index of the BSSBasicMCSSet parameter.

When an MCS from the basic STBC MCS is required in 9.7.5 and 9.7.6 but the basic STBC MCS has the value null, the STA shall select a mandatory MCS of the attached PHY.

### Basic Rate Set and Basic MCS Set for mesh STA

A mesh STA shall not establish a mesh peering with a mesh STA using a different BSSBasicRateSet (see 13.2.7 and 13.2.8).

Mesh STAs should adopt the mandatory PHY rates as the default BSSBasicRateSet to reduce the risk that a candidate peer mesh STA utilizes a different BSSBasicRateSet. If the mesh STA is also a QMG STA, it should adopt the MCSs of mandatory MCSs as the default BSSBasicMCSSet.

### Rate selection for data and management frames

#### Rate selection for non-STBC Beacon

If the BSSBasicRateSet parameter is not empty, a non-STBC Beacon shall be transmitted in a QMG PPDU using one of the rates included in the BSSBasicRateSet parameter.

If the BSSBasicRateSet parameter is empty, the frame shall be transmitted in a QMG PPDU using one of the mandatory PHY rates.

#### Rate selection for STBC group addressed data and management frames

When a STA has dot11TxSTBCOptionActivated true, it shall use the basic STBC MCS when it transmits an STBC Beacon frame or when it transmits a group addressed data or management frame that is an STBC frame.

#### Rate selection for other group addressed data and management frames

Change the last paragraph of 9.7.5.3 as follows:

If both the BSSBasicRateSet parameter, and the BSSBasicMCSSet parameter, and the BSS basic QMG-MCS and NSS set are empty (e.g., a scanning STA that is not yet associated with a BSS), the frame shall be transmitted in a QMG PPDU using one of the mandatory PHY rates.

Insert the following paragraph before the last paragraph of 9.7.5.3:

If the BSSBasicRateSet parameter is empty and the BSSBasicMCSSet parameter is empty and the BSS basic QMG-MCS and NSS set is not empty, the frame shall be transmitted in a QMG PPDU using one of the <QMG-MCS. NSS> tuples included in the BSS basic QMG-MCS and NSS set.

#### Rate selection for polling frames

A data frame of a subtype that includes CF-Poll that does not also include CF-Ack and that is sent in the CP shall be transmitted at a rate selected as follows:

a) If an initial exchange has already established protection and the Duration/ID field in the frame establishing protection covers the entire TXOP, the rate or MCS is selected according to the rules in 9.7.5.6.

b) Otherwise, the data frame shall be transmitted at a rate or MCS as defined in 9.7.5.3, treating the frame as though it has a group address in the Address 1 field, solely for the purpose of determining the appropriate rate or MCS.Change 9.7.5.6 as follows:

#### Rate selection for other data and management frames

A data or management frame not identified in 9.7.5.1 through 9.7.5.5.1 shall be sent using any data rate, MCS, or <QMG-MCS, NSS> tuple subject to the following constraints:

- A STA shall not transmit a frame using a rate or MCS that is not supported by the receiver STA or STAs, as reported in any Supported Rates element, Extended Supported Rates element, or Supported MCS Set field in management frames transmitted by the receiver STA.

- A STA shall not transmit a frame using a <QMG-MCS, NSS> tuple that is not supported by the receiver STA, as reported in any Supported QMG-MCS and NSS Set field in management frames transmitted by the receiver STA.

- If at least one Operating Mode field with the Rx NSS Type subfield equal to 0 was received from the receiver STA:

- A STA shall not transmit a frame with the number of spatial streams greater than that indicated in the Rx NSS subfield in the most recently received Operating Mode field with the Rx NSS Type subfield equal to 0 from the receiver STA.

- A STA shall not transmit a frame using a value for the CH_BANDWIDTH parameter of the TXVECTOR that is not supported by the receiver STA, as reported in any QMG Capabilities element or QMG Capabilities element received from the intended receiver.

- A STA that is a member of a BSS and that is not a QMG STA shall not transmit a frame using a value for the CH_BANDWIDTH parameter of the TXVECTOR that is not permitted for use in the BSS, as reported in the most recently received QMG Operation element with the exception transmissions on a TDLS off-channel link, which follow the rules described in 10.22.

- A QMG STA that is a member of a BSS shall not transmit a frame using a value for the CH_BANDWIDTH parameter of the TXVECTOR that is not permitted for use in the BSS, as reported in the most recently received QMG Operation element with the following exceptions:

- Transmissions on a TDLS off-channel link follow the rules described in 10.22.

- Transmissions by a QMG STA on a TDLS link follow the rules described in 10.22.

- If at least one Operating Mode field with the Rx NSS Type subfield equal to 0 was received from the receiver STA:

- A STA shall not transmit a frame using a value for the TXVECTOR parameter CH_BANDWIDTH that is not supported by the receiver STA as reported in the most recently received Operating Mode field with the Rx NSS Type subfield equal to 0 from the receiver STA.

- A STA shall not initiate transmission of a frame at a data rate higher than the greatest rate in the OperationalRateSet, or using an MCS that is not in the QMG OperationalMCSset, or using a <QMG-MCS, NSS> tuple that is not in the OperationalQMGMCS_NSSSet, which are parameters of the MLMEJOIN.request primitive.

When the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate in the BSSBasicRateSet parameter, or an MCS in the BSSBasicMCSSet parameter, or a <QMG-MCS, NSS> tuple in the BSS basic QMG-MCS and NSS set, or a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet, the BSSBasicMCSSet, and the BSS basic QMG-MCS and NSS set are empty.

The rules in this subclause also apply to A-MPDUs that aggregate MPDUs of type Data or Management with any other types of MPDU.

### Rate selection for control frames

#### General rules for rate selection for control frames

Control frames carried in an A-MPDU that does not contain a QMG single MPDU shall be sent at a rate selected from the rules defined in 9.7.5.6.

NOTE—The rules defined in 9.7.6.2 through 9.7.6.5 apply only to control frames not carried in an A-MPDU that does not contain a QMG single MPDU.

The following rules determine whether a control frame is carried in QMG PPDU:

- A control frame shall be carried in a QMG PPDU when the control frame meets any of the following conditions:

  - The control frame is sent using an STBC frame.

- A control response frame shall be carried in a QMG PPDU when the control frame is a response to a frame that meets any of the following conditions:

  - The frame eliciting the response was an RTS frame carried in a QMG PPDU; or

  - The frame eliciting the response was an STBC frame.

- A control frame may be carried in a QMG PPDU when the control frame meets any of the following conditions:

  - The control frame contains a QMG Control field with the MRQ subfield equal to 1.

- A control frame may be carried in a QMG PPDU when the control frame is an STBC frame.

- A control response frame shall be carried in a QMG PPDU if the eliciting frame was an RTS frame carried in a QMG PPDU that contains a QMG Control field with MRQ subfield equal to 1.

NOTE—In these cases, the requirements specified in 9.27, 9.28.2, and 9.29 further constrain the choice of QMG PPDU.

If an Operating Mode field has been received from the intended receiver STA, the following constraints also apply:

- If at least one Operating Mode field with the Rx NSS Type subfield equal to 0 was received from the receiver STA.

- A STA shall not transmit a frame with the number of spatial streams greater than that indicated in the Rx NSS subfield in the most recently received Operating Mode field with the Rx NSS Type subfield equal to 0 from the receiver STA.

#### Rate selection for control frames that initiate a TXOP

Change the first paragraph of 9.7.6.2 as follows:

This subclause describes the rate selection rules for control frames that initiate a TXOP and that are either a QMG single MPDU or not carried in an A-MPDU.

Insert the following paragraph at the end of 9.7.6.2:

When transmitting a QMG PPDU, a STA shall select a <QMG-MCS, NSS> tuple from the BSS basic QMG-MCS and NSS set when protection is required (as defined in 9.23) and shall select a <QMG-MCS, NSS> tuple from the Supported QMGMCS_NSSSet parameter of the intended receiver when protection is not required.

#### Rate selection for control frames that are not control response frames

Change the first paragraph of 9.7.6.4 as follows:

This subclause describes the rate selection rules for control frames that are not control response frames, are not the frame that initiates a TXOP, are not the frame that terminates a TXOP, and are either a QMG single MPDU or not carried in an A-MPDU.

Change the fourth paragraph of 9.7.6.4 as follows:

A frame that is carried in A QMG PPDU shall be transmitted by the STA using an MCS supported by the receiver STA, as reported in the Supported MCS Set field in the QMG Capabilities element received from that STA. When the supported MCS set of the receiving STA or STAs is not known, the transmitting STA shall transmit using an MCS in the BSSBasicMCSSet parameter.

Insert the following paragraph at the end of 9.7.6.4:

A frame that is carried in a QMG PPDU shall be transmitted by the STA using a <QMG-MCS, NSS> tuple supported by the receiver STA. A <QMG-MCS, NSS> tuple is supported if reported as such in the Supported QMG-MCS and NSS Set field in the QMG Capabilities element received from that STA. When the Supported QMG-MCS and NSS set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a <QMG-MCS, NSS> tuple in the BSS basic QMG-MCS and NSS set.

#### Rate selection for control response frames

##### Introduction

Change 9.7.6.5.1 as follows:

Subclauses 9.7.6.5.2 through 9.7.6.5.5 describe the rate selection rules for control response frames that are either a QMG single MPDU or not carried in an A-MPDU.

##### Selection of a rate or MCS

Change the second bullet in the dashed list of the first paragraph of 9.7.6.5.2 as follows:

- If the control response frame is carried in a QMG PPDU, then it is transmitted using an MCS or <QMG-MCS, NSS> tuple as determined by the procedure defined in 9.7.6.5.3.

Change the second paragraph of 9.7.6.5.2 as follows:

The modulation class of the control response frame shall be selected according to the following rules:

- If the received frame is of a modulation class other than QMG PPDU, the control response frame shall be transmitted using the same modulation class as the received frame.

- If the control response frame is carried in a QMG PPDU, the modulation class shall be QMG.

##### Control response frame MCS computation

Change 9.7.6.5.3 as follows:

If a control response frame is to be transmitted within a QMG PPDU, the channel width (CH_BANDWIDTH parameter of the TXVECTOR) shall be selected first according to 9.7.6.6, and then the MCS or <QMG-MCS, NSS> tuple shall be selected from a set of MCSs and <QMG-MCS, NSS> tuples called the CandidateMCSSet as described in this subclause.

If the frame eliciting the response was transmitted by a QMG STA, the Rx Supported MCS Set is determined for QMG PPDUs as described in 9.7.11.

The CandidateMCSSet is determined using the following rules:

- If the frame eliciting the response was an STBC frame, the CandidateMCSSet shall contain only the basic STBC MCS.

- If the above conditions are true, the CandidateMCSSet is the union of the BSSBasicMCSSet and the BSS basic QMG-MCS and NSS set. If the frame eliciting the response was an RTS frame carried in a QMG PPDU, then the CandidateMCSSet may additionally include the <QMG-MCS, NSS> tuple with the same MCS and number of spatial streams as the QMG PPDU. If the combined BSSBasicMCSSet parameter is empty, the CandidateMCSSet shall consist of

- The set of mandatory PHY MCSs if the STA eliciting the response is an STA that is not a QMG STA

- The set of mandatory MCSs plus the set of <QMG-MCS, NSS> tuples corresponding to the mandatory QMG PHY MCSs with NSS = 1 if the STA eliciting the response is a QMG STA.

MCS values from the CandidateMCSSet that cannot be transmitted with the selected CH_BANDWIDTH parameter value shall be eliminated from the CandidateMCSSet.

The choice of a response MCS is made as follows:

- If the frame eliciting the response is within a QMG PPDU,

- Eliminate from the CandidateMCSSet all MCSs and all <QMG-MCS, NSS> tuples that meet any of the following conditions:

- Have a data rate that is higher than the data rate of the <QMG-MCS, NSS> tuple of the received frame using the largest possible value of CH_BANDWIDTH that is no larger than the value of CH_BANDWIDTH of the received frame

- Have a number of spatial streams greater than that indicated in the Rx NSS subfield in the most recent Operating Mode field with the Rx NSS Type subfield equal to 0 from the intended receiver STA, if at least one Operating Mode field with the Rx NSS Type subfield equal to 0 was received from the intended receiver STA

- Determine the highest number of spatial streams (NSS) value of the MCSs and <QMG-MCS, NSS> tuples in the CandidateMCSSet that is less than or equal to the NSS value of the received frame. Eliminate all MCSs from the CandidateMCSSet that have an NSS value that is not equal to this NSS value. The mapping from MCS to NSS is dependent on the attached PHY.

- Find the highest rate MCS or <QMG-MCS, NSS> tuple of the CandidateMCSSet for which the modulation value of each stream is less than or equal to the modulation value of each stream of the MCS of the received frame and for which the coding rate is less than or equal to the coding rate of the MCS from the received frame. This MCS or <QMG-MCS, NSS> tuple is the primary MCS for the response transmission. The mapping from MCS or <QMG-MCS, NSS> tuple to modulation and coding rate is dependent on the attached PHY. For the purpose of comparing modulation values, the following sequence shows increasing modulation values: BPSK, QPSK, 16-QAM, 64-QAM.

- If no MCS meets the condition in step 3), remove each MCS or <QMG-MCS, NSS> tuple from the CandidateMCSSet that has the highest value of NSS in the CandidateMCSSet. If the resulting CandidateMCSSet is empty, then set the CandidateMCSSet to the QMG PHY mandatory MCSs. Repeat step 3) using the modified CandidateMCSSet.

Once the primary MCS or <QMG-MCS, NSS> tuple has been selected, the STA may select an alternate MCS according to 9.7.6.5.4. The STA shall transmit the control response frame using either the primary MCS or the alternate MCS, if one exists.

##### Selection of an alternate rate or MCS for a control response frame

An alternate rate may be selected provided that all of the following conditions are met:

- The duration of frame at the alternate rate is the same as the duration of the frame at the primary rate determined by 9.7.6.5.2.

- The alternate rate is in either the BSSBasicRateSet parameter or is a mandatory rate of the attached PHY.

- The modulation class of the frame at the alternate rate is the same class as that of the primary rate selected by 9.7.6.5.2.

An alternate MCS may be selected provided that both of the following conditions are met:

- The duration of the frame at the alternate MCS is the same as the duration of the frame at the primary MCS.

- The alternate MCS is in the CandidateMCSSet that was generated according to the procedure of 9.7.6.5.3.

##### Control response frame TXVECTOR parameter restrictions

Change 9.7.6.5.5 as follows:

A STA shall not transmit a control response frame with TXVECTOR parameter FEC_CODING set to LDPC_CODING unless it is in response to a reception of a frame with the RXVECTOR parameter FEC_CODING equal to LDPC_CODING.

#### Control frame TXVECTOR parameter restrictions

A STA shall not transmit a control frame that initiates a TXOP with the TXVECTOR parameter FEC_CODING set to a value of LDPC_CODING.

### Multiple BSSID Rate Selection

If the multiple BSSID capability is supported, Beacon frames shall be transmitted using any basic rate valid for all of the BSSs supported. If no such rate exists, then Beacon frames shall be transmitted using any mandatory PHY rate for any PHY type that all BSSs have in common.

### Modulation classes

Change 9.7.8 as follows:

Table 9-4a defines modulation classes for the rules for response frames in 9.7.

Table 9-4a—Modulation classes

| Modulation class | Description of modulation | Condition that selects this modulation class |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Clause 14 to Clause 19 PHYs and Clause 21 PHY | Clause 20 PHY | Clause 22 PHY | Clause 26 PHY |
| 14 | QMG | N/A | N/A | N/A | FORMAT is QMG. |

### Rate Selection Constraints for QMG STAS

Insert 9.7.11 as follows:

#### Rx Supported QMG-MCS and NSS Set

The Rx Supported QMG-MCS and NSS Set of a QMG STA is determined for each <QMG-MCS, NSS> tuple NSS = 1,…, 4 and bandwidth (540 MHz and 1080 MHz) from its Supported QMG-MCS and NSS Set field as follows:

- If support for the QMG-MCS for NSS spatial streams at that bandwidth is mandatory (see 26.15), then the <QMG-MCS, NSS> tuple at that bandwidth is supported by the STA on receive.

- Otherwise, the <QMG-MCS, NSS> tuple at that bandwidth is not supported by the STA on receive.

A QMG STA shall not, unless explicitly stated otherwise, transmit a QMG PPDU unless the <QMG-MCS,NSS> tuple and bandwidth used are in the Rx Supported QMG-MCS and NSS Set of the receiving STA(s).

#### Tx Supported QMG-MCS and NSS Set

The Tx Supported QMG-MCS and NSS Set of a QMG STA is determined for each <QMG-MCS, NSS> tuple

NSS = 1,…, 4 and bandwidth (540 MHz and 1080 MHz) from its Supported QMG-MCS and NSS Set field as follows:

- If support for the <QMG-MCS, NSS> tuple at that bandwidth is mandatory (see 26.15), then the <QMG-MCS, NSS> tuple at that bandwidth is supported by the STA on transmit.

- Otherwise, the <QMG-MCS, NSS> tuple at that bandwidth is not supported by the STA on transmit.

## QMG Control field operation

Insert the following sentence to the first paragraph in 9.9:

If the value of dot11QMGControlFieldSupported is true, a STA shall set the +QMGC Support subfield of the QMG Extended Capabilities field of the QMG Capabilities element to 1 in QMG Capabilities elements that it transmits.

Insert the following sentence to the second paragraph in 9.9:

A STA that has a value of true for at least one of dot11RDResponderOptionImplemented, dot11MCSFeedbackOptionImplemented, and dot11AlternateEDCAActivated shall set dot11QMGControlFieldSupported to true.

Insert the following sentence to the fifth paragraph in 9.9:

A QMG Control field shall not be present in a frame addressed to a STA unless that STA declares support for +QMGC in the QMG Capabilities Info field of its QMG Capabilities element.

Insert the following sentence to the sixth paragraph in 9.9:

NOTE—A QMG STA that does not support +QMGC that receives a +QMGC frame addressed to another STA still performs the CRC on the actual length of the MPDU and uses the Duration/ID field to update the NAV, as described in 9.3.2.4 (Setting and resetting the NAV).

Insert the following sentence to the last paragraph in 9.9:

If the QMG Control field is present in an MPDU aggregated in an A-MPDU, then all MPDUs of the same frame type (i.e., having the same value for the Type subfield of the Frame Control field) aggregated in the same A-MPDU shall contain a QMG Control field. The QMG Control field of all MPDUs containing the QMG Control field aggregated in the same A-MPDU shall be set to the same value.

## Control Wrapper operation

Insert the following sentence to the first paragraph in 9.10:

A STA supporting the QMG Control field that receives a Control Wrapper frame shall process it as though it received a frame of the subtype of the wrapped frame.

NOTE—A STA supporting the QMG Control field can reset the NAV set by a wrapped RTS frame following the rules defined in 9.3.2.4 (Setting and resetting the NAV).

## A-MSDU operation

Change the 11th paragraph of 9.11 as follows:

A STA shall support the reception of an A-MSDU, where the A-MSDU is carried in a QoS Data frame with Ack Policy equal to Normal Ack in the following cases:

- For an HT STA if the A-MSDU is not aggregated within an A-MPDU

- For a VHT STA if the A-MSDU is sent as a VHT single MPDU

- For a QMG STA if the A-MSDU is not aggregated within an A-MPDU

Insert the following sentence after the 13th of 9.11

A STA shall not transmit an A-MSDU in a QMG PPDU to a STA if the A-MSDU length exceeds the value indicated by the Maximum A-MSDU Length field of the QMG Capabilities element received from the recipient STA.

Insert the following sentence after the 14th of 9.11

A QMG STA that sets the Maximum MPDU Length in the QMG Capabilities element to indicate 3895 octets shall set the Maximum A-MSDU Length in the QMG Capabilities element to indicate 3839 octets. A QMG STA that sets the Maximum MPDU Length in the QMG Capabilities element to indicate 7991 octets or 11 454 octets shall set the Maximum A-MSDU Length in the QMG Capabilities element to indicate 7935 octets.

Insert the subsequent paragraphs and note as follows:

The length of an A-MSDU transmitted in a QMG PPDU is limited by the maximum MPDU size supported by the recipient STA (see 9.12.5).

A QMG STA shall not transmit to a recipient QMG STA an A-MSDU that includes a number of MSDUs greater than the value indicated by the Max Number of MSDUs in A-MSDU field in the Extended Capabilities element received from the recipient STA.

## PPDU duration constraint

Insert the following sentence after the second paragraph in 9.14:

A QMG STA shall not transmit a PPDU that has a duration (as determined by the PHY-TXTIME.confirm primitive defined in 6.5.8 (PLME-TXTIME.confirm)) that is greater than aPPDUMaxTime.

## STBC operation

Insert the following sentence to the first paragraph in 9.15:

A STA that has not set the Tx STBC subfield to 1 in the QMG Capabilities element may shall not transmit QMG PPDUs with a TXVECTOR parameter STBC set to a nonzero value.

Insert the following sentence to the second paragraph in 9.15:

A STA shall not send a QMG PPDU with the TXVECTOR parameter STBC set to a nonzero value to a recipient STA unless the recipient STA has indicated in the Rx STBC field of its QMG Capabilities element that it supports the reception of PPDUs using STBC with a number of spatial streams equal to or greater than the number of spatial streams in the QMG PPDU.

## Operation across regulatory domains

Insert a new subclause 9.17a after subclause 9.17 as follows:

9.17a Partial AID, Uplink Indication and COLOR in QMG PPDUs

The QMG partial AID is a non-unique identifier of a QMG STA as defined in Table 9-5a. The partial AID is carried in the TXVECTOR parameter PARTIAL_AID of a QMG PPDU and is limited to 9 bits.

A STA transmitting a QMG PPDU carrying one or more group addressed MPDUs shall set the TXVECTOR parameters PARTIAL_AID to 0.

A STA transmitting a QMG PPDU carrying one or more individually addressed MPDUs shall set the TXVECTOR parameter PARTIAL_AID as shown in Table 9-5a.

Table 9-5a—Settings for the TXVECTOR parameter PARTIAL_AID

| Condition | PARTIAL_AID |
| --- | --- |
| A frame that is not a Control frame that is addressed to an AP. |  |
| A frame that is not a Control frame that is addressed to a Mesh STA. |  |
| A frame that is not a Control frame that is sent by an AP and addressed to a STA associated with that AP or sent by a DLS or TDLS STA in a direct path to a DLS or TDLS peer STA. | where is a bitwise exclusive OR operation mod X indicates the X-modulo operation dec(A[b:c]) is the cast to decimal operator where b is scaled by and c by |
| Otherwise (see NOTE) | 0 |
| NOTE—The last row covers the following cases: — A PPDU sent to an IBSS STA — A PPDU sent by an AP to a non associated STA — Any other condition not explicitly listed elsewhere in the table |  |

In Table 9-5a:

- AID[b:c] represents bits b to c inclusive of the AID of the recipient STA with bit 0 being the first transmitted.

- BSSID[b:c] represents bits b to c inclusive of the BSSID, with bit 0 being the Individual/Group bit. In this representation, the Individual/Group bit is BSSID[0] and BSSID[47] is the last transmitted bit.

- RA[b:c] represents bits b to c inclusive of the RA field, with bit 0 being the Individual/Group bit. In this representation, the Individual/Group bit is RA[0] and RA[47] is the last transmitted bit.

A STA that transmits a QMG PPDU to a DLS or TDLS peer STA obtains the AID for the peer STA from the DLS Setup Request, DLS Setup Response, TDLS Setup Request or TDLS Setup Response frame.

A QMG AP should not assign to a QMG STA, an AID that results in the PARTIAL_AID value, as computed using Equation 9-7a, being equal to either

0 oror

where OBSSID is the BSSID of a BSS that is not the BSS of which the AP is a member and for which the AP might be heard by the STA being assigned the AID.

As an example of the PARTIAL_AID setting, consider the case of a BSS with BSSID 00-21-6A-AC-53-52 that has as a member a non-AP QMG STA assigned AID 5. In QMG PPDUs sent to an AP, the PARTIAL_AID is set to 165. In QMG PPDUs sent by the AP to the non-AP STA associated with that AP, the PARTIAL_AID is set to 37.

NOTE 1—In the example above, BSSID[47:40] = 0x52, that is, BSSID[47] = 0, BSSID[46] = 1, BSSID[45] = 0, BSSID[44] = 1, etc.

NOTE 2—As described in IEEE Std 802-2001, the use of hyphens for the BSSID indicates hexadecimal representation rather than bit-reversed representation such that the leftmost octet in the representation is the first transmitted octet for 802.11. Using the BSSID vector numbering described above, the BSSID in IEEE Std 802-2001 hexadecimal representation is BSSID[7:0]-BSSID[15:8]- BSSID[23:16]-BSSID[31:24]- BSSID[39:32]-BSSID[47:40].

A STA transmitting a QMG PPDU that is addressed to an AP shall set the TXVECTOR parameter UPLINK_INDICATION to 1. The UPLINK_INDICATION parameter shall be set to 0 for all other cases.

The TXVECTOR parameter COLOR is used to assist a receiving STA in identifying the BSS from which a reception originates so that the receiving STA might increase spatial reuse and reduce power consumption by terminating the reception process in the case when the reception is not from the BSS with which the STA is associated. A STA transmitting a QMG PPDU that is addressed to an AP need not include the TXVECTOR parameter COLOR in the TXVECTOR. A STA transmitting a QMG PPDU that is not an NDP frame and that is sent by a DLS or TDLS STA in a direct path to a DLS or TDLS peer STA shall set the TXVECTOR parameter COLOR to the value of the COLOR parameter, if present, from the RXVECTOR of the most recently received frame from its associated AP or from the DO of the IBSS of which it is a member that contained a COLOR parameter. An AP transmitting a QMG PPDU shall set the TXVECTOR parameter COLOR to a value of its choosing within the range 0 to 7 and shall maintain that value for the duration of the existence of the BSS. The AP which is a member of a Multiple BSSID Set shall set the TXVECTOR parameter COLOR for each different BSSID(i) to a same value.

An AP shall include the value within the range 0 to 7 that it is using for the TXVECTOR parameter COLOR in the COLOR field of the QMG Capabilities Info field of the QMG Capabilities element in all frames that contain that element. The COLOR field of the QMG Capabilities Info field of the QMG Capabilities element in all frames transmitted from a non-AP STA is reserved.

## HCF

### HCF contention based channel access (EDCA)

#### EDCA TXOPs

Change the last paragraph of 9.19.2.2 as follows:

A STA shall save the TXOP holder address for the BSS in which it is associated, which is the MAC address from the Address 2 field of the frame that initiated a frame exchange sequence except when this is a CTS frame, in which case the TXOP holder address is the Address 1 field. If the TXOP holder address is obtained from a control frame, a VHT STA shall save the non-bandwidth signaling TA value obtained from the Address 2 field. If a non-VHT STA receives an RTS frame is received with the RA address matching the MAC address of the STA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS, without regard for, and without resetting its NAV. If a VHT STA receives an RTS frame with the RA address matching the MAC address of the STA and the non-bandwidth signaling TA value obtained from the Address 2 field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS, without regard for, and without resetting its NAV. If a QMG STA receives an RTS frame with the RA address matching the MAC address of the STA and the TA value obtained from the Address 2 field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS, without regarding for, and without resetting its NAV. When a STA receives a frame addressed to it that requires an immediate response, except in the case for RTS, it shall transmit the response independent of its NAV. The saved TXOP holder address shall be cleared when the NAV is reset or when the NAV counts down to 0.

#### Obtaining an EDCA TXOP

Change the first paragraph of 9.19.2.3 as follows:

When a STA and the BSS, of which the STA is a member, both support multiple channel widths, an EDCA TXOP is obtained based solely on activity of the primary channel. “Idle medium” in this subclause means “idle primary channel.” Likewise “busy medium” means “busy primary channel.” Once an EDCA TXOP has been obtained according to this subclause, further constraints defined in 10.15.9 and 9.19.2.8 might limit the width of transmission during the TXOP or deny the channel access, based on the state of CCA on secondary channel, secondary 40 MHz channel, or secondary 80 MHz channel. For QMG STA, further constraints defined in 9.19.2.9 might limit the width of transmission during the TXOP or deny the channel access, based on the state of CCA on secondary 540MHz channel.

#### Multiple frame transmission in an EDCA TXOP

Change the first paragraph of 9.19.2.4 as follows:

Multiple frames may be transmitted in an EDCA TXOP that was acquired following the rules in 9.19.2.3 if there is more than one frame pending in the primary AC for which the channel has been acquired. However, those frames that are pending in other ACs shall not be transmitted in this EDCA TXOP except when sent in a VHT MU PPDU with TXVECTOR parameter NUM_USERS> 1 and if allowed by the rules in 9.19.2.3a. If a TXOP holder has in its transmit queue an additional frame of the primary AC as the the duration of transmission of that frame plus any expected acknowledgment for that frame is less than the remaining TXNAV timer value, then the STA TXOP holder may commence transmission of that frame a SIFS (or RIFS, if the conditions defined in 9.3.2.3.2 are met) after the completion of the immediately preceding frame exchange sequence, subject to the TXOP limit restriction as described in 9.19.2.2. A STA shall not commence the transmission of an RTS with a bandwidth signaling TA until at least PIFS time after the immediately preceding frame exchange sequence. A QMG STA shall not commence the transmission of an RTS until at least PIFS time after the immediately preceding frame exchange sequence. An HT STA that is a TXOP holder may transmit multiple MPDUs of the same AC within an A-MPDU as long as the duration of transmission of the A-MPDU plus any expected BlockAck response is less than the remaining TXNAV timer value.

NOTE 1—PIFS is used by a VHT STA to perform CCA in the secondary 20 MHz, 40 MHz, and 80 MHz channels before receiving RTS

NOTE 2—PIFS is used by a QMG STA to perform CCA in the secondary 540 MHz channels before receiving RTS

Insert the following subclause after 9.19.2.8

#### EDCA channel access in a QMG BSS

If the MAC receives a PHY-CCA.indication primitive with the channel-list parameter present, the channels considered idle are defined in Table 9.8b

Table 9.8b Channels indicated idle by the channel-list parameter

| PHY-CCA.indication channel-list element | Idle channel |
| --- | --- |
| primary | None |
| secondary | Primary 540MHz |

In the following description, the CCA is sampled according to the timing relationships defined in 9.3.7. Slot boundaries are determined solely by activity on the primary channel. “Channel idle for an interval of PIFS” means that whenever CCA is sampled during the period of PIFS that endsat the start of transmission, the CCA for that channel was determined to be idle.

If a STA is permitted to begin a TXOP (as defined in 9.19.2.3) and the STA has at least one MSDU pending for transmission for the AC of the permitted TXOP, the STA shall perform exactly one of the following steps:

- Transmit a 1080MHz mask PPDU if the secondary 540MHz channel is idle during an interval of PIFS immediately preceding the start of the TXOP.

- Transmit a 540MHz mask PPDU on the primary 540 MHz channel.

- Restart the channel access attempt by invoking the backoff procedure as specified in 9.19.2 as though the medium is busy on the primary channel as indicated by either physical or virtual CS and the backoff timer has a value of 0.

NOTE 1—In the case of rule c), the STA selects a new random number using the current value of CW[AC], and the retry counters are not updated (as described in 9.19.2.5; backoff procedure invoked for event a)).

NOTE 2—For QMG STA, an EDCA TXOP is obtained based on activity on the primary channel (see9.19.2.3). The width of transmission isdetermined by the CCA status of the non-primary channels during the PIFS interval before transmission (see 9.19.2.4).

## Block Acknowledgment (Block Ack)

### QMG delayed Block Ack extensions

## Reverse Direction Protocol

## PSMP operation

## Sounding PPDUs

Insert the following paragraph at the beginning of 9.27:

The behavior described in this subclause is specific to the use of QMG Control field.

Change the now third paragraph through the beginning of the fifth paragraph of 9.27 as follows:

A STA transmits sounding PPDUs when it operates in the following roles:

- MFB requester (see 9.28.2)

- QMG beamformer involved in explicit transmit beamforming (see 9.29.3)

A STA receives sounding PPDUs when it operates in the following roles:

- MFB responder (see 9.28.2)

- QMG beamformee involved in explicit transmit beamforming (see 9.29.3)

When transmitting a sounding PPDU, the transmitting STA follows the rules stated below to determine the maximum number of space-time streams for which channel coefficients can be simultaneously estimated:

- When transmitting a sounding PPDU that Contains a +QMGC frame with the MRQ subfield equal to 1, or

- Is sent as a response to a +QMGC frame with the TRQ field equal to 1, or

Is sent by a QMG beamformer involved in explicit transmit beamforming

## Link adaptation

### Introduction

In many environments the channel is changing with time as the stations move or with changes in the environment itself, such as the movement of objects in the environment, or changes in external noise sources. Link adaptation might improve performance by closely tracking and adapting the channel changes. Link adaptation can be supported using the QMG Control field in OFDM transmission. Accordingly, link adaptation can be supported using the QMG Link Measurement in SC transmission.

To fully exploit MIMO channel variations and transmit beamforming on a MIMO link in OFDM transmission, a STA can request that another STA provide MIMO channel sounding and MFB.

Link adaptation may be supported by immediate response or delayed response as described below. Unsolicited MFB is also possible.

- Immediate: An immediate response occurs when the MFB responder transmits the response in the TXOP obtained by the TXOP holder. This approach allows the MFB requester to obtain the benefit of link adaptation within the same TXOP.

- Delayed: A delayed response occurs when the MFB responder transmits the response in the role of a TXOP holder in response to an MRQ in a previous TXOP obtained by the MFB requester.

- Unsolicited: An unsolicited response occurs when a STA sends MFB independent of any preceding MRQ.

Link adaptation in SC transmission is achieved with information exchange of link measurement request frame and link measurement report frame. The QMG TPC and the QMG link margin may be supported in the mechanism in SC transmission.

### Link adaptation using the QMG Control field

The behavior described in this subclause is specific to the QMG Control field.

A STA that supports link adaptation using the QMG Control field shall set the QMG Link Adaptation Capable subfield in the QMG Capabilities Info field of the QMG Capabilities element to Unsolicited or Both, depending on its specific link adaptation feedback capability. A STA shall not send an MRQ to STAs that have not set QMG Link Adaptation Capable subfield to Both in the QMG Capabilities Info field of the QMG Capabilities element. A STA whose most recently transmitted MCS Feedback field of the QMG Extended capabilities field of the QMG Capabilities element is equal to Unsolicited or Both may transmit unsolicited MFB in any frame that contains a QMG control field.

The MFB requester may set the MRQ field to 1 in the QMG Control field of a frame to request a STA to provide link adaptation feedback. In each request the MFB requester shall set the MSI/STBC field to a value in the ranges 0 to 6, 0 to 2, or 0 to 3, depending on the settings in the Unsolicited MFB and STBC fields (see 8.2.4.6. The choice of MSI value is implementation dependent.

NOTE—The MFB requester might use the MSI subfield as an MRQ sequence number, or it might implement any other encoding of the field.

The appearance of more than one instance of a QMG Control field with the MRQ field equal to 1 within a single PPDU shall be interpreted by the receiver as a single request for link adaptation feedback.

An MFB requester shall transmit +QMGC frames with the MRQ subfield equal to 1 in one of the following ways in OFDM transmission:

—  Within a sounding PPDU, or

—  With the NDP Announcement subfield in the +QMGC frame set to 1 and following the +QMGC frame by a QMG NDP transmission

In OFDM transmission the number of OCEFs sent in the sounding PPDU or in the NDP is determined by the total number of spatial dimensions to be sounded, including any extra spatial dimensions beyond those used by the data portion of the frame.

An MFB responder that has set the QMG Link Adaptation Capable subfield to Both in the QMG Capabilities Info field of the QMG Capabilities element shall support both of the following in OFDM transmission:

—  Computation and feedback of the MFB estimate on the receipt of an MFB request (MRQ equal to 1 in the QMG Control field) in the sounding PPDU

—  Computation and feedback of the MFB estimate on the receipt of an MFB request (MRQ equal to 1 in QMG Control field) in the +QMGC frame with the NDP Announcement subfield set to 1 and the receipt of the subsequent QMG NDP(see 9.31)

On receipt of a QMG Control field with the MRQ subfield equal to 1, an MFB responder computes the QMG -MCS, NUM_STS, and SNR estimates based on the PPDU carrying the MRQ or, in the case of a +QMGC frame with the NDP Announcement subfield set to 1 carrying the MRQ, based on the subsequent QMG NDP. The MFB responder labels the result of this computation with the MSI value from the QMG Control field in the received frame carrying the MRQ. The MFB responder may include the received MSI value in the MFSI field of the corresponding response frame. In the case of a delayed response, this allows the MFB requester to associate the MFB with the soliciting MRQ.

An MFB responder that sends a solicited MFB shall set the Unsolicited MFB subfield in QMG Control field to 0.

The MFB responder may send a solicited response frame with any of the following combinations of QMG-MCS, NUM_STS, and MFSI:

—QMG -MCS = 31, NUM_STS = 3 in the MFB subfield, MFSI = 7: no information is provided for the immediately preceding request or for any other pending request. This combination is used when the responder is required to include a QMG Control field due to other protocols that use this field (e.g., the Reverse Direction Protocol) and when no MFB is available. It has no effect on the status of any pending MRQ.

—QMG -MCS = 31, NUM_STS = 3 in the MFB subfield, MFSI in the range 0 to 6: the responder is not now providing, and will never provide, feedback for the request that had the MSI value that matches the MFSI value.

—QMG -MCS = valid value, NUM_STS = valid value in the MFB subfield, MFSI in the range 0 to 6: the responder is providing feedback for the request that had the MSI value that matches the MFSI value.

An MFB responder that discards or abandons the MFB estimates computed in response to an MRQ may indicate that it has done so by setting the QMG -MCS to 31 and NUM_STS to 3 in the MFB subfield in the next frame addressed to the MFB requester that includes the QMG Control field. The value of the MFSI is set to the value of the MSI/STBC subfield of the frame that contains an MRQ for which the computation was abandoned, regardless of whether the MSI/STBC subfield contains an MSI or a Compressed MSI and STBC Indication subfields.

The STA receiving MFB may use the received MFB to compute the appropriate QMG -MCS, SNR, BW, and NUM_STS.

When computing the MCS estimate for an MFB requester whose Tx MCS Set Defined field is set to 1, the number of spatial streams corresponding to the recommended MCS shall not exceed the limit indicated by the Tx Maximum Number Spatial Streams Supported field.

Hardware and buffer capability may limit the number of MCS estimate computations that a MFB responder is capable of computing simultaneously. When a new MRQ is received either from a different MFB requester or from the same MFB requester with a different MSI value, and the MFB responder is not able to complete the computation for MRQ, the MFB responder may either discard the new request or may abandon an existing request and initiate an MCS estimate computation for the new MRQ.

A STA sending unsolicited MFB feedback using the QMG Control field shall set the Unsolicited MFB subfield to 1.

Unsolicited QMG -MCS, NUM_STS, BW, and SNR estimates reported in the MFB subfield of a QMG Control field sent by a STA are computed based on the most recent PPDU received by the STA that matches the description indicated by the STBC Indication and FB Tx Type fields in the same QMG Control field.

In an unsolicited MFB response the STBC Indication, FB Tx Type, and BW fields are set according to the RXVECTOR parameters of the received PPDU from which the QMG-MCS, SNR, BW, and NUM_STS are estimated, as follows:

—The STBC Indication field is set to 1 if the parameter STBC is equal to 1 and set to 0 if the STBC parameter is equal to 0.

—The FB TX Type field is set to 1 if the parameter BEAMFORMED is equal to 1 and set to 0 if equal to 0.

—The BW field shall indicate a bandwidth equal to or less than the bandwidth indicated by theparameter CH_BANDWIDTH.

In an MFB response solicited by an MRQ that was not carried in the +QMGC frame with the NDP Announcement subfield set to 1, the MFB is computed based on RXVECTOR parameters CH_BANDWIDTH, NUM_STS, BEAMFORMED, and STBC of the received PPDU that carried the MRQ and might additionally be based on other factors that are not part of the RXVECTOR. The NUM_STS subfield of the MFB subfield of QMG Control field shall be set to an equal or smaller value than the RXVECTOR parameter NUM_STS of the received PPDU that triggered the MRQ.

If the MFB is in the same PPDU as a QMG Noncompressed Beamforming frame or a Compressed Beamforming frame, the MFB responder should estimate the recommended MFB under the assumption that the MFB requester uses the steering matrices contained therein.

If the MFB is in the same MPDU as a QMG Compressed Beamforming frame, the MFB responder shall estimate the recommended MFB under the assumption that the beamformer will use the steering matrices contained therein. In this case the value of the NUM_STS field in the MFB subfield of the QMG Control field shall be the same as the value of the Nc Index field in the QMG MIMO Control field of the QMG Compressed Beamforming frame and, if the MFB is unsolicited, the FB Tx Type shall be set to 0. Additionally, MFB estimate shall be based on the bandwidth indicated by the Channel Width subfield of the QMG MIMO Control field of the QMG Compressed Beamforming frame. In this case the SNR and BW subfields are reserved and set to 0.

If an unsolicited MFB is not in the same MPDU as a QMG Compressed Beamforming frame, the NUM_STS subfield of the MFB subfield of the QMG Control field shall be set to an equal or smaller value than the RXVECTOR parameter NUM_STS of the received PPDU from which the MFB parameters are estimated.

After the MFB estimate computation is completed, the MFB responder should include the MFB in the MFB field in the next transmission of a frame addressed to the MFB requester that includes A QMG Control field. When the MFB requester sets the MRQ subfield to 1 and sets the MSI subfield to a value that matches the MSI subfield value of a previous request for which the responder has not yet provided feedback, the responder shall discard or abandon the computation for the MRQ that corresponds to the previous use of that MSI subfield value.

A STA may respond immediately to a current request for MFB with a frame containing an MFSI field value and MFB field value that correspond to a request that precedes the current request.

NOTE 1—If a QMG STA includes the QMG Control field in the initial frame of an immediate response exchange and the responding QMG STA includes the QMG Control field in the immediate response frame, the immediate response exchange effectively permits the exchange of QMG Control field elements.

NOTE 2—If an MRQ is included in the last PPDU in a TXOP and there is not enough time for a response, the recipient might transmit the response MFB in a subsequent TXOP.

NOTE 3—Bidirectional request/responses are supported. In this case, a STA acts as the MFB requester for one direction of a duplex link and a MFB responder for the other direction and transmits both MRQ and MFB in the same QMG data frame.

If a beamformer transmits a PPDU with the TXVECTOR EXPANSION_MAT_TYPE set to either COMPRESSED_SV or NON_COMPRESSED_SV, it should use the recommended MCS associated with those matrices reported in a Noncompressed Beamforming frame or a Compressed Beamforming frame.

### Link adaptation using the QMG Link Measurement

#### General

A STA may transmit a Link Measurement Request frame to request a STA indicated in the RA field of the frame to respond with a Link Measurement Report frame (8.5.7.5 Link Measurement Report frame format) in SC transmission. If the Link Measurement Request frame is sent within a PPDU defined in clause 21, the Link Measurement Report frame shall contain the QMG Link Margin element. The requesting STA may use values of the MCS, of the SNR and of the Link Margin to transmit frames to the STA indicated in the RA field of the Link Measurement Request frame.

The requesting STA may aggregate a Link Measurement Request frame in an A-MPDU as defined in Table 8-412 (A-MPDU contents in the data enabled immediate response context) and Table 8-413 (A-MPDU contents MPDUs in the control response context).

If the Dialog Token field in the Link Measurement Request frame is equal to a non-zero value, the responding STA shall perform the measurement on the next frame received from the requesting STA and shall send back a Link Measurement Report frame corresponding to the received frame.

The responding STA may aggregate a Link Measurement Report frame in a A-MPDU as defined in in Table 8-284 (A-MPDU contents in the data enabled immediate response context) and Table 8-287 (A-MPDU contents MPDUs in the control response context).

A QMG STA with MAC address that is equal to the value of the Link Measurement Request frame RA field shall transmit a Link Measurement Report frame addressed to the requesting STA. The RA field of the Link Measurement Report frame shall be equal to the TA field of the Link Measurement Request frame.

If the Dialog Token field in the Link Measurement Report frame is equal to the non-zero Dialog Token field of the Link Measurement Request frame, the MCS, SNR and Link Margin fields of the Link Measurement Report frame shall be computed using the measurements of the PPDU that is the subsequent frame following the Link Measurement Request frame.

If the Dialog Token field in the Link Measurement Request frame is equal to 0, the responding STA may set the MCS field in the Link Measurement Report frame to the MCS value computed based on any of the received frames from the requesting STA.

The SNR field and Link Margin field in the Link Measurement Report frame shall indicate the corresponding measurements based on the reception of the PPDU that was used to generate the MCS feedback contained in the same Link Measurement Report frame.

The Link Measurement Request and Report frames can be used to obtain Link Margin information, which can be used to determine appropriate action by the requesting STA (e.g., change MCS or control transmit power or initiate FST).

A STA may send an unsolicited Link Measurement Report frame with the Dialog Token field set to 0.

#### QMG TPC

A QMG STA that receives a Link Measurement Report frame containing a QMG Link Margin element that indicates Increase or Decrease Transmit power behaves according to the following rules:

- If the QMG STA intends to implement the recommendation indicated in the Activity field of the Link Measurement Report, it shall implement the change and send a Link Measurement Report containing a QMG Link Adaptation Acknowledgement element not later than 2×aPPDUMaxTime after it acknowledged the reception of the Link Measurement Report. The Activity field of the QMG Link Adaptation Acknowledgement element shall be set to the value of the Activity field in the received QMG Link Margin Sub-element.

- If the QMG STA does not implement the recommendation indicated in the Activity field of the Link Measurement Report, it may send the Link Measurement report containing a QMG Link Adaptation Acknowledgement element no later than 2×aPPDUMaxTime after it acknowledges the reception of the Link Measurement Report. The Activity field of the QMG Link Adaptation Acknowledgement element shall be set to 0, indication the STA prefers to not change transmit power.

A QMG STA shall not include the QMG Link Adaptation Acknowledgement element in a Link Measurement Report unless it is in response to a Link Measurement Report with Activity field set to increase or decrease transmit power.

#### QMG Fast Link Adaptation

A STA supports fast link adaptation if the Fast Link Adaptation field in the STA’s QMG Capabilities info field is 1. Otherwise, the STA does not support fast link adaptation. A STA that supports fast link adaptation shall not use fast link adaptation with a peer STA that does not support fast link adaptation.

A STA that supports fast link adaptation shall support the reverse direction protocol (see 9.25). The STA that transmits a Link Measurement Request frame as part of fast link adaptation shall be the RD initiator and the STA that responds with a Link Measurement Report frame shall be the RD responder. Transmission of Link Measurement Request, Link Measurement Report and the frames defined below shall follow the rules of the reverse direction protocol.

A STA initiates fast link adaptation by transmitting a Link Measurement Request frame of subtype

Action No Ack and that has the Dialog Token field set to 0. The PPDU containing the frame shall have the AGGREGATION parameter in the TXVECTOR set to AGGREGATED, shall not contain any other frame that requires immediate response and shall have a duration (as determined by the PHY-TXTIME.confirm primitive defined in 6.5.8) that is greater than 5.27μs.

NOTE–The PPDUs have the AGGREGATION parameter in the TXVECTOR set to AGGREGATED to allow padding of the PSDUs with MPDU delimiters of size 0, therefore meeting the transmission duration requirement.

A STA supporting fast link adaptation and that receives a Link Measurement Request frame of subtype Action No Ack, with the Dialog Token field equal to 0 and contained in a PPDU with the AGGREGATION parameter in the RXVECTOR equal to AGGREGATED shall respond with a Link Measurement Report frame in no longer than BRPIFS from the reception of the Link Measurement Request frame. The TPC Report element, QMG Link Margin element and other fields transmitted in the Link Measurement Report frame shall reflect measurements on the PPDU that contained the last received Link Measurement Request frame from the initiating STA.

The STA responding with the Link Measurement Report frame shall keep the IFS not longer than SIFS by transmitting PPDUs that do not contain frames requiring immediate response and that have a duration that is greater than 5.27μs. All transmitted PPDUs should use the same MCS and the same transmit power.

The transmitted Link Measurement Report frame shall be of subtype Action No Ack, shall be sent using MCS 1 and shall be sent within a PPDU with the AGGREGATION parameter in the TXVECTOR set to AGGREGATED. In addition, the PPDU shall not contain any frame that requires immediate response and shall have a duration that is greater than 5.27us.

If at least one of the conditions above for the transmission of the Link Measurement Report frame is not met, the STA may follow the rules in 9.37.1 General to respond to the received Link Measurement Request frame.

A STA that supports fast link adaptation and that receives a Link Measurement Report frame should respond with an unsolicited Link Measurement Report frame in no longer than BRPIFS from the reception of the Link Measurement Report frame. The TPC Report element, QMG Link Margin element and other fields transmitted in the unsolicited Link Measurement Report frame shall reflect measurements taken on one or more of the PPDUs received by the STA transmitting the unsolicited Link Measurement Report frame, starting with the received Link Measurement Report frame itself. If the unsolicited Link Measurement Report frame is transmitted longer than SIFS from the reception of the Link Measurement Report frame, the STA transmitting the unsolicited Link Measurement Report frame shall keep the IFS not longer than SIFS by transmitting one or more PPDUs before issuing the unsolicited Link Measurement Report frame.

## Transmit beamforming

### General steering matrix calculations

In order for a QMG beamformer to calculate an appropriate steering matrix for transmit spatial processing when transmitting to a specific QMG beamformee, the QMG beamformer needs to have an accurate estimate of the channel over which it is transmitting. The method of calculation for QMG beamformer based on explicit feedback is defined as follows:

When using explicit feedback, the QMG beamformee makes a direct estimate of the channel from training symbols sent to the QMG beamformee by the QMG beamformer. The QMG beamformee may prepare CSI or steering feedback based on an observation of these training symbols. The QMG beamformee quantizes the feedback and sends it to the QMG beamformer. The QMG beamformer can use the feedback as the basis for determining transmit steering vectors. See 9.29.3(Explicit feedback beamforming).

Insert the following paragraphs at the last of 9.29.1:

A QMG STA shall not transmit a PPDU with the TXVECTOR EXPANSION_MAT parameter present if dot11BeamFormingOptionActivated is false.

### Transmit beamforming with implicit feedback

### Explicit feedback beamforming

Change the third paragraph in 9.29.3 as follows:

The supported formats shall be advertised in the beamformee’s HT Capabilities element. The supported formats shall be advertised in the beamformee’s QMG Capabilities element.

Change the 5th paragraph in 9.29.3 as follows:

A beamformee’s responding capabilities shall be advertised in HT Capabilities elements contained in Beacon, Probe Request, Probe Response, Association Request, Association Response, Action, and Action No Ack frames that are transmitted by the beamformee. A beamformee’s responding capabilities shall be advertised in QMG Capabilities elements contained in Beacon, Probe Request, Probe Response, Association Request, Association Response, Action, and Action No Ack frames that are transmitted by the beamformee. Devices that are capable of acting as a beamformee shall advertise one of the following response capabilities in the Explicit Transmit Beamforming CSI Feedback subfield of the Transmit Beamforming Capability field:

- Immediate: The beamformee is capable of sending a feedback response a SIFS after receiving a sounding PPDU and/or is capable of sending a feedback response aggregated in a PPDU that contains a MAC response within the beamformer’s TXOP.

- Delayed: The beamformee is not capable of sending the feedback response within the beamformer’s TXOP, but it is capable of sending the feedback response in a TXOP that it obtains.

- Immediate and Delayed: The beamformee is capable of sending a feedback response a SIFS after receiving a sounding PPDU, sending a feedback response aggregated in a PPDU that contains a MAC response within the beamformer’s TXOP, or sending the feedback response in a TXOP that it obtains.

Insert the following paragraph after the 6th paragraph in 9.29.3:

The sounding frame types supported by the beamformee, staggered and/or NDP, are advertised in the QMG Capabilities element in frames that are transmitted by the beamformee.

Change the 7th paragraph in 9.29.3 as follows:

A STA that sets any of the Explicit Transmit Beamforming CSI Feedback Capable, Explicit Noncompressed Beamforming Feedback Capable, or Explicit Compressed Beamforming Feedback Capable fields to 1 shall transmit explicit feedback based on the receipt of a +HTC sounding PPDU in which the CSI/Steering subfield has a nonzero value. A STA that sets any of the Explicit Transmit Beamforming CSI Feedback Capable, Explicit Noncompressed Beamforming Feedback Capable, or Explicit Compressed Beamforming Feedback Capable fields to 1 shall transmit explicit feedback based on the receipt of a +QMGC sounding PPDU in which the CSI/Steering subfield has a nonzero value. This requirement is independent of the values of the Receive Staggered Sounding Capable and the Receive NDP Capable fields.

Insert the following paragraph after the 9th paragraph in 9.29.3:

A beamformer shall set the response type format indicated in the CSI/Steering subfield of the QMG Control field of any sounding frame excluding the NDP and of any PPDU with the NDP Sounding Announcement field equal to 1 to one of the nonzero values (CSI, Compressed Beamforming, or Noncompressed Beamforming) that corresponds to a type that is supported by the beamformee.

Insert the following paragraph after the 10th paragraph in 9.29.3:

The receipt of a PHY-RXSTART.indication primitive with the RXVECTOR SOUNDING parameter value equal to SOUNDING indicates a sounding packet. A non-NDP request for feedback is a sounding PPDU with a +QMGC MPDU that contains a nonzero value of the CSI/Steering subfield and that has the NDP Announcement subfield equal to 0.

Insert the following paragraph after the 11th paragraph in 9.29.3:

An NDP request for feedback is the combination of a +QMGC MPDU that contains a nonzero value of the CSI/Steering subfield and that has the NDP Announcement subfield equal to 1 and the NDP that follows.

Insert the following paragraph after the 12th paragraph in 9.29.3:

A beamformee that transmits a feedback frame in response to a sounding PPDU sent by a beamformer shall transmit a frame of the type (CSI, Compressed Beamforming, or Noncompressed Beamforming) indicated in the CSI/Steering subfield of the QMG Control field transmitted by the beamformer.

Insert the following paragraph after 13th paragraph in 9.29.3 as follows:

The Beamformee decides on any tone grouping to be used in the explicit Beamforming feedback. The value selected shall be a value supported by the Beamformer as indicated in the Minimal Grouping subfield of the Beamformer’s QMG Capabilities element.

Insert the following paragraph after the 14th paragraph in 9.29.3:

A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its QMG Capabilities element to either 2 or 3 shall transmit Explicit CSI feedback after SIFS or later in the beamformer’s TXOP as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-13. A beamformee that sets the Explicit Noncompressed Beamforming Feedback Capable field of its QMG Capabilities element to either 2 or 3 shall transmit Explicit Noncompressed Beamforming feedback after SIFS or later in the beamformer’s TXOP as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-13.

Insert the following paragraph after the 15th paragraph in 9.29.3 as follows:

A beamformee that sets the Explicit Compressed Feedback Capable field of its QMG Capabilities element to either 2 or 3 shall transmit Explicit Compressed Beamforming feedback after SIFS or later in the beamformer’s TXOP as a response to a non-NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-13

Insert the following paragraph after the 16th paragraph in 9.29.3 as follows:

A beamformee that sets the Explicit Transmit Beamforming CSI Feedback field of its QMG Capabilities element to either 2 or 3 shall transmit the Explicit CSI feedback after SIFS or later in the beamformer’s TXOP as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-14.

Insert the following paragraph after the 17th paragraph in 9.29.3 as follows:

A beamformee that sets the Explicit Noncompressed Beamforming Feedback Capable field of its QMG Capabilities element to either 2 or 3 shall transmit the Explicit Noncompressed Beamforming feedback after SIFS or later in the beamformer’s TXOP as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-14.

Insert the following paragraph after the 18th paragraph in 9.29.3 as follows:

A beamformee that sets the Explicit Compressed Beamforming Feedback Capable field of its QMG Capabilities element to either 2 or 3 shall transmit the Explicit Compressed Beamforming feedback after SIFS or later in the beamformer’s TXOP as a response to an NDP request for feedback in a frame that is appropriate for the current frame exchange sequence following Table 9-14.

Insert the following paragraph after the 19th paragraph in 9.29.3 as follows:

When the beamformee sets the Explicit Transmit Beamforming CSI Feedback field of its QMG Capabilities element to either 2 or 3 and the beamformer has transmitted an NDP or an non-NDP Explicit Beamforming CSI feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee until PIFS after transmitting the sounding request.

Insert the following paragraph after the 20th paragraph in 9.29.3 as follows:

When the beamformee sets the Explicit Noncompressed Beamforming Feedback Capable field of its QMG Capabilities element to either 2 or 3 and the beamformer has transmitted an NDP or an non-NDP Explicit Noncompressed Beamforming feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee until PIFS after the sounding request.

Insert the following paragraph after the 21th paragraph in 9.29.3 as follows:

When the beamformee sets the Explicit Compressed Beamforming Feedback Capable field of its QMG Capabilities element to either 2 or 3 and the beamformer has transmitted an NDP or an non-NDP Explicit Compressed Beamforming feedback request in a frame that does not require immediate control response, the beamformer shall not transmit the next packet to the beamformee until PIFS after transmitting the sounding request.

Insert the following paragraph after the 23th paragraph in 9.29.3 as follows:

NOTE—Error recovery in a TXOP is not affected by sounding. A beamformer that is a TXOP holder and that fails to receive an expected response to a sounding PPDU can continue transmission as specified in 9.19.2.4. A beamformee transmitting a feedback response after SIFS or later in the beamformer’s TXOP shall use an Action No Ack frame or an Action No Ack +QMGC frame (defined in 8.3.3.14).

Insert the following paragraph after the 24th paragraph in 9.29.3 as follows:

A beamformee transmitting delayed feedback response shall use an Action frame or an Action +QMGC frame to send this information within a separate TXOP.

## Antenna selection (ASEL)

## Null data packet (NDP) sounding

### NDP rules

Change 9.31.1 as follows:

Sounding may be accomplished using QMG NDP, as described in 26.6.3.5.12 (QMG Preamble format for sounding PPDUs). The MAC rules associated with sounding using QMG NDP are described in9.28.1 and 9.28.2.

A QMG STA that has set the Receive NDP Capable field of its QMG Capabilities element to 1 during association processes a QMG NDP as a sounding packet if the destination of the sounding packet is determined to match itself and if the source of the sounding packet can be ascertained.

An RXVECTOR LENGTH parameter equal to 0 indicates that the PPDU is a QMG PPDU.

A STA that is a TXOP holder or an RD responder shall not set both the QMG NDP Announcement and RDG/More PPDU subfields to 1 simultaneously.

An NDP sequence contains at least one QMG NDP and at least one PPDU that is not an NDP. Only one PPDU in the NDP sequence may contain a QMG NDP announcement. An NDP sequence begins with a QMG NDP announcement. The NDP sequence ends at the end of the transmission of the last QMG NDP that is announced by the QMG NDP announcement. A STA that transmits the first PPDU of an NDP sequence is the NDP sequence owner. In the NDP sequence, only PPDUs carrying QMG NDP and PPDUs carrying single MPDU Control frames may follow the NDP sequence’s starting PPDU.

A STA shall transmit only one QMG NDP per NDP announcement. Each PPDU in an NDP sequence shall start a SIFS after end of the previous PPDU.

A CTS frame that is a +QMGC frame shall not contain the QMG NDP Announcement subfield set to 1.

NOTE 1—A CTS frame cannot be used for QMG NDP announcement: if the CTS frame is a response to an RTS frame, the optional NAV reset timeout that starts at the end of the RTS frame does not include the additional QMG NDP and SIFS. Also, if the CTS were the first frame of an NDP sequence, it would not be possible to determine the destination address of the QMG NDP.

A STA shall transmit a QMG NDP as follows:

- A SIFS after sending a PPDU that is a QMG NDP announcement and that does not contain an MPDU that requires an immediate response.

- A SIFS after successfully receiving a correctly formed and addressed immediate response to a PPDU that is a QMG NDP announcement and that contains an MPDU that requires an immediate response.

This rule enables the NDP receiver to know that it will receive a QMG NDP and can determine the source and destination of the QMG NDP. It enables the receiver and transmitter to know when the immediate response and QMG NDP will be transmitted relative to the frame containing the QMG NDP announcement indication.

A STA that has transmitted a QMG NDP announcement in a frame that requires an immediate response and that does not receive the expected response shall terminate the NDP sequence at that point (i.e., the STA does not transmit a QMG NDP in the current NDP sequence).

Feedback information generated from the reception of a QMG NDP is transmitted using any of the feedback rules and signaling as appropriate, e.g., immediate or delayed.

### Transmission of a QMG NDP

A STA that transmits a QMG NDP shall set the LENGTH, SOUNDING, NSS and STBC parameters of the TXVECTOR as specified in this subclause.

- LENGTH shall be set to 0.

—SOUNDING shall be set to SOUNDING.

—STBC all shall be set to 0.

— NSS indicates two or more spatial streams.

The number of spatial streams sounded is indicated by the NSS parameter of the TXVECTOR and shall not exceed the limit indicated by the Channel Estimation Capability field in the Transmit Beamforming Capabilities field transmitted by the STA that is the intended receiver of the QMG NDP.

A STA shall not transmit a QMG NDP announcement with an RA corresponding to another STA unless it has received a QMG Capabilities element from the destination STA in which the Receive NDP Capable field is equal to 1.

The destination of a QMG NDP in the NDP sequence is equal to the RA of the immediately preceding MPDU within QMG NDP announcement.

The source of a QMG NDP in the NDP sequence is equal to the TA of the immediately preceding MPDU within QMG NDP announcement.

## DMG or QMG AP or PCP Cluster

### General

Change the first paragraph of 9.37.1 as follows:

An AP or PCP may use the AP or PCP clustering mechanism to improve spatial sharing and interference mitigation with other co-channel DMG or QMG BSSs. There are two types of clustering:

Change the second paragraph of 9.37.1 as follows:

AP or PCP clustering allows an AP or PCP that is a member of a cluster to schedule transmissions in nonoverlapping time periods with respect to other members of the same cluster since the AP or PCP can receive DMG or QMG Beacon and Announce frames containing the Extended Schedule element of other APs and PCPs that are members of the cluster and can also receive the Extended Schedule element from another BSS through associated non-AP and non-PCP STAs (9.37.4 (Cluster report and re-scheduling)).

Change the third paragraph of 9.37.1 as follows:

A STA is decentralized AP or PCP clustering capable if it sets the Decentralized AP or PCP Clustering field to 1 within the DMG or QMG AP or PCP Capability Information field in the DMG or QMG Capabilities element. A STA is centralized AP or PCP clustering capable if it sets the Centralized AP or PCP Clustering field to 1 within the DMG or QMG AP or PCP Capability Information field in the DMG or QMG Capabilities element. The AP or PCP employs the Clustering Control field and ECAPC Policy Enforced field defined in 8.3.4.2 (DMG Beacon) to configure the use of AP or PCP Clustering. A decentralized AP or PCP clustering capable or centralized AP or PCP clustering capable AP or PCP that transmits the Clustering Control field is decentralized clustering enabled or centralized clustering enabled, respectively.

Change the sixth paragraph of 9.37.1 as follows:

Each AP or PCP that is a member of an AP or PCP cluster shall include the Clustering Control field in each DMG or QMG Beacon frame it transmits.

Change the ninth paragraph of 9.37.1 as follows:

An AP or PCP that is a member of an AP or PCP cluster shall transmit its DMG or QMG Beacon frame during one of the Beacon SPs as specified in 9.37.2 (Cluster formation).

### Cluster formation

#### Decentralized AP or PCP cluster formation

Change the first paragraph of 9.37.2.1 as follows:

A clustering enabled AP or PCP starts a decentralized AP or PCP cluster by becoming an S-AP or S-PCP, subject to the absence of existing clusters as described below and in 9.37.2.2 (Centralized AP or PCP cluster formation). A decentralized clustering enabled AP or PCP becomes an S-AP or S-PCP of a decentralized AP or PCP cluster by transmitting a DMG or QMG Beacon frame at least once every aMinBTIPeriod beacon intervals that has the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 0 and that includes a Clustering Control field with the Beacon SP duration subfield set to the value of dot11BeaconSPDuration, the Cluster ID subfield set to the S-AP or S-PCP MAC address, and the Cluster Member Role subfield set to the value for an S-AP or S-PCP. The value of ClusterMaxMem subfield shall be chosen to keep the result of (beacon interval length/ClusterMaxMem) as an integer number of microseconds.

Change the second paragraph of 9.37.2.1 as follows:

A decentralized clustering enabled AP or PCP that receives a DMG or QMG Beacon frame with the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 0 from an S-AP or S-PCP on the channel the AP or PCP selects to establish a BSS shall monitor the channel for DMG or QMG Beacon frame transmissions during each Beacon SP for an interval of length at least aMinChannelTime. A Beacon SP is empty if no DMG or QMG Beacon frame is received during the Beacon SP over an interval of length aMinChannelTime. The AP or PCP shall not become a member of the cluster if no Beacon SP is determined to be empty during aMinChannelTime, in which case, subject to the requirements described in 9.37.2.2 (Centralized AP or PCP cluster formation), then the AP or PCP may become the S-AP or S-PCP of a new cluster, or may cease its activity on this channel and may attempt operation on a different channel.

Change the third paragraph of 9.37.2.1 as follows:

A decentralized clustering enabled AP or PCP that operates its BSS on a channel on which it discovered an S-AP or S-PCP within a decentralized AP or PCP cluster and at least one empty Beacon SP shall transmit its DMG or QMG Beacon frame during an empty Beacon SP. By transmitting its DMG or QMG Beacon frame during an empty Beacon SP and by setting the clustering control field appropriately as described in 8.3.4.2 (DMG Beacon), the AP or PCP becomes a member AP or member PCP.

Change the fifth paragraph of 9.37.2.1 as follows:

The member AP or member PCP shall transmit its DMG or QMG Beacon frame with the ECAPC Policy Enforced field set to 0, the Beacon SP duration subfield set to the value of the Beacon SP duration subfield contained in the S-AP or S-PCP DMG or QMG Beacon, the Cluster ID subfield set to the MAC address of the S-AP or S-PCP, the Cluster Member Role subfield set to the member AP or member PCP value, and the ClusterMaxMem subfield set to the value of the ClusterMaxMem field contained in the S-AP or S-PCP DMG or QMG Beacon.

Change the sixth paragraph of 9.37.2.1 as follows:

An AP or PCP with a value of Cluster Member Role that is not zero shall schedule a Beacon SP that is allocated for DMG or QMG Beacon frame transmission of other cluster member APs and PCPs within the decentralized AP or PCP cluster at each of ClusterTimeOffsetn, at any time the AP or PCP transmits its own DMG or QMG Beacon. The minimum size of the Beacon SP should be equal to the value of the Beacon SP duration subfield within the S-AP or S-PCP DMG or QMG Beacon.

#### Centralized AP or PCP cluster formation

Change part of the second paragraph of 9.37.2.2 as follows:

Verification step: The verification step is defined to be the following procedure:

- The centralized clustering enabled STA monitors the channel for DMG or QMG Beacon frames over an interval of at least aMinChannelTime.

- During this monitoring period, for each distinct Cluster ID received in a DMG or QMG Beacon frame that has the ECAPC Policy Enforced field set to 1 in the DMG or QMG Parameters field and Cluster Member Role set to 1 (S-AP or S-PCP of the cluster) or 2 (a member AP or member PCP of the cluster), the centralized clustering enabled STA determines if an S-AP with a MAC address equal to the Cluster ID belongs to the same CCSS as the centralized clustering enabled STA, via i) attempting to receive an Announce frame from the S-AP that transmitted the DMG or QMG Beacon frame according to the channel access rules described in 9.36 (DMG channel access) in order to solicit an ECAPC Policy element or ii) any other means. Any such DMG or QMG Beacon frame whose transmitter cannot be determined in this way to belong to the same CCSS as the centralized clustering enabled STA is defined to be from another ECAPC.

The verification step is defined to be unsuccessful if, at the end of the monitoring period, there are one or more received DMG or QMG Beacon frames that are from another ECAPC. Otherwise, the verification step is defined to be successful.

Change the third paragraph of 9.37.2.2 as follows:

If a centralized clustering enabled STA receives at least one DMG or QMG Beacon frame that has the ECAPC Policy Enforced field set to 1 and from a member AP, S-AP, or member PCP of another ECAPC during the monitoring period and the STA is not an ECPAC policy blind AP or PCP, the STA

- Shall cease its activity on this channel and may attempt operation on a different channel, or

- If one of the received DMG or QMG Beacon frames was sent by an S-AP, may elect to unenroll from its current CCSS and join the cluster of the S-AP as a member AP or member PCP.

NOTE 1 - A decentralized clustering enabled AP or PCP does not cease DMG or QMG Beacon frame transmission during Cluster Monitoring and S-AP or S-PCP handover. Hence, data communication is unaffected while performing these procedures.

Change the fourth paragraph of 9.37.2.2 as follows:

If a centralized clustering enabled STA receives at least one DMG or QMG Beacon frame that has the ECAPC Policy Enforced field set to 1 from an S-AP from the same CCSS during the monitoring period and the STA is not an ECPAC policy blind AP or PCP, the STA may elect either to unenroll from its current CCSS and join the cluster of the S-AP as a member AP or member PCP or to continue and become an S-AP in the CCSS.

Change the fifth paragraph of 9.37.2.2 as follows:

In order to become an S-AP in a centralized AP or PCP cluster, a centralized clustering enabled STA shall take the following actions:

— Set dot11ChannelUsageImplemented to true

— Start a BSS as an AP

— On a channel that is not listed as excluded by the CCSR, and

— At the start time and with the beacon interval configured by the CCSR

— Include in transmitted DMG or QMG Beacon frames

— The ECAPC Policy Enforced field set to 1 in the DMG or QMG Parameters field, and

— A Clustering Control field with the ClusterMaxMem subfield set to the values configured by the CCSR, the Beacon SP Duration subfield set to the value most recently configured by the CCSR, the Cluster ID subfield set to the S-AP MAC address, and the Cluster Member Role subfield set to 1 (S-AP or S-PCP of the cluster).

A member AP or member PCP within a centralized AP or PCP cluster, upon a change of the Clustering Control field received from its S-AP, shall update the Clustering Control field sent in subsequent frames. Upon a change of the ECPAC Policy Detail field received from its S-AP, a member AP or member PCP within a centralized AP or PCP cluster shall update the ECPAC Policy element sent in subsequent frames and send individually addressed Announce or Information Response frames to other STAs within the BSS in order notify them of the changes. The member AP or member PCP within the centralized AP or PCP cluster shall attempt to receive a DMG or QMG Beacon frame from its S-AP at least once every dot11DMGEcssPolicyDetailUpdateDurationMax TUs.

Change the sixth paragraph of 9.37.2.2 as follows:

An S-AP in a centralized AP or PCP cluster shall set the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field to 1 for the lifetime of the BSS.

Change the seventh paragraph of 9.37.3.2 as follows:

NOTE 2 - An assumption to allow the establishment of an S-AP or S-PCP in this case is that the APs and PCPs cannot hear each other's DMG or QMG Beacons. How a STA decides to switch the channel or to establish an S-AP or S-PCP is implementation dependent.

Change the eighth paragraph of 9.37.2.2 as follows:

An AP or PCP that

— receives a DMG or QMG Beacon frame that has the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 1 from an S-AP on a channel and

— does not receive at least one DMG or QMG Beacon frame that has the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 1 from S-AP on every channel supported by the AP or PCP in the Operating Class within the next aMinChannelTime and

— is not an ECPAC policy blind AP or PCP

shall either join the cluster of the S-AP as a member AP or member PCP if centralized clustering enabled or cease its activity on this channel and may attempt operation on a different channel. S-APs within a CCSS report the channels unused by the ECAPC via the Channel Usage procedures (see 10.24.15 (Channel usage procedures)).

Change the ninth paragraph of 9.37.2.2 as follows:

An AP or PCP within a decentralized AP or PCP cluster that

— receives a DMG or QMG Beacon frame that has the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 1 from an S-AP and

— does not receive at least one DMG or QMG Beacon frame that has the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 1 from an S-AP on every channel supported by the AP or PCP in the Operating Class within the next aMinChannelTime (i.e., does not become a ECPAC policy blind AP or PCO) and

— is not already an ECPAC policy blind AP or PCP

shall quit the decentralized AP or PCP cluster before the next TBTT + beacon interval length, then the AP or PCP shall either join the cluster of the S-AP as a member AP or member PCP if centralized AP or PCP clustering enabled or cease its activity on this channel and may attempt operation on a different channel.

Change the tenth paragraph of 9.37.2.2 as follows:

An AP or PCP that receives at least one DMG or QMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG or QMG Parameters field set to 1 sent by an S-AP on every channel supported by the AP or PCP in the Operating Class within the most recent aMinChannelTime ignores the ECPAC Policy Enforced subfield (i.e., treats this subfield as though equal to 0) in DMG or QMG Beacon frames received from S-APs for 300×aMinChannelTime. During this period the AP or PCP is called an ECPAC policy blind AP or PCP.

Change the eleventh paragraph of 9.37.2.2 as follows:

NOTE-An AP or PCP within a decentralized AP or PCP cluster does not cease DMG or QMG Beacon frame transmission when quitting a decentralized AP or PCP cluster. Hence, data communication is unaffected while performing these procedures.

Change the twelfth paragraph of 9.37.2.2 as follows:

A centralized clustering enabled AP or PCP that attempts to join the centralized AP or PCP cluster of an SAP as a member AP or member PCP shall be a STA coordinated by an MM-SME that also coordinates a second non-AP and non-PCP STA, and shall successfully perform the following steps in order:

— The AP or PCP shall monitor the channel for DMG or QMG Beacon frames during each Beacon SP over an interval of length at least aMinChannelTime. A Beacon SPn is empty if no DMG or QMG Beacon frame is received during the Beacon SPn over an interval of length aMinChannelTime.

— The second non-AP and non-PCP STA shall attempt to associate with the S-AP and thereby receive an Announce frame from the S-AP. The contents of the Announce frame are passed to the AP or PCP.

— Upon receiving an Announce frame that includes the ECAPC Policy element, the AP or PCP shall select a Cluster Time Offset index from the intersection of a) the Cluster Time Offset indices of the empty Beacon SPs with b) the indices indicated by the Available Cluster Offset Bitmap field in the ECAPC Policy element. If the intersection is empty, the AP or PCP shall select a Cluster Time

Offset index of an empty Beacon SP. The selected Cluster Time Offset index is passed to the second non-AP and non-PCP STA.

Change the fifteenth paragraph of 9.37.2.2 as follows:

The member AP or member PCP within a centralized AP or PCP cluster shall transmit its DMG or QMG Beacon frames with the ECAPC Policy Enforced subfield in the DMG or QMG Parameters field set to 1, the Beacon SP duration subfield in the Clustering Control field set to the value of the Beacon SP duration subfield contained in the most recently received S-AP DMG or QMG Beacon, the Cluster ID subfield set to the MAC address of the S-AP, the Cluster Member Role subfield set to 2 (a member AP or member PCP of the cluster), and the ClusterMaxMem subfields set to the value of the ClusterMaxMem field contained in the S-AP DMG or QMG Beacon.

Change the last paragraph of 9.37.2.2 as follows:

At any time an AP or PCP within a centralized AP or PCP cluster transmits a DMG or QMG Beacon frame, the AP or PCP shall schedule a Beacon SP that reserves time for BHI transmission by other APs and PCPs within the centralized AP or PCP cluster at each in use ClusterTimeOffsetn-1 as indicated by the most recently transmitted (if S-AP) or received (if member AP or member PCP) Available Cluster Time Offset Bitmap. The minimum size of the Beacon SP shall be equal to the value of the Beacon SP duration subfield within the DMG or QMG Beacon frame of the S-AP of the centralized AP or PCP cluster. A member AP, S-AP, or member PCP of a centralized AP or PCP cluster shall not transmit or schedule transmissions during a Beacon SP of another member AP, S-AP, or member PCP.

### Cluster maintenance

#### General cluster maintenance

Change the first paragraph of 9.37.3.1 as follows:

The TBTT of the S-AP or S-PCP provides a timing reference for the Beacon SPs of the member APs and PCPs. Timing synchronization among the member APs and PCPs facilitates equitable sharing of the common medium among the member APs and PCPs. As long as a member AP or member PCP periodically receives DMG or QMG Beacons from the S-AP or S-PCP, the member AP or member PCP is able to maintain synchronization with the S-AP or S-PCP and hence the other member APs and PCPs.

#### Decentralized AP or PCP cluster maintenance

Change the second paragraph of 9.37.3.2 as follows:

A member AP or member PCP of the decentralized AP or PCP cluster shall start an S-AP or S-PCP handover if, within a time period of 4×aMinBTIPeriod beacon intervals, it does not receive a DMG or QMG Beacon frame with the ECAPC Policy Enforced field set to 0, with the value of the Cluster ID field equal to the Cluster ID of the cluster of which the AP or PCP is a member and with the Cluster Member Role field set to the S-AP or S-PCP value. This is the first cluster monitoring period. During the next step in the S-AP or S-PCP handover, the member AP or member PCP performs another cluster monitoring period. A cluster monitoring period is a time period of 4×aMinBTIPeriod beacon intervals during which the AP or PCP listens for DMG or QMG Beacons while continuing to transmit DMG or QMG Beacons using its current Beacon SPn.

Change the fourth paragraph of 9.37.3.2 as follows:

If, during a cluster monitoring period, the member AP or member PCP receives a DMG or QMG Beacon frame with the value of Cluster Member Role set to the S-AP or S-PCP value, the member AP or member PCP shall follow the rules in 9.37.2 (Cluster formation) to become a member AP or member PCP of the cluster corresponding to the detected S-AP or S-PCP or cease operation on the channel; in either case, the cluster monitoring period is terminated.

Change the fifth paragraph of 9.37.3.2 as follows:

If, during a cluster monitoring period, the AP or PCP receives no DMG or QMG Beacons with the value of Cluster Member Role set to the S-AP or S-PCP value and one or more DMG or QMG Beacons with the ECAPC Policy Enforced field set to 0 and with Cluster ID equal to the Cluster ID of its last S-AP or S-PCP, then at the end of the cluster monitoring period the AP or PCP compares the MAC addresses of all such received DMG or QMG Beacons with its own MAC address. If its MAC address is the lowest, the AP or PCP shall become an S-AP or S-PCP according to the rules in 9.37.2 (Cluster formation). If its MAC address is not the lowest, the AP or PCP shall perform a new cluster monitoring period. If the number of cluster monitoring periods performed by the AP or PCP exceeds dot11MaxNumberOfClusteringMonitoringPeriods, the AP or PCP may cease cluster maintenance and initiate cluster formation as described in 9.37.2 (Cluster formation).

Change the sixth paragraph of 9.37.3.2 as follows:

If, during a cluster monitoring period, the AP or PCP does not receive a DMG or QMG Beacon frame that contains the value of S-AP or S-PCP in the Cluster Member Role field and does not receive a DMG or QMG Beacon frame with the ECAPC Policy Enforced field set to 0 and with Cluster ID equal to the Cluster ID of the cluster of which it is currently a member, then at the end of the cluster monitoring period the AP or PCP may become an S-AP or S-PCP according to the rules of 9.37.2 (Cluster formation), or it may cease its activity on this channel and may attempt operation on a different channel.

Change the eighth paragraph of 9.37.3.2 as follows:

If, during a cluster monitoring period, the member AP or member PCP of a decentralized AP or PCP cluster receives no DMG or QMG Beacons from clustering enabled STAs, then the AP or PCP shall establish itself as an SAP or S-PCP according to the rules in 9.37.2 (Cluster formation).

Change the last paragraph of 9.37.3.2 as follows:

If an S-AP or S-PCP of a decentralized AP or PCP cluster detects the presence of a S-AP or S-PCP of another decentralized AP or PCP cluster on the same channel, it should schedule a Beacon SP for the DMG or QMG Beacon frame transmission of the other S-AP or S-PCP if the MAC address of the other S-AP or S-PCP is lower than the MAC address of this S-AP or S-PCP. The S-AP or S-PCP with higher MAC address should become a member AP or member PCP of the cluster corresponding to the S-AP or S-PCP with the lower MAC address according to the rules in 9.37.2 (Cluster formation).

#### Centralized AP or PCP cluster maintenance

Change the last paragraph of 9.37.3.3 as follows:

In the case when a member AP or member PCP of a cluster has not received DMG or QMG Beacon frames from its S-AP for a duration exceeding 4×aMinBTIPeriod beacon intervals, and the member AP or member PCP intends to continue to operate a BSS on the channel, the AP or PCP shall either

a) Stop the current BSS then become an S-AP within the CCSS as described in 9.37.2.2 (Centralized AP or PCP cluster formation); or

b) Monitor the channel for DMG or QMG Beacon frames for an interval of length at least aMinChannelTime. During this period

1) if one or more DMG or QMG Beacon frames are received with the ECAPC Policy Enforced field set to 1 in the DMG or QMG Parameters field and the Cluster Member Role set to 1 (S-AP or S-PCP of cluster) from one or more S-APs and the AP or PCP is not an ECPAC policy blind AP or PCP, then the AP or PCP shall join a selected S-AP as a cluster member as described in 9.37.2.2 (Centralized AP or PCP cluster formation);

2) otherwise, if the AP or PCP is an ECPAC policy blind PCP or AP, then it may join a selected SAP as a cluster member;

3) otherwise, if, after the period elapses, no DMG or QMG Beacon frames are received with the ECAPC Policy Enforced field in the DMG Parameters field set to 1 and the Cluster Member Role set to 1 (S-AP or S-PCP of cluster) and if the AP or PCP is decentralized AP or PCP clustering capable, then the AP or PCP shall attempt to join a decentralized AP or PCP cluster if present as described in 9.37.2.1 (Decentralized AP or PCP cluster formation). If the AP or PCP is not decentralized AP or PCP clustering capable or a decentralized AP or PCP cluster is not present, then the AP or PCP shall set its Cluster Member Role to 0 (not currently participating in a cluster).

c) In all three cases, the AP or PCP

1) Shall set the ECAPC Policy Enforced bit to 0 and shall not include the ECAPC Policy element in (Re)Association Response, Announce, or Information Response frames and

2) Should send individually addressed Announce or Information Response frames to other STAs within the BSS to notify them of the changes.

#### Centralized AP or PCP cluster MAC requirements

Change the third paragraph of 9.37.3.4 as follows:

If the most recent ECAPC Policy element, received by a non-AP and non-PCP STA in a BSS from the member AP, S-AP, or member PCP of the BSS, has the TXSS CBAP Enforced field set to 1, then the non-AP and non-PCP STA shall perform each of its TXSSs in the DTI within one or more TXSS CBAPs. If the non-AP and non-PCP STA is the source DMG or QMG STA of an SP and if the non-AP and non-PCP determines that it needs to perform a TXSS before continuing to transmit to the destination DMG or QMG STA of the SP, then the non-AP and non-PCP STA should truncate the SP (see 9.36.8 (Dynamic truncation of service period)).

Change the fourth paragraph of 9.37.3.4 as follows:

A TXSS CBAP shall last from Tstart to Tend, as defined below, excluding any time that overlaps a BHI or an SP that has source and destination DMG or QMG AIDs set to 255 (such as for a Beacon SP).

### Cluster report and re-scheduling

Change the second paragraph of 9.37.4 as follows:

A non-AP and non-PCP STA that is a member of a BSS and that receives a DMG or QMG Beacon frame should send a Cluster Report element to its AP or PCP if the received DMG or QMG Beacon frame meets all of the following conditions:

— The DMG or QMG Beacon frame is not from the STA’s AP or PCP.

— The DMG or QMG Beacon frame contains the Clustering Control field.

— Either

— The value of the Cluster ID field within the Clustering Control field is different from the MAC address of the STA’s AP or PCP; or

— The value of the Cluster ID field within the Clustering Control field is the same as the MAC address of the STA’s AP or PCP, the TBTTs of the two BSSs are less than dot11BeaconPeriod/(2×ClusterMemMax) apart in time, and the ECAPC Policy Enforced field in the DMG Beacon frame received most recently from the STA’s AP or PCP is equal to 1.

Change the third paragraph of 9.37.4 as follows:

The non-AP and non-PCP STA shall not send a Cluster Report element to its AP or PCP if the received DMG or QMG Beacon frame does not meet the preceding conditions.

Change the fourth paragraph of 9.37.4 as follows:

A Cluster Report element meeting the conditions above shall be transmitted in an Announce or Information Response frame sent to the STA’s AP or PCP. Within the transmitted Cluster Report element, the STA shall set the Cluster report subfield to 1. The STA shall set the Clustering Control field within a transmitted Cluster Report element to the corresponding field values within the Clustering Control of the received DMG or QMG Beacon, shall set the Reported BSSID field to the BSSID of the received DMG or QMG Beacon, and shall set the Reference timestamp field to indicate the DMG or QMG Beacon frame reception time. The STA shall set the Schedule present subfield to 1 if the Extended Schedule field is present in the transmitted Cluster Report element; otherwise, it shall set Schedule present subfield to 0. The STA shall set the TSCONST present subfield to 1 if the Traffic Scheduling Constraint Set field is present in the transmitted Cluster Report element; otherwise, it shall set TSCONST present subfield to 0.

Change the fifth paragraph of 9.37.4 as follows:

The STA shall set the ECAPC Policy Enforced field in the Cluster Report Control field to the value of the ECAPC Policy Enforced field in the received DMG or QMG Beacon. The STA should attempt to receive an Announce frame from the AP or PCP that transmitted the DMG or QMG Beacon frame according to the channel access rules described in 9.36 (DMG channel access) in order to solicit an ECAPC Policy element. If the STA obtains an ECAPC Policy element from the AP or PCP that transmitted the DMG or QMG Beacon, the STA shall set the ECAPC Policy Present subfield to 1 and include the ECAPC Policy element in the transmitted Cluster Report element; otherwise, the STA shall set ECAPC Policy Present subfield to 0 and not include the ECAPC Policy element in the transmitted Cluster Report element. If present, the Extended Schedule Element field within the Cluster Report element shall be set to the corresponding field values within the Extended Schedule element of the received DMG or QMG Beacon. If present, the Traffic Scheduling Constraint Set field shall be set to indicate periods of time with respect to the TBTT of the beacon interval of the BSS the STA participates where the transmitting STA experiences poor channel conditions, such as due to interference.

Change the sixth paragraph of 9.37.4 as follows:

If the received DMG or QMG Beacon frame contains more than one Extended Schedule element entry, the STA shall repeat the aforementioned procedure and transmit a Cluster Report element corresponding to each Extended Schedule element entry.

Change the eighth paragraph of 9.37.4 as follows:

A member AP or member PCP within a centralized AP or PCP cluster should report new interference information and may report all interference information to the S-AP of the cluster

— When the member AP or member PCP receives one or more of the following:

— A DMG or QMG Beacon frame from another AP or PCP in another centralized AP or PCP cluster within the same CCSS or another CCSS

— A Cluster Report element with the Cluster Report field set to 1 from a non-AP and non-PCP STA within the same BSS characterizing an AP or PCP in another centralized AP or PCP cluster within the same CCSS or another CCSS; and

— If at least dot11DMGEcssClusterReportDurationMin TUs have elapsed since the last report.

Change the last paragraph of 9.37.4 as follows:

Via unspecified means, the S-AP might aggregate received DMG or QMG Beacon frames and Cluster Report elements and send the aggregate to the CCSR. Upon receiving this information, the CCSR might reconfigure the TSF offsets of an S-AP, reconfigure the ECAPC Policy Detail of an S-AP, update the Cluster Time Offset availability information provided in an individually addressed frame by an S-AP to a member AP or member PCP, or perform other actions.

### Decentralized AP or PCP cluster request

Change the third paragraph of 9.37.5 as follows:

If the non-AP and non-PCP STA does not receive a DMG or QMG Beacon frame from its AP or PCP with decentralized AP or PCP clustering enabled after dot11ClusterEnableTime following the transmission to its AP or PCP of a Cluster Report element with the Cluster request subfield set to 1, the non-AP and non-PCP STA may transmit an Announce frame including the last Extended Schedule element transmitted by the AP or PCP. If the Announce frame is transmitted, it shall use MCS 0, and the TA field shall be set to the broadcast address. If a DMG or QMG STA receives an Announce frame with the TA field set to the broadcast address and with the BSSID field different from the BSSID of its BSS, the STA may send a Cluster Report element containing the Extended Schedule element within the received Announce frame to its AP or PCP, which might be used by the AP or PCP to reschedule SPs in portions of the beacon interval that are nonoverlapping in time with the SPs contained in the Extended Schedule element reported by the STA.

# MLME

## Synchronization

## Power management

#### Receive operation for STAs in PS mode during the CFP

#### QMG TXOP power save

A QMG AP supports the operation of non-AP QMG STAs in TXOP power save mode in a BSS when the dot11QMGTXOPPowerSaveOptionImplemented at the AP is true. Non-AP QMG STAs that are in Active mode (see Table 10-1) and have dot11QMGTXOPPowerSaveOptionImplemented equal to true operate in TXOP power save mode. A STA that has dot11QMGTXOPPowerSaveOptionImplemented equal to true shall set the QMG TXOP Power Save field in the QMG Capabilities element to 1; otherwise, the STA shall set the field to 0. A QMG AP may allow non-AP QMG STAs in TXOP power save mode to enter the Doze state during a TXOP. A QMG AP shall indicate this by transmitting a QMG PPDU with the TXVECTOR parameter TXOP_PS_NOT_ALLOWED set to 0. The value of this parameter in the TXVECTOR of all QMG PPDUs transmitted by the QMG AP may be changed from 1 to 0 during the TXOP to enable TXOP PS for the remainder of the TXOP. The value of this parameter in the TXVECTOR of all QMG PPDUs transmitted by QMG AP shall not be changed from 0 to 1 during the TXOP. If the dot11QMGTXOPPowerSaveOptionImplemented at QMG AP is false then the QMG AP shall set the TXOP_PS_NOT_ALLOWED to 1 in the TXVECTOR of the frames.

If the AP allows non-AP QMG STAs to enter Doze state during a TXOP, then a non-AP QMG STA that is in QMG TXOP power save mode may enter the Doze state till the end of that TXOP when one of the following conditions is met:

— On receipt of a PPDU, the STA determines that the RXVECTOR parameter UPLINK_INDICATION is equal to 0 but the COLOR in the RXVECTOR parameter does not match the COLOR indicated by the AP to which the STA is associated.

—The STA finds that the COLOR in the RXVECTOR parameter matches the COLOR indicated by the AP to which the STA is associated but the RXVECTOR parameter PARTIAL_AID is not equal to 0 nor does it match the STA’s partial AID.

— The STA finds that the PARTIAL_AID in the RXVECTOR matches its partial AID but the RA in the MAC header of the corresponding frame that is received correctly does not match the MAC address of the STA.

— 540 MHz STAs receive a Beacon frame or a Set PCO frame that contains the PCO Phase field equal to 1.

— In a received PSMP frame, the STA finds that the STA_AID field in the PSMP STA Info field is not its AID nor does the PSMP Group Address ID match its Group Address.

— The STA receives a frame intended for it with the More Data field equal to 0 and the Ack Policy subfield in the QoS Control field is equal to No Ack or sends an acknowledgment if Ack Policy subfield is not equal to No Ack.

—  On receipt of a PPDU, the non-AP STA determines that the RXVECTOR parameter UPLINK_INDICATION is equal to 1

The QMG AP shall include a NAV-set sequence (e.g., RTS/CTS) at the beginning of such a TXOP with the Duration/ID value set to the remainder of the TXOP duration. A QMG AP shall not transmit frames to a non-AP QMG STA that has been allowed to enter Doze state according to the conditions above for the remainder of the TXOP.

NOTE—A QMG AP does not transmit QMG PPDUs in the current TXOP if the AP has already transmitted a QMG PPDU with the TXVECTOR parameter TXOP_PS_NOT_ALLOWED set to 0 in the same TXOP and does not want the STAs that are in Awake state to enter the Doze state.

If a QMG AP truncates the TXOP in which it allowed STAs to enter Doze state, then the QMG AP shall not transmit frames to the STAs that were allowed to enter the Doze state until the NAV set at the start of the TXOP has expired.

If the AP does not receive an acknowledgment after transmitting an individually addressed frame containing all or part of an MSDU, A-MSDU or MMPDU sent with the More Data field equal to 0 to a non-AP QMG STA that is in QMG TXOP power save mode and the AP had set the TXVECTOR parameter TXOP_PS_NOT_ALLOWED to 0, it shall retransmit that frame at least once within the same TXOP, subject to applicable retry, lifetime limit or TXOP limit. If an acknowledgment to the retransmission of this last frame in the same TXOP is not received, it may wait until the next TXOP to further retransmit that frame, subject to its applicable retry or lifetime limit.

NOTE—An AP that receives from a QMG STA in TXOP power save mode a BlockAck frame that is a response to an A-MPDU containing MPDUs with the More Data field equal to 0 cannot expect to receive a response to subsequent MPDUs retransmitted in the same TXOP because the QMG STA might be in the Doze state.

A QMG STA that is in TXOP power save mode and has entered Doze state shall continue to operate its NAV timer during Doze state and shall transition into Awake state on expiry of the NAV timer.

NOTE—The STA can contend for access to the medium immediately on the expiry of the NAV timer.

## STA authentication and association

## TPC procedures

## DFS procedures

## Radio measurement procedures

## Phased coexistence operation (PCO)

Change 10.16 as follows：

### General description of PCO

PCO is an optional coexistence mechanism in which a PCO active AP divides time into alternating 540 MHz and 1080MHz phases (see Figure 10-20). The PCO active AP reserves the 540 MHz primary channel and the 540 MHz secondary channel in turn to start the 1080 MHz phase and resets the NAV in the 540 MHz channels in the opposite order to start the 540 MHz phase. Due to the protection of the 1080 MHz period in both channels, it is tolerant of OBSSs on both 540 MHz halves of a 1080 MHz channel.

A PCO active STA that does not know the current PCO phase shall transmit using a 540 MHz PPDU.

During the 1080 MHz phase, a PCO active STA shall transmit Data frames using a 1080 MHz QMG PPDU and Control frames using a 540MHz duplicate or a 1080 MHz PPDU, with the following exceptions:

— Any CF-End frame shall be sent using only a 1080 MHz PPDU.

— A PCO active AP may transmit 540 MHz group addressed frames as defined in 9.7.5.3.

Figure 10-20—Phased coexistence operation (PCO)

A PCO active STA shall transmit Management frames in 540 MHz or 1080 MHz PPDUs according to 9.7.5 during the 1080 MHz phase, except that QMG Set PCO Phase frames shall be sent following the rules specified in 10.16.2.

During the 540 MHz phase, a PCO active STA shall not transmit frames using a 1080 MHz (or QMG duplicate) PPDU.

During the 540 MHz phase, a STA may transmit a 1080 MHz mask PPDU that is not also a 1080 MHz PPDU.

NOTE 1—This rule allows a STA to transmit 540 MHz PPDUs without requiring it to change to a 540 MHz transmit mask.

A PCO-capable AP may set the PCO Active field to 1 only if it is in a 540/1080 MHz BSS.

NOTE 2—A non-PCO-capable 540/1080 STA regards the PCO active BSS as a PCO inactive BSS. A non-PCO-capable 540/1080 STA that associates with a PCO active BSS protects its transmissions as though the BSS were a PCO inactive BSS.

The value indicated by the PCO Transition Time field in the QMG Extended Capabilities field is measured from the end of the PPDU carrying the QMG Set PCO Phase frame. The PCO active STA shall be able to receive a PPDU using the new channel width no later than the value specified by the PCO Transition Time field after the end of the PPDU carrying the QMG Set PCO Phase frame.

### Operation at a PCO active AP

A PCO-capable AP activates PCO if it decides that PCO active BSS is more appropriate than either PCO inactive BSS or 540 MHz BSS in the current circumstances. The algorithm for making this decision is beyond the scope of this standard.

A PCO active AP shall set the PCO Active field in the QMG Operation element to 1.

When a PCO active AP detects that PCO is not providing a performance benefit, the PCO active AP may deactivate PCO and operate in either a PCO inactive BSS or 540 MHz BSS. A PCO-capable AP shall set the PCO Active field in the QMG Operation element to 0 when PCO operation is disabled. Since the AP advertises the current mode in its Beacon and Probe Response frames, its associated STAs are informed of the mode change.

Values of the PCO Transition Time field in the QMG Extended Capabilities field from 1 to 3 indicate the maximum time the PCO active STA takes to switch between a 540 MHz channel width and a 1080 MHz channel width. A PCO active AP may set the PCO Transition Time field to 0 when it requires the associated PCO active STAs to be able to receive 1080 MHz frames and respond with 1080 MHz frames during the 540 MHz phase.

The PCO active AP shall increase the value of the PCO Transition Time field if the PCO active AP accepts the association of a PCO-capable STA whose value of the PCO Transition Time field exceeds the one currently used by the PCO active AP. If the PCO active AP decides not to extend its transition time to meet the value of the requesting STA, the PCO active AP shall deny the association. The AP may choose to continue PCO when a non-PCO-capable 540/1080 STA requests association, and in such cases, the PCO active AP shall be able to receive 1080 MHz frames and respond using1080 MHz frames during the 540 MHz phase.

A PCO active AP that indicates a switch to the 1080 MHz phase by a PCO Phase field in a Beacon frame or by a PCO Phase Control field in a QMG Set PCO Phase frame and that transmits a nonzero value of the PCO Transition Time field shall wait for at least the transition time specified by the PCO Transition Time field before sending a CF-End frame in the 1080 MHz channel to start the 1080 MHz operating phase.

When switching to the 1080 MHz phase, a PCO active AP indicates a NAV duration either in the CF Parameter Set element of a Beacon frame or in the Duration/ID field of a Set PCO Phase frame sent on the primary channel that shall protect up to the end of the intended 1080 MHz phase plus a transition time. A PCO active AP may continue the CFP after the 1080 MHz phase by setting a longer duration for the CFP. The value of the Duration/ID field in a CTS-to-self frame sent to protect a 1080 MHz phase shall be set to protect up to the intended end of the 1080 MHz phase plus a transition time. The CTS-to-self shall be sent in a QMG duplicate PPDU. The transmission of the CTS-to-self shall be delayed until the secondary channel CCA has indicated idle for at least a PIFS. It need not sense the primary channel because it is already reserved by a Beacon frame or a Set PCO Phase frame.

If the PCO Transition Time field is nonzero, a PCO active AP shall start a timer with a timeout value equal to the time specified by the PCO Transition Time field after transmitting a Beacon frame or a Set PCO Phase frame. If this timer expires while attempting to reserve the secondary channel, the AP shall transmit a Set PCO Phase frame indicating a switch back to the 540 MHz phase and shall transmit a CF-End frame on the primary channel.

NOTE—If this timer expires while attempting to reserve the secondary channel, the AP abandons switching to the 1080 MHz phase to avoid an unexpectedly long delay.

A PCO active AP may transmit a QMG Set PCO Phase frame in a QMG duplicate PPDU followed by a CF-End frame in a 1080 MHz HT PPDU to reserve both the primary and secondary channels again for the 1080 MHz phase or to extend the 1080 MHz phase. The value of the Duration/ID field in a QMG Set PCO Phase frame contained in a QMG duplicate PPDU for this intent shall protect up to the end of the intended 1080 MHz phase plus the transition time.

To start the 540 MHz phase, a PCO active AP shall send a QMG Set PCO Phase frame in a 1080 MHz PPDU or in a QMG duplicate PPDU with the Duration/ID field set to cover the transition time. It may also send a CF-End frame in both primary and secondary channels following the QMG Set PCO Phase frame, where a CF-End frame in the primary channel shall be sent out at least after the transition time. The Duration/ID field of the QMG Set PCO Phase frame for this case shall cover the transition time plus the duration of a CF-End frame.

A PCO active AP may broadcast a QMG Set PCO Phase frame to advertise the current PCO phase to PCO active STAs.

Although PCO improves throughput in some circumstances, PCO might also introduce jitter. To minimize the jitter, the maximum duration of 1080 MHz phase and 540 MHz phase is dot11PCO1080MaxDuration and dot11PCO540MaxDuration, respectively. Also in order for the PCO active AP to give opportunities for each STA to send frames, the minimum duration of 1080 MHz phase and 540 MHz phase is dot11PCO1080MinDuration and dot11PCO540MinDuration, respectively.

### Operation at a PCO active non-AP STA

If the PCO field in the Association Request frame to a PCO active AP is equal to 1 and the association succeeds, the STA shall operate in PCO mode. When requesting association, a PCO-capable STA shall set the PCO Transition Time field to 0 if the PCO active AP has set the PCO Transition Time field to 0. A PCO-capable STA may attempt to associate with a transition time that is larger than one currently advertised by the PCO active AP. If such an association fails, the PCO-capable non-AP STA may regard the BSS as a PCO inactive BSS and may attempt an association as a non-PCO-capable 540/1080 STA.

NOTE 1—A STA that does not support the PCO transition time indicated by an AP might still attempt association with that AP. The AP will either refuse the association based on PCO transition time or respond by adjusting its PCO transition time to suit the STA.

A PCO active non-AP STA may transmit a Probe Request frame to the associated PCO active AP to determine the current PCO phase. A PCO active STA associated with a PCO active AP shall switch its operating phase from 540 MHz channel width to 1080 MHz channel width when it receives from its AP a Beacon frame or a Probe Response frame that contains the PCO Phase field equal to 1 or a QMG Set PCO Phase frame with the PCO Phase Control field equal to 1. The value of the CFP DurRemaining field in the CF Parameter Set element of a Beacon frame or the value of the Duration/ID field of a QMG Set PCO Phase frame shall be interpreted as the duration of the PCO 1080 MHz phase.

A PCO active STA associated with a PCO active AP shall switch its operating phase from 1080 MHz channel width to 540 MHz channel width when it receives a Beacon frame or a Probe Response frame that contains the PCO Phase field equal to 0 or a QMG Set PCO Phase frame with the PCO Phase Control field equal to 0. It also may switch from 1080 MHz channel width to 540 MHz channel width based on the expiry of the value in the Duration/ID field of a QMG Set PCO Phase frame that indicated a 1080 MHz phase or based on the expiry of the value in the CFP DurRemaining field of the CF Parameter Set element of a Beacon frame that indicated a 1080 MHz phase.

A PCO active STA shall halt PCO operation if it receives a QMG Operation element from its AP with the PCO Active field equal to 0.

NOTE 2—A QMG STA might change its PCO capabilities by disassociating followed by associating with an AP.

## Tunneled direct link setup

## Quality-of-Service management frame (QMF)

## Multi-band operation

### General

A device is multi-band capable if the value of its local MIB variable dot11MultibandImplemented is true. A multi-band capable device is said to be a member of a BSS when one or more of the STAs in the device is a member of the BSS. A STA that is part of a multi-band capable device advertises the capability by including the Multi-band element in Beacon, DMG Beacon, QMG Beacon, (Re)Association Request, (Re)Association Response, Information Request, Information Response, Probe Request, Probe Response, Announce, FST Setup Request, FST Setup Response, TDLS Discovery Request, TDLS Discovery Response, TDLS Setup Request, and TDLS Setup Response frames.

## MMSL cluster operation

## QMG BSS operation

### Basic QMG BSS functionality

A QMG STA has dot11QMGOptionImplemented equal to true.

A STA that is starting a QMG BSS shall be able to receive and transmit at each of the <QMG-MCS, NSS> tuple values indicated by the Basic QMG-MCS and NSS Set field of the QMG Operation parameter of the MLME-START.request primitive and shall be able to receive at each of the <QMG-MCS, NSS> tuple values indicated by the Supported QMG-MCS and NSS Set field of the QMG Capabilities parameter of the MLME-START.request primitive.

A STA that is a QMG AP or a QMG mesh STA declares its channel width capability in the Supported Channel Width Set subfield of the QMG Capabilities element QMG Capabilities Info field as described in Table 8-183v.

A STA that is a QMG AP or a QMG mesh STA shall set the Channel Width subfield in the QMG Operation element QMG Operation Information field to indicate the BSS operating channel width as defined in Table 10-25.

Table 10-25-QMG BSS operating channel width

| QMG Operation element Channel Width field | BSS operating channel width |
| --- | --- |
| 0 | 540 MHz |
| 1 | 1080 MHz |

A QMG STA that is a member of a QMG BSS shall not transmit a 540 MHz QMG PPDU on a channel other than the primary 540 MHz channel of the BSS, except for a 540 MHz QMG PPDU transmission on an off-channel TDLS direct link as constrained by 10.22.

A QMG STA shall not transmit to a second QMG STA using a bandwidth that is not indicated as supported in the QMG Capabilities element received from that QMG STA.

### Channel selection methods for a QMG BSS

Before a STA starts a QMG BSS, the STA shall perform a minimum of dot11QMGOBSSScanCount OBSS scan operations to search for existing BSSs (see 10.40.3).

If an AP or a mesh STA starts a QMG BSS that occupies some or all channels of any existing BSSs, the AP or mesh STA may select a primary channel of the new QMG BSS that is identical to the primary channel of any one of the existing BSSs.

If an AP or a mesh STA selects a primary channel for a new QMG BSS with a 1080 MHz operating channel width from among the channels on which no beacons are detected during the OBSS scans, then the selected primary channel meets the following conditions:

— It shall not be identical to the secondary 540 MHz channel of any existing BSSs with a 1080 MHz operating channel width.

NOTE—An AP or a mesh STA operating a QMG BSS with a 1080 MHz operating channel width, on detecting an OBSS whose primary channel is the AP’s or the mesh STA’s secondary 540 MHz channel, might switch to 540 MHz BSS operation and/or move to a different channel.

### Scanning requirements for QMG STA

An OBSS scan operation is a passive or active scan of a set of channels that are potentially affected by QMG BSS operation (see 10.1). Each channel in the set may be scanned more than once during a single OBSS scan operation. OBSS scans are performed by STAs that start a QMG BSS.

During an individual scan within an OBSS scan operation, the minimum per-channel scan duration is dot11OBSSScanPassiveDwell TUs (for a passive scan) or dot11OBSSScanActiveDwell TUs (for an active scan). During an OBSS scan operation, each channel in the set is scanned at least once per dot11BSSWidthTriggerScanInterval seconds, and the minimum total scan time (i.e., the sum of the scan durations) per channel within a single OBSS scan operation is dot11OBSSScanPassiveTotalPerChannel TUs (for a passive scan) or dot11OBSSScanActiveTotalPerChannel TUs (for an active scan).

NOTE—The values provided in the previous paragraph are minimum requirements. For some combinations of parameter values the minimum might be exceeded for some parameters in order to meet the minimum value constraints of other parameters.

### Channel switching methods for a QMG BSS

A QMG AP announces a switch of operating channel by either of the following:

— Using the Channel Switch Announcement element, Channel Switch Announcement frame, or both, following the procedure described in 10.9

— Using the Extended Channel Switch Announcement element, Extended Channel Switch Announcement frame, or both, following the procedure described in 10.10.

A QMG mesh STA announces a switch attempt of operating channel by either of the following:

— Using the Channel Switch Announcement element, Channel Switch Announcement frame, or both, following the procedure described in 10.9.

— Using the Extended Channel Switch Announcement element, Extended Channel Switch Announcement frame, or both, following the procedure described in 10.10.

A QMG AP or a QMG mesh STA may also announce a switch of operating channel width, a new Country String field (possibly including a new Operating Class table number), new operating classes, or new TPC parameters for the BSS that come into effect at the same time as the switch of operating channel.

NOTE 1—Other means to switch the operating channel width are described in 10.42.

The New Channel Number field in the Channel Switch Announcement element, Extended Channel Switch Announcement element, Channel Switch Announcement frame, or Extended Channel Switch Announcement frame identifies the primary 540 MHz channel after the switch. The value of the New Channel Number field is set to the value that dot11CurrentPrimaryChannel (see 1.10) will have after the switch.

If a Channel Switch Announcement element in a Beacon frame or Probe Response frame is used to announce a switch to a 540 MHz operating channel width, then a Wide Bandwidth Channel Switch subelement in a Channel Switch Wrapper element shall not be present in the same frame.

If an Extended Channel Switch Announcement element in a frame or an Extended Channel Switch Announcement frame is used to announce a switch to a 540 MHz operating channel width, then a Wide Bandwidth Channel Switch subelement shall not be present in the same frame.

NOTE 2—The indicated operating class within the Extended Channel Switch Announcement element or frame is used to differentiate between a BSS operating channel width of 540 MHz and a BSS operating channel width greater than 540 MHz as well as indicate the location of the secondary 540 MHz channel. When switching to a 540 MHz operating channel width, the operating class indicated within the Extended Channel Switch Announcement element or frame has a channel spacing of 540 MHz. When switching to an operating channel width greater than 540 MHz, the operating class indicated within the Extended Channel Switch Announcement element or frame has a channel spacing of 1080 MHz.

If a Channel Switch Announcement element is used in a Beacon or Probe Response frame to announce a switch to a 1080 MHz operating channel width, then the Wide Bandwidth Channel Switch subelement in the Channel Switch Wrapper element shall also be present in the same frame.

If an Extended Channel Switch Announcement element is used in a Beacon or Probe Response frame to announce a switch to a 1080 MHz operating channel width, then the Wide Bandwidth Channel Switch subelement in the Channel Switch Wrapper element may be present in the same frame.

NOTE 3—The indicated operating class within the Extended Channel Switch Announcement element identifies the bandwidth and the relative position of the primary 540 MHz and secondary 540 MHz channels. Hence a Wide Bandwidth Channel Switch subelement is unnecessary when the Extended Channel Switch Announcement element is used for a channel switch to a 1080 MHz bandwidth.

If new BSS TPC parameters are announced that come into effect at the same time as the channel switch, then a STA that is a QMG AP, a QMG STA in an IBSS, or a QMG mesh STA in an MBSS shall include

— At least one New QMG Transmit Power Envelope element in a transmitted Channel Switch Announcement frame or Extended Channel Switch Announcement frame and

— At least one New QMG Transmit Power Envelope subelement in a transmitted Channel Wrapper element in Beacon and Probe Response frames.

A recipient QMG STA in the BSS STA that has dot11SpectrumManagementRequired or dot11RadioMeasurementActivated equal to true and that maintains association with the BSS after the switch shall use the parameters in these received elements and subelements in the recipient STA’s TPC calculations for the new operating channel and operating channel width (see 10.8 (TPC procedures)). If both New QMG Transmit Power Envelope elements and New QMG Transmit Power Envelope subelements are transmitted for the switch, the set of New QMG Transmit Power Envelope elements and set of subelements shall contain the same set of values for the Local Maximum Transmit Power Unit Interpretation subfield, and New QMG Transmit Power Envelope elements and subelements that have the same value for the Local Maximum Transmit Power Unit Interpretation subfield shall also have the same values for their other fields.

If a new country string, new operating classes or both, are coming into effect at the same time as the channel switch, then a STA that is a QMG AP, a QMG STA in an IBSS, or a QMG mesh STA in an MBSS shall include

— A New Country element in a transmitted Extended Channel Switch Announcement frame and

— A New Country subelement in a transmitted Channel Wrapper element.

The New Country element or subelement shall contain all the Operating Classes for the BSS after the switch. The New Country element or subelement, transmitted in an Extended Channel Switch Announcement frame or in the same frame as an Extended Channel Switch Announcement element, respectively, shall include one Operating Triplet field that contains the same Operating Class as the New Operating Class field in the Extended Channel Switch Announcement frame or Extended Channel Switch Announcement element. A recipient QMG STA in the BSS STA that has dot11MultiDomainCapabilityActivated, dot11SpectrumManagementRequired, or dot11RadioMeasurementActivated equal to true and that maintains association with the BSS after the switch shall use the parameters in these received elements and subelements in order to maintain regulatory compliance. If both New Country elements and New Country subelements are transmitted for the switch, their fields shall be the same.

A Channel Switch Wrapper element shall not be included in Beacons and Probe Responses if the element contains zero subelements.

NOTE 4—Channel Switch Wrapper is not defined to carry subelements in the case of a switch to 540 MHz and when no change to the country string, operating classes or TPC parameters are announced.

An AP that switches the BSS to a lower operating channel width may recalculate the TS bandwidth budget and may delete one or more active TSs by invoking the MLME-DELTS.request primitive with a ReasonCode value of SERVICE_CHANGE_PRECLUDES_TS.

### NAV assertion in a QMG BSS

A QMG STA shall update its NAV as described in 9.3.2.4 and 9.3.2.13 using the Duration/ID field value in any frame that does not have an RA matching the STA’s MAC address.

### QMG STA antenna indication

A QMG STA that does not change its Rx antenna pattern after association shall set the Rx Antenna Pattern Consistency subfield in the QMG Capabilities Info field to 1; otherwise, the STA shall set the Rx Antenna Pattern Consistency subfield in the QMG Capabilities Info field to 0.

A QMG STA that does not change its Tx antenna pattern after association shall set the Tx Antenna Pattern Consistency subfield in the QMG Capabilities Info field to 1; otherwise, the STA shall set the Tx Antenna Pattern Consistency subfield in the QMG Capabilities Info field to 0.

### BSS basic QMG-MCS and NSS set operation

The BSS basic QMG-MCS and NSS set is the set of <QMG-MCS, NSS> tuples that are supported by all QMG STAs that are members of a QMG BSS. It is established by the STA that starts the QMG BSS, indicated by the Basic QMG-MCS and NSS Set field of the QMG Operation element in the MLME-START.request primitive. Other QMG STAs determine the BSS basic QMG-MCS and NSS set from the Basic QMG-MCS and NSS Set field of the QMG Operation element in the BSSDescription derived through the scan mechanism (see 10.1).

A QMG STA shall not attempt to join (MLME-JOIN.request) a BSS unless it supports (i.e., is able to both transmit and receive using) all the <QMG-MCS, NSS> tuples in the BSS basic QMG-MCS and NSS set.

A QMG STA shall not attempt to (re)associate (MLME-ASSOCIATE.request and MLMEREASSOCIATE. request) with a QMG AP unless the STA supports (i.e., is able to both transmit and receive using) all the <QMG-MCS, NSS> tuples in the Basic QMG-MCS and NSS Set field in the QMG Operation element transmitted by the AP.

## Notification of operating mode changes

# Security

# MLME mesh procedures

# QMG PHY specification

## Introduction

### Introduction to the PHY

Clause 26 specifies respectively the PHY entity for a millimeter wave WLAN single carrier (SC) and orthogonal frequency division multiplexing (OFDM) system.

The maximum number of space-time streams supported in the millimeter wave WLAN SC and OFDM system is four.

A QMG STA shall support the following Clause 26 features:

- 540 MHz channel width

- Detect and decode SIG field and data field of the Control PHY and SC PHY.

- Single spatial stream MCSs 1 to 3 for SC transmission

- LDPC coding with length 672

- Normal Guard Interval as defined in Table 26-3 (Timing-related constants)

A QMG STA may support the following Clause 26 features:

- Support for 1080 MHz channel width

- QMG MCSs 4 to 8 for SC transmission

- Short Guard Interval as defined in Table 26-3 (Timing-related constants)

- LDPC coding with length 2016

- Detect and decode SIG field of the SC MIMO PHY and OFDM MIMO PHY.

- STBC (transmit and receive)

- Transmit beamforming sounding (by sending a QMG NDP)

- Responding to transmit beamforming sounding

### Scope

The services provided to the MAC by the QMG PHY consist of the following protocol functions:

- A PHY convergence function, which adapts the capabilities of the physical medium dependent (PMD) system to the PHY service. This function is supported by the physical layer convergence procedure (PLCP), which defines a method of mapping the PSDUs into a framing format (PPDU) suitable for sending and receiving PSDUs between two or more STAs using the associated PMD system.

- A PMD system whose function defines the characteristics and method of transmitting and receiving data through a wireless medium between two or more STAs. Depending on the PPDU format, these STAs support a mixture of QMG Control PHY, QMG SC PHY and QMG OFDM PHY.

### QMG PHY functions

The QMG PHY contains three functional entities: the PHY convergence function (PLCP), the layer management function (PLME) and the PMD function. Each of these functions is described in detail in 1.3 to 1.15. The QMG PHY service is provided to the MAC through the PHY service primitives defined in Clause 12.

#### QMG PLCP sublayer

In order to allow the MAC to operate with minimum dependence on the PMD sublayer, a PHY convergence sublayer is defined (PLCP). The PLCP sublayer simplifies the PHY service interface to the MAC services.

#### QMG PMD sublayer

The QMG PMD sublayer provides a means to send and receive data between two or more STAs. This subclause is concerned with SC and OFDM modulations.

#### PHY management entity (PLME)

The PLME performs management of the local PHY functions in conjunction with the MLME.

#### Service specification method

The models represented by figures and state diagrams are intended to be illustrations of the functions provided. It is important to distinguish between a model and a real implementation. The models are optimized for simplicity and clarity of presentation; the actual method of implementation is left to the discretion of the QMG PHY compliant developer. The service of a layer or sublayer is the set of capabilities that it offers to a user in the next higher layer (or sublayer). Abstract services are specified here by describing the service primitives and parameters that characterize each service. This definition is independent of any particular implementation.

### QMG PHY functions

#### General

The QMG PHY contains two functional entities: the PHY function and the physical layer management function (i.e., the PLME). Both of these functions are described in detail in 1.5, 1.6, and 1.14.

#### PHY management entity (PLME)

The QMG PHY service is provided to the MAC through the PHY service primitives defined in Clause 7. The QMG PHY service interface is described in 1.2.

#### Service specification method

The models represented by figures and state diagrams are intended to be illustrations of the functions provided. It is important to distinguish between a model and a real implementation. The models are optimized for simplicity and clarity of presentation; the actual method of implementation is left to the discretion of the QMG-PHY-compliant developer.

The service of a layer is the set of capabilities that it offers to a user in the next higher layer. Abstract services are specified here by describing the service primitives and parameters that characterize each service. This definition is independent of any particular implementation.

#### PPDU format

The structure of the PPDU transmitted by a QMG STA is determined by the TXVECTOR parameters as defined in Table 26- .

A QMG PPDU can be further categorized as a QMG SC Control PHY PPDU, a QMG SC PHY PPDU, or QMG OFDM PHY PPDU.

## QMG PHY service interface

### Introduction

The PHY provides an interface to the MAC through an extension of the generic PHY service interface defined in 7.3.4 (Basic service and options). The interface includes TXVECTOR, RXVECTOR, and PHYCONFIG_VECTOR.

The TXVECTOR supplies the PHY with per-PPDU transmit parameters. Using the RXVECTOR, the PHY informs the MAC of the received PPDU parameters. Using the PHYCONFIG_VECTOR, the MAC configures the PHY for operation, independent of frame transmission or reception.

### TXVECTOR and RXVECTOR parameters

The parameters in Table 26-  are defined as part of the TXVECTOR parameter list in the PHYTXSTART.request primitive and/or as part of the RXVECTOR parameter list in the PHYRXSTART.indication primitive.

Table 26-  TXVECTOR and RXVECTOR parameters

| Parameter | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- |
| MCS | The MCS field indicates the modulation and coding scheme used in the transmission of the packet. Values are integers in the range of 0 to 31. - An MCS value of 0 indicates the use of Control PHY. - MCS values of 1 to 8 indicate use of single carrier modulations. The value is an index to Table 26-40 to Table 26-47. - MCS values of 9 to 16 indicates use of OFDM modulations. The value is an index to Table 26-48 to Table 26-55. | Y | Y |
| LENGTH | Indicates the number of octets in the PSDU in the range of 0 to 262143. A value of zero indicates a packet in which no data part follows the SIG. | Y | Y |
| N_TX | Indicates the number of transmit chains. | Y | N |
| NSS | Indicatesthe number of spatial streams | Y | N |
| EXPANSION_MAT | EXPANSION_MAT_TYPE is COMPRESSED_SV Contains a set of compressed beamforming feedback matrices as defined in 26.6.7.12.4 (Compressed beamforming feedback matrix). The number of elements depends on the number of spatial streams and the number of transmit chains. EXPANSION_MAT_TYPE is NON_COMPRESSED_SV Contains a set of noncompressed beamforming feedback matrices as defined in1.6.8.12.3. The number of complex elements is, whereis the total number of subcarriers,is the number of columns, andis the number of rows in each matrix. EXPANSION_MAT_TYPE is CSI_MATRICES Contains a set of CSI matrices as defined in1.6.8.12.2. The number of complex elements is, whereis the total number of subcarriers,is the number of columns, andis the number of rows in each matrix. | Y | N |
| EXPANSION_MAT_TYPE | For QMG OFDM PPDU transmission and EXPANSION_MAT is present. Enumerated type: COMPRESSED_SV indicates that EXPANSION_MAT is a set of compressed beamforming feedback matrices. NON_COMPRESSED_SV indicates that EXPANSION_MAT is a set of noncompressed beamforming feedback matrices. CSI_MATRICES indicates that EXPANSION_MAT is a set of channel state matrices. | Y | N |
| CHAN_MAT | CHAN_MAT_TYPE is COMPRESSED_SV Contains a set of compressed beamforming feedback matrices as defined in 1.6.8.12.4 based on the channel measured during the training symbols of the received PPDU. The number of elements depends on the number of spatial streams and the number of transmit chains. CHAN_MAT_TYPE is NON_COMPRESSED_SV Contains a set of noncompressed beamforming feedback matrices as defined in 1.6.8.12.3 based on the channel measured during the training symbols of the received PPDU. The number of complex elements is, whereis the total number of subcarriers,is the number of columns, andis the number of rows in each matrix. CHAN_MAT_TYPE is CSI_MATRICES Contains a set of CSI matrices as defined in 1.6.8.12.2 based on the channel measured during the training symbols of the received PPDU. The number of complex elements is, whereis the total number of subcarriers,is the number of columns, andis the number of rows in each matrix. | N | Y |
| CHAN_MAT_TYPE | Enumerated type: COMPRESSED_SV indicates that CHAN_MAT is a set of compressed beamforming vector matrices. NON_COMPRESSED_SV indicates that CHAN_MAT is a set of noncompressed beamforming vector matrices. CSI_MATRICES indicates that CHAN_MAT is a set of channel state matrices. | N | Y |
| ADD-PPDU | Enumerated Type: - ADD-PPDU indicates that this PPDU is immediately followed by another PPDU with no IFS or preamble on the subsequent PPDU. - NO-ADD-PPDU indicates no additional PPDU follows this PPDU. | Y | Y |
| AGGREGATION | Indicates whether the PSDU contains an A-MPDU. Enumerated Type: - AGGREGATED indicates this is a packet with A-MPDU aggregation. - NOT_AGGREGATED indicates this is a packet without A-MPDU aggregation. | Y | Y |
| RSSI | The allowed values for the RSSI parameter are in the range from 0 through RSSI maximum. This parameter is a measure by the PHY of the power observed at the antennas used to receive the current PPDU. RSSI shall be measured during the reception of the PLCP preamble. RSSI is intended to be used in a relative manner, and it shall be a monotonically increasing function of the received power. | N | Y |
| REC_MCS | Indicates the MCS that the STA’s receiver recommends. | N | O |
| SNR | Contains an array of received SNR measurements for each spatial stream. SNR indications of 8 bits are supported. SNR shall be the sum of the decibel values of SNR per tone divided by the number of tones represented in each stream as described in 8.4.1.48. | N | Y |
| STBC | Indicates whether STBC is used. 0 indicates no STBC (NSTS=NSS in the Data field). 1 indicates STBC is used (NSTS=2NSS in the Data field). | Y | Y |
| RCPI | Is a measure of the received RF power measured over the preamble of a received frame. Refer to 21.3.10 for the definition of RCPI. | N | Y |
| TXOP_PS_NOT_ALLOWED | Indicates whether a QMG AP allows non-AP QMG STAs in TXOP power save mode to enter Doze state during the TXOP. 0 indicates that the QMG AP allows non-AP QMG STAs to enter doze mode during a TXOP. 1 indicates that the QMG AP does not allow non-AP QMG STAs to enter doze mode during a TXOP. | Y | Y |
| PARTI AL_AID | Provides an abbreviated indication of the intended recipient(s) of the PSDU (see 9.17a). Integer: range 0-511 if UPLINK_INDICATION is 1, and range 0-63 if UPLINK_INDICATION is 0. | Y | Y |
| UPLINK_INDICATION | Set to 1 if the QMG PPDU is addressed to AP. Set to 0 otherwise (See 9.17a). | Y | Y |
| COLOR | If UPLINK_INDICATION is 0, set to a value of its choosing within the range 0 to 7 and shall maintain that value for the duration of the existence of the BSS. Otherwise, not present (See 9.17a). | Y | Y |
| CH_BANDWIDTH | Indicates the channel width of the transmitted PPDU: Enumerated type: CBW540 for 540 MHz CBW1080 for 1080 MHz | Y | Y |
| DYN_BANDWIDTH | In TXVECTOR, if present, indicates whether the transmitter is capable of Static or Dynamic bandwidth operation. In RXVECTOR, if valid, indicates whether the transmitter is capable of Static or Dynamic bandwidth operation. Enumerated type: Static if the transmitter is capable of Static bandwidth operation Dynamic if the transmitter is capable of Dynamic bandwidth operation NOTE—In the RXVECTOR, the validity of this parameter is determined by the MAC based on the contents of the received MPDU. | O | Y |
| BEAMFORMED | Set to 1 if a beamforming steering matrix is applied to the waveform in a QMG OFDM PHY transmission as described in 20.3.11.11.2. Set to 0 otherwise. NOTE—When BEAMFORMED is set to 1, smoothing is not recommended. | Y | O |
| TIME_OF_DEPARTURE_REQUESTED | Enumerated type: - TRUE indicates that the MAC entity requests that the PHY PLCP entity measures and reports time of departure parameters corresponding to the time when the first frame energy is sent by the transmitting port. - FALSE indicates that the MAC entity requests that the PHY PLCP entity neither measures nor reports time of departure parameters. | O | N |
| RX_START_OF_FRAME_OFFSET | 0 to 232-1. An estimate of the offset (in 10 nanosecond units) from the point in time at which the start of the preamble corresponding to the incoming frame arrived at the receive antenna port to the point in time at which this primitive is issued to the MAC. | N | Y |
| PACKET_TYPE | Enumerated Type: - TRN-R-PACKET indicates either a packet whose data part is followed by one or more TRN-R subfields, or a packet that is requesting TRN-R subfields to be appended to a future response packet. - TRN-T-PACKET indicates a packet whose data part is followed by one or more TRN-T subfields. This field is reserved if TRN-LEN is 0. | Y | Y |
| BEAM_TRACKING_REQUEST | This parameter indicates whether beam tracking is requested. Enumerated type: Beam tracking requested or Beam tracking not requested | Y | Y |
| LAST_RSSI | In the TXVECTOR, LAST_RSSI indicates the received power level of the last packet with a valid PHY SIG that was received a SIFS period before transmission of the current packet; otherwise, it is 0 (9.3.2.3.3). In the RXVECTOR, LAST_RSSI indicates the value of the LAST_RSSI field from the PCLP header of the received packet. Valid values are integers in the range of 0 to 15: - Values of 2 to 14 represent power levels (–71+value×2) dBm. - A value of 15 represents power greater than or equal to –42 dBm. - A value of 1 represents power less than or equal to –68 dBm. - A value of 0 indicates that the previous packet was not received a SIFS period before the current transmission. | Y | Y |
| Turnaround | Set to 1 or 0 as specified in 9.3.2.3.3. | Y | Y |
| NOTE 1—In the “TXVECTOR” and “RXVECTOR” columns, the following apply: Y = Present; N = Not present; O = Optional; |  |  |  |

## Common Parameters

### Introduction

This subsection introduces some common parameters.

### Common requirements

#### Introduction

This subclause describes the common requirements for all 3 PHYs: Control PHY, SC PHY and OFDM PHY.

#### Transmit RF Delay

As defined at 18.3.8.6 and its value is implementation dependent.

#### Center frequency tolerance

##### General

The transmitter center frequency tolerance shall be ± 20 ppm maximum.

##### Center frequency convergence

The transmitter center frequency shall converge to within 1ppm of its final value within 2.5 µs from the start of the packet.

##### Symbol clock tolerance

The symbol clock frequency tolerance shall be ± 20 ppm maximum.

The transmit center frequency and the symbol clock frequency shall be derived from the same reference oscillator.

##### Transmit rampup and rampdown

The transmit power-on ramp is defined as the time it takes for a transmitter to rise from less than 10% to greater than 90% of the average power to be transmitted in the frame.

The transmit power-on ramp shall be less than 30 ns.

The transmit power-down ramp is defined as the time it takes the transmitter to fall from greater than 90% to less than 10% of the maximum power to be transmitted in the frame.

The transmit power-down ramp shall be less than 30 ns.

##### Maximum input requirement

The receiver maximum input level is the maximum power level at the receive antenna(s) of the incoming signal, in dBm, present at the input of the receiver antenna for which the error rate criterion (defined in 21.3.3.9) is met. A compliant receiver shall have a receiver maximum input level at the receive antenna(s) of at least 10 microwatts/cm2 for each of the modulation formats that the receiver supports.

##### Receive sensitivity

The packet error ratio (PER) shall be less than 10% for a PSDU length of 4096 octets with the ratedependent input levels listed in Table 26-.

Table 26- Receiver sensitivity

| MCS Index | BW 540MHz | BW 1080MHz |
| --- | --- | --- |
| 1 | -74 | -71 |
| 2 | -69 | -66 |
| 3 | -67 | -64 |
| 4 | -61 | -58 |
| 5 | -59 | -56 |
| 6 | -55 | -52 |
| 7 | -53 | -50 |
| 8 | -51 | -48 |
| 9 | -74 | -71 |
| 10 | -69 | -66 |
| 11 | -66 | -63 |
| 12 | -64 | -61 |
| 13 | -60 | -57 |
| 14 | -57 | -54 |
| 15 | -55 | -52 |
| 16 | -53 | -50 |

### Time-related parameters

Table 26-  defines the timing-related parameters for the QMG format。

Table 26-  Time-related parameters

| Parameter | CBW 540 MHz | CBW 1080 MHz | Description |
| --- | --- | --- | --- |
|  | 168 | 336 | Number of complex data numbers per OFDM symbol |
|  | 8 | 16 | Number of pilot subcarriers per OFDM symbol |
|  | 3 | 3 | Number of DC subcarriers per OFDM symbol |
|  | 179 | 355 | Total number of subcarriers per OFDM symbol |
|  | 89 | 177 | Highest data subcarrier index per OFDM symbol |
|  | 660MHz | 1320MHz | OFDM sample rate |
|  | 1.51515ns=1/ | 0.757575ns=1/ | OFDM sample time |
|  | 440MHz =2/3 | 880MHz =2/3 | SC chip rate |
|  | 2.272ns=1/ | 1.136ns=1/ | SC chip time |
| ΔF | 2.578125MHz |  | Subcarrier frequency spacing |
|  | 387.8787ns |  | IDFT/DFT period |
|  | 96.969696ns= /4 |  | Normal Guard interval duration |
|  | 48.484848ns= /8 |  | Short Guard interval duration |
|  | 484.8==1.25 |  | Long GI Symbol interval |
|  | 436.4==1.125 |  | Short GI Symbol interval |
|  | or depending on the GI used (see Table 22-8) |  | Symbol interval |
|  | 581.8ns=256 |  | ZCZ block duration |
|  | 8145.5 ns= |  | Short training field duration for Control PHY |
|  | 5818 ns= |  | Short training field duration for SC/OFDM PHY |
|  | 2327.3 ns= |  | Channel estimation field duration |
|  | 2327.3 ns = |  | SIGNAL field duration |
|  | 484.8 ns= |  | Duration of each OSTF field |
|  | 484.8 ns= |  | Duration of each OCEF field |

Table 26-  defines parameters used frequently in Clause 26.

Table 26-  Frequently used parameters

| Symbol | Explanation |
| --- | --- |
|  | Number of the blocks for SC transmission. |
|  | Number of OCEF symbols (see 22.3.8.3.5). |
|  | Number of coded bits per block. |
|  | For OFDM Transmission: Number of coded bits per OFDM symbol. For SC Transmission: Number of coded bits per constellation symbol. |
|  | Number of coded bits per symbol per spatial stream. |
|  | Number of data bits per symbol. |
|  | Number of data constellation symbols per block for SC transmission |
|  | Length of the UW in each block for SC transmission. |
|  | Number of the symbols in each block for SC transmission. |
|  | Number of coded bits per subcarrier per spatial stream. |
|  | Number of receive chains. |
|  | For pre-QMG modulated fields, = 1 (see NOTE 2). For QMG modulated fields,is the number of space-time streams. |
|  | Number of OCEF symbols |
|  | Number of spatial streams. |
|  | Number of transmit chains. |
|  | The coding rate. |
| NOTE 1—Pre-QMG modulated fields refer to the STF, CEF, and QMG SIG fields, while QMG modulated fields refer to the OCEF, and Data fields (see Figure 22-17). |  |

### Mathematical conventions in the signal description

#### Notation

indicates the element in rowand columnof matrix.

#### Subcarrier indices in use

For a 540 MHz OFDM PPDU transmission, the 540 MHz is divided into 256 subcarriers. The signal is transmitted on subcarrier -89 to -2 and 2 to 89, with -1, 0, and 1 being the center (DC) subcarriers.

For a 1080 MHz OFDM PPDU transmission, the 1080 MHz is divided into 512 subcarriers. The signal is transmitted on subcarrier -177 to -2 and 2 to 177, with -1, 0, and 1 being the center (DC) subcarriers.

#### Transmitted signal

The transmitted signal is described in complex baseband signal notation. The actual transmitted signal is defined by the complex baseband signal in Equation.

where

represents the real part of a complex variable;

represents the complex baseband signal in transmit chain;

represents the center frequency of the portion of the transmitted PDU.

The transmitted RF signal is derived by up-converting the complex baseband signal, which consists of several fields. The fields and timing boundaries for the various fields are shown in Figure 26-  to Figure 26-3.

Figure 26-  Packet structure for the SC PPDU with 540MHz

Figure 26-  Packet structure for the SC PPDU with 1080MHz

Figure 26-  Packet structure for the OFDM PPDU

The time offset,, determines the starting time of the corresponding field.

For the SC transmission with 540MHz,

For the SC transmission with 1080MHz,

For the OFDM transmission,

where

Each OFDM baseband waveform, for the fields above, is defined via the inverse discrete Fourier transform (IDFT) as:

In Equation the following notions are used:

For 540 MHz transmission,. For 1080 MHz transmission,.

The number of the space time stream

is the guard interval duration used for each OFDM symbol

is the spatial mapping matrix withrows andfor the subcarrier.

is a windowing function. An example function,, is given in 26.3.4.5.

is defined in 26.3.4.4.

is the subcarrier frequency spacing given in Table 26-3.

represents the cyclic shift per space-time stream, whose value is defined in Table 26-31.

is the frequency-domain symbol in subcarrierof space-time stream. Some of the  withinhave a value of 0, such as the DC tones.

The base band waveform for fields defined by time waveform for fields defined by time domain sequence for single carrier transmission is:

where is the constellation point.

Conversion from the sampled digital domain to the continuous time domain is beyond the scope of this document. Filtering for pulse shaping such as in GMSK is beyond the scope of this standard.

#### Definition of the tone rotation

The functionis used to represent a rotation of the tones. BW in  is determined by the TXVECTOR parameter CH_BANDWIDTH as defined in Table 26- .

Table 26-  CH_BANDWIDTH and

| CH_BANDWIDTH |  |
| --- | --- |
| CBW540 |  |
| CBW1080 |  |

For a 540 MHz PPDU transmission,

For a 1080 MHz PPDU transmission,

#### The windowing function

The windowing functionis used to smooth the transition between adjacent fields in the packet where OFDM modulation is employed. No windowing is applied to preamble fields or to SC modulated fields. The windowing function is different from being equivalent to “1” only in the transition region.

An example of a windowing function is given by

The transition region creates an overlap (with length TR) between adjacent fields. The field wave form is extended cyclically to fill the part of the transition region in which it is undefined. If the transition region vanishes (i.e., TR=0), the windowing function degenerates to a rectangular window. The choice of windowing function is implementation dependent, as long as transmit EVM and transmit mask requirements are met.

### QMG PHY Preamble

#### General

The QMG PHY preamble is the part of the PHY PPDU that is used for packet detection, AGC, frequency offset estimation, synchronization, indication of PHY frame type (Control, SC, or OFDM), bandwidth indication (540 MHz or 1080 MHz) and channel estimation.

Note: For MIMO transimission, different transmit antenna transmit a different preamble which is composed of one of the four ZCZ sequences with the order of , such as for the case of one tramsmit antenna, the preamble sequence is composed of , for the case of two transmit antennas, the preamble sequences are composed of, respectively.

#### QMG Control PHY STF

The Short Training field is composed of 14 repetitions of the sequenceof length 256. The sequence is defined in 26.8.

The waveform for the Short Training field is

where  is SC chip time of CBW 540 MHz and mod is the modulus operation.

#### QMG Control PHY CEF

The Channel Estimation field is composed of four concatenated  sequences where each sequence is multiplied by an individual sign assignment. As shown in Figure 26- , the CEF of control PHY is defined as  where the sequence  is defined in 26.8.

The waveform for the Channel Estimation field is

where  is SC chip time of CBW 540 MHz and mod is the modulus operation. The Control PHY preamble is illustrated in Figure 26- .

Figure 26-  Control PHY Preamble

Note: All fields are transmitted with SC transmission. For 1080MHz SC transmission, the STF fields, the CEF fields and the SIG fields are transmitted with duplicated style.

#### QMG SC and OFDM PHY STF

The Short Training fields of SC and OFDM PHY share the same definition. The common Short Training field is composed of 10 repetitions of the sequenceof length 256. The sequence is defined in 26.8.

The waveform for the Short Training field is

where  is SC chip time of CBW 540 MHz and mod is the modulus operation.

#### QMG SC PHY CEF

The Channel Estimation field of SC PHY is also composed of four concatenated  sequences but is with different sign assignment patterns. As shown in Figure 26-  and Figure 26- , the CEF of SC 540 MHz PHY is defined as  and the CEF of SC 1080 MHz PHY is defined as  where the sequenceis defined in 26.8.

The waveform for the Channel Estimation field of SC 540 MHz PHY is

and the waveform for the Channel Estimation field of SC 1080 MHz PHY is

where  is SC chip time of CBW 540 MHz and mod is the modulus operation.

Figure 26-  QMG SC PHY Preamble for 540 MHz

Figure 26-  QMG SC PHY Preamble for 1080 MHz

The SC PHY preambles are illustrated in Figure Figure 26-  and Figure 26- .

Note: All fields are transmitted with SC transmission. For 1080MHz SC transmission, the STF fields, the CEF fields and the SIG fields are transmitted with duplicated style.

#### QMG OFDM PHY CEF

The Channel Estimation field of OFDM PHY is also composed of four concatenated  sequences but is with different sign assignment patterns. As shown in Figure 26-  and Figure 26- , the CEF of OFDM 540 MHz PHY is defined as

and the CEF of OFDM 1080 MHz PHY is defined as

where the sequence  is defined in 26.8.

The waveform for the Channel Estimation field of OFDM 540 MHz PHY is

and the waveform for the Channel Estimation field of OFDM 1080 MHz PHY is

where  is SC chip time of CBW 540 MHz and mod is the modulus operation.

Figure 26-  QMG OFDM PHY Preamble for 540 MHz

Figure 26-  QMG OFDM PHY Preamble for 1080 MHz

The OFDM PHY preambles are illustrated in Figure 26-  and Figure 26- .

Note: The STF fields, the CEF fields and the SIG fields are transmitted with SC transmission. For 1080MHz SC transmission, the STF fields, the CEF fields and the SIG fields are transmitted with duplicated style.

### CRC calculation

The value of cyclic redundancy check (CRC) field is the ones complement Equation

where

;

is the number of bits over which the CRC is generated;

is bitof the input bit sequence;

are initialized values that are added modulo 2 to the first  bits of the input bit sequence;

is the length of CRC sequence.  for CRC of the SIG, and for the data field;

is the CRC generating polynomial;

.

Figure 26-  shows the operation of the CRC. First, the shift register is reset to all ones. The bits are then passed through the XOR operation at the input. When the last bit has entered, the output is generated by shifting the bits out of the shift register, first, through an inverter.

Figure 26-  CRC Calculation

is the CRC generating polynomial of 8 bits, and the CRC sequence is.

Figure 26-  shows the operation of the CRC with 8 bits.

Figure 26-  8 bits CRC Calculation

### Scrambler

The QMG Control PHY SIG, QMG SC/OFDM PHY SIG, and the data fields following the scrambler initialization field (including data padding bits) shall be scrambled by XORing each bit in turn with a length 127 periodic sequence generated by the polynomial S(x)=. The octets of the PSDU and the pad bits shall be placed into a bit stream with bit 0 (LSB) of each octet first and bit 7 of each octet (MSB) last. The generation of the sequence and the XOR operation are defined in Figure 26-.

For each PPDU, the transmitter shall select a non-zero seed value for the scrambler (bitsthrough). The seed value should be selected in a pseudo random fashion. The seed value is sent in the Scrambler Initialization field of the PLCP header. Each data bit in the data field of the PPDU is then XORed with the scrambler output (XOR) and the scrambler content shiftes once.

Figure 26- Scrambler

### Common LDPC parity matrices

#### Introduction

Transmission/reception of LDPC with codeword length 672 bits (expansion factor ) is mandatory, and transmission/reception of LDPC with codeword length 2016 bits (expansion factor ) is optional.

#### General

The structured LDPC code is defined by a parity check matrix  of size, which is determined by a base matrixof size, an expansion factor  and a base permutation matrix of size . The number of information bits is, the number of codeword bits is , then code rate. If each element  in the base matrix  is replaced by zero sub-block matrix of size  or the sub-block matrix, the parity check matrix can be obtained.

The base matrix  is defined as follows:

The parity check matrix  is defined as follows:

where, if, equals a zero matrix of size ; otherwise,  equals the base permutation matrix  to  power. The base permutation matrix  of size  is defined as follows:

#### Rate-1/2 LDPC code matrix

Table 26- Rate-1/2 LPDC code matrix

| -1 | 0 | -1 | 0 | -1 | 0 | -1 | 0 | 0 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -1 | -1 | 34 | -1 | 12 | -1 | 36 | 18 | 0 | -1 | -1 | -1 | -1 | -1 | -1 |
| 8 | -1 | 0 | -1 | 0 | -1 | 0 | -1 | -1 | 13 | 0 | -1 | -1 | -1 | -1 | -1 |
| -1 | 16 | 40 | -1 | 32 | -1 | 22 | -1 | -1 | -1 | 19 | 0 | -1 | -1 | -1 | -1 |
| -1 | 20 | -1 | 22 | -1 | 2 | -1 | 28 | 32 | -1 | -1 | 21 | 0 | -1 | -1 | -1 |
| 30 | -1 | 18 | -1 | -1 | 14 | -1 | 30 | -1 | 37 | -1 | -1 | 31 | 0 | -1 | -1 |
| 40 | -1 | 12 | -1 | 38 | -1 | 6 | -1 | -1 | -1 | 26 | -1 | -1 | 13 | 0 | -1 |
| -1 | 24 | -1 | 20 | 10 | -1 | 2 | -1 | -1 | -1 | -1 | 18 | -1 | -1 | 5 | 0 |

#### Rate-5/8 LDPC code matrix

Table 26- Rate-5/8 LPDC code matrix

| -1 | 0 | -1 | 0 | 0 | 0 | 0 | 0 | 0 | -1 | 0 | -1 | -1 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -1 | 0 | -1 | 32 | -1 | 22 | -1 | 18 | 0 | 19 | 0 | -1 | -1 | -1 | -1 |
| 8 | 16 | 40 | 34 | -1 | 12 | -1 | 36 | 32 | -1 | -1 | 21 | 0 | -1 | -1 | -1 |
| 30 | 20 | 18 | 22 | 38 | -1 | 6 | -1 | -1 | 13 | -1 | -1 | 31 | 0 | -1 | -1 |
| -1 | 24 | -1 | 20 | -1 | 2 | -1 | 28 | 16 | 37 | -1 | -1 | -1 | 13 | 0 | -1 |
| 40 | -1 | 12 | -1 | 10 | 14 | 2 | 30 | -1 | 19 | -1 | -1 | -1 | -1 | 5 | 0 |

#### Rate-3/4 LDPC code matrix

Table 26-  Rate-3/4 LPDC code matrix

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -1 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 16 | 40 | 34 | 32 | 12 | 22 | 36 | 18 | 13 | 19 | 0 | -1 | 0 | -1 | -1 |
| 30 | 20 | 18 | 22 | 38 | 2 | 6 | 28 | 32 | 37 | 26 | 21 | 31 | -1 | 0 | -1 |
| 40 | 24 | 12 | 20 | 10 | 14 | 2 | 30 | 16 | 19 | 34 | 18 | -1 | 13 | 5 | 0 |

#### Rate-13/16 LDPC code matrix

Table 26- Rate-13/16 LPDC code matrix

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | -1 | -1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30 | 20 | 18 | 22 | 38 | 2 | 6 | 28 | 32 | 37 | 26 | 21 | 34 | -1 | 0 | -1 |
| 40 | 24 | 12 | 20 | 10 | 14 | 2 | 30 | 16 | 19 | 34 | 18 | 8 | 13 | 5 | 0 |

### QMG PHY SIG

#### General

The QMG SIG field carries information required to interpret PPDUs. The preamble and the QMG SIG fields are all transmitted using SC modulation.  The structure of the QMG PHY SIG field is shown in Figure 26-12.

Figure 26-  QMG PHY SIG Structure

The QMG SIG field contains the fields listed in Table 26- . The mapping of the fields is also described in Table 26- .

Table 26-  Fields in the QMG PHY SIG

| Bit | Fields | Number of Bits | Description |
| --- | --- | --- | --- |
| B0-B6 | Scrambler Initialization | 7 | The initial scrambler state. |
| B7 | Short GI | 1 | Set to 0 if short guard interval is not used in the Data field. Set to 1 if short guard interval is used in the Data field. |
| B8 | Uplink Indication | 1 | Set to the value of the TXVECTOR parameter UPLINK_INDICATION.. |
| B9-B17 | PAID | 9 | If Uplink Indication is not present or set to 1, set to the value of the TXVECTOR parameter PARTIAL_AID. PARTIAL_AID provides an abbreviated indication of the intended recipient(s) of the PSDU (see 9.17a). If Uplink Indication is set to 0, B9-B11 are set to the value of the TXVECTOR parameter COLOR and B12-B17 are set to the value of the TXVECTOR parameter PARTIAL_AID. |
| B18-B35 | Length | 18 | Number of data octets in the PSDU, Range 0-262143; |
| B36-B39 | Last RSSI | 4 | For SC/OFDM PHY: Contains a copy of the parameter LAST_RSSI from the TXVECTOR. When set to 0, this field is reserved and ignored by the receiver. The value is an unsigned integer: - Values of 2 to 14 represent power levels (–71+value×2) dBm. - A value of 15 represents a power greater than or equal to –42 dBm. - A value of 1 represents a power less than or equal to –68 dBm. Value of 0 indicates that the previous packet was not received a SIFS period before the current transmission. For Control PHY: Reserved. |
| B40-B41 | Spreading Factor/Aggregation/ Additional PPDU | 2 | For Control PHY: Set to 0: spreading by 13 Set to 1: spreading by 7 Set to 2: spreading by 4 Set to 3: no spreading For SC/OFDM PHY: Aggregation: Set to 1 indicate that the PPDU in the data portion of the packet contains an A-MPDU; otherwise, set to 0. Additional PPDU: Contains a copy of the parameter ADD-PPDU from the TXVECTOR. A value of 1 indicates that this PPDU is immediately followed by another PPDU with no IFS or preamble on the subsequent PPDU. A value of 0 indicates that no additional PPDU follows this PPDU. |
| B42-B46 | Training Length | 5 | For SC/OFDM PHY: Corresponds to the TXVECTOR parameter TRNLEN. If the Beam Tracking Request field is 0, the Training Length field indicates the length of the training field. The use of this field is defined in 21.10.2.2.3. A value of 0 indicates that no training field is present in this PPDU. If the Beam Tracking Request field is 1 and the Packet Type field is 10, the Training Length field indicates the length of the training field. If the Packet Type field is 11, the Training Length field indicates the length of the training field requested for receive training. For Control PHY: Reserved. |
| B47 | Beam Tracking Request | 1 | Corresponds to the TXVECTOR parameter BEAM_TRACKING_REQUEST. Set to 1 to indicate the need for beam tracking (9.35.7); otherwise, set to 0. The Beam Tracking Request field is reserved when the Training Length field is 0. |
| B48 | Codeword Length | 1 | Set to 0 indicate that the codeword length is 672; Set to 1 indicate that the codeword length is 2016; |
| B49 | TXOP_PS_NOT_ALLOWED | 1 | Set to 0 by QMG AP if it allows non-AP QMG STAs in TXOP power save mode to enter Doze state during a TXOP. Set to 1 otherwise. The bit is reserved and set to 1 in QMG PPDUs transmitted by a non-AP QMG STA. |
| B50-B54 | MCS | 5 | Index into the Modulation and Coding Scheme table. |
| B55-B56 | Packet Type | 2 | When the Training Length field is nonzero, corresponds to the TXVECTOR parameter PACKET-TYPE. - Packet Type = 11 indicates either a packet whose data part is followed by one or more TRN-R subfields, or a packet that is requesting TRN-R subfields to be appended to a future response packet. - Packet Type = 10 indicates a packet whose data part is followed by one or more TRN-T subfields. When the Training Length field is zero and Packet Type = 01 indicates that PPDU is a sounding PPDU. This field is reserved when the Training Length field is zero and Packet Type = 00. |
| B57-B60 | Spatial Expansion | 4 | For Control PHY/SC PHY: Index into the corresponding spatial expansion method defined in Table 26- . |
|  | DB | 1 | For OFDM PHY: If the PPDU is transmitted on 1080MHz channel: set DB to 1 for allowing dynamic bandwidth; set to 0 for non-allowing dynamic bandwidth. Otherwise set to 0; (Reserved) For Control PHY, SC PHY: Reserved. |
|  | STBC | 1 | For OFDM PHY: Set to 1 if space time block coding is used and otherwise set to 0. |
|  | NSS | 2 | For OFDM PHY: Set to 0 for 1 spatial stream. Set to 1 for 2 spatial streams. Set to 2 for 3 spatial streams. Set to 3 for 4 spatial streams. |
| B61 | Turnaround | 1 | As defined in Table 21-1. |
| B62-B63 | Reserved | 2 | Reserved. |
| B64-B79 | CRC | 16 | QMG SIG check sequence. Definition of this field calculation is in 1.3.6. |
| Note 1--A value of 0 in the QMG Length field indicates a PPDU that does not include a data field, i.e., NDP. NDP transmissions are used for sounding purposes only (see 9.21.2). The packet ends after the last QMG OCEF or the QMG SIG. |  |  |  |

All the numeric fields are encoded in unsigned binary, least significant bit first.

Reserved bits are set to 0 by the transmitter and shall be ignored by the receiver.

#### Encoding of SIG

The scrambled SIG sequence consists of 80 bits, shown as  and is encoded with LDPC of rate-1/2 and length-672 as follows:

- The scrambled SIG sequence is padded with 4 zeros bits to create 84 bits. The 84-bits sequence include two contiguous subsequences with length of 42 bits, shown as , and the two subsequences are repeated once respectively to obtain 168 bits, such as . Then, the repeated data sequence is padded with 168 zero bits to create 336 bits.

- Then, using rate-1/2 LDPC base matrix and setting , the 336-bits sequence is added with parity bits  to create LDPC codeword, shown as , such that , where  is revised LDPC parity check matrix.

- The basic code is. Using Rate-matching to obtain transmitting sequence of 1024 bits: .

### Transmission of the preamble and the SIG field on 1080 MHz channel

The STF fields , the CEF fields, and the SIG fields that are transmitted using SC modulation are specified at the SC chip rateMHz. For transmission on the 1080 MHz channel, the signal is resampled with a 3 rate change, applied appreciate phase rotation for each 540 MHz subband and then summed to form the transmitted signal.

Let, where is described in 1.3.5.

Then:

where is the length of the filter defined in 1.3.11. The transmitted signal is given as

whereMHz.

### definition

The filteris defined by the following coefficients (fromto):

[1,-6	,-5,3,8,0,-8,-4,6,8,-4,-13,	-2,16,14,-9,-23,-4,23,15,	-20,-27,20,57,4,-99,	-102,70,247,144,-264,-558,-175,991,2327,2917,2327,991,-175,-558,-264,144,247,70,-102,	-99,4,57,20,-27,-20,	15,23,-4,-23,-9,14,16,-2,-13,-4,8,6,-4,-8,0,8,3,-5,-6,1].

Normalized to have a norm ofso that.

### Encoding of Data

The scrambled data fieldis encoded with LDPC codeword length of  bits, information length of  bits and expansion factor of  as follows:

- is the maximal number of data bits in each LDPC codeword. The number of LDPC codeword is calculated as.

- The number of data bits in the  LDPC codeword is:

where,

is smallest integer which is larger than or equal to real number ,  is modulus operation.

- The data field is broken into  blocks of data word,. Each data word is added with 8 bits CRC sequence, then padded with  zero bits to create  bits, such as.

- For each data word,  parity bits are added to create the LDPC codeword , such that .

- Packet encoding: The  bit set which is composed of all the jth bits of  LDPC codeword is encoded with 1-bit parity check to create a parity bit ,. All the parity bits () are combined into a parity data word  with length of  bits. That is, .

- Bits selection parameter: the punctured-bit index set of  codeword which are not belong to padding bits are (), and the punctured-bit index set of parity data word  is . The intersection of any two different sets of () is empty set. The length of  is , such as: (where, )

- if :;

- if :, ;

- .if, where, ,.

- Bits selection: the first element of  is equal to the last element of  plus 1, such as, , where,  is the length of padding bits for the  codeword. Remove the zero padding bits and the punctured bits from  to obtain codeword, where. The punctured-bit index set of parity data word  is. Remove the padding-bit-generated bits and the punctured bits from  toobtain, where, the padding-bit-generated bits are totally generated from the zero padding bits.

Example:

The data field contains  bytes. The parameter of LDPC:, , , .

- 、，the number of LDPC codeword: ；

- 、， ，；that is, the number of data bits in the  LDPC codeword is 316, and the  LDPC codeword is 315.

- 、 The data field is broken into  blocks of data word. Each data word is added with 8 bits  sequence. Then padded with zero bits: the  data word is padded with 12 bits and the  data word are padded with 13 bits.

- 、All the data word are encoded to obtain LDPC codeword，.

- 、Packet encoding: Encoded with 1-bit parity check to create , length of 672 bits.

- 、 Bits selection parameter: because of , , and , .

- 、 Bits selection: Remove the zero padding bits  and the punctured bits  from  to obtain codeword  with length of 618 bits; Remove the zero padding bits  and the punctured bits  from  to obtain codeword  with length of 617 bits; the same operating to obtain . Remove the padding-bit-generated bits  and the punctured bits  from  to obtain codeword  with length of 546 bits.

### Received channel power indicator (RCPI) measurement

The RCPI is a measure of the received RF power in the selected channel. This parameter shall be measured by the PHY of the received RF power in the channel measured over the data portion of the received frame. RCPI shall be a monotonically increasing, logarithmic function of the received power level defined in dBm. The allowed values for the Received Channel Power Indicator (RCPI) parameter shall be an 8 bit value in the range from 0 to 220, with indicated values rounded to the nearest 0.5 dB as follows:

- 0: Power < –110 dBm

- 1: Power = –109.5 dBm

- 2: Power = –109.0 dBm

- And so on up to:

- 220: Power > 0 dBm

- 221–254: reserved

- 255: Measurement not available

where RCPI = int{(Power in dBm +110) × 2} for 0 dBm > Power > –110 dBm.

RCPI shall equal the received RF power with an accuracy of ± 5 dB (95% confidence interval) within the specified dynamic range of the receiver. The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel width multiplied by 1.1. The relative error between RF power measurements made within a 1 second interval should be less than ± 1 dB.

## QMG Control PHY

### Introduction

Transmission and reception of Control PHY PPDUs is mandatory. Control PHY uses the same chip rate as SC PHY. Control PHY is transmitted when the TXVECTOR indicates MCS 0.

The modulation and coding scheme for Control PHY is shown in Table 26-11.

Table 26-  Modulation and coding scheme for the control PHY

| MCS Index | Modulation | Code rate | Data rate |
| --- | --- | --- | --- |
| 0 | BPSK | 1/2 | 25.38 Mbps for SF 13, 47.14 Mbps for SF 11, 82.5 Mbps for SF 4 and 330 Mbps for no spreading |

### Frame format

The Control PHY frame is composed of the QMG Control PHY STF, QMG Control PHY CEF, QMG Control PHY SIG, QMG control PHY Data field, and possibly AGC and TRN-R/T subfields, which is shown in Figure 26- .

Figure 26-  QMG Control PHY PPDU format

### QMG Preamble Transmission

In Control PHY preamble is transmitted using SC transmission. See 1.3.5.

### QMG Control PHY SIG Transmission

#### General

QMG Control PHY SIG and data are modulated by binary phase shift keying (BPSK) and are spread by Barker sequences.

#### Generation of CRC bits

The CRC sequence is calculated over bits 0-23 and uses CRC 16-CCITT as described in 1.3.6.

#### Scrambler

The operation of the scrambler is defined in 1.3.7. Bits x1, x2, x3, x4, x5, x6, x7 of the scrambler shift register are initialized using the bits in the scrambler initialization bits from the SIG. The SIG is scrambled starting from bit 7.

#### Encoding

Please see 1.3.9.2.

#### Modulation

The QMG Control PHY SIG is modulated using BPSK modulation, the input bit stream is mapped according to the following equation. Each output symbol is then rotated according to the following equation:.

Figure 26-  Constellation bit encoding for BPSK

#### Spreading

The constellation points are spread using the Barker sequence Barker(L) defined as follows in Table 26-  The Barker(L) sequences where L is the spreading length.

Table 26-  The Barker(L) sequences

| Length L | Barker (L) Sequences |
| --- | --- |
| 4 | +1 ,+1 ,-1 ,+1 |
| 7 | +1, +1, +1 ,−1 ,−1 ,+1, −1 |
| 13 | +1 ,+1 ,+1, +1, +1 ,−1 ,−1 ,+1, +1, −1 ,+1 ,−1 ,+1 |

The waveform for the modulated and spread data field is

where means the largest integer smaller than a real number,is the modulated constellationpoint, and  is the SC chip time of CBW 540 MHz.

For QMG Control PHY SIG, spreading factor is always Barker(13)

### QMG Control PHY Data field

#### General

The Data field consists of the payload data of the PSDU. The PSDU is scrambled, encoded, modulated and spread as described in the following subclauses.

#### Scrambler

The operation of the scrambler is defined in 1.3.7. The scrambling of the data field continues the scrambling of the QMG Control PHY SIG with no reset.

#### Encoder

Use the LDPC of rate-1/2 and length-672. Please see 1.3.12.

#### Modulation

See 1.4.4.5.

#### Spreading

Spreading factor of the QMG Control PHY data field is indicated in the Control PHY SIG. See 1.4.4.6.

### QMG Control PHY Performance requirements

#### Transmit requirements

##### Introduction

Transmitter performance requirements of the Control PHY are defined in 1.4.6.1.2.

##### Transmit EVM

The transmit EVM accuracy test shall be performed by instrumentation capable of converting the transmitted signal into a stream of complex samples, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc. The instrumentation shall perform carrier lock, symbol timing recovery, and amplitude adjustment while making the measurements. The instrumentation shall incorporate a rake receiver or equalizer to minimize error resulting from multipath. If used, the equalizer shall be trained using information in the preamble (STF/CEF). For QMG Control PHY EVM, the signal is first de-spread using the spreading Barker sequence, the EVM is then calculated on the resulting symbols according to the formula below:

whereis the number of samples to be measured andshall be 1000, is the average power of the constellation,  is the complex coordinates of the measured symbol,  is the complex coordinates of the ideal constellation point for the measured symbol.

The test equipment should use a root-raised cosine filter with roll-off factor of 0.25 for the pulse shaping filter when conducting EVM measurement.

The transmit pulse shaping used is left to the implementer.

The relative constellation error (EVM) shall not exceed an MCS dependent value according toTable 26- .

Table 26-  EVM requirement for Control PHY

| MCS index | Discription | EVM value [dB] |
| --- | --- | --- |
| 0 | Control PHY Modulation | -6 |

#### Receive requirements

##### Introduction

Subclause 1.4.6.2.2 describes the performance requirement from the Control PHY receiver.

##### CCA

The start of a valid QMG Control PHY transmission at a receive level greater than the minimum sensitivity for control PHY (–78 dBm) shall cause CCA to indicate busy with a probability > 90% within 3 μs.

## QMG SC PHY

### Introduction

Transmission and reception of QMG SC PHY PPDUs transmitted by QMG SC PHY style is mandatory for MCSs 1-3. Transmission and reception of QMG SC PHY PPDUs transmitted by QMG SC PHY MIMO style may be supported.

### Transmitter block diagram

Figure 26-15 shows the transmit process for the QMG SC PHY SIG transmission. The actual structure of the transmitter is implementation dependent.

Figure 26-  Transmitter block diagram for QMG SC PHY SIG fields

Figure 26-  shows the transmit block used to generate the data fields of 540HMz and 1080 MHz using SC transmission.

Figure 26-  Transmitter block diagram for data fields of QMG SC PHY PPDUs

Note: The operations denoted by the dots line arrow and dots line boxes in the above two figures are optional which are applied for SC MIMO transmission.

### Overview of  QMG SC PHY PPDU encoding process

#### Construction of QMG SC PHY SIG

The construction of the QMG SC PHY SIG field proceeds as follows:

- Obtain the parameters from TXVECTOR. Add the reserved bits, append the calculated CRC. The calculation of the CRC of the QMG SC PHY SIG is described in 1.3.6.

- Scrambler: Scramble the QMG SC PHY SIG bits. The Scrambler is described in 1.3.7.

- LDPC encoder: The scrambled bits are encoded using the LDPC code as described in 1.5.5.4.1.

- Constellation mapper: BPSK modulate the coded bits as described in 1.5.5.4.2 to form the transmitted symbol blocks of QMG SC PHY SIG.

- Duplication and phase rotation: Apply the appropriate phase rotations for each 540 MHz subchannel as 1.5.5.4.3.

- Symbol blocking: Divied the encoded SIG field into symbol block of length-256.

- CSD: Apply CSD for each transmit chain as described in 1.5.5.4.4.

- GI insertion: Prepend a GI for each symbol block of length-256.

- Pulse filter, Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit.

Notice: Conversion from the sampled digital domain to the continuous time domain is beyond the scope of this document. Filtering for pulse shaping such as in GMSK is beyond the scope of this standard.

#### Construction of QMG SC PHY PPDU

The construction of the data field in a QMG SC PHY PPDU LDPC encoding proceeds as follows:

- Scrambler: Scramble the PHY padded data.

- LDPC encoder: The scrambled bits are encoded using the LDPC code as described in 1.3.12.

- Blocking Padding Zero: Append the PHY pad bits to the PSDU as described in 1.5.6.4.

- Stream parser: The output of the LDPC encoder is rearranged into blocks as described in 1.5.6.4.

- Constellation mapper: Map to BPSK, QPSK, 16-QAM, or 64-QAM constellation points as described in 1.5.6.6.

- Spatial Expansion: Apply the spatial expansion mechanism according to the combination of the number of the spatial streams and the number of the transmit antennas as described in 1.5.6.7.

- Pulse filter, Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit.

### QMG SC PHY PPDU Format

A QMG SC PHY PPDU frame is composed of the QMG SC PHY STF, the QMG SC PHY CEF, the QMG SC PHY SIG field, the optional QMG SC PHY SCTF, the SC data block and the optional training fields, which is illustrated in Figure 26- .

Figure 26-  Format of QMG SC PHY PPDU

Note: SCTF filed is only transmitted for 1080 MHz channel bandwidth and is composed of a ZCZ sequence of length 512. For MIMO transimission, different transmit antenna transmit a different preamble which is composed of one the four ZCZ sequences with the order of , such as for the case of one tramsmit antenna, the preamble sequence is composed of , for the case of two transmit antennas, the preamble sequences are composed of, respectively.

### QMG SC PHY SIG Transmission

#### SIG Fields

The QMG SC PHY SIG field carries information required to interpret PPDUs. The preamble and the QMG SC PHY SIG fields all are transmitted with SC modulation. Please see 1.3.9.

#### Generation of the CRC bits of SIG

The calculation of the CRC of the QMG SC PHY SIG is described in 1.3.6.

#### Scrambler

The operation of the scrambler is defined in 1.3.7. Bits x1, x2, x3, x4, x5, x6, x7 of the scrambler shift register are initialized using the bits in the scrambler initialization bits from the QMG SC PHY SIG. The QMG SC PHY SIG is scrambled starting from bit 7.

#### Encoding and modulations

The modulation and coding scheme defines the modulation and code rate that is used in the PPDU.

##### Encoding

Please see 1.3.9.2.

##### Modulation

Please see 1.4.4.5.

##### Duplication

If the channel bandwidth is 1080 MHz, the modulated SIG symbols are duplicated as described in 1.3.10.

##### Symbol blocking, CSD and GI insertion

The modulated QMG SC PHY SIG symbols are divived into four symbols blocks of length 256. For each block, the cyclic shift value for the QMG SC PHY SIG field of the PPDU for transmit chain  out of a total of  are defined inTable 26- .

Table 26-  Cyclic shift values for QMG SIG fields

| values for QMG-SIG-A field of PPDU |  |  |  |  |
| --- | --- | --- | --- | --- |
| Total number of transmit chains() | Cyclic shift for transmit chain (in units of ns) |  |  |  |
|  | 1 | 2 | 3 | 4 |
| 1 | 0 | — | — | — |
| 2 | 0 | -128 | — | — |
| 3 | 0 | -85 | -170 | — |
| 4 | 0 | -64 | -128 | -192 |

After the CSD operations, each group of 256 symbols is pre-pended by the last 64 symbols that forms the cyclic prefix.

### Data Field Transmission

#### General

The subclause describes the transmission of the SC data field.

#### Scrambler

The operation of the scrambler is defined in 1.3.7. The scrambling of the data field continues the scrambling of the QMG SC PHY SIG with no reset for SC transmission.

#### Encoding

Please see 1.3.12.

#### Symbol Block Padding Zeros

The number of symbol blocks,, is calculated as:

where is the number of LDPC parity bits,  values are listed in Table 26-15, and NCBPB values are shown in Table 21-16..

Table 26-  Values of mSE

| NSS | NTXT | mSE |
| --- | --- | --- |
| 1 | 1 | 1 |
|  | 2 | 2 |
|  | 3 | 2 |
|  | 4 | 4 |
| 2 | 2 | 2 |
|  | 3 | 4 |
|  | 4 | 4 |
| 3 | 3 | 3 |
|  | 4 | 6 |
| 4 | 4 | 4 |

Table 26-  Values of NCBPB

| Symbol mapping | NCBPB (540MHz) |  | NCBPB (1080MHz) |  |
| --- | --- | --- | --- | --- |
|  | Long GI | Short GI | Long GI | Short GI |
| π/2-BPSK | 192 | 224 | 384 | 448 |
| π/2-QPSK | 384 | 448 | 768 | 896 |
| π/2-16QAM | 768 | 896 | 1536 | 1792 |
| π/2-64QAM | 1152 | 1344 | 2304 | 2688 |

Then the number of padding zero for each spatial stream is given by

The PSDU is concatenated withzeros. They are scrambled using the continuation of the scrambler sequence that scrambled the PSDU.

#### Stream Parser

After coding and puncturing, the data bit streams at the output of the FEC encoder are processed in groups of. Each of these groups is re-arranged intoblocks ofbits.

The number of bits assigned to a single axis (real or imaginary) of a constellation point in a spatial stream is denoted by Equation .

The sum of these over all streams is

Consecutive blocks of s bits are assigned to different spatial streams in a round robin fashion.

The following equations are an equivalent description to the above procedure. Bit i at the output of encoder is assigned to input bit k of spatial stream where

where,,.

#### Constellation mapping

The modulation and coding scheme (MCS) field specifies the modulation and code rate used in the PPDU

A QMG STA shall support all equal modulation (EQM) rates for one spatial stream (MCSs 1 to 3). MCS 4-8 are optional.

The coded and padded bit stream is converted into a stream of complex constellation points according to the modulation specified in the MCS table.

In π/2-BPSK modulation, the input bit stream is mapped according to the following equation, whereis the kth input coded (or scrambled pad) bit. Each output symbol is then rotated according to the following equation:.

NOTE – With appropriate choice of transmit filtering, π/2-BPSK is equivalent to a precoded pulse-shaped MSK (e.g., GMSK). The precoder is simply , where bin,k is the scrambled input stream, bin,-1 is 0, and bout,k is the input to the (G)MSK modulator.

In π/2-QPSK modulation, the input bit stream is mapped according to the following equation, where k is the output symbol index, k=0,1,…... Each output symbol is then rotated according to the following equation:.

Figure 26- QPSK constellation bit encoding

In π/2-16QAM modulation, the input bit stream is mapped according to the following equation

where k is the output symbol index, k=0,1,…... Each output symbol is then rotated according to the following equation:.

Figure 26-  16QAM constellation bit encoding

In π/2-64QAM modulation, the input bit stream is mapped according to the following equation

where k is the output symbol index. Each output symbol is then rotated according to the following equation: .

Figure 26-  64QAM constellation bit encoding

#### Spatial Expansion

Each spatial stream is divided into many sub-blocks that each sub-block includes NDSPB constellation symbols. Values of NDSPB and NUSPB are shown inTable 26- .

Table 26-  Values of NDSPB and NUSPB

|  | NCBPB (540MHz) |  | NCBPB (1080MHz) |  |
| --- | --- | --- | --- | --- |
|  | Long GI | Short GI | Long GI | Short GI |
| NDSPB | 192 | 224 | 384 | 448 |
| NUWPB | 64 | 32 | 128 | 64 |

The spatial stream of complex numbers,,,, is the input to the spatial expansion modular, which produces as output the stream of complex numbers,,. The relation between the spatial streamsand the output streamsis given in Table 26-  where  ,andis defined as,

where,,and

Table 26-  Constellation mapper output to spatial mapper input

| Index |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |  |  | - | - |  |
| 2 |  | 2 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
| 3 |  | 3 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
| 4 |  | 4 | 1 |  |  |  |  |  |
|  |  |  | 2 |  |  |  |  |  |
|  |  |  | 3 |  |  |  |  |  |
|  |  |  | 4 |  |  |  |  |  |
| 5 | 2 | 2 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
| 6 |  | 3 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
| 7 |  | 4 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
|  |  |  | 4 |  |  | - | - |  |
| 8 | 3 | 3 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
| 9 |  | 4 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
|  |  |  | 4 |  |  | - | - |  |
| 10 | 4 | 4 | 1 |  |  | - | - |  |
|  |  |  | 2 |  |  | - | - |  |
|  |  |  | 3 |  |  | - | - |  |
|  |  |  | 4 |  |  | - | - |  |
|  | Note 1: a=2, for and, otherwise a=1. Note 2: indicates conjugate operation. |  |  |  |  |  |  |  |

The starting index for the first symbol for π/2 rotation is 0. If the Additional PPDU field within the PLCP header is equal to 0, the final block transmitted is followed by the same ZCZ sequence guard interval. If the Additional PPDU field within the PLCP header is equal to 1, the final block transmitted of the last PPDU in an A-PPDU is followed by the same ZCZ sequence guard interval.

Figure 26-  Block transmission

### Performance Requirements

#### Transmit Requirements

##### Introduction

Transmitter performance requirements of the SC PHY are defined in 1.5.7.1.2.

##### Transmit EVM

The transmit EVM accuracy test shall be performed by instrumentation capable of converting the transmitted signal into a stream of complex samples, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc.

The instrumentation shall perform carrier lock, symbol timing recovery and amplitude adjustment and equalization while making the measurements. The equalizer shall be trained using information in the QMG SC PHY preamble (STF/CEF). For the QMG SC PHY EVM, measuring samples at the sample rate, the measured symbols should not contain the first and the last hundred symbols of a given packet (ramp up/down). The EVM is calculated according to the formula below:

whereis the number of samples to be measured andshall be 1000, is the average power of the constellation,  is the complex coordinates of the measured symbol,  is the complex coordinates of the ideal constellation point for the measured symbol, and  is the complex DC term chosen to minimize EVM.

The test equipment should use a root-raised cosine filter with roll-off factor of 0.25 for the pulse shaping filter when conducting EVM measurement.

The transmit pulse shaping used is left to the implementer.

The relative constellation error (EVM) shall not exceed an MCS dependent value according to Table 26-.

Table 26- EVM requirements for SC PHY

| MCS Index | Modulation | R | EVM value[dB] |
| --- | --- | --- | --- |
| 1 | π/2-BPSK | 1/2 | -7 |
| 2 | π/2-QPSK | 1/2 | -11 |
| 3 | π/2-QPSK | 3/4 | -13 |
| 4 | π/2-16-QAM | 1/2 | -19 |
| 5 | π/2-16-QAM | 3/4 | -21 |
| 6 | π/2-64-QAM | 5/8 | -22 |
| 7 | π/2-64-QAM | 3/4 | -24 |
| 8 | π/2-64-QAM | 13/16 | -26 |

##### Time of Departure accuracy

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPmdTxStartRMS and aTxPmdTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters:

- MULTICHANNEL_SAMPLING_RATE is 440x106 sample/s for 540MHz channel and 880 x106 sample/s for 1080MHz channel, respectively

- FIRST_TRANSITION_FIELD is Short Training field

- SECOND_TRANSITION_FIELD is Channel Estimation field

- TRAINING_FIELD is Channel Estimation field

- TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH is 80 ns

NOTE—The indicated windowing applies to the time of departure accuracy test equipment, and not the transmitter or receiver.

#### Receive Requirements

##### Introduction

This subclause describes the receiver requirements of the QMG SC PHY

##### CCA

The start of a valid QMG SC PHY transmission at a receive level greater than the minimum sensitivity for MCS 0 (–74 dBm for 540MHz, -71dBm for 1080MHz) shall cause CCA to indicate busy with a probability > 90% within 1 μs. The receiver shall hold the carrier sense signal busy for any signal 20dB above the minimum sensitivity for MCS 0.

## QMG OFDM PHY

### Introduction

Transmission and reception of OFDM PHY PPDUs is optional.

### QMG OFDM PHY PPDU Format

A QMG OFDM PHY PPDU frame is composed of the QMG OFDM PHY STF fields, the QMG OFDM PHY CEF field, the QMG OFDM PHY SIG field, , the QMG OFDM PHY Short Training Field (OSTF), the QMG OFDM PHY Channel Estimation Field (OCEF),  QMG OFDM PHY symbols and the optional training fields, as shown in Figure 26- .

Figure 26-  Format of the QMG OFDM PHY PPDU

### Transmission Block Diagram

Figure 26-  shows the transmit process for the QMG OFDM PHY SIG transmission. The actual structure of the transmitter is implementation dependent.

Figure 26-  Transmitter block diagram for QMG PHY SIG field

Figure 26-  shows the transmitter blocks used to generate the Data field of OFDM PPDU with LDPC encoding.

Figure 26-  Transmitter block diagram for the data field of QMG OFDM PHY

Note: The operations denoted by the dots line arrow and dots line boxes in the above two figures are optional which are applied for OFDM MIMO transmission.

### Overview of QMG OFDM PHY PPDU encoding process

This subclause provides an overview of the QMG OFDM PPDU encoding process.

#### Construction of QMG OFDM PHY SIG

The construction of the QMG OFDM PHY SIG field proceeds as follows:

- Obtain the parameters from TXVECTOR. Add the reserved bits, append the calculated CRC. The calculation of the CRC of the QMG OFDM PHY SIG is described in 1.3.6.

- Scrambler: Scramble the QMG OFDM PHY SIG bits. The Scrambler is described in 1.3.7.

- LDPC encoder: The scrambled bits are encoded using the LDPC code as described in 1.5.5.4.1.

- Constellation mapper: Differential BPSK modulate the coded bits as described in 1.5.5.4.2 to form the transmitted symbol blocks of QMG OFDM PHY SIG.

- Duplication and phase rotation: Apply the appropriate phase rotations for each 540 MHz subchannel as 1.5.5.4.3.

- CSD: Apply CSD for each transmit chain as described in 1.5.5.4.4. This is need only for the MIMO mechanism.

- Symbol blocking and GI insertion: Prepend a GI and generate the transmission symbol block.

- Upsampling, Filter and downsampling: Sample rate change in OFDM transmission as described in 1.6.5.3. This is only needed for the OFDM mechanism.

- Pulse filter, Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit.

Notice: Conversion from the sampled digital domain to the continuous time domain is beyond the scope of this document. Filtering for pulse shaping such as in GMSK is beyond the scope of this standard.

#### Construction of QMG OFDM PHY PPDU

The construction of the Data field in an OFDM PHY PPDU with LDPC encoding proceeds as follows:

  - Scrambler: Scramble the PHY padded data. The Scrambler is described in 1.3.7.

  - LDPC encoder: The scrambled bits are encoded using the LDPC code with the APEP_LENGTH in the TXVECTOR as described in 1.6.8.3.

  - Stream parser: The output of the LDPC encoder is rearranged into blocks as described in 1.6.8.5.

  - Constellation mapper: Map to BPSK, QPSK, 16-QAM, 64-QAM or 256-QAM constellation points as described in 1.6.8.6.

  - LDPC tone mapper: The LDPC tone mapping shall be performed on all LDPC encoded streams as described in 1.6.8.7.

  - OFDM symbol padding zeros: Append the PHY pad bits to the PSDU as described in 1.6.8.4.

  - STBC: Apply STBC as described in 1.6.8.8.

  - Pilot insertion: Insert pilots following the steps described in 1.6.8.9.

  - CSD: Apply CSD for each space-time stream as described in 1.6.8.10.

  - Spatial mapping: Apply the Q matrix as described in 1.6.8.11.

  - Phase rotation: Apply the appropriate phase rotations for each 540 MHz subchannel. As described in 1.3.4.4.

  - IDFT: Compute the inverse discrete Fourier transform.

  - Insert GI and apply windowing: Prepend a GI and apply windowing.

  - Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit.

#### Construction of OSTF

The OSTF field is defined in 1.6.6 and is constructed as follows:

    - Sequence generation: Generate the OSTF in the frequency-domain over the bandwidth indicated by CH_BANDWIDTH as described in 1.6.6.

    - Phase rotation: Apply appropriate phase rotation for each 540 MHz subchannel as described in 1.3.4.3 and 1.3.4.4.

    - CSD: Apply CSD for each space-time stream and frequency segment as described in 1.6.8.10.

    - Spatial mapping: Apply the Q matrix as described in 1.6.8.11.

    - IDFT: Compute the inverse discrete Fourier transform.

    - Insert GI and apply windowing: Prepend a GI (LONG_GI) and apply windowing as described in 1.3.4.3.

    - Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit. Refer to 1.3.4.3 and 1.6.6 for details.

#### Construction of OCEF

The OCEF field is defined in 1.6.7 and constructed as follows:

  - Sequence generation: Generate the OCEF sequence in the frequency-domain over the bandwidth indicated by CH_BANDWIDTH as described in 1.6.7.

  - Phase rotation: Apply appropriate phase rotation for each 540 MHz subchannel as described in 1.3.4.3 and 1.3.4.4..

  - matrix mapping: Apply the  matrix to the OCEF sequence and apply the  matrix to the pilot tones as described in 1.6.7.

  - CSD: Apply CSD for each space-time stream and frequency segment as described in 1.6.8.10.

  - Spatial mapping: Apply the Q matrix as described in 1.6.8.11.

  - IDFT: Compute the inverse discrete Fourier transform.

  - Insert GI and apply windowing: Prepend a GI (LONG_GI) and apply windowing as described in 1.3.4.3.

  - Analog and RF: Up-convert the resulting complex baseband waveform associated with each transmit chain to an RF signal according to the center frequency of the desired channel and transmit. Refer to 1.3.4.3 and 1.6.7 for details.

### QMG OFDM PHY SIG Fields

#### SIG Fields

See 1.5.5.1-1.5.5.4.1.

#### Modulation

Please see 1.4.5.4.

#### Sample rate change

If the channel bandwidth is 540 MHz, the preamble sequence and the QMG OFDM PHY SIG fields defined in the above subclauses and are specified at the SC chip rate (). For transmission in the OFDM (nominal) sample rate, the signal is resampled with a 3/2 rate change. The resampling is done by upsampling by a factor of 3, filtering by the filterdefined in 1.3.11, and downsampling by a factor of 2 (see equation below). To define the transmission of the preamble when the packet is an OFDM packet, the preamble waveform is defined below.

Let, then

whereis the length of the filter defined in 1.3.11, for.

If the channel bandwidth is 1080 MHz, the modulated SIG symbols are transmitted using duplication style as described in 1.3.10.

### OSTF definition

The main purpose of the OFDM Short Training Field (OSTF) is to improve automatic gain control estimation in a MIMO transmission. The duration of the OSTF field is TOFDM-STF regardless of the Short GI field setting in QMG OFDM SIG.

For a 540 MHz transmission, the frequency domain sequence is given by Equation .

where  and  .

For a 1080 MHz transmission, the frequency domain sequence is given by Equation.

The time domain representation of the signal of transmit chain shall be as specified in Equation (22-33).

where

For 540 MHz transmission, . For 1080 MHz transmission,.

is given in Table 26- .

is defined in 1.6.8.12.

is defined in 26.3.4.4.

### OCEF Definition

The OFDM Channel Estimation Field (OCEF) provides a means for the receiver to estimate the MIMO channel between the set of constellation mapper outputs (or, if STBC is applied, the STBC encoder outputs) and the receive chains. The transmitter provides training for space-time streams (spatial mapper inputs) used for the transmission of the PSDU(s). For each tone the MIMO channel that can be estimated is anmatrix. A frame transmission has a preamble that contains OCEF symbols, where the data tones of each OCEF symbol are multiplied by entries belonging to a matrix, to enable channel estimation at the receiver. The pilot tones of each OCEF symbol are multiplied by the entries of a matrixdefined in the following text. The multiplication of the pilot tones in the OCEF symbol by thematrix instead of thematrix allows receivers to track phase and frequency offset during MIMO channel estimation using the OCEF. The number of OCEF symbols,, is a function of the number of space-time streamsas shown in Table 26- . As a result the OCEF field consists of one, two or four symbols.

Table 26-  Number of OCEFs required for different numbers of space-time streams

| NSTS | NOCEF |
| --- | --- |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |
| 4 | 4 |

In a 540 MHz transmission, letandbe the sequences defined in Equation and Equation, respectively.

In a 540 MHz transmission, the OCEF sequence transmitted is given by Equation.

In a 1080 transmission, letandbe the sequences defined in Equation and Equation, respectively.

In a 1080 MHz transmission, the OCEF sequence transmitted is given by Equation .

The generation of the time domain OCEF symbols per frequency segment is shown in Figure 26-  where is given in Equationis given in Equation.

Figure 26-  Generation of OCEF symbols per frequency segment

where  is the set of subcarrier indices for the pilot tones.

For a 540 MHz transmission,.

For a 1080 MHz transmission,.

is amatrix whose elements are defined in Equation.

The time domain representation of the waveform transmitted on the transmit chain  shall be as described by Equation.

where

For 540 MHz transmission,. For 1080 MHz transmission,.

is given in Table 26- .

is defined in 1.6.8.12.

is defined in 26.3.4.4.

is the duration of each OCEF symbol

is defined in Equation .

### Data Fields

#### General

The LDPC encoding process is composed of several steps including determining the number of padding bits, padding with zeros and the coding of every word.

#### Scrambler

The operation of the scrambler is defined in 1.3.7. The scrambling of the data field continues the scrambling of the QMG-OFDM PHY SIG with no reset for OFDM transmission.

#### Encoding

Please see 1.3.12.

#### OFDM symbol padding zeros

The number of symbol blocks,, is calculated as:

where is the number of LDPC parity bits,  denotes the least common multiple ofand, and  for STBC, otherwise . Then the number of padding zero for each spatial stream is given by

The PSDU is concatenated withzeros. They are scrambled using the continuation of the scrambler sequence that scrambled the PSDU.

#### Stream Parser

After coding and puncturing, the data bit streams at the output of the FEC encoder are processed in groups of. Each of these groups is re-arranged intoblocks ofbits.

The number of bits assigned to a single axis (real or imaginary) of a constellation point in a spatial stream is denoted by Equation.

The sum of these over all streams is

Consecutive blocks of s bits are assigned to different spatial streams in a round robin fashion.

The following equations are an equivalent description to the above procedure. Bit i at the output of encoder is assigned to input bit k of spatial stream where

where,,.

#### Subcarrier modulation mapping

The OFDM subcarriers shall be modulated by using BPSK, QPSK, 16-QAM, or 64-QAM, depending on the RATE requested. The encoded and interleaved binary serial input data shall be divided into groups of(1, 2, 4, or 6) bits and converted into complex numbers representing BPSK, QPSK, 16-QAM, or 64-QAM constellation points. The conversion shall be performed according to Gray-coded constellation mappings, illustrated inFigure 26- , with the input bit,, being the earliest in the stream. The output values, d, are formed by multiplying the resulting (I+jQ) value by a normalization factor, as described in Equation.

The normalization factor,, depends on the base modulation mode, as prescribed in Table 26-21. Note that the modulation type can be different from the start to the end of the transmission, as the signal changes from SIGNAL to DATA, as shown in Figure 26-27 to Figure 26-29. The purpose of the normalization factor is to achieve the same average power for all mappings. In practical implementations, an approximate value of the normalization factor may be used, as long as the device conforms with the modulation accuracy requirements described in 1.6.9.1.2.

Table 26-  Modulation-dependent normalization factor

| Modulation |  |
| --- | --- |
| BPSK |  |
| QPSK |  |
| 16-QAM |  |
| 64-QAM |  |

Figure 26-  BPSK, QPSK, 16-QAM, and 64-QAM constellation bit encoding

For BPSK, B0 determines the I value, as illustrated inTable 26- . For QPSK, B0 determines the I value and B1 determines the Q value, as illustrated in Table 26- . For 16-QAM, B0 and B1 determine the I value and B2 and B3 determines the Q value, as illustrated in Table 26- . For 64-QAM, B0, B1 and B2 determine the I value and B3, B4, and B5 determine the Q value, as illustrated in Table 26- .

Table 26-  BPSK encoding table

| Input bit (B0) | I-out | Q-out |
| --- | --- | --- |
| 0 | –1 | 0 |
| 1 | 1 | 0 |

Table 26-  QPSK encoding table

| Input bit (B0) | I-out | Input bit (B1) | Q-out |
| --- | --- | --- | --- |
| 0 | –1 | 0 | –1 |
| 1 | 1 | 1 | 1 |

Table 26-  16-QAM encoding table

| Input bits (B0B1) | I-out | Input bits (B2B3) | Q-out |
| --- | --- | --- | --- |
| 00 | –3 | 00 | –3 |
| 01 | –1 | 01 | –1 |
| 11 | 1 | 11 | 1 |
| 10 | 3 | 10 | 3 |

Table 26-  64-QAM encoding table

| Input bits (B0B1B2) | I-out | Input bits (B3B4B5) | Q-out |
| --- | --- | --- | --- |
| 000 | –7 | 000 | –7 |
| 001 | –5 | 001 | –5 |
| 011 | –3 | 011 | –3 |
| 010 | –1 | 010 | –1 |
| 110 | 1 | 110 | 1 |
| 111 | 3 | 111 | 3 |
| 101 | 5 | 101 | 5 |
| 100 | 7 | 100 | 7 |

#### Tone Mapping

The LDPC tone mapping shall be performed on all LDPC encoded streams and using an LDPC tone-mapping distance parameter.is constant for each bandwidth and its value for different bandwidths is given in Table 26- . LDPC tone mapping shall not be performed on streams that are encoded using BCC.

Table 26-  Value of Tone mapping parameter

| Parameter | 540MHz | 1080MHz |
| --- | --- | --- |
|  | 6 | 12 |

LDPC tone mapping for LDPC-coded streams is done by permuting the stream of complex numbers generated by the constellation mappers, to obtain

where , and

Each two consecutively generated complex constellation numbers will be transmitted on two data tones that are separated by at leastfrom other data tones.

#### Space-time block coding

This subclause defines a set of optional robust transmission techniques that are applicable only when using STBC coding for SU PPDUs. In this case,spatial streams are mapped tospace-time streams. These techniques are based on STBC. When the QMG-OFDM PHY SIG STBC field is 1, a symbol operation shall occur between the constellation mapper and the spatial mapper as defined in this subclause.

If STBC is applied, the stream of complex numbers,,, , is the input to the STBC encoder, which produces as output the stream of complex numbers, , ,.For given values of  and, STBC processing operates on the complex modulation symbols in sequential pairs of OFDM symbols so that the values of and depend on and. Also, and  depend on, and. This is defined in Table 26- .

Table 26-  Constellation mapper output to spatial mapper input for STBC

| 2 | 1 | 1 |  |  |
| --- | --- | --- | --- | --- |
|  |  | 2 |  |  |
| 4 | 2 | 1 |  |  |
|  |  | 2 |  |  |
|  |  | 3 |  |  |
|  |  | 4 |  |  |

If STBC is not applied, and.

NOTE—When STBC is applied, an odd number of space-time streams is not allowed, and.

#### Pilot Subcarriers

For a 540MHz transmission, eight pilot tones shall be inserted in subcarriers. The pilot mapping  for subcarrier k for symbol n shall be as specified in Equation.

whereis defined in Table 26- .

Table 26-  Pilot values for 540MHz transmission

| 1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

For a 1080MHz transmission,16 pilot tones shall be inserted in subcarriers -165, -143, -121, -99, -77, -55, -33, -11, 11, 33, 55, 77, 99, 121, 143, 165. The pilot mapping  for subcarrier k for symbol n shall be as specified in Equation.

where is defined in Table 26- .

Table 26-  Pilot values for 1080MHz transmission

| -1 | 1 | -1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Cyclic shift diversity (CSD) for OFDM PHY data fields Transmissioin

The cyclic shift values defined in this subclause apply to the data fields of the PPDU. Cyclic shifts are applied to prevent unintended beamforming when correlated signals are transmitted in multiple space-time streams. The cyclic shift valuefor the data fields for space-time stream n out oftotal space-time streams is shown in Table 26- .

Table 26-  Cyclic shift values for the data fields of an OFDM PPDU

| values for the data fields of a PPDU |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Cyclic shift for space-time stream n (ns) |  |  |  |
|  | 1 | 2 | 3 | 4 |
| 1 | 0 | — | — | — |
| 2 | 0 | -128 | — | — |
| 3 | 0 | -170 | -85 | — |
| 4 | 0 | -128 | -64 | -192 |

#### OFDM Modulation

The time domain waveform of the Data field of a QMG PPDU from transmit chain, shall be as defined in Equation(26-75).

where:

is defined in 18.3.5.10.

For 540 MHz transmission,. For 1080 MHz transmission,.

is defined in 26.5.3.9.8.

is the transmitted constellation on subcarrier , space-time streamand Data field OFDM symboland is defined in  through .

is the (short) guard interval duration and is given in Table 26-3.

is given in 1.6.8.10.

is defined in 1.6.8.9.

is defined in 1.3.4.4.

For a 540 MHz transmission,

andis defined as

For a 1080 MHZ transmission,

andis defined as

is a spatial mapping/steering matrix withrows and columnsfor subcarrier k. may be frequency dependent. Refer to the examples of listed in 20.3.11.11.2 for examples of that could be used for OFDM PPDU. Note that implementations are not restricted to the spatial mapping matrix examples listed in 20.3.11.11.2.

#### Beamforming

##### General

QMG OFDM MIMO beamforming is used by a STA with multiple antennas (the beamformer) to steer signals using knowledge of the channel to improve throughput. With QMG OFDM MIMO beamforming, all space-time streams in the transmitted signal are intended for reception at a single STA.

For QMG OFDM MIMO beamforming, the steering matrix  can be determined from the effective channel, , or the beamforming feedback matrix,, that is sent back to the beamformer by the beamformee using CSI feedback matrix format as defined in 1.6.8.12.2, or noncompressed beamforming feedback matrix format as defined in 1.6.8.12.3, or the compressed beamforming feedback matrix format as defined in 1.6.8.12.4. The feedback report format is described in 8.5.12.1, 8.5.12.2, and 8.5.12.3, respectively.

##### CSI matrices feedback

In CSI matrices feedback, the beamformee shall remove the space-time stream CSD in Table 26-31 from the measured channel before computing a set of matrices for feedback to the beamformer. The CSI matrix,found by the beamformee u for subcarrier k shall be encoded using method described in 8.4.1.2 so that applying the procedure in 20.3.12.3.3 (CSI matrices feedback decoding procedure) optimally reconstructs the matrix.

##### Noncompressed beamforming feedback matrix

In noncompressed beamforming feedback matrix, the beamformee shall remove the space-time stream CSD in Table 26-31 from the measured channel before computing a set of matrices for feedback to the beamformer. The beamforming feedback matrices,, found by the beamformee are sent to the beamformer in the order of real and imaginary components per tone as specified in 8.5.12.2. The beamformer might use these matrices to determine the steering matrices.

The beamformee shall encode the matrices using the method described in 20.3.12.3.4 so a beamformer applying the procedure in 20.3.12.3.5 optimally reconstructs the matrix.

##### Compressed Beamforming feedback matrix

Upon receipt of a NDP sounding PPDU, the beamformee shall remove the space-time stream CSD in Table 26-31 from the measured channel before computing a set of matrices for feedback to the beamformer. The beamforming feedback matrix, found by the beamformee for subcarrier shall be compressed in the form of angles using the method described in 20.3.12.3.6. The angles, and, are quantized according to Table 8-49a.

The beamformee shall generate the beamforming feedback matrices with the number of rows () equal to theof the NDP.

After receiving the angle information, and, the beamformer reconstructs using Equation (20-79). For SU-MIMO beamforming, the beamformer can use this matrix to determine the steering matrix. The method used by the beamformer to calculate the steering matrixis implementation specific.

The beamformee decides the tone grouping value to be used in the beamforming feedback matrix . A STA with dot11QMGBeamformerOptionImplemnted equal to true shall support all tone grouping values and Codebook Information values.

#### QMG Preamble format for sounding PPDUs

##### General

The MIMO channel measurement takes place in every PPDU as a result of transmitting the OCEFs as part of the PLCP preamble. The number of OCEFs transmitted shall be determined by the number of space-time streams transmitted. The use of the same spatial transformation enables the computation of the spatial equalization at the receiver.

When the number of space-time streams, , is less than the number of transmit antennas, or less than , sending only  OCEFs does not allow the receiver to recover a full characterization of the MIMO channel, even though the resulting MIMO channel measurement is sufficient for receiving the Data field of the QMG PPDU.

However, there are several cases where it is desirable to obtain as full a characterization of the channel as possible, thus requiring the transmission of a sufficient number of OCEFs to sound the full dimensionality of the channel, which is in some cases  and in other cases . These cases of MIMO channel measurement are referred to as MIMO channel sounding. A sounding packet may be used to sound available channel dimensions. A sounding PPDU is identified by setting the Not Sounding field in the QMG SIG to 0. A sounding PPDU may have any allowed number of OCEFs satisfying as shown in Table 26- .

##### Sounding with an NDP

A STA may sound the channel using a NDP (indicated by zero in the Length field in the QMG SIG) with the Not Sounding field set to 0. The number of OCEFs is the number implied by the 2STBCNSS, which shall indicate spatial time streams. The last OCEF of an NDP shall not be followed by a Data field as shown in Figure 26- .

Figure 26-  NDP format

It is optional for a STA to process an NDP.

##### Sounding PPDU for channel quality assessment

In response to the reception of an MRQ, sent by STA A to STA B, the responding STA B returns to the requesting STA A an MCS selection that STA B determines to be a suitable MCS for STA A to use in subsequent transmissions to STA B. In determining the MCS, STA B performs a channel quality assessment, which entails using whatever information STA B has about the channel, such as an estimate of the MIMO channel derived from the sounding PPDU that carries the MRQ. To enable this calculation, the MRQ is sent in conjunction with a sounding PPDU.

The STA sending the MRQ (STA A) determines how many OCEFs to send, and whether to use an NDP or not, based on the Transmit Beamforming Capabilities field, number of space-time streams used in the PPDU carrying the MRQ, the number of transmit chains it is using (), whether the transmit and receive STAs support STBC, and in some cases, the number of receive chains at the responding STA ( ).

The maximum number of available space-time streams is set by the number of transmit and receive chains and the STBC capabilities of the transmitter and receiver, as is shown in Table 26- . While the number of receive chains at a STA is not communicated in a capabilities indicator, the maximum number of space-time streams supported may be inferred from the STBC capabilities of the receiving STA. When the number of receive chains is known at the transmitter, the number of OCEFs sent to obtain a full channel quality assessment is determined according to the maximum number of space-time streams indicated in Table 26- . The number of OCEFs to use in conjunction with the indicated number of space-time streams is determined according toTable 26- .

Table 26-  Maximum available space-time streams

|  |  | without STBC | with STBC |
| --- | --- | --- | --- |
| 1 | 1 | 1 | N/A |
| 2 | 1 | 1 | 2 |
| 4 | 1 | 1 | 2 |
| 4 | 2 | 2 | 4 |

The sounding PPDU may have non-identity spatial mapping matrix. For different receiving STAs,  may vary.

### Performance requirements

#### Transmit requirements

##### Introduction

This subclause describes the performance requirement from the OFDM PHY transmitter.

##### Transmit EVM

The transmit EVM accuracy test shall be performed by instrumentation capable of converting the transmitted signal into a stream of complex samples, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, DC offsets, phase noise, etc.

The sampled signal shall be processed in a manner similar to an actual receiver, according to the following steps, or an equivalent procedure:

- Start of frame shall be detected.

- Transition from short sequences to channel estimation sequences shall be detected, and fine timing (with one sample resolution) shall be established.

- Frequency offsets shall be estimated and corrected.

- The frame shall be de-rotated according to estimated frequency offset.

- The complex channel response coefficients shall be estimated for each of the subcarriers using information contained in the preamble (STF/CEF and/or OCEF).

- For each of the OFDM symbols: transform the symbol into subcarrier received values, estimate the phase from the pilot subcarriers, derotate the subcarrier values according to estimated phase, and divide each subcarrier value with a complex estimated channel response coefficient.

- For subcarrier, compute the Euclidean distance to the ideal location for the symbol, or pilot.

- Compute the RMS average of all errors in a packet. It is given by

where

is the number of frames

is the frame index

is the carrier index

is the set of pilot and data subcarriers {1,2, …, (NSR – 1), (NSR + NDC),(NSR+NDC+1),…, NST}

is the symbol index

is the number of symbols

is the total number of subcarriers

and  are the ideal constellation point for I and Q, respectively

is the average power of the constellationcomputed over theframe.

The measurements shall occur only on the OFDM symbols, the measurement shall be performed on at least 10 frames with 16 symbols at least in each of them. Random data shall be used.

The EVM RMS error shall not exceed an MCS dependent value as found in Table 26-  .

Table 26-  EVM requirements for OFDM

| MCS Index | Modulation | R | EVM value[dB] |
| --- | --- | --- | --- |
| 9 | BPSK | 1/2 | -7 |
| 10 | QPSK | 1/2 | -11 |
| 11 | QPSK | 3/4 | -13 |
| 12 | 16-QAM | 1/2 | -15 |
| 13 | 16-QAM | 3/4 | -19 |
| 14 | 64-QAM | 5/8 | -22 |
| 15 | 64-QAM | 3/4 | -24 |
| 16 | 64-QAM | 13/16 | -26 |

#### TX flatness

##### TX flatness for 540MHz channel

When using the OFDM PHY and only while transmitting OFDM symbols, the average energy of the OFDM symbols constellations in each of the subcarriers with indices –70 to –2 and +2 to +70 shall deviate no more than ± 2 dB from their average energy. The average energy of the constellations in each of the subcarriers with indices –71 to –89 and +70 to +89 shall deviate no more than +2/–4 dB from the average energy of subcarriers with indices –71 to –89 and +70 to +89.

##### TX flatness for 1080MHz channel

When using the OFDM PHY and only while transmitting OFDM symbols, the average energy of the OFDM symbols constellations in each of the subcarriers with indices –146 to –2 and +2 to +145 shall deviate no more than ± 2 dB from their average energy. The average energy of the constellations in each of the subcarriers with indices –147 to –177 and +147 to +177 shall deviate no more than +2/–4 dB from the average energy of subcarriers with indices –177 to –2 and +2 to +177.

##### Time of Departure accuracy

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPmdTxStartRMS andaTxPmdTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters:

- MULTICHANNEL_SAMPLING_RATE is 660x106 sample/s for 540MHz and 1320 for 1080MHz

- FIRST_TRANSITION_FIELD is Short Training field

- SECOND_TRANSITION_FIELD is Channel Estimation field

- TRAINING_FIELD is Channel Estimation field

- TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH is 80 ns

NOTE— The indicated windowing applies to the time of departure accuracy test equipment, and not the transmitter or receiver.

#### Receive requirements

##### Introduction

Subclause 1.6.9.3.2 describes the performance requirement for an OFDM receiver.

##### CCA

The start of a valid QMG OFDM PHY or QMG SC PHY transmission at a receive level greater than the minimum sensitivity for MCS 0 (–74 dBm for 540MHz, -71dBm for 1080MHz) shall cause CCA to indicate busy with a probability > 90% within 1 μs.

## Analog Beamforming PHY frame format

### TX sector sweep

The packets sent during TX sector sweep are Control PHY packets as defined in 26.4.

### Beam refinement

#### General

Beam refinement is a process where a STA can improve its antenna configuration (or antenna weight  vectors) both for transmission and reception. If the SLS beamforming training did not include an RSS, as in the case where both devices have more than one transmit sector per antenna, beam refinement can serve as the first receive antenna configuration training. The procedure of beam refinement is described in 9.38.64. In the beam refinement procedure, BRP packets are used to train the receiver and transmitter antenna. There are two types of BRP packets: BRP-RX packets and BRP-TX packets:

- BRP-RX packets are packets that have TRN-R training sequences appended to them. These packets enable receiver antenna weight vector training.

- BRP-TX packets are packets that have TRN-T training sequences appended to them. The transmitting STA may change antenna configuration at the beginning of each sequence. The receiving STA performs measurements on these sequences and sends feedback to the STA that transmits the BRP-TX packet.

#### BRP packet structure

Each BRP packet is composed of an STF, a CEF field and a data field followed by a training field containing an AGC training field and a receiver training field.

Figure 26-  BRP packet structure

#### BRP packet SIG fields

A value of 0x0 in the Packet Type field indicates a BRP-RX packet (TRN-R field is present).

A value of 0x1 in the Packet Type field indicates a BRP-TX packet (TRN-T field is present).

A value of N in the Training Length field indicates that the AGC has 4N subfields and that the TRN- R/T field has 5N subfields.

The value N in the Training Length field of a BRP-RX packet is equal to the value of the L-RX field requested by the intended receiver of the BRP-RX packet at a previously received BRP Request field (see 8.5.4).

#### Beam refinement AGC field

The beam refinement AGC fields are composed of  repetitions of the sequence  when the packet is transmitted using the Control PHY in bandwidth 540 MHz,  when the packet is transmitted using the SC PHY in bandwidth 540 MHz, and  when the packet is transmitted using the OFDM PHY in bandwidth 540 MHz.  when the packet is transmitted using the Control PHY in bandwidth 1080 MHz,  when the packet is transmitted using the SC PHY in bandwidth 1080 MHz, and  when the packet is transmitted using the OFDM PHY in bandwidth 1080 MHz. The sequences, , , ,  and are defined in 26.8. The sequences are transmitted using rotated  modulation. Any transmit signal transients that occur due to this TX AWV configuration change shall completely settle by the end of the first ,  or  subsequence in bandwidth 540 MHz, and ,  or  subsequence in bandwidth 1080 MHz.

In a BRP-TX packet, the transmitter may change the TX AWV configuration at the beginning of each AGC subfield. The set of AWVs used for the AGC subfields should be the same as that used for the TRN-T subfields. In a BRP-RX packet, the transmitter shall use the same TX AWV as in the preamble and data fields of the packet.

#### Beam refinement TRN-R field

The TRN-R fields enable receiver AWV training. The TRN-R fields have the form shown in Figure 26- .

Figure 26-  TRN-R field definition

Each subfield CEF matches the Channel Estimation field defined in1.3.5. The  subfields  through  each consist of the sequence  in bandwidth 540 MHz, and  in bandwidth 1080 MHz. The sequences , ， and  are defined in Table 26-34, and , ， and  are defined in Table 26-35, in 26.8.

#### Beam refinement TRN-T field

The TRN-T field enables transmitter AWV training. The TRN-T fields have the form shown in Figure 26-40.

Figure 26-  TRN-T field definition

Each subfield CEF matches the Channel Estimation field defined in 1.3.5. The  subfields  through  each consist of the sequence  in bandwidth 540 MHz, and  in bandwidth 1080 MHz. The sequences , ， and  are defined in Table 26-34, and , ， and  are defined in Table 26-35, in 26.8. The sequences are transmitted using rotated  modulation. When transmitting the CEF subfield, the transmitter shall use the same AWV as in the preamble and data fields of the packet. Any transmit signal transients that occur due to TX AWV configuration changes between subfields shall settle by the end of the first 64 samples of the subfield.

#### Channel measurement

The good autocorrelation properties of the ZCZ sequences enable reconstructing part of the impulse response of the channel between the transmitter and the receiver. The receiver should find the tap with largest amplitude in the channel during the CEF field of the BRP-RX. It selects thereafter the set of taps that is  measured  around  the  tap  with  the  largest  amplitude,  according  to  the  value  of dot11ChanMeasFBCKNtaps. It can select a contiguous set of taps or select a noncontiguous set of taps, and include the tap delays subfield as part of the subfield measurement. It then measures the phase and amplitude of the corresponding channel taps in each of the TRN-T field repetition (except for those using the CEF AWV configuration). The beam refinement feedback subfield  is the relative amplitude and phase of this tap in the  repetition compared to this tap in the first TRN-T subfield.

#### BRP resampling in an OFDM packet

The BRP AGC, CEF, and Tn/Rn fields are specified at the SC chip rate (). When appended to an OFDM packet, the signal is resampled as defined in 1.3.10.

## ZCZ sequence

The following ZCZ sequences set is used in the preamble and the single carrier channel estimation field. These are two ZCZ sequences sets, namely, and, with  where the subscript denotes the length of the sequences and the superscript denotes the index of the ZCZ sequence. The sets are generated using the following recursive procedure, illustrated as in Figure 26- .

Figure 26-  Generation of ZCZ sequences set

,, and used in Figure 26-  are defined as follows, respectively.

where

denotes the length of the column vector;

denotes the initial sequence;

denotes the identity matrix of the size;

denotes all zero matrix of the size;

are elements of the coefficient matrix of size ;

with denoting the diagonal matrix with diagonal element ;

are elements of the Discrete Fourier Transform (DFT) matrix of size ;

denotes all 1 column vector;

denotes Kronecker product.

Note that, the elements ofbelongs to, and.

When , , and are given by  , the generated ZCZ sequence set is listed in Table 26-  where 0, 1, 2, and 3 denote , repectively.

When , , and are given by ,  the generated ZCZ sequence set  is listed in Table 26-  where 0, 1, 2, and 3 denote , repectively.

When , , and are given by ,  the generated ZCZ sequence set  is listed in Table 26-  where 0, 1, 2, and 3 denote , repectively.

When , , and are given by ,  the generated ZCZ sequence set  is listed in Table 26-  where 0, 1, 2, 3 denote , repectively.

When ,, and are given by ,  the generated ZCZ sequence set  is listed in Table 26-  where 0, 1, 2, 3 denote , respectively.

Note that the sequence sets in the tables are normative, the description above is informative.

Table 26-  The sequence set

|  | The sequence set to be transmitted from left to right, up to down |
| --- | --- |
|  | 22110220222220132233020222002031 |
|  | 20130022202022112031000020022233 |
|  | 22330202220020312211022022222013 |
|  | 20310000200222332013002220202211 |

Table 26-  The sequence set

|  | The sequence set to be transmitted from left to right, up to down |
| --- | --- |
|  | 21010002230313112101111301210200 21012220230331332101333101212022 |
|  | 22200121202210302220123202000323 22202303202232122220301002002101 |
|  | 23030200210111132303131103230002 23032022210133312303313303232220 |
|  | 20220323222012322022103000020121 20222101222030102022321200022303 |

Table 26-  The sequence set

|  | The sequence set to be transmitted from left to right, up to down |
| --- | --- |
|  | 0011202022222013221102200022021300112020111113020033200211331320 0011202000000231221102202200203100112020333331200033200233113102 |
|  | 0213222220202211201300220220001102132222131311000231220013311122 0213222202020033201300222002223302132222313133220231220031133300 |
|  | 0033200222002031223302020000023100332002113313200011202011111302 0033200200220213223302022222201300332002331131020011202033333120 |
|  | 0231220020022233203100000202003302312200133111220213222213131100 0231220002200011203100002020221102312200311333000213222231313322 |

Table 26-  The sequence set

|  | The sequence set to be transmitted from left to right, up to down |
| --- | --- |
|  | 1302031011002330130210213322122313022132110001120231213222112330 1302031011002330201321320033233031200310332223303120102111001223 1302031011002330312032031100300113022132110001122013031000330112 1302031011002330023103102211011231200310332223301302320333223001 |
|  | 1021003312232013102111003001130210212211122302310310221123302013 1021003312232013213222110112201332030033300120133203110012231302 1021003312232013320333221223312010212211122302312132003301120231 1021003312232013031000332330023132030033300120131021332230013120 |
|  | 1100011213022132110012233120102111002330130203100033233020132132 1100011213022132221123300231213233220112312021323322122313021021 1100011213022132332230011302320311002330130203102211011202310310 1100011213022132003301122013031033220112312021321100300131203203 |
|  | 1223023110212211122313023203110012232013102100330112201321322211 1223023110212211233020130310221130010231320322113001130210211100 1223023110212211300131201021332212232013102100332330023103100033 1223023110212211011202312132003330010231320322111223312032033322 |

Table 26-  The sequence set

|  | The sequence set to be transmitted from left to right, up to down |
| --- | --- |
|  | 0011133100223120221131310000310222333113331120132211313111110213 0011133122001302221131312222132011222002002231201100202022221320 0011133100223120221131310000310233000220002231203322020222221320 2233311300223120003313130000310200111331331120130033131311110213 0011133100223120221131310000310200111331113302310033131333332031 0011133122001302221131312222132033000220220013023322020200003102 0011133100223120221131310000310211222002220013021100202000003102 2233311300223120003313130000310222333113113302312211313133332031 |
|  | 0213113302203322201333330202330020313311311322112013333313130011 0213113320021100201333332020112213202200022033221302222220201122 0213113302203322201333330202330031020022022033223120000020201122 2031331102203322023111110202330002131133311322110231111113130011 0213113302203322201333330202330002131133133100330231111131312233 0213113320021100201333332020112231020022200211003120000002023300 0213113302203322201333330202330013202200200211001302222202023300 2031331102203322023111110202330020313311133100332013333331312233 |
|  | 0033131300003102223331130022312022113131333320312233311311330231 0033131322221320223331132200130211002020000031021122200222001302 0033131300003102223331130022312033220202000031023300022022001302 2211313100003102001113310022312000331313333320310011133111330231 0033131300003102223331130022312000331313111102130011133133112013 0033131322221320223331132200130233220202222213203300022000223120 0033131300003102223331130022312011002020222213201122200200223120 2211313100003102001113310022312022113131111102132233311333112013 |
|  | 0231111102023300203133110220332220133333313122332031331113310033 0231111120201122203133112002110013022222020233001320220020021100 0231111102023300203133110220332231200000020233003102002220021100 2013333302023300021311330220332202311111313122330213113313310033 0231111102023300203133110220332202311111131300110213113331132211 0231111120201122203133112002110031200000202011223102002202203322 0231111102023300203133110220332213022222202011221320220002203322 2013333302023300021311330220332220133333131300112031331131132211 |

## Regulatory requirements

Wireless LANs (WLANs) implemented in accordance with this standard are subject to equipment certification and operating requirements established by regional and national regulatory administrations. The PHY specification establishes minimum technical requirements for interoperability, based upon established regulations at the time this standard was issued. These regulations are subject to revision or may be superseded.

## Channelization

A QMG channel is specified by the two PLME MIB fields specified in Table 26- .

Table 26-  Fields to specify QMG channels

| Field | Meaning |
| --- | --- |
| dot11CurrentChannelWidth | Channel width. Possible values represent 540 MHz and 1800 MHz channels. |
| dot11CurrentChannelCenterFrequencyIndex | For a 540 MHz, or 1080 MHz channel, denotes the channel center frequency. |

Given dot11CurrentChannelCenterFrequencyIndex, the center frequency is given by Equation for 540MHz channel.

wheredenotes the channel index decided by dot11CurrentChannelCenterFrequencyIndex.

Figure 26-  Channelization for 540 MHz

Given dot11CurrentChannelCenterFrequencyIndex, the center frequency is given by Equation for 1080MHz channel.

wheredenotes the channel index decided by dot11CurrentChannelCenterFrequencyIndex.

Figure 26-  Channelization for 1080 MHz

## Transmit spectrum mask

For a 540 or 1080 MHz mask PPDU, the interim transmit spectral mask shall have a 0 dBr (dB relative to the maximum spectral density of the signal) bandwidth of 470 or 940 MHz, –17 dBr at 330 or 660 MHz frequency offset, –22 dBr at 1350 or 2670 MHz frequency offset, and –30 dBr at 1530 or 3060 MHz frequency offset and above. The interim transmit spectral mask for frequency offsets in between 235 or 470 and 330 or 660 MHz, 330 or 660 and 1350 or 2700 MHz, and 1350 or 2700 and 1530 or 3060 MHz shall be linearly interpolated in dB domain from the requirements for 235 MHz, 330 MHz, 1350 MHz, and 1530 MHz frequency offsets. The transmit spectrum shall not exceed the maximum of the interim transmit spectral mask.

Figure 26-  Example transmit spectral mask for a PPDU

## PHY Transmit procedure

### PHY transmit procedure for an SC transmission

The PLCP transmit procedure is shown in Figure 26- . In order to transmit data, a PHY-TXSTART.request primitive shall be enabled so that the PHY entity shall be in the transmit state. Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME, as specified in 1.14. Other transmit parameters, such as MCS and transmit power, are set via the PHY-SAP with the PHYTXSTART.request(TXVECTOR), as described in Table 26- .

The PLCP shall then issue a PHY_TXSTART.request primitive, and transmission of the PLCP preamble shall start, based on the parameters passed in the PHY-TXSTART.request primitive. The PLCP shall calculate the length of the packet according the MCS and the length specified in the PHY_TXSTART.request primitive, adding padding bits if necessary.

The PHY continues with the scrambling, encoding and transmission of the QMG-SIG according to the PHY_TXSTART.req(TXVECTOR) parameters. The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The data are encoded as described in 1.5.6.3. The encoded data are then modulated as described in 1.5.6.6, depending on the MCS requested in the PHY_TXSTART.req. Transmission can be prematurely terminated by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled by receiving a PHYTXEND.request primitive.

Transmission of the PSDU is completed with the transmission of the last bits of the (encoded) PSDU. The packet transmission shall be completed, and the PHY entity shall enter the receive state (i.e., PHYTXSTART shall be disabled). Each PHY-TXEND.request primitive is acknowledged with a PHY-TXEND.confirm primitive from the PHY. In an SU transmission, normal termination occurs after the transmission of the final bit of the last PSDU octet, according to the number of symbol blocks indicated by.

Figure 26-  PHY transmit procedure for aQMG SC PHY transmission

### PHY transmit procedure for an OFDM transmission

The PLCP transmit procedures are shown in Figure 26-  for OFDM SU transmission. In order to transmit data, a PHY-TXSTART.request primitive shall be enabled so that the PHY entity shall be in the transmit state. Further, the PHY shall be set to operate at the appropriate frequency through station management via the PLME, as specified in~. Other transmit parameters, such as MCS and transmit power, are set via the PHY-SAP with the PHYTXSTART.request(TXVECTOR), as described in Table 26- .

The PLCP shall then issue a PHY_TXSTART.request primitive, and transmission of the PLCP preamble shall start, based on the parameters passed in the PHY-TXSTART.request primitive. The PLCP shall calculate the length of the packet according the MCS and the length specified in the PHY_TXSTART.request primitive, adding padding bits if necessary.

The PHY continues with the scrambling, encoding and transmission of the QMG SIG according to the PHY_TXSTART.req(TXVECTOR) parameters. The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The data are scrambled and encoded s described in 1.6.8.2 and 1.6.8.3, respectively. The encoded data are then modulated as described in1.6.8.6, depending on theMCS requested in the PHY_TXSTART.req. Transmission can be prematurely terminated by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled by receiving a PHYTXEND.request primitive.

Transmission of the PSDU is completed with the transmission of the last bits of the (encoded) PSDU. The packet transmission shall be completed, and the PHY entity shall enter the receive state (i.e., PHYTXSTART shall be disabled). Each PHY-TXEND.request primitive is acknowledged with a PHY-TXEND.confirm primitivefrom the PHY. In an SU transmission, normal termination occurs after the transmission of the final bit of the last PSDU octet, according to the number of OFDM symbols indicated by (see 1.6.8.1).

Figure 26-  PHY transmit procedure for a QMG OFDM PHY transmission

A typical state machine implementation of the transmit PHY for an SU transmission is provided in Figure 26- . This state machine does not describe the operation of optional features, such as STBC.

Figure 26-  Typical Tx state machine

## Receive procedure

### SC Receive procedure

A typical PLCP receive procedure is shown in Figure 26-  for an SC transmission. A typical state machine implementation of the receive PHY is given in Figure 26- . This receive procedure and state machine do not describe the operation of optional features, such as STBC.

Figure 26-  PHY receive procedure for a QMG SC PHY transmission

Upon receiving the transmitted PHY preamble overlapping the primary 540 MHz channel, the PHY measures a receive signal strength. This activity is indicated by the PHY to the MAC via a PHY-CCA.indication primitive. A PHY-CCA.indication(BUSY, channel-list) primitive is also issued as an initial indication of reception of a signal as specified in 1.6.9.3.2. The channel-list parameter of the PHY-CCA.indication primitive is absent when the operating channel width is 540 MHz. The channel-list parameter is present and includes the element primary when the operating channel width is 1080 MHz.

After the PHY-CCA.indication(BUSY) is issued, the PHY entity shall use the following ZCZ sequences of length 256 to synchronization, and channel estimation. The PHY demodulates the QMG SIG field. After the reception of the QMG SIG field, the PMD indicates to the PLCP the type of PHY of the packet according the SC/OFDM subfield in the QMG SIG field. If the SC/OFDM subfield in the QMG SIG field indicated a SC PHY, the receiver is capable of receiving SC PHY, and dot11SCPHYActivated is true. And the PLCP shall indicate to the PMD the MCS, length and other parameters needed for the demodulation of the packet. At the end of the data portion of the packet, the PHY shall indicate PHY_RXEND.ind(No_Error) to the MAC.

### OFDM receiver procedure

A typical PLCP receive procedure is shown in Figure 26-  for a QMG OFDM PHY transmission. A typical state machine implementation of the receive PHY is given in Figure 26- . This receive procedure and state machine do not describe the operation of optional features, such as STBC. Further, through station management (via the PLME) the PHY is set to the appropriate frequency, as specified in TBD. The PHY has also been configured with group information (i.e., group membership and position in group) so that it can receive data intended for the STA. Other receive parameters, such as RSSI and indicated DATARATE, may be accessed via the PHY-SAP.

Upon receiving the transmitted PHY preamble overlapping the primary 540 MHz channel, the PHY measures a receive signal strength. This activity is indicated by the PHY to the MAC via a PHY-CCA.indication primitive. A PHY-CCA.indication(BUSY, channel-list) primitive is also issued as an initial indication of reception of a signal as specified in 1.6.9.3.2. The channel-list parameter of the PHY-CCA.indication primitive is absent when the operating channel width is 540 MHz. The channel-list parameter is present and includes the element primary when the operating channel width is 1080 MHz.

The PHY includes the most recently measured RSSI value in the PHY-RXSTART.indication(RXVECTOR) primitive issued to the MAC.cc.

After the PHY-CCA.indication(BUSY, channel-list) is issued, the PHY entity shall begin receiving the training symbols and searching for QMG-SIG in order to set the maximum duration of the data stream. If the check of the QMG-SIG parity bit is not valid, a PHY-RXSTART.indication primitive is not issued, and instead the PHY shall issue the error condition PHY-RXEND.indication(FormatViolation) primitive. If a valid QMG-SIG parity bit is indicated, the PHY shall maintain PHY-CCA.indication(BUSY, channel-list) for the predicted duration of the transmitted PPDU. If the QMG-SIG indicates an invalid CRC or Reserved QMG-SIG Indication, the PHY shall issue the error condition PHY-RXEND.indication(FormatViolation).

After receiving a valid QMG-SIG indicating a supported mode, the PMD indicates to the PLCP the type of PHY of the packet according the SC/OFDM subfield in the QMG-SIG field. If the SC/OFDM subfield in the QMG-SIG field indicated an OFDM PHY, the receiver is capable of receiving SC PHY, and dot11OFDMPHYActivated is true. The PHY entity shall begin receiving the QMG training symbols. If an indication of a valid QMG-SIG CRC, a PHYRXSTART.indication(RXVECTOR) primitive shall be issued. The RXVECTOR associated with this primitive includes the parameters specified in Table 26- .

If signal loss occurs during reception prior to completion of the PSDU reception, the error condition PHYRXEND.indication(CarrierLost) shall be reported to the MAC. If signal loss occurs during reception prior to completion of the PSDU reception, the PHY shall set the PHY-CCA.indication(IDLE) primitive and return to the RX IDLE state.

The received PSDU bits are assembled into octets, decoded, and presented to the MAC using a series of PHY-DATA.indication(DATA) primitive exchanges. Any final bits that cannot be assembled into a complete octet are considered pad bits and discarded. After the reception of the final bit of the last PSDU octet, and possible padding bits, the receiver shall be returned to the RX IDLE state. A PHY-RXEND.indication(NoError) primitive shall be issued on entry to the RX IDLE state.

Figure 26-  PHY receive procedure for a QMG OFDM PHY transmission

Figure 26-  PHY receive state machine

## QMG PLME

### PLME SAP sublayer management primitive

Table 26-  lists the MIB attributes that may be accessed by the PHY entities and the intralayer of higher level LMEs. These attributes are accessed via the PLME-GET, PLME-SET, PLME-RESET, and PLME-CHARACTERISTICS primitives defined in 6.5.

### PHY MIB

QMG PHY management information base (MIB) attributes are defined in Annex C with specific values defined in Table 26- . The“Operational semantics” column in Table 26-  contains two types: static and dynamic.

- Static MIB attributes are fixed and cannot be modified for a given PHY implementation.

- Dynamic MIB attributes are interpreted according to the MAX-ACCESS field of the MIB attribute.

When MAX-ACCESS is read-only, the MIB attribute value may be updated by the PLME and read from the MIB attribute by management entities. When MAX-ACCESS is read-write, the MIB attribute may be read and written by management entities but shall not be updated by the PLME.

Table 26-  QMG PHY MIB attributes

| Managed Object | Default value/ range | Operational Semantics |
| --- | --- | --- |
| dot11PHYOperationTable |  |  |
| dot11PHYType | QMG | Static |
| dot11PHYQMGTable |  |  |
| dot11CurrentChannelWidth | Implementation dependent | Dynamic |
| dot11CurrentChannelCenterFrequencyIndex | Implementation dependent | Dynamic |
| dot11TxSTBCOptionImplemented | False/Boolean | Static |
| dot11TxSTBCOptionActivated | False/Boolean | Dynamic |
| dot11RxSTBCOptionImplemented | False/Boolean | Static |
| dot11RxSTBCOptionActivated | False/Boolean | Dynamic |
| dot11BeamFormingOptionImplemented | False/Boolean | Static |
| dot11BeamFormingOptionActivated | False/Boolean | Dynamic |
| dot11MaxNTxChainsImplemented | Implementation dependent | Static |
| dot11MaxNTxChainsActivated | Implementation dependent | Dynamic |
| dot11TransmitBeamformingConfigTable |  |  |
| dot11ReceiveNDPOptionImplemented | False/Boolean | Static |
| dot11TransmitNDPOptionImplemented | False/Boolean | Static |
| dot11ExplicitCSITransmitBeamformingOptionImplemented | False/Boolean | Static |
| dot11ExplicitNonCompressedBeamformingMatrixOptionImplemented | False/Boolean | Static |
| dot11ExplicitTransmitBeamformingCSIFeedbackOptionImplemented | Implementation dependent | Static |
| dot11ExplicitNoncompressedBeamformingFeedbackOptionImplemented | Implementation dependent | Static |
| dot11ExplicitCompressedBeamformingFeedbackOptionImplemented | Implementation dependent | Static |
| dot11NumberBeamFormingCSISupportAntenna | Implementation dependent | Static |
| dot11NumberNonCompressedBeamformingMatrixSupportAntenna | Implementation dependent | Static |
| dot11NumberCompressedBeamformingMatrixSupportAntenna | Implementation dependent | Static |
| dot11BeamformeeOptionImplemented | False/Boolean | Static |
| dot11BeamformerOptionImplemented | False/Boolean | Static |
| dot11NumberSoundingDimensions | Implementation dependent | Static |
| dot11BeamformeeNTxSupport | Implementation dependent | Static |

### TXTIME calculation

The value of the TXTIME parameter returned by the PLME-TXTIME.confirm primitive shall be calculated for a QMG PPDU using Equation for SC MIMO PPDU, Equation for OFDM MIMO PPDU.

where

,,,, andare defined in Table 26-3 (Timing-related constants)

For an NDP, there is no Data field and.

For a SC MIMO PPDU, the total number of data blocks in the Data field,, is given by Equation.

### PHY characteristics

The static QMG PHY characteristics, provided through the PLME-CHARACTERISTICS service primitive, shall be as shown in Table 26- . The definitions for these characteristics are given in 6.5.

Table 26-  QMG PHY characteristics

| PHY parameter | Value |
| --- | --- |
| aRIFSTime | 1 |
| aSIFSTime | 3 |
| aRxRFDelay | Implementation dependent |
| aRxPLCPDelay | Implementation dependent |
| aRxTxTurnaroundTime | <1 |
| aCCATime | <3 |
| aTxRFDelay | Implementation dependent |
| aTxPLCPDelay | Implementation dependent |
| aRxTxSwitchTime | <1 |
| aTxRampOnTime | Implementation dependent |
| aTxRampOffTime | Implementation dependent |
| aAirPropagationTime | <600 |
| aPHY-RX-START-Delay | 11 μs |
| aMACProcessingDelay | Implementation dependent |
| aSlotTime | 5 |
| aCWmin | 15 |
| aCWmax | 1023 |
| aPPDUMaxTime | 12 ms |
| aPSDUMaxLength | 262 143 octets |
| aSCBlockSize | 256 for 540 MHz 512 for 1080 MHz |
| aPreambleLength | 8.7 μs |

## Parameters for QMG-MCSs

The rate-dependent parameters for 540 MHz, and 1080 MHz are given in Table 26-  through Table 26- . Support for short GI is optional in all cases. Support for QMG-MCSs 4-16 is optional in all cases. A QMG STA shall support single spatial stream QMG-MCSs within the range QMG-MCS 0 to QMG-MCS 3 for 540 MHz channel widths for which it has indicated support regardless of the Tx or Rx Highest Supported Long GI Data Rate subfield values in the Supported QMG-MCS and NSS Set field. When more than one spatial stream is supported, the Tx or Rx Highest Supported Long GI Data Rate subfield values in the Supported QMG-MCS and NSS Set field may result in a reduced QMG-MCS range (cut-off) for. Support for 540 MHz withand SC transmission is mandatory. Support for 540 MHz, and 1080 MHz with and SC transmission is optional. Support for 540 MHz, and 1080 MHz with OFDM transmission is optional.

### Parameters for QMG-MCSs with SC Transimission

The related parameters for different modulation are summarized for 540MHz channel bandwidth with SC transmission.

Table 26-  SC MCSs for mandatory 540 MHz, Nss=1

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 165.00 | 192.50 |
| 2 | π/2-QPSK | 1/2 | 2 | 330.00 | 385.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 495.00 | 577.50 |
| 4 | π/2-16-QAM | 1/2 | 4 | 660.00 | 770.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 990.00 | 1155.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 1237.50 | 1443.75 |
| 7 | π/2-64-QAM | 3/4 | 6 | 1485.00 | 1732.50 |
| 8 | π/2-64-QAM | 13/16 | 6 | 1608.75 | 1876.88 |

Table 26-  SC MCSs for mandatory 540 MHz, Nss=2 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 330.00 | 385.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 660.00 | 770.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 990.00 | 1155.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 1320.00 | 1540.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 1980.00 | 2310.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 2475.00 | 2887.50 |
| 7 | π/2-64-QAM | 3/4 | 6 | 2970.00 | 3465.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 3217.50 | 3753.75 |

Table 26-  SC MCSs for mandatory 540 MHz, Nss=3 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 495.00 | 577.50 |
| 2 | π/2-QPSK | 1/2 | 2 | 990.00 | 1155.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 1485.00 | 1732.50 |
| 4 | π/2-16-QAM | 1/2 | 4 | 1980.00 | 2310.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 2970.00 | 3465.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 3712.50 | 4331.25 |
| 7 | π/2-64-QAM | 3/4 | 6 | 4455.00 | 5197.50 |
| 8 | π/2-64-QAM | 13/16 | 6 | 4826.25 | 5630.63 |

Table 26-  SC MCSs for mandatory 540 MHz, Nss=4 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 660.00 | 770.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 1320.00 | 1540.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 1980.00 | 2310.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 2640.00 | 3080.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 3960.00 | 4620.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 4950.00 | 5775.00 |
| 7 | π/2-64-QAM | 3/4 | 6 | 5940.00 | 6930.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 6435.00 | 7507.50 |

The related parameters for different modulation are summarized for 1080MHz channel bandwidth with SC transmission.

Table 26-  SC MCSs for optional 1080 MHz, Nss=1 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 330.00 | 385.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 660.00 | 770.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 990.00 | 1155.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 1320.00 | 1540.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 1980.00 | 2310.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 2475.00 | 2887.50 |
| 7 | π/2-64-QAM | 3/4 | 6 | 2970.00 | 3465.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 3217.50 | 3753.75 |

Table 26-  SC MCSs for optional 1080 MHz, Nss=2 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 660.00 | 770.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 1320.00 | 1540.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 1980.00 | 2310.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 2640.00 | 3080.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 3960.00 | 4620.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 4950.00 | 5775.00 |
| 7 | π/2-64-QAM | 3/4 | 6 | 5940.00 | 6930.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 6435.00 | 7507.50 |

Table 26-  SC MCSs for optional 1080 MHz, Nss=3 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 990.00 | 1155.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 1980.00 | 2310.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 2970.00 | 3465.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 3960.00 | 4620.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 5940.00 | 6930.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 7425.00 | 8662.50 |
| 7 | π/2-64-QAM | 3/4 | 6 | 8910.00 | 10395.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 9652.50 | 11261.25 |

Table 26-  SC MCSs for optional 1080 MHz, Nss=4 (Optional)

| MCS Index | Modulation | R | NCBPS | Data Rate (Mbps) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Long GI | Short GI |
| 1 | π/2-BPSK | 1/2 | 1 | 1320.00 | 1540.00 |
| 2 | π/2-QPSK | 1/2 | 2 | 2640.00 | 3080.00 |
| 3 | π/2-QPSK | 3/4 | 2 | 3960.00 | 4620.00 |
| 4 | π/2-16-QAM | 1/2 | 4 | 5280.00 | 6160.00 |
| 5 | π/2-16-QAM | 3/4 | 4 | 7920.00 | 9240.00 |
| 6 | π/2-64-QAM | 5/8 | 6 | 9900.00 | 11550.00 |
| 7 | π/2-64-QAM | 3/4 | 6 | 11880.00 | 13860.00 |
| 8 | π/2-64-QAM | 13/16 | 6 | 12870.00 | 15015.00 |

### Parameters for QMG-MCSs with OFDM Transimission

The related parameters for different modulation are summarized for 540MHz channel bandwidth with OFDM transmission.

Table 26-  OFDM MCSs for optional 540 MHz, Nss=1 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 168 | 84 | 173.25 | 192.50 |
| 10 | QPSK | 1/2 | 2 | 336 | 168 | 346.50 | 385.00 |
| 11 | QPSK | 3/4 | 2 | 336 | 252 | 519.75 | 577.50 |
| 12 | 16-QAM | 1/2 | 4 | 672 | 336 | 693.00 | 770.00 |
| 13 | 16-QAM | 3/4 | 4 | 672 | 504 | 1039.50 | 1155.00 |
| 14 | 64-QAM | 5/8 | 6 | 1008 | 630 | 1299.38 | 1443.75 |
| 15 | 64-QAM | 3/4 | 6 | 1008 | 756 | 1559.25 | 1732.50 |
| 16 | 64-QAM | 13/16 | 6 | 1008 | 819 | 1689.19 | 1876.88 |

Table 26-  OFDM MCSs for optional 540 MHz, Nss=2 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 336 | 168 | 346.50 | 385.00 |
| 10 | QPSK | 1/2 | 2 | 672 | 336 | 693.00 | 770.00 |
| 11 | QPSK | 3/4 | 2 | 672 | 504 | 1039.50 | 1155.00 |
| 12 | 16-QAM | 1/2 | 4 | 1344 | 672 | 1386.00 | 1540.00 |
| 13 | 16-QAM | 3/4 | 4 | 1344 | 1008 | 2079.00 | 2310.00 |
| 14 | 64-QAM | 5/8 | 6 | 2016 | 1260 | 2598.75 | 2887.50 |
| 15 | 64-QAM | 3/4 | 6 | 2016 | 1512 | 3118.50 | 3465.00 |
| 16 | 64-QAM | 13/16 | 6 | 2016 | 1638 | 3378.38 | 3753.75 |

Table 26-  OFDM MCSs for optional 540 MHz, Nss=3 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 504 | 252 | 519.75 | 577.50 |
| 10 | QPSK | 1/2 | 2 | 1008 | 504 | 1039.50 | 1155.00 |
| 11 | QPSK | 3/4 | 2 | 1008 | 756 | 1559.25 | 1732.50 |
| 12 | 16-QAM | 1/2 | 4 | 2016 | 1008 | 2079.00 | 2310.00 |
| 13 | 16-QAM | 3/4 | 4 | 2016 | 1512 | 3118.50 | 3465.00 |
| 14 | 64-QAM | 5/8 | 6 | 3024 | 1890 | 3898.13 | 4331.25 |
| 15 | 64-QAM | 3/4 | 6 | 3024 | 2268 | 4677.75 | 5197.50 |
| 16 | 64-QAM | 13/16 | 6 | 3024 | 2457 | 5067.56 | 5630.63 |

Table 26-  OFDM MCSs for optional 540 MHz, Nss=4 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 672 | 336 | 693.00 | 770.00 |
| 10 | QPSK | 1/2 | 2 | 1344 | 672 | 1386.00 | 1540.00 |
| 11 | QPSK | 3/4 | 2 | 1344 | 1008 | 2079.00 | 2310.00 |
| 12 | 16-QAM | 1/2 | 4 | 2688 | 1344 | 2772.00 | 3080.00 |
| 13 | 16-QAM | 3/4 | 4 | 2688 | 2016 | 4158.00 | 4620.00 |
| 14 | 64-QAM | 5/8 | 6 | 4032 | 2520 | 5197.50 | 5775.00 |
| 15 | 64-QAM | 3/4 | 6 | 4032 | 3024 | 6237.00 | 6930.00 |
| 16 | 64-QAM | 13/16 | 6 | 4032 | 3276 | 6756.75 | 7507.50 |

The related parameters for different modulation are summarized for 1080MHz channel bandwidth with OFDM transmission.

Table 26-  OFDM MCSs for optional 1080 MHz, Nss=1 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 336 | 168 | 346.50 | 385.00 |
| 10 | QPSK | 1/2 | 2 | 672 | 336 | 693.00 | 770.00 |
| 11 | QPSK | 3/4 | 2 | 672 | 504 | 1039.50 | 1155.00 |
| 12 | 16-QAM | 1/2 | 4 | 1344 | 672 | 1386.00 | 1540.00 |
| 13 | 16-QAM | 3/4 | 4 | 1344 | 1008 | 2079.00 | 2310.00 |
| 14 | 64-QAM | 5/8 | 6 | 2016 | 1260 | 2598.75 | 2887.50 |
| 15 | 64-QAM | 3/4 | 6 | 2016 | 1512 | 3118.50 | 3465.00 |
| 16 | 64-QAM | 13/16 | 6 | 2016 | 1638 | 3378.38 | 3753.75 |

Table 26-  OFDM MCSs for optional 1080 MHz, Nss=2 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 672 | 336 | 693.00 | 770.00 |
| 10 | QPSK | 1/2 | 2 | 1344 | 672 | 1386.00 | 1540.00 |
| 11 | QPSK | 3/4 | 2 | 1344 | 1008 | 2079.00 | 2310.00 |
| 12 | 16-QAM | 1/2 | 4 | 2688 | 1344 | 2772.00 | 3080.00 |
| 13 | 16-QAM | 3/4 | 4 | 2688 | 2016 | 4158.00 | 4620.00 |
| 14 | 64-QAM | 5/8 | 6 | 4032 | 2520 | 5197.50 | 5775.00 |
| 15 | 64-QAM | 3/4 | 6 | 4032 | 3024 | 6237.00 | 6930.00 |
| 16 | 64-QAM | 13/16 | 6 | 4032 | 3276 | 6756.75 | 7507.50 |

Table 26-  OFDM MCSs for optional 1080 MHz, Nss=3 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 1008 | 504 | 1039.50 | 1155.00 |
| 10 | QPSK | 1/2 | 2 | 2016 | 1008 | 2079.00 | 2310.00 |
| 11 | QPSK | 3/4 | 2 | 2016 | 1512 | 3118.50 | 3465.00 |
| 12 | 16-QAM | 1/2 | 4 | 4032 | 2016 | 4158.00 | 4620.00 |
| 13 | 16-QAM | 3/4 | 4 | 4032 | 3024 | 6237.00 | 6930.00 |
| 14 | 64-QAM | 5/8 | 6 | 6048 | 3780 | 7796.25 | 8662.50 |
| 15 | 64-QAM | 3/4 | 6 | 6048 | 4536 | 9355.50 | 10395.00 |
| 16 | 64-QAM | 13/16 | 6 | 6048 | 4914 | 10135.13 | 11261.25 |

Table 26-  OFDM MCSs for optional 1080 MHz, Nss=4 (Optional)

| MCS Index | Modulation | R | NBPSCS | NCBPS | NDBPS | Data Rate(Mbps) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Long GI | Short GI |
| 9 | BPSK | 1/2 | 1 | 1344 | 672 | 1386.00 | 1540.00 |
| 10 | QPSK | 1/2 | 2 | 2688 | 1344 | 2772.00 | 3080.00 |
| 11 | QPSK | 3/4 | 2 | 2688 | 2016 | 4158.00 | 4620.00 |
| 12 | 16-QAM | 1/2 | 4 | 5376 | 2688 | 5544.00 | 6160.00 |
| 13 | 16-QAM | 3/4 | 4 | 5376 | 4032 | 8316.00 | 9240.00 |
| 14 | 64-QAM | 5/8 | 6 | 8064 | 5040 | 10395.00 | 11550.00 |
| 15 | 64-QAM | 3/4 | 6 | 8064 | 6048 | 12474.00 | 13860.00 |
| 16 | 64-QAM | 13/16 | 6 | 8064 | 6552 | 13513.50 | 15015.00 |

## QMG PMD sublayer

### Scope and field of application

The PMD services provided to the PLCP for the QMG PHY are described in this subsection. Also defined in this subclause are the functional, electrical, and RF characteristics required for interoperability of implementations conforming to this specification. The relationship of this specification to the entire QMG PHY is shown in Figure 26- .

Figure 26-  PMD layer reference model

### Overview of interactions

The QMG PMD sublayer accepts PLCP sublayer service primitives and provides the actual means by which data are transmitted or received from the medium. The combined function of the QMG PMD sublayer primitives and parameters for the receive function results in a data stream, timing information, and associated receive signal parameters being delivered to the PLCP sublayer. A similar functionality is provided for data transmission.

### Overview of interactions

The primitives provided by the QMG PMD fall into two basic categories:

  - Service primitives that support PLCP peer-to-peer interactions

  - Service primitives that have local significance and support sublayer-to-sublayer interactions

### Basic service and options

#### Status of service primitives

All of the service primitives described in 1.16.4 are mandatory, unless otherwise specified.

#### PMD_SAP peer-to-peer service primitives

Table 26-  indicates the primitives for peer-to-peer interactions

Table 26-  PMD_SAP peer-to-peer service primitives

| Primitive | Request | Indicator | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD | X | X | — | — |

#### PMD_SAP sublayer-to-sublayer service primitives

Table 26-  indicates the primitives for sublayer-to-sublayer interactions.

Table 26-  PMD_SAP sublayer-to-sublayer service primitives

| Primitive | Request | Indicator | Confirm | Response |
| --- | --- | --- | --- | --- |
| PMD_TXSTART | X | — | — | — |
| PMD_TXEND | X | — | — | — |
| PMD_TXPWRLVL | X | — | — | — |
| PMD_TX_PARAMETERS | X | — | — | — |
| PMD_RSSI | — | X | — | — |
| PMD_RCPI | — | X | — | — |
| PMD_CHAN_MAT | — | X | — | — |

### PMD_SAP detailed service specification

#### Introduction to PMD_SAP service specification

Subclauses 1.16.5.2 through 20.5.5.13 describe the services provided by each PMD primitive.

#### PMD_DATA.request

##### Function

This primitive defines the transfer of data from the PLCP sublayer to the PMD entity.

##### Semantics of the service primitive

This primitive shall provide the following parameters: PMD_DATA.request (TXD_UNIT).

The TXD_UNIT parameter shall be the n-bit combination of 0 and 1 for one modulation symbol. This parameter represents a single block of data that, in turn, shall be used by the PHY to be encoded into a transmitted symbol.

##### When generated

This primitive shall be generated by the PLCP sublayer to request transmission of one symbol.

##### Effect of receipt

The PMD performs transmission of the data.

#### PMD_DATA.indication

##### Function

This primitive defines the transfer of data from the PMD entity to the PLCP sublayer.

##### Semantics of the service primitive

This primitive shall provide the following parameter: PMD_DATA.indication(RXD_UNIT)

The RXD_UNIT parameter shall be 0 or 1 and shall represent either a SIGNAL field bit or a data field bit after the decoding of the FEC by the PMD entity.

##### When generated

This primitive, generated by the PMD entity, forwards received data to the PLCP sublayer.

##### Effect of receipt

The PLCP sublayer decodes the bits that it receives from the PMD and either interprets them as part of its own signaling or passes them to the MAC sublayer as part of the PSDU after any necessary additional processing (e.g., descrambling).

#### PMD_TXSTART.request

##### Function

This primitive, generated by the PHY PLCP sublayer, initiates PPDU transmission by the PMD layer.

##### Semantics of the service primitive

This primitive has no parameters.

##### When generated

This primitive shall be generated by the PLCP sublayer to initiate the PMD layer transmission of the PPDU.

The PHY-TXSTART.request primitive shall be provided to the PLCP sublayer prior to issuing the PMD_TXSTART command.

##### Effect of receipt

PMD_TXSTART initiates transmission of a PPDU by the PMD sublayer.

#### PMD_TXEND.request

##### Function

This primitive, generated by the PHY PLCP sublayer, ends PPDU transmission by the PMD layer.

##### Semantics of the service primitive

This primitive has no parameters.

##### When generated

This primitive shall be generated by the PLCP sublayer to terminate the PMD layer transmission of the PPDU.

##### Effect of receipt

PMD_TXEND terminates transmission of a PPDU by the PMD sublayer.

#### PMD_TXEND.confirm

##### Function

This primitive, generated by the PMD entity, indicates the end of PPDU transmission by the PMD layer. It is generated at the 4 μs boundary following the trailing boundary of the last symbol transmitted.

##### Semantics of the service primitive

This primitive has no parameters.

##### When generated

This primitive shall be generated by the PMD entity at the 4 μs boundary following the trailing boundary of the last symbol transmitted.

##### Effect of receipt

The PLCP sublayer determines that transmission of the last symbol of the PPDU is complete. This completion is used as a timing reference in the PLCP state machines. See 1.12.

#### PMD_TXPWRLVL.request

##### Function

This primitive, generated by the PHY PLCP sublayer, selects the power level used by the PHY for transmission.

##### Semantics of the service primitive

This primitive shall provide the following parameter: PMD_TXPWRLVL.request (TXPWR_LEVEL) TXPWR_LEVEL selects which of the transmit power levels should be used for the current packet transmission. The number of available power levels shall be determined by the MIB parameter aNumberSupportedPowerLevels. The number of available power levels are determined by the PLCP.

##### When generated

This primitive shall be generated by the PLCP sublayer to select a specific transmit power. This primitive shall be applied prior to setting PMD_TXSTART into the transmit state.

##### Effect of receipt

PMD_TXPWRLVL immediately sets the transmit power level to the level given by TXPWR_LEVEL.

#### PMD_TX_PARAMETERS.request

##### Function

This primitive, generated by the PHY PLCP sublayer, selects the related parameters used by the PHY for transmission.

##### Semantics of the service primitive

This primitive shall provide the following parameters:

PMD_TX_PARAMETERS.request (MCS, CH_BANDWIDTH, CH_OFFSET, STBC, GI_TYPE, ANTENNA_SET, FEC_CODING, PMD_EXPANSIONS_MAT, PMD_EXPANSIONS_MAT_TYPE).

##### When generated

This primitive shall be generated by the PLCP sublayer to select a specific transmit parameter. This primitive shall be applied prior to setting PMD_TXSTART into the transmit state.

##### Effect of receipt

PMD_TX_PARAMETERS immediately sets the transmit parameters. The receipt of these parameters selects the values that shall be used for all subsequent PPDU transmissions.

#### PMD_RSSI.indication

##### Function

This primitive, generated by the PMD sublayer, provides the receive signal strength to the PLCP and MAC entity.

##### Semantics of the service primitive

This primitive shall provide the following parameter: PMD_RSSI.indication (RSSI)

The RSSI shall be a measure of the RF energy received by the QMG PHY. RSSI indications of up to 8 bits (256 levels) are supported.

##### When generated

This primitive shall be generated by the PMD after the reception of the QMG training fields.

##### Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RSSI may be used as part of a CCA scheme.

#### PMD_RCPI.indication

##### Function

This primitive, generated by the PMD sublayer, provides the received channel power indicator to the PLCP and MAC entity.

##### Semantics of the service primitive

The primitive shall provide the following parameter: PMD_RCPI.indication(RCPI).

The RCPI is a measure of the channel power received by the OFDM PHY. RCPI measurement and parameter values are defined in 7).

##### When generated

This primitive shall be generated by the PMD when the OFDM PHY is in the receive state. It is generated at the end of the last received symbol.

##### Effect of receipt

This parameter shall be provided to the PLCP layer for information only. The RCPI may be used in conjunction with RSSI to measure input signal quality.

#### PMD_CHAN_MAT.indication

##### Function

This primitive, generated by the PMD sublayer, provides the channel response matrices to the PLCP and MAC entity.

##### Semantics of the service primitive

This primitive shall provide the following parameter: PMD_CHAN_MAT.indication (CHAN_MAT)

The CHAN_MAT parameter contains the channel response matrices that were measured during the reception of the current frame.

##### When generated

This primitive shall be generated by the PMD when the OFDM PHY is in the receive state. It shall be available continuously to the PLCP that, in turn, shall provide the parameter to the MAC entity.

##### Effect of receipt

The PLCP sublayer passes the data to the MAC sublayer as part of the RXVECTOR

Annex B

(normative)

Protocol Implementation Conformance Statement (PICS)

Annex C

(normative)

ASN. Encoding of the MAC and PHY MIB

Annex D

(normative)

Regulatory references

Annex E

(normative)

Country elements and operating classes