IEEE P802.11
Wireless LANs

| P802.11aq MIB and PICS text changes |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd Mississauga, ON, CANADA L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to update the MIB and PICS clauses. It addresses comments from LB208

# Annex B

## (normative)

## Protocol Implementation Conformance Statement (PICS) - proforma

## B.2 Abbreviations and special symbols

### General abbreviations for Item and Support columns

<Insert the following new list item, at the end of B.2.2>

PAD preassociation discovery

## PICS proforma - IEEE Std 802.11-<year>

### IUT configuration

<Insert the following entry at the end of the IUT configuration table>

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| *CF33 | Preassociation discovery procedure | 10.25.3.2.13(ANQP-SD 10.25.3.4 (PAD) | O | Yes  No  N/A  |

Insert the following new subclause at the end of clause B.4

### B.4.27   Preassociation Discovery Extensions

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| PAD1 | Advertisement Protocol element | 8.4.2.92 | CF33 | Yes □ No □ N/A □ |
| PAD2 | Service Hint element | 8.4.2.213 | CF33 | Yes □ No □ N/A □ |
| PAD3 | Service Advertisement element | 8.4.2.214 | CF33 | Yes □ No □ N/A □ |
| PAD4 | Service Hash element | 8.4.2.215 | CF33 | Yes □ No □ N/A □ |
| PAD5 | Supported ULP element | 8.4.2.216 | CF33 | Yes □ No □ N/A □ |
| PAD5 | Unsolicited PAD Procedure | 10.25.3.4.2 | CF33 | Yes □ No □ N/A □ |
| PAD6 | Solicited PAD Procedure | 10.25.3.4.3 | CF33 | Yes □ No □ N/A □ |

# Annex C

(normative)

## ASN.1 encoding of the MAC and PHY MIB

## C.3 MIB Detail

Change the end of the “Dot11StationConfigEntry” of the “dot11StationConfig TABLE” as follows:

dot11GDDActivated TruthValue,

dot11GDDEnablementTimeLimit Unsigned32,

dot11GDDEnablementFailHoldTime Unsigned32,

dot11GDDEnablementValidityTimer Unsigned32

dot11PADActivated, Truthvalue,

dot11UnsolicitedPADActivated, Truthvalue

}}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11PADActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation.

This attribute when true, indicates that the capability of the STA to operate Solicited PAD with external networks is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <ANA> }

dot11UnsolicitedPADActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation.

This attribute when true, indicates that the capability of the STA to operate Unsolicited PAD with external networks is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <ANA> }

dot11EncapsulatedPADActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take effect as soon as practical in the implementation.

This attribute when true, indicates that the capability of the STA to operate Encapsulated PAD with external networks is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <ANA> }

Insert the following compliance statement after the “Compliance Statements – S1G” section:

-- ********************************************************************

-- * Compliance Statements - PAD

-- ********************************************************************

dot11PADComplianceGroup OBJECT-GROUP

OBJECTS {

dot11PADActivated,

dot11UnsolicitedPADActivated }

STATUS current

DESCRIPTION

"This object group provides the objects from the IEEE 802.11

MIB required to manage Preassociation Discovery functionality."

::= { dot11Groups <ANA> }

dot11PADCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"This object class provides the objects from the IEEE 802.11

MIB required to manage Preassociation discovery functionality."

MODULE -- this module

MANDATORY-GROUPS {

dot11PADComplianceGroup }

::= { dot11Compliances 6 }

1Copyright release for PICS proforma: Users of this standard may freely reproduce the PICS proforma in this annex so that it can be used for its intended purpose and may further publish the completed PICS.