IEEE P802.11
Wireless LANs

| A Possible Way of Using Service Hash in 11aq |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| Ping Fang | Huawei Technologies |  |  |  |

Abstract

This document provides the 11aq Amendment Draft text for a possible way of using service hash in 11aq to address CID 1353.

<The first change agaist D1.2>

## 3.1   Definitions

service hash:  A value that is used for representing a service and is formed by  truncating 6 consecutive octets of the SHA-256 hash output of the service name, where the range of the octet numbers of the truncation depends on the frame that carries the service hash.

<The second change against D1.2>

#### 8.4.2.173 Service Hash element

The Service Hash element consists of one or more Service Hashes. The Service Hash element is included in Beacon, Probe Request and Probe Response frames.

The format of the Service Hash element is shown in Figure 8-577e.

|  | Element ID |  | Length | Service Hash |
| --- | --- | --- | --- | --- |
| Octets | 1 | 1 |  | Multiple 6-octet service hashes |

Figure 8-577e – Service Hash element format

The Element ID and Length fields are defined in 8.4.2.1 (General).

The Service Hash field contains one or more 6-octet service hashes. A service hash in the Service Hash field of the Service Hash element is generated as follows:

service hash = L(SHA-256(service name), 0, 48).

<The third change against D1.2>

#### 8.4.5.20 Service Information Request ANQP-element

The Service Information Request ANQP-element is sent by the non-AP STA to an AP to request service information. It is included in a GAS Query Request.

The format of the Service Information Request ANQP-element is shown in Figure 8-607a.

|  | Info ID | Length |  | Service Hash | Service Information Query Length | Service Information Query |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 |  | 6 | 1 | variable |

Figure 8-607a – Service Information Request ANQP-element format

The Info ID and Length fields are defined in 8.4.5.1.

The service hash in the Service Hash field of the Service Information Request ANQP-element is generated as follows:

service hash = L(SHA-256(service name), 48, 48).

The Service information Query Length field contains the length of the Service Information Query field.

The Service Information Query field contains service-specific query, such as key-value query.

The procedure used for this element is described in clause 10.25.3.2.11.1.

#### 8.4.5.21 Service Information Response ANQP-element

The Service Information Response ANQP-element provides detailed service information between STAs, using the GAS protocol, in response to a Service Information Request ANQP-element. The Service Information Response ANQP-element is included in a GAS Query Response, sent by the AP to the non-AP STA.

The format of the Service Information Response ANQP-element is shown in Figure 8-607b.

|  | Info ID |  | Length | Service Hash | Detailed Service Information Descriptors |
| --- | --- | --- | --- | --- | --- |
| Octets | 2 | 2 |  | 6 | variable |

Figure 8-607b - Service Information Response ANQP-element format

The Info ID and Length fields are defined in 8.4.5.1.

The service hash in the Service Hash field of the Service Information Response ANQP-element is generated as follows:

service hash = L(SHA-256(service name), 96, 48).

The Detailed Service Information Descriptors field contains one or more Detailed Service Information Descriptor subfields (Figure 8-607c).

The format of the Detailed Service Information Descriptor subfield is shown in Figure 8-607c

|  | Service Instance | Service Information Query Response Length | Service Information Query Response |
| --- | --- | --- | --- |
| Octets | 4 | 2 | variable |

Figure 8-607c – Detailed Service Information Descriptor subfield format

The Service Instance field is is a 4-octet unsigned integer assigned by the Proxy for the service.

The Service Information Query Response field is a variable length field. The format of the Service Information Query Response is service-specific that contains requested service information.

The procedure used for this element is described in clause 10.25.3.2.11.2

<The fourth change against the approved (by Motion #15) Doc. 11-15-0890r1>

10.26.5 Bloom filter Hash function operation

….

X is the service hash that is mapped into the Bloom Filter Bit Array field. A service hash used to map into the Bloom Filter Bit Array field is generated as follows:

service hash = L(SHA-256(service name), 0, 48).