IEEE P802.11
Wireless LANs

| 802.11 REVmc Initial Sponsor ballot - some proposed comments resolutions (Stephens) – Part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-11-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains some proposed resolutions to SB1 comments.

## History

R1: CID 5046, 6467, 6314, 6656, 6410, 6603, (6565,) (6561,) 6657, 6477, 6788, 6821, 6456, 6433, 6560, 6756

R2: Reviewed in TGmc F2F 2015-08-20.

R3: Reviewed in TGmc F2F 2015-08-21.

R4: Updated resolution to CID 6788.  Moved approved CIDs to end.  Added CIDs 5046, 6788, 6587, 6235, 3038, and added resolutions where possible to remaining GEN CIDs: 6410, 6717, 6565, 6433, 6520, 6756, 6787, 5048, 6747, 5059, 6720, 6737.

R5: Reviewed in TGmc Telecon 2015-08-28.

R6: 2015-09-07  Added a bunch of MAC unassigned comments from Clause 8.

R7: Continued work on the MAC unassigned comments.

R8: incorporating review comments to R7 tracked like this.  Comments updated in R8 will show “R8” somewhere in the resolution.

R9: Additional changes tracked with “R9”.

R10: updated during TGmc F2F.

R12: updated during TGmc F2F, wed pm2.

R14: discussed during TGmc telecon 2015-10-09

R15: updated during TGmc F2F 2015-10-14 & 2015-10-15

R16: approved comments moved to bottom.

R16: after review by TGmc telecon 2015-11-20

## R16 List of comments contained:

5038 (Editor),  6226 & 5070 (MAC),  6415 (MAC), 6042 & 5031 (MAC),  5141 (MAC)

## R13 List of comments contained:

(parens indicate partial or no solution)

The following comments are assigned to the author.

(5038), 5048, 5059, 5070, (5141), 5310, 5863, 6051, 6052, 6088, 6226, 6274, 6297, 6323, (6396), 6410, (6433), 6456, 6525, 6531, (6560), 6565, 6587, 6667, 6668, 6669, 6717, 6720, 6737, 6747, 6756, 6787, 6789, 6819

In addition, Resolutions are proposed to the following comments, which are not assigned to the author:

MAC: 6455 (Was MAC, now EDITOR), 6456, 6254, 6054, 5041, 5071, 6413, 6415, 6414, 6532, 6533, 6202, 6098, 5061,  6042, 6044,  (5025), 5026, 5027, 5029, (5141), 6610

# Comments

| 5054 | 826.56 | 8.4.2.26 |  | Naming a field of the Extended Capabilities element as "Capabilities" can cause confusion with the Capability Information field. | Name this field "Extended Capabilities" globally. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Here are the left collocates of “Capabilities field”

#Total No. of Collocate Types: 12

#Total No. of Collocate Tokens: 129

1	35	rsn

2	29	beamforming

3	28	extended

4	4	addba

5	4	enabled

6	7	timing

7	2	time

8	1	dmg

9	7	x

10	1	ht

11	1	table

12	10	the

There are a bunch of different capabilities fields.   There is already an “HT Extended Capabilities field”,  so potentially we are adding confusion.

I do note that one of the “Extended Capabilities field” references is actually a reference to the Capabilities field.

All the references to “the Capabilities field” can safely be changed to “the Extended Capabilities field”.

Status:  This resolution was previously approved by straw poll.  It was then pulled (according to 11-15/1251/r2) because “1.1.1.2	Wish to discussion to ensure RSN Capabilities”.  I have reviewed the changes in the light of this rationale and see nothing to change.

Proposed Change:

Revised.  Globally change “the Capabilities field” to “the Extended Capabilities field”.

At 826.56, 827.01, 827.02 (2x), 827.07 change “Capabilities” to “Extended Capabilities”

Action:  Mark to research and indicate.

## CID 5038 (Editor)

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5038 |  |  |  | The "Action field format" tables are inconsistent in their interpretation as to the meaning of the "Information" field. Either it names "a field" which might contain a field, an element, or multiple of them. (e.g. the "Relay Status Code" field contains a Status Code); or the name of the field is absent, and it identifies the information that goes into it (e.g. "one or more elements"). | Recommend that this be discussed and see if a simple fix to create a single interpretation is possible. For example, to make the Information hold "[Name: ][field¦element]". This would entail adding missing when the information holds the definition of a new name, and adding field or element to every entry that does not have it. Furthermore, many of these tables are followed by a list of "the xyz field is defined in ". These could be combined into the table by moving the xref into the information column, e.g. in parens after field or element. So "Category" becomes "Category (8.4.1.11)". Or we could add a new column for that purpose. So, in 8.6.20.15, "Destination Status Code" would become "Destination Status Code: Status Code field (optional)" |  | EDITOR |

Discussion:

At the time of writing 2015-08-27, I am daunted by the potential amount of work involved.   So I am not ready to propose a resolution.  This CID is here as a placeholder.

If we need to make a decision on this comment, then it should a “reject for lack of specific detail”.

## MAC Clause 8 unassigned

| 6226 | 726.01 | 8.4.2.10 |  | It is not possible to request elements with an element ID extension | Add words to that effect |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 5070 | 726.08 | 8.4.2.10 |  | Anything that purports to identify elements using an Element ID by itself does not support the Element ID extension mechanism introduced in D4.0. The Request element needs to be extended. | Create an Extended Request element, which contains a sequence of 2 octet tuples consisting of an Element ID octet and an optional Element ID Extension octet. We might also want to create a forward compatible method to support 3-octet element IDs for some time in the future. Whereever a frame supports a Request element, add the Extended Request element. Add text in clause 9 that indicates that any non-extended element IDs requested are put in the existing Request element, and any new ones in the Extended Request element. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

It is not possible to list extended element IDs in the request element, because that element assumes they are a single octet.  If we did start listing 2-octet element IDs, a non-understanding receiver would determine the Element ID Extension field was the Element ID with unpredictable results.

Proposed Resolution: (to both CID 6226 and 5070).

Revised.  Make changes under CIDs 6226 and 5070 in <this-document>.  These changes add a new Extended Request element, with appropriate processing thereof.

Proposed changes:

At 726.180:

The Requested Element IDs are the list of elements that are requested to be included in the Probe Response or Information Response frame. The Requested Element IDs are listed in order of increasing element ID. The Requested Element IDs within a Request element transmitted in a Probe Request frame should not include an element ID that corresponds to an element that will be included in the Probe Response frame even in the absence of the Request element, or will be excluded from the Probe Response frame even in the presence of the Request element as described by the notes in Table 8-34 (Probe Response frame body). The Requested Element IDs within a Request element transmitted in an Information Request frame do not include an element ID that corresponds to an element that will be included in the Information Response. frame even in the absence of the Request element, as shown in Table 8-375 (Information Response frame Action field format). A given element ID is included at most once among the Requested Element IDs. The Request element does not support the use of extended Element IDs, i.e., Element IDs that have a defined Element ID Extension field.

Option 2.

Option 1 +

At: 709.26 insert a new extended element row

| - Element IDsinput.doc(#1429) |  |  |  |
| --- | --- | --- | --- |
| Element | Element ID | Element ID Extension (M82) | Extensible |
| Reserved for elements using the Element ID Extension field (M82) | 255 | 0–255 |  |
| Extended Request | 255 | <ANA> |  |

At 726.32 (after Request Element), insert the following new subclause:

8.4.2.10a Extended request element This element is placed in a Probe Request frame or Information Request frame(#3232) to request that the responding STA include the requested information in the Probe Response frame or Information Response frame, respectively(#3232). The format of the element is as shown in Request element. Element ID; Length; Element ID Extension; Requested Element ID; Requested Element ID Extensions. Octets:; 1; 1; 1; 1; variable. - Request element. The Element ID, Element ID Extension, and Length fields are defined in Error: Reference source not found.(#139) The Requested Element ID field contains one of the Element IDs used to indicate an extended element. The Requested Element ID Extensions field contains a list of 1-octet element ID extension values that, combined with the value of the Requested Element ID field, identify elements that are requested to be included in the Probe Response or Information Response(#3232) frame. The values in this field are listed in increasing order. The requested elements within an Extended request element transmitted in a Probe Request frame(#3232) do not identify an element that will be included in the Probe Response frame even in the absence of the Request element, or will be excluded from the Probe Response frame even in the presence of the Extended Request element as described by(Ed) the notes in Table 8-34 (Probe Response frame body). The requested elements within an Extended request element transmitted in an Information Request frame do not identify an element that will be included in the Information Response frame even in the absence of the Extended request element, as shown in Table 8-375 (Information Response frame Action field format)(#3232). A given element ID extension is included at most once in the Requested Element ID Extensions field.(#3355) See (#3354)10.1.4.3.5 (Contents of a probe response) (#3355) for additional requirements.

(New changes 2015-10-23: not shown with tracking.)

At 147.27:

Change under “Type” column: “As defined in 8.4.2.10 (Request element)” to “A list of (Extended) Element IDs”.

Change under “Valid range” column: “As defined in 8.4.2.10 (Request element)” to “As defined in 8.4.2.10 (Request element) and 8.4.2.10a (Extended Request element)”.

Change under “Description” column: “This element is optionally present” to “This parameter is optionally present”

At 151.58 change “by the Request element” to “by the Request element or Extended Request element”

At 634.16 (Probe Request frame body) insert a new row:

18 / Extended Request / The Extended Request element is optionally present if dot11RadioMeasurementActivated is true.

At 638.34 change “by the Request element of” to “by the Request element or Extended Request element(s) of”

At 741.61 insert in Optional subelement IDs for Beacon Request table a new row: (and adjust reserved accordingly)

11 / Extended Request / <blank>

At 743.63 change: “The Request, AP Channel Report, and Vendor Specific subelements have the same format”

to: “The Request, Extended Request, AP Channel Report, and Vendor Specific subelements have the same format”

At 776.52 change “all fixed fields and any elements whose Element IDs are present in the Request element in the corresponding Beacon request are included in the Reported Frame Body subelement, in the order that they appeared in the reported frame.”

to “all fixed fields and any elements identified in a Request element or Extended Request element in the corresponding Beacon request are included in the Reported Frame Body subelement, in the order that they appeared in the reported frame.”

At 1205.38, before the “Last” row, insert a new row:

“7 / Zero or more Extended Request elements”

At 1205.53 insert a new para:

“The Extended Request element is described in 8.4.2.10a (Extended Request element).”

At 1206.46 delete the para: “The Request element field is described in 8.4.2.10 (Request element).”

At 1538.41 change: “NOTE—MLME-SCAN.request primitives and resulting Probe Request frames may include a Request element that can

be used to request radio measurement information from the scanned BSSs.”

to: NOTE—MLME-SCAN.request primitives and resulting Probe Request frames may include a Request element or Extended Request element(s) that can be used to request radio measurement information from the scanned BSSs.

Change 1543.43 as follows:

10.1.4.3.5 Contents of a probe response A STA that responds to a Probe Request frameaccording to10.1.4.3.4 (Criteria for sending a probe response) shall transmit a Probe Response frame individually addressed to the STA that transmitted the Probe Request frame. If there was a Request element or Extended Request element in the Probe Request frame, then: — Each element that is listed in the Request element or Extended Request element(s) and that is supported by the STA shall be included in the Probe Response frame. An element that is listed in a Request element or Extended Request element and that is not supported by the STA shall not be included. — Elements that would not have been included otherwise shall be included after all of the elements that would have been included even in the absence of the Request element or Extended Request element. — Elements that would have been included even in the absence of the Request element or Extended Request element shall be included in their normal position (see Table 8-34 (Probe Response frame body)), and may be included again after all of the elements that would have been included even in the absence of the Request element or Extended Request element. NOTE—An element that would necessarily be included anyway is not expected to be requested. — Elements after all of the elements that would have been included even in the absence of the Request element or Extended Request element shall be included in the same order as in the Request element or Extended Request element(s). — If dot11RadioMeasurementActivatedis true and the RCPI element was requested, an RCPI element containing the RCPI of the Probe Request frame shall be included. If no measurement result is available, the RCPI value shall beset to indicate that a measurement is not available (see 8.4.2.37 (RCPI element) and Table 16-9 (RCPI values)).

Change 1669.08 as follows:

If a STA accepts a Measurement Pause request it shall delay processing of the next measurement request in the Measurement Request frame. If the Measurement Pause request is the last Measurement Request element in a repeated Measurement Request frame, the STA shall delay processing the first Measurement Request element in the Measurement Request frame for the next repeat. In each case the delay shall be no less than the Pause Time value specified in the Measurement Pause request. NOTE—Measurement pause is always supported by a STA implementing radio measurements. A measurement pause shall not be sent as the only Measurement Request element in a Measurement Request frame. A measurement pause shall not be included as the last Measurement Request element in a Measurement Request frame that has the Number of Repetitions field equal to 0.

At 1809.18 change:

A STA may transmit an Information Request frame with the length field of the Request element set to 0 and with no Extended Request element present to another STA in the PBSS to determine if the destination DMG STA is still present in the PBSS and is within range of the sending STA.

At 1809.35 change:

If there was a Request element with a Length field > 0 or there was an Extended Request element in the Information Request frame received by a STA, then: — Each element that is listed in a Request element or Extended Request element and that is supported by the STA shall be included in the Information Response frame.An element that is listed in a Request element or Extended Request element and that is not supported by the STA shall not be included. — If dot11RadioMeasurementActivated is true and the RCPI element was requested, an RCPI element containing the RCPI of the Information Request frame shall be included. If no measurement result is available, the RCPI value shall beset to indicate that a measurement is not available (see 8.4.2.37 (RCPI element)).

At 2719.26,  Rename MD5 row as MD5.1 and insert an MD5 row above:

MD5 / Request mechanism / <blank> / <blank> / <blank>

At 2719.33 insert a new row as MD5.2

MD5.2 / Extended Request element / 8.4.2.10a / CF8:M / Yes, No, N/A

At 2719.49 change “Ignore improperly formed Request element” to “Ignore improperly formed Request element or Extended Request element”

and insert reference to 8.3.3.10a in “References” column.

## CID 6415

| 6415 | 917.33 | 8.4.2.68.5 |  | "The Channel Number field indicates a current operating channel, or a center frequency index of the frequency segment (if the indicated channel comprises noncontiguous frequency segments)" -- the parenthesis makes no sense. 1) What is its precedence w.r.t. the "or"? 2) Which frequency segment is being referred to, if there is more than one? | Clarify (maybe need to say "of frequency segment 0"?) |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

We previously discussed this and aggred to a resolution along the lines of “if the corresponding operating class in Annex E has “—“ in the channel set column”

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6413 | 907.23 | 8.4.2.66.4 | V | "The Channel Number field indicates the channel number, or center frequency index of the frequency segment if the identified channel comprises noncontiguous frequency segments" -- the bit from "if" onwards makes no sense. 1) What is its precedence w.r.t. the "or"? 2) Which frequency segment is being referred to, if there is more than one? | Clarify (maybe need to say "of frequency segment 0"?) | REVISED (MAC: 2015-10-09 14:39:07Z): Make changes under CID 6413 in 11-15/1010r14 (https://mentor.ieee.org/802.11/dcn/15/11-15-1010-14-000m-revmc-sb0-stephens-resolutions-part-2.doc). These clarify that the frequency segment is that containing the primary 20MHz channel. | EDITOR |

Related CID 6413, which has been approved as follows:

Change at 907.23:

The Channel Number field indicates the channel number or (if the corresponding operating class in Annex E has “—“ in the channel set column) the center frequency index of the frequency segment containing the primary 20 MHz channel if the identified channel comprises noncontiguous frequency segments of the Peer-to-Peer Link events requested and included in the Peer-to-Peer Link event report. A Channel Number of 0 in all N+1 Channel Number subelements indicates a request to report any Peer-to-Peer Link event for any supported channel in the specified filtering Operating Class.

Seeing as the comment CID 6413 closely matches 6415,  but in a different location,  I believe the resolution should also match.

Proposed Resolution:

Revised. Make changes under CID 6415 in <this-doc>. These clarify that the frequency segment is that containing the primary 20MHz channel.

Proposed Change:

At 917.34:

The Channel Number field indicates a current operating channel or (if the corresponding operating class in Annex E has “—“ in the channel set column) the center frequency index of the frequency segment containing the primary 20 MHz channel, of the AP identified by the BSSID in the AP Descriptor.

| 6042 | 979.05 | 8.4.2.106 |  | It is confusing to read about the MCCAOP Setup Request element in the clause on the MCCAOP Setup _Reply_ element. (although it is correct, if you have read the clause on the MCCAOP Setup Request element and spent a second thought). | Since the MCCAOP Setup Request element corresponds to an MCCAOP reservation, replace "MCCAOP Setup Request element" with "MCCAOP reservation" (page 979, lines 5 and 7). Make a similar replacement in the clause on the MCCAOP Setup Request element. (page 978, lines 5 and 7) |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 5031 | 1030.36 | 8.4.2.147 |  | "It is set to 1 if the STA supports both Link cooperating type and Link switching type. It is set to 0 if a STA supports only Link switching or if the Duplex subfield is set to 1." So, what happens if the STA supports both Link cooperation, link switching and duplex? "Link cooperating type and Link switching type" -- the capitalization of the various "Link" types doesn't follow WG style. The language it poor, in what sense is a "type" supported. | Reword thus: "It is set to 1 if the STA supports both link cooperation and link switching. It is set to 0 otherwise." |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

“Link cooperating” is used as both a noun and an adjective.  As an adjective, it is followed by “type”, “mode” and “field”.

This is awkward and inconsistent.   Use of the gerund is awkward.

We should also make the same changes for “link switching type”,  although here “switching” is acceptable.

The proposed rewording to “link cooperation” doesn’t address the global problem.

Status: Action:  Adrian to check with Carlos/Payam,  what happens if a STA supports all three?

Update:  Carlos OK with it.  No repsonse from Payam.

Proposed Resolution:

Revised.

At 1030.36 change:

"It is set to 1 if the STA supports both Link cooperating type and Link switching type. It is set to 0 if a STA supports only
Link switching or if the Duplex subfield is set to 1."

To:

"It is set to 1 if the STA supports both link cooperation and link switching. It is set to 0 otherwise."

Globally change (case insensitive) “link cooperating type” to “link cooperation”.

Globally change (case insensitive) “link cooperating mode” to “link cooperation”.

At 1217.38 change “Link Cooperating” to “Link Cooperation”

Globally change (case insensitive) “link cooperating subfield” to “Link Cooperation subfield”.

Globally change (case insensitive) “link cooperating” to “link cooperation”.

Globally change (case insensitive) “link switching type” to “link switching”.

Globally change (case insensitive) “link switching mode” to “link switching”.

Globally change (case insensitive) “link switching” to “link switching”.

## MAC assigned

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5141 | 1329.56 | 9.22.2.7 |  | "All other channel access functions at the STA shall treat the medium as busy" "treat the medium as busy" is underspecified. Better to include this in the definition of virtual carrier sense. | Add to 9.3.2.1 a statement that the virtual carrier sense indicates medium busy during the TXNAV duration and reference 9.22.2.7 for its definition. |  | MAC |

Discussion:

Is there a single TXNAV timer or not?

In reviewing all occurences of TXNAV, apart from that at the cited location,  there appears to be only a single TXNAV timer.

So the question is what are the “other channel access functions” at the cited location?

Are they EDCAFs for other ACs, or are they any other channel access mechanisms,  such as transmission of the Beacon at TBTT (which is a famously individual and unnamed channel access funtion)?

I suggest we explicitly make the TXNAV time a single timer.  The previous discussion moved towards modifying the language in 9.22.2 to explicitly include the effect of the TXNAV timer.

- HCF - HCF (#2203)contention based channel access (EDCA)(11ad) - EDCA backoff procedure Each EDCAF shall maintain a state variable CW[AC], which shall be initialized to the value of the parameter CWmin[AC], for that EDCAF’s AC.(#2458) For the purposes of this subclause, (#2458)transmission failure of an MPDU(11ac) is defined as follows: - After transmitting an MPDU (even if(11ac) it is carried in an A-MPDU or as part of a VHT MU PPDU that might have TXVECTOR parameter NUM_USERS > 1)(11ac) that requires an immediate frame as a response, the STA shall wait for a timeout interval of duration of aSIFSTime + aSlotTime + aRxPHYStartDelay(#1486), starting at the PHY-TXEND.confirm primitive. If a PHY-RXSTART.indication primitive does not occur during the timeout interval, the STA concludes that the transmission of the MPDU has failed. - If a PHY-RXSTART.indication primitive does occur during the timeout interval, the STA shall wait for the corresponding PHY-RXEND.indication primitive to recognize(#2458) a valid response frame sent by the recipient of the MPDU requiring a response. The recognition of anything else, including any other valid frame, shall be interpreted as failure of the MPDU transmission.(#2458) - The nonfinal (re)transmission of an MPDU that is delivered using the GCR unsolicited retry retransmission policy (Error: Reference source not found)) is defined to be a failure.(11aa)(#2458) The backoff procedure shall be invoked by(#2458) an EDCAF when any of the following events occurs: - An MA-UNITDATA.request primitive is received that causes a frame with that AC to be queued for transmission such that one of the transmit queues associated with that AC has now become non-empty and any other transmit queues associated with that AC are empty;(#1439) the medium is busy on the primary channel(11ac) as indicated by physical CS, virtual CS, or a non-zero TXNAV timer value; and the backoff timer has a value of 0 for that AC. - The transmission of the MPDU in the final PPDU transmitted(11ac) by the TXOP holder during the TXOP for that AC has completed(#285) and the TXNAV timer has expired, and the AC was a primary AC.(11ac) (See Error: Reference source not found)(#2458) - The expected immediate response to(11ac) the initial frame of a TXOP of that AC is not received and the AC was a primary AC.(11ac) - The transmission attempt collides internally with another EDCAF of an AC that has higher priority, that is, two or more EDCAFs in the same STA are granted a TXOP at the same time.(11ac) The transmission attempt of a STA coordinated by an MM-SME collides internally with another STA coordinated by the same MM-SME (see 10.34 (MMSL cluster operation)), which is indicated to the first MAC entity with a (#2123)PHY-TXBUSY.indication(BUSY) (MDR)primitive as response to the PHY-TXSTART.request primitive. (11ad)(#2458) In addition, the backoff procedure may be invoked for an EDCAF when the transmission of the MPDUs in a non-initial PPDU by the TXOP holder fails.(11ac) NOTE 2(#1101)—A STA can perform a PIFS recovery, as described in Multiple frame transmission in an EDCA TXOP, or perform a backoff, as described in the previous paragraph, as a response to transmission failure within a TXOP. How it chooses between these two is implementation dependent. A STA that performs a backoff within its existing TXOP shall not extend the TXNAV timer value (see Multiple frame transmission in an EDCA TXOP).(#2458) NOTE 3(#1101)—In other words, the backoff is a continuation of the TXOP, not the start of a new TXOP. If the backoff procedure is invoked for reason a) above, the value of CW[AC] shall be left unchanged. If the backoff procedure is invoked because of reason b) above, the value of CW[AC] shall be reset to CWmin[AC]. NOTE 4—If condition b) or c) occurs for a secondary AC, the backoff for the associated EDCAF continues without change to the backoff counter or to the value of CW[AC].(11ac) If the backoff procedure is invoked because of a failure event [reason c), d), or e)(11ad) above or the transmission failure of a non-initial frame by the TXOP holder], the value of CW[AC] shall be updated as follows before invoking the backoff procedure: - If the QSRC[AC] or the QLRC[AC] (#1056)has reached dot11ShortRetryLimit or dot11LongRetryLimit respectively, CW[AC] shall be reset to CWmin[AC]. - If dot11RobustAVStreamingImplemented is true and either(#1056) the QSDRC[AC] or the QLDRC[AC] has reached dot11ShortDEIRetryLimit or dot11LongDEIRetryLimit, respectively, CW[AC] shall be reset to CWmin[AC].(11aa) - Otherwise, - If CW[AC] is less than CWmax[AC], CW[AC] shall be set to the value (CW[AC] + 1)×2 – 1. - If CW[AC] is equal to CWmax[AC], CW[AC] shall be left(#2458) unchanged.(#2458) - EDCA TXOPs There are three(11ac) modes of EDCA TXOP defined:(Ed) initiation of an EDCA TXOP, sharing an EDCA TXOP,(11ac) and multiple frame transmission within an EDCA TXOP. Initiation of the TXOP occurs when the EDCA rules permit access to the medium. Sharing of the EDCA TXOP occurs when an EDCAF within an AP that supports DL-MU-MIMO(#2458) has obtained access to the medium, making the corresponding AC the primary AC, and includes traffic from queues associated with other ACs in VHT MU PPDUs transmitted during the TXOP.(11ac) Multiple frame transmission within the TXOP occurs when an EDCAF retains the right to access the medium following the completion of a frame exchange sequence, such as on receipt of an (#1198)Ack frame. - Obtaining an EDCA TXOP Each EDCAF(#2458) shall maintain a backoff timer, which has a value measured in backoff slots as described below.(#2458) When the backoff procedure is invoked, the(#2458) backoff timer is set to an integer value chosen randomly with a uniform distribution taking values in the range [0,CW[AC]] inclusive. The duration AIFS[AC] is a duration derived from the value AIFSN[AC] by the relation AIFS[AC] = AIFSN[AC] × aSlotTime + aSIFSTime. In an infrastructure BSS, AIFSN[AC] is advertised by an EDCA AP in the EDCA Parameter Set element in Beacon and Probe Response frames transmitted by the AP.(#2458) The value of AIFSN[AC] shall be greater than or equal to 2 for non-AP STAs.(#2437)(#2458) The value of AIFSN[AC] shall be greater than or equal to 1 for APs. An EDCA TXOP is granted to an EDCAF when the EDCAF determines that it shall initiate the transmission of a frame exchange sequence. Transmission initiation shall be determined according to the following rules: EDCAF(#2458) operations shall be performed at slot boundaries, defined as follows on the primary channel,(#2458) for each EDCAF: - Following AIFSN[AC] × aSlotTime – aRxTxTurnaroundTime of idle medium after SIFS (not necessarily idle medium during the SIFS(#156)) after the last busy medium on the antenna that was the result of a reception of a frame with a correct FCS. - Following EIFS – DIFS + AIFSN[AC] × aSlotTime + aSIFSTime – aRxTxTurnaroundTime of idle medium after the last indicated busy medium as determined by the physical CS mechanism that was the result of a frame reception that has resulted in FCS error, or PHY-RXEND.indication (-RXERROR) primitive where the value of RXERROR is not NoError. - When any other EDCAF at this STA transmitted a frame requiring acknowledgment, the earlier of - The end of the (#1627)(#3338)AckTimeout interval timed from the (#1601)PHY-TXEND.confirm primitive, followed by AIFSN[AC] (#1630)× aSlotTime + aSIFSTime – aRxTxTurnaroundTime of idle medium, and - The end of the first AIFSN[AC] × aSlotTime – aRxTxTurnaroundTime of idle medium after SIFS (not necessarily medium idle during the SIFS(#156), the start of the SIFS(#156) implied by the length in the PHY(#61) header of the previous frame) when a PHY-RXEND.indication primitive occurs as specified in Error: Reference source not found. - Following AIFSN[AC] × aSlotTime – aRxTxTurnaroundTime of idle medium after SIFS (not -necessarily medium idle during the SIFS(#156)) after the last busy medium on the antenna that was the result of a transmission of a frame for any EDCAF and which did not require an –acknowledgment or the expiration of the TXNAV timer, if non-zero. - Following AIFSN[AC] × aSlotTime + aSIFSTime – aRxTxTurnaroundTime of idle medium after the last indicated idle medium as indicated by the CS mechanism that is not covered by a) to d). - Following aSlotTime of idle medium, which occurs immediately after any of these conditions, a) to f), is met for the EDCAF. On these(#2458) specific slot boundaries(#2458) each EDCAF shall make a determination to perform one and only one of the following functions: - Decrement the backoff timer.(#2458) - Initiate the transmission of a frame exchange sequence.(#2458) - Invoke the backoff procedure due to an internal collision. - Do nothing.(#2458) NOTE—In the case that an EDCAF gains access to the channel and transmits MSDUs, A-MSDUs, or MMPDUs from a secondary AC, the EDCAF of the secondary AC is not affected by this operation. If the EDCAF of a secondary AC experiences an internal collision with the EDCAF that gained access to the channel, it performs the backoff procedure regardless of the transmission of any of its MSDUs, A-MSDUs, or MMPDUs (See Error: Reference source not found).(11ac) At each of the above-described specific slot boundaries, each EDCAF shall decrement the backoff timer if the backoff timer for that EDCAF has a nonzero value. At each of the above-described specific slot boundaries, each EDCAF shall initiate a transmission sequence if - There is a frame available for transmission at that EDCAF, and - The backoff timer for that EDCAF has a value of 0, and - Initiation of a transmission sequence is not allowed to commence at this time for an EDCAF of higher UP. At each of the above-described specific slot boundaries, each EDCAF shall report an internal collision (which is handled in Error: Reference source not found) if - There is a frame available for transmission at that EDCAF, and - The backoff timer for that EDCAF has a value of 0, and - Initiation of a transmission sequence is allowed to commence at this time for an EDCAF of higher UP. An example showing the relationship between AIFS, AIFSN, DIFS, and slot times immediately following a medium busy condition (and assuming that medium busy condition was not caused by a frame in error) is shown in Error: Reference source not found. In this case, with AIFSN = 2, the EDCAF may decrement the backoff counter for the first time at 2 × aSlotTime following the (#1610)TxSIFS (where (#1610)TxSIFS is the time at which the MAC responds to the end of the medium busy condition if it is going to respond “after SIFS”). If, in this example, the backoff counter contained a value of 1 at the time the medium became idle, transmission would start as a result of an EDCA TXOP on-air at a time aSIFSTime + 3 × aSlotTime following the end of the medium busy condition. A STA shall save the TXOP holder address for the BSS in which it is associated, which is the MAC address from the Address 2 field of the frame that initiated a frame exchange sequence except when this is a CTS frame, in which case the TXOP holder address is the Address 1 field. If the TXOP holder address is obtained from a Control frame(Ed), a VHT STA shall save the (MDR)nonbandwidth signaling TA value obtained from the Address 2 field. If a non-VHT STA receives(11ac) an RTS frame with the RA address matching the MAC address of the STA and the MAC address in the TA field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV. If a VHT STA receives an RTS frame with the RA address matching the MAC address of the STA and the (MDR)nonbandwidth signaling TA value obtained from the Address 2 field in the RTS frame matches the saved TXOP holder address, then the STA shall send the CTS frame after SIFS, without regard for, and without resetting, its NAV.(11ac) When a STA receives a frame addressed to it that requires an immediate response, except for RTS, it shall transmit the response independent of its NAV. The saved TXOP holder address shall be cleared when the NAV is reset or when the NAV counts down to 0. - EDCA channel access in a VHT or TVHT(11af)(Ed) BSS(11ac) If the MAC receives a PHY-CCA.indication primitive with the channel-list parameter present, the channels considered idle are defined in Channels indicated idle by the channel-list parameter. - Channels indicated idle by the channel-list parameter(11ac). PHY-CCA.indication (MDR)primitive channel-list element; Idle channels. primary; None. secondary; Primary 20 MHz channel. secondary40; Primary 20 MHz channel and secondary 20 MHz channel. secondary80; Primary 20 MHz channel, secondary 20 MHz channel, and secondary 40 MHz channel. When a STA and the BSS, of which the STA is a member, both support multiple channel widths, an EDCA TXOP is obtained based solely on activity of the primary channel. “Idle medium” in this subclause means “idle primary channel.” Likewise “busy medium” means “busy primary channel.” Once an EDCA TXOP has been obtained according to this subclause, further constraints defined in 10.16.9 (STA CCA sensing in a 20/40 MHz BSS) and Error: Reference source not found might limit the width of transmission during the TXOP or deny the channel access, based on the state of CCA on secondary channel, secondary 40 MHz channel, or secondary 80 MHz channel.(11ac) In the following description, the CCA is sampled according to the timing relationships defined in Error: Reference source not found. Slot boundaries are determined solely by activity on the primary channel. “Channel idle for an interval of PIFS” means that the STATE parameter of the most recent PHY-CCA.indication primitive was IDLE, and no PHY-CCA.indication(BUSY) occurred during the period of PIFS that ends at the start of transmission(#3510), the CCA for that channel was determined to be idle. If a STA is permitted to begin a TXOP (as defined in Obtaining an EDCA TXOP) and the STA has at least one MSDU pending for transmission for the AC of the permitted TXOP, the STA shall perform exactly one of the following actions:(#2458) - Transmit a 160 MHz or 80+80 MHz mask PPDU if the secondary channel, the secondary 40 MHz channel, and the secondary 80 MHz channel were idle during an interval of PIFS immediately preceding the start of the TXOP. - Transmit an 80 MHz mask PPDU on the primary 80 MHz channel if both the secondary channel and the secondary 40 MHz channel were idle during an interval of PIFS immediately preceding the start of the TXOP. - Transmit a 40 MHz mask PPDU on the primary 40 MHz channel if the secondary channel was idle during an interval of PIFS immediately preceding the start of the TXOP. - Transmit a 20 MHz mask PPDU on the primary 20 MHz channel. - Restart the channel access attempt by invoking the backoff procedure as specified in HCF as though the medium is busy on the primary channel as indicated by either physical or virtual CS, the TXNAV timer value has a value of 0, and the backoff timer has a value of 0. - Transmit a TVHT_4W or TVHT_2W+2W mask PPDU if the secondary TVHT_W channel and the secondary TVHT_2W channel were idle during an interval of PIFS immediately preceding the start of the TXOP.(11af) - Transmit a TVHT_2W or TVHT_W+W mask PPDU if the secondary TVHT_W channel was idle during an interval of PIFS immediately preceding the start of the TXOP.(11af) - Transmit a TVHT_W mask PPDU on the primary TVHT_W channel.(11af) NOTE 1—In the case of rule e), the STA selects a new random number using the current value of CW[AC], and the retry counters are not updated (as described in Multiple frame transmission in an EDCA TXOP; backoff procedure invoked for event a)). NOTE 2—For both an HT and a VHT STA, an EDCA TXOP is obtained based on activity on the primary channel (see Obtaining an EDCA TXOP). The width of transmission is determined by the CCA status of the (MDR)nonprimary channels during the PIFS interval before transmission (see VHT description in Error: Reference source not found).(#2458) - Multiple frame transmission in an EDCA TXOP A frame exchange may be one of the following:(11ac) - A frame not requiring immediate acknowledgment (such as a group addressed frame or a frame transmitted with an acknowledgement policy that does not require immediate acknowledgement) or an A-MPDU containing only such frames - A frame requiring acknowledgment (such as an individually addressed frame transmitted with an acknowledgement policy that requires immediate acknowledgement) or an A-MPDU containing at least one such frame, followed after SIFS by a corresponding acknowledgment frame - Either - a VHT NDP Announcement frame followed after SIFS by a VHT NDP, or - a Beamforming Report Poll frame followed after SIFS by a PPDU containing one or more VHT Compressed Beamforming frames The TXNAV timer is a single timer, shared by the EDCAFs within a STA, that is initialized with the duration from the Duration/ID field in the frame most recently successfully transmitted by the TXOP holder, except for PS-Poll frames.(#2458) The TXNAV timer begins counting down from the end of the transmission of the PPDU containing that frame. An(#2458) HT STA may retransmit unacknowledged MPDUs within the same TXOP or in a subsequent TXOP. Multiple frames may be transmitted in an EDCA TXOP that was acquired following the rules in Obtaining an EDCA TXOP if there is more than one frame pending in the primary(11ac) AC for which the channel has been acquired. However, those frames that are pending in other ACs shall not be transmitted in this EDCA TXOP except when sent in a VHT MU PPDU with TXVECTOR parameter NUM_USERS > 1 and if allowed by the rules in Error: Reference source not found.(11ac) If a TXOP holder has in its transmit queue an additional frame of the primary(11ac) AC and the duration of transmission of that frame plus any expected acknowledgment for that frame is less than the remaining TXNAV timer value, then the TXOP holder(11ac) may commence transmission of that frame a SIFS (or RIFS, if the conditions defined in Error: Reference source not found are met(11ac)) after the completion of the immediately preceding frame exchange sequence, subject to the TXOP limit restriction as described in Error: Reference source not found(#3382). A STA shall not commence the transmission of an RTS with a bandwidth signaling TA until at least PIFS time after the immediately preceding frame exchange sequence.(11ac) An HT STA that is a TXOP holder may transmit multiple MPDUs of the same AC within an A-MPDU as long as the duration of transmission of the A-MPDU plus any expected (#192)BlockAck frame response is less than the remaining TXNAV timer value. NOTE 1—PIFS is used by a VHT STA to perform CCA in the secondary 20 MHz, 40 MHz, and 80 MHz channels before receiving RTS (see Error: Reference source not found).(11ac) NOTE 2—An RD responder can transmit multiple MPDUs as described in Error: Reference source not found.(#241) After a valid response to the initial frame of a TXOP, if the Duration/ID field is set for multiple frame transmission and there is a subsequent transmission failure, the corresponding channel access function may transmit after the CS mechanism (see Error: Reference source not found) indicates that the medium is idle at the TxPIFS slot boundary (defined in Error: Reference source not found) provided that the duration of that transmission(Ed) plus the duration of any expected acknowledgment and applicable IFS is less than the remaining TXNAV timer value.(#251) At the expiry of the TXNAV timer, if the channel access function has not regained access to the medium, then the EDCAF shall invoke the backoff procedure that is described in Error: Reference source not found. Transmission failure is defined in Error: Reference source not found. Note that, as for an EDCA TXOP, a multiple frame transmission is granted to an EDCAF, not to a STA, so that the multiple frame transmission is permitted only for the transmission of a frame of the same AC as the frame that was granted the EDCA TXOP, unless the EDCA TXOP obtained is used by an AP for a PSMP sequence or a VHT MU PPDU with TXVECTOR parameter NUM_USERS > 1.(11ac) In the case of PSMP,(11ac) this AC transmission restriction does not apply to either the AP or the STAs participating in the PSMP sequence, but the specific restrictions on transmission during a PSMP sequence described in Error: Reference source not found do apply.(#2458) If a TXOP is protected by an RTS or CTS frame carried in a non-HT or a non-HT duplicate PPDU, the TXOP holder shall set the TXVECTOR parameter CH_BANDWIDTH of a PPDU as follows:(11ac) - To be the same or narrower than RXVECTOR parameter CH_BANDWIDTH_IN_NON_HT of the last received CTS frame in the same TXOP, if the RTS frame with a bandwidth signaling TA and TXVECTOR parameter DYN_BANDWIDTH_IN_NON_HT set to Dynamic has been sent by the TXOP holder in the last RTS/CTS exchange. - Otherwise, to be the same or narrower than the TXVECTOR parameter CH_BANDWIDTH of the RTS frame that has been sent by the TXOP holder in the last RTS/CTS exchange(Ed) in the same TXOP. If there is no RTS/CTS exchange in non-HT duplicate format in a TXOP and there is at least one non-HT duplicate frame exchange in a TXOP, the TXOP holder shall set the CH_BANDWIDTH parameter in TXVECTOR of a PPDU sent after the first non-HT duplicate frame to be the same or narrower than the CH_BANDWIDTH parameter in TXVECTOR of the initial frame in the first non-HT duplicate frame exchange in the same TXOP.(11ac) If there is no non-HT duplicate frame exchange in a TXOP, the TXOP holder shall set the TXVECTOR parameter CH_BANDWIDTH of a non-initial PPDU to be the same or narrower than the TXVECTOR parameter CH_BANDWIDTH of the preceding PPDU that it has transmitted in the same TXOP.(11ac) If a TXOP is protected by a CTS-to-self frame carried in a non-HT or non-HT duplicate PPDU, the TXOP holder shall set the TXVECTOR parameter CH_BANDWIDTH of a PPDU to be the same or narrower than the TXVECTOR parameter CH_BANDWIDTH of the CTS-to-self (#3236)frame in the same TXOP.(11ac) NOTE—The bandwidth of a PS-Poll frame does not constrain the bandwidth of an immediate data response to that PS-Poll frame.(11ac) Note that(#1288) when transmitting multiple frames in a TXOP using acknowledgment mechanisms other than Normal Ack, a protective mechanism should be used (such as RTS/CTS or the protection mechanism described in Error: Reference source not found). A QoS AP or a mesh STA may send group addressed frames without using any protection mechanism. In a QoS IBSS, group addressed frames shall be sent one at a time, and backoff shall be performed after the transmission of each of the group addressed frames. In an MBSS, a mesh STA may send multiple group addressed frames in a TXOP, bounded by the TXOP limit, without performing backoff after the TXOP is obtained.

# Agreed Comment Resolutions

| 6410 |  |  |  | Use of >> on signed quantities is implementation-defined in C | Only use unsigned values with >> (cf. e.g. p. 3511) |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6717 |  |  |  | Need to make sure >> is only used for logical shift, not arithmetic shift. This is an issue for pseudo-C-ode with the shifted amount being a signed variable. E.g. "int data[4] ... data[1] >> 31" is very suspect... | As it says in the comment |  | GEN |

Discussion:

I did review use of “>>” in the standard, and did not see any that appeared to depend a difference between signed and unsigned integers.  Mark also did a review, and suggested the only items that need attention are as shown in this resolution, adding as explanation:

While right-shifting of a signed quantity is implementation-defined in C, only two's complement implementations are extant, and for such implementations a shift followed by a suitable bitwise AND or a cast is probably sufficient to resolve any possible ambiguity (i.e. whether the shift is arithmetic or logical). The only issue is that in some implementations an 8-bit type might not exist, so a suitable cast does not exist.

Proposed resolution:

Revised.

At 3536.40 replace “(UINT8) (bssidIndex >> 3)”  with “(bssidIndex >> 3) &  0xff”

At 3538.08 replace “(UINT8) (aid >> 3)” with “(aid >> 3) & 0xff”

At 3536.41 and 3538.09 delete “(UINT8)” and remove the outermost parentheses;

| 6565 |  |  |  | The relationship between PHYs is not clear (e.g. are 11 Mbps PPDUs ERP PPDUs?) | Clarify the relationship between PHYs, and whether the PPDUs sent by a PHY which another PHY depends on are considered PPDUs of that other PHY |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment has not indicated  a specific issue to address,  just hints that there might be one.  I believe the question is moot on non-legacy bands. HT and VHT define the VHT PPDU in terms of *VECTOR parameters, so this should be unambiguous.

There is no such definition for ERP PPDU.  It would seem reasonable by extension that a PPDU generated by an ERP that uses a “pre ERP” waveform is a non-ERP PPDU.  This is consistent with: 1382.41:

When these mechanisms are used, non-ERP STAs do not interfere with frame exchanges using ERP PPDUs between ERP STAs

So my general understanding is that an <x> PPDU means “PPDU with format <x>”,  not “PPDU generated by an <x> PHY”.

Having done this work, the question remains:  “what is broken”?  If we know what is broken we can have an attempt at fixing it.

Mark Rison’s research is shown below:

Regarding CID 6565, I have been unable to find the "ERP PPDU" which I had in mind. So I can only point at the following, which could do with definition of the terms ("X PPDU"): 1382.42: non-ERP STAs do not interfere with frame exchanges using ERP PPDUs between ERP STAs [the one you found; probably doesn't matter too much as basically just introductory] 2227.9: CCA Mode 5: A combination of CS and energy above threshold. CCA shall report busy at least while a high rate PPDU with energy above the ED threshold is being received at the antenna. [Does CCA Mode 5 detect DSSS PPDUs with energy above the threshold? I suspect the answer is yes] 2468.21: "Clause 20 PPDU extended by 22.2.3" [includes ERP and (HR/)DSSS PPDUs? That's what the definition of HT PPDU suggests] Note that your claim in 15/1010r2 that "HT doesn't define an HT PPDU" isn't true: 31.58: high throughput (HT) physical layer (PHY) protocol data unit (PPDU): A Clause 20 (High Throughput (HT) PHY specification) PPDU with the TXVECTOR FORMAT parameter equal to HT_MF or HT_GF.

There was an email thread amongst various PHY experts.  The consensus appears to be that in 2227.09 “high rate PPDU” is intended to include a DSSS PPDU.

Brian also commented:

Then … analogous to what we did in 11n and 11ac, we need - To require that a HR PHY must also support the mandatory DSSS requirements To define a single interface in clause 17 that is used by the MAC, then describe how the clause 17 PHY “calls” the clause 16 PHY capabilities, in terms of a) interface mappings, b) TX and RX procedures and c) maybe some other things???

I agree that this work would improve the clarity of specification. Unless somebody volunteers to do the work in this round,  I propose a conservative resolution of this comment.

Status: R8.  This may interfere with work that Mark Rison is doing.  If he wants to pull this resolution,  I will do so without complaint.

Proposed Resolution:

Revised.  At 2227.09 change “a high rate PPDU” to “an HR/DSSS PPDU”

(Note to editors, this resolution is a subset of the changes proposed for CID 6302)

| 6433 |  |  |  | The terms "transmit power" and "transmit power level" are both used but not defined. Is there a difference? | Change "transmit power level" to "transmit power" except where it specifically refers to an index rather than a power (e.g. 2277.57) |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Proposed Resolution:

Revised.  A 2277.57, change “The transmit power level.” To “The transmit power level index.”

Change all “TXPWR_LEVEL” to “TXPWR_LEVEL_INDEX”.

In reply to the commenter, transmit power and transmit power level are conventionally used as synonyms, and we are chooseing not to enforce a different interpretation in the standard.

| 6560 |  |  |  | The term "MMPDU" causes no end of confusion as people read it as "Management MPDU" (i.e. a type of MPDU) rather than "MAC Management PDU" (i.e. a type of PDU) | Change "MMPDU" to "MLMEPDU" throughout, defining the term "MLMEPDU" in 3.2 as follows: "MAC layer management entity (MLME) medium access control (MAC) protocol data unit (MPDU): Information that is delivered as a unit between MAC layer management entities." |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There was sentiment to change the name in a previous comment cycle:

EDITOR: 2012-10-12 15:14:32Z -

Straw poll:  A - change name of MMPDU 1111 (4)

B - reject the comment,  don't change name. 11 (2)

However, we never did get round to changing it.

There are two difficulties with this change in my mind:

- There are about 447 places to touch.   We’ll miss about 10 of them, which probably means this many additional comments in the next ballot cycle.

- MLMEPDU hardly trips off the tongue.

Let’s redo the straw poll.

Do you prefer:

- To leave MMPDU as is 1111

- To change it to MLMEPDU 0

Proposed Resolution:

Rejected. The MMPDU term is defined unambiguously in the standard.  It is not clear that changing it as proposed would lessen confusion.

| 6756 |  |  |  | "The RCPI field contains an RCPI value as specified for certain PHYs in Clause ..." -- the list is incomplete. And how is the RCPI element to be used if the PHY does not specify how the RCPI is determined? | Make the RCPI value mapping the same for all PHYs, then the problem goes away |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 6610 |  |  | The mapping from the RCPI to the RCPI value in MMPDUs is defined identically in all the PHYs | Put it all in one common place | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Siaxuaaion:

The existing definitions are identical, and in fact we put a lot of effort removing the gratuitous variability in these definitions.  It makes sense to have a single definition where this is appropriate.

The following changes unify the definitions:

Proposed Resolution: (to both)

Revised. Make changes under CID 6756 in <this-document>.  These changes move the repetitive definition of the encoding of this field into Clause 8.

Proposed Changes:

At 859.18 change as follows.  Text copied from 16.4.6.6 is shown with this tracking.  Changes to that copied text are shown with this tracking.:

- RCPI element The RCPI element indicates the received frame power level at the receiving STA as shown in RCPI element format. Element ID; Length; RCPI. Octets:; 1; 1; 1. - RCPI element format. The Element ID and Length fields are defined in Error: Reference source not found.(#139) The RCPI field contains an RCPI value which(#1607) is an indication of the received RF power in the selected channel for a received frame. RCPI is a monotonically increasing, logarithmic function of the received power level. The allowed values for the RCPI field are defined in Table 8-X -- RCPI values, where P is the received power level in dBm. - Table 8-X -- RCPI values(#2177). RCPI Value; Description. 0; Represents P < –109.5 dBm. 1–219; Power levels in the range are represented by. 220; Represents dBm. 221–254; Reserved. 255; Measurement not available. (#2177)

At 2196.36:

- Received Channel Power Indicator Measurement The RCPI(#1607) is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY(#1023) of the received RF power in the channel measured over the entire received frame or by other equivalent means that meet the specified accuracy. The encoding of received power to RCPI is defined in 8.4.2.37. Table 8-X -- RCPI values (#2177) RCPI shall equal the received RF power within an accuracy of ±5 dB (95% confidence interval) within the specified dynamic range of the receiver. The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

At 151.40, 158.03, 173.39, 174.03, 177.29, 180.43, 187.41, 188.03, 191.08, 194.42, 269.24, 269.37, : (Valid Range Column)

As defined in 18.4.2.37.

At 778.60:

Average RCPI is a logarithmic function of the received signal power, as defined in 8.4.2.37.

At 779.02:

Last RCPI is a logarithmic function of the received signal power, as defined 8.4.2.37.

At 812.07:

RCPI is a logarithmic indication of the received channel power of the corresponding Link Measurement Request frame, as defined in 8.4.2.37.

At 910.31 and 2944.40, 3078.18:

The Source RCPI is a logarithmic function of the received signal power, as defined 8.4.2.37.

At 910.46 and 2944.65, 3078.48:

Target RCPI is a logarithmic function of the received signal power, as defined in 8.4.2.37.

At 1109.32:

RCPI indicates the received channel power of the corresponding Link Measurement Request frame, which is a logarithmic function of the received signal power, as defined in 8.4.2.37 (RCPI element)

At 1544.04:

If dot11RadioMeasurementActivatedis true and the RCPI element was requested, an RCPI element containing the RCPI of the Probe Request frame shall be included. If no measurement result is available, the RCPI value shall beset to indicate that a measurement is not available (see 8.4.2.37 (RCPI element)).

At 16.2.3.6 and make matching changes at 2233.01:

16.2.3.6 RXVECTOR RCPI The allowed values for the RCPI parameter are in the range from 0 to 255, as defined in 8.4.2.37. This parameter is a measure by the PHY of the received channel power. The performance requirements for the measurement of RCPI are defined in 16.4.6.6.

At 2227.42 and make matching changes at 2265.08, 2369.30, 2410.24:

17.3.8.6 Received Channel Power Indicator Measurement The RCPI indicator is a measure of the received RF power in the selected channel for a received frame. This parameter shall be a measure by the PHY of the received RF power in the channel measured over the entire received frame or by other equivalent means that meet the specified accuracy. The RCPI encoding is defined in 8.4.2.37. RCPI shall equal the received RF power within an accuracy of ±5 dB (95% confidence interval) within the specified dynamic range of the receiver. The received RF power shall be determined assuming a receiver noise equivalent bandwidth equal to the channel bandwidth multiplied by 1.1.

At 2279.07:  no change proposed

The 8-bit RCPI value is described in 18.2.3.6 (RXVECTOR RCPI) and 17.3.8.6 (Received Channel Power Indicator Measurement).

At 2983.39, 2988.24, 2988.58, :

as defined 8.4.2.37.

| 6789 |  |  |  | Inconsistency about presence/absence of space after/before colon in dated ISO references (i.e. ISO 1234:1999). Shouldn't the colon be a hyphen anyway, for consistency with IEEE? | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:R8

Revised.

At 111:10 change “1: 1994” to “1:1994”

Globally change “2: 1998” to “2:1998”

Globally change “ISO-14962-1997” to “ISO 14962:1997”

Globally change “ISO-639” to “ISO 639”

Globally change “ISO -1” to “ISO 3166-1”

Globally change “ISO -2” to “ISO 3166-2”

Globally delete “[ISO.-2]”

Globally change “ISO/IEC -1” to “ISO 3166-1”

Globally Change “14977 : 1996” to “14977:1996”

Globally Change “7498-1: 1994” to “7498-1:1994”

At 5.62 add: “ISO/IEC 3166-2, Codes for the representation of names of countries and their subdivisions— Part 2: Country subdivision code.”

| 6787 |  |  |  | What is X in the penultimate column of Table 17-2 Example of LENGTH calculations for CCK? | Clarify |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Mark R writes:

Proposed Resolution:

Revised.  At 2203.34,  replace “X” by “LENGTH x 11/8.  Delete the parentheses in ‘(x 8/11) and (x 11/8)’ on the same line.”

| 5048 |  |  |  | There are MIB variables left that relate to the removed FH PHY. | Remove (or deprecate) all the *ot11Hopping* objects. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  These changes deprecate the “Hopping” MIB variables.

For dot11HoppingPatternTable itself and for each of the OBJECT-TYPE definitions under dot11HoppingPatternTable:

- Change STATUS to deprecated

- At the start of the DESCRIPTION insert the following new para (within the quotes): “This attribute is deprecated because the frequency hopping PHY is no longer present in IEEE Std 802.11-<year>.”

| 5059 |  |  |  | There are a number of placeholder clauses. | The group needs to determine whether to keep the placeholders, in an attempt to preserve the current clause numbering, or to delete them. Any changes should be done asap. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

I know that Mark H wants to move a clause into the architecture session.  This will displace clause numbers.

We have the following options:

- No changes to clause numbers.  Artificially constrain Mark’s moved material to a second level heading.  Leave placeholders for all removed clause numbers.

- Limited changes to clause numbers. Allow Mark’s moved material.   This will renumber all the MAC clauses.   Leave all but one PHY placeholder in place,  which leaves the PHY clauses unchanged.

- Allow all changes.   This means that all MAC and PHY clauses get renumbered,  and all placeholders removed.

Proposed Resolution:

Revised. Delete the placeholder clauses:  14 & 15;  Annexes: I, J and K.

Review the following figures and manually update references as necessary: Figures 20-21, 20-27, 22-1, 22-2, 22-3, and 22-37.

| 6737 |  |  |  | Shouldn't have years for docs which are fully identified otherwise (e.g. FIPS PUB 180-3). | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The FIPS PUB references with years are:

180-3-2008

197-2001

180-3-2003 was superseded by 180-4 in 2012.

The original reference to 180-3 was to define SHA-1.  As 180-4 supersedes 180-3,  and contains the definition of SHA-1,  the reference can be updated.

197-2001 is still current.

The IEEE-SA style guide states:

17.2 Citing standards in a bibliography Standards listed shall include designation and title. They can be either dated or undated, whichever is appropriate to a particular entry. Example: [B1] ASME BPVC-I-2004, Boiler and Pressure Vessel Code, Section 1—Power Boilers. [B2] Code of Federal Regulations Title 29 Part 1210 Section 354 (29CFR1210.354), Health and Safety Standards—Head injury. [B3] ISO/IEC 7498-4, Information processing systems—Open Systems Interconnection—Basic Reference Model—Part 4: Management framework.

The ISO “how to write standards” states:

Remember to date your references if reference is made to a specific clause, subclause, figure, table etc., in another document.

The ISO directives are summarized as:

Normative references may be undated or dated. They are dated if a specific element in the referenced publication (clause, subclause, figure, title, annex etc.) is cited in the text. In the case of an enquiry or final draft, the date of publication shall be replaced by a dash together with a footnote "To be published", and a full title (see ISO/IEC Directives Part 2:2011,6.6.7.5). If no specific element is cited, the reference can be left undated, signifying that the latest edition of the publication and any amendments apply (see ISO/IEC Directives Part 2:2011, 6.6.7.5.2 and 6.6.7.5.3).

It is therefore our choice to leave or remove,  as either are permitted.  We have previously removed such year numbers,  so to be consistent we should do so here.

Proposed Resolution:

Revised.  Globally replace “FIPS PUB 180-3-2008” with “FIPS PUB 180-4” (2 instances).

Globally replace “FIPS PUB 197-2001” with “FIPS PUB 197” (3 instances).

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6400 | 685.22 | 8.4.1.40 |  | Calling a field in Auth frames "Element" is incredibly confusing | Rename to "Finite Field Element" at least |  | GEN |

Discussion:

This is marked “discuss” in the database with notes:

GEN: 2015-09-17 07:14:17Z -

REJECTED (GEN: 2015-09-17 07:13:06Z) The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

GEN: 2015-09-17 07:12:52Z - Proposed resolution: Revise - change field name "Element Field". Change  to "Finite Field Element" need to have list of locations for the change

I propose we should avoid using the word “element” in refering to this field.

A neutral way to handle it is to call it the “FFE” field,  with an acronym to explain.

Proposed Resolution:

Revised.

At 53.24 insert an abbreviation: “FFE	finite field element”

At 685.22 replace “Element” by “Finite field element (FFE)”.

At 685.25, 685.29, 685.34, 1887.45, and 1887.51,  replace “Element” by “FFE”.

| 6405 | 1048.35 | 8.4.2.162 |  | Where is the use/non-use of Transmit Power Envelope subelements in a Channel Switch Wrapper specified? | If there are no rules/restrictions, then there should be at least a NOTE to that effect |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.

At 1049.26 insert new para:  “The Transmit Power Envelope subelement is present as described in 10.40.4.”

## GEN “Definitions” comment group

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6274 | 3.06 | 1.5 |  | There are no 2-param Floor examples | Add "For example, Floor (3.3, 2) is 2 and Floor (-3.3, 2) is -4." (minuses, not hyphens, natch) to the end of the first para of the referenced subclause |  | GEN |

Discussion:  We have a similar example for Ceil(),  so might as well have one for Floor.

Proposed Resolution:

Accepted

| 6088 | 10.61 | 3.1 |  | A given DS (and therefore a given ESS) can only have one (logical) portal. Otherwise, the DS does not know how to route outbound traffic destined for more than one non-802.11 network, the DS could create a "bridge" between the non-802.11 LANs, independent of the WLAN, and this can result in loops, etc. | Change "portals" to "the portal" at 10.61, 11.6, 14.2, 3551.16, 3551.47, 3551.48, 3553.1 Change "portals" to "a portal" at 11.8, 110.50 Change "access point (AP), and portal entities" to "an optional portal, and the access point (AP) entities" at 13.2, and 21.22. Change "zero or more portals" to "zero or one portal" at 13.3, 21.25 and 3545.36. Add a note after the definiton of portal (P17.18), "NOTE--For the purposes of this Standard, there is at most one portal in a given ESS's infrastructure. In an implementation, a single logical portal function may be provided by multiple devices that provide integration services for the ESS. How such multiple devices coordinate to appear as a single logical portal is implementation dependent." Change "The WLAN system includes the DS, AP, the AP's STA, and portal entities." to "The WLAN system includes the DS, an optional portal, APs, and the APs' STAs." at 3545.33. Change "zero or more [0..*] portals" to "zero or one [0..1] portal." at 3545.58. Also change "Each portal" to "The portal" at the same location. Change "portal(s)" to "portal" at 3550.15. Remove the second (dotted line) Portal box from Figure R-1. Change "APs, mesh gates, and portals, except the origninating portal." to "APs and mesh gates." at 3553.2. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Changes:

Change "portals" to "the portal" at 10.61, 11.6, 14.2, 3551.16, 3551.47, 3551.48, 3553.1
Change "portals" to "a portal" at 11.8, 110.50
Change "access point (AP), and portal entities" to "an optional portal, and the access point (AP) entities" at 13.2, and 21.22.
Change "zero or more portals" to "zero or one portal" at 13.3, 21.25 and 3545.36.
Add a note after the definiton of portal (P17.18), "NOTE--For the purposes of this Standard, there is at most one portal in a given ESS's infrastructure. In an implementation, a single logical portal function may be provided by multiple devices that provide integration services for the ESS. How such multiple devices coordinate to appear as a single logical portal is implementation dependent."
Change "The WLAN system includes the DS, AP, the AP's STA, and portal entities." to "The WLAN system includes the DS, an optional portal, APs, and the APs' STAs." at 3545.33.
Change "zero or more [0..*] portals" to "zero or one [0..1] portal." at 3545.58. Also change "Each portal" to "The portal" at the same location.
Change "portal(s)" to "portal" at 3550.15.
Remove the second (dotted line) Portal box from Figure R-1.
Change "APs, mesh gates, and portals, except the origninating portal." to "APs and mesh gates." at 3553.2.

Proposed Resolution:

Accepted

| 6669 | 21.00 | 3.2 |  | Definition of "40 MHz physical layer (PHY) protocol data unit (PPDU):" can merge the first and last (HT and VHT). | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  yes, we could merge, but what is the benefit?

Location 23.31

Proposed Resolution:

Rejected. The cited occurances are not incorrect as they stand.  Each reflects a distinct type of 40 MHz PPDU.

| 6668 | 21.00 | 3.2 |  | The second half in "directional multi-gigabit (DMG) physical layer (PHY) protocol data unit (PPDU): A Clause 21 (Directional multi-gigabit (DMG) PHY specification) PPDU transmitted or received using the Clause 21 (Directional multi-gigabit (DMG) PHY specification) physical layer (PHY)." is obvious and should be deleted. | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

It is somewhat repetitive.

Proposed resolution:

Revised.  At 27.30 delete “transmitted or received using the Clause 21 (Directional multi-gigabit (DMG) PHY specification) physical layer (PHY)”.

| 6667 | 21.00 | 3.2 |  | Add VHT to the definition of "non-high throughput (non-HT) physical layer (PHY) protocol data unit (PPDU)". | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

At 36.05:

non-high throughput (non-HT) physical layer (PHY) protocol data unit (PPDU): A Clause 20 (High Throughput (HT) PHY specification) or Clause 22 (…) physical layer (PHY) PPDU with the TXVECTOR FORMAT parameter equal to NON_HT.

Discussion:  this is a partial fix.  Need to understand whether:

- 802.11g/a STA

- TVT STA

- DMG STA

Should also be included in this definition.

Proposed Resolution:

Revised.

Replace definition at 36.04 with

non-high throughput (non-HT) physical layer (PHY) protocol data unit (PPDU): A PPDU that is transmitted not using a TXVECTOR FORMAT parameter equal to HT_MF, HT_GF or VHT.

| 6819 | 21.00 | 3.2 |  | "individually addressed bufferable unit (BU): An individually addressed MSDU, individually addressed A-MSDU (HT STAs only) or individually addressed bufferable MMPDU." needs to say "DMG STAs" as for bufferable unit (BU). | As it says in the comment |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Note, this change has been made for another comment.  Not sure which one,  so proposing wording that is non-conflicting.

Proposed Resolution: R8

Non proposed here. This is addressed in 11-15/762.

| 6531 | 21.21 | 3.1 |  | The definition of "wireless local area network (WLAN) system" seems to be specific to infrastructure BSSes, and essentially a duplicate of the definition of "infrastructure". It is not clear why an IBSS etc. is not a WLAN system | Delete the definition |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Agree the definitions are essentially duplicates.  The WLAN system term is used only in annex P.

We could replace all references to “WLAN system”,  but this creates two awkwardnesses:

- “An infrastructure” as a noun is awkward,  because elsewhere it is used as an adjective.

- Many figures in annex P (for which we do not have sources) would need to be edited.  The editors would have to redraw the figures,  which is a bit of a grind.

Proposed Resolution:

Revised.

Delete definition of WLAN system at 21.21.

At 3541.14, 3541.15, 3541.21 3541.22 change “WLAN system” to “ESS’s infrastructure”

At 3543.20 delete “system”

At 3544.22 insert:

“Definition: in this annex a wireless local area network (WLAN) system is a system that includes the distribution system (DS), access points (APs), and portal entities. It is also the logical location of distribution and integration service functions of an extended service set (ESS). A WLAN system contains an optional portal and one or more APs in addition to the DS.”

| 6396 | 36.04 | 3.2 |  | "non-high throughput (non-HT) physical layer (PHY) protocol data unit (PPDU)" definition does not include PPDUs sent by PHYs other than the HT PHY. Does this work for all instances of "non-HT PPDU" in the spec? | Make definition cover PPDUs sent by pre-HT PHYs and by the VHT PHY |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Asked Assaf.  He says:

I have attached a proposed resolution. The resolution covers all usages of the terms non-HT PPDU except one in clause 9.7.6.5.3. I think the whole clause assumes that the PHY connected to the MAC layer is a VHT or HT PHY. It therefore assumes that a clause 16-19 PHY that elicits a (control) response is a non-HT PPDU. This is probably not correct if both the transmitter and the receiver are non-HT PHYs – i.e. they do not advertise any HT PHY capabilities. I think that in this case the problem is in clause 9.7.6. Either it should be specified that it refers only to VHT and HT PHYs or add subclauses for rate selection of clause 16-29 PPDUs. This is beyond by expertise, but I can give it a try. I am not sure it should be within the resolution of this comment.

I reviewed 9.7.6.5.3 and didn’t see any particular issue.

See CID 6667.

Discussion:

The non-HT PPDU is used throughout the spec to refer to the transmission of clause 16, clause 17, clause 18 and clause 19 PHYs PPDUs, when transmitted by a clause 20 (HT), clause 22, (VHT) or clause 23 (TVWS) PHYs.  Therefore, the definition should be changed to include clause 22 and clause 23.

Proposed Resolution:

Revised.

Replace definition at 36.04 with

non-high throughput (non-HT) physical layer (PHY) protocol data unit (PPDU): A PPDU that is transmitted not using a TXVECTOR FORMAT parameter equal to HT_MF, HT_GF or VHT.

| 6297 | 36.42 | 3.2 |  | It says "nontransmitted basic service set (BSS) identifier (BSSID): A BSSID that is not transmitted explicitly, but that can be derived from the information encoded in a Beacon frame." but nontransmitted BSSIDs can also be derived from information in other frames | Delete "Beacon" in the cited text |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  The “multiple BSSID” element provides this information,  and is present in at least the Probe Response frame.  So the commenter’s assertion is correct.

But the change would leave a misleading statement,  implying that the information can be determined from any frame,  which is clearly wrong.

Proposed change:

Revised. At cited location replace “a Beacon frame” with “Probe Response, Beacon and DMG Beacon frames and Neighbor reports”

| 6051 | 41.12 | 3.2 |  | The term RSNA-capable is only ever used to refer to a STA (or AP, or peer - which is equivalent). We don't need a definition that covers "equipment" (whatever that means). | Change the definition to (deleting "equipment"), "robust-security-network-association- (RSNA-) capable station (STA): A STA that is able to create RSNAs." |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.

Changes cited definition to read:

robust-security-network-association- (RSNA-) capable: Pertains to a station (STA) that is able to create RSNAs.

| 6052 | 41.15 | 3.2 |  | The term RSNA-enabled is only ever used to refer to a STA (or AP, or peer - which is equivalent). We don't need a definition that covers "equipment" (whatever that means). | Change the definition to (deleting "equipment"), "robust-security-network-association- (RSNA-) enabled station (STA): A STA when it is RSNAcapable and dot11RSNAActivated is true." |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

If there is a term that is not used, it should be deleted.  I can find no reference to the term.

Proposed resolution:

Revised.  Delete cited definition.

| 5863 | 51.42 | 3.4 |  | The document does not correctly include terms of concern to the IEEE Registration Authority. Use of CID should be clear when either an OUI or CID is applicable, and with the number of places where this is appropriate, CID should be entered into the acronyms list. | Detailed requested changes will be provided through RAC mandatory coordination comments. |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

We can do nothing useful with this comment.   The individual cases are cited in other comments and changes have been made to resolve them.  This is arguably a “reject”,  but as we did make changes,  even though they are not detailed here, a “revised” is just about tolerable.

Proposed Resolution.

Revised.  We thank the RAC for their coordination comments.  A total of 29 other more detailed comments were received from the RAC.  These comments have all been accepted or revised, except where changes are proposed to deprecated or obsolete text.

| 6044 | 988.58 | 8.4.2.114 |  | The calculation of the value of the Length field is only correct with a generous interpretation: The presence of the External Destination Address field is determined for each destination independently by the corresponding Flags field. So, the Length is actually 2+N1*13+N2*19 with N1+N2=N. | The easiest resolution is to delete the sentence "The Length field set to (2 + 13 x Number of Destinations) or to (2 + 19 x Number of Destinations) octets." |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited sentence is trying to be helpful.  Unfortunately,  as the comment indicates,  it is inaccurate.  Being unnecessary and inaccurate we can fix it or delete it.  I prefer the latter.

Proposed Resolution:

Revised.  At cited location, delete “The Length field set to (2 + 13 x Number of Destinations) or to (2 + 19 x Number of Destinations) octets.”

| 5025 | 1006.42 | 8.4.2.128 |  | "While associated with an AP or PCP, a STA overrides the value of dot11PSRequestSuspensionInterval with the value of this subfield when itreceives this element from its AP or PCP." This is not describing a frame format, so it doesn't belong in clause 8. | Move to clause 10 somewhere relevant. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1006.41

The MinBHIDuration subfield indicates the minimum duration of the BHI, which can include the BTI, ABFT, and ATI and is specified in microseconds. While associated with an AP or PCP, a STA overrides the value of dot11MinBHIDuration variable with the value of this subfieldwhen it receives this element from its AP or PCP.

Discussion:

This is an awkward bit of specification.  What exactly does “override” mean?  Does it mean the value of the mib variable is written at the time of association and reverts afterward?  Does a read of the MIB variable by an external entity during this time produce the original or new value.

As nobody really cares about the MIB,  and as no behaviour related to this variable appears to be specified for an unassociated STA,  I really don’t think these questions matter.

We can resolve the awkwardness by having a STA simple write to this MIB variable on association.

Status:  Carlos agrees with this resolution,  but highlights that the following fields are also at issue:

PSRequestSuspensionInterval MinBHIDuration BroadcastSTAInfoDuration MinPPDuration SPIdleTimeout MaxLostBeacons

In addition to the list above, AssocRespConfirmTime also has the same issue.

The proposed resolution addresses the issues in all these fields:

Proposed Resolution:

Revised.

At 1006.36 delete “While associated with an AP or PCP, a STA overrides the value of dot11PSRequestSuspensionInterval with the value of this subfield when itreceives this element from its AP or PCP.”

At 1006.42 delete “While associated with an AP or PCP, a STA overrides the value of dot11MinBHIDuration variable with the value of this subfield when it receives this element from its AP or PCP.”

At 1006.48 delete “While associated with an AP or PCP, a STA overrides the value of dot11BroadcastSTAInfoDuration with the value of this subfield when it receives this element from its AP or PCP.”

At 1006.54 delete “While associated with an AP or PCP, a STA overrides the value of dot11AssocRespConfirmTime with the value of this subfield when it receives this element from its AP or PCP.”

At 1006.60 delete “While associated with an AP or PCP, a STA overrides the value of dot11MinPPDuration with the value of this subfield when it receives this element from its AP or PCP.”

At 1007.03 delete “While associated with an AP or PCP, a STA overrides the value of its local dot11SPIdleTimeout variable with the value of this subfield when it receives this element from its AP or PCP.”

At 1007.07 delete: “While associated with an AP or PCP, a STA overrides the value of dot11MaxLostBeacons with the value of this subfield when it receives this element from its AP or PCP.”

At 1595.18 insert a new list item c) and subitems and increment the existing list items accordingly: “If an Association Response frame is received with a status code of SUCCESS, a DMG STA shall write to each of the following MIB attributes the value of the corresponding subfield of the DMG BSS Parameter Configuration field of the DMG Operation element received from the AP or PCP to which it requested association:

- dot11PSRequestSuspensionInterval from the PSRequestSuspensionInterval subfield

- dot11MinBHIDuration from the MinBHIDuration subfield

- dot11BroadcastSTAInfoDuration from the BroadcastSTAInfoDuration subfield

- dot11AssocRespConfirmTime from the AssocRespConfirmTime subfield

- dot11MinPPDuration from the MinPPDuration subfield

- dot11SPIdleTimeout from the SPIdleTimeout subfield

- dot11MaxLostBeacons from the MaxLostBeacons subfield”

Make the same change at 1599.11, (before list item d)) substituting “reassociation” for “association” (preserving case).

| 5026 | 1011.48 | 8.4.2.131 |  | "expected to communicate" - passive voice is considered dangerous. Who or what does the expecting? Ditto on the next line. Ditto at 1014.19 "wishes to communicate" | Reword to avoid the passive voice and remove any expectations by inanimate objects. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1011.48

The Destination AID subfield indicates the AID of a STA that is expected to communicate with the source DMG STA during the allocation or broadcast AID if all STAs are expected to communicate with the source DMG STA during the allocation. If the Source AID and the Destination AID subfields for an SP allocation contain the broadcast AID, then during the SP a non-AP and non-PCP STA does not transmit unless it receives a Poll or Grant frame from the AP or PCP.

1014.19:

The Destination AID subfield contains the AID of a STA that the requesting STA wishes to communicate with during the allocation.

Discussion:

Although it might seem editorial,  use of passive voice obscures who is making a decision or performing an action.  Use of anthropomorphic words such as “expects”, “desires”, “wishes” hides how decisions are made or communicated.

Proposed Changes:

At 1011.48:

The Destination AID subfield is set to the AID of the STA that the source STA targets during the allocation, or to the broadcast AID if the source STA targets more than one STA during the allocation. If both Source AID and Destination AID subfields for an SP allocation are set to the broadcast AID, then during the SP a non-AP and non-PCP STA does not transmit unless it receives a Poll or Grant frame from the AP or PCP.

1014.19:

The Destination AID subfield contains the AID of a STA that the requesting STA targets during the allocation.

Proposed Resolution:

Revised.  Make changes under CID 5026 in <this-document>.  These changes address the issues raised in the comment.

| 5027 | 1029.45 | 8.4.2.147 |  | "The Relay Capabilities element is not used in an IBSS" - this is not frame format, and doesn't below in Clause 8. | Remove cited sentence or move to clause 9 or 10. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1029.43

A STA that intends to participate in relay operation (10.36 (DMG relay procedures)) advertises its capabilities through the Relay Capabilities element. The Relay Capabilities element is defined in Figure 8-537 (Relay capabilities element format). The Relay Capabilities element is not used in an IBSS.

Discussion:

I think what it’s really saying is that DMG relay is not supported in an IBSS.

There’s a worms in this can.  At 155.48 we have:

The DMG Beacon frame doesn’t describe this element as being present.  This is presumably covered by “One or more elements

can appear in this frame.” At 644.38.   There is another comment on 644.38 that requests filling in the blank (i.e.,  doing the work that .11ad should have done originally).   So,  I suppose this is not actually an inconsistency and will get fixed.

However, the Probe Response frame,  does not carry this wildcard statement, and does not carry the Relay Capabilities element,  so this is an inconsistency that needs to get fixed here.

Mark R writes:

CID 5027: Missing the canonical "and is not present otherwise" [[Adrian]] This is for debate. If you look in Table 8-34, the “not present otherwise” is in the minority. This reflects some folks opinions that you shouldn’t proscribe what you didn’t need to. The original mistake was to get rid of the "if and only if"s. I should have spoken up more strongly then. However, now that we have done that, we have fallen back to saying that we should always spell it out, i.e. set to 1 if $blah, and to 0 otherwise.

Proposed Resolution:

Revised.

Deleted cited sentence.

Before the “Vendor Specific” row of table 8-34 (Probe Response) add a new row:

“68”, “Relay Capabilities”, “The Relay Capabilities element is present if dot11RelayActivated is true; otherwise not present.”

At 1521.61 (Relay operation, General) add a new paragraph: “DMG relay operation is not supported by an IBSS STA.  An IBSS STA shall set dot11RelayActivated to false.”

| 5029 | 1030.04 | 8.4.2.147 |  | "Relay Usability" -- this is a poor choice of field name. | Globally change to "Relay Client". |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This term is used in 3 locations.  The definition of the field is “The Relay Usability subfield indicates whether the STA is capable of using frame-relaying through a relay STA. It is set to 1 if the STA supports transmission and reception of frames through a relay STA; otherwise, it set to 0”.   This confirms its use in the relay client role.

Proposed Resolution:

Accepted

| 6414 | 917.29 | 8.4.2.68.5 |  | "The Operating Class field contains an enumerated value from Annex E specifying a channel width or frequency segment index and width (if the indicated channel comprises noncontiguous frequency segments)" -- how can an OC indicate a frequency segment index? And anyway, the OCs give the max width, not the operating width | Clarify |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

The Operating Class field contains an enumerated value from Annex E specifying a channel width or frequency segment index and width (if the indicated channel comprises noncontiguous frequency segments) as well as the frequency band in which the Channel Number is valid.

Proposed Change:

Revised.  Replace sentence at 917.29 “The Operating Class field contains an enumerated … as well as the frequency band in which the Channel Number is valid.”  With

“The Operating Class field contains an enumerated value from Annex E that identifies the mapping from Channel Number to frequency.”

| 6532 | 938.04 | 8.4.2.75 |  | "Request subelement" is too generic | Change to "FMS Request subelement" throughout |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.

At 938.13 delete “one or more Request Subelements”

At 938.17 change “Request sublements” to “FMS Request Subelements”

In 8.4.2.75 change “Request [s|S]ubelement” to “FMS Request [s|S]ubelement”, preserving the case of the “S”.

| 6533 | 939.41 | 8.4.2.76 |  | "Status subelement" is too generic | Change to "FMS Response subelement" throughout |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.

At 939.48 delete “one or more Status Subelements”

At 940.24 delete “The FMS Status Subelements field contains one or more FMS Status subelements.”

In 8.4.2.76 change “Status [s|S]ublement” to “FMS Response [s|S]ublement”, excluding “FMS Status subelement”, preserving the case of the “S”

| 6202 | 943.61 | 8.4.2.78 |  | The term "an RSN protected frame" is used, but the term is never defined | Change the cited text to "a protected frame" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context 943.61:

The Protected Keep-Alive Required subfield is set to 1 to indicate that only an RSN protected frame indicates activity.

1752.56:

A STA may send security protocol protected or unprotected keepalive frames, as indicated in the Idle Options field

1753.01:

If the Idle Options field requires security protocol protected keepalive frames, then the AP may disassociate the STA if no protected frames are received from the STA for a period indicated by the Max IdlePeriod field of the BSS Max Idle Period element.

Discussion:  The behaviour described is not limited to “RSN protected”,  so agree with the proposed change.

Proposed Resolution:

Accepted.

| 6098 | 964.28 | 8.4.2.92 |  | Error in the Reserved values range. | Change the Reserved table entry value range from '4-220' to '5-220' |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5061 | 964.29 | 8.4.2.92 |  | The reserved range "4-220" is incorrect. | Change to "5-220" |  | MAC |

Proposed Resolution:

Accepted

| 6254 | 861.09 | 8.4.2.39 |  | It says "In a Beacon report or , the Antenna ID always identifies the antenna used" -- or what? | Add something before the comma |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 861.01

The Antenna ID field contains the identifying numberfor the relevant antenna(s). When included in a Beacon, Probe Response, or Location Track Notification frame, the Antenna ID identifies the antenna(s) used to transmit the Beacon, ProbeResponse, or Location Track Notification frame. When included in a measurement report, orLocation Track Notification frame, the Antenna ID identifies the antenna(s) used for the reported measurement or transmission of the Location Track Notification frame. The valid range for the Antenna ID is 1 to 254. The value 0 indicates that the antenna identifier is unknown. The value 255 indicates that this measurement was made with multiple antennas, i.e., antennas were switched during the measurement duration. In a Beacon report or , the Antenna ID always identifies the antenna used to receive the reported beacon or frame. If during frame reception, different antennas are used to receive the preamble and body, the antenna ID identifies the antenna that receives the frame body. In these cases, the value 255 is not used. The value 1 is the only value used for a STA with only one antenna. STAs with more than one antenna assign Antenna IDs to each antenna and each antenna configuration as consecutive, positive integers starting with 1. Each Antenna ID number represents a unique antenna or unique configuration of multiple antennas characterized by a fixed relative position, a fixed relative direction, and a fixed peak gain for that position and direction.

Discussion:

Antenna Element Present in: Probe Response, Beacon,

Field present in Antenna element, Noise Histogram report, Beacon report, Frame report, Location Parameters / Radio subelement, Link Measurement report

The Beacon report can report a single frame, and it is then appropriate to report an Antenna ID.  I’m less sure of the value of reporting it in the Frame Report,  which reports counts of frame types.  Perhaps it was there before and has been removed.

I’d say a link measurement report would be far more interested in the antenna ID.

Anyhow,  in the interests of doing no harm and minimizing changes,  I propose the following.

.”.

Proposed Resolution:

Revised. Delete cited sentence.

| 5041 | 875.21 | 8.4.2.51 |  | "DSE Registered Location Element Body field" -- having both element and body in the name of a field causes confusion. | Replace "Element Body" with "Information" in this context globally. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  Globally change “DSE Registered Location Element Body” to “DSE Registered Location Information”

| 5071 | 880.28 | 8.4.2.55.2 |  | "Reserved and shall be set to 0 or 3 otherwise" -- no, reserved is defined to be "set to 0 on transmit" | Either remove "reserved and" or remove "and shall be set to 0 or 3". |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This is a duplication of CID 6651,  which has already been approved.

For discussion: We might want to consider creating a new definition in Clause 8 conventions:

“Undefined fields and subfields are set to any value they can represent upon transmission and are ignored upon reception”

Proposed resolution:

Revised. Replace cited sentence with:

"Otherwise this subfield is set to 0 or 3 upon transmission,  and is ignored upon reception."

(Note to editor,  this is the same as the resolution to CID 6651)

| 6413 | 907.23 | 8.4.2.66.4 |  | "The Channel Number field indicates the channel number, or center frequency index of the frequency segment if the identified channel comprises noncontiguous frequency segments" -- the bit from "if" onwards makes no sense. 1) What is its precedence w.r.t. the "or"? 2) Which frequency segment is being referred to, if there is more than one? | Clarify (maybe need to say "of frequency segment 0"?) |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

The Channel Number field indicates the channel number, or center frequency index of the frequency segment if the identified channel comprises noncontiguous frequency segments, of the Peer-to-Peer Link events requested and included in the Peer-to-Peer Link event report. A Channel Number of 0 in all N+1 Channel Number subelements indicates a request to report any Peer-to-Peer Link event for any supported channel in the specified filtering Operating Class.

Youhan proposes the following changes:

Change at 907.23:

The Channel Number field indicates the channel number or (if the corresponding operating class in Annex E has “—“ in the channel set column) the center frequency index of the frequency segment containing the primary 20 MHz channel if the identified channel comprises noncontiguous frequency segments of the Peer-to-Peer Link events requested and included in the Peer-to-Peer Link event report. A Channel Number of 0 in all N+1 Channel Number subelements indicates a request to report any Peer-to-Peer Link event for any supported channel in the specified filtering Operating Class.

Proposed Resolution: R9:

Revised.  Make changes under CID 6413 in <this-document>.  These clarify that the frequency segment is that containing the primary 20MHz channel.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6455 | 1916.54 | 11.4.3.4.4 | J | "The PN shall be implemented as a 48-bit monotonically incrementing non-negative integer," -- what does "monotonically incrementing" mean? I think 1, 1, 1, 1 is a monotonically increasing (and monotonically decreasing, in fact), sequence | According to Wiki, if it never stays the same, it's "stricty increasing", so use that term instead | REJECTED (MAC: 2015-06-26 17:25:18Z): Wiki is not an authoritative source for IEEE Std 802.11 and 1, 1, 1, 1 are not values for a "monotonically incrementing" integer because it is not incrementing. | EDITOR |

| 6456 |  |  |  | "monotonic" (/ly) (increasing, incrementing, etc.) is used about 45 times, but in many of those instances it seems the intent is that it be strictly increasing (i.e. it may not stay the same) | Examine the 45 instances of "monotonic" and replace them with "strictly increasing" or similar, where need be (an example where it need not be is for the mapping from power to RCPI/RSSI such as at 2175.48) |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

A similar comment (CID 6455) was resolved thus:

REJECTED (MAC: 2015-06-26 17:25:18Z): Wiki is not an authoritative source for IEEE Std 802.11 and 1, 1, 1, 1 are not values for a "monotonically incrementing" integer because it is not incrementing.

Discussion:

What TGmc wishes to assert is up to TGmc members.  However the definition of monotonically increasing (and by extension, also monotonically incrementing) is well defined and means “non-decreasing” and not “strictly increasing”.

The term “incremented monotonically” is essentially meaningless.   Monotonically relates to a property of adjacent members of a sequence.  It does not describe the process used to construct the sequence.

If we care about how these should be interpreted we should use only the following well-defined terms: “monotonically increasing” (or non-decreasing), “strictly increasing” - plus their inverses.

I have reviewed all uses of “monotonically” and identified the following changes:

Status:  This was previously marked “ready for motion” in this document.

The telecon minutes from Aug 28th state:

“Removed CID 6456, as it conflicts with CID 6455 and needs the document reference fixed up.  We’ll fix it up and bring it back for motion.”

I think this was a database update issue,   and not an issue with the proposed resolution below.  But I would like to revisit that to determine that this is indeed the case.

Propose Resolution:  (also make this the resolution of CID 6455).

Revised. Make changes under CID 6456 in <this-document>.  These changes revise the use of “monotonically” throughout the standard as necessary according to its interpretation as a change in a consistent direction, or no change.

Proposed Changes:

At 1919.22 change “monotonically” to “strictly”

At 1916.54, 1922.32 change “monotonically incrementing non-negative” to “strictly increasing”

At 2117.13, change “HWMP SNs are incremented monotonically as unsigned integers.” to

“HWMP SNs are strictly increasing unsigned integers.”

At 2941.39 and all occurrences matching “way is to monotonically increase”:

One easy way is to increment the EventIndex for new reports being written in the table.

At 3590.22:

The airtime cost constants (Table 13-4(Airtime cost constants)) and estimates of the average data rate and frame error rate will vary from one implementation and configuration of the IEEE Std 802.11 PHY and MAC to the other. While no mechanism is defined to measure the average data rate and the frame error rate, it is expected that numeric values will not exhibit large variations in amplitude over the lifetime of a path. Unstable measurements might cause path selection instabilities.

At 1965.59:

The Supplicant shall maintain a separate key replay counter for sending EAPOL-Key request frames to the Authenticator; the Authenticator also shall maintain a separate replay counter to filter received EAPOL-Key request frames.

| 6577 | 575.37 | 8.2.4.5.5 |  | Calling it "No explicit acknowledgement or PSMP Ack" implies it's not always used for PSMP Ack. However the rest of the spec just calls it PSMP Ack, implying it must be used for PSMP. | Is that Ack Policy used for anything other than PSMP? If not, delete the "No ... or". If it is, fix all the places which suggest it isn't |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

Uses are described for this Ack Policy for the two cases of bit 6.  One covers the “no immediate ack” case, the other covers the “PSMP” case.  In the case of bit 6 being zero, this ack policy is used for “QoS CF-Poll and QoS CF-Ack +CF-Poll Data frames”  as described at 575.43.

| 6461 | 575.63 | 8.2.4.5.4 |  | It says "The recipient can expect a BlockAckReq frame in the future" but if this is in an A-MPDU, it might instead get an A-MPDU with ack policy Normal Ack or Implicit Block Ack Request | Add ",or alternatively a frame with an Ack Policy indicating Normal Ack or Implicit Block Ack Request," after "The recipient can expect a BlockAckReq frame in the future" in the cited text |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This is a reasonable change.

For example a sequence of A-MPDUs in which only the last solicits a BA using the implicit block ack mechanism is a reasonable sequence, and arguably not convered by the original text.  The change supports this usage.

Proposed Resolution: R8

Revised. At 575.63, change “The recipient can expect a BlockAckReq frame in the future” to “The recipient can expect a BlockAckReq frame or implicit block ack request in the future”

| 6101 | 595.31 | 8.3.1.1 |  | There are two bits marked as "B13" in Figure 8-18. | Change the second "B13" to "B14" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Propose

Proposed Resolution:

Accepted.

| 5119 | 595.61 | 8.3.1.2 |  | The sentence "The TA field is the address of the STA transmitting the RTS frame or a bandwidth signaling TA" gives the impression that bandwidth signaling TA is also applicable for other types of STAs, such as DMG STA. The following sentence in the same paragraph does not help, because it makes no mention to other types of STAs. The same situation happens in 8.3.1.5, 8.3.1.8.1, 8.3.1.9.1 | The sentence should be rewritten for clarity. Propose "The TA field is the address of the STA transmitting the RTS frame, except that for a VHT STA this field can be used as a bandwidth signaling TA". The other subclauses should also be clarified accordingly. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This really is a philosophical point.  At how many locations do we need to say that <Y> feature is supported for an <X> STA.

I believe we should avoid duplication of normative specification.

Note CID 5867 also touches this text.

For example, we used to have statements for the CTS frame saying “the CTS frame is not transmitted by a DMG STA”.  That sentence was removed because it was redundant.

Straw poll:

A: reject the comment - 4

B: accept the direction of the comment - 2

Proposed Resolution:

Rejected.  The cited statement is correct.   Support for bandwidth signalling for VHT STAs is clearly indicated at 77.01.   There is no need to add the requested limitation to VHT also at the cited location.  Also note that the cited text is in Clause 8 defining frame formats.

| 6252 | 616.46 | 8.3.2.2.2 |  | There is no direct normative constraint on the max MPDU size for DMG, just on the max MSDU/A-MSDU size (7920/7935) | Delete "and 7995 octets for DMG STAs" at the referenced location |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Reference is 617.46

Proposed Resolution: R10 (Based on D4.0)

Accepted.

| 6055 | 617.28 | 8.3.2.2.2 |  | Text says, "i.e., all of the MSDUs are intended to be received by a single receiver". But, there is no restriction on group addressed A-MSDU transmission. In fact, GCR explicitly uses group addressed A-MSDUs. | Change "be received by a single receiver" to "be received by a single receiver, or by all receivers of a single group address" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

1309.52 states: “The Address 1 field of an MPDU carrying an A-MSDU shall be set to an individual address or to the GCR concealment address.”

Proposed Resolution: R8

Revised. Make changes under CID 6055 in <this-document>.  This excludes GCR from the cited statement.

At 617.27

Excluding A-MSDUs sent using GCR, an A-MSDU contains only MSDUs whose DA and SA parameter values map to the same receiver address (RA) and transmitter address (TA) values, i.e., all of the MSDUs are intended to be received by a single receiver, and necessarily they are all transmitted by the same transmitter. The rules for determining RA and TA are independent of whether the frame body carries an A-MSDU. The contents of an A-MSDU sent using GCR are described in 9.24.10.3.

Consider an accept.

| 5979 | 625.53 | 8.3.3.5 |  | The correct field name is "Capability Information" (same comment for Tables 8-29/30/31/32) | Correct "Capability" field in Tables 8-29, 8-30, 8-31 and 8-32 to "Capability Information" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The commenter is correct.  The fixed size field is the “Capability Information field”.

Proposed Change:

Revised. Change “Capability” to “Capability Information” in Table 8-27, Table 8-29, Table 8-30, Table 8-31,  Table-32, Table-8-34,  Table 8-40.

| 6351 | 640.10 | 8.3.3.11 |  | The rows with "REJECTED_WITH_SUGGESTED_BSS_TRANSITION" are incompatible with the other rows with the same alg and seqno | Change "Status" in those rows to "Not REJECTED_WITH_SUGGESTED_BSS_TRANSITION" and in other rows to "Any" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Agree that the cited row is an exception and a conflict to the previous row.   Don’t see the need to change “Status” to “Any” elsewhere.

Proposed Resolution: R8

Revised.  At 640.10, 640.28 and 640.64 change “Status” to “Not REJECTED_WITH_SUGGESTED_BSS_TRANSITION”

| 5083 | 640.28 | 8.3.3.11 |  | Informal reference to "Status" should be formal. | Replace "Status is" with "Status Code field is" throughout this table. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

For discussion:   is the Meaning of the “Status” column clear – e.g., “Status”, “reserved” and various conditions.

Proposed Resolution: R8

Revised

Replace "Status is" with "the Status Code field is" throughout this table.

| 6343 | 642.34 | 8.3.3.13 |  | Can't Action No Acks have MICEs or AMPEEs? | Add MICEs and AMPEEs to Table 8-39 as in Table 8-38 |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

For discussion:

Action no ack was introduced to carry transient management information (Beamforming) and avoid the overhead of an ack,  on the assumption that if it was lost,  it was better to re-generate than re-transmit.  It is hard to imagine what security threats could prompt the need to protect this information.   Is it reasonable to continue with this assumption?    If so,  should we document it?

Proposed Resolution:

Rejected.  There is no reason for Action No Ack to carry MIC or AMPE elements.  The Action No Ack carries information such as beamforming that is of time critical but transient value.  There is no need to cryptographically authenticate such data.  The Action No Ack frame is not used for mesh peering exchanges, so the AMPE element is not present.

| 6053 | 650.25 | 8.4.1.4 |  | STAs support frame formats, not "devices". | Change "devices" to "STAs" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

At 650.25

An ERP STA sets dot11ShortPreambleOptionImplementedto true as all ERP devices support both long and short preamble formats.

Proposed Resolution:

Accepted

| 6247 | 669.47 | 8.4.1.26 |  | In Figure 8-91 there's a B48 | Change the last bit position to B47 |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 5980 | 687.16 | 8.4.1.46 |  | There are combinations that are not covered by the definition of BSS Type subfield in Table 8-63, and this paragraph. For example, value of the BSS Type field in Reassociation Request. This has led to some interop confusion. Suggest to make it clear that the field value is undefined in other cases. We can also make the field "resereved". | Add this sentence to the end of the paragraph: "The value of the BSS Type field in all other cases is undefined.", or "reserved" instead of "undefined", if reserevd does not mandate setting the field to 0 in these cases. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This is one of those cases where the specification is arguably underconstrained, resulting in incompatible implementations.  Normally, we would mark such a field as reserved, but this does two bad things:

- It implicitly marks a field as available for later use, which is not safe if it’s already being used by some implementations

- It makes non-compliant any device that is transmitting a value in this field.

Proposed Changes:

At 687.16:

The BSS Type subfield is defined in Table 8-63 (The BSS Type subfield) for specific types of frame cited below. An AP sets the BSS Type subfield to 3 within transmitted DMG Beacon, Probe Response, or (Re)Association Response frames. A PCP sets the BSS Type subfield to2 within transmitted DMG Beacon, Probe Response, or (Re)Association Response frames. An IBSS STA or a STA that is not a member of a BSS sets the BSS Type subfield to 1 within transmitted DMG Beacon or Probe Response frames. The BSS Type subfield is reserved for all other types of frame.

Proposed Resolution:

Revised. Make changes under CID 5980 in <this-document>.  This defines the subfield as reserved for all other types of frame.

| 5955 | 706.12 | 8.4.1.52 |  | the description field for "channel width" provides information regarding VHT and TVHT STA use of this field, but no information for other STA types, such as HT. The bit that indicates support for the use of the operation mode notification frame is in the extended capabilities IE, which can be transmitted by, for example HT STA, the operation mode notification frame is defined as an action frame of category VHT, so the question becomes - is there a limitation on the types of STA that can set the extended capabilities bit for operation mode notification support? | Clarify the list of STA types that can support operation mode notification. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

At 706.09:

At 1847.34:

10.42 Notification of operating mode changes A STA whose dot11OperatingModeNotificationImplemented is true shall set the Operating Mode Notification field in the Extended Capabilities element to 1. A VHT STA shall set dot11OperatingModeNotificationImplemented to true. A STA that has the value true for dot11OperatingModeNotificationImplemented is referred to as operating mode notification capable.

Discussion: R8

Encodings are provided to support VHT and TVHT.

The Operating Mode Notification frame is a VHT action frame.

It is clear that VHT intended this to be a re-usable feature.   TVHT chose to re-use it.

This interpretation is supported by Annex B:

My interpretation is that only the VHT and TVHT STAs can transmit an operating mode notification frame with defined contents.

Question does this need further clarification?

Yes:

No:

Proposed Resolution:

Revised.

At 2675.38 delete “O” from the “Status” column.

At 1847.37 After “A VHT STA shall set dot11OperatingModeNotificationImplemented to true.”  Add  “A STA that is not a VHT STA shall set dot11OperatingModeNotificationImplemented to false.”

| 6068 | 803.01 | 8.4.2.21.13 |  | The Standard doesn't say whether Location Reference ASCII string is null-terminated or not. Presumably it is not, as it has an explicit Length, and it appears that all ASCII strings in the Standard are not null-terminated (although some don't say explicitly). | Seach out all ASCII strings, and make sure it is clear that they are not null-terminated (most say explicitly, but several don't). Perhaps it makes sense to say once in 8.2.2 that all ASCII strings are not null-terminated. Although, maybe it's easier for the reader to say it explicitly at each usage. |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There are 12 “ASCII string”,  of which none appear to be null terminated,  and most carry the phrase “This string is not null terminated.”

There are 40 instances of “ASCII”.  None of these appear to be null terminated.

The concept of null-termination is specific to certain programming languages.  The question is whether “ASCII string” ambiguously implies the use of such a representation.

Conversation with Mark R:

[[Adrian]] OK, removing the 5 instances that are duplications. I think we should mop up the rest, i.e. make the statement in Clause 8 cover UTF-8 strings too, and in the subclause I'm adding in Clause 9 to say that reserved fields are set to 0 on tx and ignored on rx, also say that ASCII strings are not null-terminated and then remove the 5 instances.

Straw poll: prefer

Single case 0

Convention 5

Proposed Change: R8.

Revised.  At 561.01 insert a new para: “ASCII strings are not null terminated”.

Remove the 5 instances of “This string is not null terminated” in Clause 8.

| 6647 | 825.53 | 8.4.2.24.4 |  | Bit 8 is now used | Delete "8 and" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Bit 8 is indeed used.

Context:

Bits 8 and 14–15: Reserved. The remaining subfields of the RSN Capabilities field are reserved.

Proposed resolution:

Revised. At cited location delete “8 and”.

| 6476 | 827.34 | 8.2.4.26 |  | "In Beacon and Probe Response frames transmitted by an AP: Set to 0 if the AP does not support PSMP operation Set to 1 if the AP supports PSMP operation In Beacon frames transmitted by a non-AP STA: Set to 0 Otherwise: Set to 0 if the STA does not support PSMP operation Set to 1 if the STA supports PSMP operation" can be simplified | Change the cited text to: "In Beacon frames transmitted by a non-AP STA: Set to 0 Otherwise: Set to 0 if the STA does not support PSMP operation Set to 1 if the STA supports PSMP operation", i.e. delete the "In Beacon and Probe Response frames transmitted by an AP:" bit |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Straw poll:

Accept 5

Reject 2

Proposed Resolution:

Accepted.

| 6402 | 831.35 | 8.4.2.26 |  | b59 of Extended Capabilities is not defined | Define it as reserved |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

Agreed.  Furthermore,  it somehow got deleted from the ANA database – which will be fixed at next upload.

Proposed Resolution:

Revised.  Add a row for bit 59 at the cited location and mark it Reserved.

| 6061 | 831.40 | 8.4.2.26 |  | This phrasing makes no sense, "bandwidth for a TDLS direct link on the base channel" and "wider bandwidth on the base channel". There can't be a "wider bandwidth" on the base channel. | Change "The TDLS Wider Bandwidth subfield indicates whether the STA supports a wider bandwidth than the BSS bandwidth for a TDLS direct link on the base channel. The field is set to 1 to indicate that the STA supports a wider bandwidth on the base channel and to 0 to indicate that the STA does not support a wider bandwidth on the base channel." to "The TDLS Wider Bandwidth subfield indicates whether the STA supports a wider bandwidth than the BSS bandwidth for a TDLS direct link with a primary channel equal to the base channel. The field is set to 1 to indicate that the STA supports a wider bandwidth and to 0 to indicate that the STA does not support a wider bandwidth." |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The thing that makes no sense is that a TDLS link cannot both be “on the base channel” and wider than the base channel.

The technical point to debate is whether a TDLS link that is wider than the base channel can operate its base channel on the “wider” bit of that link, or should be constrained to the same channel as the primary channel of the AP.   To do so would require the TDLS link to simultaneously treat the same channel as primary for transmissions to the AP and as secondary for transmissions to its peer.

The proposed change does not allow for an 80 MHz extension of a 40MHz base channel – i.e., “primary channel equal to the base channel” assumes the base channel doesn’t have a primary/secondary.

Proposed Resolution:

Revised.

At cited location change "The TDLS Wider Bandwidth subfield indicates … base channel." to

"The TDLS Wider Bandwidth subfield indicates whether the STA supports a wider bandwidth than the BSS bandwidth for a TDLS direct link with a primary channel equal to the primary or only channel of the base channel. The field is set to 1 to indicate that the STA supports a wider bandwidth and to 0 to indicate that the STA does not support a wider bandwidth."

| 6436 | 838.17 | 8.4.2.29 |  | It's not a BA setup mechanism which is used, it's a BA mechanism | Delete "setup" at the referenced location |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

Block Ack: A separate block ack setup mechanism described in 9.24 (Block acknowledgment (block ack)) is used

Proposed Resolution:

Accepted

| 6054 | 865.13 | 8.4.2.44 |  | STAs have RM capability, not "devices". | Change "device" to "STA" |  | MAC |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

The RM Enabled Capabilities element signals support for radio measurements in a device.

Proposed Resolution:

Accepted.

| 6525 | 36.48 | 3.2 |  | "off-channel: Channel that is not the base channel." -- but "base channel" is not defined | Add a definition of the term "base channel" (and not "Channel that is not the off-channel"!) |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:   technical change alert!

As “base channel” is not defined, we can’t determine whether my proposal changes anything!

Proposed Resolution:

Revised.

Replace cited definition with:

off-channel: A channel used by a tunneled direct link setup (TDLS) station (STA) that does not overlap the channel(s) used by the access point (AP) with which the TDLS STA is associated.

## CID 5046

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5046 |  |  |  | The following MIB variable names occur once (case sensitive) in the draft, which makes it likely to be a spelling error or typo: dot112040BSSCoexistenceManagementSupported, dot11AckFailure, dot11AntennasListGroup, dot11APChannelReport, dot11APGeospatialLocation, dot11AssociationResposeTimeout, dot11BeaconMACAddress, dot11BeaconRssi, dot11BSSTransitionManagementActivated, dot11ChannelAvailabilityActivated, dot11ContactVerificatonSignalInterval, dot11DefaultKeys, dot11DSE, dot11DSEAssociateFailHoldTime, dot11DSEAssociateTimeLimit, dot11DSETransmitDivider, dot11ESSLinkIdentifier, dot11ExendedChannelSwitchActivated, dot11FineTimingMsmtActivated, dot11GasComebackDelay, dot11GCActivated, dot11LSigTxopProtectionOptionImplemented, dot11MacGroupAddresses, dot11MeshHoldingTimeOut, dot11MultiBSSIDEnabled, dot11MultidomainCapabilityActivated, dot11MultipleRetryCountThreshold, dot11NameTable, dot11NonAPStationAuthSinkMulticast, dot11nonAPStationAuxMaxHCCAHEMMrate, dot11NonAPStationBackgroundRate, dot11NonAPStationCipherSuite, dot11NonAPStationHCCA, dot11nonSTBCCTSSuccessCountt, dot11NumberSupportedPowerLevels, dot11OFDMEDThresholddot11STATransmitPowerClass, dot11PHYDMG, dot11PhyTVHT, dot11PhyVHT, dot11ProtectedQLoadActivated, dot11ProtectedTXOPNegotiationActivating, dot11ProtectedTXOPNegotiationImplemented, dot11PublicTXOPNegotiationImplemented, dot11QMFReconfiguationActivated, dot11RMActiveBeaconMeasurementActivated, dot11RMMeasurementPilotCapability, dot11RMNonOperatingChannelMeasurementActivated, dot11RMParallelMeasurementActivated, dot11RMParallelMeasurenmentActivated, dot11RMRequestRowStatus, dot11RMRqstQoSDelayThresholdRange, dot11RMRqstTrigdQoSMEasurementCount, dot11RSNAConfigNumberofGTKSAReplayCounters, dot11RSNAEnable, dot11RSNAProtectedManagmentFramesActivated, dot11RSNIMeasurementActivated, dot11ShortGIOptionIn80Activated, dot11STAStatisticsPSMPUTTGrantDuration, dot11SupportedDataratesRxValue, dot11SupportRxAntenna, dot11SupportTxAntenna, dot11TimeAdvertisementIntervalDTIMs, dot11transmitted, dot11TVHTOptionImpelemented, dot11TVHTStationConfig, dot11VHTSUBeamformeeActivated, dot11VHTSUBeamformeeImplemented, dot11WirelessMgmtOptions, dot11WNMRequestNextIndex, dot1xAuthTxPeriod, dot11APC | Review all such uses and use the correct name |  | GEN |

Status: pending action “6.4.2.3	Concern on “dot11APGeospatialLocation”

6.4.2.3.1	dot11APLCITable is correct table replacement

6.4.2.3.2	dot11APCivicLocation change to dot11APCivicLocationTable

6.4.2.3.3	The inconsistency in the text vs MIB was the intent of the changes proposed here.

6.4.2.3.4	ACTION ITEM: Stephen MCCAAN/Mark HAMILTON – will provide a comment to change the MIB and text to provide the correct labelling.”

Proposed Resolution.

Revised.  Make changes under CID 5046 in <this-document>.  This document indicates a disposition for each of the cited occurences.

Proposed Changes:

(The changes are shown in bold italic.)

dot112040BSSCoexistenceManagementSupported

This is a typo for dot112040BSSCoexistenceManagementSupport

At 3560.44 change:

S.5.2 Establishing a 20/40 MHz BSS Before starting a 20/40 MHz BSS, a 40MC HT AP isrequired by the rules defined in 10.16.5 (Scanning requirements for 40-MHz-capable STA) to examine the channels of the current regulatory domain to determine whether the operation of a 20/40 MHz BSS might unfairly interfere with the operation of existing 20 MHz BSSs. The AP (or some of its associated HT STAs) is required toscan all of the channels of the current regulatory domain in order to ascertain the operating channels of any existing 20 MHz BSSs and 20/ 40 MHz BSSs. This type of scanning is called OBSS scanning. The particulars of OBSS scanning are controlled by the following MIB attributes: — dot11FortyMHzOptionImplemented — dot112040BSSCoexistenceManagementSupport — dot11FortyMHzIntolerant — dot11BSSWidthTriggerScanInterval — dot11BSSWidthChannelTransitionDelayFactor — dot11OBSSScanPassiveDwell — dot11OBSSScanActiveDwell

dot11AckFailure

This is a typo

At 3004.37 change “dot11AckFailure” to “dot11AckFailureCount”

dot11AntennasListGroup

At 2187.16 change “…Group” to “…Entry”

dot11APChannelReport – harmless (occurs as a “topic” heading in the MIB).

dot11APGeospatialLocation

This is addressed in the resolution to comment 5047.

dot11AssociationResposeTimeout,

Typo

At 184.08 change:

Specifies a time limit (in TU) from dot11AssociationResponseTimeout, after which the reassociation procedure is terminated.

dot11BeaconMACAddress,

dot11BeaconRssi,

These will be necessarily covered in the treatment of CID 5022 and 6208.  No change is proposed here.

dot11BSSTransitionManagementActivated,

This is a typo

At 1743.43 change

“dot11BSSTransitionManagementActivated” to

“dot11BSSTransitionActivated”

dot11ChannelAvailabilityActivated,

At 832.69 change:

The STA sets the Channel Availability Query field to 1 when dot11GDDActivated is true and sets it to 0 otherwise. See 10.44.4 (Channel availability query (CAQ) procedure).

At 1853.51 change “dotGDDActivated” to “dot11GDDActivated”

dot11ContactVerificatonSignalInterval,

Typo.

At 1854 change “dot11ContactVerificatonSignalInterval” to “dot11ContactVerificationSignalInterval”

dot11DefaultKeys,

Discussion:  This is in 11.2.2.3 “WEP state”. As this is deprecated, we can ignore the error.

dot11DSE,

At 3352.24 Usage OK as subsequent text explains intended meaning.

dot11DSEAssociateFailHoldTime,

At 2919.37 change:

dot11DSEEnablementFailHoldTime OBJECT-TYPE SYNTAX Unsigned32 MAX-ACCESS read-only STATUS current DESCRIPTION "This is a control variable. It is written by the SME when the device is initialized. dot11DSEEnablementFailHoldTime indicates the number of seconds that a dependent STA must not transmit in a DSE frequency band when it fails to attain enablement with an enabling STA within dot11DSEEnablementTimeLimit seconds. Unless another value is mandated by regulatory authorities, the value is 512 seconds." DEFVAL { 512 } ::= { dot11LCIDSEEntry 20}

dot11DSEAssociateTimeLimit,

At 2919.21 change:

dot11DSEEnablementTimeLimit OBJECT-TYPE SYNTAX Unsigned32 MAX-ACCESS read-only STATUS current DESCRIPTION "This is a control variable. It is written by the SME when the device is initialized. dot11DSEEnablementTimeLimit indicates the maximum number of seconds that a dependent STA may transmit in a DSE frequency band while attaining enablement with an enabling STA. Unless another value is mandated by regulatory authorities, the value is 32 seconds." DEFVAL { 32 } ::= { dot11LCIDSEEntry 19 }

dot11DSETransmitDivider,

Typo

At 1688.18 change: “dot11DSETransmitDivider” to “dot11DSETransmitDivisor”

dot11ESSLinkIdentifier

Harmless, occurs in MIB comment.

dot11ExendedChannelSwitchActivated,

Typo

At 636.16 change “dot11ExendedChannelSwitchActivated” to “dot11ExtendedChannelSwitchActivated”

dot11FineTimingMsmtActivated,

This is the subject of CID 5172,  so we don’t need to handle it here.

dot11GasComebackDelay,

Typo

At 1774.61 change “dot11GasComebackDelay” to “dot11GASComebackDelay”

dot11GCActivated,

Typo

At 1757.30 change “dot11GCActivated” to “dot11GCRActivated”

dot11LSigTxopProtectionOptionImplemented,

At 3153.46 change “dot11LSigTxopProtectionOptionImplemented” to “dot11LsigTxopProtectionOptionImplemented”

dot11MacGroupAddresses,

Inventive creation of expected name.

At 2658.43 change “dot11MacGroupAddresses” to “dot11GroupAddressesTable”

dot11MeshHoldingTimeOut,

Typo at 2094.27 change “…Out” to “…out”

dot11MultiBSSIDEnabled,

Wrong name used.

At 644.22 change “…Enabled” to “…Activated”

dot11MultidomainCapabilityActivated,

At 643.09 change “dot11MultidomainCapabilityActivated” to “dot11MultiDomainCapabilityActivated”

dot11MultipleRetryCountThreshold,

This term doesn’t appear in the standard.  A related term is the valid name of a field. No change required.

dot11NameTable,

At 3267.26 change “dot11NameTable” to “dot11DomainName”

dot11NonAPStationAuthSinkMulticast,

Discussion:  This variable is a work of fiction.  The intent of V.4.2.7 is clear, but the cited variable just doesn’t exist.  A similar named variable “dot11NonAPStationAuthSourceMulticast” does not have the meaning described in this subclause.

At 3579.45 change:

V.4.2.7 Authorized Service Access Type This per-non-AP STA parameter indicates the access type allowed for the non-AP STA based on the SSPN decision. The AP will use this information for authorization requests from the STA, e.g., allow or disallow direct link operation and group addressed services. The parameter uses TruthValues to indicate the service type authorized. The following MIB attributes are used: — dot11NonAPStationAuthDls is to authorize a non-AP STA to use DLS — dot11NonAPStationAuthMaxSourceMulticastRate isto authorize a non-AP STA to source group addressed stream(s) to toward the network

dot11nonAPStationAuxMaxHCCAHEMMrate,

Typo

At 142.38 change

“dot11nonAPStationAuxMaxHCCAHEMMrate” to

“dot11NonAPStationAuthMaxHCCAHEMMRate”

dot11NonAPStationBackgroundRate,

Wrong name

At 142.34 change

“dot11NonAPStationBackgroundRate” to

“dot11NonAPStationAuthMaxBackgroundRate”

dot11NonAPStationCipherSuite,

Discussion:  This variable doesn’t exist, but two variables dot11NonAPStationUnicastCipherSuite and  dot11NonAPStationBroadcastCipherSuite do exist.  However, V4.2.4 indicates this is relevant only to unicast traffic.

At 3578.46 change:

V.4.2.4 Link Layer Encryption Method This parameter indicates the link layer encryption method selected during the RSNA establishment process for protecting the unicast communication between the non-AP STA and the AP. The cipher suite format of this parameter is drawn from the RSNE defined in 8.4.2.24 (RSNE). The AP obtains this information about the STA via the MLME SAP. In the interworking service, the SSPNalso participates in the selection of the cipher suite selection, as described in 10.25.5. Therefore, the link layer encryption method selectedwill meet or exceed the security requirement of the SSPN. NOTE—In interworking, the SSPN can require visibility and configurability of the STA access. With this information available to the SSPN, the operator would be able to have better control, e.g., barring access to IEEE Std 802.11 networks if null encryption is used. This is also related to the operator network’s configuration, e.g., if preauthentication should be supported. The following MIB attribute is used: — dot11NonAPStationUnicastCipherSuite

dot11NonAPStationHCCA,

This variable doesn’t exist.  It’s presence in this comment is an artefact of a hyphen having been inserted in the variable name to avoid ugly justificaiton.

At 3580.48 change: “— dot11NonAPStationHCCA-HEMMMSDUCount,” to “— dot11NonAPStationHCCAHEMMMSDUCount,”   (note deletion of hyphen)

dot11nonSTBCCTSSuccessCountt,

Typo

At 3014.31 change “dot11nonSTBCCTSSuccessCountt” to “dot11nonSTBCCTSSuccessCount”

dot11NumberSupportedPowerLevels,

Wrong term.

At 2564.18 change “dot11NumberSupportedPowerLevels” to “dot11NumberSupportedPowerLevelsImplemented”

dot11OFDMEDThresholddot11STATransmitPowerClass,

This term doesn’t exist in the standard.  No change required.

dot11PHYDMG,

At 3220.65 change “the dot11PHYDMG Table” to “dot11PHYDMGTable”

dot11PhyTVHT,

Harmless – occurs in MIB comment.  No change proposed.

dot11PhyVHT,

Harmless – occurs in MIB comment.  No change proposed.

dot11ProtectedQLoadActivated,

Wrong term

At 1797.16 change “dot11ProtectedQLoadActivated” to “dot11ProtectedQLoadReportActivated”

dot11ProtectedTXOPNegotiationActivating,

Wrong term.

Change 1797.16:

Pairwise cipher suite selectors WEP-40, WEP-104, and TKIP shall not be used as the pairwise cipher suite when dot11MeshSecurityActivated, dot11ProtectedTXOPNegotiationActivated, or dot11ProtectedQLoadReportActivated is true.

dot11ProtectedTXOPNegotiationImplemented,

dot11PublicTXOPNegotiationImplemented,

Wrong terms.

At 1800.01:

An HCCA AP for which dot11PublicTXOPNegotiationActivated is true or dot11ProtectedTXOPNegotiationActivated is true shall be able to maintain one or more dot11APCEntry(s) for each collaboration candidate in the dot11APCTable. These fields indicate the schedules that the AP should try to avoid using when creating schedules for new TS requests.

dot11QMFReconfiguationActivated,

Typo

At 1793.10 change “dot11QMFReconfiguationActivated” to “dot11QMFReconfigurationActivated”

dot11RMActiveBeaconMeasurementActivated,

Discussion:  I can’t find anything like this MIB variable.   Also its role in the sentence cited below is unclear:  is the “when” an additional criterion, or an explanation of the purpose of this non-existence MIB variable?

At 2860.01 change:

The value of ProbeDelay to be used when making a beacon type measurement with measurement mode active .

dot11RMMeasurementPilotCapability,

This and nothing like it appears to exist.  I think it must be a “hang on” from an earlier draft of .11k.  Propose to delete mention of this.

At 203.09 change:

This element is optionally present when dot11RMMeasurementPilotActivated is a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 10.11.14 (Multiple BSSID Set)) with two or more members, or if dot11MultiBSSIDActivated is true

dot11RMNonOperatingChannelMeasurementActivated,

I can’t find anything that this might be.

At 1653.34 change:

10.11.2 Measurement on operating and nonoperating channels Measurements on nonoperating channels might need the measuring STA to interrupt its data services on the operating channel, switch channels, and make measurements. Measurements on the operating channel might not require the STA to interrupt its data services.

dot11RMParallelMeasurementActivated,

Typo

At 1656.61 change “dot11RMParallelMeasurementActivated” to “dot11RMParallelMeasurementsActivated”

dot11RMParallelMeasurenmentActivated,

Typo

At 1656.57 change “dot11RMParallelMeasurenmentActivated” to “dot11RMParallelMeasurementsActivated”

dot11RMRequestRowStatus,

Typo

At 2953.40 change “dot11RMRequestRowStatus” to “dot11RMRqstRowStatus”

dot11RMRqstQoSDelayThresholdRange,

Wrong term

At 2962.44 change “dot11RMRqstQoSDelayThresholdRange” to “dot11RMRqstTrigdQoSDelayThresholdRange”

dot11RMRqstTrigdQoSMEasurementCount,

Typo

At 2960.59 change “dot11RMRqstTrigdQoSMEasurementCount” to “dot11RMRqstTrigdQoSMeasurementCount”

dot11RSNAConfigNumberofGTKSAReplayCounters,

Typo

At 825.12 change “dot11RSNAConfigNumberofGTKSAReplayCounters” to “dot11RSNAConfigNumberOfGTKSAReplayCounters”

Globally change “dot11RSNAConfigNumberofPTKSAReplayCounters” to

“dot11RSNAConfigNumberOfPTKSAReplayCounters” (3 instances)

dot11RSNAEnable,

Typo

At 1946.17 change “dot11RSNAEnable” to “dot11RSNAEnabled”

dot11RSNAProtectedManagmentFramesActivated,

Typo

At 2027.05 change

“dot11RSNAProtectedManagmentFramesActivated” to “dot11RSNAProtectedManagementFramesActivated”

dot11RSNIMeasurementActivated,

Wrong term

At 187.57 change “dot11RSNIMeasurementActivated” to “dot11RMRSNIMeasurementActivated”

dot11ShortGIOptionIn80Activated,

At 1317.32 change “dot11ShortGIOptionIn80Activated” to “dot11VHTShortGIOptionIn80Activated”

dot11STAStatisticsPSMPUTTGrantDuration,

This appears to be a valid variable.  No change proposed.

dot11SupportedDataratesRxValue,

At 2379.07 change “dot11SupportedDataratesRxValue” to “dot11SupportedDataRatesRxTable”

dot11SupportRxAntenna,

At 2213.54 change “dot11SupportRxAntenna” to “dot11SupportedRxAntenna”

dot11SupportTxAntenna,

At 2213.55 change “dot11SupportTxAntenna” to “dot11SupportedTxAntenna”

dot11TimeAdvertisementIntervalDTIMs,

Wrong term

At  623.30 change “dot11TimeAdvertisementIntervalDTIMs” to “dot11TimeAdvertisementDTIMInterval”

dot11transmitted,

Typo

At 784.35 change “dot11transmitted” to “dot11Transmitted”

dot11TVHTOptionImpelemented,

Typo

At 78.10 change “dot11TVHTOptionImpelemented” to “dot11TVHTOptionImplemented”

dot11TVHTStationConfig,

Harmless – in MIB comment.  No change proposed.

dot11VHTSUBeamformeeActivated,

No such MIB variable exists.  The line at 78.16 already catches the “Implemented” case.

At 78.27 delete ‘— “dot11TVHTSUBeamformeeActivated” replaces “dot11VHTSUBeamformeeActivated”.’

dot11VHTSUBeamformeeImplemented,

At 1436.04 replace “dot11VHTSUBeamformeeImplemented” with “dot11VHTSUBeamformeeOptionImplemented”

dot11WirelessMgmtOptions,

Harmless – in MIB comment.  No change proposed.

dot11WNMRequestNextIndex,

Although unusual, this is valid.

dot1xAuthTxPeriod,

Discussion:  This variable exists in dot1x.  As it is quoted from “e.g.”,  this is a non-normative refereuce.  Don’t propose to change anything.

The PTK shall not be used longer than the PMK lifetime as determined by the minimum of the PMK lifetime indicated by the AS, e.g., Session-Timeout + dot1xAuthTxPeriod or from dot11RSNAConfigPMKLifetime. When RADIUS is used and the Session-Timeout attribute is not in the RADIUS Accept message, and if the key lifetime is not otherwise specified, then the PMK lifetime is infinite.

dot11APC

Harmless – in MIB comment.  No change proposed.

## CID 6712 (Editor)

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6712 |  |  | V | Do not allow "A-MPDU" and "A-MSDU" to span lines (i.e. make the hyphen always non-break). | As it say in the comment | REVISED (EDITOR: 2015-05-01 11:33:52Z) - This comment will be passed to the IEEE-SA publication editor for consideration. | EDITOR |

Discussion:

A revised was proposed as shown above.  The ad-hoc notes stated: EDITOR: 2015-05-01 11:33:17Z - There are ~1200 instances of these terms.  Insertion of the non-breaking hyphen is a manual operation if the font is to be preserved properly.

Since then Mark Rison has determined that a global search & replace operation is feasible using \+ in frame-maker as the replacement string.

He has identified the following likely list of 480 strings for replacement of the hyphen with a non-breaking hyphen:

1349 non-AP 621 A-MSDU 492 A-MPDU 414 non-HT 280 L-SIG 187 A-BFT 176 S-AP 162 00-0F-AC 159 U-APSD 150 CF-End 137 VHT-SIG-A 137 CF-Poll 127 HT-SIG 123 VHT-SIG-B 122 Non-HT 120 HT-greenfield 112 MA-UNITDATA 108 MM-SME 108 HT-mixed 104 L-LTF 100 L-STF 95 HT-immediate 84 Non-AP 81 HT-LTF 73 S-PCP 72 CF-Pollable 69 GCR-SP 66 R1KH-ID 63 HT-delayed 62 PS-Poll 59 CCA-ED 47 U-PID 45 CF-Ack 43 R0KH-ID 42 TRN-R 42 On-channel 36 BRP-RX 35 non-GCR-SP 35 L-RX 34 power-on 34 TX-RX 33 over-the-DS 33 TRN-T 32 AP-initiated 31 A-PPDU 27 in-band 27 HT-ELTF 24 DL-MU-MIMO 23 on-demand 22 In-Motion 22 BC-REQ 21 HT-STF 20 R-MID 19 TVHT-SIG-B 19 HT-DLTF 18 Over-the-DS 18 HD-DF 17 TX-TRN-REQ 16 in-progres 16 HT-GF-STF 16 BS-FBCK 15 TVHT-SIG-A 15 FD-AF 14 Y-coordinate 14 X-coordinate 14 SU-MIMO 14 S-PSMP 14 IEEE-SA 14 I-MID 13 a-mpdu-end 13 STA-A 13 S-box 13 MA-UNITDATA-STATUS 13 I-BC 12 scalar-op 12 co-channel 12 UP-to-AC 12 TX-to-RX 12 NT-MLME 12 HC/non-AP 12 BRP-TX 11 TR-MLME 11 RX-to-TX 11 MU-MIMO 11 HT-LTF1 11 GCR-A 11 BC-Grant 10 STA-B 10 R-BC 10 Q-Ack 10 ITU-T 9 TE-B 9 TE-A 9 GID-H 9 C-PSDU 8 elem-op 8 TX-TRN-REQ=1 8 On-demand 8 HT-SIG2320 8 Co-Located 7 on-the-air 7 follow-up 7 SU-only 7 S1KH-ID 6 no-ack¦block-ack 6 co-located 6 Z-coordinate 6 TX-TRN-OK 6 S0KH-ID 6 S-APSD 6 Non-GCR-SP 6 No-Ack/No-Retry 6 NAV-SA 6 NAV-DA 6 MSGCF-ESS-LINK-UP 6 MD-ID 6 IPI-ON 6 HT-Training 6 HT-GF 6 FT-PMKR1-SA 6 AA-AA-03-00-00-00-08-00 5 ppdu-ba 5 on-channel 5 initiator-sequence-ba 5 in-phase 5 STA-to-AP 5 MFSI/GID-L 5 HT-SIG2322 5 HT-Data 5 GID-L 5 FT-R0-AUTH 5 FT-INIT-AUTH 5 FT-Full-Auth 5 FF-FF 5 DS-UNITDATA 4 y-coordinate 4 x-coordinate 4 wi-fi 4 re-scheduling 4 non-FT 4 intra-AC 4 ht-txop-sequence 4 cf-ack-piggybacked-qos-poll-sequence 4 cf-ack-piggybacked-qos-data-sequence 4 burst-ba 4 Tx-Mode 4 TX-train-response 4 TX-TRN-OK=1 4 TPK-TK 4 Space-T 4 RX-train-response 4 RX-Train-response=1 4 Q-out 4 Non-PCP/Non-AP 4 No-LLC 4 No-Fragmentation 4 L-sig 4 L-RX>0 4 ITU-R 4 I-out 4 HT-SIG2321 4 HT-SIG2 4 HT-SIG1 4 HT-LTF2 4 HT-Greenfield 4 FT-Transition-Auth 4 FT-PTK-START 4 FT-PTK-INIT-DONE 4 FT-PTK-CALC-NEGOTIATING 4 FT-INIT-ASSOC 4 DS-STA-NOTIFY 4 CTS-to-AP 4 AA-AA-03-08-00-07-80-9B 4 AA-AA-03-00-00-00-81-37 4 AA-AA-03-00-00-00-80-F3 3 so-called 3 re-unite 3 ppdu-rd 3 ma-no-ack-htc 3 TE-D 3 TE-C 3 S-R 3 PAME-BI 3 MU-capable 3 L-sig-protected-sequence 3 FT-R0-SEND-PMKR1SA 3 FT-INIT-START 3 FT-INIT-R1-SA 3 FT-INIT-GET-R1 3 FT-AUTH 3 Authenticator-to-AS 3 AA-AA-03-00-00-00-08-06 3 9-7<E2><80><94>Non-HT 3 8-42<E2><80><94>Non-AP 3 ----------------N 3 --------------------------------------T 3 --------------- 3 +HTC-HT 2 <E2><80><9C>GCR-SP 2 vht-bf 2 vendor-id 2 up-sampled 2 re-schedule 2 on-line 2 no-ack 2 n-k2432 2 n-k2419 2 n-k 2 n-j 2 m-th 2 m-i 2 in-order 2 i-- 2 ht-nav-protected-sequence 2 ht-ack-sequence 2 dial-in 2 co-factor 2 co-exist 2 cf-sequence 2 burst-rd 2 burst-ba-rd 2 Y-axi 2 XX-SET 2 XX-GET 2 X-axi 2 Wi-Fi 2 VHT-SI 2 VHT-S 2 TX-trainresponse 2 SIG-B 2 S-PCP<E2><80><99> 2 S-D 2 S-Box 2 RX-train-response=1 2 RD-capable 2 R-D 2 PU-APSD 2 NO-ADD-PPDU 2 N-th 2 MLME-of 2 MLME-in 2 L-sig-protection-set 2 KDF-z 2 I/R-BC 2 HT-SIG2341 2 HT-SIG2340 2 HT-Preamble 2 HT-LTFn 2 H-SHA-1 2 G-A 2 FT-START 2 FT-RV-HANDSHAKE-NEGOTIATING 2 FT-R0-AUTH-CLEANUP 2 FT-PTK-CALC-NEGOTIATING3 2 FT-PTK 2 FT-PMK-R1-SA-RECD 2 FT-PMK-R1-SA-PUSH 2 FT-Initial-Association 2 FT-Initial 2 FT-INIT-R1 2 FT-INIT 2 FT-HANDSHAKE-DONE 2 FT-GET-PMK-R1-SA 2 E-mail 2 De-aggregat 2 CF-Polling 2 CF-Pollability 2 CF-Pol 2 CF-End1332 2 CF-End+Ack 2 C-language 2 AP-defined 2 AA-AA-03-00-00-F8-81-37 2 AA-AA-03-00-00-F8-80-F3 2 AA-AA-03-00-00-00-80-9B 2 9-1<E2><80><94>UP-to-AC 2 8-133<E2><80><94>ACI-to-AC 2 22-23<E2><80><94>VHT-SIG-B 2 22-22<E2><80><94>VHT-SIG-B 2 22-15<E2><80><94>VHT-SIG-B 2 21-24<E2><80><94>TRN-R 2 12-8<E2><80><94>Over-the-DS 2 12-5<E2><80><94>Over-the-DS 2 12-12<E2><80><94>Over-the-DS 2 12-11<E2><80><94>Over-the-DS 2 10-15<E2><80><94>Non-AP 2 ---------------N 2 -------------------------S 2 --------------------------r 2 ----------------------------------T 2 ----------------------------------------T 2 --------------------------------------------T 1 yyyy-mm-dd 1 x-coordinate<E2><80><94>i 1 up-sampling 1 unit-to-AP 1 un-admitted 1 tu-b 1 to-ap 1 soon-to-be 1 scalor-op 1 re-scrambled 1 re-request 1 re-order 1 re-initiate 1 ramp-up 1 ppdu-ba-rd 1 per-non-AP 1 per-TS 1 on-going 1 on-air 1 on-QoS 1 non-PS 1 non-PC 1 no-transition 1 no-more-psmp 1 no-acknowledgment 1 n-k=L2433 1 n-k=L2420 1 mu-user-respond 1 mu-user-not-respond 1 mu-ppdu-end 1 m-i1886 1 look-up 1 l-th 1 l-Assoc 1 inverse-op 1 inter-PC 1 inter-AP 1 in-use 1 in-service 1 in-room 1 in-proces 1 in-motion 1 i-th 1 ht-nav-protected 1 f-f2396 1 exclusive-OR 1 de-spread 1 de-rotated 1 co-resident 1 clean-up 1 cf-frame-exchange-sequence 1 cf-ack-piggybacked-qos-poll-sequence= 1 c-b 1 boot-up 1 ab-ae-a5-b8-fc-ba 1 a-mpdu 1 Z-axi 1 XX-RESET 1 VHT-LT 1 US-ASCII 1 UP-based 1 U-PSMP 1 U-NII 1 TX-train-response=1 1 TX-Train-response=1 1 TX-FBCK-REQ=1 1 T-SIG-B 1 S-Ap 1 S-AP<E2><80><99> 1 Re-beamforming 1 RX-trainresponse 1 PS-Poll] 1 PRF-n 1 PIDF-LO 1 PC-capable 1 On-line 1 On-board 1 On-Demand 1 No-transition 1 No-Retry 1 No-Ack/NoRetry 1 No-Ack/ 1 NOTE-th 1 NON-HT 1 NON-AP 1 N-bit 1 MS-MPPE-Send-Key 1 MS-MPPE-Recv-Key 1 MLME-DEAUTHENTI-i 1 MA-UNITDATASTATUS 1 List-R1KH-ID 1 LU-L 1 LU-G 1 LU-D 1 L-STF2495 1 L-STF2381 1 L-STF2309 1 L-SIG2309 1 L-S 1 L-Preamble 1 L-Pre 1 L-LTF2495 1 L-LTF2381 1 L-LTF2309 1 L-LT 1 L-LENGTH 1 KDF-X 1 In-sequence 1 IG-A2 1 IG-A1 1 IF-MIB 1 HT-greenfield880 1 HT-SiG 1 HT-STF2381 1 HT-STF2309 1 HT-SIG2381 1 HT-SIG2309 1 HT-Mixed 1 HT-LTFs2381 1 HT-LTFs2310 1 HT-LTF12381 1 HT-LTF12310 1 HT-GF-STF2381 1 HT-GF-STF2309 1 HT-GF-LTF1 1 HT-Duplicate 1 HT-DATA 1 H-Television 1 FTE[R1KH-ID 1 FT-related 1 FT-RV-DS-CONFIRM 1 FT-RV-AIR-CONFIRM 1 FT-RESERVE-2 1 FT-RESERVE 1 FT-R0-SENDPMKR1SA 1 FT-PTK-CALC 1 FT-NO-RV-CONFIRM 1 FT-FULL-AUTH 1 FT-DS-REQUEST 1 FT-DONE 1 FT-AIR-REQUEST 1 FR-NC 1 FF-FF-FF-FF-FF-FF 1 ESS-Link-Up 1 EDXL-DE 1 De-aggregation 1 DS-to-mobile 1 DS-UNITDATA3554 1 DS-UNITDATA3552 1 DMG-P 1 DMG-M 1 D-SLP 1 Co-located 1 CTSto-AP 1 CF-polling 1 CF-Frame 1 CF-End+stbc+QAP 1 CF-End+non-stbc+QAP 1 CF-End+non-QAP 1 CF-End+QAP 1 CF-Ack2671 1 CE-CP2402 1 CALC-PMK-R 1 CA-QC 1 CA-ON 1 CA-BC 1 C-MPDU 1 BF-FBCK 1 BC-grant 1 AU-NT 1 AU-NSW 1 AU-ACT 1 AP-to-AP 1 ACI-to-AC 1 AC-VO 1 AC-VI 1 AC-DE-48-12-7B-80 1 AC-BK 1 AC-BE 1 AA-AA-03-00-00-00-81-00-87-65VLAN-tagged 1 AA-AA-03-00-00-00-81-00-87-65- 1 AA-AA-03-00-00-00-81-00-87-65 1 AA-AA-03-00-00-00-08-003542 1 A-serie 1 A-MDSU 1 A-MA-MSDU 1 00-AF-AC 1 00-0F-A

I suggest targetting the top 30.  This will take 0.5-1.0 editing day.  Anything more than this is of limited value for editing cost, IMHO.

Straw poll:  I prefer to change

A: the top 30 by frequency of the original list 111111

B: the top 30 by frequency of the list with pattern [A-Za-z]-  or –[A-Za-z]$ 1

C: don’t care, don’t know 11

So the proposed resolution is:

Proposed Resolution:

Revised.  In the following list of terms replace hyphens by non-breaking hyphens.

1349 non-AP

621 A-MSDU

492 A-MPDU

414 non-HT

280 L-SIG

187 A-BFT

176 S-AP

162 00-0F-AC

159 U-APSD

150 CF-End

137 VHT-SIG-A

137 CF-Poll

127 HT-SIG

123 VHT-SIG-B

122 Non-HT

120 HT-greenfield

112 MA-UNITDATA

108 MM-SME

108 HT-mixed

104 L-LTF

100 L-STF

95 HT-immediate

84 Non-AP

81 HT-LTF

73 S-PCP

72 CF-Pollable

69 GCR-SP

66 R1KH-ID

63 HT-delayed

62 PS-Poll

Additionally, This comment will also be passed to the IEEE-SA publication editor for consideration.

(Note to editor, non-breaking hyphen is coded as “\+” in the frame-maker find & replace operation.  Also,  don’t attempt to flag changes.)

## CID 5310

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5310 | 738.35 | 8.4.2.20.6 |  | "A Yes in the Extensible column": but what is in the column is "Yes" (in quotes because that is the symbol that is actually in the column). | Replace 'A Yes in the Extensible' with 'A "Yes" in the Extensible'. | See cid 5311. | EDITOR |

Discussion:

We fixed this as a side effect of resolving comment 6707,  by introducing the folowing “common” text:

At the location in this standard that a subelement is defined, the definition might indicate if the subelement is extensible, typically using a table column called “Extensible” in the table in which subelement IDs are defined. A subelement that is indicated as extensible (typically with “Yes” in the “Extensible” column) might be extended in future revisions or amendments of this standard. A subelement that is indicated as extensible through subelements (typically with “Subelements” in the “Extensible” column”) might be extended in future revisions or amendments of this standard by defining (additional) subelements within the subelement.

Proposed Resolution:

Revised.  Incorporate changes in 11-15/758r7 under CID 6707.  These changes make the change proposed by the commenter, in addition to restructuring the description of subelements.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6587 |  |  |  | We need to distinguish between an MCS for a specific PHY and the general concept of an MCS | Introduce the term "HT-MCS" and use that for MCSes which are specific to HT, only using plain "MCS" for the generic concept |  | MAC |

Ad-hoc notes:

MAC: 2015-08-21 10:09:37Z: BRC accepts in principle.  Submission required to identify specific usages (with global search-and-replace is fine) and changes.  Note, include "Basic MCS set" as well as "Supported MCS" and so forth.

Discussion:

The terms to consider are

Supported MCS Set

SupportedMCSSet

CandidateMCSSet

Basic MCS Set

STBC MCS

MCS index

MCS parameters

DMG MCS

HT MCS

MCS <integer>

MCS Selector

MCS feedback

MCS request

primary MCS

receive MCS

Rx MCS Bitmask

Rx MCS set

MCS parameter

Tx MCS set

mandatory MCS set

MCS set of the DMG

MCS (not in above contexts)

There are 796 instances of MCS.   This is too large a number to handle individually because we don’t have enough time or patience so to do.

So we can only consider changing the terminology if we can find bullet-proof HT-specific patterns.  Perhaps the main impediment is that MCS is also used by the DMG PHY (see 2397.11) and MAC (see 1001.38), although “rate” is sometimes the term used in the DMG MAC.

Looking at “Supported MCS Set” Of the 50 instances roughly 20 were clearly HT, and roughly 10 were clearly DMG, and the remainder required some work.

Clearly it is not going to be possible to find a search expression that makes this distinction.

I believe that each instance of MCS needs individual attention to determine whether it should be HT or DMG.

The only way this is going to be feasible is to ask an editor to make the changes speculatively in the draft, and then have TGmc review them.

I think the instructions to the editor should be to look for MCS not preceeded by VHT or TVHT with or without the hyphen and determine whether the scope is clearly HT, clearly DMG,  learly generic or unknown,  and to substitute MCS by HT-MCS, DMG-MCS, <leave alone> and X-MCS respectively.

So the question is whether we should undertake this work and whether any editor is willing to give it a go.

Proposed Resolution:

Rejected. The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

## CID 6680 (MAC)

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6680 | 1865.00 | 11 |  | "An MPDU of type Data with the Protected Frame subfield of the Frame Control field equal to 1 is called a WEP MPDU." -- even if the standard is consistent when using this term, it's a very confusing one now that other forms of protection are defined and indeed preferred. | Remove the term "WEP MPDU" or at least restrict it to MPDUs where WEP has indeed been used as the cipher |  | MAC |

This was discussed on an email thread between Dan, Mark R, Mark H, Jon & Dorothy.   The following is the proposed resolution from Dan,  which didn’t attract any further response on the thread.

Proposed Changes:

1. Modify text on page 2015 lines 42-47:

“An WEP-encapsulated MPDU of type Data with the Protected Frame subfield of the Frame

Control field equal to 1 is called a WEP MPDU. All Oother MPDUs of type Data are called

non-WEP MPDUs.

A STA shall not transmit WEP-encapsulated MPDUs when dot11PrivacyInvoked is false."

2. Change text on page 2016 lines 27-30:

When the boolean dot11ExcludeUnencrypted is true, non-WEP MPDUs of type Data with the

Protected Frame subfield of the Frame Control field equal to 0 shall not be indicated at the MAC

service interface, and only MSDUs successfully reassembled from successfully decrypted MPDUs

shall be indicated at the MAC service interface.

Proposed Resolution:

Revised.  Make changes under CID 6680 in <this-doccument>.  These changes reword text to avoid the use of the term “WEP MPDU”.

## CID 6788

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6788 |  |  |  | "bufferable" v. "buffered" -- there is confusion in these terms. Probably move to "buffered" for most cases | As it says in the comment |  | EDITOR |

Discussion:

“bufferable” is appropriate to unit types that is elgible to be buffered. This is an MSDU, A-MSDU or MMPDU.  “bufferable management frame” is a bit of a misnomer,  but comes from the 1:1 mapping between MMPDU and the frame that carries it.  I can live with this.

“buffered” should relate to instances of a bufferable type of thing.

Also note that the unit of buffering is a “BU”.

There’s also use of “buffered traffic”, which could be replaced with “buffered BU” to be more precise.

Note 576.57 implies that “traffic” is data-only.  So we might also give consideration to replacing “buffered traffic” with “buffered BU” to explicitly include buffered MMPDUs.

I reviewed all uses of “bufferable” and “buffered” and propose the following changes would improve clarity.

Status:  Changes in 1010r3 reviewed acceptable in TGmc.

The changes in 11-15/762 under CID 6788 were also reviewed and found acceptable.

This author took an Action to merge those resolutions.  In retrospect, I believe that is rather difficult,

because the changes in Doc 762 address more than just the terminology of buffered/bufferable.  The changes in that document are disjoint, so they can be considered and approved separately.

However, I have taken input from Mark Rison to extend the locations considered for bufferable/buffered.

Proposed Resolution:

Revised.  Make changes in this document under CID 6788.  This ensure that “buffered” and “bufferable” are used correctly in the context of the power saving mechanisms.

Proposed changes:  (those that are added after the initial TGmc review are shown “new at”)

at 15.22 change:

multiple BSSID capability: The capability to advertise information for multiple basic service set identifiers (BSSIDs) using a single Beacon or Probe Response frame instead of using multiple Beacon or Probe Response frames, each corresponding to a single BSSID, and the capability to indicate buffered bufferable units (BUs) for these multiple BSSIDs using a single traffic indication map (TIM) element in a single Beacon

At 26.08 change:

bufferable unit (BU):An MSDU, A-MSDU (HT STAs and DMG STAs only) or bufferable MMPDU .. NOTE—Bufferable units are used in power saving protocols (see 10.2).

At 82.12 change:

The Multiple BSSID capability also enables the indication of buffered BUs for multiple BSSIDs using a single TIM element in a single beacon.

New at 567.37:

The More Data field is 1 in individually addressed frames transmitted by a mesh STA to a peer mesh STA that is either in light sleep mode or in deep sleep mode for the corresponding mesh peering, when additional buffered BUs remain to be transmitted to this peer mesh STA.

At 567.50 change:

A non-DMG STA sets the More Data field to 1 in non-GCR-SP group addressed frames transmitted by the AP when additional group addressed buffered bufferable units (BUs) that are not part of an active GCR-SP remain to be transmitted by the AP during this beacon interval.

New at 567.59:

The More Data field is set to 1 in GCR-SP group addressed frames transmitted by the AP when additional buffered group addressed BUs that are part of an active GCR-SP remain to be transmitted by the AP during this GCRSP. The More Data field is set to 0 in GCR-SP group addressed frames transmitted by the AP when no more buffered group addressed BUs that are part of an active GCR-SP remain to be transmitted by the AP during this GCRSP.

New at 568.01:

The More Data field is 1 in group addressed frames transmitted by a mesh STA when additional buffered group addressed BUs remain to be transmitted. The More Datafield is 0 in group addressed frames transmitted by a mesh STA when no more buffered group addressed BUs remain to be transmitted. A DMG STA sets the More Data field as follows: — In directed frames, it is set to1 to indicate that the STA has MSDUs or A-MSDUs buffered for transmission to the frame’s recipient during the current SP or TXOP. — It is set to 1 in group addressed frames transmitted by the AP when additional buffered group addressed BUs remain to be transmitted by the AP during this beacon interval. The More Data field is set to 0 in group addressed frames transmitted by the AP when no more buffered group addressed BUs remain to be transmitted by the AP during this beacon interval.

At 665.01 change:

The Max SP Length subfield is 2 bits in length and indicates the maximum number of total buffered BUs

Throughout Table 8-47 (665.09) replace “MSDUs, A-MSDUs, and MMPDUs” with “BUs”.

At 720.52 change:

This bit is set to 1 in TIM elements with a value of 0 in the DTIM Count field when one or more group addressed BUs are buffered at the AP or the mesh STA

At 721.01 change:

Bit number N indicates the status of buffered, individually addressed BUs for the STA whose AID is N. It is determined as follows: — If the STA is not using APSD, and any individually addressed BUs for that STA are buffered and the AP or the mesh STA is prepared to deliver them, then bit number Nin the traffic indication virtual bitmap is 1. — If the STA is using APSD, and any individually addressed BUs for that STA are buffered in at least one nondelivery-enabled AC (if there exists at least one nondelivery-enabled AC), then bit number Nin the traffic indication virtual bitmap is 1. — If the STA is using APSD, all ACs are delivery-enabled, and any individually addressed BUs for that STA are buffered in any AC, then bit number Nin the traffic indication virtual bitmap is 1. — Otherwise,

At 721.39:

The bits 1 to k of the bitmap are used to indicate that one or more group addressed BUs are buffered for each AP corresponding to a nontransmitted BSSID. The AIDs from 1 to kare not allocated to a STA. The AIDs from (k+1) to (2n – 1) are reserved and set to 0. The remaining AIDs are shared by the BSSs corresponding to the transmitted BSSID and all nontransmitted BSSIDs. — When the DTIM Count field is 0 for a BSS that has a nontransmitted BSSID, and one or more group addressed BUs are buffered at the AP for this BSS, the corresponding bits from bit 1 to bit kis set to 1. --Each bit starting from bit 2n in the traffic indication virtual bitmap corresponds to individually addressed traffic buffered for a specific STA within any BSS corresponding to a transmitted or nontransmitted BSSID at the time the Beacon frame is transmitted. The correspondence is based on the AID of the STA.

At 722.25:

For both Method A and Method B, when there are no BUs buffered for any BSS corresponding to a transmitted or nontransmitted BSSID supported, the Partial Virtual Bitmap field is encoded as a single octet equal to 0, the Bitmap Offset subfield is 0, and the Length field is 4. When there are no buffered individually addressed BUs for any BSS corresponding to a transmitted or nontransmitted BSSID, but there are buffered group addressed BUs for one or more of the BSSs, the Partial Virtual Bitmap field consists of the octets number 0 to N0 – 1 where N0 is the smallest positive integer such that (N0 × 8 – 2 n < 8). In this case, the Bitmap Offset subfield value containsthe number 0, and the Length field is N0+3.

At 868.61:

The FMS Descriptor element is included in the Nontransmitted BSSID Profile subelement if the Multiple BSSID element is included in a Beacon frame and if the TIM field indicates there are buffered group addressed BUs for this nontransmitted BSSID.

New at 937.34:

The FMS counters are used by the non-AP STA to identify the DTIM beacon after which buffered group addressed BUs assigned to a particular delivery interval are transmitted. A single FMS Counter is shared by all FMS streams that use the same delivery interval.

New at 937.52:

The Current Count field indicates how many DTIM Beacon frames (includingthe current one) appear before the next DTIM Beacon frame after which the buffered group addressed BUs assigned to a particular delivery interval are scheduled to be transmitted. The Current Count field is 0 on transmission and ignored upon reception when the FMS Counter field is included in the FMS Status subelement.

At 1185.27:

The TIM Element field contains a TIM element as specified in 8.4.2.6 (TIM element). The bit corresponding to buffered group addressed BUs is 0 for all BSSIDs and ignoredupon reception

At 1280.49:

If there are non-GCR-SP buffered group addressed BUs, the PC shall transmit these prior to any individually addressed A-MSDUs, MSDUs, and MMPDUs.

New at 1262.29:

NOTE—Group addressed retransmissions of buffered BUs use the same sequence number as the initial group addressed transmission of the BU. Unicast retransmissions of a buffered group addressed BU delivered via DMS use the same sequence number as the initial unicast transmission of the BU. When a buffered BU is delivered both using group addressing and unicast (e.g., when DMS is active but there are other associatedSTAs not using DMS), the sequence number might differ between the group addressed and unicast transmissions of the same BU.

At 1282.36:

Because the Beacon frame that initiates the CFP contains a DTIM element, if there are associated STAs using PS mode, the buffered group addressed BUs that are not delivered via the GCR-SP delivery method shall be sent immediately after any Beacon frame containing a TIM element with a DTIM count field with a value of 0.

At 1336.08:

The HC shall perform delivery of buffered non-GCR-SP group addressed BUs following DTIM Beacon frames.

New at 1549.30:

In a BSS operating under the DCF, or during the CP ofa BSS using the PCF, upon determining that a BU is currently buffered in the AP, a STA operating in the normal (non-APSD) PS mode transmits a PS-Poll frame to the AP, which responds with the corresponding buffered BU immediately, or acknowledges the PS-Poll and respond with the corresponding buffered BU at a later time. If the TIM indicating the buffered BU is sent during a CFP, a CF-Pollable STA operating inthe PS mode does not send a PS-Poll frame, but remains active until the buffered BU is received (or the CFP ends).

New at 1549.38:

A non-AP QoS STA may be in PS mode before the setup of DLS or block ack. Once DLS is set up, both of the QoS STAs associated with a DLS link suspend the PS mode and shall be awake. When a STA enters normal (non-APSD) PS mode, any downlink block ack agreement without an associated schedule is suspended for the duration of this PS mode. Buffered BUs for a TID without a schedule are sent using Normal Ack following a PS-Poll as described in rest of 10.2.2. Uplink block ack agreements, block ack agreements for any TID with a schedule, and any block ack agreements to APSD STAs continue to operate normally.

New at 1551.44:

Non-GCRSP buffered group addressed BUs are sent by the AP subsequent to the transmission of a Beacon frame containing a DTIM. The DTIM is indicated by the DTIM count field of the TIM element having a value of 0.

New at 1552.01:

APSD defines two delivery mechanisms, namely unscheduled APSD(U-APSD) and scheduled APSD (S-APSD). STAs may use U-APSD to have some or all of their buffered BUs delivered during unscheduled SPs. STAs may use S-APSD to schedule delivery of some or all of their buffered BUs during scheduled SPs. If there is no unscheduled SP in progress, the unscheduled SP begins when the AP receives a trigger frame from a STA, which is a QoS Data or QoS Null frame using an AC the STA has configured to be trigger enabled. An A-MPDU that contains one or more trigger frames acts as a trigger frame. An unscheduled SP ends after the AP has attempted to transmit at least one buffered BU using a delivery-enabled AC and destined for the STA, but no more than the number indicated in the Max SP Length field of the QoS Capability element of the STA’s (Re)Association Request frame if the field has a nonzero value. By setting the EOSP field to 1 in the last frame sent during the SP, an unscheduled SP may be terminated before the maximum number of buffered BUs in the SP has been reached. In order to configure an AP to deliver buffered BUs during anunscheduled SP, a STA designates one or more of its ACs to be delivery-enabled and one or more of its ACto be trigger-enabled.

New at 1553.04:

If the SI is nonzero, a scheduled SP for a GCR group ends after the AP has attempted to transmit at least one buffered BU associated with the GCR group but no more than the number indicated in the Max SP Length field of the QoS Capability element of the STA’s (Re)Association Request frame. The last frame of the GCR SP shall have the EOSP field set to 1

At 1553.33:

If a scheduled SP overlaps the period during which the AP is required to transmit non-GCR-SP group addressed frames and individually addressed frames toSTAs in PS mode that follow a DTIM beacon that has at least 1 bit set to 1 in the partial virtual bitmap of its TIM, the scheduled SP shall be deferred until the AP has transmitted all such buffered BUs.

New at 1553.47:

If scheduled SPs are supported in a BSS, a STA may use both unscheduled and scheduled APSD on different ACs at the same time. The GCR-SP deliverymethod may be used on any AC, irrespective of any non-GCR unscheduled or scheduled APSD flows. When a STA establishes scheduled delivery for an AC the AP shall not transmit buffered BUs using that AC during an SP that is initiated by a trigger frame, and it shall not treat buffered BUs using the AC that are received from the STA as trigger frames. The AP shall decline any ADDTS Request frame that indicates the use of both scheduled and unscheduled APSD to be used on non-GCR-SP frames of the same AC at the same time. APSD shall be used only to deliver buffered individually addressed BUs and buffered GCR-SP BUs to a STA. Non-GCR and non-GCR-SP buffered group addressed BU delivery shall follow the frame delivery rules defined for buffered group addressed BUs as defined in 10.2.2.7 (AP operation during the CFP).

New at 1555.64:

The AP transmits buffered BUs destined for the STA and using delivery-enabled ACs during an unscheduled SP.

New at 1556.09:

When dot11FMSActivated is true and the AP has established an FMS delivery interval for a multicast stream, the AP shall transmit all non-GCR-SP buffered group addressed BUs belonging to particular FMS stream immediately after the DTIM that has the Current Count field value of the FMS Counter field set to 0 for that particular FMS stream.

New at 1556.26:

When the AP transmits an STBC DTIM or TIM Beacon frame, the AP shall retransmit all buffered non-GCR-SP group addressed BUs that were transmitted following the non-STBC DTIM or TIM Beacon frame except that they are transmitted using the basic STBC MCS. It may be the case that a complete set of buffered non-GCR-SP group addressed BUs is sent over a period of time during which non-STBC and STBC transmissions are interleaved, but the transition from non-STBC group addressed transmissions to STBC group addressed transmissions shall be preceded by the transmission of an STBC Beacon frame and the transition from STBC group addressed transmissions to non-STBC group addressed transmissions shall be preceded by the transmission of a non-STBC Beacon frame.

New at 1556.43:

For a STA using U-APSD, the AP transmits one buffered BU destined for the STA from any AC that is not delivery-enabled in response to PS-Poll from the STA. When all ACs associated with the STA are delivery-enabled, the AP transmits one buffered BU from the highest priority AC that has a buffered BU. The AP can respond with either an immediate Data or Management frame or with an Ack frame, while delaying the responding Data or Management frame.

New at 1556.63:

At each scheduled APSD SP for a STA, the APSD-capable AP(i.e., an AP for which dot11APSDOptionImplemented is true) shall attempt to transmit at least one buffered BU, using admitted TSPECs with the APSD and Schedule subfields both set to 1, that are destined for the STA. At each unscheduled SP for a STA, the AP shall attempt to transmit at least one buffered BU, but no more than the value specified in the Max SP Length field in the QoS Capability element from delivery-enabled ACs, that are destined for the STA. The More Data bit of the individually addressed Data or bufferable Management frame using delivery-enabled ACs and destined for that STA indicates that more BUs are buffered for the delivery-enabled ACs. The More Data bit equal to 1 in Data or bufferable Management frames using nondelivery-enabled ACs and destined for that STA indicates that more BUs are buffered for the nondelivery-enabled ACs. For all frames except for the final frame of the SP, the EOSP subfield of the QoS Control field of the QoS Data frame shall be set to 0 to indicate the continuation of the SP. An AP may also set the More Data bit to 1 in a QoS +CF-Ack frame in response to a QoS Data frame to indicate that it has one or more pending BUs buffered for the target STA identified by the RA in the QoS +CF-Ack frame. If the QoS Data frame is using a delivery-enabled AC, the More Data bit in the QoS +CF-Ack frame indicates more BUs buffered for all delivery-enabled ACs. If the QoS Data frame is not using a delivery-enabled AC, the More Data bit in the QoS +CF-Ack frame indicates more BUs buffered for all ACs that are not delivery-enabled.

At 1558.23:

All non-GCR-SP group addressed BUs except those with a service class ofStrictlyOrdered shall be buffered if any associated STAs are in the PS mode, regardless of whether those STAs are CFPollable.

New at 1558.31:

When dot11FMSActivated is true and the AP has set up an FMS delivery interval for a multicast stream, the AP shall send all buffered non-GCR-SP group addressed BUs belonging to a particular FMS stream immediately after the DTIM with the Current Count field value of the FMS Counter field set to 0 for that particular FMS stream.

New at 1558.47:

When the AP transmits an STBCDTIM or TIM Beacon frame, the AP shall retransmit all buffered non-GCR-SP group addressed BUs that were transmitted following the non-STBC DTIM or TIM Beacon frame except that they are transmitted using the basic STBC MCS. It may be the case that a complete set of buffered non-GCR-SP group addressed BUs is sent over a period of time during which non-STBC and STBC transmissions are interleaved, but the transition from non-STBC group addressed transmissions to STBC group addressed transmissions shall be preceded by the transmission of a STBC Beacon frame and the transition from STBC group addressed transmissions to non-STBC group addressed transmissions shall be preceded by the transmission of a non-STBC Beacon frame.

New at 1551.58:

QoS STAs use the Power Management field in the Frame Control field of a frame to indicate whether it is in active or PS mode. As APSD is a mechanism for the delivery of downlink buffered BUs to power-saving STAs, the frames transmitted by a STA in PS mode that is using APSD have the Power Management bit in the Frame Control field set to 1, thereby causingbuffering to take place at the AP.

New at 1557.06:

The More Data bit of the individually addressed frames containing all or part of a buffered BU using delivery-enabled ACs and destined for that STA indicates that more BUs are buffered for the delivery-enabled ACs. The More Data bit equal to 1 in frames containing all or part of a buffered BU using nondelivery-enabled ACs and destined for that STA indicates that more BUs are buffered for the nondelivery-enabled ACs.

New at 1557.30:

If the AP does not receive an acknowledgment to an individually addressed frame containing all or part of a buffered BU sent to a STA in PS mode following receipt of a PS-Poll from that STA, it may retransmit the frame for at most the lesser of the maximum retry limit and dot11QAPMissingAckRetryLimit times before the next Beacon frame, but it shall retransmit that frame at least once before the next Beacon frame, timepermitting and subject to its appropriate lifetime limit. If an acknowledgment to the retransmission is not received, it may wait until after the next Beacon frame to further retransmit that frame subject to its appropriate lifetime limit.

New at 1559.15:

A STA in PS mode shall operate as follows to receive a buffered BU from the AP when no PC is operating and during the CP when a PC is operating.

New at 1559.33:

The STA shall remain in the awake state until it receives the buffered BU in response to its poll or it receives another Beacon frame whose TIM indicates that the AP does not have any BUs buffered for this STA. If the bit corresponding to the STA’s AID is1 in the subsequent TIM, the STA shall issue another PS-Poll to retrieve the buffered BU. When a STA that is using U-APSD and has all ACs delivery-enabled detects that the bit corresponding toits AID is 1 in the TIM, the STA shall issue a trigger frame or a PS-Poll frame to retrieve the buffered BU.

New at 1559.40:

If the More Data field in the received frame containing all or part of a buffered BU indicates that more traffic for that STA is buffered, the STA, at its convenience, shall poll until no more BUs are buffered for that STA.

New at 1559.52:

A STA that stays awake to receive buffered group addressed BUs shall elect to receive all group addressed non-STBC transmissions or all group addressed STBC transmissions and remain awake until the More Data field of the appropriate type (non-STBC or STBC) of buffered group addressed BUs indicates there are no further buffered group addressed BUs of that type, or until a TIM is received indicating there are no more buffered group addressed BUs of that type, or until an FMS Descriptor element is received indicating that there are no further buffered group addressed BUs for which the STA has previously received an FMS Response element in a frame that has a value in Address 1 that matches its MAC address or that has an Address 1 value that is a group address corresponding to a group of which it is a member and that was transmitted by the AP with which it is associated and which had an Element Status value in FMS Status subelement of “Accept”.

New at 1560.09:

A STA in PS mode that is associatedas CF-Pollable shall operate as follows in a BSS with an active PC to receive buffered BUs from the AP during the CFP

New at 1560.15:

To receive buffered group addressed BUs, the STA shall wake up early enough to be able to receive either every non-STBC DTIM or every STBC DTIM thatmay be sent during the CFP. A STA receiving buffered group addressed BUs shall elect to receive all group addressed non-STBC transmissions or all group addressed STBC transmissions and remain awake until the More Data field of the frames containing the buffered group addressed BUs indicates there are no further buffered group addressed BUs of that type, or until a TIM is received indicating there are no more group addressed BUs of that type buffered or until an FMS Descriptor element is received indicating that there are no further buffered group addressed BUs for which the STA has previously received an FMS Response element in a frame that has an Address 1 value that matches its MAC address or that has an Address 1 value that is a group address corresponding to a group of which it is a member and that was transmitted by the AP with which it is associated and which had an Element Status value in FMS Status subelement of “Accept”. See also 9.3.6 (Group addressed MPDU transfer procedure). When the STA detects that the bit corresponding to its AID is 1 in the DTIM at the start of the CFP (or in a subsequent TIM during the CFP), the STA shall remain in the awake state for at least that portion of the CFP throughthe time that the STA receives a buffered individually addressed BU from the AP carried in a frame with the More Data field in the Frame Control field indicating that no further traffic is buffered.

New at 1560.36:

If the More Data field in the Frame Control field of the last frame containing all or part of a buffered BU received from the AP indicates that more traffic for the STA is buffered, then, when the CFP ends, the STA may remain in the awake state and transmit PS-Poll frames during the CP to request the delivery of additional buffered BUs, or may enter the doze state during the CP (except at TBTTs for DTIMs expected during the CP), awaiting the start of the next CFP.

New at 1560.45:

A STA using APSD shall operate as follows to receive a buffered BU from the AP: a) If a scheduled SP has been set up, the STA wakes up at its scheduled start time. (The STA shall wake up early enough to receive transmissions at the scheduled SP.) b) If the STA is initiating an unscheduled SP, the STA wakes up and transmits a trigger frame to the AP. When one or more ACs are not delivery-enabled, the STA may retrieve buffered BUs using those ACs by sending PS-Poll frames to the AP.

New at 1560.57:

The STA may send additional PS-Poll frames if the More Data subfield is 1 in downlink individually addressed frames containing all or part of a buffered BU that do not use any delivery-enabled ACs. The STA may send additional trigger frames if the More Data subfield is 1 in downlink individually addressed frames containing all or part of a buffered BU that use delivery-enabled ACs

New at 1566.49:

A non-AP STA that does not use FMS wakes every DTIM interval and follows buffered group addressed BU reception rules as defined in 10.2.2.8 (Receive operation for STAs in PS mode during the CP).

New at 1568.32:

Once synchronized with the FMS Current Count, the non-AP STA need not wake up at every DTIM interval to receive buffered group addressed BUs.

New at 1575.08:

It is possible that an ATIM frame may be received from more than one STA, and that a STA that receives an ATIM frame may receive more than a single buffered BU from the transmitting STA.ATIM frames are only addressed to the destination STA of the buffered BU. An ATIM for a buffered BU shall have a destination address identical to that of the buffered BU.

At 1576.27:

A STA may enter PS mode if the value of the ATIM window in use within the IBSS is greater than 0. A STA shall not enter PS mode if the value of the ATIM window in use within the IBSS is equal to 0. A STA shall set the Power Management subfield in the Frame Control field of frames containing all or part of a buffered BU or individually addressed Probe Request frame that it transmits using the rules in 8.2.4.1.7 (Power Management field).

At 1577.03:

If power management is in use within an IBSS, a STA shall buffer individually addressed BUs for STAs that are known to be in PS mode. Buffered BUs may be sent to STAs in active mode at any valid time.

New At 1577.42:

Immediately following the ATIM Window, a non-DMG STA shall begin transmission of any buffered group addressed BUs for which an ATIM was previously transmitted. Following the transmission of any group addressed BUs, any buffered BUs addressed to STAs for which an acknowledgment for a previously transmitted ATIM frame was received shall be transmitted. A STA shall use the backoff procedure defined in 9.3.4.3 (Backoff procedure for DCF) for transmission of the first frame following the ATIM Window. All remaining frames shall be transmitted using either the DCF (for non-QoS STAs) or the EDCAF (for QoS STAs).

New at 1577.50:

If a buffered BU is transmitted using fragmentation and if the buffered BU has been partially transmitted when the next Beacon frame is sent in a non-DMG IBSS or when the next beacon interval begins in a DMG BSS, the STA shall retain the buffered BU and announce the remaining fragments by transmitting an ATIM frame during the next ATIM Window/Awake Window. If a STA is unable to transmit a buffered BU during the beacon interval in which it was announced, for example due to contention withother STAs, the STA shall retain the buffered BU and announce the buffered BU again by transmitting an ATIM frame during the next ATIM Window/Awake Window.

New at 1579.51:

A non-AP STA shall buffer BUs addressed to other non-AP STAs in doze state. Buffered BUs shall be transmitted only at designated times (10.2.6.2 (Non-AP and non-PCP STA power management mode)).

At 1586.41:

Buffered BUs that are to be transmitted to a STA that is in PS mode are first announced through ATIM frames during the awake window

At 1586.48:

If a STA receives or transmits an ATIM frame during the awake window, it shall be awake during the CBAPs within the current beacon interval that havethe source AID or destination AID described by the ATIM frame to wait for the announced buffered BUs to be received and/or to transmit announced buffered BUs.

At 1767.29:

An AP advertises that a group addressed stream is subject to GCR-SP within a GCR Response subelement. The subelement indicates the start of each SP. See 10.2.2.5 (Power managementwith APSD). When the Service Interval field in the Schedule element of the DMS Response frame is greater than 0, at every scheduled SP, the AP schedules for transmission buffered GCR-SP group addressed BUs assigned to that particular group address.

At 2081.36:

A mesh STA that receives and accepts a Mesh Peering Open frame (see 13.3.6.2 (Mesh Peering Open frame processing)) shall assign a unique AID among its neighborpeer mesh STAs to the transmitter of the frame. The AID is used in the encoding ofthe TIM element in the Beacon frame (see 8.4.2.6 (TIM element)). AID 0 (zero) is reserved to indicate the presence of buffered group addressed BUs (see 13.14.4 (TIM transmissions in an MBSS)).

At 2164.44:

13.14.4 TIM transmissions in an MBSS The TIM element identifies the peer mesh STAs for which traffic is pending and buffered in the reporting mesh STA. This information is coded in a partial virtual bitmap, as described in 8.4.2.6 (TIM element). In addition, the TIM contains an indication whether group addressed traffic is pending. Every neighbor peer mesh STA is assigned an AID by the reporting mesh STA as part of the mesh peering establishment process (see 13.3.1 (General)). The mesh STA shall identify those peer mesh STAs for which it is prepared to deliver buffered BUs by setting bits in the TIM’s partial virtual bitmap that correspond to the appropriate AIDs.

At 2164.56:

13.14.5 TIM types There are two different TIM types: TIM and DTIM. A mesh STA shall transmit a TIM with every Beacon frame. Every DTIMPeriod, a TIM of type DTIM is transmitted with a Beacon frame. After transmitting a Beacon frame containing a DTIM, the mesh STA shall send the buffered group addressed BUs, before transmitting any individually addressed frames. The More Data field of each group addressed frame shall be set to indicate the presence of further buffered group addressed BUs. The mesh STA sets the More Data field to 0 in the last transmitted group addressed frame following the transmission of the DTIM Beacon frame.

At 2167.21:

The mesh STA may return to the doze state after the beacon reception from this peer mesh STA, if the peer mesh STA did not indicate buffered BUs. If an indication of buffered individually addressed BUs is received, the light sleep mode mesh STA shall send a peer trigger frame with the RSPI field set to 1 to initiate a mesh peer service periodwith the mesh STA that transmitted the Beacon frame (see 13.14.9.2 (Initiation of a mesh peer service period)). If an indication of buffered group addressed BUs is received, the light sleep mode mesh STA shall remainin awake state after the DTIM Beacon reception to receive group addressed BUs. The mesh STA shall remain awake state until the More Data field of a received group addressed frame is set to 0 or if no group addressed frame is received within the PHY specific Group Delivery Idle Time. (See 13.14.5 (TIM types).)

At 3532.26:

The first example is one in which there are no group addressed BUs buffered in the AP but there is traffic for two STAs queued in the AP. STAs with AID 2 and AID 7 have BUs buffered in the AP. Figure O-1 (Partial Virtual Bitmap example #1)shows the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM element for this example.

At 3533.36:

The first example with Multiple BSSID is one in which there are eight BSSIDs and the lowest possible AID that can be assigned to any STA in this example is 8.There are no group addressed BUs buffered in the AP for any of the eight BSSIDs. However, STAs with AID 9 and AID 11 have an individually addressed BUs buffered in the AP. Figure O-4 (Partial Virtual Bitmap example #4, Method A and Method B) shows the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM element for this example when either Method A or Method B is used. It is noted that Method B reduces to Method A in this example.

At 3533.60:

In the next example, there are eight BSSIDs and the lowest possible AID that can be assigned to any STA in this example is 8. There are group addressed BUs buffered at the AP for the transmitted BSSID, and the DTIM Count field in the TIM element of the transmitted BSSID is 0. The nontransmitted BSSID with BSSID Index 3 also has the DTIM Count field set to 0 and has buffered group addressed BUs. All other nontransmitted BSSIDs have no buffered group addressed BUs. In addition, STAs with AID 12, AID 17, AID 22 and AID 24 have BUs buffered at the AP. FigureO-5 (Partial Virtual Bitmap example #5, Method A or Method B) shows the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM element for this example wheneither Method A or Method B is used. It is noted that Method B reduces to Method A in this example

At 3534.25:

In the third example, there are sixteenBSSIDs and the lowest possible AID that can be assigned to any STA is 16 (n=4, k=15, see 8.4.2.6 (TIM element). There are no group addressed BUs buffered at the AP for the transmitted BSSID, and the DTIM Count field in the TIM element of the transmitted BSSID is 0. The nontransmitted BSSID Index 3 also has the DTIM Count field set to 0 and has group addressed BUs buffered at the AP. All other nontransmitted BSSIDs have no buffered group addressed BUs. In addition, the STA with AID 39 has individually addressed BUs buffered at the AP. Figure O-6 (Partial Virtual Bitmap example #6, Method A) and Figure O-7 (Partial Virtual Bitmap example #6, Method B) show the values of the Bitmap Control and Partial Virtual Bitmap fields that would be part of the TIM element for this example when Method A (N2=4, see 8.4.2.6 (TIM element)) and Method B (N0=2, N1=4, N2=4, see 8.4.2.6 (TIM element)) are used, respectively.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5047 | 1075.28 | 8.4.5.13 |  | "This information is taken from dot11ApCivicLocation". There is no such object. There are 5 references to it. | Reword such references to the name of the table, or the objects within the table. |  | MAC |

Proposed Resolution:

Revised.  Make changes under CID 5047 in <this-document>.

At 1075.28 change “dot11ApCivicLocation” to “dot11APCivicLocationTable”

Proposed Changes:

At 3237.60:

-- dot11APLCITable ::= { dot11imt 3 } -- dot11APCivicLocationTable ::= { dot11imt 4 }

At 3581.01 change:

approximated using the AP’s location information. This parameter includes two type of formats, Geospatial and Civic Location. The following MIB attributes are used: — dot11APLCITable — dot11APCivicLocationTable

| 6314 |  |  |  | The terms "RF chain" and "transceiver" are used but are never defined | Define the terms |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

CID 5049 defines RF chain.

The term “transceiver” is defined in dictionaries (e.g., http://www.oxforddictionaries.com/definition/english/transceiver), and that definition appears to be relevant.

Proposed Resolution:

Revised. Incorporate the change in 11-15/897r2 (https://mentor.ieee.org/802.11/dcn/15/11-15-0897-02-000m-sb-location-related-comment-resolutions-part-2.docx).  This defines the term “RF chain”.

The term “transceiver” doesn’t need special definition. The dictionary definition applies to 802.11.

(Note to editor, this change is also specified for CID 5049).

| 6603 |  |  |  | There seems to be confusion between behavior limits and behavior limits sets. As far as I can tell, the intent is that in a given operating class there is a set of behavior limits, i.e. a behavior limits set, but the text is not always consistent with this interpretation (e.g. 10.9.8.5 heading) | Use "behavior limit" when referring to a specific constraint and "behavior limits set" when referring to all the constraints in a given OC |  | GEN |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

In my opinon:

A behavior limit is an individual constraint of some kind.

A behavior limit set, is the set of all such constraints that apply for the current operating class.

The term “behavior limits”,  could also reasonably imply “behavior limits set”.

If this opinion is correct,  the following changes are necessary:

Proposed Changes:

At 666.37 + matching changes at 666.41, 890.14, :

... includes a value of 13 or 14 in the behavior limits set as specified in Annex E

At 1649.18:

10.9.8.5 HT-greenfield transmissions in operating classes that include a behavior limit of 16

At 1694.18 + matching changes at 1694.21, 1694.24:

… if the Behavior limits set column of the selected row contains the value 13.

At 3331.01:

Behavior limits are listed in Table D-2 (Behavior limits sets). Table D-2—Behavior limits

At 3331.07 change the column heading “Behavior limits set” to “Behavior limit”

At 3349.47:

For Behavior limit GeoDB, the channel starting frequency shall be the frequency that results in the regulatory domain’s channel number being the RLAN channel number.

At 3352.53:

STAs operating under the behavior limit 17 in Table D-2 (Behavior limits sets) are required to be registered with the FCC ULS.

Proposed Resolution:

Revised.  Make changes under CID 6603 in <this-document>.  These changes adjust the use of “set” according to context to clarify that the set is the list of behavior limits,  not a single one of them.