IEEE P802.11
Wireless LANs

| REVmc - some GEN comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document contains some proposed resolutions to SB0 comments.

CID 6759, 6760, 6763

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6759 |  | 8.4.2.68.5 |  | Should Certificate ID really be UTF-8 (as opposed to printable ASCII)? Is that safe for the URL? | Make it printable ASCII |  | GEN |

Discussion

Revised.  The “Certificate ID” itself does not need to be printable ASCII. The standard does not need to describe how this is used to perform a web look-up. At 924.37 edit the clause as noted in document 11-15-1018r1.

Proposed Resolution

At 924.36 change:

The Certificate ID field contains an UTF-8 string indicating an identifier assigned to the STA in a manner outside the scope of the standard. the assigned identifier for the STA. This string is not null terminated. The Certificate ID typically takes the form of “WFA3991” and might be used by a receiving STA to look up the certificate assigned to that ID using a web lookup such as http:// certifications.wi-fi.org/pdf_certificate.php?cid= WFA3991.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6760 |  |  |  | Is the NAI Realm NUL-terminated? More generally, are "strings" NUL-terminated? | Clarify |  | GEN |

Discussion

Clause 8.4.5.10 states “The NAI Realm Encoding Type is a 1-bit subfield. It is set to 0 to indicate that the NAI Realm in the NAI Realm subfield is formatted in accordance with IETF RFC 4282. It is set to 1 to indicate it is a UTF-8 formatted character string that is not formatted in accordance with IETF RFC 4282.”

Therefore the NAI Realm is either formatted according to IETF RFC 4282 or is UTF-8. The standard is silent as to whether it is NUL-terminated (sic).

Regarding the general comment about “strings”, this is an unanswerable question, as “strings” are defined in different ways.

Proposed Resolution:

Reject. The format of the NAI Realm is clearly defined in clause 8.4.5.10 being either IETF RFC 4282 or UTF-8.  Regarding “strings” it is not possible to state whether they are all NUL-terminated (sic) or not, as they have differing definitions within the document, e.g. the Language Code field (1066.48) may be a null terminated string, whereas the Certificate ID field (924.36) is not.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6763 |  |  |  | What is the case of the non-decimal hex characters in the URI formed by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash? The example suggests lowercase, but is this mandatory (if so, where is the shall; if not, then this should at least be NOTEd)? | As it says in the comment |  | GEN |

Discussion:

Clause 8.4.5.16 states “The Emergency Alert URI field is formatted in accordance with IETF RFC 3986”, where RFC 3986 defines a URI.

However, it appears that the citred clause for CID 6763 uses the term “Emergency Alert Server URI” which is a typo.

So, although the resolution to the format is “please see RFC 3986”, there is a typo which needs to be corrected.

Proposed Resolution:

Revised: Please see clause 8.4.5.16 which states that the Emergency Alert URI field is formatted in accordance with IETF RFC 3986.  This field name is used erroneously in clause 10.25.7 and is required to be changed.

Proposed Changes:

(The changes are shown in bold italic.)

At 1786.20 change:

— A URI formed by concatenating the Emergency Alert Server URI with the hexadecimal numerals of the Alert Identifier Hash converted to ASCII encoded characters and the “.xml” file extension. For example, if the Emergency Alert Server URI is http://eas.server.org and the Alert Identifier Hash is 0x1234567890abcdef, then the URI would be http://eas.server.org/1234567890abcdef.xml (the mechanism by which the URI is retrieved is outside the scope of this standard). The XML file is formatted in accordance with OASIS EDXL. The non-AP STA retrieves the Emergency Alert Server URI (see 8.4.5.16 (Emergency Alert URI ANQP-element)) using an ANQP query according to the procedures in 10.25.3.2 (ANQP procedures). This method is recommended for non-AP STAs in the associated state.