IEEE P802.11
Wireless LANs

| REVmc - some MAC comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-08-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document contains some proposed resolutions to SB0 comments.

CID 5395, 6355

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5395 | 1063 | 8.4.5.1 |  | Presumably the ANQP-element has its unique hyphenated name because it has longer ID and Length fields. But there is no reason to limit this longer format to ANQP. At least future-proofing will be helped by having a longer element form available. To do this, create a more generic element name, maybe "longelement". Then "ANQP-element" becomes "ANQP longelement". | Replace "ANQP-element" with "ANQP longelement" throughout the draft. |  | MAC |

Proposed Resolution.

Reject.  The use of the term ANQP-element has nothing to do with its “longer ID”. The commenter needs to explain the use of the term ‘longer’ in more detail. In other words, longer than what? Indeed this format is also used for RLQP in clause 8.4.5

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6355 | 3575 | V.3.3 |  | Table V-2---Example Enterprise DSCP to UP/AC mapping shows UP 2 being AC_BE, but shouldn't it be AC_BK per Table 9-1? | Change the two AC_BEs for UP 2 to AC_BKs |  | MAC |

Proposed Resolution.

Accept.

Proposed Changes:

(The changes are shown in bold italic.)

Table V-2—Example Enterprise DSCP to UP/AC mapping

| Application Class | Per-hop behavior (PHB) | IEEE Std(#130) 802.1D(#3 128)) User Priority | Access Category |
| --- | --- | --- | --- |
| Network Control | CS6 | 7 | AC_VO |
| Telephony | EF | 6 | AC_VO |
| RT Interactive | CS4 | 6 | AC_VO |
| Multimedia Conference | AF4x | 5 | AC_VI |
| Signaling | CS5 | 5 | AC_VI |
| Broadcast Video | CS3 | 4 | AC_VI |
| Multimedia Stream | AF3x | 4 | AC_VI |
| Low Latency Data | AF2x | 3 | AC_BE |
| High Throughput Data | AF1x | 2 | AC_BK |
| OAM | CS2 | 2 | AC_BK |