IEEE P802.11
Wireless LANs

| Channel Models for IEEE 802.11ay |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Alexander Maltsev | Intel | Turgeneva str., 30, Nizhny Novgorod, 603024, Russia | +7-831-2969461 | alexander.maltsev@ intel.com |

Abstract

This document is an amendment to the “Channel Models for 60 GHz WLAN Systems” doc. IEEE 802.11-09/0334r8. It provides an update of the legacy indoor channel models for the conference room, enterprise cubicle and living room environments and defines new channel models for IEEE 802.11ay.

Revision History

r0 – Sept. 2015 – Initial version contains high level description of the proposed channel models to be used in IEEE 802.11ay group.

Table of Contents

1	Introduction	3

2	Channel Model Requirements	3

2.1	TGay Use Cases and Evaluation Scenarios	3

2.2	Channel Model Requirements	4

2.3	Legacy Indoor Channel Models Extension	5

2.4	New TGay Channel Models	5

3	MIMO Extension for Legacy Indoor Channel Models	5

3.1	Conference room	5

3.2	Enterprise cubicle	5

3.3	Home living room	5

4	Channel Model for Open Area Outdoor Hotspot Access	5

4.1	Modelling Scenario	5

4.2	Measurement Results and Ray Tracing Modelling	6

4.3	Model Development Methodology	6

4.4	Channel Model Description	7

4.5	Model of Dynamical Human Blockage and Doppler Effect	9

4.6	Polarization Impact Modelling	9

4.7	Path Loss Model	9

5	Channel Model for Outdoor Street Canyon Hotspot Access	9

5.1	Modelling Scenario	9

5.2	Measurement Results and Ray Tracing Modelling	11

5.3	Model Development Methodology	14

5.4	Channel Model Description	14

5.5	Model of Dynamical Human Blockage and Doppler Effect	18

5.6	Polarization Impact Modelling	18

5.7	Path Loss Model	18

6	Channel Model for Large Hotel Lobby Scenario	19

6.1	Modelling Scenario	19

6.2	Measurement Results and Ray Tracing Modelling	20

6.3	Model Development Methodology	22

6.4	Channel Model Description	22

6.5	Model of Dynamical Human Blockage and Doppler Effect	23

6.6	Polarization Impact Modelling	23

6.7	Path Loss Model	23

7	Channel Model for Street Canyon Backhauling	24

7.1	Modelling Scenario	24

7.2	Measurement Results and Ray Tracing Modelling	24

7.3	Model Development Methodology	24

7.4	Channel Model Description	24

7.5	Polarization Impact Modelling	24

7.6	Path Loss Model	24

8	References	25

# Introduction

The TGay group started development of the new standard enhancing the efficiency and performance of existing IEEE 802.11ad specification providing Wireless Local Area Networks (WLANs) connectivity in 60 GHz band. The 11ay effort aims to significantly increase the data transmission rates defined in IEEE 802.11ad from 7 Gbps up to 30 Gbps which satisfies growing demand in network capacity for new coming applications, [1].

The scope of the considered use cases covers a wide variety of indoor and outdoor applications including high speed wireless docking connectivity, ultra-short range communications, 8K UHD wireless transfer at smart home, augmented reality headsets and high-end wearables, data center inter-rack connectivity, mass-data distribution or video on demand system, mobile offloading and multi-band operation, mobile front-hauling, and wireless backhaul [2], [3].

This document describes the channel models for 60 GHz WLANs systems based on the results of experimental measurements. The goal of the document is to support channel modeling for the mentioned scenarios and assist TGay standardization process. The document provides an extension of the legacy indoor Single Input Single Output (SISO) channel models for the conference room, living room, and enterprise cubicle environments proposed in [4] and implemented in [5] for the case of Multiple Input Multiple Output (MIMO) systems. As well as it describes channel models for new use cases proposed in TGay.

Similar to the document [4] reference antenna models which may be applied in the beamforming algorithm to set up a directional link are considered. These are isotropic antenna, basic steerable directional antenna, Phased Antenna Array (PAA), and large aperture array based on the Modular Antenna Array (MAA) technology.

# Channel Model Requirements

## TGay Use Cases and Evaluation Scenarios

TGay proposes a variety of use cases to be used for performance evaluation of the future IEEE 802.11ay systems, [6]. The summary of the use cases proposed in [2] and supplemented with docking station scenario proposed in [3] is provided in Table 1.

Table 1: Summary of the proposed use cases in TGay.

| # | Applications and Characteristics | Propagation Conditions | Throughput | Topology |
| --- | --- | --- | --- | --- |
| 1 | Ultra Short Range (USR) Communications: -Static,D2D, -Streaming/Downloading | LOS only, Indoor <10cm | ~10Gbps | P2P |
| 2 | 8K UHD Wireless Transfer at Smart Home: -Uncompressed 8K UHD Streaming | Indoor, LOS with small NLOS chance, <5m | >28Gbps | P2P |
| 3 | Augmented Reality and Virtual Reality: -Low Mobility, D2D -3D UHD streaming | Indoor, LOS/NLOS <10m | ~20Gbps | P2P |
| 4 | Data Center NG60 Inter-Rack Connectivity: -Indoor Backhaul with multi-hop* | Indoor, LOS only <10m | ~20Gbps | P2P P2MP |
| 5 | Video/Mass-Data Distribution/Video on Demand System: - Multicast Streaming/Downloading - Dense Hotspots | Indoor, LOS/NLOS <100m | >20Gbps | P2P P2MP |
| 6 | Mobile Wi-Fi Offloading and Multi-Band Operation (low mobility): -Multi-band/-Multi-RAT Hotspot operation | Indoor/Outdoor, LOS/NLOS <100m | >20Gbps | P2P P2MP |
| 7 | Mobile Fronthauling | Outdoor, LOS <200m | ~20Gbps | P2P P2MP |
| 8 | Wireless Backhauling with Single Hop: -Small Cell Backhauling with single hop -Small Cell Backhauling with multi-hop | Outdoor, LOS <1km <150m | ~2 – 20 Gbps | P2P P2MP |
| 9 | Office docking | Indoor LOS/NLOS < 3 m | ~13.2 Gbps | P2P P2MP |

As it follows from Table 1 the proposed use cases differ from each other by the throughput, latency, and topology configuration. Moreover the same use cases can be considered in different propagation environments.

## Channel Model Requirements

## Legacy Indoor Channel Models Extension

## New TGay Channel Models

# MIMO Extension for Legacy Indoor Channel Models

## Conference room

## Enterprise cubicle

## Home living room

# Channel Model for Open Area Outdoor Hotspot Access

The university campus channel model represents the scenario with large open areas with low and rare buildings like university campus, park areas, city squares.

## Modelling Scenario

Figure 4–1: Open Area Outdoor Hotspot Access Scenario

## Measurement Results and Ray Tracing Modelling

## Model Development Methodology

## Channel Model Description

## Model of Dynamical Human Blockage and Doppler Effect

## Polarization Impact Modelling

## Path Loss Model

# Channel Model for Outdoor Street Canyon Hotspot Access

The street canyon (outdoor access ultra-high-rate hot-spots) channel model represents typical urban scenario: city Street with pedestrians’ sidewalks along the tall long buildings. The access link between the APs on the lampposts and the STAs at human hands is modeled in this scenario

## Modelling Scenario

The geometry of the street canyon access scenario that is used for channel model parameters evaluation via ray-tracing simulations is shown in Figure 5–2. The corresponding numerical parameters are summarized in Table 5-2.

Figure 5–2: Street canyon (outdoor access ultra-high-rate hot-spots) scenario

Table 5-2: Street canyon (outdoor access ultra-high-rate hot-spots) scenario parameters

| Parameter | Value |
| --- | --- |
| AP height, Htx | 6 m |
| STA height, Hrx | 1.5m |
| AP distance from nearest wall, Dtx | 4.5 m |
| Sidewalk width | 6 m |
| Road width | 16 m |
| Street length | 100 m |
| AP-AP distance, same side | 100 m |
| AP-AP distance, different sides | 50 m |
| Road and sidewalk material | asphalt |
| Road and sidewalk r | 4+0.2j |
| Road and sidewalk roughness σg (standard deviation) | 0.2 mm |
| Building walls material | concrete |
| Building walls r | 6.25+0.3j |
| Building walls roughness σw (standard deviation) | 0.5 mm |

## Measurement Results and Ray Tracing Modelling

## Model Development Methodology

## Channel Model Description

Figure 5–3: Street canyon (outdoor access ultra-high-rate hot-spots) scenario reflected rays illustration

## Model of Dynamical Human Blockage and Doppler Effect

## Polarization Impact Modelling

## Path Loss Model

# Channel Model for Large Hotel Lobby Scenario

The hotel lobby (indoor access large public area) channel model represents typical indoor scenario: large hall with multiple users within. Similar indoor channel models were considered in the [4], with statistical approach to the channel modeling, suitable for link layer simulations. The proposed here quasi-deterministic approach based on the specified STA location and may be used also for the system level simulations.

## Modelling Scenario

The basic parameters and geometry are summarized in Table 6-3 and illustrated in Figure 6–4:

Table 6-3: Hotel lobby (indoor access large public area) scenario parameters

| Parameter | Value |
| --- | --- |
| AP height, Htx | 3.5 m |
| AP position | Middle of the nearest wall (see Figure 6–4 ) |
| STA height, Hrx | 1.5m |
| Room height | 4 m |
| Room width | 10 m |
| Room length | 15 m |
| Floor material | Concrete |
| Floor rf | 4 + 0.2j |
| Floor roughness σf (standard deviation) | 0.1 mm |
| Walls material | Concrete |
| Walls rw | 4 + 0.2j |
| Walls roughness σw (standard deviation) | 0.2 mm |
| Ceiling material | Plasterboard |
| Ceiling rc | 6.25+0.3j |
| Ceiling roughness σc (standard deviation) | 0.2 mm |

Figure 6–4: Hotel lobby (indoor access large public area) scenario

## Measurement Results and Ray Tracing Modelling

## Model Development Methodology

## Channel Model Description

## Model of Dynamical Human Blockage and Doppler Effect

## Polarization Impact Modelling

## Path Loss Model

# Channel Model for Street Canyon Backhauling

## Modelling Scenario

## Measurement Results and Ray Tracing Modelling

## Model Development Methodology

## Channel Model Description

## Polarization Impact Modelling

## Path Loss Model

# References

- IEEE doc. 802.11-14/0606r0, Next Generation 802.11ad: 30+ Gbps WLAN, C. Cordeiro, et al., May 2014.

- IEEE doc. 802.11-15/0625r2, IEEE 802.11 TGay Use Cases, Rob Sun, et al., May 2015.

- IEEE doc. 802.11-15/0830r0, Docking Usage Model, T. Solomon, July 2015.

- IEEE doc. 802.11-9/0334r8, Channel Models for 60 GHz WLAN Systems, A. Maltsev, et al., May, 2010.

- IEEE doc. 802.11-10/0854r3, Implementation of 60 GHz WLAN Channel Model, R. Maslennikov and A. Lomayev, May 2010.

- IEEE doc. 802.11-15/0866r1, TGay Evaluation Methodology, G. Venkatesan and L. Cariou, July 2015.

- H. Sawada, “Intra-cluster response model and parameter for channel modeling at 60 GHz (Part 3),” IEEE doc. 802.11-10/0112r1, January 2010.