IEEE P802.11
Wireless LANs

| 802.11 ”No Preference” for when the first Burst Instance starts (Relative to Draft 4.2) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission addresses the following:

- Provide a means for an ISTA to indicate in IFTMR that it has no preference for when the first Burst Instance starts.

- Replaced references to FTM Trigger frame with equivalent description of a FTM Request frame.

Editor: Insert the following paragraph as shown:

8.4.2.166 Fine Timing Measurement Parameters element

|  | B0 B1 | B2 B6 | B7 | B8 B11 | B12 B15 | B16 B23 | B24 B39 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Status Indication | Value | Reserved | Number of Bursts Exponent | Burst Duration | Min Delta FTM | Partial TSF Timer |
| Bits | 2 | 5 | 1 | 4 | 4 | 8 | 16 |

|  | B40 | B41 | B42 | B43 B47 | B48 B49 | B50 B55 | B56 B71 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Partial TSF Timer No Preference | ASAP Capable | ASAP | FTMs Per Burst | Reserved | FTM Format and Bandwidth | Burst Period |
| Bits | 1 | 1 | 1 | 5 | 2 | 6 | 16 |

Figure 8-569—Fine Timing Measurement Parameters field format

NOTE TO EDITOR: Insert before Figure 8-571 – Calculation of Partial TSF Timer field.

The Partial TSF Timer No Preference field is reserved when the FTM Parameters element is included in an initial Fine Timing Measurement frame.

The initiating STA indicates a preferred time for the start of the first burst instance by setting the Partial TSF Timer No Preference field to 0 and by setting the Partial TSF Timer field to indicate the preferred start of the first burst instance. Otherwise the Partial TSF Timer No Preference field is set to 1 and the Partial TSF Timer field is reserved. Editor: Change Cl. 8.4.2.166 as shown below:

The ASAP field indicates the initiating STA’s request to start the first burst instance of the FTM session as soon as possible. When the ASAP field is set to 0 and the Partial TSF Timer No Preference field is set to 0 by an initiating STA, the initiating STA requests the start of the first burst instance specified by the Partial TSF Timer field in the initial Fine Timing Measurement Request frame. When the ASAP field is set to 1 and the Partial TSF Timer No Preference field is set to 0 by an initiating STA, the Partial TSF Timer field in the Fine Timing Measurement Request frame indicates the requested start of the first burst instance if the ASAP field is set to 0 in the initial Fine Timing Measurement frame. The ASAP field is also used by the responding STA to signal whether that request has been honored or not. When the ASAP field is set to 0 by the responding STA, the Partial TSF Timer field in the initial Fine Timing Measurement frame indicates the start time of the first burst instance and the earliest time the Fine Timing Measurement RequestFTM Trigger frame (see 10.24.6.4 (Measurement exchange)) should be sent by the initiating STA. When the ASAP field is set to 1 by the responding STA, the Partial TSF Timer field in the initial Fine Timing Measurement frame indicates the start time of the first burst instance and the earliest time the initial Fine Timing Measurement frame will be sent. The responding STA sets the ASAP field to 1 to indicate the STA’s intent to send a Fine Timing Measurement frame as soon as possible.

# Editor: Edit the last paragragh of Cl. 10.24.6.1 as shown below:

#### 10.24.6.1 Overview

The initiating STA in Figure 10-33 (Concurrent FTM sessions) establishes sessions with responding STA 1 and responding STA 2 on different channels. The sessions’ burst instance periodicity might be different as well as the STAs’ clock offsets and thus, over time, some temporal conflicts may occur. To overcome this, during each burst instance the initiating STA indicates its availability by transmitting a Fine Timing Measurement Request frame (see 10.24.6.4 (Measurement exchange)). During each burst instance, the responding STA transmits one or more Fine Timing Measurement frames as negotiated.

# Editor: Delete the second paragraph and change the third paragraph in Cl. 10.24.6.4 as shown below:

#### 10.24.6.4 Measurement exchange

The initiating STA shall transmit a Fine Timing Measurement Request frame without a Measurement Request element, without a Fine Timing Measurement Parameters element and with the Trigger field set to 1  as soon as it is available on channel during a burst instance. This indicates to the responding STA its availability for the remainder of the burst instance. In response, the responding STA shall transmit an ACK frame and should transmit FTMs per Burst Fine Timing Measurement frames before the end of the burst instance. In addition, the initiating STA shall be ready to receive a Fine Timing Measurement frame. These Fine Timing Measurement frames shall not include a Fine Timing Measurement Parameters element (for additional constraints on the Fine Timing Measurement frame see 10.24.6.7 (LCI and Location Civic retrieval using fine timing measurement procedure). Within a burst instance, consecutive Fine Timing Measurement frames shall be spaced at least Min Delta FTM apart. Within a burst instance the initiating STA shall perform fine timing measurement on each Fine Timing Measurement frame addressed to it, except the last Fine Timing Measurement frame. The initiating STA may perform fine timing measurement on the last Fine Timing Measurement frame in a burst instance,

NOTE – If the initiating STA successfully transmits a non-initial Fine Timing Measurement frame  late in a burst instance, fewer than FTMs per Burst might be successfully transmitted by the responding STA in the burst instance. If a Fine Timing Measurement frame, except for the initial Fine Timing Measurement frame in the ASAP=0 case, is sent outside a burst instance, it might not be acknowledged.

# Editor: Change the end-3 and end-2 paragraphs of Cl. 10.24.6.4 as shown below:

NOTE— This value of the Partial TSF Timer field ought to result in the Fine Timing Measurement Request frame not being transmitted before a successful transmission of FTM.

When neither an Ack to the initial Fine Timing Measurement frame nor a Fine Timing Measurement Request frame has been received by the responding STA, the responding STA shall not terminate the FTM session before the time indicated by the Partial TSF timer plus the Burst Duration.