IEEE P802.11
Wireless LANs

| 11mc MAC operation comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-10-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Dorothy Stanley | HP (Aruba Networks) | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the proposed resolutions to some “MAC Operation” category MAC CIDs: 6204, 6207, 6213, 6217, 6233

6437, 6439, 6440, 6444, 6445,

6454, 6498

R1: Resolutions as agreed in Cambridge 2015-10-15 AM1. Agreed to proposed resolutions to all except CID 6233 (Submission required, assign to commenter).

CID 6204- MAC

| 6204 | 1383.19 | 9.26.2 |  |  | "If a protection mechanism is being used, a fragment sequence shall use ERP-OFDM modulation for the final fragment and control response." -- why? | Delete the cited sentence |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is in clause 9.26.2 Protection mechanism for non-ERP receivers (next page, line 19):

The commenter’s proposed change is to delete the cited sentence.

A prior submission 11-04-0886r0 (page 24) proposed deleting both the cited and following sentences.

The proposed deletion was not done.

Cambridge discussion: agree that text can be deleted; serves no purpose.

Action: DS to notify reflector.

Proposed resolution: Accepted

CID 6207 - MAC

| 6207 | 1382.54 | 9.26.2 |  |  | It says "Protection mechanisms frames shall be sent using one of the mandatory Clause 16 or Clause 17 rates and using one of the mandatory Clause 16 or Clause 17 waveforms" -- well, which is it? The mandatory rates in HR/DSSS are not the same as those in DSSS. Ditto Table 9-11---Protection requirements for HT Protection field values nonmember protection mode and non-HT mixed mode | Pick one of Clause 16 and Clause 17 only |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below, sentence strating at line 54:

And Table 9-11

Discussion:

Table 9-11 is in the “9.26.3 Protection mechanisms for transmissions of HT PPDUs “ clause, and is repeating the “Use Protection” definition.

The cited sentence (simplifying clause titles for readability) is

Protection mechanisms frames shall be sent using

one of the mandatory Clause 16 (DSSS PHY for the 2.4 GHz band)

or

Clause 17 ( (HR/DSSS) PHY) rates

and

using one of the mandatory Clause 16 (DSSS PHY for the 2.4 GHz band)

or

Clause 17 ( (HR/DSSS) PHY)

waveforms, so all STAs in the BSA are able to learn the duration of the exchange even if they cannot detect the

ERP-OFDM signals using their CCA function.

Proposed resolution: Revised

The requirement is indicated at 1383.26 (9.26.2, which is referenced in the table) that either can be used based on the rates in the BSSBasicRateSet parameter of the protection mechanism frame. The indicated text in Table 9-11 is duplicative.

The text at 1382.54 is retained, as it covers the case when protection is required but no clause 16 or 17 rate is in the BSSBasicRateSet parameter.

At 1386.31 delete “The frames that are used for providing the protection shall be sent

at a Clause 16 (DSSS PHY specification for the 2.4 GHz band

designated for ISM applications) or Clause 17 (High rate direct

sequence spread spectrum (HR/DSSS) PHY specification) rate.”

CIDs 6213 (MAC)

| 6213 | 1285.45 | 9.5 |  |  | It says "the number of octets in the fragment (before processing by the security mechanism) shall be determined by dot11FragmentationThreshold" but that's just an upper limit; as other parts of the spec make clear (e.g. the next para!), a fragment (even a non-final one) might be smaller | Change "determined" to "limited" |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below

The commenter’s proposed change is:

When data are to be transmitted, the number of octets in the fragment (before processing by the security mechanism) shall be limited by dot11FragmentationThreshold and the number of octets in the MPDU that have yet to be assigned to a fragment at the instant the fragment is constructed for the first time.

Discussion:

The current “determined” is correct and imples a limit, albeit not as specific as “limited”.

Preposed Resolution: Accepted

CID 6217 (MAC)

| 6217 | 1310.24 | 9.12 |  |  | The max A-MSDU size should apply when an A-MSDU is sent in a non-HT PPDU too (as Table 8-19 indicates) | Delete "in an HT PPDU" in "A STA shall not transmit an A-MSDU in an HT PPDU to a STA if the A-MSDU length exceeds the value indicated by the Maximum A-MSDU Length field of the HT Capabilities element received from the recipient STA."; also delete "received in an HT PPDU" at 2921.61 |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below:

And at 2921.61:

The commenter’s proposed change is:

A STA shall not transmit an A-MSDU to a STA if the A-MSDU length exceeds the value

indicated by the Maximum A-MSDU Length field of the HT Capabilities element received from the recipient

STA.

Discussion:

Agree that Table 8-19 allows this.

Proposed resolution: Accepted

CID 6233 (MAC)

| 6233 | 1276.29 | 9.3.7 |  |  | Table 9-5 needs to be extended for PHYs after 11n | Add info on VHT (and DMG?) PPDUs |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below:

Changes are not specified by the commenter

Proposed resolution: Rejected

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

Submission required; assign to commenter.

CID 6437

| 6437 | 1268.28 | 9.3.4.3 |  |  | What does "CWindow" indicate in Figure 9-15, exactly? It's not CW, since it seems to be the same for all the STAs. Is it CWmin? | Delete "CWindow" from Figure 9-15, including the key |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is shown below:

Discussion:

CWindow indicates “Contention Window” as shown in the definition in the figure.

The standard defines “CW” for contention window (at 51.64). The text following the figure refers to “CW”, at 1268.58. The value of the contention window is likely (re)set to aCWmin (per 1266.55) but that level of detail is not shown in this figure. Suggest aligning the terminology.

Proposed resolution: Rejected

Cwindow is defined in the figure, “Contention Window”. The figure is an illustrative example, and
does not show the case with widely varying values of the contention window.

CID 6439 (MAC)

| 6439 | 1237.00 | 9 |  |  | Figures 9-4, 9-10, 9-14 say or at least suggest there is immediate access when the medium is idle for DIFS or AIFS. This is only the case if the CW is zero (see e.g. 1268.50) | Add "and CW is zero" in the identified figures |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below for Figure 9-4.

Discussion:

The contention window is drawn with “slash” marks indicating a range of possible values.

See 1267.19, where there is no mention of contention window.

“In general, a STA may transmit a pending MPDU when it is operating under the DCF access method, either in

the absence of a PC, or in the CP of the PCF access method, when the STA determines that the medium is idle

for greater than or equal to a DIFS, or an EIFS if the immediately preceding medium-busy event was caused by

detection of a frame that was not received at this STA with a correct MAC FCS value.”

The cited text for Figures 9-10 and 9-14 are shown above and below.

Proposed resolution: Rejected.

The figures are correct in stating that there is immediate access after DIFS/AIFSN. This applies when an MSDU arrives for transmission and no backoff is currently in operation.

See 1267.19, where there is no mention of contention window.

“In general, a STA may transmit a pending MPDU when it is operating under the DCF access method, either in

the absence of a PC, or in the CP of the PCF access method, when the STA determines that the medium is idle

for greater than or equal to a DIFS, or an EIFS if the immediately preceding medium-busy event was caused by

detection of a frame that was not received at this STA with a correct MAC FCS value.”

CIDs 6440 (MAC)

| 6440 | 1237.00 | 9 |  |  | The second para of 9.3.4.2 and item c) of 9.22.2.2 suggest the use of CW/backoff depends on whether you received the MSDU from the SAP during medium busy or not. This seems like a recipe for people not doing backoff ("I received the MSDU when the medium was idle, honest!") | Always require backoff, irrespective of whether the medium was busy when the MSDU was received at the SAP |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is shown below for the locations cited in the comment (initial page and line number are not correct):

Discussion:

It’s not clear where the “suggest the use of CW/backoff depends on whether you received the MSDU from the SAP during medium busy or not” is. MSDUs are not mentioned n the text.

Proposed Resolution: Rejected.

The text refers to “pending MPDUs”, not the SAP interface. Each TXOP is preceeded by a backoff but note that the backoff may have occurred after a prior transmission. If additional MSDUs arrive for transmission and the medium has been idle for DIFS/AIFSN at that time, then the transmission proceeds without further delay.

CID 6444 (MAC)

| 6444 | 1287.13 | 9.7 |  |  | The multirate rules are an impenetrable mess. It's impossible to determine whether they are complete, let alone whether they are correct | Rewrite as a flowchart or table, so that it can be seen that the rules are complete and correct |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Not enough detail in the proposed resolution.

Proposed resolution: Rejected

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

CIDs 6445 (MAC)

| 6445 | 1298.33 | 9.7.6.5.4 |  |  | There is no such thing as an alternate rate. By definition, an alternate rate or MCS will cause the duration to differ | Delete the referenced subclause |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is shown below:

Discussion:

This text was added in 802.11n, and revisited in 11ac with no changes made (11-11-1439), implying there is a use for this.

Discuss proposed resolution in TGmc

Proposed resolution: Rejected

Alternate rates that differ slightly may result in a PPDU of the same duration given that durations are quantized to symbol duration. MCSs may vary, for example varying the number of space time streams without changing duration.
CID 6454 (MAC)

| 6454 | 1272.13 | 9.3.4.4 |  |  | It says "The AP shall attempt to deliver one MSDU to the STA that transmitted the PS-Poll" -- but an A-MSDU (typically consisting of more than one MSDU) can also be delivered in response to a PS-Poll | Update this subclause (and any other subclause which is similarly erroneous) to allow an A-MSDU in response to a PS-Poll. See also the ad-hoc notes for CID 201: "In fact, this paragraph also several times uses the phrase "data frame" when it should say BU. Scrub this entire paragraph, and make it align with clause 10.2." |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is below:

Discussion:

Insufficient detail in the proposed resolution

Proposed resolution: Rejected

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

CID 6498 (MAC)

| 6498 | 1286.54 | 9.6 |  |  | "A STA shall support the concurrent reception of fragments of at least three MSDUs or MMPDUs." -- frankly, this is not good enough nowadays, even for non-AP STAs (consider QoS, for example) | Add "A STA should support the concurrent reception of fragments of at least one MSDU per access category. An AP should support the concurrent reception of at least on MSDU per access category per associated STA." |
| --- | --- | --- | --- | --- | --- | --- |

The cited text is shown below:

Discussion:

The proposed change seems reasonable, and is a should, allowing existing devices to remain compliant.

Proposed resolution: Rejected

The use of fragmentation is expected to decline, as 11n and 11ac support additiona aggregation.

Do not see the need to add requirements, even if “should”.

References:

https://mentor.ieee.org/802.11/dcn/15/11-15-0532-17-000m-revmc-sponsor-ballot-comments.xls