IEEE P802.11
Wireless LANs

| 802.11 REVmc Initial Sponsor ballot - some proposed comments resolutions (Stephens) – Part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-10-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains some proposed resolutions to initial sponsor ballot comments.

R1:  additional comments added.

# Comments

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc | Comment Group |
| --- | --- | --- | --- | --- | --- | --- |
| 6223 |  |  | Should there be a minimum fragment size? dot11FragmentationThreshold only specifies what will be fragmented, not what might be fragmented. It might be necessary to fragment e.g. to meet a TXOP Limit constraint | Add a dot11FragmentationLimit | MAC |  |

Proposed Resolution:

Rejected.

The comment does not adequately demonstrate a need for such a change.

| 6227 |  |  | What is a "mesh TKSA"? | Change to "mesh PTKSA" throughout | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:  (wording initially from Jouni)

Rejected.

A Mesh TKSA is the security association that does not contain a PTK and as such, is not called a PTKSA. It contains a mesh TK and consequently, is called mesh TKSA.

| 6234 |  |  | Basic rates/MCSes are by definition a property of the BSS, so "BSS" is superfluous | Delete "BSS" (and fix subsequent capitalisation, where necessary) at 1607.64, 1606.65, 1247.22, 1288.57, 1289.50, 1289.52, 1289.57, 1291.26, 1291.29, 1293.27, 1294.49, 1296.53, 1846.20, 1846.22, 1846.26, 1846.32 | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

While accepting that the Basic Rate is a necessarily a property of a BSS, to include “BSS” in the name of the basic rate related fields and parameters is not incorrect.

| 6236 |  |  | What happens if dot11SupportedDataRatesRxTable does not match the OperationalRateSet passed over the MLME SAP? Ditto TxTable and BasicRateSet | Remove either the SAP parameters or the MIB variables | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I have some sympathy with the sentiment of the comment, but none for wasting any more of my time fixing an unused MIB.

I presume the current model, if we can dignify the co-evolution of the MIB and MLME with that term, is that the PHY tells the SME what it can do using the MIB variable, and then the SME tells the MAC what to use/advertise, which might be the same,  might be a subset and must not be a superset.  How the SME decides to do this is unspecified.

Propose Resolution:

Rejected.

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

| 6245 |  |  | It's not clear which non-Action frames are robust Management frames (the implication is that only those with MME are, i.e. Disassoc and Deauth). | State explicitly in Clause 11 that Disassoc and Deauth are the only robust Management frames that are not Action frames | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

105.05 suffices: “The robust Management frames are Disassociation, Deauthentication, and robust Action frames.”

| 6290 |  |  | The AID is not 1-2007 for DMG STAs | Change the text at 173.20, 180.31, 187.26, 1940.30, to say the limit is 2007 for non-DMG STAs and 254 for DMG STAs. Delete "The value of the AID is in the range 1--2007." at 568.61, 569.29, 720.58, 721.25, 722.5, 722.17, 3535.35 (!), 3539.10, 3539.46, | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The changes proposed below are conservative.  The indicated change “. Delete "The value of the AID is in the range 1--2007."” is incorrect because most of the listed locations don’t contain that text,  but the value 2007.

Additional changes could be made to the description of the TIM bitmap and encoding process to indicate a lower AID limit (and therefore bitmap size) for DMG.   I have not proposed to do this,  because in my mind it is self evident that if the maximum AID is smaller,  the bitmap is also constrained.

Proposed Resolution:

Revised.

At 173.20, 180.31, 187.26, 1940.30

change “1—2007 (inclusive)” to “Non-DMG: 1—2007 <newline>DMG: 1—254”

Delete "The value of the AID is in the range 1--2007." at 568.61

| 6350 |  |  | Does reassociation apply to PCPs? | Change the text where it indicates reassociation might apply to a PCP | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Carlos provided the following resolution:

Proposed Resolution.

Rejected.

Re-association to the same PCP enables a STA to change its capabilities/operational parameters while retaining its association. In addition, inclusion of this mechanism for DMG STAs maintains a consistent implementation across PBSS and infrastructure BSS in DMG and between DMG and non-DMG.

| 6357 |  |  | What is the difference, if any, between "neighbour peer mesh STA", "neighbour mesh STA" and "peer mesh STA"? | Clarify. If "peer" means some kind of peering agreement and "neighbour" means some kind of physical proximity, make sure the terms are all always used correctly | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Guido writes:

• A neighbor mesh STA is a mesh STA that is in communication range. A neighbor mesh STA may be a mesh STA of the same MBSS or of a different MBSS. Therefore, the set of neighbor mesh STAs contains all mesh STAs that a mesh STA shares the wireless medium with. E.g., neighbor mesh STAs can mutually set the NAV. • A neighbor peer mesh STA is a neighbor mesh STA that a mesh STA has peered with. All peer mesh STAs belong to the same MBSS. Once two mesh STAs have peered and as long as they are in mutual communication range they can exchange MSDUs. • A peer mesh STA is a mesh STA that has peered with. Once two mesh STAs have an active peering the mesh STAs can exchange frames as long as they are also neighbor mesh STAs. If mesh STA's neighbor peer mesh STA "walks" away it becomes a peer mesh STA. The peering is still active but the two mesh STAs cannot exchange MSDU anymore as they are no longer in range over a single instance of the wireless medium. Once a peer mesh STA walks back in range it immediately becomes a neighbor peer mesh STA and communication can be established again. Being a peer mesh STA means that credentials etc. have already been exchanged. I believe the current text is fine, because: • The term "neighbor STA" explains that "A STA that is in direct communication range over a single instance of the wireless medium." This is the physical proximity. • The term "peer mesh STA" explains the "agreement" that the commenter asks for. The standard reads "A mesh STA to which a mesh peering has been established." • A neighbor peer mesh STA is the union of "peer mesh STA" and "neighbor STA." A neighbor mesh STA is a mesh STA that fulfills the conditions of being a neighbor STA and being a peer mesh STA.

Proposed Resolution:

Rejected.

The comment fails to identify a specific issue with the balloted draft. The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

In reply to the commenter,

• The term "neighbor STA" explains that "A STA that is in direct communication range over a single instance of the wireless medium." This is the physical proximity.

• The term "peer mesh STA" explains the "agreement" that the commenter asks for. The standard reads "A mesh STA to which a mesh peering has been established."

• A neighbor peer mesh STA is the union of "peer mesh STA" and "neighbor STA." A neighbor mesh STA is a mesh STA that fulfills the conditions of being a neighbor STA and being a peer mesh STA.

| 6424 |  |  | The concept of a "successful transmission" (or "transmitting successfully", etc.) is poorly defined. There are some hints in some places, but they are either restricted in scope (e.g. 9.2.2/9.3.4.3 for DCF, 9.22.2.2 only for that subclause) or incomplete (e.g. 9.2.2/9.3.4.3 for DCF only referring to Acks) | Define the concept of a successful transmisison in one place (e.g. either putting a frame on the air which does not require any response after SIFS, or getting the right response after SIFS) | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

1238.55 (9.2.2 DCF) states: “A transmission is successful either when an Ack frame is received from the STA addressed by the RA field of the transmitted frame or when a frame with a group address in the RA field is transmitted completely.”

There are 21 references to *successful transmission, of which the majority are in Clause 10.

As some of these relate to Data frames, the question arises as to whether any other acknowledgement mechanism can be used in these cases (e.g.  Block Ack), if so 9.2.2 is arguably incomplete.

Proposed Resolution:

Revised.

At 1238.55 delete: “A transmission is successful either when an Ack frame is received from the STA addressed by the RA field of the transmitted frame or when a frame with a group address in the RA field is transmitted completely.”

In subclause 3.2 insert (in alphabetic order)

“successful transmission: A transmission and the reception of its expected acknowledgement or a transmission for which no acknowledgement is expected.”

| 6430 |  |  | The material related to DMG has a lot of "TXTIME"s without an indication of the PHY parameters (especially the MCS) to be used to determine this | Add an indication of the PHY parameters (especially the MCS) to be assumed when TXTIME(frame) is mentioned in the context of DMG (and anything else which might suffer from the same problem) | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Please assign to Carlos Cordeiro.

| 6432 |  |  | The spec often talks of doing things based on the "most recently received" MPDU or MMDU. However, this is not always correct. For instance, if a reassociation was attempted but failed, then the parameters remain those in the (Re)Association Response which led to the current BSS participation, not those in the Reassociation Respon`se which signalled failure. Also, what if a response (or even request) which was in some way invalid was "most recently received" -- does that count or not? Also, if it's a capability, it should be static. Examples: 664.57, 1759.47, 1788.5 | Check the use of each of the 114 instances of "recently" in the spec and correct those which aren't strictly correct | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

We have previous removed “most recently received” related to capabilities, on the assumption that these are static.  However, capabilities might change between association attempts, so references to capabilities in (re)association arguably benefit from this language.

There are 164 instances of “most recent”. I reviewed them all and propose that the following changes be made:

At 542.21:  Comment:  This section is pretty value-free.  No change proposed.

At 1759.57, 1761.53, 1763.07, we see this kind of language: “Extended Capabilities element in their most recently received mesh Beacon frame” related to peer mesh STAs.

I think a peer mesh STA must hold these capabilities constant while it has a peering relationship, but I don’t see anything to prevent it changing when it has no pairing relationship.  So I think the correct statement is “any mesh Beacon frame received from the peer mesh STA during it latest mesh peering”.  Given that that is longer,  and more complicated,  I don’t feel any incentive to change it.

Proposed changes:

At 664.55:

A TSPEC as described in 10.2.2.5 (Power management with APSD) is to be used to make a particular AC exclusively either trigger-enabled or delivery-enabled. These subfields are set to 0 when the APSD subfield in the Capability Information field received from the AP with which the non-AP STA is associating is equal to 0.

At 1791.24:

An associated non-AP QMF STA may transmit a QMF Policy Change to the QMF AP in its BSS only if the Extended Capabilities element received from the AP has its QMFReconfigurationActivated subfield equal to 1.

Proposed Resolution:

Revised.

Make changes under CID 6432 in <this-document>.  These changes were a result of a survey of uses of “most recent”, which identified two incorrect uses.

| 6443 |  |  | Is a frame which was not "correctly received" a frame at all? We got rid of the miscreants under CID 152 but a few new ones have cropped up | Deal with the miscreants as they were dealt with under CID 152 | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Changes:

At 1620.34:

In a DMG BSS and in the case of a TS that is established between non-AP STAs (PTP TSPEC), the timeout of the recipient is based on the arrival of MSDUs that belong to the TS within the MAC after any decryption, A-MSDU unpacking, and reassembly.

At 3170.21:

It is written by the MAC when an MSDU is received that is addressed to the STA (either individually or globally). This counter is incremented by the number of octets in the MSDU.

Proposed Resolution:

Revised.  Make changes under CID 6443 in <this-document>. These change remove any unnecessary “correctly received”.

| 6457 |  |  | The GCM nonce is missing the priority | Add a priority, as for CCM | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Jouni provided the following resolution:

Proposed Resolution:

Rejected.

Frame priority is protected by the GCMP AAD construction and the GCMP nonce construction ensures uniqueness of the nonce by including the A2 field and PN which the transmitter is required to manage as a single counter for all frames using the same key. The comment did not identify any technical reason for the proposed change and the proposed change would result in the existing implementations being incompatible with the new design.

| 6460 |  |  | What exactly does "mandatory" mean in the context of rates(/MCSs/preambles/etc.) in PHYs? If a rate is "mandatory", does that mean it has to be included in the operational rate set? | Add a "NOTE---A STA is not required to include mandatory rates in its operational rate set." | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

There is nothing that excuses a STA from including mandatory rates in an operational rate set.

So, the proposed NOTE creates a conflict.  Or, if taken to be correct, might lead an implementation into making a technical change.

Proposed Resolution:

Rejected.   The comment does not indicate a problem to solve.  The proposed interpretation might create possible interoperability issues with STAs that understood and depended on the current rule.

| 6475 |  |  | If in a U-APSD SP an AP ends the SP part-way through a fragmented MSDU/MMPDU, what happens at the next SP? Does the AP start from the beginning? Does the part-BU count as one or zero (if the Max SP Length was not indeterminate)? | Clarify (see CID 1482) | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

The comment asks questions, but does not identify an issue.

The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

| 6483 |  |  | The term "supported rate set" is undefined | Change throughout to "operational rate set", except at 1277.47, 1383.45, 1384.1 (change to "basic rate set") and maybe 892.52, 1383.50, 1384.5 (not sure what is intended there) | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The AP broadcasts its supported rates in the “Supported Rates and BSS Membership Selectors” element combined with the “Extended Supported Rates and BSS Membership Selectors”.  It indicates which rates are “basic” by setting the top bit of a rate.

The MLME-START.request supplies these values in a combination of the BSSBasicRateSet, OperationalRateSet and BSSMembershipSelectorSet parameters.

A non-AP uses the same elements to indicate its supported rates.

The MLME-JOIN.request specifies an OperationalRateSet parameter, but not a BSS Membership Selector.   The MLME-ASSOCIATE.request contains no rate information.

There is no definition of “operational rate set”, but it presumably relates to one or more of the parameters cited above.  Neither is there a formal definition of “basic rate set”,  even though that term is frequently used.  I shall assume it’s OK to use “basic rate set”.

Basically we have two choices:  which of “operational” or “basic” was intended, and how to convey the definition of “operational” (as the latter is less firmly entrenched).

Proposed Changes:

At 648.11:

NOTE—No subfield is supplied for ERP as a STA supports ERP operation if it includes all of the Clause 19 (Extended Rate PHY (ERP) specification) mandatory rates in its operational rate set (determined by the OperationalRateSet parameter of the MLME-START.request or MLME-JOIN.request primitive based on which it started or joined a BSS, as appropriate).

At 892.53:

A Management frame (excluding a Probe Request) is received that contains a Supported Rates and BSS Membership Selectors element and optionally an Extended Supported Rates and BSS Membership Selectors element where the supported rate set indicated by these elements includes only Clause 16 (DSSS PHY specification for the 2.4 GHz band designated for ISM applications), Clause 18 (Orthogonal frequency division multiplexing (OFDM) PHY specification), Clause 17 (High rate direct sequence spread spectrum (HR/DSSS) PHY specification), and Clause 19 (Extended Rate PHY (ERP) specification) rates

At 1277.47:

The characteristic rate set is equal to the IBSS’s basic rate set when the STA is operating as a member of an IBSS. It is equal to the AP’s operational rate set (indicated by the AP’s Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) when the STA is associated with an AP.

At 1291.21: + same change at 1293.05 and 1294.32

When the operational rate set of the receiving STA or STAs (indicated by its or their Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) is not known, the transmitting STA shall transmit using a rate in the BSSBasicRateSet parameter, or anMCS in the Basic MCS Set field of the HT Operation parameter of the MLME-START.request primitive or Basic MCS Set field of the HT Operation parameter of the SelectedBSS parameter of the MLME-JOIN.request primitive, or a <VHT-MCS, NSS> tuple in the BSS basic VHT-MCS and NSS set, or a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet, the Basic MCS Set field of the HT Operation parameter of the MLME-START.request primitive or Basic MCS Set field ofthe HT Operation parameter of the SelectedBSS parameter of the MLME-JOIN.request primitive, and the BSS basic VHT-MCS and NSS set are empty.

At 1383.45:

b) In an IBSS if a Beacon frame isreceived from one of the IBSS participants where the operational rate set (indicated by the Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) contains only Clause 16 (DSSS PHY specification for the 2.4GHz band designated for ISM applications) or Clause 17 (High rate direct sequence spread spectrum (HR/DSSS) PHY specification) rates. c) A Management frame (excluding a Probe Request) is received where the operational rate set (indicated by the Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) includes only Clause 16 (DSSS PHY specification for the 2.4 GHz band designated for ISM applications) or Clause 17 (High rate direct sequence spreadspectrum (HR/DSSS) PHY specification) rates.

At 1384.01:

— A Beacon frame is received from a neighbor STA where the operational rate set (indicated by the Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) contains only Clause 16 (DSSS PHY specification for the 2.4 GHzband designated for ISM applications) or Clause 17 (High rate direct sequence spread spectrum (HR/DSSS) PHY specification) rates, or — A Management frame (excluding Probe Request) isreceived where the operational rate set (indicated by the Supported Rates and BSS Membership Selectors, and, if present, Extended Supported Rates and BSS Membership Selectors elements) includes only Clause 16 (DSSS PHY specification for the 2.4 GHz band designated for ISM applications) or Clause 17 (High rate direct sequence spreadspectrum (HR/DSSS) PHY specification) rates

Proposed resolution:

Revised.  Make changes under CID 6483 in <this-document>.  These changes essentially replace instances of “supported rate set” with either basic or operational rate set.

| 6484 |  |  | Sometimes the spec refers to Association Request/Response but fails to cover Reassociation Request/Response | Add "(Re)" where appropriate | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I have no sympathy for this comment.  The commenter clearly uncovered an instance where this was the case, but couldn’t be bothered to enter it into the comment.

Not surprisingly, I can’t be bothered to do this work either.

Needs submission

Assign to commenter

| 6485 |  |  | Introduce the term PPDU Transmission Options to encompass all the things which can be selected when transmitting a PPDU (which is essentially the things which appear in the SIG(NAL) field, though there are a few others, e.g. preamble length) | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Possibly useful.  Needs submission.

Assign to commenter

| 6488 |  |  | Non-HT BA is obsolete, as is HT-delayed BA. Their presence needlessly complicates the spec | Mark them all as so (or deprecated -- I can't remember which one you're supposed to use for the black spot) | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I agree with the sentiment. Implementers generally started implementing Block Ack with .11n.

We need to determine usage of the term.  I’ll put out an email to the 802.11 reflector.

Status: pending email poll response

| 6536 |  |  | There are a bunch of "*BSS network"s, which seems pleonastic (about 20 instances) | Delete the "network"s | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Agree with the sentiment.  “Pleonastic” is going to be my mot du jour.

We have plenty of *BSS without an accompanying network, so the term is generally regarded as sufficient of itself, and a noun.

There is also ESS network.  I think the same argument applies to it.

Proposed Resolution:

Revised.  Make changes under CID 6536 in <this-document>. These changes remove “network” as a qualifier to *BSS and ESS.

Proposed changes:

At 66.60 change “One (or more) IBSS or ESS networks” to “One (or more) IBSSs or ESSs”

At 102.14 change “both ESS and IBSS networks” to “both ESSs and IBSSs”

At 928.45:

Except in Location Track Notification frames sent in an IBSS, the Indication Multicast Address field specifies the destination address to which the Location Track Notification frames are sent.

At 1730.46:

1) A non-IBSS STA shall transmit the Location Track Notification frames to the Indication Multicast Address field in the Location Indication Parameters subelement configured by the Location Configuration Request frame. 2) … 3) An IBSS STA shall transmit the Location Track Notification frames to the destination address of the STA that configuredthe STA using Location Configuration Request frames.

At 2938.55:

"This attribute is the destination address to which the Location Track Notification frames are sent, excluding in an IBSS;

Globally change “SS network” to “SS”.

(Note to editor, the global change will also change some “BSS networks” to “BSSs”.  This is intended.)

| 6543 |  |  | References to "Clause frames" (n = 16, 17, etc.) make no sense as frames are a MAC concept. "rates" is suspect too because a given rate may be used by more than one PHY (e.g. 11g and 11a, and probably some variants of 11a and 11n). Other forms like "Clause waveforms" or "PPDUs" or "formats" make sense but should be consistent | Pick one valid term and use it consistently | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I agree with the sentiment.  Other comments have questioned whether a Clause 16 frame can be transmitted by a Clause 20 PHY etc…

But, this is probably a lot of work, and I’m not volunteering for it.

Assign to commenter, submisison required.

| 6570 |  |  | It's a bad idea to say which things X is present in in more than one place, as this is almost guaranteed to become out-of-date | Remove all but a single normative instance of things like "Present in Beacon, Probe Response, Mesh Peering Open and Mesh Peering Confirm frames." Search for "present" and "is used in" to achieve this. See ad-hoc notes for CID 141 for examples | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

This is permission to do a *lot* of work.

Submission required.

Assign to commenter.

| 6572 |  |  | The distinctions made in the specification w.r.t. TS/TC/TSID/TID are incomprehensible | Make the definitions comprehensible. E.g. what does "UP for either TC or TS" mean? | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Well, clearly the distinctions have been comprehended by current implementers, so the commenter’s assertion is demonstrably untrue.

Submission required.

Assign to commenter.

| 6589 |  |  | Colons in MAC addresses and suchlike imply bit-reversed notation, which is definitely Not The Done Thing anymore | Change the colons to hyphens, or put an explicit note that they are not meant to imply bit-reversed notation | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I really don’t want to interpose myself in a debate where some folks take highly polarized positions, and I’m myself really don’t care.  I don’t remember the conventions, and need to look them up every time I need to understand them.

But we might find out whether the commenter should bring a proposal by a straw poll.

Straw poll:   Change colons to hyphens in MAC addresses, and make any necessary bit reversals.

Yes

No

Abstain

| 6617 |  |  | Do we really have to say "Multiple Vendor Specific subelements are optionally present in the list of optional subelements." a million times? | Say it once at most | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The proposed resolution is written assuming a rejection.

We might do better than the rejection below.

Straw poll:  Do you agree with this statement: “If an element supports subelements, it should by convention support a Vendor Specific subelement, and the numbering of this subelement should be the same for all elements” ?

Yes

No

Abstain

Proposed Resolution:

Rejected.

An Action frame can always carry Vendor Specific elements, because this is part of the defined structure of the Action frame.  But an element doesn’t have a common “payload” format.  Some elements contain subelements.  Some do not.  Each element gets to choose independently whether to contain subelements, and whether a Vendor Specific subelement is supported, and even what Subelement ID should be used if a Vendor Specific subelement is supported.

| 6624 |  |  | When do we say "Address 1/2" and when do we say "TA/RA" (see e.g. 9.19.2.2)? There doesn't appear to be any rhyme or reason to the choice | Be consistent | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Submission required.

Assign to commenter.

| 6671 |  |  | The last bullet point of NOTE 2 in Table 8-173--HT Operation element fields and subfields seems useless (OBSS is covered by the second bullet and associated STAs are covered by the first) and dangerous (what, any old Management frame?!). Maybe ditto 9.26.2 "A Management frame (excluding a Probe Request) is received where...". [these may be references to D3.0] | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution.

Rejected.   This comment is out of scope, as it is not on the balloted draft (i.e., Table 8-173 is not HT Operation element fields).

| 6706 |  |  | Sometimes spec says "Supported Rates element" but should also say "Extended Supported Rates element". | Add "and Extended Supported Rates element" wherever missing | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I reviewed uses of “Supported Rates element” and found none that lacked the “Extended Supported Rates element”.

This is because the text tends to refer to “basic rate set” and “operational rate set” (fixed by CID 6483), which are implicitly references to these elements.

Proposed Resolution:

Rejected.  There are no instances of “Supported Rates element” that need an additional “Extended Supported Rates element”

| 6708 |  |  | DMG mixes up the operational rate set and the basic rate set (and it's not really the rate it's the MCS). | Unmix DMG | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I agree DMG mixed this all up.  Unmixing it will take a lot of work.  I don’t expect assigning this to any of the .11ad folks will result in any changes.

Submission required.

Assign to commenter.

| 6710 |  |  | "In an A-MSDU, the Mesh Control field is located in the A-MSDU Subframe Header (see Figure 8-55 (A-MSDU subframe structure for Mesh Data)). In an MMPDU, the Mesh Control field is located within the MMPDU (see 8.6.18 (Multihop Action frame details)). Such Mesh Control fields need to be taken into account if a maximum A-MSDU or MMPDU size constraint applies" -- why does the MCf needs to be taken into account if a maximum A-MSDU size constraint applies, since it's not within the A-MSDU? | Delete "A-MSDU or" in the cited text | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is on 586.57.

In the case of the pre-HT and HT PPDU formats, the maximum A-MSDU is constrained, but at a value much larger than the maximum MSDU size, which also constrains the size of the payload. So the A-MSDU constraint has no effect on the payload size.

In the case of VHT, there is no A-MSDU constraint.

In the case of DMG, there would be a constraint,  but DMG STAs don’t support mesh,  so the cited text doesn’t apply.

Proposed Resolution:

Accepted.

| 6765 |  |  | One instance of "AES-GCMP" (all others are "AES-GCM"). | Change the errant one to "AES-GCM" | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I fail to find this string in the draft.

Proposed Resolution:

Rejected.

The cited term is not present in the balloted draft.

| 6768 |  |  | Constructs like "AP's BSSID" are dubious because the BSSID is a property of the BSS, not the AP (even though the BSSID is the MAC address of the AP). Especially with Multiple BSSID support | Change to "AP's MAC address" throughout | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion.  A BSSI is, by definition, the MAC address of the AP.  The question is whether an AP can claim to “have” a BSSID.  Having reviewed the uses, I think putting “MAC” rather than BSSID adds clarity.

Arguably the sentence at 2031.26-28 is redundant after the change.  But I’m no proposing to remove it.

Proposed Resolution:

Accepted.

(Note to editor, there are 9 instances.)

| 6770 |  |  | It says "Because the IEEE Std 802.11 null Data frame does not derive from an MA-UNITDATA.request" -- "null Data frame"? | I think it means a Null frame here | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc | Comment Group |
| --- | --- | --- | --- | --- | --- | --- |
| 6813 |  |  | Should not use "Data" after "Null"/"QoS Null". | As it says in the comment | MAC | Editorials |

Discussion:

There are 9 instance of the term “null Data frame”:

iates a calibration procedure. It shall be a  QoS Null Data frame with the Ack Policy field set to Normal Ack.   In

frames that are Beacon or ATIM frames; or (QoS) Null Data frames shall be transmitted. ATIM frame  transmission t

other than RTS, CTS, Ack, Beacon, ATIM and  (QoS) Null Data frames during the ATIM Window.  e) Individually address

ransmit individually addressed or group addressed Null Data frames within  the ATIM window to indicate the STA’s in

essfully received for all individually  addressed Null Data frames or after the STA has transmitted group addressed

or after the STA has transmitted group addressed Null Data frames at  least dot11BSSBroadcastNullCount times.   10

n occurring.   NOTE 2—Because the IEEE Std 802.11 null Data frame does not derive from an MA-UNITDATA.request  prim

attribute specifies the number of group addressed Null Data frames a   STA may transmit before it changes power man

There is no “Null Data frame”,  there is only a “Null frame” – see 564.13.

Likewise there is no QoS Null Data frame, only “QoS Null frame”.

Proposed Resolution: (to both comments 6770 and 6813)

Revised.

Globally change “Null Data frame” (case insensitive) to “Null frame”

| 6774 |  |  | "The SME shall generate an MLME-ASSOCIATE.response primitive addressed to the non-AP and non-PCP STA." -- primitives are not addressed to STAs, frames are. | Change to refer to the field which gives the target STA | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is at 1597.01.

Proposed Resolution:

Revised.

At cited location change “The SME shall generate an MLME-ASSOCIATE.response primitive addressed to the non-AP and non-PCP STA.”

to “The SME shall generate an MLME-ASSOCIATE.response primitive with the PeerSTAAddress parameter set to the MAC address of the the non-AP and non-PCP STA.”

| 6786 |  |  | "However, an acknowledgment shall be sent in response to a duplicate fragment of an individually addressed MSDU." -- also MMPDU | Add "or MMPDU" after "MSDU" | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment is at 1287.10.

I agree, this is a clear omission.

Proposed resolution:

Accepted.

| 6791 |  |  | References to both 802.1Q-2004 and 2011 in clause 2/annex A. Classifier type 2 should probably not specify the year, therefore | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The comment does not propose any change.

Regardless, there are two cited 802.1Q year numbers (2003 and 2011). (10 instances) and 18 instances without year number.

The hits are:

ion Protocol (SRP) Support for SRP from IEEE Std 802.1Q-2011 enables integration of the TSPEC ADDTS reques Type 2, the Classifier Parameter is the IEEE Std 802.1Q-2003 [B22] VLAN Tag TCI. The 12 endianness of the the 802.1Q VLAN TCI field is as defined IEEE Std 802.1Q-2003 [B22] for the VLAN Tag 13 TCI. The Frame Cla red. For Classifier Type 5 when used to match an 802.1Q-2011 frame, the classifier parameters are: Priorit I subfield contains the value to match against an 802.1Q-2011 frame header, in the LSB; the 7 MSBs are res D subfield contains the value to match against an 802.1Q-2011 frame header, in the 12 LSBs; the 4 MSBs are selection procedures defined in 8.6.8 of IEEE Std 802.1Q-2011 using two queues, the primary and alternate. tocol (SRP) as described in Clause 35 of IEEE Std 802.1Q-2011 is an example of a 43 higher layer protoc by the MSRP DMN are described in C.3 of IEEE Std 802.1Q-2011. 51 52 NOTE 2.If the higher layer SRP Rese dures are described in 54 Clause 35 of IEEE Std 802.1Q-2011. 56 10.4.4.4 TS setup procedures for both

It is valid to refer to a specific year, if the specific contents of that version apply (such as referencing a specific subclause number).

Alternatively, it is possible to reference the standard without the year, in which case the latest version applies.  So the question is:

- Are the references to -2003 necessarily -2003?   On review, I don’t think so.

- Are the references to -2011 necessary?  I think only those 4 citing specific clauses are (the last 4 above).

- Should the normative reference state a year?   I think so, if any of the references cite a year.

Status: checking with Mark / Donald.

Proposed resolution:

Revised

Replace all 802.1Q-2003 with 802.1Q.

Replace all 802.1Q-2011 with 802.1Q, except at: 1242.36, 1610.42, 1610.51 and 1610.54.

| 6332 | 819.44 | 8.4.2.24.2 | It says "default pairwise cipher suite and default group cipher suite for Data frames in an RSNA" but what does default mean here? | Is it referring to the case where the RSNE is truncated before these fields? If so, say so; if not, say what it means | MAC | Frame formats 8.4 |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The inclusion of the items in this table in an RSNE is optional.

So when they are not included, some mechanism is necessary to supply a default value.

Status:

Although I think I’ve got it right,  Jouni tells me privately that I’ve got it wrong.    Therefore I suggest he be assigned the comment.

Proposed Resolution:

At 819.46 (suite 4) add “. See NOTE 1.”

At 819.52 (suite 6) add “. See NOTE 2.”

At 819.55 (suite 8) add “. See NOTE 1, See NOTE 2.”

After 820.10 add a table last row, merged containing:

“NOTE 1—This value is used for the parwise cipher suite is none is specified in the RSNE.

NOTE 2—This value is used for the group cipher suite is none is specified in the RSNE.”

| 6799 |  |  | Table 11-8---Integrity and key-wrap algorithms top row has "Deprecated" instead of an AKM in the first cell -- need to give the AKM and indicate the deprecation somewhere else. | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I can’t find a published standard (802.11i, 802.11-2007, 802.11-2012) that includes this table where this value is defined.  Given that no value is defined for the AKM,  which appears to be the “key” of this table, the entry should be removed.

You can’t deprecate something that never existed,  so indicating deprecation anywhere is a mistake.

Proposed Resolution:

Revised.

Remove cited table row (1971.21).

| 6811 |  |  | "When dot11DMGOptionImplemented is true, dot11MeshActivated shall be set to false." -- shouldn't the first be Activated? In theory you could implement DMG but not have it activated, and (implement and) activate mesh instead. | Change as suggested in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.  There is no dot11DMGOptionActivated MIB variable.   Changing at this specific location would create a reference to a variable that does not exist.

| 6826 |  |  | "The Request element field is described in 8.4.2.10 (Request element)." in 8.6.20.5 Information Response frame format does not seem correct. Was some description of the Request information information (sic) intended? | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The intended reference is 1206.46.

The cited sentence clearly doesn’t belong here, because there is no such element in this frame format.

The distinction between “requested elements” and “provided elements” is in the frame format is unclear.

The related procedure is described in 10.30.1 (1809.10).  It indicates that the response frame can be unsolicited, but does not indicate what it may carry in this case.  This is presumably the “provided” information.

Propose Resolution:

Revised.  Delete paragraph at 1206.46.

Change the para at 1206.48 “The provided elements are elements, as described in 8.4.2 (Elements), that the transmitter of this frame is providing to the destination of the frame.”

to read

“The requested elements are those returned in response to an Information Request frame, as described in 10.30.1.

The provided elements are elements, as described in 8.4.2 (Elements), that the transmitter of this frame provides to the destination of the frame, either in addition to the requested elements, or in an unsolicited Information Request frame.”

| 6827 |  |  | "The format of Channel Entry field is shown in Figure 8-391 (Channel Entry field format)" -- OK, but the description of the subfields is in terms of Location Track Notification usage, which is not appropriate to Channel Usage use. | Fix the wording | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The cited location is 952.47.

The commenter is correct that the wording (at 930.29) is inappropriate to be “re-used” from the Channel Usage element.

I propose to move the Channel Entry field to the common fields definitions and rename it to something more descriptive.

Proposed Resolution:

Revised.  Make changes under CID 6827 in <this-document>.  These changes move the referenced field structure into its own subclause in 8.4.1,  make it generic,  and reference the new subclause from the cited location.

Proposed Changes:

At 666.57 insert a new subclause:

8.4.1.21a Operating Class and Channel field The Operating Class and Channel field is used in the Location Indication Channels subelement of the Location Parameters element and in the Channel Usage element. The Operating Class and Channel field indicates an operating class and channel. The format of the field is defined in Figure 8-aaa. Operating Class; Channel. Octets:; 1; 1. Figure 8-aaa – Operating Class and Channel field The Operating Class field each indicates an operating class value as defined in Annex E. The operating class is interpreted in the context of the country specified in the Beacon frame. The Channel field includes a channel number, which is interpreted in the context of the indicated operating class. Channel numbers are defined in Annex E.

At 930.20:  Delete Figure 8-392.

At 930.17:

The Channel Entry field includes one or more Operating Class and Channel fields (see 8.4.1.21a). <figure not shown, it is deleted by the editing instruction above> The Operating Class subfield of the Operating Class and Channel field indicates the frequency band on which a STA transmits Location Track Notification frames. The Channel subfield of the Operating Class and Channel field includes a channel number on which a STA sends or an ESS expects to receive Location Track Notification frames. Operating Class and Channel fields can be grouped together to identify a noncontiguous channel. A noncontiguous channel is indicated by a group of N+1 Operating Class and Channel fields where the first NChannel Operating Class and Channel fields contain an Operating Class subfield with an 80+ Behavior Limit and the last Operating Class and Channel field in the group contains an Operating Class subfield without an 80+ Behavior Limit (as defined in Annex E)

At 952.47:

The Channel Entry field includes zero or more Operating Class and Channel fields. The format of the Operating Class and Channel field is defined in 8.4.1.21a.. Operating Class and Channel fields may be grouped together to identify a noncontiguous channel as described in 8.4.2.70.3 (Location Indication Channels subelement).

| 6828 |  |  | "For an ADDBA set up between STAs where one is a non-HT STA, the Block Ack Policy and Buffer Size fields in the ADDBA Request frame are advisory and may be changed by the recipient. The Buffer Size field in the ADDBA Request frame is advisory and may be changed by the recipient for an ADDBA set up between HT STAs. The Block Ack Timeout Value field in the ADDBA Request frame is advisory and may be changed by the recipient for an ADDBA set up between HT STAs" -- why the inconsistency between HT STAs and non-HT STAs? | Clarify | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

The comment identifies a difference in behaviour between different types of STA, but does not indicate a problem to resolve.

| 6808 |  |  | "References in this standard to a ' report', where corresponds to one of the Measurement Types in Table 8-113 (Measurement Type definitions for measurement reports) is equivalent to (according to context) a) 'a Measurement Report frame or Radio Measurement Report frame carrying a Measurement Report element with the Measurement Type equal to ' or b) 'a Measurement Report element with the Measurement Type equal to '." -- should be "a Spectrum Management Measurement Report frame or ...". Probably ditto request. | As it says in the comment | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.  While the “Measurement Report” frame is a spectrum management action frame, the name of the frame is itself not prefixed with “Spectrum Management”.  Likewise for the “Measurement Request” frame.

| 6298 |  |  | People keep confusing MPDUs, MSDUs, MMPDUs, A-MSDUs and A-MPDUs | Say something somewhere like "An MSDU or MMPDU is transmitted in one or more MPDUs. An MSDU may be carried in an A-MSDU. An A-MSDU is transmitted in one MPDU. An MSDU or MMPDU may be carried (in an MPDU) in an A-MPDU." | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Yes, folks confuse all these terms, probably because they look alike.

We have, hopefully, already unambiguous definitions for these terms.

Adding a note “somewhere” will not necessarily improve matters.

Proposed Resolution:

Rejected.

The comment does not indicate an error in the standard.

| 6317 |  |  | The term "RTT" is used but in all cases it actually refers to the time of fight | Replace the term "RTT" with the term "TOF" in the glossary, and update the 4 instances of the term "RTT" accordingly | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

Two of the uses (1741.27, 3566.21) of RTT refer validly to the round trip time, not the one-way time of flight. One of the uses (82.59) could validly be either.

| 6447 |  |  | What is a "QoS Data frame" or "QoS Data MPDU"? Does this refer to all frames of Type Data and Subtype 1000-1111, or all frames of Type Data and Subtype 1000-1011 (i.e. not QoS Null or immediate relatives), or all frames of Type Data and Subtype 1000 (i.e. not QoS Data + CF-anything)? 568.44 is an example of the first of these, 572.33 seems to be an example of the last of these and 574.52 seems to be an example of the middle of these | Use "QoS data subtype" for the first, "QoS (+)Data frame" for the second and "QoS Data frame" for the last, throughout | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Submission Required

Assign to commenter.

| 6568 |  |  | The ProbeDelay isn't a probe delay other than in scanning | Rename it to something more generic, like NAVSyncDelay | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The commenter is correct.  It’s a relatively easy fix.

However, I am not proposing to change the *ProbeDelay named MIB variables,  which results in more extensive changes.

Proposed Resolution:

Revised.

Change ProbeDelay to NAVSyncDelay at:

159.05, 159.28, 200.46, 200.43, 1550.34, 1649.03

| 6665 |  |  | " adopt" -- what does this mean? | Clarify (see CID 3018) | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.  The comment doesn’t indicate an issue to resolve.  The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

| 6757 |  |  | There is confusion between "RSNA" (the association, so necessarily point-to-point) and "RSN" (the network, so general). | Use the right term in each case | MAC | Terminology |
| --- | --- | --- | --- | --- | --- | --- |

Submission Required.

Assign to commenter.

| 6055 | 617.28 | 8.3.2.2.2 | Text says, "i.e., all of the MSDUs are intended to be received by a single receiver". But, there is no restriction on group addressed A-MSDU transmission. In fact, GCR explicitly uses group addressed A-MSDUs. | Change "be received by a single receiver" to "be received by a single receiver, or by all receivers of a single group address" | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

Context: 617.28

An A-MSDU contains only MSDUs whose DA and SA parameter values map to the same receiver address (RA) and transmitter address (TA) values, i.e., all of the MSDUs are intended to be received by a single receiver, and necessarily they are all transmitted by the same transmitter. The rules for determining RA and TA are independent of whether the frame body carries an A-MSDU.

Proposed Resolution:

Accepted

| 5954 | 617.45 | 8.3.2.2.2 | The maximum MPDU length of 4095 given in Note3 for non-DMG STAs contradicts the values found in table 8-19--Maximum data unit sizes (in octets) and durations (in microseconds) | Probably best to include a reference in the note to table 8-19. | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

We have three comments on this NOTE – this, CID 6250 (Assigned to Matt F) and CID 6252 (Approved,  deletes part of the cited note).

I’m not sure the NOTE is worth what it’s costing us.  The existence of the constraint is made clear at 587.30 and the cited NOTE.

Proposed Resolution.  (to CID 6250 also.  CID 6252 can stay as it is, no conflict.).

Revised.

Remove the cited note.  Table 8-19 suffices to make the constraints expressed in the note clear.

| 6107 | 620.45 | 8.3.3.2 | It is possible (even without the inclusion of Vendor Specific elements) for the length of the Beacon frame to exceed the maximum frame size for (at least) the PHYs defined in clauses 16 (DSSS), 17 (HR), 18 (OFDM), and 19 (ERP). If this ever occurs it creates an significant risk of interoperability problems because a conformant implementation is supposed to meet all mandatory normative requirements, but in this situation two of said requirements -- the contents of the Beacon frame and the maximum frame size -- are in direct conflict. | There are several ways to resolve this situation, and I consider it of primary importance for there to be an unambiguous statement of what should be transmitted in cases where this situation arises. What I consider to be the simplest approach that does not preclude behaviors that are currently permitted is to define which elements are mandatory in all Beacon frames, which elements only need to be sent every Nth beacon, and a rule to sequence the elements of the second type such that each does get sent at least once in every N Beacons. Document 11-15-0531-00-000m (a submission by this commenter to the May 2015 session in Vancouver) contains a proposal for achieving this. However, this commenter is certainly open to other approaches of resolving this problem. | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

Assign to commenter to present his submission.

| 5968 | 632.53 | 8.3.3.9 | Probe Request and Probe Response have been growing in szie, which is undesirable in particular at low rates sucuh as 1 Mbps. | Reduce the size of Probe Request and Probe Response, for example by including only a couple supported rates and by defining a shorthand notation for frequently used configurations. | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

Needs Submission.

Assign to Commenter (Menzo)

| 6108 | 632.61 | 8.3.3.10 | It is possible (even without the inclusion of Vendor Specific elements) for the length of the Probe Response frame to exceed the maximum frame size for (at least) the PHYs defined in clauses 16 (DSSS), 17 (HR), 18 (OFDM), and 19 (ERP). If this ever occurs it creates an significant risk of interoperability problems because a conformant implementation is supposed to meet all mandatory normative requirements, but in this situation two of said requirements -- the contents of the Probe Response frame and the maximum frame size -- are in direct conflict. | There are several ways to resolve this situation, and I consider it of primary importance for there to be an unambiguous statement of what should be transmitted in cases where this situation arises. This problem is closely related to the equivalent situation that exists for Beacon frames, but is harder solve because the alternative of sending certain elements only in every Nth Beacon, as discussed in document 11-15-0531-00-000m (a submission by this commenter to the May 2015 session in Vancouver) is not applicable to Probe Responses. The only obvious approach for Probe Responses is to define which elements are mandatory and which elements may optionally be omitted if insufficient space is available (perhaps with a precedence for deciding which to omit). The document cited above contains some suggestions in this regad. | MAC | Frame formats |
| --- | --- | --- | --- | --- | --- | --- |

See CID 6107.  Assign to commenter to present.

| 5031 | 1030.36 | 8.4.2.147 | "It is set to 1 if the STA supports both Link cooperating type and Link switching type. It is set to 0 if a STA supports only Link switching or if the Duplex subfield is set to 1." So, what happens if the STA supports both Link cooperation, link switching and duplex? "Link cooperating type and Link switching type" -- the capitalization of the various "Link" types doesn't follow WG style. The language it poor, in what sense is a "type" supported. | Reword thus: "It is set to 1 if the STA supports both link cooperation and link switching. It is set to 0 otherwise." | MAC | Frame formats 8.4 |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

Note comment 5030 globally changed “link cooperating” to “link cooperation”

Proposed Resolution:

Revised.

Globally change “link cooperating type” to “link cooperation”

Globally change “link cooperating” to “link cooperation”

Globally change “Link switching type” to “link switching”

Globally change “Link switching” to “link switching”

At 1030.36 replace “It is set to 0 if a STA supports only Link switching or if the Duplex subfield is set to 1.” with “It is set to 0 otherwise.”

(Note to editor, comment 5030 also globally changes “link cooperating” to “link cooperation”).

| 6666 | 1552.1 |  | At 1552.1 in D3.0 add "and a maximum delivery interval" after "a delivery interval". | As it says in the comment | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:  I can’t find the cited text at the cited location.

Proposed Resolution:

Rejected.

The comment does not indicate a problem to resolve.  The cited location is not in the balloted draft.

| 6403 | 1941.63 |  | Do the msbs need to be irretrievably deleted at 1941.63 (EAPOL-Key MIC), 1896.4 (BIP-CMAC) and 959.5 (Emergency Alert Identifier Hash) in D3.0? | Add a Truncate-128() for the first one, and generalise Truncate-128() to Truncate-n(), so the latter two can use Truncate-64() | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

I’m not getting into this argument.

Propose to assign to Dan or Jouni.

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc | Comment Group |
| --- | --- | --- | --- | --- | --- | --- |
| 6607 | 2198 | 17 | Some bits of the spec state/imply "HR/DSSS/short" is included in "HR/DSSS", others exclude it | Be consistent | GEN | PHY |

Discussion:

The comment is not specific enough, for example by citing a contradiction.

Proposed Resolution:

Rejected.   The comment asserts an inconsistency, but does not back this up with references. The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.

| 6294 | 2202.35 | 17.2.3.6 | It says "if the rounding performed by the Ceil() operation took less than 8/11 or a 1 if the rounding took more than or equal to 8/11." -- what does it mean for rounding to take more or less than a number? | Clarify (perhaps it refers to the difference between the rounded number and the original number?). Also replace Ceil() with the floor symbols | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I agree this “took” language is awkward.  The “shall” is unnecessary, because that at 2202.29 suffices.  Also,  as it has been named previously,  it is unnecessary to re-specify it as b7.

Proposed Resolution:

Revised.

Replace “the service field (b7) bit shall indicate a 0 if the rounding performed by the Ceil() operation took less than 8/11 or a 1 if the rounding took more than or equal to 8/11.”

with:

“the length extension bit is 0 if  ((number of octets x 8/11)-Length) < 8/11 and is 1 otherwise.”

| 5067 | 2238.64 | 18.3.2.5 | "The normative specifications of generating the transmitted waveforms shall utilize the rectangular pulse shape" Who is responsible for actioning this "shall". I claim, as written, it is a "shall" directed at the authors of this specification. | Rewrite so that the normative requirements on an implementer are clear, and avoid creating normative specifications for the authors of this documents. | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |

Context: 2238.62:

In the case of vanishing TTR , the windowing function degenerates into a rectangular pulse of duration T. The normative specifications of generating the transmitted waveforms shall utilize the rectangular pulse shape. In implementation, higher TTRis typically implemented in order to smooth the transitions between the consecutive subsections. This creates a small overlap between them, of duration TTR , as shown in Figure 18-2 (Illustration of OFDM frame with cyclic extension and windowing for (a) single reception or (b) two receptions of the FFT period).

Discussion:

I think this is a simple case of excessive “shall-ness”.  What it is trying to say is the standard assumes a rectangular pulse, but implementations will do something different (as they are probably forced to do to meet the transmit mask and band edge requirements).

Proposed Resolution:

Revised.

At cited location replace “The normative specifications of generating the transmitted waveforms shall utilize the rectangular pulse shape.”

with

“This standard describes transmitted waveforms using a rectangular pulse shape.”

Also change the following sentence to read:

“In a typical implementation, a higher TTR is used in order to smooth the transitions between consecutive subsections.”

| 6268 | 2360.26 | 20.3.16 | It says "The OFDM PHY" | Change to "The HT PHY" | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

The standard is intended to specify what is needed for interoperability, not to keep the manufacturer out of gaol.  This subclause it an attempt to stop manufacturers doing something illegal,  and has no other content.  As such, it has no place in the standard.

Proposed Resolution:

Revised.  At 2360.25 delete subclause 20.3.16 (Transmit and receive in-band and out-of-band spurious transmissions) in its entirety.

| 6260 | 2367.41 | 20.3.20.2 | It says "compliant with the OFDM PHY" | Change to "compliant with the HT PHY" | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |
| 6262 | 2368.04 | 20.3.20.3 | It says "compliant with the OFDM PHY" | Change to "compliant with the HT PHY" | GEN | PHY |

Context: 2367.41

The interfering signal in the adjacent channel shall be a signal compliant with the OFDM PHY, unsynchronized with the signal in the channel under test. For a conforming OFDM PHY, the corresponding rejection shall be no less than specified in Table 20-23 (Receiver minimum input level sensitivity). The interference signal shall have a minimum duty cycle of 50%.

Discussion:

An HT STA might validly encounter an OFDM (i.e. 802.11a) signal as an interferer.  Such a signal will also look identical to an 802.11g transmission.   Stating that this is OFDM avoids answering the question as to the format (legacy, MF, GF) and width of the interfering signal.

But if we do need to change it we should:

- Supply the missing information about format and width

- Determine that this does not make legacy devices non-compliant.

I got the following suggestion from Youhan, and Vinko approves it:

REVISED. Change “raising the power of the interfering signal until” to “raising the power of the interfering signal of 20 MHz bandwidth until” at D4.0 P2367L55. Change “raising the power of the interfering signal until” to “raising the power of the interfering signal of 40 MHz bandwidth until” at D4.0 P2367L64. Change “OFDM PHY” to “HT PHY” at D4.0 P2367L41 and P2368L5. Change “OFDM PHY” to “VHT PHY” at D4.0 P2552L9 and P2553L20. (I think below requires some wordsmithing – just giving the general thought…) Add “The interfering signal shall be a signal compliant with the TVHT PHY.” at D4.0 P2626L55 (11af). Add “The interfering signal shall be a signal compliant with the TVHT PHY.” at D4.0 P2627L42 (11af).

Youhan’s suggestion is that the interfering signal should be transmitted by the HT PHY without indicating any more than its width.  As its only purpose is to act as an interferer, and as it will get well and truly mangled by the receiver’s filter, its format probably doesn’t matter much.

Youhan’s changes don’t address the case of the adjacent channel.  I’m assuming that matching changes should be made there.

Proposed Resolution: (to both CID 6260 and 6262)

Revised.

At 2367.41, 2367.42, 2368.05, 2368.06 change “OFDM PHY” to “HT PHY”.

At 2367.25, 2367.55 change “the interfering signal” to “an interfering signal of 20 MHz bandwidth”.

At 2367.35, 2367.64 change “the interfering signal” to “an interfering signal of 40 MHz bandwidth”.

At 2552.09, 2552.12, 2553.20, 2553.23 change “OFDM PHY” to “VHT PHY”

| 6261 | 2367.42 | 20.3.20.2 | It says "For a conforming OFDM PHY" | Change to "For an HT PHY" | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |
| 6263 | 2368.05 | 20.3.20.3 | It says "For a conforming OFDM PHY" | Change to "For an HT PHY" | GEN | PHY |

Discussion:

This is the device under test, and should be the HT PHY.

Proposed resolution: (to both)

Accepted.

(Note to editor,  these changes are a subset of the changes for CID 6260)

| 6465 | 2384.4 | 20.5 | "NOTE---Support of 400 ns GI is optional on transmit and receive." applies to more than the table this is in | Promote the NOTE to the main text of the referenced subclause, and add it to Clauses 21 to 23 too | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I suppose why the note was thought necessary was that the table is captioned “mandatory”.  None of the other Clause 20 tables have “mandatory” in their title,   so that is a reason why the note is attached only here.

Clause 21 (DMG) does not have “mandatory”,  neither does Clause 23 (TVHT).

Clause 22 does in three places.   I propose a “conservative” change to bring Clause 22 into line with Clause 20 usage of this note.

Proposed Resolution:

At 2569.32, 2573.33, and 2577.33 after “400 ns GI” insert “(See NOTE)”

At 2569.52, 2573.53, and 2577.53 insert a new last table row,  merged,  containing:

“NOTE—Support of 400 ns GI is optional on transmit and receive”

In reply to the comment The NOTE applies specifically to this table because it is the only Clause 20 MCS table that includes “mandatory” in its caption, and reminds the reader that support of short GI is not mandatory.  Change to Clauses 21 and 23 are not necessary,  because these do not label their MCS tables with “mandatory”.

| 6466 | 2385.09 | 20.5 | "NOTE---The 400 ns GI rate values are rounded to 1 decimal place." applies to more than the table this is in | Promote the NOTE to the main text of the referenced subclause, and add it to Clauses 21 to 23 too | GEN | PHY |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I agree that the note is in the wrong place for Clause 20.

I do not know whether the Clause 21-23 tables are actually rounded or not.

Status:  need input from DMG, VHT and TVHT PHY experts.

Proposed Resolution:

Revised.

At 2384.55, delete “(see NOTE)”.   At 2385.09 delete the last table row containing the NOTE.

At 2384.13 insert a new para: “In the MCS parameter tables that follow, data rates for a 400 ns GI are rounded to 1 decimal place.”

| 6093 | 3550.39 | Q.7 | This paragraph is hopeless confusion of logical architecture and implementation options, and uses what seems like logical terms (AU) in ambiguous and contradictory ways. I don't think this paragraph adds anything (except confusion) to the Standard at this point in time. Without this paragraph, Q.7 has no purpose. | Delete subclause Q.7. | MAC |  |
| --- | --- | --- | --- | --- | --- | --- |

Discussion:

I read Q.7, and ended up none the wiser.

Proposed Resolution:

Accepted.