IEEE P802.11
Wireless LANs

| REVmc - MAC power management comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-10-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document contains some proposed resolutions to SB0 comments.

CID 5862, 6373, 6374

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5862 | 567 | 8.2.4.1.7 |  | The paragraph reads:"In an MBSS, the Power Management field is valid only in frame exchanges as described per the mesh power mode transitions rules in 13.14 (Power save in a mesh BSS)." It does not provide definition of the PM bit. It should be better to have field value definition here. | Insert the following sentences to the end of 8.2.4.1.7 (Power Management field). "In an MBSS, the Power Management field is valid only in frame exchanges as described per the mesh power mode transitions rules in 13.14 (Power save in a mesh BSS). A value of 1 in an individually addressed frames toward peer mesh STA indicates that the STA will be in either light sleep mode or deep sleep mode for the peering. A value of 1 in other frames indicates that the STA's non-peer mesh power mode is deep sleep mode. A value of 0 in an individually addressed frames toward peer mesh STA indicates that the STA will be in active mode for the peering. A value of 0 in other frames indicates that the STA's non-peer mesh power mode is active mode." |  | MAC |

Discussion

It appears that the use of the word “towards” in the mesh description means “within messages transmitted to”.

There is a reasonable description of the Power Management field behaviour within clause 13.14, and it does not appear trivial to summarise it within clause 8.4.2.1.7

So, either the suggested resolution is reasonable (albeit with some editorial corrections), or the suggested resolution is not necessary.

Proposed Resolution.

Revised.  The proposed change is to add the following sentence “In such exchanges, the value indicates the STA’s power management mode (see 13.14)”.

At 567.6 change:

In an MBSS, the Power Management field is valid only in frame exchanges as described per the mesh power mode transitions rules in 13.14 (Power save in a mesh BSS). In such exchanges, the value indicates the STA’s power management mode (see 13.14).

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6373 | 566 | 8.4.2.1.7 |  | "The Power Management field is valid only in frame exchanges as described in 10.2.2.2" -- but DMG STAs in an infra BSS have their own PM field subclause | Add "or 10.2.6" |  | MAC |

Discussion

In addition to clause 10.2.2.2, the Power Management field is mentioned and has defined behaviour in clause 10.2.2.5.1 (QoS STA), 10.2.2.14 (TDLS peer), 10.2.2.15.1 (TDLS general). 10.2.6.2.2 (Non-AP and non-PCP STA).

The cited reference should be changed, but also broadened.

Proposed Resolution.

Revised. Change the text

“The Power Management field is valid only in frame exchanges as described in 10.2.2.2”

to

“The Power Management field is valid only in frame exchanges as described in 10.2.2 and 10.2.6”

At 566.54 change:

The Power Management field is valid only in frame exchanges as described in 10.2.2 and 10.2.6 . In such exchanges, a value of 1 indicates that the STA will be in PS mode. A value of 0 indicates that the STA will be in active mode.

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6374 | 567 | 8.4.2.1.7 |  | "The Power Management field is valid only in frame exchanges as described in 10.2.2.2" -- do DMG STAs follow the normal rules (that would be a first...) when in an IBSS? | Clarify |  | MAC |

Discussion

It’s not clear what the commenter is looking for.  Perhaps “normal rules” needs to be described a little more, especially with regard to power management.

Proposed Resolution.

Reject: Insufficent detail.

Proposed Changes: