IEEE P802.11
Wireless LANs

| Specification Framework for TGay |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-03-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Carlos Cordeiro | Intel | 2111 NE 25th Ave, Hillsboro OR 97124, USA | +1-503-712-9356 | carlos.cordeiro@intel.com |

Abstract

This document provides the framework from which the draft TGay amendment will be developed. The document provides an outline of each the functional blocks that will be a part of the final amendment. The document is intended to reflect the working consensus of the group on the broad outline for the draft specification. As such it is expected to begin with minimal detail reflecting agreement on specific techniques and highlighting areas on which agreement is still required. It may also begin with an incomplete feature list with additional features added as they are justified. The document will evolve over time until it includes sufficient detail on all the functional blocks and their inter-dependencies so that work can begin on the draft amendment itself.

- Definitions and acronyms

  - Acronyms

| DL | Downlink |
| --- | --- |
| DMG | Directional Multi-Gigabit |
| EDMG | Enhanced DMG |

  - Definitions

enhanced directional multi-gigabit (EDMG) basic service set (BSS): A BSS in which a DMG Beacon frame transmitted by an EDMG station (STA) includes the EDMG Capabilities element.

enhanced directional multi-gigabit (EDMG) station (STA): A directional multi-gigabit (DMG) STA whose radio transmitter is capable of transmitting and receiving EDMG physical layer (PHY) protocol data units (PPDUs).

- Frame formats

  - DMG Beacon

A EDMG Supported field shall be defined by using one reserved bit from the DMG Parameters field. An EDMG STA shall set the EDMG Supported field to 1.

  - Block Ack

The size of the Block Ack window bitmap for EDMG STAs shall be 1024 bits.

  - Information elements

  - MIMO

An EDMG STA may transmit a “MIMO Setup frame” (e.g., RTS or DMG CTS-to-self) prior to the transmission of a SU or MU MIMO PPDU. The “MIMO Setup frame” indicates the destination STA(s) addressed by the PPDU.

A “MIMO Setup frame” (e.g., RTS) transmission may trigger a response (e.g., DMG CTS or Ack) from destination STA(s).

- MAC sublayer

  - Channel access

To enable backward compatibility with non-EDMG STAs, in an EDMG BSS the BTI, A-BFT and ATI is always present on the primary channel of the BSS. The channel width of the primary channel of an EDMG BSS is 2.16 GHz.

    - Control trailer

A Control Trailer may be used in Control mode PPDUs in place of TRN-Units to carry control data. The Control Trailer is encoded as Control mode LDPC codewords. The presence of the Control Trailer is signaled in the Control mode header using the existing Training Length and Packet Type fields, and by setting reserved bits 22 and 23 both to 1.

    - Channel access over multiple channels

When using multiple channels, the following apply:

- Full carrier sense (physical and virtual) shall be maintained on a primary channel.

- An EDMG STA may transmit a frame to a peer EDMG STA to indicate intent to perform channel bonding transmission to the peer STA. This allows an EDMG STA to choose to operate over multiple channels only after receiving such a frame, thus saving power.

The 11ay specification supports that, when using multiple channels, a PCP or an AP can simultaneously transmit to multiple STAs allocated to different channels individually.

The 11ay specification shall support allocation of SP(s) and scheduled CBAP(s) over more than one channel and/or over a bonded channel.

    - MIMO channel access

An EDMG STA may transmit a frame to one or more EDMG STAs to indicate a target time in which the transmitting STA intends to contend for the channel to send a MIMO PPDU to the STA(s).

  - DMG beamforming

The 11ay SLS beamforming protocol shall enable feedback of one or more sectors per TX and RX antenna.

The 11ay beamforming protocol shall enable TX and RX training using the same BRP frame.

The 11ay beamforming protocol shall enable simultaneous BF training of transmit DMG antennas.

The 11ay beamforming protocol shall support simultaneous RX antenna training.

The 11ay beamforming protocol shall provide the means to enable TX sector down selection as part of beamforming training.

The 11ay beamforming protocol shall support RX sector down selection, for STAs with antenna pattern reciprocity, as part of beamforming training.

The 11ay beamforming protocol shall support a mode of operation in which, in a single phase, the initiator trains its transmit antennas/sectors and multiple responders train their receive antennas/sectors.

The 11ay beamforming protocol supports multi-beamforming for multiple array antennas. Multi-beamforming means that a transmitter simultaneously sends SSW frames in multiple polarized directions. 11ay provides the means of multi-beamforming capability exchange in which the initiator and responders exchange the information on whether they have the capability of multi-beamforming.

The 11ay beamforming protocol should support simultaneous multi-STA RSS over multiple channels during the A-BFT.

An EDMG STA shall not transmit a Short SSW packet to another STA that is not also an EDMG STA.

For SLS using Short SSW frames, LBIFS is equal to 2×TXTIME(Short SSW) + 3×SBIFS.  An EDMG STA performing an SLS using Short SSW frames shall increase the value of the CDOWN field within the Short SSW frame by two for each LBIFS contained as part of a sector sweep.

  - SU MIMO beamforming

BF for SU MIMO comprises the following phases:

- SLS:  Coarse BF training to allow TX sector down selection for Initiator and Responder including feedback with candidate sectors.

- TX-RX Sector/Antenna Mapping:  BF transining of RX sectors/antennas/RF chains for each of the candidate TX sectors to determine best TX-RX configuration for SU MIMO.

- Others:  TBD

  - MU MIMO beamforming

BF for MU-MIMO supports training of more STAs, including collecting their feedback, than the number of STAs transmitted to in each MU-MIMO transmission.

- MLME

  - Spatial reuse and interference mitigation

11ay shall provide means to extend the DMG spatial sharing mechanism for operation in a multi-channel configuration.

The EDMG spatial sharing mechanism shall enable an EDMG STA to perform concurrent measurements employing multiple RX antenna configurations as are used for receiving multiple streams from the target EDMG STA based on the same measurement configuration.

- Security

An EDMG STA may include the Authentication IE within Management and Extension frames during discovery and association phase to parallelize the pre-shared key based authentication and key generation, and speed up the process of authentication and association.

- EDMG PHY

  - General

The 11ay specification shall support:

- SU-MIMO

- Downlink MU-MIMO

- Channel bonding of at least two 2.16 GHz

- Contiguous and non-contiguous channel aggregation of at least two 2.16 GHz channels

11ay shall enable both SC and OFDM modulations for SU-MIMO and MU-MIMO data transmission.

11ay supports channel-wise DL OFDMA. This means that in a bonded channel, a PCP or AP can simultaneously transmit to multiple STAs allocated to different frequency resources in the unit of one channel bandwidth.

  - Common parameters

    - Introduction

    - EDMG PPDU format

A single PPDU format is defined for all EDMG PHYs: the EDMG PPDU format. Figure 1 shows the EDMG PPDU format and all possible fields. Not all fields are transmitted in an EDMG PPDU. Fields are included depending on whether the PPDU is a SU PPDU or is a MU PPDU.

| L-STF | L-CEF | L-Header | EDMG-Header-A | EDMG-STF | EDMG-CEF | EDMG-Header-B | Data | AGC | TRN |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- —EDMG PPDU format

The fields of the EDMG PPDU format are summarized in Table 1.

- —Fields of the EDMG PPDU

| Field | Description |
| --- | --- |
| L-STF | Non-EDMG Short Training field |
| L-CEF | Non-EDMG Channel Estimation field |
| L-Header | Non-EDMG Header field |
| EDMG-Header-A | EDMG Header A field |
| EDMG-STF | EDMG Short Training field |
| EDMG-CEF | EDMG Channel Estimation field |
| EDMG-Header-B | EDMG Header B field |
| Data | The Data field carriers the PSDU(s) |
| AGC | Automatic Gain Control field |
| TRN | Training sequences field |

The EDMG-Header-A, EDMG-STF, EDMG-CEF and EDMG-Header-B fields exist only in EDMG PPDUs.

    - EDMG preamble

      - Introduction

      - Non-EDMG portion of EDMG format preamble

The structure of the L-Header field is defined as follows:

- For a control mode PPDU, the L-Header field is the same as the DMG control mode header field (see Table 20-11) with the following change: the reserved bits 22 and 23 shall be both set to 1 to indicate the presence of the EDMG-Header-A field. This implies that the PPDU is an EDMG PPDU.

- For a SC mode PPDU or an OFDM mode PPDU, the L-Header field is the same as the DMG SC mode PHY header (see Table 20-17) with the following change: the reserved bit 46 shall be set to 1 to indicate the presence of the EDMG-Header-A field. This implies that the PPDU is an EDMG PPDU.

      - EDMG portion of EDMG format preamble

An EDMG STA shall be able to determine the primary channel and occupied bandwidth from any EDMG PPDU it receives.

    - Channelization

The channelization used by EDMG STAs is shown in Figure 2. The circumstances in which a channel can be used in a regulatory domain is determined by local regulatory rules and any additional rules prescribed by the 11ay specification.

- —Channelization used by EDMG STAs

    - Transmit mask

  - EDMG control PHY

  - EDMG SC PHY

  - EDMG OFDM PHY

  - EDMG Beamforming

The format of the Short SSW packet is shown in Figure 3 and its fields are defined in Table 2.

|  | Packet Type | Addressing | CDOWN | RF Chain ID | Short SSW Feedback | Direction | Reserved | FCS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits | 2 | 16 | 11 | 2 | 11 | 1 | 1 | 4 |

- - Short SSW packet format

- - Short SSW field definition

| Field | Definition |
| --- | --- |
| Packet Type | Indicates the type of the packet. Possible values: 0: Short SSW 1-3: Reserved |
| Addressing | The format is TBD. |
| CDOWN | A down-counter indicating the number of remaining Short SSW packet transmissions and LBIFSs to the end of the TXSS/RXSS across all antennas. This field is set to 0 in the last Short SSW packet transmission. |
| RF Chain ID | Identifies the RF chain the transmitter is currently using for this transmission. |
| Short SSW Feedback | In a RSS, contains the value of the CDOWN field of the Short SSW packet that was received with best quality in the immediately preceding sector sweep. The determination of which packet was received with best quality is implementation dependent. This field is reserved when transmitted as part of an ISS. |
| Direction | Indicates the direction of the transmission. The Direction field is set to 0 to indicate that the frame is transmitted by the beamforming initiator and set to 1 to indicate that the frame is transmitted by the beamforming responder. |
| FCS | The four MSBs of the FCS |