IEEE P802.11
Wireless LANs

| 802.11 REVmc Initial Sponsor Ballot – MAC Comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd, Mississauga, ON. CANADA. L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |

Abstract

This document contains some proposed resolutions to initial sponsor ballot comments..

# Comments

| 5042 | 253.40 | 6.3.29.2.2 | "Specifies additional parameters associated with the block ack agreement." -- this is about as useful as a chocolate teapot. When you have nothing to say, say nothing. Ditto at 255.06, 256.23 and 257.46. | Remove cited sentence. |
| --- | --- | --- | --- | --- |

Discussion:

- Looks like a reasonable change.

Proposed Resolution:

Accept.

| 5043 | 253.32 | 6.3.29.2.2 | Use of "block ack" without a noun to qualify it ambiguous. Does it relate to the mechanism, a specific agreement, a specific frame? Suggest that all such uses be qualified with one of these terms, rewording as necessary. | Review all such uses and add qualifying noun where missing. |
| --- | --- | --- | --- | --- |

Discussion:

Proposed Resolution:

Revised. Change all occurences of "Block ACK is" to "Block Ack agreement is" throughout the document.

| 5044 | 401.50 | 6.3.74.2.1 | "QoS Map" is used as both an adjective and a noun. Recommend it be considered to be an adjective. | Review all uses of "QoS Map" and add any missing noun. e.g., at the cited location, add "frame". |
| --- | --- | --- | --- | --- |

Discussion:

- Replace all occurrences where QoS Map is used as a noun and ensure that it is only used as an adjective.

Proposed Resolution:

Revised.

At 401.50, change "QoS Map" to "QoS Map Configure frame"

At 402.9, change "QoS Map" to "QoS Map information"

At 402.53, change "QoS Map" to "QoS Map information"

At 628.15, change "QoS Map" to "The QoS Map element"

At 632.18, change "QoS Map" to "The QoS Map element"

At 965.63, change "QoS Map" to "QoS Map element"

At 966.11, change "QoS Map" to "QoS Map element" - note 2 occurences.

At 966.43, change "QoS Map" to "QoS Map element".

At 1787.14, change "QoS Map" to "QoS mapping"

At 1787.20, change "QoS Map" to "QoS mapping"

At 1787.21, change "QoS Map" to "QoS mapping"

At 1787.29, change "QoS Map" to "QoS mapping"

At 3576.23, change "QoS Map" to "QoS mapping"

At 107.31, change "QoS Map" to "QoS mapping"

| 5045 | 202.10 | 6.3.11.2.2 | DMG use of "operationalRateSet" instead of "basic MCS Set" is awkward. Firstly, the semantics relate to "basic" rather than "operational", secondly it relates to MCSs, not rates. | If this does not affect OTA signalling, and I don't think it does, change DMG throughout to use "basic MCS Set". |
| --- | --- | --- | --- | --- |
| 5267 | 149.59 | 6.3.3.3.2 | About the BSSBasicRateSet parameter of the BSSDescription set parameter of the MLME-SCAN.confirm primitive: "DMG BSS: Empty". But a parameter of a primitive is something that is present in the primitive, whether or not the invoker of the primitive has put any values in it. When a parameter (such as BSSBasicRateSet) is numerical, then some number or other will be present in the parameter. If no number is defined as meaning "Empty", then there is no such thing as "Empty" in the BSSBasicRateSet parameter. | Since the valid values of BSSBasicRateSet are 1-127, it would be easy enough to define the value 0 as meaning "Empty". So insert that definition into this description. (In the normative text below, references to BSSBasicRateSet being empty, or not, seem to be consistent with such a definition.) |

Discussion:

- In the context of DMG, it makes more sense to use Operational rate set. Also, this does not refer to OTA signalling.

- “Empty is defined for the Operational Rate Set.

Proposed Resolution:

Revised

At 201.58.  Change "Empty." to "The set of MCS indexes that the STA uses for communication within the BSS."

At 202.10. Change ""The set of MCS indexes that the STA uses for communication within the BSS." to "Empty".

| 6059 | 398.47 | 6.3.73.3.2 | "The mapping of Status Code received in the GAS Response frame is mapped to the corresponding Result Code in Table 8-45 (Status codes)." How does Table 8-45 show this mapping? Similar problem at P1612.9, and 1625.48. | Either add Result Code mappings to Table 8-45 (another column, probably), add a new table that shows these mappings, or reword these sentences to something the Standard tells the implementer how to do. |
| --- | --- | --- | --- | --- |

Discussion:

- Need to match ResultCode with Status codes.

Proposed Resolution:

| 6071 | 170.08 | 6.3.7.2.2 | The AssociateFailureTimeout parameter to MLME-ASSOCIATE.request (and MLME-REASSOCIATE.request) is described as being set to the value of the MIB attribute dot11AssociationResponseTimeOut. Why do we need a parameter to pass a value that is already in a MIB attribute? | Delete the AssociateFailureTimeout parameter, and delete the ReassociateFailureTimeout parameter. |
| --- | --- | --- | --- | --- |

Discussion:

- Looks like the MIB variable covers this behaviour and no MLME primitive parameters are needed

.

Proposed Resolution:

Revised.

At 169.32, remove  AssociateFailureTimeout,

At 170.08, remove the AssociationFailureTimeout row.

At 183.34, remove ReassocateFailutreTimeout,

At 184.08, remove the ReassociationFailureTimeout row.

At 1595.31, replace “the AssociateFailureTimeout expires” with “the association fails to complete within dot11AssociationResponseTimeOut”

At 1599.18,  replace “the ReassociateFailureTimeout expires” with “the reassociation fails to complete within dot11AssociationResponseTimeOut:”

| 6220 | 340.00 | 6.3.58 | The uses of t1-t4 are not clearly explained for the MLME-FINETIMINGMSMT primitives. In the .request, t1 and t4 are the values from the relevant previous .confirm. In the .confirm they are the values measured for the frame which has just gone out, and the Ack which came back (but there needs to be a way to signal "no Ack"). In the .indication t1 and t4 are from the frame but t2 and t3 are measured | Update the descriptions in the tables as it says in the comment |
| --- | --- | --- | --- | --- |

Discussion:

- Email sent to commenter.

Proposed Resolution:

| 6408 | 143.00 | 6 | "The MLME subsequently issues an MLME-foo.confirm" -- not if nothing is received and there's no timeout | Add "In the case that a response is received from the responder STA", as for MLME-ENABLEMENT, for any primitive where a response from the peer is needed and there's no timeout |
| --- | --- | --- | --- | --- |

Discussion:

- Looks like a reasonable change

Proposed Resolution:

Revised. Make the following changes:

At 162.58, 171.63, 186.09, 245.60, 416.22, 419.52, 426.04, change "The MLME subsequently..." to "In the case that a response is received from the responder STA, the MLME subsequently…"

| 6428 | 143.00 | 6 | The parameters called "BSSBasicRateSet" or "OperationalRateSet" or "SupportedRates" in the MLME SAP are not always rates, they can sometimes be BSS selectors | Rename the parameter to allow for this, and if there are any cases where a BSS selector is not allowed, reduce the range (from 1-127) |
| --- | --- | --- | --- | --- |

Discussion:

- Email sent to commenter

Proposed Resolution:

| 6604 | 144.00 | 6.3 | It says "Content of FT Authentication elements" | Replace all instances with "Content of FT Authentication frame" (cf. "Content of SAE Authentication frame") |
| --- | --- | --- | --- | --- |

Discussion:

- FT Authentication frames are  included in Authentication as well as Public Action frames. SAE are solely included in Authentication frames.

Proposed Resolution:

Reject. The FT Authentication is a set of elements that is included in either an Action frame or an Authentication frame.

| 6170 | 147.18 | 6.3.3.2.3 | From Guido Hiertz:The "Channel list" provides an "Ordered set of integers." In which order? Lowest first? Highest first? Which way are the integers ordered | Define the ordering of integers |
| --- | --- | --- | --- | --- |
| 6860 | 147.18 | 6.3.3.2.3 | The "Channel list" provides an "Ordered set of integers." In which order? Lowest first? Highest first? Which way are the integers ordered | Define the ordering of integers |

Discussion:

- Using set looks to be the correct convention at this point.

Proposed Resolution:

Revised. At 147.18, change "Ordered set" to "Set"