IEEE P802.11
Wireless LANs

| Proposed Resolution to CID 2-7 on TGaj D1.0 in LB127 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-01-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Shiwen HE | SEU |  |  | shiwenhe@seu.edu.cn |

Abstract

This document proposes resolutions CID 2-7 on TGaj D1.0 in LB127.

Editorial Comments

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 26.3.5.2 | 215 | 24 | T | Equation(26-10) | Sequence Z(n mod 256) in Equation(26-10) should be multipled by exp((jn/2) . That is, rSTF(nTc)=Z(n mod 256) exp(jn/2). |  |
| 3 | 26.3.5.3 | 215 | 42 | T | Equation(26-11) | All sequences Z(n mod 256) in Equation(26-11) should be multipled by exp(jn/2). |  |
| 4 | 26.3.5.4 | 216 | 4 | T | Equation(26-12) | All sequences Z(n mod 256) in Equation(26-12) should be multipled by exp(jn/2). |  |
| 5 | 26.3.5.5 | 216 | 23, 33 | T | Equation(26-13) and Equation(26-14) | All sequences Z(n mod 256) in Equation(26-13) and Equation(26-14) should be multipled by exp(jn/2). |  |
| 6 | 26.3.5.6 | 217 | 8, 18 | T | Equation(26-15) and Equation(26-16) | All sequences Z(n mod 256) in Equation(26-15) and Equation(26-16) should be multipled by exp(jn/2). |  |

Proposed resolution: Revised

We have revised the Equations (26-10)- to Equation (26-16) according the suggestions.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7 | 26.5.6.7 | 239 | 33-37 | T | Spatial Expansion Intex 3 | In order to avoding the unintentional beamforming, suggest to revise the transmission styple of one stream by three number of antennas by a betterl style, such as B. Hochwald, T. L. Marzetta and C. B. Papadias, "A transmitter diversity scheme for wideband CDMA systems based on space-time spreading", IEEE Journal on Selected Areas in Commun., vol. 19, no. 1, Jan. 2001, pp. 48-60. In additional, there are typos in the table. |  |

Proposed resolution: Revised

We have revised the spatial expansion mechanism according the suggestion in the given reference and corrected these typos.