IEEE P802.11
Wireless LANs

| Proposed Resolution to CID 120, 121, 125, 137, 138, 139, and 140 on TGaj D1.0 in LB217 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Shiwen HE | SEU |  |  | shiwenhe@seu.edu.cn |

Abstract

This document proposes resolutions CID 120, 121, 125, 137, 138, 139, and 140 on TGaj D1.0 in LB217.

Editorial Comments

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 120 | 6.3.26.2.2 | 14 | 3 | T | (scheduling) should not be in parenthesis since it only applies to scheduled operation | Reword to remove parenthesis and be grammatically correct |  |
| 121 | 6.3.26.3.2 | 14 | 45 | T | (scheduling) should not be in parenthesis since it only applies to scheduled operation | Reword to remove parenthesis and be grammatically correct |  |

Proposed resolution: Revised

Insert the following row before the ‘DMG TSPEC’ row in the untitled table in 6.3.26.2.2:

| 45MG TSPEC | 45MG TSPEC element | As defined in 8.4.2.133 (DMG and 45MG TSPEC element and 45MG TSPEC element) | Specifies the characteristics and QoS (scheduling) requirements of the 45MG allocation request. |
| --- | --- | --- | --- |

Insert the following rows before the ‘DMG TSPEC’ row in the untitled table in 6.3.26.3.2:

| 45MG TSPEC | 45MG TSPEC element | As defined in 8.4.2.133 (DMG and 45MG TSPEC element and 45MG TSPEC element) | Specifies the characteristics and QoS (scheduling) requirements of the 45MG allocation request. |
| --- | --- | --- | --- |

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 125 | 26 |  |  | T | It is desirable for silicon vendors to unify the QMG PHY format with 11ad formats and CDMG formats, even thought their Nyquist sampling rates (bandwidth) are different. | Unify the PHY preamble formats of control/SC/OFDMA modes with 11ad and CDMG. |  |

Proposed resolution: Reject

IEEE 802.11aj 45MG draft is a new draft and 45MG STA only operates on 45GHz band. So it does not need to coexist with the existing standards operating on 2.4/5/60GHz. Compared to the 11ad and CDMG, IEEE 802.11aj 45MG draft has many new features that 11ad and CDMG can’t support. Based on these, new PHY preamble formats of control/SC/OFDM modes are defined for 11aj 45MG STA.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 137 | 26.4.2 | 227 | 61 | T | In REVmc, the title "Frame format" is replaced by "PPDU format". | Change "26.4.2 Frame format" to "26.4.2 PPDU format". |  |

Proposed resolution: Accept

26.4.2. FramePPDU format

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 138 | 26.1.2 | 202 | 6 | T | ...This function is supported by the physical layer convergence procedure (PLCP)...". Here the "PLCP" is removed from REVmc. See "21.1.1 Scope" as reference. Change this paragraph accordingly. | Change this paragraph to " a) A function that defines a method of mapping the PHY service data units (PSDU) into a framing format (PPDU) suitable for sending and receiving PSDUs between two or more STAs." |  |

Proposed resolution: Revised

- A PHY convergence function, which adapts the capabilities of the physical medium dependent (PMD) system to the PHY service. This function is supported by the physical layer convergence procedure (PLCP), which defines a method of mapping the PSDUs into a framing format (PPDU) suitable for sending and receiving PSDUs between two or more STAs using the associated PMD system.A function that defines a method of mapping the PSDUs into a framing format (PPDU) suitable for sending and receiving PSDUs between two or more STAs.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 139 | 26.1.3.1 | 202 | 25 | T | "PLCP" is removed from REVmc. | Remove subclause 26.1.3.1 QMG PLCP sublayer. |  |

Proposed resolution: Accept

26.1.3.1 QMG PLCP sublayer

In order to allow the MAC to operate with minimum dependence on the PMD sublayer, a PHY convergence sublayer is defined (PLCP). The PLCP sublayer simplifies the PHY service interface to the MAC services.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 140 | 26.1.3 | 202 | 25 | T | "The QMG PHY contains three functional entities: the PHY convergence function (PLCP), the layer management function (PLME) and the PMD function.". "PLCP" and "PMD" are removed from REVmc. | Change this sentence to "The QMG PHY contains two functional entities: the PHY and the layer management function (PLME).". |  |

Proposed resolution: Accept

The QMG45MG PHY contains threetwo functional entities: the PHY convergence function (PLCP), the PHY function and the physical layer management function (PLME) and the PMD function. Each of these functions is described in detail in Error: Reference source not found to Error: Reference source not foundBoth of these functions are described in detail in Error: Reference source not found, Error: Reference source not found, Error: Reference source not found and Error: Reference source not found.

The QMG45MG PHY service is provided to the MAC through the PHY service primitives defined in Clause 7. The 45MG PHY service interface is described in Error: Reference source not found.