IEEE P802.11
Wireless LANs

| Proposed Comment Resolution Text for Clause 11.25a.6 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 165, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |
| James Lepp | BlackBerry | 1001 Farrar Road, Ottawa, Canada |  | jlepp@ieee.org |

Abstract

This document provides the 11aq Amendment Draft text for resolving LB216 CIDs 2334 and 2097. The baseline document used is D3.2.

- Proposed resolution text:

- Change clause 11.25a.6 in D3.2 as follows:

11.25a.6 Service hash procedures

A service hash is generated from a service name after all single-octet upper-case alphabetic characters (A-Z) in the service name are converted into corresponding low-case ones (a-z).

A service hash contained in the Service Hash field of the Service Hash element or used to map into the Bloom Filter Bit Array field is generated as follows:

service hash = L(SHA-256(service name), 0, 48).

A service hash contained in a Service Name subfield of the Service Information Request ANQP-element is generated as follows:

service hash = L(SHA-256(service name), 48, 48).

A service hash contained in a Service Name subfield of the Service Information Response ANQP-element is generated as follows:

service hash = L(SHA-256(service name), 96, 48).

For example, for the service name of "_ipp._tcp":

1) the service hash contained in the Service Hash field of the Service Hash element or used to map into the Bloom Filter Bit Array field is “bfd39037d25c” in hexadecimal;

2) the service hash contained in a Service Name subfield of the Service Information Request ANQP-element is “b99322def844” in hexadecimal; and

3) the service hash contained in a Service Name subfield of the Service Information Response ANQP-element is “48964b3a97f9” in hexadecimal.