IEEE P802.11
Wireless LANs

| Proposed resolutions to comments on clause 26.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ke Yao | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | yao.ke5@zte.com.cn |
| Bo Sun | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | Sun.bo1@zte.com.cn |
| Weimin Xing | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | Xing.weimin@zte.com.cn |

Abstract

This submission contains proposed comment resolutions to comments 4/28/2016 on D0.1.

The comments assigned to the author in Clause 26.2.2 are:

1925,  835, 870, 880, 1105, 2676, 2136, 2148, 2153, 2243, 2342, 2517.

The resolutions proposed in this document are marked with green.

The changes marked in this document are based on TGax Draft 0.1.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# Comments for Clause 26.2.2

## CID 1925, 2243, 2675, 2153, 1609, 870

| CID | Page. Line | Clause Number | Comment | Proposed Change | Comment Group |
| --- | --- | --- | --- | --- | --- |
| 1925 | 72.55 | 26.2.2 | Describe concept of Beam Change | Given that "beam change" is a new concept, provide a description or definition. | TX/RXVECTOR |
| 2243 | 72.63 | 26.2.2 | TX Vector BEAM_CHANGE definition | Otherwise set to 0 | TX/RXVECTOR |
| 2675 | 72.62 | 26.2.2 | BEAM_CHANGE is meaningful only in HE_SU PPDU. Therefore, there's no need to have TXVECTOR BEAM_CHANGE in other format. | Modify the text from "Set to 1" to "Not present". | TX/RXVECTOR |
| 2153 | 72.00 | 26.2.2 | BEAM_CHANCE: Applicablity of TXVECTOR/RXVECTOR not included | Include details | TX/RXVECTOR |
| 1609 | 72.58 | 26.2.2 | You can't have a shall in a NOTE | If this is normative, take away the "NOTE---". If this is informative, change "shall be" to "is" | TX/RXVECTOR |
| 870 | 72.55 | 26.2.2 | The content of the table26-1 is missing. | It is the first time to use the term "pre-HE and HE modulated fields". Suggest to add a further definition by quoting Figure26-21 in subclause 26.3.8. | TX/RXVECTOR |

Proposed resolution for CID 1925:

REVISED.

Reason:

Describe the concept of beam change and add the case when it’s set to 0.

Proposed resolution for CID 2243:

REVISED.

Reason:

Describe the concept of beam change.

Otherwise not present as presented by CID 2675.

Proposed resolution for CID 2675:

ACCEPTED.

Reason:

Modify as the proposed change.

Proposed resolution for CID 2153:

REVISED.

Reason:

Add "Y" to both TXVECTOR and RXVECTOR.

Proposed resolution for CID 1609:

REVISED.

Reason:

Since beamforming for pre-HE portion is optional, I suggest to describe in what cases this function is used instead of the current description of when to keep the original approach (without beamforming). Modify as follows.

Proposed resolution for CID 870:

ACCEPTED.

Reason:

Add reference there.

Instruction to Editor:

Please make the following changes in TGax D0.1 P72L 55 Table 26-1 under subclause 26.2. 2

Table 26-1—TXVECTOR and RXVECTOR parameters

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| BEAM_CHANGE | FORMAT is HE_SU | Indicates if beam changes or not between pre-HE and HE modulated fields. Refer to subclause 26.3.8 for the definition of pre-HE and HE modulated fields. 1 indicates that the beam changes between the two portions. 0 indicates that the beam does not change beween the two portions. NOTE—BEAM_CHANGE may be set to 0 if NSS<=2 or the PPDU is not the first PPDU in a TXOP. | Y | Y |
|  | Otherwise | Not present |  |  |

## CID 835

| 835 | 73.04 | 26.2.2 | The range of BSS color can be 0 to 63 with 6bit BSS color field defined in HE-SIG-A. | Modify "TBD" by "63". | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID 835:

ACCEPTED.

Reason: same as what the commenter mentioned.

Instruction to Editor:

Please make the following changes in TGax D0.1 P73L 3 Table 26-1 under subclause 26.2. 2

Table 26-1—TXVECTOR and RXVECTOR parameters

| Parameter | Condition | Value | TXVECTOR | RXVECTOR |
| --- | --- | --- | --- | --- |
| BSS_COLOR | FORMAT is HE_SU or HE_MU or HE_EXT_SU or HE_TRIG. | Set to a value of the AP’s choosing within the range 0 to 63 (see Error: Reference source not found (STA ID, Uplink Flag and BSS Color in HE PPDUs)). | Y | Y |
|  | Otherwise | Not present | N | N |

## CID 880

| 880 | 72.07 | 26.2.2 | What if "Q" matrix in the data portion does not change? Is the BEAM_CHANGE still set to 1 for Nss>2 or the PPDU is the first PPDU in the TXOP | In Table 26-1, BEAM_CHANGE field, is the BEAM_CHANGE always set to 1 for the cases of Nss>2 or the 1st PPDU in TXOP? even though the Q matrix does not change between pre-HE and HE-modulated fields | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID 880:

REJECTED.

For Nss>2 or the first PPDU in the TXOP, beam always changes between pre-HE and HE modulated part. That means pre-HE is processed without beamforming, just like non-HT and there is no Q matrix actually for pre-HE.

## CID 1105, 2342

| 1105 | 72.58 | 26.2.2 | Parameter "BEAM_CHANGE" should be set to 1 in the case of not "N_ss > 2" but "N_ss >= 2." | As in comment. | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |
| 2342 | 72.58 | 26.2.2 | Parameter "BEAM_CHANGE" should be set to 1 in the case of not "N_ss > 2" but "N_ss >= 2." | As in comment. | TX/RXVECTOR |

Proposed resolution for CID 1105 & 2342:

REJECTED.

Reason:

Since the performance showed that there is benefit when Nss=2, so only need to exclude Nss >=3. Refer to "11-15-1322-00-00ax"

## CID 2676

| 2676 | 73.30 | 26.2.2 | "MU" may also indicate per user parameters for HE trigger-based PPDU, especially for RXVECTOR column. For example, SNR/FEC_CODING/MCS/etc parameters for HE trigger-based PPDU need to be per-user based. | Modify RXVECTOR column for per-user parameters when the FORMAT is HE_TRIG from "Y" to "MU", and modify the NOTE accordingly. | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID 2676:

REVISED.

Reason:

Modify the note as following.

There is no need to modify “Y” to “MU” for the format of HE_TRIG, because the current parameters of HE_TRIG are PE_DURATION and BSS_COLOR, both of which are same for different STAs in HE_TRIG reception. The SIGA of different STAs should be same and AP may choose to ignore the content in SIGA which AP already knows about.

Instruction to Editor:

Please make the following changes in TGax D0.1 P73L 29 in Table 26-1 under subclause 26.2. 2

Table 26-1—TXVECTOR and RXVECTOR parameters

NOTE 1—In the “TXVECTOR” and “RXVECTOR” columns, the following apply: Y = Present; N = Not present; O = Optional; MU indicates that the parameter is present once for an HE SU PPDU and HE extended range SU PPDU and present per user for an HE MU PPDU. For HE trigger-based PPDU, MU indicates that the parameter is present once in TXVECTOR, and MU indicates the parameter is present per user in RXVECTOR. Parameters specified to be present per user are conceptually supplied as an array of values indexed by u, where u takes values 0 to NUM_USERS-1.

## CID 2136

| 2136 | 72.59 | 26.2.2 | Can beamchange be used with STBC? Then Nss=1 would max Nsts_total to 2 (Fig 26-7). Does Nsts_total in (26-14) refer to total number of STS in data fields? | Clarify Nsts_total definition and STBC+beamchange | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID 2136:

REJECTED.

Reason:

“Can beamchange be used with STBC?”

- Beamforming can be used in pre-HE-STF portion when STBC is used in data potion.  Since STBC is not used in HE-LTF1, pre-HE-STF portion which should keep consistence with HE-LTF1 can not use STBC either. But they are independent settings.

“Does Nsts_total in (26-14) refer to total number of STS in data fields?”

- yes, the N_STS in equation 26-14 should be the same as that in HE-LTF1 and equals the num of STS in data field.

“Clarify Nsts_total definition and STBC+beamchange”

3. Therefore, the NSTS_total actually has the same meaning as before, no need to clarify further.

## CID 2148

| 2148 | 72.00 | 26.2.2 | Is this support AP or STA modes only? what about ad-doc (peer-to-peer modes) ? |  | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID :

REJECTED.

Reason:

There is no such limitation for only AP or STA mode. Ad-hoc can also apply beam change if needed.

## CID 2517

| 2517 | 72.58 | 26.2.2 | Is there a notion of "first PPDU in a TXOP" in a PHY clause? | How does PHY know if this is the first PPDU in a TXOP? For example, do we need to add a TXVECTOR parameter indicating whether a PPDU is the first PPDU in a TXOP? | TX/RXVECTOR |
| --- | --- | --- | --- | --- | --- |

Proposed resolution for CID 2517:

REJECTED.

Reason:

The MAC layer knows if this PPDU is the first or not and decides the value for the BEAM_CHANGE parameter. PHY needn't to know that info. Here is just an information note for reader.