IEEE P802.11
Wireless LANs

| 802.11 REVmc Initial Sponsor ballot – proposed comment resolution for CID 7742 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Aldana | Qualcomm Corporation |  |  | caldana@qca.qualcomm.com |

Abstract

This document contains proposed resolution to CID 7742 and uses REVmc_Draft 5.3 as its baseline.

## History

R0: CIDs  7742

# Comments

| CID | Clause Number | Page | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 7742 | 11.24.6.3 |  | "The responding STA's selection of the Number of Bursts Exponent value shall be 0 when the initiating STA requests it to be 0." should be a "should". An AP is required to support non-ASAP since a STA is not required to be ASAP-capable. ASAP=1 can be problematic for scheduling. | I think this came in the context of a discussion with Carlos ALDANA, so I hope he can work out what this was about! |

Discussion:

It is is important to state expected behavior for various AP and STA combinations regarding ASAP and non-ASAP FTM sessions.

Proposed Resolution: Revised.

Proposed changes:  In Section 11.24.6.3, add the language shown in red below

If the request was successful

— The responding STA’s selection of the format and bandwidth in the Format And Bandwidth field should be the same as that requested by the initiating STA. The responding STA shall not choose a bandwidth wider than requested. The responding STA shall not choose a VHT format if HT-mixed or non-HT format was requested. The responding STA shall not choose an HT format if non-HT format was requested.

— An initiating STA performing a fine timing measurement procedure with a responding STA that is an AP shall support non-ASAP operation.

— A responding STA that is an AP shall support and select non-ASAP operation when so requested by an initiating STA.

— An initiating STA performing a fine timing measurement procedure with a responding STA that is not an AP shall support ASAP operation.

— A responding STA that is not an AP shall support and select ASAP operation when so requested by an initiating STA.

— If the responding STA is ASAP capable, the responding STA’s selection of ASAP should be the same as that requested by the initiating STA.