IEEE P802.11
Wireless LANs

| 802.11 REVmc Initial Sponsor ballot – modification to Figure 9-680 in Section 9.6.8.33 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Aldana | Qualcomm Corporation |  |  | caldana@qca.qualcomm.com |

Abstract

This document contains a modification to Figure 9-680 in REVmc_Draft 5.3.

## History

R0: Initial submission

Discussion:

Figure 9-680 has the FTM Synchronization Information element (Element ID=255) in an undesired order.   It should be at the end of figure, not in the middle.  This document proposes a revised Figure to address this.

- Fine Timing Measurement frame format(#46)

NOTE TO EDITOR:  Please replace the Figure below with the one that follows.

|  | Category | Public(M56) Action | Dialog Token | Follow Up Dialog Token | TOD | TOA |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 | 6 |
|  | TOD Error | TOA Error | FTM Synchronization Information (optional) | LCI Report (optional) | Location Civic Report (optional) | Fine Timing Measurement Parameters(#3465) (optional) |
| Octets: | 2 | 2 | variable (#6419) | variable (#2403) | variable (#2403) | variable (#2164) |
| - Fine Timing Measurement Action field(#2042) format |  |  |  |  |  |  |

|  | Category | Public(M56) Action | Dialog Token | Follow Up Dialog Token | TOD | TOA |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 | 6 |
|  | TOD Error | TOA Error | LCI Report (optional) | Location Civic Report (optional) | Fine Timing Measurement Parameters(#3465) (optional) | FTM Synchronization Information (optional) |
| Octets: | 2 | 2 | variable (#6419) | variable (#2403) | variable (#2403) | variable (#2164) |
| - Fine Timing Measurement Action field(#2042) format |  |  |  |  |  |  |