IEEE P802.11
Wireless LANs

| LB217 Comment Resolutions to CID 51, 104, and 105 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jianhan Liu | Mediatek |  |  | jianhan.liu@mediatek.com |

Abstract

This document proposes resolutions to TGaj LB217 CIDs: 51, 104 and 105.

Technical Comments

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 51 | 25.4.3.2.1 | 178 | 57 | T | All reserved bits from the Control PHY header are being used. This is a bad design, because it does not allow for any future extension. For example, 11ay will not be able to use any reserved bit. | Use only a single reserved bit. | See the follows for details. |

Proposed resolution: Rejected.

Discussions:

To enable the flexible spreading for efficiency, there are two extra spreading modes introduced. There in fact do have a reserved mode for 11ay (using the setting “11”). 11ay already adopted to use the “11” reserved mode for expansion.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 104 | 25.4.3.2.1 | 176 | 44 | T | CPHY scrambler is very short. | Scrambler should be at least 7bits | See the follows for details. |

Proposed resolution: Rejected.

Discussion:

4-bits are just the initial scrambler state of the scrambler. The sequence that is generated using the initial scrambler state is long enough. In DMG, the same 4-bits initial scrambler state is used.

| CID | Clause | Page | Line | Type | Comment | Proposed Change | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 105 | 25.4.3.2.1 | 178 | 59 | T | CPHY has no reserved bits, only one set in "Robust PHY Mode Indication/Reserved bits" | Add at least 8 reserved bits | See the follows for details |

Proposed resolution: Rejected.

Discussions:

To enable the flexible spreading for efficiency, there are two extra spreading modes introduced. There in fact do have a reserved mode for 11ay (using the setting “11”). 11ay already adopted to use the “11” reserved mode for expansion.