IEEE P802.11
Wireless LANs

| 802.11 Resolutions to CID #7074 and #7077 (relative to Draft 5.0) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-05-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ganesh Venkatesan | Intel Corporation | 2111 NE 25th Ave, Hillsboro, OR 97124 | 503 334 6720 | ganesh.venkatesan@intel.com |

Abstract

This submission addresses the following:

| 7074 | 586.05 | 9.2.4.6.2 | " all or part of an MSDU or A-MSDU should discard the MSDU or any MSDUs containedwithin the A-MSDU in preference toMSDUs carried in MPDUs whose DEI subfield is equal to 0." - this is clearly behaviour, and doesn't belong in Clause 9. | Move to Clause 10 | MAC | Submission Required | Clarity/consistency (Med scope) Mpve statements related behaviour based on DEI setting to Clause-10. |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 7077 | 1000.45 | 9.4.2.121 | " If there are insufficient resources, a STA should discard theMSDUs or A-MSDUs of a TS with a Drop Eligibilitysubfield equal to 1," - this is not frame format. Should not be in clause 9. | Move to Clause 10/11 | MAC | Submission Required | EDITOR: 2016-02-04 15:50:44Z - I agree the text should not be here. I reviewed uses of "Drop Eligibility" and there does not appear to be any statement about how to use this field, except in certain conditions (e.g. EDCAF collision), to choose to drop an MSDU. Needs an expert. Suggest Ganesh. Transferring to MAC Move statements related to behavior based on DEI setting to Clause-10 |

Editor: Edit the last paragraph of 9.2.6.2 as shown below.

### 9.2.4.6.2 HT variant

### The DEI subfield of the HT Control field is 1 bit in length and is set by the transmitting STA to indicate the suitability of the corresponding MSDU or A-MSDU to be discarded if there are insufficient resources at the receiving STA. See 10.9 (MPDControl field operation) and 11.27.2 (SCS procedures). In an MMPDU, the DEI subfield is reserved. The mechanisms for

Editor: Edit 9.2.6.3 as shown below.

#### 9.2.4.6.3 VHT variant

The format of the HT Control Middle subfield of the VHT variant HT Control field is shown in Figure9-12 (HT Control Middle subfield of the VHT variant HT Control field).

|  | B1 | B2 | B3 B5 | B6 B8 | B9 B23 | B24 B26 | B27 | B28 | B29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DEI | MRQ | MSI/STBC | MFSI/GID-L | MFB | GID-H | Coding Type | FB Tx Type | Unsolicited MFB |
| Bits | 1 | 1 | 3 | 3 | 15 | 3 | 1 | 1 | 1 |

Figure 9-12—HT Control Middle subfield of the VHT variant HT Control field

The subfields of VHT variant HT Control field are defined in Table9-17 (VHT variant HT Control field subfields).

Editor: Add a new row to top of Table 9-17 as shown below:

| Subfield | Meaning | Definition |
| --- | --- | --- |
| DEI | Drop Eligibility Indicator | Set to 1 by the transmitting STA to indicate the suitability of the corresponding MSDU or A-MSDU to be discarded if there are insufficient resources at the receiving STA. See 10.9 (HT Control field operation) and 11.27.2 (SCS procedures). In an MMPDU, the DEI subfield is reserved. |
| MRQ | VHT-MCS Feedback Request | Set to 1 to request VHT-MCS feedback (solicited MFB); otherwise, set to 0. |

Editor: Edit the last paragraph of 9.4.2.121 as shown below.

#### 9.4.2.121 Intra-Access Category Priority element

The Drop Eligibility subfield is 1 bit in length and is used to indicate the suitability of this TS to be discarded if there are insufficient resources. See 11.26.2 (QMF policy advertisement and configuration procedures)

Editor: Edit NOTE 2 in 10.3.2.9 as shown below:

### 10.3.2.9 Ack procedure

NOTE 2—The receiver STA performs the Ack procedure on all received frames requiring acknowledgment, even if an MSDU or A-MSDU is carried partly or wholly within the frame and is subsequently discarded due to drop eligibility (see DEI subfield in 9.2.4.6.2 (HT Variant) and 9.2.4.6.3 (VHT Variant)).

Editor: Add a new paragraph to 10.9 HT Control field operation as shown below:

## 10.9 HT Control field operation

If the value of dot11HTControlFieldSupported is true, a STA shall set the +HTC-HT Support subfield of the HT Extended Capabilities field of the HT Capabilities element to 1 in HT Capabilities elements that it transmits. If the value of dot11VHTControlFieldOptionImplemented is true, a STA shall set the +HTC-VHT Support subfield of the VHT Capabilities Information field of the VHT Capabilities element to 1 in VHT Capabilities elements that it transmits.

A STA in which at least one of dot11RDResponderOptionImplemented, dot11MCSFeedbackOptionImplemented and dot11AlternateEDCAActivated is true, shall set dot11HTControlFieldSupported or dot11VHTControlFieldOptionImplemented or both to true.

An HT variant HT Control field shall not be present in a frame addressed to a STA unless that STA declares support for +HTC-HT in the HT Extended Capabilities field of its HT Capabilities element (see 9.4.2.56 (HT Capabilities element)).

A VHT variant HT Control field shall not be present in a frame addressed to a STA unless that STA declares support for +HTC-VHT in the VHT Capabilities Information field of its VHT Capabilities element. NOTE—An HT STA that does not support +HTC (HT or VHT variant) that receives a +HTC frame addressed to another STA still performs the CRC on the actual length of the MPDU and uses the Duration/ID field to update the NAV, as described in 10.3.2.4 (Setting and resetting the NAV).

If the HT Control field is present in an MPDU aggregated in an A-MPDU, then all MPDUs of the same frame type (i.e., having the same value of the Type subfield of the Frame Control field) aggregated in thesame A-MPDU shall contain an HT Control field. The HT Control field of all MPDUs containing the HTControl field aggregated in the same A-MPDU shall be set to the same value.

If the HT Control field is present in an MPDU, the DEI subfield in the HT Control field of the received MPDU provides information on the eligibility of the contents of the received MPDU. When there are insufficient resources in a STA, the STA arbitrarily discards frames in order to recover from the insufficient resources condition. With the information from the DEI subfield, a STA can selectively drop frames with the DEI subfield set to 1 in preference to frames with the DEI subfield set to 0, under insufficient resources condition. Note that this may not help in the recovery in all conditions, and the STA will still have to fallback to the arbitrary discard strategy. The mechanisms for determining whether the resources are insufficient or when to discard MSDUs or A-MSDUs are beyond the scope of this standard.