IEEE P802.11
Wireless LANs

| Some REVmc d6 comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-07-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jouni Malinen | Qualcomm |  |  | jouni@qca.qualcomm.com |

Abstract

This document proposed comment resolutions to some P802.11-REVmc/D6.0 comments.

Rev 1: address more comments

Rev 2: updates based on the first discussion in the group: hide text of ready-for-motion CIDs, update needs-more-work comments

Rev 3: updates based on the second discussion in the group, hide text of ready-for-motion CIDs

Rev 4: even more fine-tuning of Public Action frame addressing text

# CID 8144

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 9.4.2.25.2 Page: 224 Line: 23 | What is the Cipher Suite Selector used for? It appears that the intent is that you can configure keys for various possible cipher suites, and then the MAC is to use the one which ends up being used for the link | At 224.37 change "(as defined by the Key Type, Key ID and Address elements)" to "(as defined by the Key Type, Key ID, Address and Cipher Suite Selector parameters)". Change the following bullet to "The MAC installs the key with the associated Key ID such that received frames for the cipher suite indicated by the Cipher Suite Selector, of the appropriate type, and containing the matching Key ID are processed using that key and its associated state information." |

Discussion

REVmc/D6.0 page 224 (SetKeyDescriptor for MLME-SETKEYS.request):

Proposed resolution for CID 8144:

Accepted.

# CID 8059

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 6.3.19.1.2 Page: 224 Line: 3 | It is not clear whether the SME picks the Key ID for transmission of encrypted frames (using the Key ID parameter of the MLME-SETKEYS.request) or whether the MAC does so. Additionally there are various issues with the security pseudocode | Make the changes shown under CID 7572 in 16/0276r14 |

Discussion

CID 7572 resolution: “REVISED (MAC: 2016-05-27 15:32:42Z): Incorporate the changes under CID 7572 at 223.44 and 224.34  in 11-16/276r12 (https://mentor.ieee.org/802.11/dcn/16/11-16-0276-12-000m-resolutions-for-some-comments-on-11mc-d5-0-sbmc1.docx).”

This incorporated only a small part of the 17/0276r14 changes for CID 7572+7604. The parts that were not applied are:

At 224.15, 484.5 and 484.46 change “N/A” to

“0–3 shall be used

with WEP, TKIP,

CCMP, and

GCMP;

4–5 with BIP; and

6–4095 are

reserved”.

(Jouni: Though, it looks like this was applied into D6.0, so maybe another comment pulled those in.)

Change 12.9.2.2 onwards as follows (followed by a significant cleanup of the Tx pseudo-code subclauses)

Adhoc notes for CID 7572 note this about discussion and motions:

MAC: 2016-05-27 15:17:19Z - Reviewed 11-16/276r11.  Minor changes, revved to r12.  Some expressed concern over the scope of the changes in security pseudo-code at this late stage in the Revision.  Brought to motion on telecon - motion failed 6-3-1.

Suggested: make only the changes:

Change 223.44 as follows: — If the Direction element of athe SetKeyDescriptor indicates Transmit or Both then the MAC uses the

key information (as defined by the Key and Length elements) for the transmission of all subsequent frames to which the key applies (as defined by the Key Type, Key ID and Address elements).

Change 224.34 as follows:  Receipt of this primitive causes the MAC to delete all the temporal keys identified by eachthe DeleteKeyDescriptors in the Keylist (as defined by the Key Type, Key ID and Address elements), and to cease using them.

Motion passes 4-0-6.

REVmc/D6.0 page 224 (SetKeyDescriptor for MLME-SETKEYS.request):

Proposed resolution for CID 8059:

Rejected. Some of the requested changes are already present in the balloted draft. The changes to the Tx pseudo code were previously discussed in the previous ballot and the motion to accept them failed. The group did not have consensus to accept the changes in this ballot either. The strawpoll results were 1-12-5.

# CID 8111

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.6.1.1 Page: 1966 Line: 6 | There are 5 instances of SAs being described as cached for their lifetime (1967.4, 1968.13, 1968.32, 1971.15, 1971.34). This is misleading: they simply have that lifetime | Change the sentences for the 5 instances referenced as follows, respectively: "PMKSAs have a certain lifetime."; "Mesh PMKSAs have a certain lifetime."; "PTKSAs have the same lifetime as the PMKSA or PMK-R1 security association, whichever comes first." ; "STKSAs have the same lifetime as the SMKSA or the STSL, whichever comes first."; "The TPKSA has the lifetime indicated in the TPK handshake or the lifetime of the TDLS direct link, whichever comes first." |

Discussion

The proposed changes look reasonable, but are in a format that makes it a bit complex to review. The proposed resolution provides more detailed list of changes for easier review and editing. The end result should be identical to accepting this comment.

REVmc/D6.0 page 1967:

REVmc/D6.0 page 1968:

REVmc/D6.0 page 1971:

Proposed resolution for CID 8111:

Revised. Do the proposed changes: On page 1967 line 4, replace “PMKSAs are cached for up to their lifetimes” with “PMKSAs have a certain lifetime”. On page 1968 line 13, replace “Mesh PMKSAs are cached for up to their lifetimes” with “Mesh PMKSAs have a certain lifetime”. On page 1968 line 32, replace “PTKSAs are cached for the life of the PMKSA or PMK-R1 security association” with “PTKSAs have the same lifetime as the PMKSA or PMK-R1 security ssociation, whichever comes first”. On page 1971 line 15, replace “STKSAs are cached for the life of the SMKSA or until the STSL ends, whichever comes first” with “STKSAs have the same lifetime as the SMKSA or the STSL, whichever comes first”. On page 1971 line 34, replace “The TPKSA is cached per the lifetime indicated in the TPK handshake or until the TDLS direct link is torn down, whichever comes first” with “The TPKSA has the lifetime indicated in the TPK handshake or the lifetime of the TDLS direct link, whichever comes first”.

# CID 8112

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.6.1.1 Page: 1966 Line: 6 | Some of the SAs do not have text before the bullets to describe their lifetime | Before the last sentence before the bulleted list in 12.6.1.1.3, add "It has a certain lifetime". Before the last sentence before the second bulleted list in 12.6.1.1.4, add "It has a certain lifetime". Change the last sentence before the bulleted list in 12.6.1.1.7 to "It has a certain lifetime.". Before the last sentence before the bulleted list in 12.6.1.1.8 add "It has the same lifetime as the BSS, unless superseded." After the last sentence of the first para in 12.6.1.1.9 add "An IGTKSA has the same lifetime as the BSS, unless superseded." Before the last sentence before the second bulleted list in 12.6.1.1.11, add "It has a certain lifetime". |

Discussion

The change in 12.6.1.1.7 would replace “The mesh TKSA shall be deleted when the lifetime expires” with a sentence that has no shall requirement. This may not be desirable unless there is a different location in the standard with the shall delete language. However, such text seems to exist in 12.6.16 (RSNA key management in an MBSS): “A mesh TKSA is logically a child of the mesh PMKSA. A mesh TKSA shall be deleted if the corresponding mesh PMKSA, which was used by the AMPE to create it, is deleted.”.

In addition, there is only one bulleted list in 12.6.1.1.11, so the proposed change needs some edits, if this were to be accepted (revised).

REVmc/D6.0 page 1967:

REVmc/D6.0 page 1969:

REVmc/D6.0 page 1970:

REVmc/D6.0 page 1971:

Proposed resolution for CID 8112:

Revised. At page 1967 line 26 (12.6.1.1.3), add after the “This security association is bidectional” sentence: "It has a certain lifetime". At page 1967 line 51 (12.6.1.1.4) add after the “This security association is bidirectional” sentence: "It has a certain lifetime". At page 1969 lines 6-7 (12.6.1.1.7), replace “The mesh TKSA shall be deleted when the lifetime expires” with "It has a certain lifetime". At page 1969 line 39 (12.6.1.1.8) add before the “A GTKSA consists of the following elements” sentence: "It has the same lifetime as the BSS, unless superseded." At page 1969 line 65 (12.6.1.1.9), add at the end of the paragraph: "An IGTKSA has the same lifetime as the BSS, unless superseded." At page 1970 line 60 (12.1.1.11), add before “The SMKSA consists of the following elements” sentence: "The SMKSA has a certain lifetime".

# CID 8146

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 6.3.19.1.2 Page: 224 Line: 18 | The use of the Receive Sequence Count is not described | Change the second bullet of 6.3.19.1.4 to "The MAC installs the key with the associated Key ID such that received frames of the appropriate type and containing the matching Key ID are processed using that key and its associated state information, subject to validation based on the Receive Sequence Count." |

Discussion

This is talking about the Receive Sequence Count parameters in SetKeyDescriptor passed to MLME-SETKEYS.request primitive. The proposed change adds “, subject to validation based on the Receive Sequence Count” to the end of the identified sentence.

REVmc/D6.0 page 224:

Proposed resolution for CID 8146:

Accepted.

# CID 8176

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.7.9.4.4 Page: 2042 Line: 28 | "The TDLS responder STA shall discard the message, the TDLS responder STA shall abandon the TPK handshake" is garbled | Delete "TDLS responder STA shall discard the message, the" in the cited text |

Discussion

This is not garbled; this is a list of items that the TDLS responder STA shall do with the two items that the comment identifies being the first two items in that list (“shall discard the message” and “shall abandon the TPK handshake..”).

REVmc/D6.0 page 2042:

Proposed resolution for CID 8176:

Rejected. The identified text is not garbled; it is a list of actions that the TDLS responder STA shall do if any of the following checks fails.

# CID 8175

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.7.9.4.4 Page: 2042 Line: 30 | ", and delete existing TPK handshake key state for this sequence" -- what is a TPK handshake key state? | Delete the cited text |

Discussion

I’d assume “key state” is referring to the keys and temporary key-like material derived during the TPK handshake up to this point. Those keys need to be deleted, so the cited text should not be deleted. There is another instance of “key state” in the standard, but it might be clearer to simply refer to all TPK handshake state for this instance of the handshake.

REVmc/D6.0 page 2042:

Proposed resolution for CID 8175:

Revised. On page 2042 line 31, replace “delete existing TPK handshake key state” with “delete existing TPK handshake state”.

# CID 8303

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.7.6.7 Page: 2022 Line: 58 | Re CID 7249 -- why is the GTK "encapsulated" but the IGTK not so? | Delete "encapsulated" at 2010.4, 2010.20, 2010.37, 2013.16, 2014.22, 2018.10, 2018.34, 2022.58, 2025.6, 2025.9 (fixing any preceding indefinite article) |

Discussion

“Encapsulation” here might refer to the GTK being encrypted in the frame. IGTK is protected in similar manner, so it should really have the same language here. Whether to include “encapsulation” or not is subject to some debate, but it does not seem to do much harm to delete it. It should be noted that the indicated location 2025.9 is actually proposing “encapsulated” to be removed from IGTK (and 2013.16 from SMK). 2010.4 is talking about “encapsulated PMKID” which is not even protected, so the theory above regarding protection as the source for “encapsulation” does not work there.

REVmc/D6.0 page 2010:

REVmc/D6.0 page 2013:

REVmc/D6.0 page 2014:

REVmc/D6.0 page 2018:

REVmc/D6.0 page 2022:

REVmc/D6.0 page 2025:

Proposed resolution for CID 8303:

Accepted.

# TOPIC: Mesh

Discussion:

Most of (and all of my) mesh comments below came from implementation review that identified multiple mismatches between an implementation and the standard. That review and following implementation fixes ended up finding incomplete or unclear areas in the standard. While some of these comments might be strictly speaking out-of-scope of this recirculation, they do point out at significant issues in the standard and it would be good to get those fixed rather sooner than later.

# CID 8113

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.6.1.1.7 Page: 1969 Line: 22 | What is the lifetime of a Mesh TKSA? | Add ", as defined in 12.x.y.z" to the end of the referenced line |

Discussion

Mesh TKSA has the same lifetime as the mesh PMKSA that was used to create it. This is described in 12.6.16.

REVmc/D6.0 context (page 1969):

page 1985:

Proposed resolution for CID 8113:

Revised. On page 1969 line 22, replace “Lifetime” with “Lifetime, as defined in 12.6.16”.

# CID 8115

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 12.6.1.1.10 Page: 1970 Line: 47 | What is the lifetime of a Mesh GTKSA? | Add ", as defined in 12.x.y.z" to the end of the referenced line |

Discussion

If a Mesh GTKSA had a defined lifetime, the comment could be resolved with something like this: Revised. On page 1970 line 47, replace “Lifetime” with “Lifetime, as defined in 12.x.y.z“.

However, Mesh GTKSA does not really have a separate lifetime, i.e., it is valid until deleted/replaced as part of mesh termination or GTK rekeying (with the same Key ID and from the same peer as far as the “receive Mesh GTKSA” is concerned). While this could be explicitly described, it is likely simplest to just delete the Lifetime item from Mesh GTKSA.

Rev 2: Verified whether the other comments (mainly, CID 8111) have impatch on the mesh GTKSA lifetime. It appears that they don’t. Mesh GTKSA was not included in CID 8111 changes and there does not seem to be any other comment either that would modify mesh GTKSA (apart from editorial changes to its name) or this 12.6.1.1.10 clause. As such, it looks safe to remove the Lifetime item from mesh GTKSA. It should also be noted that as pointed during discussion, GTKSA does not include the Lifetime item in its list of included elements.

REVmc/D6.0 context (page 1970):

Proposed resolution for CID 8115:

Revised. On page 1970 line 47, delete the Lifetime item.

# CID 8325

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.6.3 Page: 2146 Line: 14 | The contents of the Authentication Mesh Peering Exchange element in Mesh Group Key Inform frames is described as far as the GTKdata field is concerned. However, there is no note about the IGTKdata field here. That field should be noted here as it would need to be present when management frame protection is used. | On page 2146 line 16, add a new item to the list after the GTKdata field item: "-- If management frame protection is used, the IGTKdata field shall be present." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm.. My proposed action on this should not come as a surprise.

Rev 2: Added IGTKdata details included in the existing text for GTKdata to make the new item more consistent. 14.5.4 is not as detailed for IGTKdata, so referencing the AMPE element definition instead.

REVmc/D6.0 context:

Page 2145:

Page 2146:

Proposed resolution for CID 8325:

Revised. On page 2146 line 16, add a new item to the list after the GTKdata field item:

"-- If management frame protection is used, the IGTKdata field shall be present and shall contain the data for the IGTK from IGTK source. The components of the IGTKdata are specified in 9.4.2.118 (Authenticated Mesh Peering Exchange element)."

# CID 8324

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.6.4 Page: 2147 Line: 1 | The contents of the Authenticated Mesh Peering Exchange element in Mesh Group Key Acknowledge frames is somewhat unclear due to the undefined "shall be left blank" language. What does this mean? Is the meaning different for the Selected Pairwise Cipher suite field (page 2147 line 1) and the GTKdata field (page 2147 line 11)? As far as the element format definition is concerned, the Selected Pairwise Cipher Suite field is a field with a fixed length (4 octets). Would being "blank" mean this is a sequence of four 0x00 octets? GTKdata on the other hand is an optional, variable length field. What would it mean for it to be "blank"? Wouldn't it be clearer to say that the GTKdata field shall not be present? And what about the IGTKdata field that would optionally follow the GTKdata field? There is no comment about that here at all.. | On page 2147 line 1, replace "The Selected Pairwise Cipher Suite field shall be left blank" with "The Selected Pairwise Cipher Suite field shall be set to four octets of zero." On page 2147 line 11, replace "The GTKdata field shall be blank" with "The GTKdata field shall not be present". Add the following item after the page 2147 line 7 (GTKdata): "-- The IGTKdata field shall not be present." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm.. My proposed action on this should not come as a surprise.

rev 2: Address one more “shall be left blank” at page 2146 line 3

REVmc/D6.0 context:

Page 2146:

Page 2147:

Proposed resolution for CID 8324:

Revised.

On page 2146 line 2, replace "The Selected Pairwise Cipher Suite field shall be left blank" with "The Selected Pairwise Cipher Suite field shall be set to four octets of zero."

On page 2147 line 1, replace "The Selected Pairwise Cipher Suite field shall be left blank" with "The Selected Pairwise Cipher Suite field shall be set to four octets of zero."

On page 2147 line 11, replace "The GTKdata field shall be blank" with "The GTKdata field shall not be present".

Add the following item after the page 2147 line 7 (GTKdata): "-- The IGTKdata field shall not be present."

# CID 8322

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 9.4.2.118 Page: 999 Line: 45 | Figure 9-488 (Authenticated Mesh Peering Exchange element format) identifies IGTKdata as being optional. However, GTKdata and Key Replay Counter fields are similarly optional as noted in the text description (page 1000 lines 1 and 7). | On page 999 line 45 (Figure 9-488): - replace "Key Replay Counter" with "Key Replace Counter (optional)" - replace "GTKdata" with "GTKdata (optional)" |

Discussion

As MarkR pointed out, there’s a typo in the proposed change (“Key Replace Counter” should be “Key Replay Counter”). Other than that, note that this is my own comment, so someone else might want to take a quick look to confirm.. My proposed action on this should not come as a surprise.

REVmc/D6.0 context:

Page 999:

Page 1000:

Proposed resolution for CID 8322:

Revised. On page 999 line 45 (Figure 9-488):

- replace "Key Replay Counter" with "Key Replay Counter (optional)"

- replace "GTKdata" with "GTKdata (optional)"

# CID 8321

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.5.5.3.1 Page: 2139 Line: 43 | The contents of the Authenticated Mesh Peering Exchange element in Mesh Peering Confirm frames for AMPE is describe to not include GTKdata field. However, there is no note here about the IGTKdata field which is supposed to be handled in the same way as the GTKdata field when management frame protection is enabled. In any case, IGTKdata field shall not be present in Mesh Peering Confirm frames. | On page 2139 line 43, insert the following item to the list after "The GTKdata field shall not be present": "-- The IGTKdata field shall not be present." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm.. My proposed action on this should not come as a surprise.

REVmc/D6.0 context (page 2139):

Proposed resolution for CID 8321:

Accepted.

# CID 8052

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.5.5.2.2 Page: 2139 Line: 15 | The rules for processing Mesh Peering Open frames for AMPE cover the receive MGTK SA, but not the receive IGTK SA. Both the MGTK and IGTK are received in this frame when management frame protection is used and as such, this subclause should really describe also the possibility of IGTK extraction. | On page 2139 line 16, insert following text at the end of the "The peer mesh STA's MGTK extracted" paragraph: "When management frame protection is used, the peer mesh STA's IGTK extracted from the Mesh Peering Open frame shall be added to the Receive IGTK SA in which the peer's MAC address equals the Authenticator MAC address." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm..

MarkR: What is a "Receive IGTK SA"?  Such a SA is not described in 12.6.1.1.

Jouni: “Receive IGTK SA” was trying to refer to the IGTKSA used for receiving frames from the specific peer in MBSS. 12.6.1.1 does not define this, but it does have Mesh GTKSA (and Mesh GTKSA description in 12.6.1.1.10 mentions “transmit mesh GTKSA” and “receive mesh GTKSA”). Maybe a Mesh IGTKSA should be defined similarly to this? Or simply talk about IGTKSA (of which there would be zero or more for each peer). It should be noted that the existing sentence is this paragraph talks about “Receive MGTK SA” (which is not described in 12.6.1.1, but is mentioned in its subclause 12.6.1.1.10). The comment proposed language that is similar to that for the “Receive … SA” part.

It is not critical to get this addressed in REVmc, so may be better to postpone to REVmd for a more complete cleanup.

Relevant REVmc/D6.0 text:

Page 2139:

Redline showing possible changes for CID 8052:

The peer mesh STA’s MGTK extracted from the Mesh Peering Open frame shall be added to the Receive MGTK SA in which the peer’s MAC address equals the MGTK Source mesh STA MAC address. When management frame protection is used, the peer mesh STA's IGTK extracted from the Mesh Peering Open frame shall be added as an IGTKSA in which the peer's MAC address equals the Authenticator MAC address and the Direction vector equals ‘receive’."

Proposed resolution for CID 8052:

Revised. On page 2139 line 16, insert the following text at the end of the "The peer mesh STA's MGTK extracted" paragraph:

"When management frame protection is used, the peer mesh STA's IGTK extracted from the Mesh Peering Open frame shall be added as an IGTKSA in which the peer's MAC address equals the Authenticator MAC address and the Direction vector equals ‘receive’."

Reject - out-of-scope (clean up in REVmd to cover receive/transmit mesh IGTK SA)

# CID 8006

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.5.7 Page: 2144 Line: 35 | The encoding of LinkIDs in the KDF input for deriving MTK is not very clear. The rules here describe how min/max of the unsigned integer values are to be used, but this does not describe how the integers are encoded (size and byte order). This could result in interoperability issues due to different possible interpretations of the standard. | Add to the end of page 2144 line 35 (after the ".. of the two unsigned integers." sentence): "In the KDF context, LinkIDs are encoded as 16-bit unsigned integers, represented using the bit ordering conventions of 9.2.2." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm.. My proposed action on this should not come as a surprise.

REVmc/D6.0 context (page 2144):

Proposed resolution for CID 8006:

Accepted.

# CID 8005

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 14.5.7 Page: 2144 Line: 16 | REVmc seems to have hardcoded 128 bits as the only acceptable length of MTK. This is not correct and would prevent use of pairwise ciphers that require different key length (e.g., CCMP-256 or GCMP-256). IEEE Std 802.11-2012 used a variable length KDF here (KDF-X) and REVmc/D5.0 used KDF-Hash-Length, but it looks like some "clean up" removed this in D6.0. | On page 2144 line 16, replace "KDF-Hash-128" with "KDF-Hash-Length". On page 2144 line 24, replace "KDF-Hash-128" with "KDF-Hash-Length". On page 2144 line 27, insert a new item before the "Selected AKM Suite" line: "Length is cipher-suite dependent and is defined by the TK_bits value in Table 12-4 (Cipher suite key lengths)." |

Discussion

Note that this is my own comment, so someone else might want to take a quick look to confirm..

MarkR: I'm not sure about this one.

The reason it was cleaned up after D5.0 is that D5.0 said:

MTK <- KDF-Hash-Length([...])

where KDF-Hash-Length is the key derivation function defined in 12.7.1.7.2 (Key derivation function (KDF)) using the hash algorithm defined by the AKM in Table 9-132 (AKM suite selectors) to generate an MTK of a specified length. Both CCMP and GCMP use Length = 128.

[802.11-2012 said:

MTK <- KDF-X([...])

CCMP uses X = 128.

so I guess there was some other comment between -2012 and D5.0 saying or leading to "what is it for GCMP?" and the answer was "also 128".]

So D6.0 is consistent with D5.0 and thence -2012.  Nothing was broken by D6.0.

Jouni: I disagree with that “nothing was broken by D6.0” part. D5.0 was indeed incomplete by mentioning Length value (= 128) only for some of the ciphers (CCMP and GCMP). It should really have listed values for all the possible ciphers. That said, it implied that the Length could be variable and I would claim that it would be a reasonable interpretation of the D5.0 text to use Length=256 for CCMP-256 and GCMP-256. The proposed changes in the comment is fixing this issue by providing the details for all ciphers with a reference to a table defining those lengths.

REVmc/D6.0 redline showing the change from D5.0:

REVmc/D6.0 context:

14.5.7, page 2144:

12.7.2 (EAPOL-Key frames), page 2006:

Proposed resolution for CID 8005:

Accepted.

# TOPIC: Public Action frame addressing

Discussion

This topic on what should be used as the BSSID field value in the header of Public Action frames came from some discussions related to interoperability and behavior of deployed devices that implement GAS/ANQP for network selection. While these comments may be out-of-scope by being on unmodified text, this area has known interoperability issues with deployed devices and as such, it would be valuable to clarify the situation in the IEEE 802.11 standard.

The specific case that is being discussed is the contents of the BSSID field in GAS frames that are sent by an unassociated non-AP STA to an AP. The current IEEE 802.11 standard requires the Wildcard BSSID to be used in such a case while there are implementations that use the BSSID of the AP. While the GAS Public Action frame in such a case is not between STAs in a BSS, it is directed to the specific BSS and not to all BSSes which is what the Wildcard BSSID could also be interpreted as meaning (e.g., Probe Request frames). There does not seem to be much harm in allowing a non-AP STA to use the specific BSSID of the target AP in the BSSID field during a GAS exchange before association with that AP. Similarly, the GAS response from the AP to a non-associated STA could allow the AP’s BSSID to be used instead of requiring Wildcard BSSID to be used in the BSSID field.

There are couple of alternatives on how to handle these three comments:

- accept changes proposed in CID 8326 and CID 8327, i.e., allow AP BSSID to be used, and reject CID 8051 (no note is needed anymore with the AP BSSID allowed); leave Wildcard BSSID as an allowed value to avoid making deployed (if any) implementation compliant

- reject changes proposed in CID 8326 and CID 8327, i.e., maintain requirement for Wildcard BSSID (which was added by 802.11n and is present in 802.11-2012) and accept CID 8051 to add the note to help implementors figure out how to produce interoperable devices; this expect vendors to fix their deployed devices over time

- reject all three CIDs with reasoning pointing out that the comments are out-of-scope for this recirculation (“The comment is out of scope:  i.e., it is not on changed text, text affected by changed text or text that is the target of an existing valid unsatisfied comment.”)

Rev 2: Updated redline to cover the option (1) and cleaned up language. Added proposed resolution to achieve (1).

Rev 3 + 4: Cleaned up redline text.

Context in P802.11/D6.0:

# CID 8051

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 11.2 Page: 1742 Line: 55 | The Public Action frame addressing rules described in the standard (originally from P802.11n) do not match the behavior in number of deployed devices and trying to use these rules can result in interoperability issues, e.g., due to a deployed device not replying to a GAS request. There is quite long history on why this happened due to parallel development of P802.11n and P802.11u and separate specifications using GAS, but anyway, the end result is somewhat inconvenient situation where one cannot follow the IEEE 802.11 standard requirement while being able to interoperate with already deployed devices. While there is ongoing effort in trying to get deployed devices and various uses of GAS to become more IEEE 802.11 standard compliant, it could be useful to add an informative note into the standard on this difference since the issue is not going to fully disappear any time soon (I'd expect this to take years at minimum).. | Add the following information note as a new paragraph at the end of 11.20 (page 1743 line 3): "NOTE--Some deployed devices and protocols outside the scope of this standard do not use the Wildcard BSSID value in the BSSID field (Address 3) in Public Action frames, e.g., with GAS. To interoperate with such devices, a STA might need to use matching behavior that would not be fully compliant with the rules described in this subclause." |

Discussion

See above.

Proposed resolution for CID 8051:

Revised. Adopt the redline changes for CID 8326/8327/8051 from <this document>.

# CID 8327

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 11.2 Page: 1743 Line: 1 | What is the reason that the BSSID field (address 3) of an individually addressed PA can not be the BSSID of the recipient's BSS, if known? | Add to the stat of the sentence, "Except as described in the next sentence, ... " Add a sentence to the end of the paragraph, "If the recipient STA is an AP, or is known to be associated to an AP, and the BSSID of the recipient STA's BSS is known, the BSSID field may be set to the recipient STA's BSS's BSSID." |

Discussion

See above.

Proposed resolution for CID 8327:

Revised. Adopt the redline changes for CID 8326/8327/8051 from <this document>.

# CID 8326

| Location | Comment | Proposed Change |
| --- | --- | --- |
| Clause: 11.2 Page: 1742 Line: 54 | What is the reason that the BSSID field (address 3) of an individually addressed PA can not be the BSSID of the recipient's BSS, if known? | Add "and whose currenttly associated BSS's BSSID is not known or doesn't exist" after "same BSS as the transmitting STA". Add a sentence to the end of the paragraph, "If the recipient STA is an AP, or is known to be associated to an AP, and the BSSID of the recipient STA's BSS is known, the BSSID field may be set to the recipient STA's BSS's BSSID." |

Discussion

See above.

Proposed resolution for CID 8326:

Revised. Adopt the redline changes for CID 8326/8327/8051 from <this document>.

Redline changes for CID 8326/8327/8051:

11.20 Public Action frame addressing

A STA that is a member of a BSS and that transmits an individually addressed Public Action frame to a STA that is not a member of the same BSS shall set the BSSID field of the frame as follows:

- If the recipient STA is known to be a member of another BSS (including being an AP) and that BSS’s BSSID is known, the BSSID field shall be set to the BSS’s BSSID or the wildcard BSSID value.

- Otherwise, the BSSID field of the frame shall be set to the wildcard BSSID value.

A STA that is a member of a BSS and that transmits a group addressed Public Action frame shall set the BSSID field of the frame to the BSS’s BSSID or the wildcard BSSID value.

A STA that is a member of a BSS and that transmits an individually addressed Public Action frame to a STA that is a member of the same BSS shall set the BSSID field of the frame to the BSS’s BSSID.

A STA that is not a member of a BSS and that transmits a Public Action frame shall set the BSSID field of the frame as follows:

- If the frame is individually addressed, the recipient STA is known to be a member of a BSS (including being an AP), and that BSS’s BSSID is known, the BSSID field shall be set to the BSS’s BSSID or the wildcard BSSID value.

- Otherwise, the BSSID field of the frame shall be set to the wildcard BSSID value.