IEEE P802.11
Wireless LANs

| Resolution to CID # 5001, 5004 and 5009 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-08-29 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| SK Yong | Apple Inc. | 6 Infinite Loop, Cupetino, CA, USA |  | skyong [at] apple.com |

Abstract

Revisiting CIDs: 5004, 5009, 5001.

| CID | Comment | Proposed Change |
| --- | --- | --- |
| 5001 | "The Service Combination field is present only if the Number of Available Services field is set to 0." In other words: it is only valid in a "query" type of operation. However, the Service Hash element is defined only for Beacon and ProbeResponse frames, which are used for a "response" type operation. | Remove Service Combination field and associated text from the Service Hash element, or allow the Service Hash element in a ProbeRequest frame, so the Service Combination field can be used in a "query" operation. |
| 5009 | the format of the Servicece Hash element is bizzare. Since the service hash is only included in Probe Response/Beacon, why the AP wants to indicate if the AP offered different combinations of different services (with "and" and "or" permutations). This added unnecessary complexity with no benifit | remove flags field and service combination fields and assocaited text |

Instruction to Editor

Update the section 9.4.2.217 as follows:

- 9.4.2.217 Service Hash element

2

3	The Service Hash element contains one or more service hashes and a logical function to interpret the combi-

4

- nation of service hashes. The format of the Service Hash element is shown in Figure 9-586m (Service Hash

- element format).

7

8

| 9 Element ID | Length | Element ID Extension | Flags | Service Hashes | Service Com- bination |
| --- | --- | --- | --- | --- | --- |

10

11

12	Octets:	1	1	1	2	variable	variable

13

14

- Figure 9-586m—Service Hash element format

16

17

18

19

20	The Element ID, Length and Element ID Extension fields are defined in 9.4.2.1 (General).

21

22	The Flags field is defined in Figure 9-586n (Flags field format).

23

24

| 25 B0 B5 | B6 B11 | B12 B15 |
| --- | --- | --- |
| Number of Included Ser- vices | Number of Available Ser- vices | Reserved |

26

27

28

29

30

31	Bits:	6	6	4

32

33

- Figure 9-586n—Flags field format

35

36

37

38

- The Number of Included Services field indicates the number of service hashes that are included in the ele-

- ment. This field is set to a nonzero value.

41

- A value of r for the Number of Available Services field indicates availability of   at most r services among

those specified by the service hashes included in the element. Any value of the Number of Available Ser-

44

- vices field that is equal to or greater than the value of the Number of Included Services field indicates avail-

- ability of all the services included in the element. The field is set to 0 only when providing a combination of

- services included in the element that cannot be described by an “any r-of-n” format.

48

49	The Service Hashes field contains one or more 6-octet service hash values. See 11.25a.4 (Service hash pro-

50

51	cedures) for procedures for generating a service hash used in the Service Hash element.

52

- The Service Combination field is present only if the Number of Available Services field is set to 0. The Ser-

- vice Combination field carries a service combination bitmap that is 2n bits in length, where n is the number

of service hashes in the element, and is organized into 2n/8octets such that bit number b (0 ≤ b < 2n) in the

56

- bitmap corresponds to bit number (b mod 8) in octet number b/8, where the low order bit of each octet is

- bit number 0, and the high order bit is bit number 7. The service combination bitmap is the sum-of-products

- representation of a boolean function of n boolean variables x1,....,xn where xi (i = 1,...,n) indicates search for

- the service corresponding to the i-th service hash included in the element. Specifically, bit b (0 ≤ b < 2n) in

the bitmap corresponds to minterm mb in a sum-of-products representation.

62

63

- NOTE—To illustrate the Service Combination field format consider a STA that provides service S1 or service S2 or both

- services S3 and S4, where services S1, S2, S3 and S4 appear in the Service Hash element in that order. The provided ser-

- vice combination can be represented by the boolean function x1 + x2 + x3.x4, or the sum of minterms m1, m2, m3, m5,

- m6, m7, m9, m10, m11, m12, m13, m14, and m15  using the sum-of-product representation. The resulting bitmap is

1111111011101110 binary, and the value of the Service Combination field is 0xFEEE.

| CID | Comment | Proposed Change |
| --- | --- | --- |
| 5004 | The below definition is flaw for the purpose of 11aq. "service information registry: A logical entity that contains caches of information about services that may be available via the BSS for the STA to connect to after it associates with the AP." The cached service information (at least some portion of them) shall be available prior to association rather than only available after the STA associated to the AP. . Similarly in subclause 4.5.9.1.3, this needs to be fix. | make changes to reflect the cahced info is available for preassocaition discovery |

Instruction to Editor

Update the definition of “service information registry” in section 9.4.2.217 as follows:

- service information registry: A logical entity that contains caches of information about services that is

- available via the BSS for the STA during pre-association and association state.