IEEE P802.11
Wireless LANs

| Proposed resolutions to comments on clause 26.3.10.8 in D0.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-6 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F1-17-A011S, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 | +86 18126392367 | ross.yujian@huawei.com |
| Kaushik Josiam | Samsung |  |  | k.josiam@samsung.com |

Abstract

This submission contains proposed comment resolutions to comments on D0.1.

The remaining comments assigned to the author in Clause 26.3.10.8 (D0.5) are:

(Note that this chapter was 26.3.9.8 in D0.1)

308, 1619, 2874

The changes marked in this document are based on TGax Draft 0.5.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# Comments for Clause 26.3.10.8

## CID 308

| CID | Page. Line | Clause Number | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 308 | 116.35 | 26.3.9.8.4 | In 80 and 160Mhz, it should be mentioned that the position of the user fields along with the RU allocation signaling indicate the specific RU allocated for that user. | Clarify mapping of user-fields to the RU allocation signaling for 80 and 160 MHz | REVISED Agree in principal. Make detail changes Instruction to Editor: Please make the following changes in TGax D0.5 P237 L41 under subclause 26.3.10.8.4 according to the changes in doc 16-1371r0 |

Instruction to Editor:

Please make the following changes in TGax D0.5 P237 L41 under subclause 26.3.10.8.4 according to the changes below:

The number of RU Allocation fields in the Common field depends on the PPDU bandwidth

- In the default mode when the SIGB Compression field in the HE-SIG-A field of an HE MU PPDU is set to 0, for a 20 MHz and a 40 MHz PPDU, each HE-SIG-B content channel contains one RU Allocation field in the Common field followed by multiple User fields. The position of the User field in the User Specific field together with the 8-bit RU Allocation field indicates the RU assignment to the user.

- In the default mode for an 80 MHz PPDU, each HE-SIG-B content channel contains two RU Allocation fields for a total of 16 bits of RU allocation signaling, one each for the RUs in the two 20 MHz segments of the HE-SIG-B content channel. The position of the User field in the User Specific field together with the 8-bit RU Allocation field indicates the RU assignment to the user. The User fields corresponding to the first RU Allocation field are followed by the User fields indicated by the second RU Allocation field in the User Specific field.

- In the default mode for a 160 MHz PPDU, each HE-SIG-B content channel contains four RU Allocation fields for a total of 32 bits of RU allocation signaling, one each for the RUs in the four 20 MHz segments of the HE-SIG-B content channel. The position of the User field in the Use Specific field together with the 8-bit RU Allocation field indicates the RU assignment to the user. The User fields for each of the 20 MHz segments in the content channel are arranged by the order in which their RU Allocation signaling fields appear in the Common field.(#2784)

## CID 1619

| 1619 | 118.12 | 26.3.9.8.4 | It says "reference to MAC section(?)" | Put a proper reference | REVISED The CID has already been resolved by CID 1003 in doc 11-16/928r2, and the changes are shown in D0.5. Instruction to Editor: Please refer to CID 1003 in doc 11-16/928r2 for the changes |
| --- | --- | --- | --- | --- | --- |

## CID 2874

| 2874 | 118.35 | 26.3.9.8 | Exactly "20MHz" can be applied to the both rules described here?specification information after deciding the TBDs. | If not, replace use "<" in L33 or replace "size 20MHz or greater" with "size greater than 20MHz" | REVISED The CID has already been resolved by CID 310 in doc 11-16/928r2, and the changes are shown in D0.5. Instruction to Editor: Please refer to CID 310 in doc 11-16/928r2 for the changes |
| --- | --- | --- | --- | --- | --- |