IEEE P802.11
Wireless LANs

| Proposed change |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ross Jian Yu | Huawei Technologies | F1-17-A011S, Huawei Base, Bantian, Longgang, Shenzhen, Guangdong, China, 518129 | +86 18126392367 | ross.yujian@huawei.com |
| John Jun Zhu | Huawei Technologies |  |  | zhujun75@huawei.com |
| Ming Gan | Huawei Technologies |  |  | ming.gan@huawei.com |

Abstract

This submission contains some proposed changes on D0.5.

The changes marked in this document are based on TGax Draft 0.5.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Delelted proposed change #1,7,8 as they have already passed the motion. |

## Proposed change 2

| 230.24 | 26.3.10.8.1 | The sentence "The last User Block field may contain information for one or two STAs depending on whether the number of users indicated by the RU allocation signaling is odd and depending on the signaling of the center 26-tone RU" is not so clear. Even in the case that the number of users indicated by the RU allocation signaling is even, the last User Block field can contain information for one or two STAs depending on the signaling of the center 26-tone RU. | Instruction to the editor: Replace the sentence: “The last User Block field may contain information for one or two STAs depending on whether the number of users indicated by the RU allocation signaling is odd and depending on the signaling of the center 26-tone RU” on L24 P230 by "The last User Block field may contain information for one or two STAs depending on the number of users indicated by the RU allocation signaling and the signaling of the center 26-tone RU" |
| --- | --- | --- | --- |

## Proposed change 3

| 231.52 | 26.3.10.8.2 | In Figure 26-23, “and center 26 tone RU” should be removed in HE‐SIG‐B Content Channel 2. | Instruction to Editor: Please refer to Proposed change 3 in doc 11-16/1372r1 for the changes |
| --- | --- | --- | --- |
| 231.46-56 | 26.3.10.8.2 | remove "and center 26 tone RU" in the user specific field of HE SIG B content channel 2. | Instruction to Editor: Please refer to Proposed change 3 in doc 11-16/1372r1 for the changes |
| 231.46-56 | 26.3.10.8.2 | add "(1-bit)" in common field of HE-SIG-B content channel 2. | Instruction to Editor: Please refer to Proposed change 3 in doc 11-16/1372r1 for the changes |

Instruction to Editor:

Please replace the below figure:

Figure 26-23—Default mapping of the two HE-SIG-B channels and their duplication in an 80 MHz PPDU

with the newly proposed figure:

Figure 26-23—Default mapping of the two HE-SIG-B channels and their duplication in an 80 MHz PPDU

## Proposed change 4

| 232.4 | 26.3.10.8.2 | In "The signaling for the presence of the user field corresponding to a center 26-tone RU in the 80 MHz segment with the lower sub-carrier index is carried in HE-SIG-B content channel 1 as a 1-bit sub-field placed at the end of the Common field. Similarly, signaling for the center 26-tone RU in the 80 MHz segment with the higher subcarrier index is carried in HE-SIG-B content channel 2 as 1-bit sub-field placed at the end of the common field.", the signaling for center 26-tone RU is not placed at the end of the Common field because of CRC and Tail Subfield. | Modify the sentence by "The signaling for the presence of the user field corresponding to a center 26-tone RU in the 80 MHz segment with the lower sub-carrier index is carried in HE-SIG-B content channel 1 as a 1-bit sub-field placed after the RU Allocation Subfield in the Common field. Similarly, signaling for the center 26-tone RU in the 80 MHz segment with the higher subcarrier index is carried in HE-SIG-B content channel 2 as 1-bit sub-field placed after the RU Allocation Subfield in the Common field." | Instruction to Editor: Please refer to Proposed change 4 in doc 11-16/1372r1 for the changes |
| --- | --- | --- | --- | --- |

Instruction to Editor:

Please make the changes on P232 L4 as shown below:

The signaling for the presence of the User field corresponding to a center 26-tone RU in the 80 MHz segment with the lower sub-carrier index is carried in HE-SIG-B content channel 1 as a 1-bit Center 26-tone RU field placed after the RU Allocation field in the Common field. Similarly, signaling for the center 26-tone RU in the 80 MHz segment with the higher subcarrier index is carried in HE-SIG-B content channel 2 as 1-bit Center 26-tone RU field placed after the RU Allocation field in the Common field. When assigned, the User field corresponding to the center 26-tone RU in the 80 MHz segments is carried as the last user field in their respective content channels.(#1692) When RUs greater than 242 subcarriers are signaled in the RU Allocation field in a portion of the bandwidth, the signaling is carried in both content channels placed in the order of the absolute sub-carrier index.

## Proposed change 5

| 242.21 | 26.3.10.8.5 | "For bandwidths larger than 20 MHz, the User block fields are split equitably between two HE-SIG-B content channels, i.e., for a k user MU-MIMO PPDU, 1,...,ceil(k/2) User block fields are carried in HE-SIG-B content channel 1 and ceil(k/2)+1,...,k User block fields in HE-SIG-B content channel 2." is contracted with "For an MU-MIMO allocation of RU size greater than 242 subcarriers, the User Block subfields are dynamically split between the two HE-SIG-B content channels (1/2) and the split is decided by the AP (on a per case basis)." in 232P43L. Need to clarify the condition clearly. | Instruction to Editor: Please refer to Proposed change 5 in doc 11-16/1372r1 for the changes |
| --- | --- | --- | --- |

Instruction to Editor:

Please make the changes on P242 L21 as shown below:

When SIGB Compression bit in HE-SIG-A of a HE-MU-PPDU is set to 1 (indicating full bandwidth MU-MIMO transmission), the number of STAs in the MU-MIMO group is indicated in the ‘SIGB Number of Symbols/Number of MU-MIMO users field’ in HE-SIG-A field.  When SIGB Compression bit in HE-SIG-A of a HE-MU-PPDU is set to 1, for bandwidths larger than 20 MHz, the User fields are split equitably between two HE-SIG-B content channels, i.e., for a k user MU-MIMO PPDU, 1, ….,  User fields are carried in HE-SIG-B content channel 1 and +1,……, k  User fields in HE-SIG-B content channel 2. (#2543)

Please make the changes on P242 L21 as shown below:

When SIGB Compression bit in HE-SIG-A of a HE-MU-PPDU is set to 0, for an MU-MIMO allocation of RU size greater than 242 subcarriers(#2546), the User fields are dynamically split between the two HE-SIG-B content channels (1/2) and the split is decided by the AP (on a per case basis). See 26.3.10.8.4 (HE-SIG-B common content) and 26.3.10.8.5 (HE-SIG-B per-user content)(#1006) for more details.

## Proposed change 6

| 216.13 | 26.3.10.5 | η_LSIG,k explenation should be moved to under Eq. (26-13), as it firsst appears there | Instruction to Editor: η_LSIG,k explenation should be moved to under Eq. (26-13), as it firsst appears there |
| --- | --- | --- | --- |