IEEE P802.11
Wireless LANs

| Proposed Text Changes to TXRXVECTOR in Clause 26.2.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ke Yao | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | yao.ke5@zte.com.cn |
| Bo Sun | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | Sun.bo1@zte.com.cn |
| Weimin Xing | ZTE Corporation | Xi’an, Shaanxi, China, 710114 |  | Xing.weimin@zte.com.cn |
| Bin Tian | Qualcomm |  |  |  |
| Lochan Verma | Qualcomm |  |  |  |
| Yongho Seok | Newracom |  |  |  |

Abstract

This submission proposes modifications to TXRXVECTOR in clause 26.2.2 in TGax Draft 0.5.

# Revision Notes

| R0 | Initial revision |
| --- | --- |
| R1 | Remove discussion 6 in R0 which is RSSI related text change, and renumber all of the discussion items. |

## Discussion 1

“Not present” on line 58 page 167 is not correct. For all HE PPDU, N_TX should indicate the number of transmit chains.

The motion passed at IEEE 802.11 F2F meeting in 2016.07, resolution is included in document 11-16-0813 r5, but editor copied the wrong text.

## Proposed change 1

TGax Editor: Please make the following change in line 58 on page 167 in 26.2.2 in TGax D0.5:

| N_TX | FORMAT is HE_SU or HE_MU or HE_EXT_SU or HE_TRIG | Indicates the number of transmit chains. | Y | N |
| --- | --- | --- | --- | --- |
|  | Otherwise | See corresponding entry in Table 21-1 |  |  |

## Discussion 2

Wrong reference to section in line 17 and 25 on page 168 for EXPANSION_MAT parameter.

## Proposed change 2

TGax Editor: Please make the following change in line 17 on page 168 in 26.2.2 in TGax D0.5:

Contains a vector in the number of selected subcarriers con-

taining feedback matrices as defined in 26.3.19.2 Beamforming feedback matrix V

based on the channel measured during the

training symbols of previous HE NDP PPDU.

TGax Editor: Please make the following change in line 25 on page 168 in 26.2.2 in TGax D0.5:

For each user, contains a vector in the number of all the sub-

carriers within the RU which is assigned to this user. The vec-

tor for each subcarrier contains feedback matrices as defined in

in 26.3.19.2 Beamforming feedback matrix V based on the

channel measured during the training symbols of previous HE

NDP PPDUs or VHT NDP PPDUs

## Discussion 3

For SNR parameter, the description for each HE PPDU type are kind of redundant, suggest modifying it in a clear way.

## Proposed change 3

TGax Editor: Please make the following change in line 18 on page 169 in 26.2.2 in TGax D0.5, and delete the rows of HE-MU AND HE-TRIG.

| SNR | FORMAT is HE_SU or HE_MU or HE_EXT_SU or HE_TRIG | For HE_SU or HE_EXT_SU PPDU, contains an array of average values of received SNR measurements for each spatial stream. For HE_MU PPDU, contains an array of average values of received SNR measurements for each spatial stream of the receiver. For HE_TRIG PPDU, contains an array of average values received SNR measurements for each spatial stream per user. SNR indications of 8 bits are supported. Average value of SNR shall be the sum of the decibel values of SNR per tone divided by the number of tones. | N | M |
| --- | --- | --- | --- | --- |
|  | Otherwise | See corresponding entry in Table 21-1 |  |  |

## Discussion 4

Suggest describing CQI parameter with more details and add section reference.

Since CQI is not reported on other frames except for HE NDP, delete the entry of non-HE PPDU cases.

## Proposed change 4

TGax Editor: Please make the following change in line 42 on page 169 in 26.2.2 in TGax D0.5:

| CQI | FORMAT is HE_SU and PSDU_LENGTH equals 0 | Contains an array of received per-RU average SNRs for each space-time stream, where each per-RU average SNR is the arithmetic mean of the SNR in decibels over a 26-tone RU as described in 9.4.1.65 (HE CQI-only Report field). | N | Y |
| --- | --- | --- | --- | --- |
|  | Otherwise | Not present | N | N |

## Discussion 5

For STBC parameter description,

A typo in “Set to 1 to that indicate STBC is used in the payload” should be “Set to 1 to indicate that STBC is used in the payload”

Suggest including all scenarios where STBC is not allowed. Therefore, two more conditions as follows should be added:

STBC is applied only for a single spatial stream (Nss = 1).

DCM is not used in conjunction with STBC

## Proposed change 5

TGax Editor: Please make the following change in line 13 on page 170 in 26.2.2 in TGax D0.5:

| STBC | FORMAT is HE_SU or HE_MU or HE_EXT_SU or HE_TRIG | Indicates if STBC is used. In an HE MU PPDU where each RU includes no more than 1 user: Set to 1 to indicate all RUs are STBC encoded in the payload Set to 0 to indicate all RUs are not STBC encoded in the payload In HE SU PPDU, HE extended range SU PPDU or HE trigger-based PPDU: Set to 1 to indicate that STBC is used in the payload Set to 0 to indicate that STBC is not used in the payload STBC is not applied in MU-MIMO RUs, in case of HE MUPPDU if any RU is assigned to more than 1 user; STBC is set to 0 to indicate all RUs are not STBC encoded in the payload. STBC is applied only for a single spatial stream (Nss = 1). DCM is not used in conjunction with STBC. | Y | Y |
| --- | --- | --- | --- | --- |
|  | Otherwise | See corresponding entry in Table 21-1 |  |  |

## Discussion 6

A typo of “or or”, modify to “or”

## Proposed change 6

TGax Editor: Please make the following change in line 31 on page 171 in 26.2.2 in TGax D0.5:

FORMAT is HE_SU or

HE_MU or HE_EXT_SU

or HE_TRIG

## Discussion 7

Channel bonding is known as preamble puncturing, modify it.

## Proposed change 7

TGax Editor: Please make the following change in line 35 on page 172 in 26.2.2 in TGax D0.5:

Indicates the channel width occupied by preamble, sup-

porting channel bonding. See the field of Bandwidth in Table

26-26—HE-SIG-A field of an HE MU PPDU

## Discussion 8

11ac has APEP_LENGTH as "O" in RXVECTOR. Follow the same convention.

## Proposed change 8

TGax Editor: Please make the following change in line 12 on page 173 in 26.2.2 in TGax D0.5:

| APEP_LENGTH | FORMAT is HE_SU or HE_MU or HE_EXT_SU or HE_TRIG | If equal to 0, indicates an HE NDP PPDU. If greater than 0, indicates the number of octets in the range 1 to 1048575 in the A-MPDU pre-EOF padding (see 10.13.2 (A-MPDU length limit rules)) carried in the PSDU. | MU | O |
| --- | --- | --- | --- | --- |
|  | Otherwise | See corresponding entry in Table 21-1 |  |  |

## Discussion 9

NUM_STS for RX should also start from 1. The reason is:

In .11ac, within a MU-MIMO group it is possible to allocate ‘0’ NUM_STS to a STA. However, .11ax has gotten away with the MU-MIMO group concept. Therefore, .11ax should not support ‘0’ for NUM_STS in RXVECTOR anymore.

In addition, suggest reordering the words of the text for easy reading

## Proposed change 9

TGax Editor: Please make the following change in line 44 on page 173 in 26.2.2 in TGax D0.5:

| NUM_STS | FORMAT is HE_SU | Indicates the number of space-time streams. Integer in the range 1-8 | Y | Y |
| --- | --- | --- | --- | --- |
|  | FORMAT is HE_EXT_SU | Indicates the number of space-time streams. Integer in the range 1-2. | Y | Y |
|  | FORMAT is HE_MU | Indicates the number of space-time streams. Integer in the range: 1-4 per user per MU-MIMO RU in TXVECTOR 1-4 per MU-MIMO RU in RXVECTOR 1-8 per non MU-MIMO RU in TXVECTOR and RXVECTOR NUM_STS summed over all users per RU is not greater than 8. | MU | Y |
|  | FORMAT is HE_TRIG | Indicates the number of space-time streams. Integer in the range: 1-4 for MU-MIMO RU in TXVECTOR 1-4 per user per MU-MIMO RU in RXVECTOR 1-8 for non MU-MIMO RU in TXVECTOR and RXVECTOR. NUM_STS summed over all users per RU is not greater than 8. | Y | MU |
|  | Otherwise | See corresponding entry in Table 21-1 (TXVECTOR and RXVECTOR parameters) |  |  |

## Discussion 10

Suggest clarify the position of B0 and B6, in the words of LSB and/or MSB.

## Proposed change 10

TGax Editor: Please add the following sentence in line 34 on page 174 in 26.2.2 in TGax D0.5:

NOTE—B1-B6 indicates an integer, where B1 is MSB.

## Discussion 11

Define enumeration for HE_LTF TYPE

## Proposed change 11

TGax Editor: Please make the following changes in line 38 on page 175 in 26.2.2 in TGax D0.5:

Indicates the type of HE-LTF.

Enumerated type:

1x_HE-LTF for 3.2 us

2x_HE-LTF for 6.4 us

4x_HE-LTF for 12.8 us

See 26.3.12.10 (HE-LTF) for details.

## Discussion 12

Define enumeration type for PE_DURATION

## Proposed change 12

TGax Editor: Please make the following changes in line 41 on page 176in 26.2.2 in TGax D0.5:

Determines the duration of PE field in an HE PPDU.

Possible values are 0 µs, 4 µs, 8 µs, 12 µs and 16 µs.

Enumerated type:

PE0 for 0µs

PE1 for 4 µs

PE2 for 8 µs

PE3 for 12 µs

PE4 for 16 µs