IEEE P802.11
Wireless LANs

| Proposed SB0 Comment Resolution Text for CID 6179 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yunsong Yang | Huawei Technologies | 10180 Telesis Court, STE 400, San Diego, CA 92121 | +1-858-754-3638 | yangyunsong@huawei.com |

Abstract

This document provides the 11aq Amendment Draft text for resolving CID 6179.

Insert the following new level-3 subclause under Clause 6.3.

#### 6. Layer Management

#### 6.3 MLME SAP interface

#### …

### 6.3.x Update

#### 6.3.x.1 Introduction

This mechanism supports the process of updating one or more parameters used in the BSS without re-starting the BSS.

#### 6.3.x.2 MLME-UPDATE.request

##### 6.3.x.2.1 Function

This primitive requests that the MAC entity to initiate a BSS update procedure to update one or more parameters used in the BSS without re-starting the BSS.

##### 6.3.x.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-START.request(

ServiceHint,

ServiceHast,

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ServiceHint | Service Hint element | As defined in 9.4.2.217 (Service Hint element) | Provides an indication of the services advertised in Beacon and Probe Response frames. The element is optionally present if dot11UnsolicitedPAD-Activated is true, and absent otherwise. |
| ServiceHash | Service Hash element | As defined in 9.4.2.218 (Service Hash element) | Specifies services advertised in Beacon and Probe Response frames. The element is optionally present if dot11UnsolicitedPAD-Activated is true, and absent otherwise. |
| VendorSpecificInfo | A set of elements | As defined in 9.4.2.26 (Vendor Specific element) | Zero or more elements. |

##### 6.3.x.2.3 When generated

This primitive is generated by the SME of an AP or PCP STA when one or more parameters used in the BSS are to be changed without re-starting the BSS.

##### 6.3.x.2.4 Effect of receipt

If the MLME of an AP or PCP STA receives an MLME-UPDATE.request primitive with a ServiceHint and/or ServiceHash parameters, the AP or PCP STA shall use these parameters to update the Service Hint element and/or Service Hash element in Beacon and Probe Response frames that the AP or PCP STA transmits.

#### 6.3.x.3 MLME-UPDATE.confirm

6.3.x.3.1 Function

This primitive reports the results of a BSS update procedure.

##### 6.3.x.3.2 Semantics of the service primitive

The primitive parameter is as follows:

MLME-UPDATE.confirm(

ResultCode

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, | Indicates the result of the MLME-UPDATE.request primitive. |

6.3.x.3.3 When generated

This primitive is generated by the MLME as a result of an MLME-UPDATE.request primitive to update one or more parameters used in the BSS without re-starting the BSS.

6.3.x.3.4 Effect of receipt

The SME is notified of the results of the BSS update procedure.