IEEE P802.11
Wireless LANs

| Service Hash Response ANQP-element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2017-01-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen McCann | BlackBerry Ltd | The Pearce Building, West Street, Maidenhead, Berkshire, SL6 1RL, UK | +44 1753 667099 | smccann@blackberry.com |

Abstract

This document proposes text changes to update the P802.11aq D7.0 for the following SBO CIDs:

6045, 6054, 6057, 6059, 6114

##### 9.4.5 Access Network Query Protocol (ANQP) elements

9.4.5.1 General

Insert new rows as follows to the end of the table, with corresponding adjustment to “Reserved” value:

Table 9-271—ANQP-element definitions

| ANQP-element name | Info ID | ANQP-element (subclause) |
| --- | --- | --- |
| Service Hash Request | 288 | 9.4.5.27 (Service Hash Request ANQP-element) |
| Service Information Request | 289 | 9.4.5.28 (Service Infor- mation Request ANQP- element) |
| Service Information Response | 290 | 9.4.5.29 (Service Infor- mation Response ANQP- element) |
| Service Hash Response | <ANA> | 9.4.5.30 (Service Hash Response ANQP-element) |

Insert the new subclause as follows:

##### 9.4.5.29 Service Information Response ANQP-element

The Service Information Response ANQP-element contains the detailed service information in response to a Service Information Request ANQP-element.

The format of the Service Information Response ANQP-element is shown in Figure 9-625j (Service Infor- mation Response ANQP-element format).

| Info ID | Length | Service Information Response Tuples |
| --- | --- | --- |

Octets:	2	2	variable

##### Figure 9-625j—Service Information Response ANQP-element format

The Info ID and Length fields are defined in 9.4.5.1 (General).

The Service Information Response Tuples field contains one or more Service Information Response Tuple subfields.

The format of the Service Information Response Tuple subfield is shown in Figure 9-625k (Service Informa- tion Response Tuple subfield format).

| Service Name Length | Service Name | Instance Name Length | Instance Name | Service Infor- mation Query Response Length | Service Informa- tion Query Response |
| --- | --- | --- | --- | --- | --- |

Octets:	1	variable	1	variable	2	variable

##### Figure 9-625k— Service Information Response Tuple subfield format

The Service Name Length subfield and the Service Name subfield are defined in 9.4.5.28 (Service Information Request ANQP-element).

The Instance Name Length subfield and the Instance Name subfield are defined in 9.4.5.28 (Service Information Request ANQP-element). The Instance Name Length subfield contains a nonzero value.

The Service Information Query Response Length subfield is the length of the Service Information Query Response subfield. If the Service Information Query Response Length subfield is equal to 0, the Service Information Query Response subfield is not included.

The Service Information Query Response subfield is a variable length field. The content of the Service Information Query Response subfield is service-specific based on the requested service information and is specified in 11.25a.3 (Solicited PAD procedure).

Insert new subclause as follows:

##### 9.4.5.30 Service Hash Response ANQP-element

The Service Hash Response ANQP-element contains the detailed service information in response to a Service Hash Request ANQP-element.

The format of the Service Hash Response ANQP-element is shown in Figure 9-625xx (Service Hash Response ANQP-element format).

| Info ID | Length | Service Hash Response Tuples |
| --- | --- | --- |

Octets:	2	2	variable

##### Figure 9-625xx—Service Hash Response ANQP-element format

The Info ID and Length fields are defined in 9.4.5.1 (General).

The Service Hash Response Tuples field contains one or more Service Hash Response Tuple subfields.

The Service Hash Response Tuples field is defined as in Figure 9-625l

| Service Name Length | Service Name | Instance Name Length | Instance Name |
| --- | --- | --- | --- |

Octets:	1                    variable	      1               variable

##### Figure 9-625l— Service Hash Response Tuple subfield format

The Service Name Length subfield and the Service Name subfield are defined in 9.4.5.28 (Service Information Request ANQP-element).

The Instance Name Length subfield and the Instance Name subfield are defined in 9.4.5.28 (Service Information Request ANQP-element).

## 11. MLME

#### 11.25 WLAN interworking with external networks procedures

##### 11.25.3 Interworking procedures: generic advertisement service (GAS)

11.25.3.2 ANQP procedures 11.25.3.2.1 General

Insert new rows to table as follows:

Table 11-15—ANQP usage

|  |  |  | BSS |  | IBSS |
| --- | --- | --- | --- | --- | --- |
| ANQP-element name | ANQP-element (subclause) | ANQP- element type | AP | Non-AP and non-PCP STA | STA |
| Service Hash Request | 9.4.5.27 (Service Hash Request ANQP- element) | Q | R | T | — |
| Service Information Request | 9.4.5.28 (Service Information Request ANQP- element) | Q | R | T | — |
| Service Information Response | 9.4.5.29 (Service Information Response ANQP-element) | S | T | R | — |
| Service Hash Response | 9.4.5.30 (Service Hash Response ANQP- element) | S | T | R | — |

##### 11.25a.3 Solicited PAD procedure

When dot11SolicitedPADActivated is true, a non-AP and non-PCP STA may transmit to an AP or PCP a Service Hash Request ANQP-element. This element includes one or more service hashes generated from the service name(s) of the service(s) that the non-AP and non-PCP STA is searching, as well as valid combina- tions of services of interest. An AP or PCP might advertise support for the Solicited PAD procedure by set- ting the Solicited PAD field of the Extended Capabilities element to 1 in its Beacon and Probe Response frames.

When dot11SolicitedPADActivated is true, an AP or PCP shall use the information from the Service Hash Request ANQP-element (that it receives from a non-AP and non-PCP STA) to determine if it can provide the requested service(s) or combination of services. Determination is based on the service hash values in the Service Hashes field of the received Service Hash Request ANQP-element, and valid service combinations specified through the Flags and Service Combination fields of the Service Hash Request ANQP-element. If the AP or PCP determines that it can provide the requested service(s) or combination of services, it shall respond by transmitting a Service Hash Response ANQP-element that contains a Service Information Response Tuple subfield for each service that satisfies the request.

NOTE—For example, an AP or PCP that receives a Service Hash Request ANQP-element frame that includes hash val- ues for 4 services S1, S2, S3 and S4 (in that order) and a value of 0xFEEE in its Service Combination field, responds to the request if and only if it can provide service S1 or service S2 or both services S3 and S4. The Service Hash Response ANQP-element can contain Service Information Response Tuple subfields for any set of available services that satisfy the ANQP request, e.g., S1, S3 and S4.

The requesting non-AP and non-PCP STA shall process the Service Hash Response ANQP-element in the received ANQP response to select a service combination that satisfies the non-AP and non-PCP STA request.

If there is a matching service name, the non-AP and non-PCP STA might decide to proceed with the service information procedure (see 11.25.3.2.16 (Service information procedure) or authentication and association procedure (see 11.3 (STA authentication and association)) depending on the nature of the service (see exam- ples illustrated in W.1 (Pre-association discovery usage scenarios)).