IEEE P802.11
Wireless LANs

| Scalable Beamforming Training |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2016-12-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| James Wang | MediaTek |  |  | james.wang@mediatek.com |

Abstract

This document provides the draft text for the scalable beamforming training.

    - Scalable Beamforming Train

The scalable beamforming training allows an initiator to simultaneously train a large number of responder STAs with antenna pattern reciprocity in a single phase. In the scalable beamfroming training, the initiator trains its transmit antennas/sectors while multiple responders train their receive antennas/sectors. The responders can determine the initiator’s transmit antenna/sector which was received with the best quality and, with antenna pattern reciprocity, the responders can also determine the corresponding transmit antenna/sector with best quality. Since the scalable beamforming training involves only the initiator transmitting training frames/fields, it is independent of the number of responders. Generally, the number of receive TRN fields used in  the scalable beamforming training is determined by the maximum number of receive sectors/antennas among all responders.

The scalable beamforming training can be enabled by appending TRN-R to beacon for TXSS by a AP or PCP and for multiple STAs to perform receive antenna training. It can also be enabled by using BRP frames with TRN fields that sweep through all TX antennas with training fields RX sectors where the number RX TRN should be max of RX sectors from all intended STAs.

A scalable beamforming training allocation during BTI or DTI is scheduled ScalableScalablethrough an scalable beamforming information element. A PCP or AP may schedule the periodic scalable beamforming training which occurs every integer number of beacon intervals at a fixed time offset to TBTT by transmitting a scalable beamforming information element in probe response, association response, or other frame.

2.3.x Scalable Beamforming Information Element

Field

Number

of bits

Start bit

Description

ABT_N_BI

4

3

Indicate the interval, in number of beacon intervals, at which the scalable

beamforming

training occurs

ABT_Next_BI

4

7

Indicate the number of beacon interval in which the scalable

beamforming

training is not

present

ABT_TBTT_Offset

32

8

Indicate the offset relative to the TBTT in microseconds. A value of 0

indicate that the

sclable

beamforming

training occurs at BTI with the TRN field appended to the end of the beacon